TECHNICAL EVALUATION

&

PRELIMINARY DETERMINATION

APPLICANT

National Aeronautics and Space Administration (NASA)

Kennedy Space Center (KSC)

Mail Code: AA-B

Kennedy Space Center, FL 32899

Kennedy Space Center

Facility ID No. 0090051

PROJECT

Project No. 0090051-032-AC

Application for Air Construction Permit

Construction Permit Modification

COUNTY

Brevard County, Florida

PERMITTING AUTHORITY

Florida Department of Environmental Protection

Permitting and Waste Cleanup Programs

Central District Office

Orlando, Florida 32803-3767

November, 2016

- General Project INFORMATION

Air Pollution Regulations

Projects at stationary sources with the potential to emit air pollution are subject to the applicable environmental laws specified in Section 403 of the Florida Statutes (F.S.).  The statutes authorize the Department of Environmental Protection (Department) to establish regulations regarding air quality as part of the Florida Administrative Code (F.A.C.), which includes the following applicable chapters:  62-4 (Permits); 62-204 (Air Pollution Control – General Provisions); 62-210 (Stationary Sources – General Requirements); 62-212 (Stationary Sources – Preconstruction Review); 62-213 (Operation Permits for Major Sources of Air Pollution); 62-296 (Stationary Sources - Emission Standards); and 62-297 (Stationary Sources – Emissions Monitoring).  Specifically, air construction permits are required pursuant to Chapters 62-4, 62-210 and 62-212, F.A.C.

In addition, the U. S. Environmental Protection Agency (EPA) establishes air quality regulations in Title 40 of the Code of Federal Regulations (CFR).  Part 60 specifies New Source Performance Standards (NSPS) for numerous industrial categories.  Part 61 specifies National Emission Standards for Hazardous Air Pollutants (NESHAP) based on specific pollutants.  Part 63 specifies NESHAP based on the Maximum Achievable Control Technology (MACT) for numerous industrial categories.  The Department adopts these federal regulations in Rule 62-204.800, F.A.C.

Glossary of Common Terms

Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of this permit.

Facility Description and Location

The National Aeronautics and Space Administration (NASA) operates the Kennedy Space Center (KSC) facility.  The facility is categorized under Standard Industrial Classification Code No. 9661 and North American Industry Classification System Code No. 927110. The facility is located in Brevard County at Kennedy Space Center, Florida. NASA KSC is a spacecraft and payload processing and launch facility which contains the following four categories of permitted air emissions units (EUs):

- Hot Water Generators/Boilers With An Individual Heat Input Rating ≥ 10 MMBtu/hr

- Permitted Surface Coating Operations

- Internal Combustion Engines used for both emergency and non-emergency purposes

- Chromate Conversion Operation

- Portable Materials Crushing Operation, which provides for a portable aggregate crushing plant (that will have its own air permit  per Rule 62-210.310(2), F.A.C.) to be occasionally used at the facility.

The NASA KSC facility also contains miscellaneous insignificant emissions units and/or activities.

Facility Regulatory Categories

- The facility is not a major source of hazardous air pollutants (HAP).

- The facility does not operate units subject to the acid rain provisions of the Clean Air Act (CAA).

- The facility is a Title V source of air pollution in accordance with Chapter 62-213, F.A.C.

- The facility is not a major stationary source in accordance with Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

- The facility operates units subject to the Standards of Performance for New Stationary Sources (NSPS)

- The facility operates units subject to the National Emission Standard for Hazardous Air Pollutants (NESHAP)

- Project Description

- This permitting creates a clean new air construction permit for the facility which re-permits the facility with this permit, replacing all previously issued air construction permits for the facility.  The permit reorganize emissions units and incorporate the latest NSPS and NESHAP requirements.  This Construction Permit is being processed concurrently with Title V Air Operation Permit Renewal project 0090051-033-AV for this facility in order to incorporate the conditions of the new Construction Permit into the renewed Title V Permit.

- Processing Schedule

9/23/16	Received application for a major source air pollution construction permit.

09/14/16	Application determined complete.

- PSD Applicability

The project is located in Brevard County which is in an area that is currently in attainment with the AAQS.  For areas currently in attainment with the AAQS or areas otherwise designated as unclassifiable, the Department regulates major stationary sources of air pollution in accordance with Florida’s PSD preconstruction review program as defined in Rule 62-212.400, F.A.C.  Under preconstruction review, the Department first must determine if a project is subject to the PSD requirements (“PSD applicability review”) and, if so, must conduct a PSD preconstruction review.  The facility and the proposed project is not subject to the PSD requirements; therefore, the project is not subject to a PSD preconstruction review.

- Department review

Brief Discussion of Emissions

As provided by the applicant, annual potential emissions are expected to remain below the following values:

| Emissions unit1 | Annual Emission (tons per year) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | CO | NOx | PM10 | PM2.5 | SOx | VOC | HAPs |
| 100 – Hot Water generators/boilers | 14.35 | 8.54 | 1.3 | 1.3 | 0.1 | 0.94 | 0.32 |
| 101 – Surface Coating Operations | - | - | 5.17 | 5.17 | - | 31.03 | 9.85 |
| 102 – Non- emergency RICE NESHAP Engines | 0.35 | 33.08 | 0.4 | 0.4 | 0.02 | 0.65 | 0.03 |
| 103 – Emergency NSPS Engines | 2.24 | 7.39 | 0.16 | 0.16 | 0.01 | 0.11 | 0.01 |
| 104 – Non-emergency NSPS Engines | 1.33 | 0.82 | 0.05 | 0.05 | 0.002 | 0.11 | 0.003 |
| 105 – Emergency NSPS Fire Pumps | 1.87 | 2.34 | 0.10 | 0.08 | 0.003 | 0.08 | 0.01 |
| 106 – Chromate Conversion Operations | - | - | Negligible | Negligible |  | Negligible | Negligible |
| 107 – Portable Materials Crushing Operation | 0.01 | 0.04 | 0.45 | - | - | 0.02 | 0.002 |
| Insignificant Units2 | 45.60 | 64.90 | 11.72 | 11.72 | 0.28 | 23.13 | 2.08 |
| Total | 65.74 | 122.10 | 19.34 | 18.9 | 0.42 | 56.07 | 12.3 |

- As provided in Section 11 of the application, emissions as tabulated above are based on the following assumptions:

- EU 100 – Hot Water Generators/Boilers – Assumption that two out of the three, 20 MM/BTU/hr boilers are operating for 8,712 hours per year firing natural gas, and all three boilers are operating for 48-hours per year on fuel oil.  Assumption of AP 42- emission factors from Subsection 1.4, Tables 1.4-1 and 1.4-2 in units of pounds per 1,000-gallons and pounds per million feet of fuel oil and natural gas burned.  Heat input value of 1020 Btu/cubic foot for natural gas and 140 MMBtu/per 100 gallons for fuel oil.

- EU 101 – Surface Coating Operations – Assumption that ten surface coating booths are in use for 1-shift per day and 52-weeks per year. Maximum annual paint and solvent usage taken from calendar year 2008 usage (the maximum material usage during the past 10 years).  An additional 25% was used to assume maximum potential emissions.

- EU 102 and EU 104 – Non-emergency Engines (13 engines) – Emissions were calculated based on 250 hours operation per year, assuming 200 hours at 100% load (150 hours for CILC plus 50 other use hours), and 50 additional maintenance hours based on the applicable maintenance requirements.  The assumed 250 hours per year usage of each engine is based on a review of site-specific operational records, CILC agreement requirements, historic usage data, and applicable maintenance job plans.

- EU 103 and EU 105 – Emergency Engines (53 engines) – Emissions were calculated based on 150 hours per year (using the RICE NESHAP and NSPS IIII emergency engine threshold of 100 hours per year for non-emergency use, plus an additional 50 hours emergency use).  The assumed 150 hours per year usage of each engine is based on a review of site-specific operational records, historic usage data, and applicable maintenance job plans

Emission factors for each engine (both emergency and non-emergency) are either:

- engine manufacturer cut sheet data when available,

- if no manufacturer data is available, EPA emission limits for the Engine Tier based on year of manufacture, or

- for Tier 0 engines EPA AP-42 emission factors based on engine size.

- EU 106 - Chromate Conversion Operations – Emissions from this operation are considered negligible.  The immersion tank is not heated, uses mechanical agitation with a closed tank cover in place, and tank is kept covered with a tight tank cover when the process is idle.

- EU 107 - Portable Materials Crushing Operations – This operation would be conducted by a portable crushing operation which would have its own relocatable air permit.  Emissions were estimated using an assumed maximum hourly amount of material processed of 200 tons per hour and the 250 hours as the maximum time that one instance of material crushing would take place at NASA KSC in any year.

- As provided in Section 11 of the application, emissions for insignificant sources include emissions from the following insignificant sources:

- Boilers/hot water heaters rated at < 10 MMBtu/hr (83 units)

- Unpermitted Coating Operations (three outside areas which conduct coating operations)

- Insignificant Compression Ignition Engines (30 engines that were either manufactured before April 1, 2006 (July 1, 2006 for fire pump engines) or modified or reconstructed before July 11, 2005).  These engines are exempt from RICE NESHAP Subpart ZZZZ requirements since NASA KSC meets the definition of an institution.

- Abrasive Blasting Operations (four, fully-enclosed, abrasive blasting operations) that are currently active.

- Hypergol Servicing (loading hypergolic propellants into various payloads and satellites which are prepared for launch).  Venting systems with scrubbers are used to remove residual vapors emitted during transfer operations or to capture emissions in the event of an accidental spill or release)

- Fog Fluid Use (Theatrical fog is used during various shows at NASA KSC’s Visitor Center). Maximum annual usage of the fog fluid concentrate is two, 55-gallon drums per year.

- Fuel Storage and Transfer from five gasoline tanks and 86 diesel tanks in use at NASA KSC.

- Ethylene Oxide from one ethylene oxide sterilizer in operation at NASA KSC.

- Propane Flares used at NASA KSC to prevent excess hydrogen from collecting under rockets during a launch.   The average amount of propane burned per launch was calculated for years 2007 through 2011, which had multiple launches plus two standard deviations was used as a maximum amount of fuel burned per launch. AP-42 emission factors were used to calculate annual emissions.

- Chemical Usage (facility-wide), scaled up to 1 shift per day, 52 weeks per year.

- Remediation Activities from 11 different remediation activities at NASA KSC.

The facility wide emissions and discussion in the Application reviewed by the Department provide the basis for the Department’s determination that based on the capacity and operating schedule, the facility-wide emissions will remain less than the potential emissions tabulated above.  Compliance can be demonstrated by applicable monitoring and recordkeeping requirements contained in the permit.

State Requirements

This project is subject to the preconstruction review requirements of Chapter 403, Florida Statutes, and Chapters 62-204 through 62-297, Florida Administrative Code (F.A.C.), as indicated below.

| Subject to: | Y/N | Comments |
| --- | --- | --- |
| Rule 62-210.300, F.A.C. – Stationary Source, General Requirements | Y | Not exempt from permitting requirements |
| Rule 62-212.400, F.A.C. – Prevention of Significant Deterioration | N | The facility is not PSD major source |
| Rule 62-296.320(4), F.A.C. - General Particulate Emission Limiting Standards | Y | Operations are a source of unconfined particulate matter emissions. |
| Rules 62-296.320(1) and (2), F.A.C. - General Pollutant Emission Limiting Standards (VOCs and Odor) | Y | Operations are a source of VOC and odors. |
| Rule 62-296.400, F.A.C. - Stationary Source Emission Standards | N | There is no applicable source category |
| Rule 62-296.400 (10), F.A.C. - Best Available Control Technology (BACT) | N | There is no applicable source category |
| Rule 62-296.700, F.A.C. - Reasonably Available Control Technology (PM) | N | There is no applicable source category. |
| Rule 62-204.800, F.A.C. - Standards of Performance for New Stationary Sources (NSPS) | Y | Emissions Units EU 103, 104, and 105 are subject to 40 CFR 60, NSPS Subpart IIII, Standards of Performance for Stationary Compression Ignition Internal Combustion Engines for Stationary CI RICE |
| Rule 62-204.800, F.A.C. National Emission Standard for Hazardous Air Pollutants (NESHAPS – 40 CFR 61 and 63) | Y | Emissions Unit EU 102 is subject to 40 CFR 63, Subpart ZZZZ -National Emissions Standards for Hazardous Air Pollutants for Stationary Compression Ignition Internal Combustion Engines. Emissions Unit EU 106 is subject to 40 CFR 63, Subpart WWWWWW -National Emissions Standards for Hazardous Air Pollutants for Plating and Polishing Operations |
| Rule 62-213, F.A.C. – Operation Permits for Major Source of Air Pollution | Y | Facility is a Title V source |

Federal NSPS Provisions

The Facility operates emissions units subject to 40 CFR 60, NSPS Standards.

Emissions Units EU 103, 104, and 105 are subject to NSPS Subpart IIII, Standards of Performance for Stationary Compression Ignition Internal Combustion Engines for Stationary CI RICE.

Emissions Unit 107 is subject to 40 CFR 60, Subpart OOO, Nonmetallic Mineral Processing Plants.

Federal NESHAP Provisions

The Facility operates emissions units subject to 40 CFR 63, NESHAP Standards.

Emission Units EU 102 and 104 are subject to 40 CFR 63, Subpart ZZZZ - National Emissions Standards for Hazardous Air Pollutants for Stationary Compression Ignition Internal Combustion Engines.

Emission Units EU 106 is subject to 40 CFR 63, Subpart WWWWWW - National Emissions Standards for Hazardous Air Pollutants for Plating and Polishing Operations.

Other Draft Permit Requirements

The emission limits proposed by the applicant will meet all of the requirements of Chapters 62-204 through 297, F.A.C.

The General and Specific Conditions listed in the proposed permit (attached) will assure compliance with all the applicable requirements of Chapters 62-204 through 297, F.A.C.

- Preliminary Determination

The Department makes a preliminary determination that the proposed project will comply with all applicable state and federal air pollution regulations as conditioned by the draft permit.  This determination is based on a technical review of the complete application, reasonable assurances provided by the applicant, and the conditions specified in the draft permit.  No air quality modeling analysis is required because the project does not result in a significant increase in emissions.  Stephen Amirault is the project engineer responsible for reviewing the application and drafting the permit.  Additional details of this analysis may be obtained by contacting the project engineer Stephen Amirault, at the Florida Department of Environmental Protection, Waste Air and Stormwater Permitting Program, Central District Office, 3319 Maguire Blvd., Suite 232, Orlando, Florida 32803-3767, 407/897-4100, or via email at Stephen.amirault@dep.state.fl.us