TECHNICAL EVALUATION

&

PRELIMINARY DETERMINATION

APPLICANT

Owens Corning

1035 Talleyrand Ave

Jacksonville, Florida 32206

Jacksonville Plant

Facility ID No. 0310050

PROJECT

Project No. 0310050-024-AC

Application for Minor Source Air Construction Permit

(EU002) Limestone Transfer System (Upper Surge Hopper) new baghouse

COUNTY

Duval County, Florida

PERMITTING AUTHORITY

Florida Department of Environmental Protection

Northeast District Office

8800 Baymeadows Way West, Suite 100

Jacksonville, Florida 32256

September 28, 2016

- General Project INFORMATION

  - Air Pollution Regulations

Projects at stationary sources with the potential to emit air pollution are subject to the applicable environmental laws specified in Section 403 of the Florida Statutes (F.S.).  The statutes authorize the Department of Environmental Protection (Department) to establish regulations regarding air quality as part of the Florida Administrative Code (F.A.C.), which includes the following applicable chapters:  62-4 (Permits); 62-204 (Air Pollution Control – General Provisions); 62-210 (Stationary Sources – General Requirements); 62-212 (Stationary Sources – Preconstruction Review); 62-213 (Operation Permits for Major Sources of Air Pollution); 62-296 (Stationary Sources - Emission Standards); and 62-297 (Stationary Sources – Emissions Monitoring).  Specifically, air construction permits are required pursuant to Chapters 62-4, 62-210 and 62-212, F.A.C.

In addition, the U. S. Environmental Protection Agency (EPA) establishes air quality regulations in Title 40 of the Code of Federal Regulations (CFR).  Part 60 specifies New Source Performance Standards (NSPS) for numerous industrial categories.  Part 61 specifies National Emission Standards for Hazardous Air Pollutants (NESHAP) based on specific pollutants.  Part 63 specifies NESHAP based on the Maximum Achievable Control Technology (MACT) for numerous industrial categories.  The Department adopts these federal regulations in Rule 62-204.800, F.A.C.

  - Glossary of Common Terms

Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of this permit.

  - Facility Description and Location

Jacksonville Plant is an existing roofing production and asphalt processing facility, which is categorized under Standard Industrial Classification Code No. 2952.  The existing Jacksonville Plant is located in Duval County at 1035 Talleyrand Avenue in Jacksonville, Florida.  The UTM coordinates of the existing facility are Zone 17, 439.5 km East and 3356.2 km North.  Latitude: 30º 20` 14.9675" North and Longitude: 81º 37` 48.2012" West.  This site is in an area that is in attainment (or designated as unclassifiable) for all air pollutants subject to Ambient Air Quality Standards (AAQS).

FIGURE 1: LOCATION DUVAL COUNTY, FLORIDA 				FIGURE 2: AERIAL VIEW

  - Facility Regulatory Categories

- The facility is not a major source of hazardous air pollutants (HAP).

- The facility is a Title V major source of air pollution in accordance with Chapter 62-213, F.A.C.

- The facility is not a major stationary source in accordance with Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

The Jacksonville plant is a Title V major source for carbon monoxide (CO), sulfur dioxide (SO2), volatile organic

compounds (VOC), and particulate matter (PM) with an aerodynamic diameter of 10 microns or less (PM10) as

emissions of these pollutants exceed the major source threshold of 100 tons per year (TPY). However, the plant

is a synthetic minor source of hazardous air pollutants (HAP) with limitations such that individual HAP

emissions are less than 10 TPY and combined HAP emissions are less than 25 TPY.

Project Description

- Air construction permit, which authorizes replacement of an existing baghouse controlling the emissions from the (EU002) Limestone Transfer System (Upper Surge Hopper).

The facility proposes to replace an existing baghouse controlling PM emissions from EU002 at the Jacksonville plant.  Exhaust from EU002 are currently controlled by a bin vent dust collector.  Owens Corning is proposing to replace the existing baghouse with a new, larger baghouse rated with a maximum air flow rate of 2,370 acfm1.

There will be no increase in production rate or other process throughputs as a result of the proposed project.

However, since the air exhaust flow rate is increasing, the maximum allowable emissions of PM will increase as

- a result of the proposed project.

- 1This represents an air-to-cloth ratio of 5:1. See page 9 (Process Dust Collector) for manufacturer drawings.

  - Processing Schedule

9/1/2016	Received the application for a minor source air pollution construction permit.

PROCESS FLOW DIAGRAM (A-1)

PROCESS FLOW DIAGRAM (A-2)

PROCESS FLOW DIAGRAM (A-3)

PROCESS FLOW DIAGRAM (A-4)

PROCESS FLOW DIAGRAM (A-5)

- PSD Applicability

General PSD Applicability

For areas currently in attainment with the AAQS or areas otherwise designated as unclassifiable, the Department regulates major stationary sources of air pollution in accordance with Florida’s PSD preconstruction review program as defined in Rule 62-212.400, F.A.C.  Under preconstruction review, the Department first must determine if a project is subject to the PSD requirements (“PSD applicability review”) and, if so, must conduct a PSD preconstruction review.  A PSD applicability review is required for projects at new and existing major stationary sources.  In addition, proposed projects at existing minor sources are subject to a PSD applicability review to determine whether potential emissions from the proposed project itself will exceed the PSD major stationary source thresholds.  A facility is considered a major stationary source with respect to PSD if it emits or has the potential to emit:

- 5 tons per year or more of lead;

- 250 tons per year or more of any regulated air pollutant; or

- 100 tons per year or more of any regulated air pollutant and the facility belongs to one of the following 28 PSD-major facility categories:  fossil fuel-fired steam electric plants of more than 250 million British thermal units per hour heat input, coal cleaning plants (with thermal dryers), Kraft pulp mills, Portland cement plants, primary zinc smelters, iron and steel mill plants, primary aluminum ore reduction plants, primary copper smelters, municipal incinerators capable of charging more than 250 tons of refuse per day, hydrofluoric, sulfuric, and nitric acid plants, petroleum refineries, lime plants, phosphate rock processing plants, coke oven batteries, sulfur recovery plants, carbon black plants (furnace process), primary lead smelters, fuel conversion plants, sintering plants, secondary metal production plants, chemical process plants, fossil fuel boilers (or combinations thereof) totaling more than 250 million British thermal units per hour heat input, petroleum storage and transfer units with a total storage capacity exceeding 300,000 barrels, taconite ore processing plants, glass fiber processing plants and charcoal production plants.

Once it is determined that a project is subject to PSD preconstruction review, the project emissions are compared to the “significant emission rates” defined in Rule 62-210.200, F.A.C. for the following pollutants:  carbon monoxide (CO); nitrogen oxides (NOX); sulfur dioxide (SO2); particulate matter (PM); particulate matter with a mean particle diameter of 10 microns or less (PM10); particulate matter with a mean particle diameter of 2.5 microns or less (PM2.5); volatile organic compounds (VOC); lead (Pb); fluorides (F); sulfuric acid mist (SAM); hydrogen sulfide (H2S); total reduced sulfur (TRS), including H2S; reduced sulfur compounds, including H2S; municipal waste combustor organics measured as total tetra- through octa-chlorinated dibenzo-p-dioxins and dibenzofurans; municipal waste combustor metals measured as particulate matter; municipal waste combustor acid gases measured as SO2 and hydrogen chloride (HCl); municipal solid waste landfills emissions measured as non-methane organic compounds (NMOC); and mercury (Hg).  In addition, significant emissions rate also means any emissions rate or any net emissions increase associated with a major stationary source or major modification which would construct within 10 kilometers of a Class I area and have an impact on such area equal to or greater than 1 μg/m3, 24-hour average.

If the potential emission equals or exceeds the defined significant emissions rate of a PSD pollutant, the project is considered “significant” for the pollutant and the applicant must employ the Best Available Control Technology (BACT) to minimize the emissions and evaluate the air quality impacts.  Although a facility or project may be major with respect to PSD for only one regulated pollutant, it may be required to install BACT controls for several “significant” regulated pollutants.

PSD Applicability for Project

The project is located in Duval County which is in an area that is currently otherwise designated as unclassifiable for the air pollutants particulate matter less than or equal to ten (10) micrometers, sulfur dioxide, and lead; and in an air quality maintenance area for ozone, and in the area of influence of an air quality maintenance area for particulate matter pursuant to Chapter 62-204, FAC, and Rule 2.201, Jacksonville Environmental Protection Board (JEPB).  The plant is presently a minor source with respect to the Prevention of Significant Deterioration (PSD) program since potential regulated pollutant emissions are less than 250 TPY each.  The proposed project will not increase emissions for any PSD pollutant; therefore, the project is not subject to a PSD preconstruction review.

- INFORMATION PROVIDED BY THE APPLICANT

3.1. PM/PM10/PM2.5 EMISSIONS

An exit grain loading rate method is used for determining the controlled PM emissions for material handling

processes at the Jacksonville plant.  This method uses the exit grain loading rate for each control device that

releases to the atmosphere.  The PM emissions from material handling sources at the plant are controlled by

separate dust collectors.  This emissions calculation method uses the control device inlet air flow rate and exit

grain loading rate as opposed to control device efficiencies for each piece of control equipment.  Potential hourly

PM emissions for a source are calculated using the exit grain loading rate and the maximum inlet air flow rate of

the associated control device that releases to the atmosphere.  The potential annual PM emissions for emission

sources are calculated by multiplying the hourly emission rate by continuous operation (8,760 hour /year).  The exit grain loading rate for dust collectors is conservatively assumed to be 0.02 grains per dry standard cubic feet of air (gr/DSCF).  Owens Corning assumes PM emissions are equivalent to PM10 and PM2.5 emissions from controlled emission points.  Calculations for potential PM/PM10/PM2.5 emissions from the baghouse are shown below:

Hourly PM Emissions = Grain Loading Rate (gr DSCF) × Air Flow Rate (DSCF min) × (60 min hour) × (1 lb

7000 gr).

Hourly PM Emissions = 0.02 (gr DSCF) × 2,370 (DSCF min) × (60 min hour) × (1 lb 7000 gr) = 0.41 (lb hour).

Table 1. SUMMARY OF AFFECTED SOURCES POTENTIAL EMISSIONS

3.2.

3.3.

PROCESS DUST COLLECTOR

- Department review

Brief Discussion of Emissions

PM/PM10 emissions - The air exhaust flow rate is increasing.  Therefore, the maximum allowable emissions of PM will increase as a result of the proposed project.

4.1.  RULES AND REGULATIONS

State Requirements

EU002 is currently regulated under Reasonably Available Control Technology (RACT) requirements including Specific RACT Emission Limiting Standards for Stationary Emission Units Rules 62-296.700(3), (4), (5), (6)(a)2, (6)(b); 62-297.310(8)(b)1; 62-296.711(2)(a); and 62-296.711(3), F.A.C.

There are no new State Requirements as a result of the permitting action.

Federal NSPS Provisions

This EU is not subject to any NSPS Provisions.

Federal NESHAP Provisions

This EU is not subject to any NESHAPS Provisions.

Local Requirements

Rule 2.1101, JEPB; and Rule 2.1201, JEPB.

  - Other Draft Permit Requirements

The baghouse to be added to EU002 is exempt from CAM Plan requirements because its uncontrolled emissions are under the Title V major source threshold.  No additional production, operation limitations, monitoring requirements, etc. as a result of this permitting action.

- APPLICANT FORMAL ENFORCEMENT HISTORY

It is the interpretation of the Department that the request and review of an applicant’s enforcement history is an inherent part of the Reasonable Assurance Rule, 62-4.070(1-5), F.A.C.; Environmental Resource Permitting Rule, 62-330.301, F.A.C.; and the Coastal Construction and Excavation Rule, 62B-33.005(4), F.A.C. The enforcement history should be non-program specific (i.e. include enforcement for all programs regardless of the program the permit application is for), include enforcement of federal regulations, Department and delegated local program or agency rules, statues or orders, and include reportable spills and releases as well as formal enforcement. The period history shall be a period of previous 5 years.

5.1.  AirInfo: Past 5-year Air Compliance History- Incompliance.  No violations.

Enforcement and Compliance History Online (ECHO): Past 5-year Air/Water Compliance History- Incompliance.  No violations.

- Preliminary Determination

The Department makes a preliminary determination that the proposed project will comply with all applicable state and federal air pollution regulations as conditioned by the draft permit.  This determination is based on a technical review of the complete application, reasonable assurances provided by the applicant, and the conditions specified in the draft permit.  No air quality modeling analysis is required because the project does not result in a significant increase in emissions.  Leslie Maybin is the project engineer responsible for reviewing the application and drafting the permit.  Additional details of this analysis may be obtained by contacting the project engineer at the Department’s Northeast District Office, 8800 Baymeadows Way West, Suite 100, Jacksonville, Florida 32256, phone (904) 256-1567, or by email leslie.maybin@dep.state.fl.us.