TECHNICAL EVALUATION

&

PRELIMINARY DETERMINATION

APPLICANT

West Fraser, Inc.

401 Champion Drive

McDavid, Florida, 32568

McDavid Softwood Converting Facility

Facility ID No. 0330260

PROJECT

Project No. 0330260-014-AC

Application for Minor Source Air Construction Permit

Project Name

Installation of Canter and Merchandiser

COUNTY

Escambia County, Florida

PERMITTING AUTHORITY

Florida Department of Environmental Protection

Northwest District Permitting Program

160 W. Government Street, Suite 308

Pensacola, Florida 32502-5740

September 28, 2018

- General Project INFORMATION

Air Pollution Regulations

Projects at stationary sources with the potential to emit air pollution are subject to the applicable environmental laws specified in Section 403 of the Florida Statutes (F.S.).  The statutes authorize the Department of Environmental Protection (Department) to establish regulations regarding air quality as part of the Florida Administrative Code (F.A.C.), which includes the following applicable chapters:  62-4 (Permits); 62-204 (Air Pollution Control – General Provisions); 62-210 (Stationary Sources – General Requirements); 62-212 (Stationary Sources – Preconstruction Review); 62-213 (Operation Permits for Major Sources of Air Pollution); 62-296 (Stationary Sources - Emission Standards); and 62-297 (Stationary Sources – Emissions Monitoring).  Specifically, air construction permits are required pursuant to Chapters 62-4, 62-210 and 62-212, F.A.C.

In addition, the U. S. Environmental Protection Agency (EPA) establishes air quality regulations in Title 40 of the Code of Federal Regulations (CFR).  Part 60 specifies New Source Performance Standards (NSPS) for numerous industrial categories.  Part 61 specifies National Emission Standards for Hazardous Air Pollutants (NESHAP) based on specific pollutants.  Part 63 specifies NESHAP based on the Maximum Achievable Control Technology (MACT) for numerous industrial categories.  The Department adopts these federal regulations in Rule 62-204.800, F.A.C.

Glossary of Common Terms

Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of this permit.

Facility Description and Location

West Fraser, Inc. operates the existing McDavid Softwood Converting Facility which processes southern yellow pine logs and has the capacity to produce up to 250 million board feet of lumber per year.  The facility contains three lumber drying kilns and a planer mill.  The principal processes at the mill include:  Log storage and processing (debarking and sawing); Sawmill operations (chipping and sawing); Drying of green lumber using three indirect steam-heated kilns; Product lumber finishing, sorting, and shipping

The West Fraser, Inc., McDavid Softwood Converting Facility is an existing Sawmill and Planing Mill which is categorized under Standard Industrial Classification Code No. 2421.  The existing facility is located in Escambia County at 401 Champion Drive in McDavid, Florida.  The UTM coordinates of the existing facility are Zone 16, 469.03 km East, and 3406.43 km North.  This site is in an area that is in attainment (or designated as unclassifiable) for all air pollutants subject to Ambient Air Quality Standards (AAQS).

Facility Regulatory Categories

- The facility is a major source of hazardous air pollutants (HAP).

- The facility is a Title V major source of air pollution in accordance with Chapter 62-213, F.A.C.

- The facility is a major stationary source in accordance with Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

- The facility does operate units subject to the NSPS of 40 CFR 60.

- The facility does operate units subject to the NESHAP of 40 CFR 63.

- Project Description

McDavid Lumber Mill is proposing a project to modify the fugitive emissions source listed in unregulated emissions unit (EU) 005, that will include replacing and upgrading the existing canter system with a more advanced process. The advances in this technology maximizes recovery of lumber from logs and reduces “waste” through generation of byproducts.  The proposed changes include new loading and handling equipment, scanners, canter, merchandiser, profiler, and gang saw.  This project is expected to increase actual emissions at the mill.  The project will not increase the potential to emit (PTE) of any source because the facility PTE is limited by throughput which is unchanged. The project will not modify any other existing sources at the mill.  However, the project will increase actual production at the following unmodified but affected sources: Natural Gas Fired Boilers #1 and #2 (EU 001 and EU002), Lumber Drying Kilns (EU 003), and Planer Mill Operations (EU 004).

- The following existing emissions units (EU) will be affected by this project.

| EU No. | Emission Unit Description |
| --- | --- |
| 001 | Natural Gas Fired Boiler No. 1 |
| 002 | Natural Gas Fired Boiler No. 2 |
| 003 | Lumber Drying Kilns 1, 2 and 3 |
| 004 | Planer Mill Operations |
| Unregulated Emissions Units |  |
| 005 | Fugitive PM Emissions (only subject to facility-wide specific conditions) |

- Processing Schedule

May 24, 2018	Received the application for a minor source air pollution construction permit.

June 1, 2018	Requested additional information.

August 6, 2018	Received additional information.

- PSD Applicability

General PSD Applicability

For areas currently in attainment with the AAQS or areas otherwise designated as unclassifiable, the Department regulates major stationary sources of air pollution in accordance with Florida’s PSD preconstruction review program as defined in Rule 62-212.400, F.A.C.  Under preconstruction review, the Department first must determine if a project is subject to the PSD requirements (“PSD applicability review”) and, if so, must conduct a PSD preconstruction review.  A PSD applicability review is required for projects at new and existing major stationary sources.  In addition, proposed projects at existing minor sources are subject to a PSD applicability review to determine whether potential emissions from the proposed project itself will exceed the PSD major stationary source thresholds.  A facility is considered a major stationary source with respect to PSD if it emits or has the potential to emit:

- 250 tons per year or more of any regulated air pollutant; or

- 100 tons per year or more of any regulated air pollutant and the facility belongs to one of the following 28 PSD-major facility categories:  fossil fuel-fired steam electric plants of more than 250 million British thermal units per hour heat input, coal cleaning plants (with thermal dryers), Kraft pulp mills, Portland cement plants, primary zinc smelters, iron and steel mill plants, primary aluminum ore reduction plants, primary copper smelters, municipal incinerators capable of charging more than 250 tons of refuse per day, hydrofluoric, sulfuric, and nitric acid plants, petroleum refineries, lime plants, phosphate rock processing plants, coke oven batteries, sulfur recovery plants, carbon black plants (furnace process), primary lead smelters, fuel conversion plants, sintering plants, secondary metal production plants, chemical process plants, fossil fuel boilers (or combinations thereof) totaling more than 250 million British thermal units per hour heat input, petroleum storage and transfer units with a total storage capacity exceeding 300,000 barrels, taconite ore processing plants, glass fiber processing plants and charcoal production plants.

Once it is determined that a project is subject to PSD preconstruction review, the project emissions are compared to the “significant emission rates” defined in Rule 62-210.200, F.A.C. for the following pollutants:  carbon monoxide (CO); nitrogen oxides (NOX); sulfur dioxide (SO2); particulate matter (PM); particulate matter with a mean particle diameter of 10 microns or less (PM10); particulate matter with a mean particle diameter of 2.5 microns or less (PM2.5); volatile organic compounds (VOC); lead (Pb); fluorides (F); sulfuric acid mist (SAM); hydrogen sulfide (H2S); total reduced sulfur (TRS), including H2S; reduced sulfur compounds, including H2S; municipal waste combustor organics measured as total tetra- through octa-chlorinated dibenzo-p-dioxins and dibenzofurans; municipal waste combustor metals measured as particulate matter; municipal waste combustor acid gases measured as SO2 and hydrogen chloride (HCl); municipal solid waste landfills emissions measured as non-methane organic compounds (NMOC); and mercury (Hg).  In addition, significant emissions rate also means any emissions rate or any net emissions increase associated with a major stationary source or major modification which would construct within 10 kilometers of a Class I area and have an impact on such area equal to or greater than 1 μg/m3, 24-hour average.

If the potential emission equals or exceeds the defined significant emissions rate of a PSD pollutant, the project is considered “significant” for the pollutant and the applicant must employ the Best Available Control Technology (BACT) to minimize the emissions and evaluate the air quality impacts.  Although a facility or project may be major with respect to PSD for only one regulated pollutant, it may be required to install BACT controls for several “significant” regulated pollutants.

PSD Applicability for Project

As provided in the application, the following table summarizes projected actual emissions and PSD applicability for the project.

Table 1.  Summary of the Applicant’s PSD Applicability Analysis

| Pollutant | Annual Emissions, Tons/Year |  |  |  | Subject to PSD? |
| --- | --- | --- | --- | --- | --- |
|  | Baseline Actual | Adjusted Projected Actual | Increase | Significant Emissions Rate |  |
| CO | 1.26 | 70.17 | 68.91 | 100 | No |
| NOX | 14.82 | 38.98 | 24.16 | 40 | No |
| PM | 30.59 | 32.95 | 2.36 | 25 | No |
| PM10 | 15.99 | 18.36 | 2.37 | 15 | No |
| PM2.5 | 14.19 | 16.56 | 2.37 | 10 | No |
| SO2 | 0.15 | 0.23 | 0.08 | 40 | No |
| VOC | 166.49 | 168.71 | 2.22 | 40 | No |
| Pb | 0.001 | 0.0002 | 0.00 |  |  |
| CO2e | 29,489 | 45,759 | 16,270 |  |  |

This facility is a major stationary source with respect to PSD because potential emissions of VOC are greater than 250 TPY as shown in the emissions discussion section below.  As shown in the table above, this project is not a major modification under PSD regulations, since the differences between the baseline actual emissions and projected actual emissions do not reach or exceed the significant emissions rates.  Based on these facts it is determined that the project is not subject to PSD preconstruction review.

- Department review

Brief Discussion of Emissions

Actual emissions will increase at the following unmodified but affected sources: Natural Gas Fired Boilers #1 and #2, Lumber Drying Kilns, and planer Mill Operations.  However potential emissions for these sources and the facility remain unchanged because the facility permitted throughput remains unchanged.

Facility-Wide Potential to Emit (Tons/Year)

| EU ID | Description | NOx | VOC | CO | SO2 | PM | PM10 | PM2.5 | HAP | CO2e |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 001 | Boiler 1 | 19.49 | 3.12 | 35.08 | 0.12 | 0.68 | 0.68 | .068 | 0.37 | 22,806 |
| 002 | Boiler 2 | 19.49 | 3.12 | 35.08 | 0.12 | 0.68 | 0.68 | 0.68 | 0.37 | 22,806 |
| 003 | Kilns 1, 2, 3 |  | 355 |  |  | 4.63 | 4.63 | 4.63 | 33.3 |  |
| 004 | Planer Mill |  |  |  |  | 9.01 | 9.01 | 9.01 |  |  |
| 005 | Process Fugitive Emissions |  |  |  |  | 42.7 | 23.1 | 21.7 |  |  |
|  | By Product Handling Fugitive Emissions |  |  |  |  | 1.06E-01 | 4.99E-02 | 7.56E-03 |  |  |
|  | Haul Road Fugitive Emissions |  |  |  |  | 15.5 | 3.1 | 0.8 |  |  |
|  | Total Fugitive Emissions |  |  |  |  | 58.4 | 26.3 | 22.4 |  |  |
| 006 | Emergency Fire Pump | 0.98 | 0.03 | 0.04 | 0.10 | 0.01 | 0.01 | 0.01 | 2.99E-03 | 77.17 |
| 007 | Air Curtain Incinerator * | 0.38 |  | 19.5 | 0.04 | 8.25 | 8.25 | 8.25 |  |  |
| Total Emissions: |  | 40.34 | 361.27 | 89.71 | 0.37 | 81.62 | 49.55 | 45.69 | 33.99 | 45,690 |

* ACI Project 012-AC

State Requirements

Rule 62-296.320, F.A.C. – General Pollutant Emission Limiting Standards

Rule 62-210.200(PTE), F.A.C.

Rule 62-210.300(1), F.A.C.

Rule 62-212, F.A.C.

Rule 62-213, F.A.C.

Federal NSPS Provisions

None applicable to this project.

Federal NESHAP Provisions

None applicable to this project.

Other Draft Permit Requirements

None.  The proposed project includes changes within the log preparation and sawmill operations of the mill. Emissions from this area are quantified as Fugitive PM Emissions as listed in the Unregulated EU 005.  The fugitive emissions included in EU 005 result from log debarking and sawing, wood by-product screening, handling and storage, sawmill operations, and truck traffic. Potential emissions of fugitive PM/PM10/PM2.5 emissions in this area are low due to the high moisture content of the wood materials being processed.

- Preliminary Determination

The Department makes a preliminary determination that the proposed project will comply with all applicable state and federal air pollution regulations as conditioned by the draft permit.  This determination is based on a technical review of the complete application, reasonable assurances provided by the applicant, and the conditions specified in the draft permit.  No air quality modeling analysis is required because the project does not result in a significant increase in emissions.  Angelia Butler is the project engineer responsible for reviewing the application and drafting the permit.  Additional details of this analysis may be obtained by contacting the project engineer at the Department’s Northwest District Office, 160 W. Government Street, Suite 308, Pensacola, Florida 32502-5740, phone (850) 595-0598, or by email angelia.butler@floridadep.gov.