Permittee

| CEMEX Construction Materials Florida, LLC 10311 Cement Plant Road Brooksville, FL 34601 Authorized Representative: Alberto Calleros, Plant Manager | Air Permit No. 0530010-048-AC/PSD-FL-233A Permit Expires: 12/31/2019 Minor Air Construction Permit Brooksville North Cement Plant Removal of SO2 CEMS and Kiln Feed Limits |
| --- | --- |

Project

This is the final air construction permit, which authorizes the removal of the requirement to continuously monitor sulfur dioxide (SO2) emissions through continuous emissions monitoring system (CEMS) on In-Line Kiln Nos 1; remove thallium monitoring from In-Line Kiln No. 1; replace kiln production from a kiln feed basis to clinker produced basis; and convert kiln emissions limits from pounds per ton of dry kiln feed to pounds per ton of clinker produced.  The proposed work will be conducted at the existing Brooksville North Cement Plant, which is a Portland cement plant categorized under Standard Industrial Classification No. 3241.  The existing facility is located in Hernando County at 16301 Ponce De Leon Boulevard in Brooksville, Florida.  The UTM coordinates are Zone 17, 356.1 kilometers (km) East and 3169.3 km North.

This final permit is organized into the following sections:  Section 1 (General Information); Section 2 (Administrative Requirements); Section 3 (Emissions Unit Specific Conditions); and Section 4 (Appendices).  Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of Section 4 of this permit.

Statement of Basis

This air pollution construction permit is issued under the provisions of:  Chapter 403 of the Florida Statutes (F.S.) and Chapters 62-4, 62-204, 62-210, 62-212, 62-296 and 62-297 of the Florida Administrative Code (F.A.C.).  The permittee is authorized to conduct the proposed work in accordance with the conditions of this permit.  This project is subject to the general preconstruction review requirements in Rule 62-212.300, F.A.C. and is not subject to the preconstruction review requirements for major stationary sources in Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

Upon issuance of this final permit, any party to this order has the right to seek judicial review of it under Section 120.68 of the Florida Statutes by filing a notice of appeal under Rule 9.110 of the Florida Rules of Appellate Procedure with the clerk of the Department of Environmental Protection in the Office of General Counsel (Mail Station #35, 3900 Commonwealth Boulevard, Tallahassee, Florida, 32399-3000) and by filing a copy of the notice of appeal accompanied by the applicable filing fees with the appropriate District Court of Appeal.  The notice must be filed within 30 days after this order is filed with the clerk of the Department.

Executed in Tallahassee, Florida

(DRAFT)

For:

Syed Arif, P.E., Program Administrator

Office of Permitting and Compliance

Division of Air Resource Management

CERTIFICATE OF SERVICE

The undersigned duly designated deputy agency clerk hereby certifies that this Final Air Construction Permit package was sent by electronic mail, or a link to these documents made available electronically on a publicly accessible server, with received receipt requested before the close of business on the date indicated below to the following persons.

# Mr. Alberto Calleros, CEMEX:  alberto.calleros@cemex.com

# Mr. Brent Steele, CEMEX:  brentc.steele@cemex.com

# Ms. Lillian DePrimo, CEMEX:  lillianf.deprimo@cemex.com

# Dr. Max Lee, P.E., Koogler & Associates, Inc.:  mlee@kooglerassociates.com

# Ms. Upasna Rai, Koogler & Associates, Inc.:  urai@kooglerassociates.com

# Southwest District Air Compliance:  SWD_Air@dep.state.fl.us

# Southwest District Air Permitting:  SWD_Air_Permitting@dep.state.fl.us

# US EPA Region 4:  R4TitleVFL@epa.gov

# Ms. Lynn Scearce, DEP OPC:  lynn.scearce@dep.state.fl.us

# Clerk Stamp

# FILING AND ACKNOWLEDGMENT FILED, on this date, pursuant to Section 120.52(7), Florida Statutes, with the designated agency clerk, receipt of which is hereby acknowledged.

(DRAFT)

Facility Description

This facility produces portland cement and masonry cement as a final product and consists of two Polysius GEPOL preheater kilns (Kilns 1 and 2), two clinker coolers, raw mills, finish mills, clinker handling and storage, cement handling and storage, cement rail and truck loadout equipment, cement bagging equipment, coal bins and coal handling equipment, two coal grinding mills and associated air pollution control devices and support equipment. The permitted capacity of each kiln is 780,000 tons per year of clinker.

Raw materials used to manufacture cement clinker include but are not limited to limestone, sand, fly ash and mill scale; sources of calcium, silica, aluminum and iron.  These materials are metered into two pre-mix bins prior to introduction to the raw mills (one for each kiln).  The raw mills dry and grind the raw materials to a fineness that is suitable for the preheater kiln systems.  The raw meal product is transported to the two raw meal blending silos where it is further mixed to assure physical and chemical consistency in the kiln feed (raw mill).  The raw meal is then metered to each of the preheaters. In the rotary kilns, the raw meal is heated to temperatures exceeding 2600 0 F, which results in the production of cement clinker.  The clinker is then cooled in clinker coolers (one for each kiln) and transported to the finish mills (3), where it is mixed with gypsum (and other additives) and ground to produce cement of various grades.  The finished product is stored in cement silos prior to being shipped off-site in bulk by truck or rail cars, or sent to cement bagging lines where it is bagged prior to shipment.

The two kilns are permitted to use fuel oil, natural gas, coal and tire-derived fuel (Kiln 1 only) as fuels.  Kiln exhaust gases are used to pre-heat the incoming raw feed in the preheaters and to dry raw material in the raw mill grinding systems before being vented to the kiln baghouses of each kiln.  Air pollution emission controls include Selective Non-Catalytic Reduction (SNCR) systems on both kilns for control of NOx emissions, and baghouse dust collectors on most of the emission units for control of particulate matter (PM/PM10).

Emissions from each kiln system (kiln, preheater and raw mill) are vented through separate kiln baghouses and exhaust stacks.  Emissions from each of the clinker coolers are also vented to separate clinker cooler baghouses and exhaust stacks.  The remainders of the facility emissions are vented from individual baghouse exhaust stacks/vents or are non-stack/vent fugitive emissions.

Each of the two kiln stacks and the two clinker cooler stacks are equipped with continuous opacity monitoring systems (COMS).  The two kiln stacks are also equipped with nitrogen oxides (NOx), carbon monoxide (CO), sulfur dioxide (SO2 as a surrogate for hydrogen chloride, (HCl)), total hydrocarbons (THC), mercury (Hg), carbon dioxide (CO2), oxygen (O2) and stack gas flow continuous monitoring systems (CMS).

The existing facility consists of the following emissions units (EU).

| EU No. | ## Brief Description |  |
| --- | --- | --- |
| Brooksville North Cement Line 1 and Line 2 |  |  |
| Raw Meal Handling and Storage System |  |  |
| 031 |  | Raw Material (Limestone) Quarrying, Crushing and Storage |
| 011 |  | Raw Material Storage Silos & Feed System (Baghouses C-11 & C-11A) |
| 024 |  | Raw Material Pre-Mix Bins (Baghouse M-2280) |
| Brooksville North Cement Line 1 |  |  |
| Pyroprocessing System |  |  |
| 008 |  | Kiln No. 1 Blending Silo Nos. 1 and 2 (Baghouses E-36 & F-17) |
| 002 |  | Kiln No. 1 Feed System (Baghouse D-31) |
| 003 |  | In-Line Kiln/Raw Mill No. 1 (Baghouse E-55) |
| 004 |  | Clinker Cooler No. 1 (Baghouse F-18) |
| Clinker Handling and Storage System |  |  |
| 006 |  | Clinker Storage Silo Nos. 1 & 2 (Baghouse F-31) |
| Finish Mill System |  |  |
| 025 |  | Additive Material Storage Bin and Clinker Hoppers (Baghouse M-1171) |
| 005 |  | Finish Mill Nos. 1 & 2 (Baghouses G-23-1 & G-23-2) |
| Brooksville North Cement Line 2 |  |  |
| Pyroprocessing System |  |  |
| 012 |  | Kiln No. 2 Blending Silo (Baghouse G-11) |
| 013 |  | Kiln No. 2 Feed System (Baghouse H-13) |
| 014 |  | In-Line Kiln/Raw Mill No. 2 (Baghouse E-19) |
| 015 |  | Clinker Cooler No. 2 (Baghouse K-09) |
| Clinker Handling and Storage System |  |  |
| 016 |  | Clinker Storage Silo No. 3 (Baghouse L-07) |
| 017 |  | Clinker/Gypsum Transfer Belt (Baghouse M-09) |
| Finish Mill System |  |  |
| 018 |  | Finish Mill No. 3 Clinker Day Tank & Gypsum Day Silo (Baghouse M-10) |
| 019 |  | Finish Mill No. 3 (Baghouse N-23) |
| Brooksville North Cement Line 1 and Line 2 |  |  |
| Cement Silos and Loadout |  |  |
| 009 |  | Cement Storage Silo Nos. 1-5, Side No. 1 Railcar/Truck Loadout Systems (Baghouse H-03) |
| 021 |  | Cement Silo Nos. 7 & 8 (Baghouse P-05) |
| 022 |  | Masonry Cement Storage Silo (Baghouse P-07) |
| 023 |  | Side No. 2 Truck Loadout System (Baghouse Q-17) |
| 026 |  | Cement Bagging Line No. 1 (Baghouse M-3514) |
| 027 |  | Cement Bagging Line No. 2 (Baghouse M-3515) |
| Other |  |  |
| 028 |  | Facility Wide Fugitive Particulate Matter Emissions |
| 032 |  | Coal Handling and Coal Mills |
| 034 |  | Two Emergency Spark Ignition Engines (4SRB existing stationary RICE) |

PROPOSED PROJECT

This project authorizes the removal of the requirement to continuously monitor sulfur dioxide (SO2) emissions through a continuous emissions monitoring system (CEMS) on In-Line Kiln No 1; remove thallium monitoring from In-Line Kiln No. 1; replace kiln production from a kiln feed basis to a clinker produced basis; and convert kiln emissions limits from pounds per ton of dry kiln feed to pounds per ton of clinker produced.

The following emissions units will be affected by this project.

| EU No. | Emission Unit Description |
| --- | --- |
| 003 | In-Line Kiln/Raw Mill No. 1 (Baghouse E-55) |
| 004 | Clinker Cooler No. 1 (Baghouse F-18) |
| 014 | In-Line Kiln/Raw Mill No. 2 (Baghouse E-19) |
| 015 | Clinker Cooler No. 2 (Baghouse K-09) |

Facility Regulatory Classification

- The facility is a major source of hazardous air pollutants (HAP).

- The facility does not operate units subject to the acid rain provisions of the Clean Air Act (CAA).

- The facility is a Title V major source of air pollution in accordance with Chapter 62-213, F.A.C.

- The facility is a major stationary source in accordance with Rule 62-212.400(PSD), F.A.C.

- The facility operates units subject to the New Source Performance Standards (NSPS) of Title 40 Part 60 of the Code of Federal Regulations (40 CFR 60).

- The facility operates units subject to the National Emissions Standards of Hazardous Air Pollutants (NESHAP) of 40 CFR 63.

- Permitting Authority:  The permitting authority for this project is the Office of Permitting and Compliance in the Division of Air Resource Management of the Department of Environmental Protection (Department).  The Office of Permitting and Compliance mailing address is 2600 Blair Stone Road (MS #5505), Tallahassee, Florida  32399-2400.

- Compliance Authority:  All documents related to compliance activities such as reports, tests, and notifications shall be submitted to the Southwest District at:  13051 North Telecom Parkway, Temple Terrace, Florida  33637-0926.

- Appendices:  The following Appendices are attached as a part of this permit:  Appendix A (Citation Formats and Glossary of Common Terms); Appendix B (General Conditions); Appendix C (Common Conditions); and Appendix D (Common Testing Requirements).

- Applicable Regulations, Forms and Application Procedures:  Unless otherwise specified in this permit, the construction and operation of the subject emissions units shall be in accordance with the capacities and specifications stated in the application.  The facility is subject to all applicable provisions of: Chapter 403, F.S.; and Chapters 62-4, 62-204, 62-210, 62-212, 62-213, 62-296 and 62-297, F.A.C.  Issuance of this permit does not relieve the permittee from compliance with any applicable federal, state, or local permitting or regulations.

- New or Additional Conditions:  For good cause shown and after notice and an administrative hearing, if requested, the Department may require the permittee to conform to new or additional conditions.  The Department shall allow the permittee a reasonable time to conform to the new or additional conditions, and on application of the permittee, the Department may grant additional time.  [Rule 62-4.080, F.A.C.]

- Modifications:  The permittee shall notify the Compliance Authority upon commencement of construction.  No new emissions unit shall be constructed and no existing emissions unit shall be modified without obtaining an air construction permit from the Department.  Such permit shall be obtained prior to beginning construction or modification.  [Rules 62-210.300(1) and 62-212.300(1)(a), F.A.C.]

- Construction and Expiration:  The expiration date shown on the first page of this permit provides time to complete the physical construction activities authorized by this permit, complete any necessary compliance testing, and obtain an operation permit.  Notwithstanding this expiration date, all specific emissions limitations and operating requirements established by this permit shall remain in effect until the facility or emissions unit is permanently shut down.  For good cause, the permittee may request that a permit be extended.  Pursuant to Rule 62-4.080(3), F.A.C., such a request shall be submitted to the Permitting Authority in writing before the permit expires.  [Rules 62-4.070(3) & (4), 62-4.080 & 62-210.300(1), F.A.C.]

- Source Obligation:

- At such time that a particular source or modification becomes a major stationary source or major modification (as these terms were defined at the time the source obtained the enforceable limitation) solely by virtue of a relaxation in any enforceable limitation which was established after August 7, 1980, on the capacity of the source or modification otherwise to emit a pollutant, such as a restriction on hours of operation, then the requirements of subsections 62-212.400(4) through (12), F.A.C., shall apply to the source or modification as though construction had not yet commenced on the source or modification.

- At such time that a particular source or modification becomes a major stationary source or major modification (as these terms were defined at the time the source obtained the enforceable limitation) solely by exceeding its projected actual emissions, then the requirements of subsections 62-212.400(4) through (12), F.A.C., shall apply to the source or modification as though construction had not yet commenced on the source or modification.

[Rule 62-212.400(12), F.A.C.]

This section of the permit addresses the following emissions units.

| EU No. | Emission Unit Description |
| --- | --- |
| 003 | In-Line Kiln/Raw Mill No. 1 (Baghouse E-55) |
| 004 | Clinker Cooler No. 1 (Baghouse F-18) |
| 014 | In-Line Kiln/Raw Mill No. 2 (Baghouse E-19) |
| 015 | Clinker Cooler No. 2 (Baghouse K-09) |

{Permitting Note:  These emissions units are regulated under 40 CFR 63, Subpart A (General Provisions) and Subpart LLL (National Emission Standards for Hazardous Air Pollutants from the Portland Cement Manufacturing Industry) adopted by reference in Rule 62-204.800(8)(b), F.A.C.}

Previous Applicable Requirements

- Other Permits:  The conditions of this permit supplement all previously issued air construction and operation permits for the In-Line Kiln/Raw Mill Nos 1 and 2.  Unless otherwise specified, these conditions are in addition to all other applicable permit conditions and regulations.  [Rule 62-4.070, F.A.C.]

##### Testing Requirements

- Annual Compliance Tests:  During each calendar year (January 1st to December 31st), Kiln Nos. 1 and 2 (EUs 003 and 014) shall be tested to demonstrate compliance with the emissions standards for SO2.  [Rule 62-297.310(8)(a)1, F.A.C.]

- Test Requirements:  The permittee shall notify the Compliance Authority in writing at least 15 days prior to any required tests.  Tests shall be conducted in accordance with the applicable requirements specified in Appendix D (Common Testing Requirements) of this permit.  [Rule 62-297.310(9), F.A.C.]

- Test Methods:  Required tests shall be performed in accordance with the following reference methods.

| Method | Description of Method and Comments |
| --- | --- |
| 1-4 | Traverse Points, Velocity and Flow Rate, Gas Analysis, and Moisture Content |
| 6 or 6C | Method for Determining Sulfur Dioxide Emissions |

The above methods are described in Appendix A of 40 CFR 60 and are adopted by reference in Rule 62-204.800, F.A.C.  No other methods may be used unless prior written approval is received from the Department.  [Rules 62-204.800, F.A.C.; and Appendix A of 40 CFR 60]

permits modified

Permit No. AC27-240349

- Specific Condition 17.

- Daily sampling of the baghouse dust for the No. 1 kiln is required.  The concentration of thallium in the baghouse dust shall not exceed 1.5%.  Compliance shall be demonstrated using the “Thallium Concentration Monitoring and Analysis Procedure” as described in Mr. Bob Roger’s letter to Dr. John Koogler, dated January 12, 1994 (Attachment #9).

[Application No. 0530010-048-AC/PSD-FL-233A]

Permit No. 0530010-003-AC/PSD-FL-233

- Table 1-1.  Air Pollutant Standards and Terms.

| E.U. ID# | Description | Pollutant ID | Fuel(s) | Allowable Emissions(2) |  |  | Equivalent Emissions(3) | Regulation(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | lb/ton dry kilnph feed clinker | lb/hr @ 150 90 TPH | lb/hr @ 165 100 TPH | TPY |  |
| ARMS # 003 | Kiln No. 1 | PM/PM10 | Coal/ gas/ oil/ WTDF | 0.18 0.31 | 27.0 | 29.7 | 118 | Rule 62-212.400(6), F.A.C. |
| ARMS # 003 | Kiln No. 1 | SO2(1) | Coal/ gas/ oil/ WTDF | 0.10 0.17 | 15.0 | 16.5 | 66 | Rule 62-4.070(3), F.A.C. |
| ARMS # 003 | Kiln No. 1 | NOx | Coal/ gas/ oil/ WTDF | 1.83 2.0 | 275 | 301 | 1318 | Rule 62-4.070(3), F.A.C. |
| ARMS # 003 | Kiln No. 1 | CO | Coal/ gas/ oil/ WTDF | 1.20 2.0 | 180.0 | 198.0 | 788 | Rule 62-212.400(6), F.A.C. |
| ARMS # 003 | Kiln No. 1 | VOC | Coal/ gas/ oil/ WTDF | 0.09 0.15 | 13.6 | 14.9 | 60 | Rule 62-4.070(3), F.A.C. |
| ARMS # 003 | Kiln No. 1 | Be, Pb, Hg | Coal/ gas/ oil/ WTDF | To Be Determined (4) | (4) | (4) | (4) |  |
| ARMS # 003 | Kiln No. 1 | 20% VE | Coal/ gas/ oil/ WTDF |  |  |  |  | Rule 62-204.800, F.A.C. |
| ARMS # 004 | Cooler No. 1 | 10% VE |  |  |  |  |  | Rule 62-204.800, F.A.C. |
| ARMS # 004 | Cooler No. 1 | PM/PM10 |  | 0.09 0.15 | 13.6 | 14.9 | 60 | Rule 62-204.800, F.A.C. |
| ALLOWABLE OPERATING RATES Hours of Operation 8760 (Kiln 1); 8760(Cooler 1) Kiln preheater feed rate (kiln ph) Clinker produced 100 165 TPH (One-hour maximum); 90 150 TPH (30-day average) Kiln Heat Input 300 MMBtu/hr |  |  |  |  |  |  |  |  |
| NOTES - Emissions of SO2 will not exceed 15 lbs/hr (150 90 TPH) and 16.5 lbs/hr (165 100 TPH). Annual testing is required in lieu of sulfur restrictions. [AC27-258571] - Compliance Units. This facility shall demonstrate compliance based on these standards. - “Equivalent Emissions” are based on annual average emissions at the 30-day feed clinker production rate of 90 150 TPH. The “Equivalent Emissions” are also listed to assess applicable Title V fees and for PSD recordkeeping tracking purposes. - To confirm emissions of these pollutants are under the PSD threshold levels. |  |  |  |  |  |  |  |  |

[Application No. 0530010-048-AC/PSD-FL-233A]

- Table 1-2.  Air Pollutant Standards and Terms.

| E.U. ID# | Description | Pollutant ID | Fuel(s) | Allowable Emissions(2) |  |  | Equivalent Emissions(3) | Regulation(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | lb/ton dry kilnph feed clinker | lb/hr @ 150 90 TPH | lb/hr @ 165 100 TPH | TPY |  |
| ARMS # 014 | Kiln No. 2 | PM/PM10 | Coal/ gas/ oil | 0.18 0.31 | 27.0 | 29.7 | 118 | Rule 62-212.400(6), F.A.C. |
| ARMS # 014 | Kiln No. 2 | SO2(1) | Coal/ gas/ oil | 0.10 0.17 | 15.0 | 16.5 | 66 | Rule 62-212.400(6), F.A.C. |
| ARMS # 014 | Kiln No. 2 | NOx | Coal/ gas/ oil | 1.72 2.0 | 258.0 | 283.8 | 1130 | Rule 62-212.400(6), F.A.C. |
| ARMS # 014 | Kiln No. 2 | CO | Coal/ gas/ oil | 1.20 2.0 | 180.0 | 198.0 | 788 | Rule 62-212.400(6), F.A.C. |
| ARMS # 014 | Kiln No. 2 | VOC | Coal/ gas/ oil | 0.09 0.15 | 13.6 | 14.9 | 60 | Rule 62-212.400(6), F.A.C. |
| ARMS # 014 | Kiln No. 2 | Be, Pb, Hg | Coal/ gas/ oil | (4) | (4) | (4) | (4) | Rule 62-4.070(3), F.A.C. |
| ARMS # 014 | Kiln No. 2 | 10% VE | Coal/ gas/ oil |  |  |  |  | Rule 62-212.400(6), F.A.C.. |
| ARMS # 015 | Cooler No. 2 | 10% VE |  |  |  |  |  | Rule 62-212.400(6), F.A.C. |
| ARMS # 015 | Cooler No. 2 | PM/PM10 |  | 0.09 0.15 | 13.6 | 14.9 | 60 | Rule 62-212.400(6), F.A.C.. |
| ALLOWABLE OPERATING RATES Hours of Operation 8760 (Kiln 2); 8760(Cooler 2) Kiln preheater feed rate (kiln ph) Clinker produced 100 165 TPH (One-hour maximum); 90 150 TPH (30-day average) Kiln Heat Input 300 MMBtu/hr |  |  |  |  |  |  |  |  |
| NOTES - Emissions of SO2 will not exceed 15 lbs/hr (150 90 TPH) and 16.5 lbs/hr (165 100 TPH). Annual testing is required in lieu of sulfur restrictions. [AC27-258572] - Compliance Units. This facility shall demonstrate compliance based on these standards. - “Equivalent Emissions” are based on annual average emissions at the 30-day feed clinker production rate of 90 150 TPH. The “Equivalent Emissions” are also listed to assess applicable Title V fees and for PSD recordkeeping tracking purposes. - To confirm emissions of these pollutants are under the PSD threshold levels. |  |  |  |  |  |  |  |  |

[Application No. 0530010-048-AC/PSD-FL-233A]

Permit No. 0530010-026-AC

- Section III, Subsection A, Specific Condition 3

- Process Rate Limitations:  The maximum clinker produced by process dry preheater feed rate for each kiln shall not exceed 100 165 tons per hour (one-hour maximum) and 90 150 tons per hour (rolling 30-operating day average).  In addition to the short-term clinker production preheater feed rate limits, the clinker produced by dry preheater feed rate for each kiln shall not exceed 780,000 1,300,000 tons during any consecutive 12-month period. [Application No. 0530010-048-AC; and Rule 62-4.070(3), F.A.C.; construction permits AC27-186923, AC27-240349, AC-258571, and 0530010-003-AC]

[Rule 62-297.310(10), F.A.C. and Application No. 0530010-048-AC/PSD-FL-233A]

Permit No. 0530010-030-AC (BART Project)

- Section 3, Subsection A, Specific Condition 8.

- SO2 CEMS Installation: In accordance with the requirements in Appendix D (Standard CEMS Requirements for SO2 Monitoring) of this permit, the permittee shall properly install, calibrate, maintain, and operate a CEMS to measure and record emissions of SO2 from Kiln No. 1.  The SO2 CEMS shall satisfy all requirements of Appendix D no later than January 1, 2014.  Effective January 1, 2014 the emissions data collected with the CEMS shall be used to demonstrate compliance with the corresponding emissions standards. [Rules 62-296.340 (BART) and 62-4.070(3), F.A.C. Application No. 0530010-048-AC/PSD-FL-233A]

- Section 3, Subsection A, Specific Condition 10.

- SO2 Emissions Standard:  For purposes of BART, beginning January 1, 2014, SO2 emissions from Kiln No. 1 shall not exceed the following emissions standards.

| Pollutant | Emission Limit | Averaging Time | Compliance Method | Basis |
| --- | --- | --- | --- | --- |
| SO2 | 0.17 lb/ton Clinker | 24-operating 3 hour rolling | CEMS Annual Stack Test | BART Application No. 053010-048-AC |
|  | 15 lb/hr @150 TPH of feed 16.5 lb/hr @ 165 TPH of feed 15 lb/hr @ 90 TPH of clinker 16.5 lb/hr @ 100 TPH of clinker |  |  |  |

[Application No. 0530010-048-AC/PSD-FL-233A]