### TECHNICAL EVALUATION

&

PRELIMINARY DETERMINATION

#### APPLICANT

Genpak, LLC

26 Republic Plaza

Middletown, New York 10940-6543

Genpak Sebring Facility

Facility ID No. 0550006

#### PROJECT

# Project No. 0550006-008-AC

Application for Major Source Air Construction Permit

Modification

#### COUNTY

Highlands, Florida

PERMITTING AUTHORITY

Florida Department of Environmental Protection

Air Resource Section

South District Office

2295 Victoria Avenue, Suite 364

P.O. Box 2549

Fort Myers, Florida  33902

June 30, 2010

1.  General Project INFORMATION

Air Pollution Regulations

Projects at stationary sources with the potential to emit air pollution are subject to the applicable environmental laws specified in Section 403 of the Florida Statutes (F.S.).  The statutes authorize the Department of Environmental Protection (Department) to establish regulations regarding air quality as part of the Florida Administrative Code (F.A.C.), which includes the following applicable chapters:  62-4 (Permits); 62-204 (Air Pollution Control – General Provisions); 62-210 (Stationary Sources – General Requirements); 62-212 (Stationary Sources – Preconstruction Review); 62-213 (Operation Permits for Major Sources of Air Pollution); 62-296 (Stationary Sources - Emission Standards); and 62-297 (Stationary Sources – Emissions Monitoring).  Specifically, air construction permits are required pursuant to Rules 62-4, 62-210 and 62-212, F.A.C.

In addition, the U. S. Environmental Protection Agency (EPA) establishes air quality regulations in Title 40 of the Code of Federal Regulations (CFR).  Part 60 specifies New Source Performance Standards (NSPS) for numerous industrial categories.  Part 61 specifies National Emission Standards for Hazardous Air Pollutants (NESHAP) based on specific pollutants.  Part 63 specifies NESHAP based on the Maximum Achievable Control Technology (MACT) for numerous industrial categories.  The Department adopts these federal regulations on a quarterly basis in Rule 62-204.800, F.A.C.

Glossary of Common Terms

Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of this permit.

Facility Description and Location

Genpak LLC is an existing polystyrene foam product manufacturing plant, which is categorized under Standard Industrial Classification Code No. 3087.  The facility is located in Highlands County at 116 Shicane Drive, Sebring, Florida.  The UTM coordinates of the existing facility are Zone 17, 464.79 East and 3036.83 North.  This site is in an area that is in attainment (or designated as unclassifiable) for all air pollutants subject to state and federal Ambient Air Quality Standards (AAQS).

Facility Regulatory Categories

- The facility is not a major source of hazardous air pollutants (HAP).

- The facility has no units subject to the acid rain provisions of the Clean Air Act.

- The facility is a Title V major source of air pollution in accordance with Chapter 213, F.A.C.

- The facility is a major stationary source in accordance with Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

- Project Description

- The proposed project will modify air construction permits nos. 0550006-003-AC and 0550006-006-AC. The modification will clearly define all emissions units at the facility, remove emissions units no longer at the facility, designate existing emission units as insignificant, clearly state emission factors, and provide for a defined method for determining compliance.  The proposed project revises emission factors previously defined to be used to calculate emissions.

Processing Schedule

January 8, 2010 	Received the application for a major source air pollution construction permit.

March 9, 2010		Requested additional information.

April 22, 2010		Received additional information; application complete.

2.  PSD Applicability

General PSD Applicability

For areas currently in attainment with the state and federal AAQS or areas otherwise designated as unclassifiable, the Department regulates major stationary sources of air pollution in accordance with Florida’s PSD preconstruction review program as defined in Rule 62-212.400, F.A.C.  Under preconstruction review, the Department first must determine if a project is subject to the PSD requirements (“PSD applicability review”) and, if so, must conduct a PSD preconstruction review.  A PSD applicability review is required for projects at new and existing major stationary sources.  In addition, proposed projects at existing minor sources are subject to a PSD applicability review to determine whether potential emissions from the proposed project itself will exceed the PSD major stationary source thresholds.  A facility is considered a major stationary source with respect to PSD if it emits or has the potential to emit:

- 250 tons per year or more of any regulated air pollutant; or

- 100 tons per year or more of any regulated air pollutant and the facility belongs to one of the following 28 PSD-major facility categories:  fossil fuel-fired steam electric plants of more than 250 million British thermal units per hour heat input, coal cleaning plants (with thermal dryers), Kraft pulp mills, portland cement plants, primary zinc smelters, iron and steel mill plants, primary aluminum ore reduction plants, primary copper smelters, municipal incinerators capable of charging more than 250 tons of refuse per day, hydrofluoric, sulfuric, and nitric acid plants, petroleum refineries, lime plants, phosphate rock processing plants, coke oven batteries, sulfur recovery plants, carbon black plants (furnace process), primary lead smelters, fuel conversion plants, sintering plants, secondary metal production plants, chemical process plants, fossil fuel boilers (or combinations thereof) totaling more than 250 million British thermal units per hour heat input, petroleum storage and transfer units with a total storage capacity exceeding 300,000 barrels, taconite ore processing plants, glass fiber processing plants and charcoal production plants.

Once it is determined that a project is subject to PSD preconstruction review, the project emissions are compared to the “significant emission rates” defined in Rule 62-210.200, F.A.C. for the following pollutants:  carbon monoxide (CO); nitrogen oxides (NOX); sulfur dioxide (SO2); particulate matter (PM); particulate matter with a mean particle diameter of 10 microns or less (PM10); volatile organic compounds (VOC); lead (Pb); fluorides (Fl); sulfuric acid mist (SAM); hydrogen sulfide (H2S); total reduced sulfur (TRS), including H2S; reduced sulfur compounds, including H2S; municipal waste combustor organics measured as total tetra- through octa-chlorinated dibenzo-p-dioxins and dibenzofurans; municipal waste combustor metals measured as particulate matter; municipal waste combustor acid gases measured as SO2 and hydrogen chloride (HCl); municipal solid waste landfills emissions measured as non-methane organic compounds (NMOC); and mercury (Hg).  In addition, significant emissions rate also means any emissions rate or any net emissions increase associated with a major stationary source or major modification which would construct within 10 kilometers of a Class I area and have an impact on such area equal to or greater than 1 μg/m3, 24-hour average.

If the potential emission exceeds the defined significant emissions rate of a PSD pollutant, the project is considered “significant” for the pollutant and the applicant must employ the Best Available Control Technology (BACT) to minimize the emissions and evaluate the air quality impacts.  Although a facility or project may be major with respect to PSD for only one regulated pollutant, it may be required to install BACT controls for several “significant” regulated pollutants.

PSD Applicability for Project

As provided in the application, the following table summarizes potential emissions and PSD applicability for the project.

Table A.  Potential Emissions (Tons/Year) and PSD Applicability

| Pollutant | Project Potential Emissions (TPY) | *Significant Emissions Rate (TPY) | Subject To PSD? |
| --- | --- | --- | --- |
| CO | 0 | 100 | No |
| NOX | 0 | 40 | No |
| PM/PM10 | 0 | 25/15 | No |
| SO2 | 0 | 40 | No |
| VOC | 0 | 40 | No |

*Significant Emissions Rate (increase) is defined as a rate equal or exceeding the established values

shown for specific pollutants. Ref. 62-210-200 (280) (a).

As shown in the above table, total potential project emissions will not exceed the PSD significant emissions rate

for this project; therefore, the project is not subject to PSD preconstruction review.

3.  Application Review

Discussion of Emissions

The facility emits VOC emissions.  The VOC emissions come from the blowing agent, butane, used to create voids in the polystyrene foam sheets produced.  The percentage of butane emitted during extrusion, roll stock and thermoforming are based on process knowledge.  Two percent of the butane is emitted during extrusion, four percent during roll stock storage and two percent during thermoforming.  Scrap material from the extrusion process and the thermoforming process emits 100% of the butane remaining in the product after each process.  The process flow and butane emissions are illustrated in Attachment 1.

The facility has taken a limit on production from the extruders and thermoformers to keep their emissions at 289 TPY VOC.  At the time of the last renewal Title V permit (five years ago) the facility requested an increase in production.  An increase in emissions was authorized, however, they took a limit on production to avoid PSD review.  During this construction modification the facility requested an increase in emissions and production.  However, because the facility had removed permitted emissions units and their five year historical production and emissions were less than 50% of the permitted VOC emission limit.  The request for increase emission was unjustified. The Department and applicant agreed to maintain the VOC limit at 289 TPY VOC and the facility would take a limit on their production to keep the VOC emissions at 289 TPY.

State Requirements

Compliance shall be demonstrated by a 12 month consecutive rolling total on extruder production, thermoformer production and VOC emission.  Compliance shall be demonstrated by record keeping and the equation in condition A.6 of the permit.

4.  Preliminary Determination

The Department makes a preliminary determination that the proposed project will comply with all applicable state and federal air pollution regulations as conditioned by the draft permit.  This determination is based on a technical review of the complete application, reasonable assurances provided by the applicant, and the conditions specified in the draft permit.  Susan Machinski is the project engineer responsible for reviewing the application and drafting the permit.  Additional details of this analysis may be obtained by contacting the project engineer at the Permitting Authority’s South District Office at 2295 Victoria Avenue, Suite 364, Fort Myers, Florida 33902.

ATTACHMENT 1

GENPAK SEBRING FACILITY

0550006-008-AC

Technical Evaluation and Preliminary Determination

Extruded material

Roll Stock Storage

Thermoformed material

Thermoforming Scrap to Grinding

Extrusion scrap to Grinding

2% Butane emitted from extrusion process (98% butane remaining in product after extrusion)

98% butane emitted to atmosphere from extrusion scrap (100% - 2% emitted from extrusion)

2% Butane emitted from thermoforming process (92% butane remaining in product after thermoforming)

100% Butane injected

4% Butane emitted during roll stock storage (94% butane remaining in product after roll storage)

92% Butane emitted from thermoforming scrap [100-(2% extrusion +4% roll stock +2% thermoforming)]