TECHNICAL EVALUATION

PRELIMINARY DETERMINATION

Blacklidge Emulsions, Inc., Plant #3

Air Construction Permit

Application Number

0570461-019-AC

Environmental Protection Commission of

Hillsborough County

Tampa, Florida

April 18, 2012

# Project Description

## Applicant:

Danny Sison

Plant Manager

Blacklidge Emulsions, Inc.

2701 East 2nd Avenue

Tampa, Florida 33605

## Engineer:

Dean H. Myers, P.E.

General Environmental Engineering, Inc.

205 W. Palmetto Street

Lakeland, FL 33815

Registration No. 47666

## Project and Location:

On January 26, 2012, Blacklidge Emulsions submitted an AC permit application for the following modifications at Plant #3 and changes to Permit No. 0570461-017-AC: (1) Rename tanks T6, T8 and T9 to become S4, S1 and S2, respectively, and add an additional NG burner (1 MMBtu/hr) to each tank, for a total of two NG burners (2 MMBtu/hr) at each tank S1, S2 and S4; (2) Add one NG burner (1 MMBtu/hr) to the existing asphalt emulsion tank E4;  (3) Relocate tank T6 (or future S4) to the same area with other S series tanks S1, S2 and S3;  (4) Construct a new 8,800 gal soap tank (S3) with electric heat, which will be listed as a source exempt from permitting since there is no VOC emitted from this tank;  (5) Remove the 2 MMBtu/hr hot oil heater and replace it with NG heaters.

The project has been assigned the following NEDS Source Classification Codes (SCCs): 3-05-002-06 (Asphalt Concrete, Asphalt Heater: Natural Gas); 3-05-002-99 (Asphalt Concrete, Comment: Asphalt Storage Tank).  The primary business at this facility is identified by the Standard Industrial Classification (SIC) for Petroleum and Coal Products – 2951, Asphalt Paving Mixtures and Blocks (NAICS 324121).  The project is located at 2701 East 2nd Avenue, Tampa, Hillsborough County, Florida; at longitude 82° 26' 47" W, latitude 27° 57' 24" N.

## Process and Controls:

Blacklidge Emulsions, Inc., Plant #3 is an asphalt blending and mixing facility that produces paving asphalt products.  The facility receives liquid asphalt cement and “off-spec” or unusable paving asphalt by tank truck and stores the asphaltic material in holding tanks, mixing tanks, or “cook-off” tanks.

This permit also incorporates some of the modifications approved under Permit No. 0570461-017-AC, which authorized the construction of one (1) new 18,000 gallon asphalt tank, T6, two (2) new 30,000 gallon asphalt tanks, T8 and T9, and a 2.0 MMBtu/hr hot oil heater to provide auxiliary heat for tanks T8 and T9.  Each new asphalt tank, T6, T8, and T9 would have been equipped with a 1.0 MMBtu/hr natural gas burner under this permit.  As a result, the total heat input at the facility was 22 MMBtu/hr.  Currently, Tanks T8 and T9 are still under construction, as the piping has not been connected to the tanks.  Tank T6 is currently in operation.  Tank T7 is a flux oil storage tank which was determined to be exempt from permitting pursuant to Rule 62-210.300(3)(b)(1), F.A.C.

The AC Permit (-017-AC) also authorized the modification of the Air-ClearTM Fiber-Bed Mist Collection System to increase the volume of displaced fumes collected from all asphaltic material tanks except the emulsion tanks, E1 through E4.  A minimum of 50 acfm (10 acfm for an additional margin of safety) was used to determine the minimum air flow required to control 15 tanks (instead of 12 tanks) for a total of 750 acfm.  The Air-ClearTM system uses a variable frequency drive fan with a maximum capacity of 2,600 acfm.  Therefore, the minimum air flow of 750 acfm is 29% of the maximum capacity and is measured by the velocity flow meter at the inlet of the filter chamber.

This project does not change the permitted material throughput limits.  However, the project will result in an increase in the total heat input for all natural gas heaters to 24 MMBtu/hr from 22 MMBtu/hr.  As a result, the PTE for VOC emissions will be 26.8 TPY, and the PTE for PM emissions will be 5.1 TPY.  The PTE for NOx, CO and SO2 will be 10.5 TPY, 8.8 TPY and 0.1 TPY, respectively.

## Application Information:

Received on:  January 26, 2012

Request for Additional Information:  N/A

Application Complete:  January 26, 2012

# Rule Applicability

This project is subject to the preconstruction review requirements of Chapter 403, Florida Statutes, Chapters, 62-204, 62-210, 62-212, 62-296, and 62-297, Florida Administrative Code (F.A.C.) and Chapter 1-3 of the Rules of the Environmental Protection Commission of Hillsborough County.

This project is subject to the requirements of Rule 62-212.300, General Preconstruction Review Requirements, F.A.C., since the project is not exempt from the permit requirements in Rule 62-210.300, F.A.C.

This project is not subject to the requirements of Rule 62-212.400, Prevention of Significant Deterioration, F.A.C., since the facility is a minor source for PSD.

This project is not subject to the requirements Rule 62-212.500, Preconstruction Review for Nonattainment Areas, F.A.C., since the facility is a minor source and is not located in a designated nonattainment area.

This project is not subject to the requirements Rule 62-213, Operation Permits for Major Sources of Air Pollution, F.A.C., since the facility is a Synthetic Minor Non-Title V source.

This project is subject to the requirements of Rule 62-296.320, General Pollutant Emission Limiting Standards, F.A.C., since the project is a source of particulate matter, volatile organic compounds, and a potential source of odors.

This project is not subject to the requirements of Rule 62-296.401 through 62-296.480, Specific Emission Limiting and Performance Standards, F.A.C., since there is not an applicable source specific category in this rule.

This project is not subject to the requirements of Rule 62-296.500, Reasonably Available Control Technology (RACT) - Requirements for Volatile Organic Compounds (VOC) and Nitrogen Oxides (NOx) Emitting Facilities, F.A.C., since there is not an applicable source specific category in this rule.

This project is not subject to the requirements of Rule 62-296.600, Reasonably Available Control Technology - Lead, F.A.C., since there is not an applicable source specific category in this rule.

This project is not subject to the requirements of Rule 62-296.700, Particulate Matter Reasonably Available Control Technology, F.A.C., since the potential PM emissions are less than 5 pounds per hour and 15 tons per year, pursuant to Rule 62-296.700, F.A.C.

This project is not subject to the requirements of Rule 62-204.800, Federal Regulations Adopted by Reference, F.A.C., since there is not an applicable source specific category in this rule.

This project is subject to the requirements of Chapter 84-446, Laws of Florida and Chapters 1-1.07, Paragraph 2 and 1-3.52, Paragraph 2, Rules of the Environmental Protection Commission of Hillsborough County.

# Summary of Emissions

| Emission Unit (EU) | Regulated Parameter | Actual Emissions (tons/yr) a | Potential Emissions (tons/yr) b | Allowable Limits |
| --- | --- | --- | --- | --- |
| 002 –Tanks & Truck Loading | PM | 0.3 | 0.6 |  |
|  | VOC | 15.4 | 26.2 |  |
|  | VE |  |  | 5% Opacity c |
| 003 – Asphalt Emulsion Tanks | PM | 2.5 | 4.3 |  |
|  | VOC | 11.5 | 19.6 |  |
|  | VE |  |  | 5% Opacity c |
| 004 – Natural Gas Heaters | PM | 0.04 | 0.8 |  |
|  | VOC | 0.03 | 0.6 |  |
|  | NOx | 0.5 | 10.5 |  |
|  | SOx | 0.003 | 0.1 |  |
|  | CO | 0.4 | 8.8 |  |
|  | VE |  |  | 20% Opacity |
| TOTAL d | VOC | 15.4 | 26.8 |  |
|  | PM | 2.6 | 5.1 |  |

# a.	The actual emissions are based on 2010 AOR production of 57,500 tons of paving asphalt and a total natural gas consumed estimated at 96,600 therms (9.66 MMCF).

# b.	The PTE are based on annual production of 98,000 tons of paving asphalt and a total natural gas heat input of 24 MMBtu/hr.

# c.	EUs 002 and 003 are subject to Rule 1-3.52, Rules of the EPC.

# d.	Total emissions are the summation of EU004 and the worst case emissions for either EU002 or EU003 based on the maximum of 98,000 tons of asphalt processed and 4 transfers.

# Conclusions

The revisions proposed by the applicant will meet all of the requirements of Chapters 62-204, 62-210, 62-212, 62-296, and 62-297, F.A.C., and Chapter 1-3, Rules of the Environmental Protection Commission.

The General and Specific Conditions listed in the proposed permit (attached) will assure compliance with all the applicable requirements of Chapters 62-204, 62-210, 62-212, 62-296, and 62-297, F.A.C.

# Proposed Agency Action

Pursuant to Section 403.087, Florida Statutes and Rule 62-4.070, Florida Administrative Code the Environmental Protection Commission of Hillsborough County hereby gives notice of its intent to issue an Air Construction Permit for the construction of four (4) new asphalt tanks, four (4) additional natural gas-fired burners, alteration of the Air-ClearTM fiber-bed mist control system to control 15 asphaltic material tanks, and the removal of the hot oil heater.

CERTIFIED MAIL

In the Matter of an	File No.:	0570461-019-AC

Application for Permit by:	County:	Hillsborough

Danny Sison

Plant Manager

Blacklidge Emulsions, Inc., Plant #3

2701 East 2nd Avenue

Tampa, FL 33605

______________		____________________/

INTENT TO ISSUE

The Environmental Protection Commission of Hillsborough County (EPC), as delegated by the Florida Department of Environmental Protection (DEP) gives notice of its intent to issue a permit (copy attached) for the proposed project as detailed in the application specified above, for the reasons stated below.

The applicant, Blacklidge Emulsions, Inc., applied on January 26, 2012 to the EPC for the following modifications at Plant #3 and changes to the current AC Permit No. 0570461-017-AC: (1) Rename tanks T6, T8 and T9 to S4, S1 and S2, respectively, and add an additional NG burner (1 MMBtu/hr) to each tank for a total of two NG burners (2 MMBtu/hr) at each tank S1, S2 and S4;  (2) Add one NG burner (1 MMBtu/hr) to the existing asphalt emulsion tank E4;  (3) Relocate tank T6 (or future S4) to the same area with other S series tanks S1, S2 and S3;  (4) Construct a new 8,800 gal soap tank (S3) with electric heat, which will be listed as a source exempt from permitting since there is no VOC emitted from this tank S3;  (5) Remove the hot oil heater and replace it with NG heaters.  The facility is located a 2701 East 2nd Avenue, Tampa, Florida.

The EPC has permitting jurisdiction under Chapter 403 Florida Statutes (F.S.) and Florida Administrative Code (F.A.C.) Chapters 62-4 and 62-210.  The project is not exempt from permitting procedures.  The EPC has determined that an air pollution construction permit is required to commence or continue operations at the described facility.

The EPC intends to issue this permit based on the belief that reasonable assurances have been provided to indicate that operation of the source will comply with the appropriate provisions of Florida Administrative Code (F.A.C.) Chapters 62-204 through 62-297 and 62-4.

Pursuant to Section 403.815 and 403.0872, F.S. and Rules 62-103.150 and 62-210.350(3), F.A.C., you (the applicant) are required to publish at your own expense the enclosed Notice of Intent to Issue Permit.  The notice shall be published one time as soon as possible, in the legal advertisement section of a newspaper of general circulation in the area affected.  For the purpose of this rule, "publication in a newspaper of general circulation in the area affected" means publication in a newspaper meeting the requirements of Sections 50.011 and 50.031, F.S., in the county where the activity is to take place.  Where there is more than one newspaper of general circulation in the county, the newspaper used must be one with significant circulation in the area that may be affected by the permit.  If you are uncertain that a newspaper meets these requirements, please contact the EPC at the address or telephone number listed below.  The applicant shall provide proof of publication to the EPC, Air Permitting Section, at 3629 Queen Palm Drive, Tampa, Florida  33619 (Phone 813-627-2600 - FAX 813-627-2660) within 7 (seven) days of publication.  Failure to publish the notice and provide proof of publication within the allotted time may result in the denial of the permit pursuant to Rule 62-103.150(6), F.A.C.

The EPC will issue the final permit with the attached conditions unless a timely petition for an administrative hearing is filed pursuant to Section 120.569 and 120.57 F.S. before the deadline for filing a petition.  The procedures for petitioning for a hearing are set forth below.

A person whose substantial interests are affected by the proposed permitting decision may petition for an administrative proceeding (hearing) under Sections 120.569 and 120.57, F.S.  The petition must contain the information set forth below and must be filed (received) in the Legal Department of the EPC at 3629 Queen Palm Drive, Tampa, Florida 33619, Phone 813-627-2600, Fax 813-627-2602.  Petitions filed by the permit applicant or any of the parties listed below must be filed within 14 (fourteen) days of receipt of this notice of intent.  Petitions filed by any persons other than those entitled to written notice under Section 120.60(3), F.S. must be filed within 14 (fourteen) days of publication of the public notice or within 14 (fourteen) days of receipt of this notice of intent, whichever occurs first.  Under Section 120.60(3), however, any person who asked the EPC for notice of agency action may file a petition within 14 (fourteen) days of receipt of that notice, regardless of the date of publication.  A petitioner shall mail a copy of the petition to the applicant at the address indicated above, at the time of filing.  The failure of any person to file a petition within the appropriate time period shall constitute a waiver of that person's right to request an administrative determination (hearing) under Sections 120.569 and 120.57, F.S.; or to intervene in this proceeding and participate as a party to it.  Any subsequent intervention will be only at the approval of the presiding officer upon the filing of a motion in compliance with Rule 28-106.205, F.A.C.

A petition that disputes the material facts on which the EPC’s action is based must contain the following information:

- The name and address of each agency affected and each agency’s file or identification number if known;

- The name, address, and telephone number of the petitioner and the name, address, and telephone number of each petitioner’s representative, if any, which shall be the address for service purposes during the course of the proceedings; and an explanation of how the petitioner’s substantial interests will be affected by the EPC’s determination;

- A statement of how and when the petitioner received notice of the EPC action;

- A statement of all disputed issues of material fact. If there are none, the petition must so indicate;

- A concise statement of the ultimate facts alleged, including the specific facts the petitioner contends warrant reversal or modification of the EPC’s proposed action;

- A statement of specific rules or statutes the petitioner contends requires reversal or modification of the EPC’s proposed action; and

- A statement of the relief sought by the petitioner, stating precisely the action petitioner wishes the EPC to take with respect to the EPC’s proposed action.

A petition that does not dispute the material facts upon which the EPC’s action is based shall state that no such facts are in dispute and otherwise shall contain the same information as set forth above as required by Rule 28-106.301, F.A.C.

Because the administrative hearing process is designed to formulate final agency action, the filing of a petition means that the EPC's final action may be different from the position taken by it in this notice of intent. Persons whose substantial interests will be affected by any such final decision of the EPC on the application have the right to petition to become a party to the proceeding, in accordance with the requirements set forth above.

Mediation under section 120.573, F.S. is not available in this proceeding.

This action is final and effective on the date filed with the Clerk of the EPC unless a petition is filed in accordance with above.  Upon the timely filing of a petition, this order will not be effective until further order of the EPC.

In addition to the above, a person subject to regulation has a right to apply to the Department of Environmental Protection for a variance from or waiver of the requirements of particular rules, on certain conditions, under Section 120.542, F.S.  The relief provided by this state statute applies only to state rules, not statutes, and not to any federal regulatory requirements.  Applying for a variance or waiver does not substitute or extend the time for filing a petition for an administrative hearing or exercising any other right that a person may have in relation to the action proposed in this notice of intent.

The application for a variance or waiver is made by filing a petition with the Office of General Counsel of the Department of Environmental Protection, 3900 Commonwealth Boulevard, Mail Station #35, Tallahassee, FL 32399-3000.  The petition must specify the following information:

- The name, address, and telephone number of the petitioner,

- The name, address, and telephone number of the attorney or qualified representative of the petitioner, if any,

- Each rule or portion of a rule from which a variance or waiver is requested,

- The citation to the statute underlying (implemented by) the rule identified in (c) above,

- The type of action requested,

- The specific facts that would justify a variance or waiver for the petitioner,

- The reason by the variance or waiver would serve the purposes of the underlying statute (implemented by the rule), and

- A statement whether the variance or waiver is permanent or temporary and, if temporary, a statement of the dates showing the duration of the variance or waiver requested.

The Department will grant a variance or waiver when the petition demonstrates both that the application of the rule would create a substantial hardship or violate principles of fairness, as each of the those terms is defined in Section 120.542(2), F.S., and that the purpose of the underlying statute will be or has been achieved by other means by the petitioner.

Persons subject to regulation pursuant to any federally delegated or approved air program should be aware that Florida is specifically not authorized to issue variances or waivers from any requirements of any such federally delegated or approved program.  The requirements of the program remain fully enforceable by the Administrator of United States Environmental Protection Agency and by any person under the Clean Air Act unless and until the Administrator separately approves any variance or waiver in accordance with the procedures of the federal program.

Any person listed below may request to obtain additional information, a copy of the application (except for information entitled to confidential treatment pursuant to Section 403.111, F.S.), all relevant supporting materials, and all other materials available to the EPC that are relevant to the permit decision.  Interested persons may contact Diana M. Lee, P.E., at the above address or call (813) 627-2600, for additional information.

Any party to this order has the right to seek judicial review of it under Section 120.68 of the Florida Statues, by filing a notice of appeal under rule 9.110 of the Florida rules of Appellate Procedure with the EPC’s Legal Office at 3629 Queen Palm Drive, Tampa, Florida 33619 and with the clerk of the Department of Environmental Protection in the Office of General Counsel, Mail Station 35, 3900 Commonwealth Boulevard, Tallahassee, Florida 32399-3000, and by filing a copy of the notice of appeal accompanied by the applicable filing fees with the appropriate district court of appeal.  The notice must be filed within thirty days after this order is filed with the clerk of the Department.

Executed in Tampa, Florida

ENVIRONMENTAL PROTECTION COMMISSION

OF HILLSBOROUGH COUNTY

__________________________________

Richard D. Garrity, Ph.D.

Executive Director

cc:  Dean H. Meyers, P.E. - General Environmental Engineering, Inc.

## CERTIFICATE OF SERVICE

The undersigned duly designated clerk hereby certifies that this INTENT TO ISSUE and all copies were mailed by certified mail before the close of business on _________________ to the listed persons.

FILING AND ACKNOWLEDGEMENT

FILED, on this date, pursuant to Section 120.52(7), Florida Statutes, with the designated clerk, receipt of which is hereby acknowledged.

_____________________   _____________

Clerk                                       Date

ENVIRONMENTAL PROTECTION COMMISSION

# OF HILLSBOROUGH COUNTY

NOTICE OF INTENT TO ISSUE PERMIT

The Environmental Protection Commission of Hillsborough County (EPC), as delegated by the Florida Department of Environmental Protection (DEP) gives notice of its intent to issue an air pollution Permit No. 0570461-019-AC, to Blacklidge Emulsions, Inc., a bulk asphalt blending plant. The air construction permit authorizes the modification of tanks and the addition of natural gas heaters. The emissions from all the 15 asphaltic material tanks are controlled by the Air-Clear TM Fiber-Bed Mist Collection System. The facility is a Synthetic Non-Title V source located at 2701 East 2nd Avenue, Tampa, Florida.

A Best Available Control Technology (BACT) determination was not required.

The EPC will issue the Final permit with the conditions of the DRAFT permit unless a timely petition for an administrative hearing is filed pursuant to Sections 120.569 and 120.57, F.S. before the deadline for filing a petition.  The procedures for petitioning for hearing are set forth below.

A person whose substantial interests are affected by the proposed permitting decision may petition for an administrative proceeding (hearing) under Sections 120.569 and 120.57, F.S.  The petition must contain the information set forth below and must be filed (received) in the Legal Department of the EPC at 3629 Queen Palm Drive, Tampa, Florida 33619, Phone 813-627-2600, Fax 813-627-2602.  Petitions filed by the permit applicant or any of the parties listed below must be filed within 14 (fourteen) days of receipt of this notice of intent.  Petitions filed by any persons other than those entitled to written notice under Section 120.60(3), F.S. must be filed within 14 (fourteen) days of publication of the public notice or within 14 (fourteen) days of receipt of this notice of intent, whichever occurs first.  Under Section 120.60(3), however, any person who asked the EPC for notice of agency action may file a petition within 14 (fourteen) days of receipt of that notice, regardless of the date of publication.  A petitioner shall mail a copy of the petition to the applicant at the address indicated above, at the time of filing.  The failure of any person to file a petition within the appropriate time period shall constitute a waiver of that person's right to request an administrative determination (hearing) under Sections 120.569 and 120.57, F.S., or to intervene in this proceeding and participate as a party to it.  Any subsequent intervention will be only at the approval of the presiding officer upon the filing of a motion in compliance with Rule 28-106.205 of the F.A.C.

A petition that disputes the material facts on which the EPC’s action is based must contain the following information:

- The name and address of each agency affected and each agency’s file or identification number if known;

- The name, address, and telephone number of the petitioner, and the name, address, and telephone number of each petitioner’s representative, if any, which shall be the address for service purposes during the course of the proceedings; and an explanation of how the petitioner’s substantial interests will be affected by the agency determination;

- A statement of how and when petitioner received notice of the EPC action;

- A statement of all disputed issues of material fact.  If there are none, the petition must so indicate;

- A concise statement of the ultimate facts alleged, including the specific facts the petitioner contends warrant reversal or modification of the EPC proposed action;

- A statement of specific rules or statutes the petitioner contends requires reversal or modification of the EPC’s proposed action; and

- A statement of the relief sought by the petitioner, stating precisely the action petitioner wishes the EPC to take with respect to the EPC’s proposed action.

A petition that does not dispute the material facts upon which the EPC’s action is based shall state that no such facts are in dispute and otherwise shall contain the same information as set forth above as required by Rule 28-106.301.

Because the administrative hearing process is designed to formulate final agency action, the filing of a petition means that the EPC's final action may be different from the position taken by it in this notice of intent.  Persons whose substantial interests will be affected by any such final decision of the EPC on the application have the right to petition to become a party to the proceeding, in accordance with the requirements set forth above.

Mediation under section 120.573, F.S. is not available in this proceeding.

This action is final and effective on the date filed with the Clerk of the EPC unless a petition is filed in accordance with above.  Upon the timely filing of a petition this order will not be effective until further order of the EPC.

Any party to this order has the right to seek judicial review of it under Section 120.68 of the Florida Statues, by filing a notice of appeal under rule 9.110 of the Florida rules of Appellate Procedure with the EPC’s Legal Office at 3629 Queen Palm Drive, Tampa, Florida 33619 and with the clerk of the Department of Environmental Protection in the Office of General Counsel, Mail Station 35, 3900 Commonwealth Boulevard, Tallahassee, Florida 32399-3000, and by filing a copy of the notice of appeal accompanied by the applicable filing fees with the appropriate district court of appeal.  The notice must be filed within thirty days after this order is filed with the clerk of the Department.

The complete project file is available for public inspection during normal business hours, 8:00 a.m. to 5:00 p.m., Monday through Friday, except legal holidays, at the Environmental Protection Commission of Hillsborough County, 3629 Queen Palm Drive, Tampa, Florida 33619.  The complete project file includes the proposed Permit, the application, and the information submitted by the responsible official, exclusive of confidential records under Section 403.111, F.S.  Interested persons may contact Diana M. Lee, P.E., at the above address, or call 813-627-2600, for additional information.  Any written comments filed shall be available for public inspection.  If written comments received result in a significant change in the proposed agency action, the EPC shall revise the proposed permit and require, if applicable, another Public Notice.

ENVIRONMENTAL PROTECTION COMMISSION OF

HILLSBOROUGH COUNTY, as Delegated by

STATE OF FLORIDA

DEPARTMENT OF ENVIRONMENTAL PROTECTION

NOTICE OF PERMIT

Danny Sison

Plant Manager

Blacklidge Emulsions, Inc., Plant #3

2701 East 2nd Avenue

Tampa, FL 33605

Re:  Hillsborough County - AP

Dear Mr. Sison:

Enclosed is Permit Number 0570461-019-AC for the modification of the asphalt tanks and heaters, and the elimination of the hot oil heater at Plant #3, issued pursuant to Section 403.087, Florida Statutes.

Any party to this order (permit) has the right to seek judicial review of the permit pursuant to Section 120.68, Florida Statutes, by the filing of a Notice of Appeal pursuant to Rule 9.110, Florida Rules of Appellate Procedure, with the Clerk of the EPC in the Legal Department at 3629 Queen Palm Dr., Tampa, FL  33619; and by filing a copy of the Notice of Appeal accompanied by the applicable filing fees with the appropriate District Court of Appeal.  The Notice of Appeal must be filed within 30 days from the date this Notice is filed with the clerk of the EPC.

Executed in Tampa, Florida.

Sincerely,

Richard D. Garrity, Ph.D.

Executive Director

RDG/KRZ/krz

cc:  Dean H. Meyers, P.E. - General Environmental Engineering, Inc.

CERTIFICATE OF SERVICE

This is to certify that this NOTICE OF PERMIT and all copies were mailed before the close of business on _________________________ to the listed persons.

Clerk Stamp

FILED, on this date, pursuant to Section 120.52(7), Florida Statutes, with the designated clerk, receipt of which is hereby acknowledged.

___________________________   _____________

Clerk	Date

PERMITTEE:	PERMIT/CERTIFICATION

Danny Sison, Plant Manager	Permit No.: 0570461-019-AC

Blacklidge Emulsions, Inc., Plant #3	County: Hillsborough

2701 East 2nd Avenue	Expiration Date:  April 30, 2013

Tampa, Florida 33605	Project: Modification – Tanks, NG/Oil Heaters

This permit is issued under the provisions of Chapter 403, Florida Statutes (F.S.), and Florida Administrative Code (F.A.C.) Chapters 62-204, 62-210, 62-212, 62-213, 62-296, 62-297, and Chapter 62-4.  The above named permittee is hereby authorized to perform the work or operate the facility shown on the application and approved drawing(s), plans and other documents, attached hereto or on file with the Environmental Protection Commission (EPC) of Hillsborough County and made a part hereof and specifically described as follows:

This permit authorizes the following modifications at Plant #3: (1) Rename tanks T6, T8, and T9 to become S4, S1, and S2, respectively, and add an additional natural gas (NG) burner (1 MMBtu/hr) to each tank for a total of two NG burners (2 MMBtu/hr) at each tank S1, S2, and S4; (2) Add one NG burner (1 MMBtu/hr) to the existing asphalt emulsion tank E4; (3) Relocate tank T6 (or future S4) to the same area with other S series tanks S1, S2 and S3; (4) Construct a new 8,800 gal soap tank (S3) with electric heat, which will be listed as a source exempt from permitting since there is no VOC emitted from this tank; and (5) Remove the 2 MMBtu/hr hot oil heater and replace it with NG heaters.

Blacklidge Emulsions, Inc., Plant #3 is an asphalt blending and mixing facility that produces paving asphalt products.  The facility receives liquid asphalt cement and “off-spec” or unusable paving asphalt by tank truck and stores the asphaltic material in holding tanks, mixing tanks, or “cook-off” tanks.  Tanks, T1, T2 and T3 are the “cook-off” tanks, T4 and T5 are for mixing and storage of recycled asphalt (RA). Tanks S1, S2 and S4 (identified as T8, T9 and T6 under Permit No. 0570461-017-AC) will store modified asphalt blend, and Tank S4 will be relocated to the same area with other S-series tanks.  The 8,000 gallon tank, T7, stores flux oil.  The new 8,800 gallon tank, S3, which will be constructed under this AC Permit, will store soap with electric heat.  Both Tanks T7 and S3 were determined to be exempt from permitting pursuant to Rule 62-210.300(3)(b)(1), F.A.C.

The particulate matter and volatile organic compounds emitted from the heated asphalt cook-off, mixing, and holding tanks are controlled by the Air-Clear TM Fiber-Bed Mist Collection system and Carbon Adsorber Unit.  An induced-draft fan located downstream of the carbon adsorber provides sufficient air flow through the air pollution and odor control system to evacuate the headspace of all heated asphalt tanks during tank loading or unloading, mixing and blending, and maintain negative pressures within the tank when the tank hatch is opened to add polymer, aggregate, or any other additives to manufacture paving asphalt products.

This AC Permit does not change the permitted material throughput limits.  However, the AC Permit will result in an increase in the total heat input for all natural gas heaters to 24 MMBtu/hr.  All 15 asphaltic material tanks will be controlled by the Air-ClearTM fiber-bed mist control system and carbon adsorber.

Location:  2701 East 2nd Avenue, Tampa, Hillsborough County

UTM Coordinates: 17-359.5 E  3093.2 N

NEDS No.:  0570461

| EU ID# | Description | Capacity |
| --- | --- | --- |
| 002 – Tanks and Truck Loading | Raw Asphalt Storage tank (H1) Raw Asphalt Storage tank (H2) Raw/Product Asphalt Storage tank (H3) Product Asphalt Storage tank (H4) Product Asphalt Storage tank (H5) “Cook-off” Tank (T1) “Cook-off” Tank (T2) “Cook-off” Tank (T3) Recycled Asphalt Tank (T4) Recycled Asphalt Tank (T5) Modified Blended Asphalt Tank (S4) - new Modified Blended Asphalt Tank (S1) - new Modified Blended Asphalt Tank (S2) - new Polymer Mix Tank (P1) Polymer Mix Tank (P2) Asphalt Product Truck Loading | 25,000 gallons 25,000 gallons 25,000 gallons 25,000 gallons 25,000 gallons 5,000 gallons 5,000 gallons 10,000 gallons 15,000 gallons 15,000 gallons 18,000 gallons 30,000 gallons 30,000 gallons 25,000 gallons 25,000 gallons |
| 003 – Asphalt Emulsion Tanks | Asphalt Emulsion Tank (E1) Asphalt Emulsion Tank (E2) Asphalt Emulsion Tank (E3) Asphalt Emulsion Tank (E4) | 13,500 gallons 13,500 gallons 29,000 gallons 29,000 gallons |
| 004 – Natural Gas Heaters | H1 Natural Gas-fired Tank Heater H2 Natural Gas-fired Tank Heater H3 Natural Gas-fired Tank Heater H4 Natural Gas-fired Tank Heater H5 Natural Gas-fired Tank Heater T1 Natural Gas-fired Tank Heater T2 Natural Gas-fired Tank Heater T3 Natural Gas-fired Tank Heater T4 Natural Gas-fired Tank Heater T5 Natural Gas-fired Tank Heater S4 Natural Gas-fired Tank Heater - new S1 Natural Gas-fired Tank Heater - new S2 Natural Gas-fired Tank Heater - new P1 Natural Gas-fired Tank Heater P2 Natural Gas-fired Tank Heater E4 Asphalt Emulsion Tank Heater | 1 MMBtu/hr 1 MMBtu/hr 1 MMBtu/hr 1 MMBtu/hr 1 MMBtu/hr 1 MMBtu/hr 1 MMBtu/hr 2 MMBtu/hr 2 MMBtu/hr 2 MMBtu/hr 2 MMBtu/hr 2 MMBtu/hr 2 MMBtu/hr 2 MMBtu/hr 2 MMBtu/hr 1 MMBtu/hr |

Reference Permit No.: 0570461-017-AC, 0570461-016-AF

- A part of this permit is the attached General Conditions. [Rule 62-4.160, F.A.C.]

- Issuance of this permit does not relieve the permittee from complying with applicable emission limiting standards or other requirements of Chapters 62-204, 62-210, 62-212, 62-296 and 62-297, F.A.C., or any other requirements under federal, state, or local law. [Rule 62-210.300, F.A.C.]

- All applicable rules of the Environmental Protection Commission of Hillsborough County including design discharge limitations specified in the application shall be adhered to.  The permit holder may also need to comply with county, municipal, federal, or other state regulations prior to construction. [Rule 62-4.070(7), F.A.C.]

- The use of property, facilities, equipment, processes, products, or compounds, or any other act that causes or materially contributes to a public nuisance is prohibited, pursuant to the Hillsborough County Environmental Protection Act, Section 16, Chapter 84-446, Laws of Florida, as Amended.

Operation and Emission Limitations

- The facility is permitted to operate continuously; 8,760 hours per consecutive 12 month period.  [Rule 62-210.200 (Potential to Emit), F.A.C.]

- In order to establish this facility as a Synthetic Non-Title V Source, and as requested by the permittee, the facility shall not produce more than 98,000 tons of paving asphalt product in any consecutive 12 month period.  [Rule 62-4.070(3), F.A.C., Air Construction (AC) Permit No. 0570461-017-AC and AC Permit Application received January 26, 2012]

- In order to establish this facility as a Synthetic Non-Title V Source, the facility operations shall be limited as follows:

  - The temperature of the asphalt material mixing and storage tanks shall not exceed 375 °F;

  - Only asphalt cement as presented in the Construction Application shall be used.  The use of lower boiling asphalt cement or cutback asphalt without prior approval by the EPC is prohibited;

  - The tank heaters for Tanks T1, T2, H1, H2, H3, H4, H5 and E4 shall be fired by natural gas with a maximum heat input of 1 MMBTU/hour per tank;

  - The tank heaters for Tanks P1, P2, T3, T4, T5, S1, S2 and S4 shall be fired by natural gas with a maximum heat input of 2 MMBTU/hour per tank;

[Rules 62-4.070(3), F.A.C., AC Permit No. 0570461-017-AC and AC Permit Application received January 26, 2012]

- The maximum allowable visible emissions from the Tank & Truck Loading activities (EU002) and the Asphalt Emulsion Tanks (EU003) shall not be equal to or greater than 5% opacity.  The maximum allowable visible emissions from the natural gas-fired tank heaters (EU004) shall not be equal to or greater than 20% opacity.

| Emission Unit | Emission Point | Description | VE Limit (% opacity) |
| --- | --- | --- | --- |
| 002 | n/a | Tanks and Truck Loading (vented through single control device) | 5 |
| 003 | 001 002 003 004 | Emulsion Tank E1 Emulsion Tank E2 Emulsion Tank E3 Emulsion Tank E4 | 5 5 5 5 |
| 004 | n/a | Natural Gas Heaters (each heater) | 20 |

[Rule 62-296.320, F.A.C., and Chapter 1-3.52, Rules of the Environmental Protection Commission of Hillsborough County (EPC)]

- The facility shall maintain the system to control Visible Emissions and Odors from EU002.  The system shall be configured with the following specifications:

A.  An Air-Clear TM fiber-bed mist collection system with a maximum exhaust flow of 2,600 acfm;

B.  As recommended by the manufacturer, the inlet temperature of the Air-Clear TM system 	shall be in the range of 120° - 140° Fahrenheit (F).  In addition, since the cooling of the exhaust 	prior to the Air-Clear TM system is imperative in effectively  collecting the submicron 	particulates that cause opacity, the permittee shall make all attempts to have the exhaust be on 	the lower side of the temperature range;

C.  The facility shall install, calibrate, operate, and maintain an inlet thermometer and inlet 	temperature chart recorder in degrees Fahrenheit for the Air-Clear TM fiber-bed mist collection 	system;

D.  The facility shall install, calibrate, operate, and maintain a pressure drop measuring device 	in inches of water across the main fiber-bed diffusion filters of the Air-Clear TM system.  As 	recommended by the manufacturer, the pressure drop of the main fiber-bed filters shall be in the 	range of 0 – 3 inches water gauge (w.g.) with clean filters;

E.  The facility shall install, calibrate, operate, and maintain a velocity pressure gauge at the 	inlet of the Air-Clear TM fiber-bed mist collection system that is integrated with the variable 	frequency drive fan to verify adequate air flow through the system.  As indicated in the Air-	Clear TM O&M Manual (Attachment B), the normal operating range should fall between 1” 	and 4” w.g.;

F.  Prior to each instance of the addition of polymer additive to the polymer mix tanks, the 	facility shall verify adequate air flow as measured by the velocity pressure gauge;

G.  A complete spare filter set (enough to replace all the filters in the unit) for the Air-Clear TM 	system shall be kept onsite;

H.  A Vapor Phase Carbon Adsorber Unit containing at least 1,600 pounds of activated carbon 	and designed for an airflow of up to 4,000 acfm shall be connected to the exhaust of the Air-	Clear TM fiber-bed mist collection system;

I.  The Carbon Adsorber Unit shall be monitored and maintained as prescribed in the Carbon 	Adsorber OM&M Plan (Attachment C) to maintain the efficiency of the odor control system;

J.  As per manufacture’s recommendation, the facility shall measure the inlet and outlet 	temperature of the Carbon Adsorber Unit immediately upon installation of a new carbon 	canister and monitor the outlet temperature every four (4) hours for the first twenty-four (24) 	hours of operation.

[Rule 62-4.070(3), F.A.C., Permit No. 0570461-018-AC, Construction Application received January 26, 2012, and Carbon Adsorber and Air ClearTM O&M Manuals]

- [RESERVED]

- No person shall store, pump, handle, process, load, unload, or use in any process or installation any VOC/OS without applying known and existing vapor emission control devices or systems deemed necessary and ordered by the Environmental Protection Commission of Hillsborough County.  The following procedures shall be utilized to minimize pollutant emissions:

- Maintain covers, lids, etc., on all containers of VOC/OS when they are not being handled, tapped, etc.

- Prevent excessive air turbulence across exposed VOC/OS.

- Where possible and practical, procure/fabricate a tightly fitting cover for any open trough, basin, bath, etc., of VOC/OS so that it can be covered when not in use.

- All fittings, valve lines, etc., shall be properly maintained.

- All VOC/OS spills shall be attended to in a timely manner and the waste properly disposed of, recycled, etc.

[Rule 62-296.320(1), F.A.C.]

- The permittee shall not cause, suffer, allow or permit the discharge of air pollutants which cause or contribute to an objectionable odor.  An objectionable odor is any odor present in the outdoor atmosphere which by itself or in combination with other odors, is or may be harmful or injurious to human health or welfare, which unreasonably interferes with the comfortable use and enjoyment of life or property, or which creates a nuisance.  During tank loading/loadout and tank to tank/blending operations, a representative of Blacklidge Emulsions shall daily monitor the downwind property boundary for odors.  If objectionable odors are noted, the operation(s) shall be curtailed or halted.  If the Environmental Protection Commission of Hillsborough County receives odor complaints attributed to this facility and they are determined to be a nuisance, the permittee will need to evaluate their operations and propose corrective actions that may include installing additional odor control measures for this facility.  [Rules 62-4.070(3), 62-296.320, F.A.C., and Chapter 1-3.22, Paragraph 3, Rules of the EPC.]

- All reasonable precautions shall be taken to prevent and control generation of unconfined emissions of particulate matter in accordance with the provision in Rule 62-296.320, F.A.C.  These provisions are applicable to any source, including, but not limited to, vehicular movement, transportation of materials, construction, alterations, demolition or wrecking, or industrial related activities such as loading, unloading, storing and handling.  Reasonable precautions shall include, but not limited to, the following:

- Maintenance of parking areas and yards.  Use water or dust suppressants, if necessary;

- Removal of particulate matter from paved areas and work areas under the control of the owner/operator;

- Monitor each tank loading operation and observe the associated equipment and stack for visible emissions;

- Curtail or halt operation if visible emissions are noted.

[Rule 62-296.320, F.A.C.]

- The permittee shall not circumvent any air pollution control device, or allow the emissions of air pollutants without the applicable air pollution control device operating properly.  [Rules 62-4.070(3), and 62-210.650, F.A.C.]

- Excess emissions which are caused entirely or in part by poor maintenance, poor operation, or any other equipment or process failure which may reasonably be prevented during startup, shutdown, or malfunction shall be prohibited. The pollution control equipment (fiber-bed mist eliminator, odor control system, and associated equipment) shall be maintained in good repair to perform adequately the function for which it was intended.  The permittee shall lower the temperature in the tanks as necessary to comply with the limitations in Specific Condition Nos. 8 and 9.  Maintenance shall include, but is not limited to, monthly inspections and repair or replacement of faulty equipment when necessary.  [Rules 62-4.070(3), 62-210.700(4), and 62-210.700(5), F.A.C.]

##### Testing and Compliance Requirements

- In order to demonstrate compliance with Specific Condition No. 8, the permittee shall conduct a visible emission test on the carbon absorber exhaust stack within thirty (30) days of the start-up of all new tanks controlled by the Air-ClearTM fiber-bed mist collection system.  [Rules 62-297.310(7)(a), F.A.C. and Chapter 1-3.50, Rules of the EPC]

- Test emission units 002, 003, and 004 for visible emissions annually, during each federal fiscal year (October 1 through September 30).  Concurrent with the VE test for EU 002, perform an odor evaluation (test team’s opinion whether objectionable odors are being emitted from the facility).  The odor evaluation shall be conducted during normal tank loading/loadout and tank to tank/blending operations.  The odor evaluation report shall include the names of the odor evaluation team members, date, time, wind direction, observation, tank loading/loadout and tank to tank/blending operations taking place at the time, and corrective actions taken.  [Rule 62-297.310(4)(a)2 and (8), F.A.C.]

- Visible emissions shall be tested by a certified observer in accordance with EPA Method 9 for a minimum of thirty (30) minutes and shall include the period during which the highest opacity emissions can reasonably be expected to occur.  [Rules 62-297.310(4)(a)2, and 62-297.401(9)(a) F.A.C., and Chapter 1-3.50, Rules of the EPC]

- The permittee shall notify the Air Compliance Section of the EPC at least 15 days prior to the date on which each formal compliance test is to begin, of the date, time, and place of each such test, and the test contact person who will be responsible for coordinating and having such test conducted.  [Rule 62-297.310(7)(a)(9), F.A.C.]

- The permittee will submit two (2) copies of the compliance test report required by this permit to the Air Compliance Section of the EPC within forty-five (45) days after the test is complete.  The test report shall contain sufficient detail on the source tested and the test procedures used to allow the EPC to determine if the test was properly conducted and the test results properly computed.  [Rule 62-297.310(8), F.A.C.]

- When the Environmental Protection Commission of Hillsborough County (EPC) after investigation, has good reason (such as complaints, increased visible emissions, or questionable maintenance of control equipment) to believe that any applicable emission standard contained in Rules 62-204, 62-210, 62-212, 62-296, or 62-297, F.A.C., or in a permit issued pursuant to those rules is being violated, it shall require the owner or operator of the source to conduct compliance tests which identify the nature and quantity of pollutant emissions from the source and to provide a report on the results of said tests to the EPC. [Rule 62-297.310(7)(b), F.A.C.]

# Monitoring and Recordkeeping Requirements

- The permittee shall maintain a monthly log of the operation.  The log shall include, at a minimum, the following items and be kept for a minimum of three (3) years and made available for inspection to representatives of the Environmental Protection Commission of Hillsborough County upon request.

- Daily maximum temperature of each blend batch (RA asphalt, polymer modified asphalt, and emulsified asphalt) of paving asphalt product;

- Total weight of emulsified asphalt produced, in short tons, on a monthly basis and the rolling total for the previous consecutive 12 month period;

- Total weight of all paving asphalt produced, in short tons, on a monthly basis and the rolling total for the previous consecutive 12 month period;

- Copies of the inlet temperature charts to the Air-ClearTM system as required in Specific Condition No. 9.C.;

- Weekly pressure drop, in inches of water, of the main fiber-bed diffusion filters for the Air-ClearTM system when unloading, transferring, mixing, or loading occurs.

- Weekly velocity pressure gauge reading for the Air-ClearTM system when unloading, transferring, mixing, or loading occurs;

- Record of odor monitoring downwind of property per Specific Condition No. 9.H. that shall include date, time, wind direction, observation, tank loading/loadout, tank to tank/blending operations taking place at the time, operator initials, and corrective actions taken;

- Any maintenance/repair performed on the Air-ClearTM system.

[Rule 62-4.070(3) and 62-4.160(14)(b), F.A.C. and AC Permit No. 0570461-017-AC]

- The permittee shall submit to the Environmental Protection Commission of Hillsborough County, by April 1st of each calendar year, a completed DEP Form 62-210.900(5), "Annual Operating Report (AOR) for Air Pollutant Emitting Facility", for the preceding calendar year.   [Rule 62-210.370(3), F.A.C.]

Permit Applications and Transfers

- If the permittee wishes to transfer this permit to another owner, an "Application for Transfer of Permit" (DEP Form 62-210.900(7)) shall be submitted, in duplicate, to the Environmental Protection Commission of Hillsborough County within 30 days after the sale or legal transfer of the permitted facility. [Rule 62-4.120, F.A.C.]

- The permittee shall provide timely notification to the Environmental Protection Commission of Hillsborough County prior to implementing any changes that may result in a modification to this permit pursuant to Rule 62-210.200, F.A.C., Modification.  The changes do not include normal maintenance, but may include, and are not limited to, the following, and may also require prior authorization before implementation:

- Alteration or replacement of any equipment or major component of such equipment;

- Installation or addition of any equipment which is a source of air pollution.

Note:	Items A and B are not applicable to routine maintenance, repair, or replacement of component parts of an air emissions unit. [Rules 62-210.300 and 62-4.070(3), F.A.C.]

- A complete application for an air operation permit shall be submitted to the Environmental Protection Commission of Hillsborough County within 60 days of completion of the visible emissions test required under Specific Condition No. 16 or at least 90 days prior to the expiration date of this permit, whichever occurs first.  To properly apply for an operation permit, the applicant shall submit the following:

- The appropriate Department application form [see Rule 62-210.900, F.A.C. (Forms and Instructions)];

- The appropriate operation permit application fee(s);

- Copies of the records specified in Specific Condition No. 22 for the most recent three (3) months of operation.

[Rules 62-4.070(3), 62-4.090, 62-210.300(2), and 62-210.900, F.A.C.]

ENVIRONMENTAL PROTECTION COMMISSION

OF HILLSBOROUGH COUNTY

_________________________________

Richard D. Garrity, Ph.D.

Executive Director