TECHNICAL EVALUATION

&

PRELIMINARY DETERMINATION

APPLICANT

Lee County Solid Waste Division

10500 Buckingham Road

Fort Myers, Florida  33905

Lee County Solid Waste Resource Recovery Facility

Facility ID No. 0710119

PROJECT

Project No. 0710119-014-AC

PSD-FL-151H

Application for Minor Source Air Construction Permit

Re-Permitting of Facility

COUNTY

Lee County, Florida

PERMITTING AUTHORITY

Florida Department of Environmental Protection

Division of Air Resource Management

Office of Permitting and Compliance

2600 Blair Stone Road, MS#5505

Tallahassee, Florida  32399-2400

April 04, 2017

- General Project INFORMATION

  - Air Pollution Regulations

Projects at stationary sources with the potential to emit air pollution are subject to the applicable environmental laws specified in Section 403 of the Florida Statutes (F.S.).  The statutes authorize the Department of Environmental Protection (Department) to establish regulations regarding air quality as part of the Florida Administrative Code (F.A.C.), which includes the following applicable chapters:  62-4 (Permits); 62-204 (Air Pollution Control – General Provisions); 62-210 (Stationary Sources – General Requirements); 62-212 (Stationary Sources – Preconstruction Review); 62-213 (Operation Permits for Major Sources of Air Pollution); 62-296 (Stationary Sources - Emission Standards); and 62-297 (Stationary Sources – Emissions Monitoring).  Specifically, air construction permits are required pursuant to Chapters 62-4, 62-210 and 62-212, F.A.C.

In addition, the U. S. Environmental Protection Agency (EPA) establishes air quality regulations in Title 40 of the Code of Federal Regulations (CFR).  Part 60 specifies New Source Performance Standards (NSPS) for numerous industrial categories.  Part 61 specifies National Emission Standards for Hazardous Air Pollutants (NESHAP) based on specific pollutants.  Part 63 specifies NESHAP based on the Maximum Achievable Control Technology (MACT) for numerous industrial categories.  The Department adopts these federal regulations in Rule 62-204.800, F.A.C.

  - Glossary of Common Terms

Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of this permit.

  - Facility Description and Location

The Lee County Resource Recovery Facility (RRF) is an existing municipal waste combustion facility, which is categorized under Standard Industrial Classification Code No. 4953.  The existing Lee County RRF is in Lee County at 10500 Buckingham Road in Fort Myers, Florida.  The location of Lee County is shown in Figure 1, and a map indicating the location of the facility is shown in Figure 2.  The UTM coordinates of the existing facility are Zone 17, 424.21 kilometers (km) East, and 2945.7 km North.  This site is in an area that is in attainment (or designated as unclassifiable) for all air pollutants subject to Ambient Air Quality Standards (AAQS).

Lee County

Figure 1.  Location of Lee County, Florida.

Figure 2.  Location of Lee County RRF, indicated with a red star.

The facility consists of three municipal waste combustion (MWC) units, an ash handling system, and several small emergency engines.  MWC Units 1 and 2 began commercial operation in 1994, and MWC Unit 3 began operation in 2007.  The three units combined can produce up to 60 megawatts of electricity.

Units 1 and 2 each have a nominal design rated capacity of 660 tons of municipal solid waste (MSW) per day and 186,200 lbs/hour steam production with MSW having a heating value of 5,000 Btu/lb.  Short term capacity is restricted by limiting steam production to 186,200 lbs/hour, which effectively limits heat input.  Odor is controlled by drawing combustion air from the refuse tipping floor area.  Spray dry absorbers and baghouses are used for the control of acid gases and particulate matter, selective non-catalytic reduction (SNCR) systems for the control of nitrogen oxides (NOx), and activated carbon injection (ACI) systems for the control of mercury (Hg) and certain organic emissions.  Activated carbon is injected into the flue gases prior to the baghouses to control Hg emissions.  Captured dry ash particles fall into hoppers and are transported by an enclosed conveyor system to the bottom ash discharger where they are wetted to prevent dust, and mixed with the bottom ash.

Unit 3 has similar characteristics to Units 1 and 2.  Unit 3 has a nominal design rated capacity of 660 tons MSW per day and 197,400 lbs/hour steam production with MSW having a heating value of 5,000 Btu/lb.  Short term capacity is restricted by limiting steam production to 197,400 lbs/hour, which effectively limits heat input.  The pollution controls on Unit 3 are very similar to those on Units 1 and 2, though Unit 3 is also equipped with an option flue gas recirculation system for the control of NOx.

A photo of the facility, from the Lee County Solid Waste Division’s website, is shown in Figure 3.  The stack shown in this figure includes three separate flues, one for each MWC unit.

  - Facility Regulatory Categories

- The facility is a major source of hazardous air pollutants (HAP).

- The facility does not operate units subject to the acid rain provisions of the Clean Air Act.

- The facility is a Title V major source of air pollution in accordance with Chapter 62-213, F.A.C.

- The facility is a major stationary source in accordance with Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

Figure 3.  Photo of Lee County RRF.

  - Project Description

- The initial PSD permit (Permit No. PSD-FL-151) for the facility, including the construction of Units 1 and 2, was issued in 1992.  This permit was modified through letters from the Department several times in 1994 and 1995.  A permit modifying the initial PSD permit (PSD-FL-151B) was issued in 2000.  Another PSD permit for the construction of Unit 3 (0710119-002-AC/PSD-FL-151C) was issued in 2003.  These permits have been slightly modified several times over the intervening years through a series of construction permits.  These permits have included the provisions of federal rules in 40 CFR 60, Subpart Cb (for Units 1 and 2), and Subpart Eb (for Unit 3).  Additionally, small changes have been made to the permitted slate of wastes that are permitted to be burned at the facility, including the addition of biosolids as a permitted waste.  Also, changes to the monitoring requirements for ammonia emissions have been part of these various air construction permits.

- The permittee has requested that the valid provisions of these previous air construction permits be collected into one new permit that would supersede all previous air construction permits, including PSD permits.  This permit would not remove any applicable requirements from these permits or add new requirements; rather the requested permit simply collects all of the valid requirements from all previous permits into one clean document.  No new limits are established in this permit, and no valid requirements are removed.

- The following existing emissions units (EU) will be affected by this project.

| - EU No. | - Description |
| --- | --- |
| 001 | Municipal Waste Combustor Unit 1 |
| 002 | Municipal Waste Combustor Unit 2 |
| 004 | Ash Handling System |
| 006 | Municipal Waste Combustor Unit 3 |
| 008 | Diesel Emergency Generator - 330 horsepower (HP) |
| 009 | Fire Pump No. 1 - 305 HP |
| 010 | Fire Pump No. 2 - 300 HP |
| 011 | Fire Pump No. 3 - 64 HP (Waste-to-Energy (WTE) Transfer Station) |

- No emissions units will be added or removed by this project.

  - Processing Schedule

February 27, 2017	Department received the complete application for an air pollution construction permit and a concurrent Title V air operation permit revision.

April 4, 2017		Department issued Draft Permit package.

- PSD Applicability

  - General PSD Applicability

For areas, currently in attainment with the AAQS or areas otherwise designated as unclassifiable, the Department regulates major stationary sources of air pollution in accordance with Florida’s PSD preconstruction review program as defined in Rule 62-212.400, F.A.C.  Under preconstruction review, the Department first must determine if a project is subject to the PSD requirements (“PSD applicability review”) and, if so, must conduct a PSD preconstruction review.  A PSD applicability review is required for projects at new and existing major stationary sources.  In addition, proposed projects at existing minor sources are subject to a PSD applicability review to determine whether potential emissions from the proposed project itself will exceed the PSD major stationary source thresholds.  A facility is considered a major stationary source with respect to PSD if it emits or has the potential to emit:

- 5 tons/year or more of lead;

- 250 tons/year or more of any regulated air pollutant; or

- 100 tons/year or more of any regulated air pollutant and the facility belongs to one of the following 28 PSD-major facility categories:  fossil fuel-fired steam electric plants of more than 250 million British thermal units per hour (MMBtu/hour) heat input, coal cleaning plants (with thermal dryers), Kraft pulp mills, Portland cement plants, primary zinc smelters, iron and steel mill plants, primary aluminum ore reduction plants, primary copper smelters, municipal incinerators capable of charging more than 250 tons of refuse/day, hydrofluoric, sulfuric, and nitric acid plants, petroleum refineries, lime plants, phosphate rock processing plants, coke oven batteries, sulfur recovery plants, carbon black plants (furnace process), primary lead smelters, fuel conversion plants, sintering plants, secondary metal production plants, chemical process plants, fossil fuel boilers (or combinations thereof) totaling more than 250 MMBtu/hour heat input, petroleum storage and transfer units with a total storage capacity exceeding 300,000 barrels, taconite ore processing plants, glass fiber processing plants and charcoal production plants.

The list of the 28 source categories does include the category of “municipal incinerators capable of charging more than 250 tons of refuse per day.”  Therefore, the Lee County RRF has a major stationary source threshold is 100 tons/year or more of any PSD pollutant.  As already mentioned, this facility is a major stationary source for the PSD of Air Quality because it emits more than 100 tons/year or more of a regulated air pollutant.

Once it is determined that a facility is a PSD major source, PSD applicability for any project at the facility is based on emissions thresholds known as the “significant emission rates” (SER) as defined in Rule 62-210.200(Definitions), F.A.C.  Emissions of PSD pollutants from the project meet or exceed these rates are considered “significant” and the Best Available Control Technology (BACT) must be employed to minimize emissions of each PSD pollutant.  Although a facility may be “major” for only one PSD pollutant, a project must include BACT controls for any PSD pollutant that equals or exceeds the corresponding SER.  SER threshold by pollutants are given in Table 1.

Table 1 – List of SER by PSD Pollutant.

| Pollutant | SER (TPY) | Pollutant | SER (TPY) |
| --- | --- | --- | --- |
| CO | 100 | NOX | 40 |
| PM/PM10/PM2.5 | 25/15/10 | Ozone (VOC) 2 | 40 |
| PM2.5 (NOX) | 40 | PM2.5 (SO2) | 40 |
| Ozone (NOX) 2 | 40 | Sulfuric Acid Mist (SAM) | 7 |
| SO2 | 40 | Lead (Pb) | 0.6 |
| Hg | 0.1 | GHG | 75,000 (CO2e) 3 |
| - Excluding fluoride and those pollutants defined for Pulp and Paper, MWC, MSW landfills. - Ozone (O3) is regulated by its precursors (VOC and NOX). PSD for PM2.5 can be triggered by its precursors (NOX and SO2). - In making the CO2e calculation, the values listed in 40 CFR 98, Subpart A, Table A-1 are used to weight emissions by their respective Global Warming Potential (GWP). For example, the current GWP factors for four of the GHGs are: CO2 = 1; CH4 = 25; N2O = 298 and SF6 = 22,800. |  |  |  |

  - PSD Applicability for Project

The project is in Lee County, which is currently in attainment with all ambient air quality standards or otherwise designated as unclassifiable.  This project involves re-permitting the Lee County RRF with the result being a “clean” new construction permit incorporating all relevant conditions with regard to federal and state rules and requirements.  In addition, all applicable previous permitting actions are incorporated into this new “clean” air construction permit, and obsolete and redundant conditions have been removed.  No modification to existing emission units nor any increase pollutant emissions is authorized by this project.  Consequently, PSD does not apply to this project.

- Department review

The permittee and the Department have identified all previous air construction permits to be incorporated into this new consolidation permit.  None of these permits have been issued since the most recent renewal of the facility’s Title V air operation permit, so this new permit will be very similar to the facility’s current Title V permit, which has already incorporated all relevant conditions from all previous air construction permits, including PSD permits.

These permit changes are included in a Title V permit revision, which is being processing concurrently.

- Preliminary Determination

The Department makes a preliminary determination that the proposed project will comply with all applicable state and federal air pollution regulations as conditioned by the draft permit.  This determination is based on a technical review of the complete application, reasonable assurances provided by the applicant, and the conditions specified in the draft permit.  No air quality modeling analysis is required because the project does not result in a significant increase in emissions.  John Dawson is the project engineer responsible for reviewing the application and drafting the permit.  Additional details of this analysis may be obtained by contacting the project engineer at the Department’s Office of Permitting and Compliance at Mail Station #5505, 2600 Blair Stone Road, Tallahassee, Florida  32399-2400 at 850-717-9085 or by email John.Dawson@dep.state.fl.us.