|  | Florida Department of Environmental Protection Northwest District 160 W. Government Street, Suite 308 Pensacola, Florida 32502-5740 | Rick Scott Governor Carlos Lopez-Cantera Lt. Governor Ryan E. Matthews Interim Secretary |
| --- | --- | --- |

Permittee

| North Florida Lumber, Inc. dba Rex Lumber, Bristol, Florida Highway 12 South Bristol, Florida 32321 Authorized Representative: Kenny Sparks, General Manager | Air Permit No. 0770007-022-AC Permit Expires: March 15, 2020 Title V Air Construction Permit Bristol Lumber Mill Replacement of Multiclone Control Devices on Boiler #2 |
| --- | --- |

Project

This is the final air construction permit, which authorizes the replacement of two in-series dust collectors (multi-clones) for control of particulate matter at Boiler #2.  There will be no increase in material throughput at the plant or regulated emissions from Boiler #2 related to the proposed project.  The proposed work will be conducted at the existing Bristol Lumber Mill, which is a Softwood Lumber Mill categorized under Standard Industrial Classification No. 2421, Sawmills, Planning Mills, General.  The existing facility is located in Liberty County on Highway 12 South in Bristol, Florida.  The UTM coordinates are Zone 16, 691.48 km East and 3358.62 km North.

This final permit is organized into the following sections:  Section 1 (General Information); Section 2 (Administrative Requirements); Section 3 (Emissions Unit Specific Conditions); and Section 4 (Appendices).  Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of Section 4 of this permit.  As noted in the Final Determination provided with this final permit, only minor changes and clarifications were made to the draft permit.

Statement of Basis

This air pollution construction permit is issued under the provisions of:  Chapter 403 of the Florida Statutes (F.S.) and Chapters 62-4, 62-204, 62-210, 62-212, 62-296 and 62-297 of the Florida Administrative Code (F.A.C.).  The permittee is authorized to conduct the proposed work in accordance with the conditions of this permit.  This project is subject to the general preconstruction review requirements in Rule 62-212.300, F.A.C. and is not subject to the preconstruction review requirements for major stationary sources in Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

Upon issuance of this final permit, any party to this order has the right to seek judicial review of it under Section 120.68 of the Florida Statutes by filing a notice of appeal under Rule 9.110 of the Florida Rules of Appellate Procedure with the clerk of the Department of Environmental Protection in the Office of General Counsel (Mail Station #35, 3900 Commonwealth Boulevard, Tallahassee, Florida, 32399-3000) and by filing a copy of the notice of appeal accompanied by the applicable filing fees with the appropriate District Court of Appeal.  The notice must be filed within 30 days after this order is filed with the clerk of the Department.

Executed in Pensacola, Florida

March 16, 2017

Andrew Joslyn			Date

Permitting Program Administrator

CERTIFICATE OF SERVICE

The undersigned duly designated agency clerk hereby certifies that this Air Permit package was sent by electronic mail, or a link to these documents made available electronically on a publicly accessible server, with received receipt requested before the close of business on the date indicated below to the following persons.

# Mr. Kenny Sparks, Responsible Official, Rex Lumber, Bristol, Florida, ksparks@rex-lumber.com

# Mr. Vernon Adams, Application Contact, PLE Consulting, vernonadams@hotmail.com

# Mr. Richard Brinker, P.E., PLE Consulting, richardbrinker@bellsouth.net

# Clerk Stamp

March 16, 2017

Signature		Date

# FILING AND ACKNOWLEDGMENT FILED, on this date, pursuant to Section 120.52(7), Florida Statutes, with the designated agency clerk, receipt of which is hereby acknowledged.

Facility Description

The facility consists of a log de-barker, sawmill, wood waste handling and storage system, four lumber drying kilns, planer mill, and three boilers that provide heat to the kilns.  Pine logs are delivered to the facility via logging trucks and stored in the log storage yard.  The logs are debarked and conveyed into the sawmill to be cut into various sizes of dimensional lumber.  The lumber is automatically sorted by size and collected in corresponding piles that are transferred to the kilns for drying.  After the lumber is dried, it is transferred to the planer mill where it is sized to specifications.  The finished lumber is stored in miscellaneous storage sheds until it is shipped off site.  The facility also receives debarked and peeled poles from the Apalachee Pole Company (adjacent property owner) to be dried in the facility’s drying kilns.  Once dried, the poles are transported back to the Apalachee Pole Company for further processing.  The facility has a maximum production rate of 195 million board-feet per year of lumber including 18 million equivalent board-feet (1,500,000 cubic feet) per year of poles.

Continuous drying Kilns 1 and 2 are used to dry lumber produced at Rex.  These kilns are hybrid direct-fired and steam heated continuous kilns that consist of three chambers: pre-conditioning, drying, and post-conditioning.  The direct-fired heat utilized in the kilns is provided by waste heat from the boilers.  Steam from Boilers 1, 2, and 3 provide heat through steam coils in the kilns.  Green lumber is loaded by forklift onto steel trams sitting on rails, and the trams are pushed into the kilns by pushers on a slow continuous basis.  In the initial (in-bound) pre-conditioning chamber, the green lumber is pre-heated by steam generated in the drying chamber.  The central drying chamber is heated by a combination of steam and exhaust gases from the boilers.  In the center chamber the water in the green lumber is driven off.  This process generates VOC, and HAP (primarily methanol and formaldehyde) from the sap in the green lumber.  In the post-conditioning chamber, the dried wood is gradually cooled to minimize splitting and warping.  Due to the open-ended design of these continuous kilns, control equipment cannot be incorporated into or retrofitted so there are no external emission controls on Kilns 1 and 2.

The existing facility consists of the following emissions units.

| EU No. | Emission Unit Description |
| --- | --- |
| Regulated Emissions Units |  |
| 001 | Boiler No 1 (29.6 MMBtu/hr) and Boiler No.3 (28.7 MMBtu/hr) |
| 002 | Boiler No. 2 (42.9 MMBtu/hr) |
| 015 | Four Lumber Drying Kilns |
| 016 | One New Diesel Powered Emergency CI RICE |
| Unregulated Emissions Units |  |
| 014 | Wood Waste Handling and Storage |

PROPOSED PROJECT

The proposed project is for the replacement of two existing in-series dust collectors (multi-clones) on Boiler #2 which were originally installed in 1989.  The multi-clones have experienced normal wear and tear and are due for replacement with similar units.  There will be no increases in material throughput or process emissions related to the project.

Replacement of the control devices will take place at the following emissions unit.

| ID No. | Emission Unit Description |
| --- | --- |
| 002 | Boiler No. 2 (42.9 MMBtu/hr) |

Facility Regulatory Classification

- The facility is a major source of hazardous air pollutants (HAP).

- The facility does not operate units subject to the acid rain provisions of the Clean Air Act (CAA).

- The facility is a Title V major source of air pollution in accordance with Chapter 62-213, F.A.C.

- The facility is a major stationary source in accordance with Rule 62-212.400(PSD), F.A.C.

Facility Emissions

Total potential estimated emissions of applicable airborne contaminants following completion of the project will be the same as estimated under 0770007-021-AC:

| Pollutant | Facility Post-Project Potential Emissions TPY* |
| --- | --- |
| CO | 266 |
| NOX | 97 |
| PM | 136 |
| PM10 | 120 |
| PM2.5 | 85 |
| SO2 | 12 |
| VOC | 471.15 |
| Total HAP | 40.39 |
| Methanol | 22.77 |
| Formaldehyde | 1.54 |
| CO2 | 86,435 |

- Permitting Authority:  The permitting authority for this project is the Northwest District Office of the Department of Environmental Protection (Department).  The District Office mailing address is 160 W. Government Street, Suite 308 Pensacola, Florida 32502-5740.

- Compliance Authority:  All documents related to compliance activities such as reports, tests, and notifications shall be submitted to the Northwest District Office at:  160 W. Government Street, Suite 308, Pensacola, Florida 32502-5740.

- Appendices:  The following Appendices are attached as a part of this permit:  Appendix A (Citation Formats and Glossary of Common Terms); Appendix B (General Conditions); Appendix C (Common Conditions); Appendix D (Common Testing Requirements).

- Applicable Regulations, Forms and Application Procedures:  Unless otherwise specified in this permit, the construction and operation of the subject emissions units shall be in accordance with the capacities and specifications stated in the application.  The facility is subject to all applicable provisions of: Chapter 403, F.S.; and Chapters 62-4, 62-204, 62-210, 62-212, 62-213, 62-296 and 62-297, F.A.C.  Issuance of this permit does not relieve the permittee from compliance with any applicable federal, state, or local permitting or regulations.

- New or Additional Conditions:  For good cause shown and after notice and an administrative hearing, if requested, the Department may require the permittee to conform to new or additional conditions.  The Department shall allow the permittee a reasonable time to conform to the new or additional conditions, and on application of the permittee, the Department may grant additional time.  [Rule 62-4.080, F.A.C.]

- Modifications:  The permittee shall notify the Compliance Authority upon commencement of construction.  No new emissions unit shall be constructed and no existing emissions unit shall be modified without obtaining an air construction permit from the Department.  Such permit shall be obtained prior to beginning construction or modification.  [Rules 62-210.300(1) and 62-212.300(1)(a), F.A.C.]

- Construction and Expiration:  The expiration date shown on the first page of this permit provides time to complete the physical construction activities authorized by this permit, complete any necessary compliance testing, and obtain an operation permit.  Notwithstanding this expiration date, all specific emissions limitations and operating requirements established by this permit shall remain in effect until the facility or emissions unit is permanently shut down.  For good cause, the permittee may request that a permit be extended.  Pursuant to Rule 62-4.080(3), F.A.C., such a request shall be submitted to the Permitting Authority in writing before the permit expires.  [Rules 62-4.070(3) & (4), 62-4.080 & 62-210.300(1), F.A.C.]

- Source Obligation:

- Authorization to construct shall expire if construction is not commenced within 18 months after receipt of the permit, if construction is discontinued for a period of 18 months or more, or if construction is not completed within a reasonable time.  This provision does not apply to the period of time between construction of the approved phases of a phased construction project except that each phase must commence construction within 18 months of the commencement date established by the Department in the permit.

- At such time that a particular source or modification becomes a major stationary source or major modification (as these terms were defined at the time the source obtained the enforceable limitation) solely by virtue of a relaxation in any enforceable limitation which was established after August 7, 1980, on the capacity of the source or modification otherwise to emit a pollutant, such as a restriction on hours of operation, then the requirements of subsections 62-212.400(4) through (12), F.A.C., shall apply to the source or modification as though construction had not yet commenced on the source or modification.

- At such time that a particular source or modification becomes a major stationary source or major modification (as these terms were defined at the time the source obtained the enforceable limitation) solely by exceeding its projected actual emissions, then the requirements of subsections 62-212.400(4) through (12), F.A.C., shall apply to the source or modification as though construction had not yet commenced on the source or modification.

[Rule 62-212.400(12), F.A.C.]

- Application for Title V Permit:  This permit authorizes construction of the permitted emissions units and initial operation to determine compliance with Department rules.  A Title V air operation permit is required for regular operation of the permitted emissions unit.  The permittee shall apply for a Title V air operation permit at least 90 days prior to expiration of this permit, but no later than 180 days after commencing operation.  To apply for a Title V operation permit, the applicant shall submit the appropriate application form, compliance test results, and such additional information as the Department may by law require.  The application shall be submitted to the appropriate Permitting Authority with copies to the Compliance Authority.  [Rules 62-4.030, 62-4.050 and Chapter 62-213, F.A.C.]

This section of the permit addresses the following existing emissions unit.

| EU No. | Emission Unit Description |
| --- | --- |
| 002 | Boiler No. 2 (42.9 MMBtu/hr) |

This emissions unit (boiler) is equipped with a wood gasification system and is fueled by the wood waste generated during the processing of lumber at the facility.  Wood waste is moved to the gasifier via a variable speed screw conveyor.  Steam production at the boiler is continuously measured and recorded.

Boiler No. 2 was manufactured by Industrial Boiler and has a maximum heat input capacity of 42.9 million British thermal units per hour (MMBtu/hr).  Particulate emissions are currently controlled by two (in-series) multiple-tube dry collectors (multi-clones) manufactured by Zurn Boilers.  Both are Model MTSA 30-9-CYT-A units.  Ash collected by the multi-clones is emptied into containers located under the multi-clones.  Heat generated at the boiler is ducted to Kilns Nos. 1 and 2 to provide additional heat for drying wood.  Combustion gases from the boiler vent through the multi-clones to the atmosphere through Kilns 1 and 2 which are subject to the visible emissions standard of 20% / 27% opacity, per Rule 62-296.410(2)(a), F.A.C.  The existing multi-clones, originally installed in 1989, have experienced normal wear and tear and are due for replacement with similar units.

The boiler is subject to 40 CFR 63, Subpart DDDD, National Emission Standards for Hazardous Air Pollutants: Plywood and Composite Wood Products, because an affected source under the Plywood MACT includes, but is not limited to, green end operations, refining, and drying operations (including any combustion unit exhaust stream routinely used to direct fire process units [40 CFR 63.2232(b)].  The boiler is exempt from regulation under the Boiler MACT (40 CFR 63 Subpart DDDDD), pursuant to 40 CFR 63.7491(h).

This project does not impact any emissions standards or requirements imposed by the existing operation permit 0770007-020-AV and the current construction permit 0770007-021-AC.

The following specific conditions apply to new Multi-clones #1 and #2:

- Previous Permits:  This permit supplements all previously issued air construction and operation permits for this emissions unit. Except for the change noted below, the unit remains subject to the conditions of all other legally binding air construction and operating permits.  [Rules 62-4.070(3) and 62-210.300(1), F.A.C.]

- New Equipment:  Except for the change noted below, the new equipment authorized by this permit shall be subject to the same requirements, such as, but not limited to; emissions standards, recordkeeping, compliance testing and fuels limitations previously established for this emissions unit in all other legally binding air construction and operating permits.  [Rules 62-4.070(3) and 62-210.300(1), F.A.C.]

Equipment

- Replacement of Multi-clones at Boiler #2:  The permittee is authorized to install two (in-series) multiple-tube dry particulate matter (PM) collectors similar to the existing units.  Both will be manufactured by Hurst Boilers and will be models 9UP-30T or equivalent units.  [Rules 62-4.030 and 62-4.070(3), F.A.C., and Application No. 0770007-022-AC]

Emissions Standards

{Permitting Note:  This project does not change any existing emissions standards for this emissions unit.}

Testing requirements

- Compliance Requirements:  The emissions unit shall be tested to demonstrate initial compliance with the emissions standards for particulate matter (PM).  The initial tests shall be conducted within 60 days after achieving permitted capacity, but not later than 180 days after initial operation of the unit.  [Rules 62-4.070(3) and 62-297.310(8)(b)1, F.A.C.]

- Test Requirements:  The permittee shall notify the Compliance Authority in writing at least 15 days prior to any required tests.  Tess shall be conducted in accordance with the applicable requirements specified in Appendix D (Common Testing Requirements) of this permit.  [Rule 62-297.310(9), F.A.C.]

- Test Methods:  Required tests shall be performed in accordance with the following reference methods.

| Method | Description of Method and Comments |
| --- | --- |
| 1-4 | Traverse Points, Velocity and Flow Rate, Gas Analysis, and Moisture Content |
| 5 | Method for Determining Particulate Matter Emissions |

The above methods are described in Appendix A of 40 CFR 60 and are adopted by reference in Rule 62-204.800, F.A.C.  No other methods may be used unless prior written approval is received from the Department.  [Rules 62-204.800, F.A.C.; and Appendix A of 40 CFR 60]

RECORDS AND REPORTS

- Test Reports:  The permittee shall prepare and submit reports for all required tests in accordance with the requirements specified in Appendix D (Common Testing Requirements) of this permit.  For each test run, the report shall also indicate the control efficiency of each device.  [Rule 62-297.310(10), F.A.C.]

Back to Top