Permittee

| Trojan Florida Powder Coating Company, Inc. 1300 Hardin Ave. Sarasota, FL 34243 Authorized Representative: Carl Troiano, Director | Air Permit No. 0810257-001-AC Permit Expires: 12/18/2018 Minor Air Construction Permit Trojan Florida Facility Initial Construction |
| --- | --- |

Project

This is the final air construction permit, which authorizes the application of finishes and coatings to metal parts and extrusions.  The proposed work will be conducted at the existing Trojan Florida facility, which is a metal coating facility categorized under Standard Industrial Classification No. 3479.  The facility is located in Manatee County at 1300 Hardin Ave. in Sarasota, Florida.  The UTM coordinates are Zone 17, 346.82 km East and 3032.26 km North.

This final permit is organized into the following sections:  Section 1 (General Information); Section 2 (Administrative Requirements); Section 3 (Emissions Unit Specific Conditions); and Section 4 (Appendices).  Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of Section 4 of this permit.

Statement of Basis

This air pollution construction permit is issued under the provisions of:  Chapter 403 of the Florida Statutes (F.S.) and Chapters 62-4, 62-204, 62-210, 62-212, 62-296 and 62-297 of the Florida Administrative Code (F.A.C.).  The permittee is authorized to conduct the proposed work in accordance with the conditions of this permit.  This project is subject to the general preconstruction review requirements in Rule 62-212.300, F.A.C. and is not subject to the preconstruction review requirements for major stationary sources in Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

Upon issuance of this final permit, any party to this order has the right to seek judicial review of it under Section 120.68 of the Florida Statutes by filing a notice of appeal under Rule 9.110 of the Florida Rules of Appellate Procedure with the clerk of the Department of Environmental Protection in the Office of General Counsel (Mail Station #35, 3900 Commonwealth Boulevard, Tallahassee, Florida, 32399-3000) and by filing a copy of the notice of appeal accompanied by the applicable filing fees with the appropriate District Court of Appeal.  The notice must be filed within 30 days after this order is filed with the clerk of the Department.

Executed in Hillsborough County, Florida

_________________________________

Pamala Vazquez

Program Administrator

Permitting & Waste Cleanup Program

Southwest District

CERTIFICATE OF SERVICE

The undersigned duly designated deputy agency clerk hereby certifies that this Air Permit package was sent by electronic mail, or a link to these documents made available electronically on a publicly accessible server, with received receipt requested before the close of business on the date indicated below to the following persons.

# Carl Troiano, Trojan Florida, carl@trojanpowder.com

# Dr. S. E. Beladi, PE, General Mechanics Consulting Engineers, gmce001@gmail.com

# Max Grondahl, Florida DEP Southwest District, Max.Grondahl@floridadep.gov

# Clerk Stamp

# FILING AND ACKNOWLEDGMENT FILED, on this date, pursuant to Section 120.52(7), Florida Statutes, with the designated agency clerk, receipt of which is hereby acknowledged.

(DRAFT)

______________________________________________

(Clerk)                            (Date)

PROPOSED PROJECT

The facility proposes to apply surface coatings to metal parts and extrusions in three (3) indoor spray booths (Spray Booth 1, Spray Booth 2, Spray Booth 3). Each booth is equipped with pleated filters to capture particulates associated with the spray-application of paint products. These operations are a potential source of Volatile Organic Compounds (VOCs) and Hazardous Air Pollutants (HAPs). Spray Booth 1 and Spray Booth 2 are each equipped with two (2) exhaust ducts which vent to the atmosphere. Drying Booth 1, adjoined to Spray Booth 1, has one (1) exhaust duct. The facility also applies powder coatings to metal parts. Prior to spray painting or powder coating, metal parts are cleaned and prepared in a series of dip tanks. Ovens and drying heaters are used to finish parts.

This project will add the following emissions units.

| Facility ID No. 0810257 |  |
| --- | --- |
| ID No. | Emission Unit Description |  |
| 001 | Surface Coating Operations |  |

EXEMPT EMISSION SOURCES/ACTIVITIES

The following natural gas-fired equipment is exempt from permitting per Rule 62-210.300(3)(a)33, F.A.C. (Categorical and Conditional Exemptions - external combustion heating units with heat input capacity equal to or less than 10 million Btu per hour):

- 	Heated dip tank with immersion burner (1.75 MMBtu/hr)

-	Dry oven (1.67 MMBtu/hr)

-	Cure oven (1.67 MMBtu/hr)

-	Two (2) Spray Booth Drying Heaters for Spray Booth 1/Drying Booth 1 (2.05 MMBtu/hr each)

Facility Regulatory Classification

- The facility is not a major source of hazardous air pollutants (HAP).

- The facility does not operate units subject to the acid rain provisions of the Clean Air Act (CAA).

- The facility is not a Title V major source of air pollution in accordance with Chapter 62-213, F.A.C.

- The facility is not a major stationary source in accordance with Rule 62-212.400(PSD), F.A.C.

- This facility is a natural minor source of air pollution.

# Permit History/Affected Permits

No previous air permits have been issued for this facility.

administrative REQUIREMENTS

- Permitting Authority:  The permitting authority for this project is the Southwest District of the Department of Environmental Protection (Department).  The mailing address, phone number and e-mail address is:

Florida Department of Environmental Protection

Southwest District Office

Air and Solid Waste Permitting Program

13051 North Telecom Parkway, Suite 101

Temple Terrace, Florida 33637-0926

Telephone: 813-470-5700

E-mail: SWD_Air_Permitting@dep.state.fl.us

All documents related to applications for permits shall be submitted to the above e-mail address and/or address.

- Compliance Authority:  All documents related to compliance activities such as reports, tests, and notifications shall be submitted to the Florida Department of Environmental Protection (Department), Southwest District Office’s Compliance Assurance Program.  The mailing address, phone number and e-mail address is:

Florida Department of Environmental Protection

Southwest District Office

Compliance Assurance Program

13051 North Telecom Parkway, Suite 101

Temple Terrace, Florida 33637-0926

Telephone: 813-470-5700

E-mail: SWD_Air@dep.state.fl.us

- Appendices:  The following Appendices are attached as a part of this permit:

- Appendix A (Citation Formats and Glossary of Common Terms);

- Appendix B (General Conditions);

- Appendix C (Common Conditions); and

- Appendix D (Common Testing Requirements).

- Applicable Regulations, Forms and Application Procedures:  Unless otherwise specified in this permit, the construction and operation of the subject emissions units shall be in accordance with the capacities and specifications stated in the application.  The facility is subject to all applicable provisions of: Chapter 403, F.S.; and Chapters 62-4, 62-204, 62-210, 62-212, 62-213, 62-296 and 62-297, F.A.C.  Issuance of this permit does not relieve the permittee from compliance with any applicable federal, state, or local permitting or regulations.

- New or Additional Conditions:  For good cause shown and after notice and an administrative hearing, if requested, the Department may require the permittee to conform to new or additional conditions.  The Department shall allow the permittee a reasonable time to conform to the new or additional conditions, and on application of the permittee, the Department may grant additional time.

[Rule 62-4.080, F.A.C.]

- Modifications:  The permittee shall notify the Compliance Authority upon commencement of construction.  No new emissions unit shall be constructed and no existing emissions unit shall be modified without obtaining an air construction permit from the Department.  Such permit shall be obtained prior to beginning construction or modification.

[Rules 62-210.300(1) and 62-212.300(1)(a), F.A.C.]

- Construction and Expiration.  The expiration date shown on the first page of this permit provides time to complete the physical construction activities authorized by this permit, complete any necessary compliance testing, and obtain an operation permit.  Notwithstanding this expiration date, all specific emissions limitations and operating requirements established by this permit shall remain in effect until the facility or emissions unit is permanently shut down.  For good cause, the permittee may request that that a permit be extended.  Pursuant to Rule 62-4.080(3), F.A.C., such a request shall be submitted to the Permitting Authority in writing before the permit expires.

[Rules 62-4.070(4), 62-4.080 & 62-210.300(1), F.A.C.]

- Application for Non-Title V Air Operation Permit: This permit authorizes construction of the permitted emissions unit(s) and initial operation to determine compliance with Department rules.  A Non-Title V air operation permit is required for continued operation of the permitted emissions unit(s).  The permittee shall apply for a Non-Title V air operation permit at least 90 days prior to expiration of this permit, but no later than 180 days after commencing operation.  Commencing operation means setting into operation of any emissions unit for any purpose.  To apply for a Non-Title V air operation permit, the applicant shall submit the following:

- the appropriate permit application form (see current version of Rule 62-210.900, F.A.C. (Forms and Instructions), and/or FDEP Division of Air Resource Management website at: http://www.dep.state.fl.us/air/);

- the appropriate operation permit application fee from Rule 62-4.050(4)(a), F.A.C.; and

- copies of the most recent month of records/logs specified in Specific Condition No. A.5.

The application shall be submitted to the Permitting Authority with a copy to the Compliance Authority.

[Rules 62-4.030 and 62-4.050, F.A.C.]

This section of the permit addresses the following emissions unit.

| EU No. | Emission Unit Description |
| --- | --- |
| 001 | Surface Coating Operations: The surface coating operations take place in three (3) indoor spray booths (Spray Booth 1, Spray Booth 2, Spray Booth 3). Each booth is equipped with pleated filters to capture particulates associated with the spray-application of paint products. These operations are a potential source of Volatile Organic Compounds (VOCs) and Hazardous Air Pollutants (HAPs). Spray Booth 1 and Spray Booth 2 are each equipped with two (2) exhaust ducts which vent to the atmosphere. Drying Booth 1, adjoined to Spray Booth 1, has one (1) exhaust duct. |

Performance Restrictions

- Restricted Operation:  The hours of operation are not limited (8760 hours per year).

[Rules 62-4.070(3) and 62-210.200(PTE), F.A.C.]

##### Emissions Standards

- Emissions Standards:

  - Volatile organic compounds (VOC) emissions shall not exceed 24.8 tons in any consecutive 12-month period.

  - Total hazardous air pollutants (HAPs) emissions shall not exceed 9.82 tons in any consecutive 12-month period.

[Application No. 0810257-001-AC and Rule 62-210.200(PTE), F.A.C.]

- General Standards: Volatile Organic Compound Emissions and/or Organic Solvent Emissions: The permittee shall allow no person to store, pump, handle, process, load, unload or use in any process or installation, volatile organic compounds (VOC) or organic solvents (OS) without applying known and existing vapor emission control devices or systems deemed necessary and ordered by the Department. The following procedures shall be utilized to minimize pollutant emissions:

  - all equipment, pipes, hoses, lids, fittings, etc., shall be operated/maintained in such a manner as to minimize leaks, fugitive emissions and spills of solvent materials;

  - tightly cover or close all VOC and/or solvent containing vessels, drums, totes, etc., when they are not in use;

  - immediately confine and clean up VOC and/or solvent spills, and make sure wastes are placed in closed containers for reuse, recycling or proper disposal;

  - used cleanup solvents shall be captured and stored in closed containers and recycled or disposed of as required by the appropriate waste disposal regulations; and

  - all spray painting shall be confined to the spray booths.

[Rules 62-4.070(3) and 62-296.320(1), F.A.C.]

- General Standards: Unconfined Particulate Matter: All reasonable precautions shall be taken to prevent and control the generation of unconfined emissions of particulate matter in accordance with the provisions in Rule 62-296.320(4)(c), F.A.C. These provisions are applicable to any source, including but not limited to industrial related activities such as loading, unloading, storing and handling of materials. At a minimum, the following precautions shall be taken by the permittee:

  - all spray painting shall be confined to the spray booths;

  - all powder coating shall be confined to the powder coating booths; and

  - hoods, fans, filters and similar equipment shall be used to contain, capture and/or vent particulate matter.

[Rules 62-4.070(3) and 62-296.320(4)(c), F.A.C.]

Records and Reports

- Monthly Log:  The permittee shall keep monthly logs to document compliance with the VOC and Total HAP limitations of Specific Condition No. A.2. The logs may be based on the beginning and ending inventories, deliveries, shipments, etc. At the option of the permittee, “purchases” may be used instead of “usage” in determining amount of coating used and the VOC/HAP emissions, provided that no material is used that was not purchased. At a minimum, the logs shall contain the following:

- VOC Monthly Records

  - the quantity (gallons) and VOC content (pounds/gallon) of each VOC-containing material (surface coatings, solvents, etc.) used for the month;

  - the calculated monthly total of VOC emissions (tons/month); and

  - a cumulative total of VOC emissions for the most recent consecutive 12-month period (tons/consecutive 12-month period).

- Total HAPs Monthly Records

  - the quantity (gallons) and HAPs content (pounds/gallon) of each HAP-containing material (surface coatings, solvents, etc.) used for the month;

  - the calculated monthly total of HAP emissions (tons/month); and

  - a cumulative total of the HAP emissions for the most recent consecutive 12-month period (tons/consecutive 12-month period).

Monthly records shall be completed by the end of the following month. These records shall be maintained at the facility for at least 3 years and made available to the Department upon request. Supporting documentation (SDS sheets, purchase orders, emissions factors, etc.) shall be kept for each VOC and HAP containing material.

[Rule 62-4.070(3) and 62-4.160(14)(b), F.A.C.]