TECHNICAL EVALUATION

&

PRELIMINARY DETERMINATION

APPLICANT

Trojan Florida Powder Coating Company, Inc.

1300 Hardin Ave.

Sarasota, FL 34243

Trojan Florida Facility

Facility ID No. 0810257

PROJECT

Project No. 0810257-001-AC

Application for Minor Source Air Construction Permit

Initial Construction

COUNTY

Manatee County, Florida

PERMITTING AUTHORITY

Florida Department of Environmental Protection

Air and Solid Waste Permitting

Southwest District Office

13051 North Telecom Parkway, Suite 101

Temple Terrace, Florida 33637-0926

November 30, 2017

Prepared by Rachel Tift

- General Project INFORMATION

Air Pollution Regulations

Projects at stationary sources with the potential to emit air pollution are subject to the applicable environmental laws specified in Section 403 of the Florida Statutes (F.S.).  The statutes authorize the Department of Environmental Protection (Department) to establish regulations regarding air quality as part of the Florida Administrative Code (F.A.C.), which includes the following applicable chapters:  62-4 (Permits); 62-204 (Air Pollution Control – General Provisions); 62-210 (Stationary Sources – General Requirements); 62-212 (Stationary Sources – Preconstruction Review); 62-213 (Operation Permits for Major Sources of Air Pollution); 62-296 (Stationary Sources - Emission Standards); and 62-297 (Stationary Sources – Emissions Monitoring).  Specifically, air construction permits are required pursuant to Chapters 62-4, 62-210 and 62-212, F.A.C.

In addition, the U. S. Environmental Protection Agency (EPA) establishes air quality regulations in Title 40 of the Code of Federal Regulations (CFR).  Part 60 specifies New Source Performance Standards (NSPS) for numerous industrial categories.  Part 61 specifies National Emission Standards for Hazardous Air Pollutants (NESHAP) based on specific pollutants.  Part 63 specifies NESHAP based on the Maximum Achievable Control Technology (MACT) for numerous industrial categories.  The Department adopts these federal regulations in Rule 62-204.800, F.A.C.

Glossary of Common Terms

Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of this permit.

Facility Description and Location

Trojan Florida is an existing facility that is proposing to apply surface coatings to metal parts and extrusions, which is categorized under Standard Industrial Classification Code No. 3479.  The Trojan Florida facility is located in Manatee County at 1300 Hardin Ave in Sarasota, Florida.  The UTM coordinates of the existing facility are Zone 17, 346.82 km East, and 3032.26 km North.  This site is in an area that is in attainment (or designated as unclassifiable) for all air pollutants subject to Ambient Air Quality Standards (AAQS).

Facility Regulatory Categories

- The facility is not a major source of hazardous air pollutants (HAP).

- The facility is not a Title V major source of air pollution in accordance with Chapter 62-213, F.A.C.

- The facility is not a major stationary source in accordance with Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

- This facility is a natural minor source of air pollution.

- Project Description

- This is the initial air construction permit for an existing facility that applies finishes and coatings to metal parts and extrusions.

- Processing Schedule

8/20/2017	Received the application for a minor source air pollution construction permit.

9/14/2017	Requested additional information.

11/14/2017 Received additional information.

11/21/2017 Received additional information; application complete.

- PSD Applicability

The project is located in Manatee County which is in an area that is currently in attainment with the AAQS or is otherwise designated as unclassifiable.  The facility is not a PSD major source and therefore the proposed project is not subject to a PSD preconstruction review.

- Department review

- The facility operations include surface cleaning, surface coating, powder coating, drying, curing, and packaging of metal parts and extrusions. The proposed surface coating operations are a potential source of VOC and HAP emissions and are limited to three indoor spray booths (Spray Booth 1, Spray Booth 2, Spray Booth 3). All spray booths are equipped with pleated filters to capture particulates associated with the spray-application of paint products. The following natural gas-fired equipment is exempt from permitting in accordance with the provisions of Rule 62-210.300(3)(a)33, F.A.C.: heated dip tank with immersion burner (1.75 MMBtu/hr), drying oven (1.67 MMBtu/hr), curing oven (1.67 MMBtu/hr), and two (2) spray booth drying heaters for Spray Booth 1/Drying Booth 1(2.05 MMBtu/hr each).

- List of Emissions Units

| EU ID No. | Emissions Unit Description |
| --- | --- |
| 001 | Surface Coating Operations |

- Summary of Emissions

| Pollutant | EU No. | Potential Emissions* (tpy) | Allowable Emissions (tpy) |
| --- | --- | --- | --- |
| VOC | 001 | 24.8 | 24.8 |
|  | Total | 24.8 | 24.8 |
| Total HAP | 001 | 9.82 | 9.82 |
|  | Total | 9.82 | 9.82 |

* The potential emissions for VOC and Total HAP are based on the requested maximum allowable

emissions.

| Visible Emissions |  |  |
| --- | --- | --- |
| EU No. | Emission Limiting Standard (% Opacity) | Applicable Rule |
| 001 | 20 | Rule 62-296.320(4)(b), F.A.C |

- State Requirements and Federal Regulations

This project is subject to the preconstruction review requirements of Chapter 403, Florida Statutes and Chapters 62-204 through 62-297, Florida Administrative Code (F.A.C.), as indicated below.

| Subject to: | Y/N | Comments |
| --- | --- | --- |
| Rule 62-210.300, F.A.C. – Permits Required | Y | subject to general permitting requirements |
| Rule 62-212.400, F.A.C. - Prevention of Significant Deterioration | N | facility is not a PSD major source |
| Rule 62-296.320(4), F.A.C. - General Particulate Emission Limiting Standards | Y | painting operations could be a source of unconfined particulate matter emissions |
| Rules 62-296.320(1) and (2), F.A.C. - General Pollutant Emission Limiting Standards (VOCs and Odor) | Y | painting operations are a source of VOC and odors |
| Rule 62-296.401-418, F.A.C. - Stationary Source Emission Standards | N | there is no applicable source category |
| Rule 62-296.500, F.A.C. - Reasonably Available Control Technology (VOC) | N | Manatee County is not an air quality maintenance area for ozone |
| Rule 62-296.700, F.A.C. - Reasonably Available Control Technology (PM) | N | there is no applicable source category |
| Rule 62-204.800, F.A.C. - Standards of Performance for New Stationary Sources (NSPS) | N | there is no applicable source category |
| Rule 62-204.800, F.A.C. National Emission Standard for Hazardous Air Pollutants (NESHAPS – 40 CFR 61) | N | there is no applicable source category |
| Rule 62-204.800, F.A.C. National Emission Standard for Hazardous Air Pollutants for Source Categories a.k.a. MACT (NESHAPS –40 CFR 63) | N | there is no applicable source category |
| Chapter 62-213, F.A.C. - Operation Permits for Major Sources of Air Pollution | N | facility is a natural minor source |
| Rule 62-297.310, F.A.C. - General Compliance Test Requirements, F.A.C. | N | Testing is not required. Compliance is demonstrated via recordkeeping. |

- Preliminary Determination

The Department makes a preliminary determination that the proposed project will comply with all applicable state and federal air pollution regulations as conditioned by the draft permit.  This determination is based on a technical review of the complete application, reasonable assurances provided by the applicant, and the conditions specified in the draft permit.  No air quality modeling analysis is required because the project does not result in a significant increase in emissions.  Rachel Tift is the project engineer responsible for reviewing the application and drafting the permit.  Additional details of this analysis may be obtained by contacting the project engineer at the Department’s Air and Solid Waste Permitting Section in the Southwest District Office, 13051 North Telecom Parkway, Suite 101, Temple Terrace, Florida 33637-0926, phone 813-470-5755, or by email rachel.tift@floridadep.gov.