Permittee

| Best Engineering Surface Technologies, LLC DBA Windsor Metal Finishing 1820 Avenue A Kissimmee, FL 34758 Authorized Representative: William Faucett, President | Air Permit No. 0970042-013-AC Permit Effective: TBD Permit Expires: April 30, 2019 Air Construction Permit for Title V Facility Windsor Metal Finishing Facility Expansion |
| --- | --- |

Project

This is the final air construction permit, which authorizes construction of the Engineered Coatings Batch Application Equipment (Emission Unit 002) and HPS (High Performance System) Overhead Line Application System (Emission Unit 003).  Additionally, this project increases the volatile organic compound (VOC) emissions limit for the facility from 90 tons per consecutive 12-month period to 230 tons per consecutive 12-month period.  The total hazardous air pollutant (HAP) emissions limit of 90 tons per consecutive 12-month period was removed.  However, HAP emissions from the facility are limited by emission limitations in 40 CFR 63, Subpart MMMM, Surface Coating of Miscellaneous Metal Parts and Products.  Filter systems control particulate emissions.  The proposed work will be conducted in Osceola County at 5004 Mercantile Lane, Kissimmee, Florida, which is approximately one (1) mile from the existing main facility.   The existing main facility, Windsor Metal Finishing, which is a surface coating operation categorized under Standard Industrial Classification No. 3479, is in Osceola County at 1820 Avenue A in Kissimmee, Florida.  The UTM coordinates are Zone 17, 451.57 km East and 3125.21 km North.

This final permit is organized into the following sections:  Section 1 (General Information); Section 2 (Administrative Requirements); Section 3 (Emissions Unit Specific Conditions); and Section 4 (Appendices).  Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of Section 4 of this permit.

Statement of Basis

This air pollution construction permit is issued under the provisions of:  Chapter 403 of the Florida Statutes (F.S.) and Chapters 62-4, 62-204, 62-210, 62-212, 62-296 and 62-297 of the Florida Administrative Code (F.A.C.).  The permittee is authorized to conduct the proposed work in accordance with the conditions of this permit.  This project is subject to the general preconstruction review requirements in Rule 62-212.300, F.A.C. and is not subject to the preconstruction review requirements for major stationary sources in Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

Upon issuance of this final permit, any party to this order has the right to seek judicial review of it under Section 120.68 of the Florida Statutes by filing a notice of appeal under Rule 9.110 of the Florida Rules of Appellate Procedure with the clerk of the Department of Environmental Protection in the Office of General Counsel (Mail Station #35, 3900 Commonwealth Boulevard, Tallahassee, Florida, 32399-3000) and by filing a copy of the notice of appeal accompanied by the applicable filing fees with the appropriate District Court of Appeal.  The notice must be filed within 30 days after this order is filed with the clerk of the Department.

EXECUTION AND CLERKING

Executed in Orlando, Florida.

STATE OF FLORIDA DEPARTMENT OF ENVIRONMENTAL PROTECTION

DRAFT

________________________

Kimberly Rush, P.E.

Program Administrator

Permitting and Waste Cleanup Program

CERTIFICATE OF SERVICE

The undersigned duly designated deputy clerk hereby certifies that this permit and all copies were sent on the filing date below to the following listed persons:

# William Faucett, Best Engineering Surface Technologies, bfaucett@windsorinc.net

# Heather Stokes, Best Engineering Surface Technologies, hstokes@windsorinc.net

# Stephanie Deery, RPS, Stephanie.Deery@rpsgroup.com

# Lisa Bongiovanni, P.E., RPS, Lisa.Bongiovanni@RPSGroup.com

FDEP: Kim Rush, P.E., Nathan Hess, Chris Ferraro, P.E., Jeff Rustin, P.E.

FILING AND ACKNOWLEDGMENT

FILED, on this date, pursuant to Section 120.52, F. S., with the designated Department Clerk, receipt of which is hereby acknowledged.

DRAFT

________________	____________

Clerk                    	       Date

Facility Description

The existing facility consists of the following emissions unit.

| Facility ID No. 0970042 |  |
| --- | --- |
| ID No. | Emission Unit Description |  |
| 001 | Surface Coating Operation |  |

The surface coating operation consists of a power washer, drying oven, cure oven, and six paint spray booths where primer, top and/or clear coat materials are electrostatically applied to miscellaneous metal/aluminum parts.

PROPOSED PROJECT

This project authorizes construction of the Engineered Coatings Batch Application Equipment (Emission Unit 002) and HPS (High Performance System) Overhead Line Application System (Emission Unit 003).  Additionally, this project increases the VOC (volatile organic compound) emissions limit from 90 tons per consecutive 12-month period to 230 tons per year (TPY) on a 12-month rolled monthly basis.  This VOC limit is a Federally Enforceable Limit and is an applicant requested limit to avoid becoming subject to the requirements of Prevention of Significant Deterioration (PSD) permitting that are contained in Rule 62-212.400 of the Florida Administrative Code (F.A.C.).

The total hazardous air pollutant (HAP) emissions limit of 90 tons per consecutive 12-month period was removed.  However, HAP emissions from the facility are limited by emission limitations in 40 CFR 63, Subpart MMMM, Surface Coating of Miscellaneous Metal Parts and Products.  Filter systems control particulate emissions.  The proposed work will be conducted in Osceola County at 5004 Mercantile Lane, Kissimmee, Florida, which is approximately one (1) mile from the existing main facility.   The existing main facility, Windsor Metal Finishing, which is a surface coating operation categorized under Standard Industrial Classification No. 3479, is in Osceola County at 1820 Avenue A in Kissimmee, Florida.

This project will add the following emissions units.

| Facility ID No. 0970042 |  |
| --- | --- |
| ID No. | Emission Unit Description |  |
| 002 | Engineered Coatings Batch Application Equipment |  |
| 003 | HPS (High Performance System) Overhead Line Application System |  |

Exempt Units

The 2 (two) NFG Paint Cure Ovens, NGF Heater Dip Tank Number 3, NGF Dry-Off Oven, and the NGF Paint Bake Oven are exempt units pursuant to Rule 62-210.300(3)(a)33., F.A.C.  The Dip Wash Line and Solvent Reclaim unit are exempt pursuant to Rule 62-210.300(3)(b)1., F.A.C.  These units will be classified as insignificant units in the facility’s Title V air operation permit during the next Title V permit revision.  The units will be included in Appendix I, List of Insignificant Units.

Facility Regulatory Classification

- The facility is a major source of hazardous air pollutants (HAP).

- The facility does not operate units subject to the acid rain provisions of the Clean Air Act (CAA).

- The facility is a Title V major source of air pollution in accordance with Chapter 62-213, F.A.C.

- Windsor Metal Finishing is subject to a facility-wide synthetic VOC emission limit of 230 tons per year (TPY) on a 12-month rolled monthly basis.  This VOC limit is an applicant requested limit to avoid becoming subject to the requirements of Prevention of Significant Deterioration (PSD) permitting that are contained in Rule 62-212.400 of the Florida Administrative Code (F.A.C.).

# Permit History/Affected Permits

This is an Air Construction Permit for this facility and the facility is currently permitted under 0970042-010-AV, which expires on May 2, 2021.

- Permitting Authority:  The permitting authority for this project is the Florida Department of Environmental Protection, Central District Waste Cleanup and Permitting Programs.  The Central District’s mailing address and phone number is:

Florida Department of Environmental Protection

Central District Office

3319 Maguire Boulevard, Suite. 232

Orlando, Florida 32803-3767

Telephone: 407-897-4100

Except as otherwise specified in this permit, the required submittals, such as certifications, monitoring reports, notifications, etc., shall be submitted to the Department in a digital format via electronic mail, CD or DVD, or through file transfer site, when practicable. The electronic mail address is DEP_CD@dep.state.fl.us. All submittals shall clearly identify the Air Permit No. 0970042-013-AC.

- Compliance Authority:  All documents related to compliance activities such as reports, tests, and notifications shall be submitted to the Central District’s Compliance Assurance Program. (Use the above mailing or e-mail address).

- Appendices:  The following Appendices are attached as a part of this permit:  Appendix A (Citation Formats and Glossary of Common Terms); Appendix B (General Conditions); Appendix C (Common Conditions); Appendix D (NESHAP, 40 CFR 63, Subpart A, General Provisions); Appendix E (NESHAP, 40 CFR 63, Subpart MMMM, Surface Coating of Miscellaneous Metal Parts and Products); and Appendix F (Vendor’s or Manufacturer’s Information for Paint Booths and Blasting Operations).

- Applicable Regulations, Forms and Application Procedures:  Unless otherwise specified in this permit, the construction and operation of the subject emissions units shall be in accordance with the capacities and specifications stated in the application.  The facility is subject to all applicable provisions of: Chapter 403, F.S.; and Chapters 62-4, 62-204, 62-210, 62-212, 62-213, 62-296 and 62-297, F.A.C.  Issuance of this permit does not relieve the permittee from compliance with any applicable federal, state, or local permitting or regulations.

- New or Additional Conditions:  For good cause shown and after notice and an administrative hearing, if requested, the Department may require the permittee to conform to new or additional conditions.  The Department shall allow the permittee a reasonable time to conform to the new or additional conditions, and on application of the permittee, the Department may grant additional time.  [Rule 62-4.080, F.A.C.]

- Modifications:  The permittee shall notify the Compliance Authority upon commencement of construction.  No new emissions unit shall be constructed and no existing emissions unit shall be modified without obtaining an air construction permit from the Department.  Such permit shall be obtained prior to beginning construction or modification.  [Rules 62-210.300(1) and 62-212.300(1)(a), F.A.C.]

- Construction and Expiration:  The expiration date shown on the first page of this permit provides time to complete the physical construction activities authorized by this permit, complete any necessary compliance testing, and obtain an operation permit.  Notwithstanding this expiration date, all specific emissions limitations and operating requirements established by this permit shall remain in effect until the facility or emissions unit is permanently shut down.  For good cause, the permittee may request that that a permit be extended.  Pursuant to Rule 62-4.080(3), F.A.C., such a request shall be submitted to the Permitting Authority in writing before the permit expires.  [Rules 62-4.070(4), 62-4.080 & 62-210.300(1), F.A.C.]

- Annual Operating Report (AOR):  The information required by the Annual Operating Report for Air Pollutant Emitting Facility (DEP Form No. 62-210.900(5)) shall be submitted by April 1 of each year, for the previous calendar year, to the Department of Environmental Protection’s (DEP) Central District Office.  All synthetic non-Title V sources or all facilities with the potential to emit 10 tons/year or more of volatile organic compounds (VOC) or 25 tons/year or more of nitrogen oxides (NOX) and located in an ozone nonattainment area or ozone air quality maintenance area shall submit a completed DEP Form 62-210.900(5) unless the annual operating report is submitted using the DEP’s electronic annual operating report software.  Emissions shall be computed in accordance with the provisions of subsection 62-210.370(2), F.A.C.  [Rule 62-210.370(3), F.A.C.]

{Permitting Note:  Resources to help you complete your AOR are available on the electronic AOR (EAOR) website at:  http://www.dep.state.fl.us/air/emission/eaor.  If you have questions or need assistance after reviewing the information posted on the EAOR website, please contact the Department by phone at (850) 717-9000 or email at eaor@dep.state.fl.us.}

- Application for Title V Permit (revision):  This permit authorizes construction of the permitted emissions units and initial operation to determine compliance with Department rules.  A Title V air operation permit revision is required for regular operation of the permitted emissions unit.  The permittee shall apply for a Title V air operation permit at least 90 days prior to expiration of this permit, but no later than 180 days after commencing operation.  To apply for a Title V operation permit, the applicant shall submit the appropriate application form, compliance test results, and such additional information as the Department may by law require.  The application shall be submitted to the appropriate Permitting Authority with copies to the Compliance Authority.  [Rules 62-4.030, 62-4.050 and Chapter 62-213, F.A.C.]

- the appropriate permit application form (see current version of Rule 62-210.900, F.A.C. (Forms and Instructions), and/or FDEP Division of Air Resource Management website at: http://www.dep.state.fl.us/air/ );

- the appropriate operation permit application fee from Rule 62-4.050(4)(a), F.A.C.;

- copies of the most recent two months of records/logs specified in Specific Condition No. A.8.

[Rules 62-4.030, 62-4.050, 62-4.070(3), 62-4.090, 62-4.220, 62-210.300(2), and 62-210.900, F.A.C.]

- Facility-Wide Volatile Organic Compound (VOC) Emission Limit: Windsor Metal Finishing is subject to a facility-wide synthetic VOC emission limit of 230 tons per year (TPY) on a 12-month rolled monthly basis.  This VOC limit is an applicant requested limit to avoid becoming subject to the requirements of Prevention of Significant Deterioration (PSD) permitting that are contained in Rule 62-212.400 of the Florida Administrative Code (F.A.C.).  This facility-wide 230 TPY VOC limit replaces the previous VOC limit of 90 tons per consecutive twelve months, updated monthly, which was authorized by Permit No. 0970042-003-AC.  The facility-wide 12-month, rolled monthly VOC emissions in TPY shall be calculated according to the following equation:

The number of gallons of each coating, thinner and cleanup material employed shall be calculated on a monthly basis, and then used in the above equation, to calculate facility-wide emissions of VOC.  Monthly recordkeeping (log) shall be kept in accordance to Specific Condition Number 8.  [Emission limit proposed by applicant in accordance to email correspondence, “Windsor – Solvent Information & VOC Tracking Equation”, dated January 4, 2018; Rules 62-4.070(3) and 62-210.370, F.A.C.]

{Permitting Note:  Upon final issuance of this air construction permit, the first monthly facility-wide VOC emissions calculated using the above equation shall be added to the previous 11 months of VOC emissions from the facility to ensure that the 230 TPY VOC emission limit has not been exceeded.}

- Removal of 90 tons Hazardous Air Pollutant (HAP) emission limit per consecutive 12-month period:  The 90 tons HAP emission limit per consecutive 12-month period of Permit No. 0970042-003-AC is removed and replaced by the emission limitations in 40 CFR 63, Subpart MMMM, Surface Coating of Miscellaneous Metal Parts and Product contained in this permit.  [40 CFR 63, Subpart MMMM, Surface Coating of Miscellaneous Metal Parts and Product, and Rule 62-4.070(3), F.A.C.]

This section of the permit addresses the following emissions units.

| EU No. | Emission Unit Description |
| --- | --- |
| 001 | Surface Coating Operation |
| 002 | Engineered Coatings Batch Application Equipment |
| 003 | HPS (High Performance System) Overhead Line Application System |

- EU 001 Surface Coating Operation:  The Surface Coating Operation (EU 001) consists of a power washer, drying oven, cure oven, and six paint spray booths where primer, top and/or clear coat materials are electrostatically applied to miscellaneous metal/aluminum parts.

- EU 002 Engineered Coatings Batch Application Equipment:  The new Engineered Coatings Batch Application Equipment (EU 002) operations will be used to apply coatings to smaller metal items like pumps and valves. The smaller items will be placed on carts for operations.  First the items will be prepped in the blast room which will contain two manually operated abrasive blast units. The room is enclosed and equipped with cartridge filters at the room exhausts. The exhaust emits into the building.  Next the parts will be coated in the paint booth. One side will be coated and then transferred to one of the two natural gas fired (NGF) ovens.  After one side of the coating is cured it will be returned to the paint booth to coat the other side. The paint booths are equipped with fabric filters that exhaust through the roof.  The ovens have a combined roof stack.  Solvents are used to thin the coatings and for paint gun cleanout. The solvents will be reclaimed in an on-site unit.

- EU 003 HPS (High Performance System) Overhead Line Application System:  Windsor purchased a conveyor coating system from the High-Performance Systems, Inc. Winter Haven Facility (Facility ID No. 1050158) that previously operated under Title V Air Operating Permit No. 1050158-009-AV.  The conveyor line will operate at a rate of approximately 10 feet per minute.  Metal parts are loaded onto the conveyor system.  Parts will be initially washed in the dip coat tanks.  There shall be a total of six (6) tanks at the site that include:

- Tank 1: Sodium hydroxide / detergent (or equivalent)

- Tank 2: Water

- Tank 3: Chromium phosphate (or equivalent)

- Tank 4: Water

- Tank 5: Acidulated water (or equivalent)

- Tank 6: Water

Tank 3 has a NGF heater with a roof exhaust. The parts are then dried in a NGF oven.  The conveyor line takes the parts through five (5) paint booths. The first two booths are used to apply primer to each side of the part.  The next two booths apply the topcoat and the fifth booth applies clear coat or touch up.  The paint booths are equipped with fabric filters and have roof exhausts.  Each booth has one stack except for the fifth booth which has two stacks.  After coating application, the parts move to the Paint Bake Oven to cure the coating.  The emissions from the oven are heated in an afterburner that vents to the dip tanks to provide indirect heat for Tanks 1 and 6. These emissions vent through the Tank 3 roof exhaust. The afterburner is not used as a control device; therefore, no emissions reduction is taken in the emissions calculations.  There is also a stack for the paint mix room; however, these emissions are included in the paint booth emissions and are not included as an emission unit.  In addition, the wastewater treatment system is not expected to have air emissions and is not included as an emission unit.

Equipment

- Engineered Coatings Batch Application Equipment and HPS Overhead Line Application System:  The permittee is authorized to install/construct the Engineered Coatings Batch Application Equipment and the HPS (High Performance System) Overhead Line Application System.  [Application No. 0970042-013-AC]

Performance Restrictions

- Permitted Capacity:  The permitted capacity of these EUs (EU Numbers 001, 002, and 003) is limited by the facility-wide VOC emission limit of 230 TPY per Specific Condition 10 in Section 2 of this permit.  This facility-wide VOC limit, caps the usage in gallons of each coating, thinner and cleanup material that contain VOC used in these EUs. [Application No. 0970042-013-AC; Rule 62-210.200(223), “Potential to Emit,” F.A.C., and Email Correspondence dated January 4, 2018, “Windsor - Solvent Information & VOC Tracking Equation”]

- Restricted Operation:  The hours of operation are not limited (8760 hours per year).  [Application No. 0970042-013-AC and Rules 62-4.070(3) and 62-210.200(223), “Potential to Emit,” F.A.C.]

- Filter Systems:  The filter systems used for the paint booths and blasting operations shall be in good operation condition.  The filters shall be inspected monthly at a minimum and replaced as appropriate, to maintain control design efficiency provided by vendor or manufacturer, and use replacement filters that meet same control efficiency.   This information can be found in Appendix F, Vendor’s or Manufacturer’s Information for Paint Booths and Blasting Operations.   The filter information in Appendix F can be updated at any time without an Air Construction Permit.  [Application No. 0970042-013-AC and Rule 62-4,070(3), F.A.C.; and Email correspondence dated January 10, 2018, “RE: Windsor - Solvent Information & VOC Tracking Equation (response to 2 questions)”]

- Options for meeting the organic HAP emission rate:  The facility must use at least one of the two compliance options listed below to meet organic HAP emission limits (Conditions A.6.c. and A.6.d.) or the Facility-Specific Emission Limit (Conditions A.6.e.).  The facility cannot use different compliance options at the same time on the same coating operation.  If the facility switches between compliance options for any coating operation or group of coating operations, the facility must document this switch as required by 40 CFR 60.3930(c), and it must be reported in the next semiannual compliance report required in 40 CFR 63.3920.

- Compliant Material Option:  Demonstrate that the organic HAP content of each coating used in the coating operation is less than or equal to the organic HAP emission limit in Condition A.6.c. (Emission Units 001 and 003) and Condition A.6.d. (Emission Unit 002), and that each thinner and/or other additive, and cleaning material used contains no organic HAP.  The facility must meet all the requirements of 40 CFR 63.3940, 40 CFR 63.3941, and 40 CFR 63.3942 to demonstrate compliance with the emission limit using this option.

[Application No. 0970042-013-AC and 40 CFR 63.3891(a)]

- Emission Rate Without Add-On Controls Option:  Demonstrate that, based on the coatings, thinners and/or other additives, and cleaning materials used in the coating operation, the organic HAP emission rate for the coating operations is less than or equal to the Facility-Specific Emission Limit in Condition A.6.e., calculated as a rolling 12-month emission rate and determined on a monthly basis.  The facility must meet all the requirements of 40 CFR 63.3950, 40 CFR 63.3951, and 40 CFR 63.3952 to demonstrate compliance with the emission limit using this option.	
[Application No. 0970042-013-AC and 40 CFR 63.3891(b)]

##### Emissions Standards

- Emissions Standards:

- Visible Emissions:  Visible Emissions shall not exceed 20 percent opacity.
[Rule 62-296.320(4)(b)1., F.A.C.]

- VOC Emissions:  VOC emissions from the facility shall not exceed 230 tons per year (TPY) on a 12-month rolled monthly basis.
[Application No. 0970042-013-AC and additional information provided on 01/04/18 by email]

- Organic HAP Emissions (EU 001 and EU 003): Limit organic HAP emissions to no more than 27.5 lb. organic HAP per gallon coating solids used during each 12-month compliance period.
Application No. 0970042-013-AC and 40 CFR 63.3890(b)(2)]

- Organic HAP Emissions (EU 002): Limit organic HAP emissions to no more than 1.9 lb. organic HAP per gallon coating solids used during each 12-month compliance period.  This emission limit applies to the general use coatings that are used at Emission Unit 002.  General Use Coatings means any material that meets the definition of coating but does not meet the definition of high performance coating, rubber-to-metal coating, magnet wire coating, or extreme performance fluoropolymer coating as defined in 40 CFR 63.3981.
[Application No. 0970042-013-AC and 40 CFR 63.3890(b)(2)]

- Facility-Specific Emission Limit: Limit organic HAP emissions to no more than the following emission limit:

[Application No. 0970042-013-AC and 40 CFR 63.3890(c)(2)]

{Permitting Note: Pursuant to Application No. 0970042-013-AC, as planning for the new emissions units continues, Windsor Metal Finishing will evaluate additional coatings and may change the compliance option (options listed under 40 CFR 63.3891) for meeting the emission limits when applying for the Title V Operation Permit Revision.  An additional air construction permit is not needed to switch options.}

Records and Reports

- Options for meeting the organic HAP emission rate:  If the facility switches between compliance options for any coating operation or group of coating operations, the facility must document this switch as required by 40 CFR 60.3930(c), and it must be reported in the next semiannual compliance report required in 40 CFR 63.3920.

- Monthly Log:  In order to demonstrate compliance with Specific Condition No. A.6.b., A.6.c., A.6.d., and A.6.e., the permittee shall maintain a log at the facility for a period of at least 5 years from the date the data is recorded.  The log shall be made available to the Department upon request. The log shall contain the following:

- Facility Name, Facility ID No. (i.e., 0970042), Emission Unit No. (i.e., EU 001);

- Month and year of record;

- Most recent monthly total of VOC emissions, including;

  - Name and identification of each coating, thinner and clean up material employed;

  - Number of gallons of each coating, thinner and clean up material employed;

  - The VOC content for each coating, thinner, and clean up material employed, as applied, in pounds per gallon;

  - Total VOC emissions, in tons and rolling 12-month emissions, calculated in accordance with the formula contained in Specific Condition 10 of Section 2 of this permit;

  - If purchase data is used to track monthly emissions; the following will be subtracted from the total purchase amount on a monthly basis: the unused volume of coating, thinner and clean up material; volume sent offsite for proper disposal; and volume of solvent stored for or having been reclaimed,

- Most recent monthly total of total gallons of coating solids used;

- Most recent monthly inspection results of filter systems used for the paint booths and blasting operations;

- Most recent consecutive 12-month total of VOC emissions;

- Most recent consecutive 12-month total of gallons of coating solids used; and

- If using the Compliant Material Option:

  - Record the organic HAP content for each thinner and/or other additive, and cleaning material used during the month.

  - Record the organic HAP content of each coating used in the coating operation. Verify whether it is less than or equal to the organic HAP emission limit (Conditions A.6.c. or A.6.d.); and

- If using the Emission Rate Without Add-On Control Option: Calculate the Facility Specific Emission Rate based on the most recent consecutive 12 months.  Verify whether it is less than or equal to the Facility Specific Emission Limit.  It shall be calculated as a consecutive 12-month emission rate and determined on a monthly basis.

Monthly logs shall be completed by the end of the following month.

A consecutive 12-month total is equal to the total for the month in question plus the totals for the eleven months previous to the month in question.  A consecutive 12-month total treats each month of the year as the end of a 12-month period.  A 12-month total is not a year-to-date total.  Facilities that have not been operating for 12 months should retain 12-month totals using whatever number of months of data is available until such a time as a consecutive 12-month total can be maintained each month. 	[Rule 62-4.070(3), F.A.C.]

- Additional Recordkeeping Requirements:  Records specified in Specific Condition No. A.8. (previous condition) must document the method, calculations, and formulas used in determining the usage rate and the emission rate.  All calculations, including those used to derive emissions, must be clearly documented; they may be presented in the form of a template of sample calculations.  The records must be available for review on site by the Department.

[Rule 62-4.070(3), F.A.C.]

- Supporting Documentation:  Supporting documentation (chemical usage tracking logs, “As Supplied” sheets, Safety Data Sheets, EPA data sheets, purchase orders, waste shipment/disposal records, etc.) shall be kept for each chemical and associated products which includes sufficient information to determine usage rates and emissions for a minimum of five (5) years and made available to the Department upon request.                  [Rule 62-4.070(3), F.A.C.]

- Semi-Annual Monitoring Reports:  The permittee shall submit reports of any required monitoring at least every six (6) months.  All instances of deviations from permit requirements must be clearly identified in such reports.  If the facility switches between compliance options for any coating operation or group of coating operations, the facility must document this switch as required by 40 CFR 60.3930(c), and it must be reported in the next semiannual compliance report required in 40 CFR 63.3920.  [Rule 62-213.440(1)(b)3.a., F.A.C.]

Other Requirements

- Federal Rule Requirement:   In addition to the specific conditions listed above, Emissions Unit Numbers 001, 002, and 003 are also subject to the applicable requirements contained in 40 CFR 63, Subpart A, General Provisions and 40 CFR 63, Subpart MMMM, Surface Coating of Miscellaneous Metal Parts and Products.  These conditions are incorporated into this permit (attached and part of this permit, See Appendices). 	[Rules 62-213.440, 62-204.800(11)(d)1., and 62-204.800(11)(b)69., F.A.C.; 40 CFR 63, Subpart A; 40 CFR 63, Subpart MMMM]