TECHNICAL EVALUATION

&

PRELIMINARY DETERMINATION

APPLICANT

JELD-WEN, Inc.

1700 Avenue A

Kissimmee, Florida 34758

Kissimmee Facility

Facility ID No. 0970059

PROJECT

Project No. 0970059-008-AO

Application for Synthetic Minor Source Air Operation Permit

Air Operation Permit Renewal

COUNTY

Osceola County, Florida

PERMITTING AUTHORITY

Florida Department of Environmental Protection

Permitting and Waste Cleanup Programs

Central District Office

Orlando, Florida 32803-3767

March 15, 2018

- General Project INFORMATION

Air Pollution Regulations

Projects at stationary sources with the potential to emit air pollution are subject to the applicable environmental laws specified in Section 403 of the Florida Statutes (F.S.).  The statutes authorize the Department of Environmental Protection (Department) to establish regulations regarding air quality as part of the Florida Administrative Code (F.A.C.), which includes the following applicable chapters:  62-4 (Permits); 62-204 (Air Pollution Control – General Provisions); 62-210 (Stationary Sources – General Requirements); 62-212 (Stationary Sources – Preconstruction Review); 62-213 (Operation Permits for Major Sources of Air Pollution); 62-296 (Stationary Sources - Emission Standards); and 62-297 (Stationary Sources – Emissions Monitoring).  Specifically, air construction permits are required pursuant to Chapters 62-4, 62-210 and 62-212, F.A.C.

In addition, the U. S. Environmental Protection Agency (EPA) establishes air quality regulations in Title 40 of the Code of Federal Regulations (CFR).  Part 60 specifies New Source Performance Standards (NSPS) for numerous industrial categories.  Part 61 specifies National Emission Standards for Hazardous Air Pollutants (NESHAP) based on specific pollutants.  Part 63 specifies NESHAP based on the Maximum Achievable Control Technology (MACT) for numerous industrial categories.  The Department adopts these federal regulations in Rule 62-204.800, F.A.C.

Glossary of Common Terms

Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of this permit.

Facility Description and Location

JELD-WEN, Inc. operates an existing wood door manufacturing facility, which is categorized under Standard Industrial Classification Code No. 2431.  The facility is located in Osceola County at 1700 Avenue A in Kissimmee, Florida.  The UTM coordinates of the existing facility are Zone 17, 451.6 km East, and 3125.74 km North.  This site is in an area that is in attainment for all air pollutants subject to Ambient Air Quality Standards (AAQS).

Facility Regulatory Categories

- The facility is not a major source of hazardous air pollutants (HAP).

- The facility is not a Title V major source of air pollution in accordance with Chapter 62-213, F.A.C.

- The facility is not a major stationary source in accordance with Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

- Because of information submitted for previous permit applications, this facility is a synthetic minor source of air pollution for HAPs and individual HAPs.  Based on its Annual Operating Reports, the facility could be classified as a natural minor source.  However, the facility requested during processing of Construction Permit 00970059-006-AC to continue to be classified as a synthetic minor source

- Project Description

- This is an Air Operation Permit Renewal for 0970059-007-AO.

- Processing Schedule

3/5/2018	Received the application for a minor source air pollution construction permit.

3/6/2018	Received the appropriate permit fee.

3/15/2018	Application determined complete.

- PSD Applicability

The project is located in Osceola County which is in an area that is currently in attainment with the AAQS.  The proposed project will not increase emissions for any PSD pollutant.  Therefore, the project is not subject to a PSD preconstruction review.

- Department review

Brief Discussion of Emissions

As provided by the applicant, VOC emissions from this permitting action are expected to remain below the following values:

| Pollutant | EU No. and brief description | Emissions Limitation (tons per any consecutive 12-month period) |
| --- | --- | --- |
| Total VOCs | EU 001 – Wood Door Manufacturing Facility | Less than 49.0 |
| Total HAPs | EU 001 – Wood Door Manufacturing Facility | Less than 25.0 |
| Individual HAPs | EU 001 – Wood Door Manufacturing Facility | Less than 10.0 |
| VE | Facility Wide | Less than twenty percent (20%) opacity |

- The VOC and HAP limitations were established in Construction Permit 0970059-001-AC, dated 11/14/97.  In accordance discussions with the facility during processing of Construction Permit 0970059-006-AC, the applicant requested no changes to emission limitations and to remain a synthetic minor source controlled by a synthetic minor permit, in order to maintain future operational flexibility.  No further changes regarding air emission sources have taken place at the facility since the processing of permit 0970059-006-AC.

State Requirements

This project is subject to the preconstruction review requirements of Chapter 403, Florida Statutes and Chapters 62-204 through 62-297, Florida Administrative Code (F.A.C.), as indicated below.

| Subject to: | Y/N | Comments |
| --- | --- | --- |
| Rule 62-210.300, F.A.C. – Stationary Source General Requirements | Y | Not exempt from general permitting requirements |
| Rule 62-212.400, F.A.C. - Prevention of Significant Deterioration | N | Facility is not a PSD major source |
| Rule 62-296.320(4), F.A.C. - General Particulate Emission Limiting Standards | Y | The facility is a potential source of unconfined particulate matter emissions. Visible emissions are limited to less than 20 percent opacity. |
| Rules 62-296.320(1) and (2), F.A.C. - General Pollutant Emission Limiting Standards (VOCs and Odor) | Y | Door assembly and coating operations are a potential source of VOCs and odor. |
| Rule 62-296.400, F.A.C. - Stationary Source Emission Standards | N | There is no applicable source category |
| Rule 62-296.500, F.A.C. - Reasonably Available Control Technology (VOC) | N | Osceola County is an attainment area for ozone |
| Rule 62-296.700, F.A.C. - Reasonably Available Control Technology (PM) | N | There is no applicable source category |
| Rule 62-204.800, F.A.C. - Standards of Performance for New Stationary Sources (NSPS– 40 CFR 60) | N | There is no applicable source category |
| Rule 62-204.800, F.A.C. National Emission Standard for Hazardous Air Pollutants (NESHAPS – 40 CFR 61) | N | There is no applicable source category |
| Rule 62-204.800, F.A.C. National Emission Standard for Hazardous Air Pollutants for Source Categories a.k.a. MACT (NESHAPS –40 CFR 63) | N | There is no applicable source category |
| Chapter 62-213, F.A.C. - Operation Permits for Major Sources of Air Pollution | N | Facility is a synthetic minor source |

Federal NSPS Provisions

There are no applicable federal NSPS provisions.

Federal NESHAP Provisions

There are no applicable federal NESHAP provisions.

Other Draft Permit Requirements

Provide a brief description of additional production and operation limitations, monitoring requirements, etc.

Example: The emission limits proposed by the applicant will meet all of the requirements of Chapters 62-204 through 297, F.A.C.

The General and Specific Conditions listed in the proposed permit (attached) will assure compliance with all the applicable requirements of Chapters 62-204 through 297, F.A.C.

- Preliminary Determination

The Department makes a preliminary determination that the proposed project will comply with all applicable state and federal air pollution regulations as conditioned by the draft permit.  This determination is based on a technical review of the complete application, reasonable assurances provided by the applicant, and the conditions specified in the draft permit.  No air quality modeling analysis is required because the project does not result in a significant increase in emissions.  Morgan Hampton is the project manager responsible for reviewing the application and drafting the permit.  Additional details of this analysis may be obtained by contacting the project manager at the Florida Department of Environmental Protection, Permitting and Waste Cleanup Program, Central District Office, 3319 Maguire Boulevard., Suite 232, Orlando, Florida 32803-3767, (407) 897-4100, or via email at morgan.hampton@dep.state.fl.us.