TECHNICAL EVALUATION

&

PRELIMINARY DETERMINATION

APPLICANT

Johnson Controls, Inc.

507 E. Michigan Street

Milwaukee, WI 53202

Johnson Controls – Largo Plant

Facility ID No. 1030577

PROJECT

Project No. 1030577-001-AC

Application for Minor Source Air Construction Permit

Initial Construction of four (4) fin presses

COUNTY

Pinellas County, Florida

PERMITTING AUTHORITY

Florida Department of Environmental Protection

Air and Solid Waste Permitting

Southwest District Office

Temple Terrace, Florida 33637-0926

8/18/2016

Prepared by Quaid Noor

- General Project INFORMATION

Air Pollution Regulations

Projects at stationary sources with the potential to emit air pollution are subject to the applicable environmental laws specified in Section 403 of the Florida Statutes (F.S.).  The statutes authorize the Department of Environmental Protection (Department) to establish regulations regarding air quality as part of the Florida Administrative Code (F.A.C.), which includes the following applicable chapters:  62-4 (Permits); 62-204 (Air Pollution Control – General Provisions); 62-210 (Stationary Sources – General Requirements); 62-212 (Stationary Sources – Preconstruction Review); 62-213 (Operation Permits for Major Sources of Air Pollution); 62-296 (Stationary Sources - Emission Standards); and 62-297 (Stationary Sources – Emissions Monitoring).  Specifically, air construction permits are required pursuant to Chapters 62-4, 62-210 and 62-212, F.A.C.

In addition, the U. S. Environmental Protection Agency (EPA) establishes air quality regulations in Title 40 of the Code of Federal Regulations (CFR).  Part 60 specifies New Source Performance Standards (NSPS) for numerous industrial categories.  Part 61 specifies National Emission Standards for Hazardous Air Pollutants (NESHAP) based on specific pollutants.  Part 63 specifies NESHAP based on the Maximum Achievable Control Technology (MACT) for numerous industrial categories.  The Department adopts these federal regulations in Rule 62-204.800, F.A.C.

Glossary of Common Terms

Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of this permit.

Facility Description and Location

Johnson Controls – Largo Plant will be a new refrigeration and heating equipment manufacturing facility, which is categorized under Standard Industrial Classification Code No. 3585.  The new Johnson Controls – Largo Plant will be located in Pinellas County at 8575 Largo Lakes Blvd in Largo, Florida.  The UTM coordinates of the new facility are Zone 17, 327.00 km East, and 3085.00 km North.  This site is in an area that is in attainment (or designated as unclassifiable) for all air pollutants subject to Ambient Air Quality Standards (AAQS).

Facility Regulatory Categories

- The facility is not a major source of hazardous air pollutants (HAP).

- The facility is not a Title V major source of air pollution in accordance with Chapter 62-213, F.A.C.

- The facility is not a major stationary source in accordance with Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

- This facility is a natural minor source of air pollution.

- Project Description

- The project is for the construction of four (4) fin process for a facility that manufactures products for the commercial heating, ventilating, and air conditioning (HVAC) industry.  The product manufactured at the facility include variable air volume terminals and controls, fan coil units, and indoor air handlers.  The fin presses will emit volatile organic compounds (VOC) fugitively.

- The facility will have miscellaneous exempt emission units/activities.

- Processing Schedule

07/25/2016	Received the application for a minor source air pollution construction permit.

07/27/2016	Requested additional information.

08/05/2016	Received additional information; application complete.

- PSD Applicability

The project is located in Pinellas County which is in an area that is currently in attainment with the AAQS or is otherwise designated as unclassifiable.  The facility is not a PSD major source and therefore the proposed project is not subject to a PSD preconstruction review.

- Department review

- The project is a concurrent processing of an air construction permit and an air operation permit.  This is due the fact that the facility does not have to show compliance though any testing.  Recordkeeping will be the only mode of showing compliance.

- List of Emissions Units

| EU ID No. | Emissions Unit Description |
| --- | --- |
| 001 | 4 Fin Presses |

- Summary of Emissions

| Pollutant | EU No. | Potential Emissions (tpy) | Allowable Emissions (tpy) |
| --- | --- | --- | --- |
| VOC | 001 | 20.6 | 20.6 |

{Note:  Stamping fluid is the VOC emissions source (90% VOC) and no HAPs.  Maximum annual rate is calculated based on 2015 usage (in the old site) (3,300 gallons in 4000 hours) extrapolated to unrestricted usage (8,760 hrs.) to be 20.53 tons/year rounded to 20.6 tons/year, emissions factor is 4.69 lb/hour.}

- State Requirements and Federal Regulations

This project is subject to the preconstruction review requirements of Chapter 403, Florida Statutes and Chapters 62-204 through 62-297, Florida Administrative Code (F.A.C.), as indicated below.

| Subject to: | Y/N | Comments |
| --- | --- | --- |
| Rule 62-210.300, F.A.C. – Permits Required | Y | subject to general permitting requirements |
| Rule 62-212.400, F.A.C. - Prevention of Significant Deterioration | N | facility is not a PSD major source |
| Rule 62-296.320(4), F.A.C. - General Particulate Emission Limiting Standards | N | there is no applicable source category |
| Rules 62-296.320(1) and (2), F.A.C. - General Pollutant Emission Limiting Standards (VOCs and Odor) | Y | fin press operations are a source of VOC and odors |
| Rule 62-296.401-418, F.A.C. - Stationary Source Emission Standards | N | there is no applicable source category |
| Rule 62-296.500, F.A.C. - Reasonably Available Control Technology (VOC) | N | Pinellas County is not an air quality maintenance area for ozone |
| Rule 62-296.700, F.A.C. - Reasonably Available Control Technology (PM) | N | is a new source, therefore not subject to PM RACT rule |
| Rule 62-204.800, F.A.C. - Standards of Performance for New Stationary Sources (NSPS) | N | there is no applicable source category |
| Rule 62-204.800, F.A.C. National Emission Standard for Hazardous Air Pollutants (NESHAPS – 40 CFR 61) | N | there is no applicable source category |
| Rule 62-204.800, F.A.C. National Emission Standard for Hazardous Air Pollutants for Source Categories a.k.a. MACT (NESHAPS –40 CFR 63) | N | there is no applicable source category |
| Chapter 62-213, F.A.C. - Operation Permits for Major Sources of Air Pollution | N | facility is a natural non-Title V source |
| Rule 62-297.310, F.A.C. - General Compliance Test Requirements, F.A.C. | N | Compliance is demonstrated via recordkeeping. |

- Preliminary Determination

The Department makes a preliminary determination that the proposed project will comply with all applicable state and federal air pollution regulations as conditioned by the draft permit.  This determination is based on a technical review of the complete application, reasonable assurances provided by the applicant, and the conditions specified in the draft permit.  No air quality modeling analysis is required because the project does not result in a significant increase in emissions.  Quaid Noor is the project engineer responsible for reviewing the application and drafting the permit.  Additional details of this analysis may be obtained by contacting the project engineer at the Department’s Air and Solid Waste Permitting Section in the Southwest District Office, 13051 North Telecom Parkway, Temple Terrace, Florida 33637-0926, phone 813-470-5726, or by email quaid.noor@dep.state.fl.us.