Permittee

| Mosaic Fertilizer, LLC 13830 Circa Crossing Drive Lithia, Florida 33547 Authorized Representative: Keith Nadaskay, Sr. Environmental Specialist | Air Permit No. 1050055-028-AC Permit Expires: 06/29/2019 Minor Air Construction Permit South Pierce Facility Modification of SAP No. 10 |
| --- | --- |

Project

This is the final air construction permit, which authorizes the replacement of the interpass absorber’s (IPA) Heat Recovery System (HRS) superheater, the IPA tower first stage distributor and the final absorption tower (FAT) acid distribution system and the upgrade of the sulfuric acid mist (SAM) eliminator candles in the HRS absorber and final absorption tower. The proposed work will be conducted at the existing South Pierce Facility, which is a phosphate fertilizer manufacturer categorized under Standard Industrial Classification No. 2874.  The existing facility is in Polk County at 7450 Highway 630 in Mulberry, Florida.  The UTM coordinates are Zone 17, 407.53 km East and 3071.51 km North.

This final permit is organized into the following sections:  Section 1 (General Information); Section 2 (Administrative Requirements); Section 3 (Emissions Unit Specific Conditions); and Section 4 (Appendices).  Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of Section 4 of this permit.

Statement of Basis

This air pollution construction permit is issued under the provisions of:  Chapter 403 of the Florida Statutes (F.S.) and Chapters 62-4, 62-204, 62-210, 62-212, 62-296 and 62-297 of the Florida Administrative Code (F.A.C.).  The permittee is authorized to conduct the proposed work in accordance with the conditions of this permit.  This project is subject to the general preconstruction review requirements in Rule 62-212.300, F.A.C. and is not subject to the preconstruction review requirements for major stationary sources in Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

Upon issuance of this final permit, any party to this order has the right to seek judicial review of it under Section 120.68 of the Florida Statutes by filing a notice of appeal under Rule 9.110 of the Florida Rules of Appellate Procedure with the clerk of the Department of Environmental Protection in the Office of General Counsel (Mail Station #35, 3900 Commonwealth Boulevard, Tallahassee, Florida, 32399-3000) and by filing a copy of the notice of appeal accompanied by the applicable filing fees with the appropriate District Court of Appeal.  The notice must be filed within 30 days after this order is filed with the clerk of the Department.

Executed in Hillsborough County, Florida

(DRAFT)

_________________________________

Pamala Vazquez

Program Administrator

Permitting & Waste Cleanup Program

Southwest District

CERTIFICATE OF SERVICE

The undersigned duly designated deputy agency clerk hereby certifies that this Air Permit package was sent by electronic mail, or a link to these documents made available electronically on a publicly accessible server, with received receipt requested before the close of business on the date indicated below to the following persons.

# Keith Nadaskay, Mosaic Fertilizer, LLC, keith.nadaskay@mosaicco.com

# Rama Iyer, PE, ERM, ramaiyer.gte@gmail.com

# Scott Borderieux, Florida DEP Southwest District, scott.borderieux@dep.state.fl.us

# Max Grondahl, Florida DEP Southwest District, max.grondahl@dep.state.fl.us

# Clerk Stamp

# FILING AND ACKNOWLEDGMENT FILED, on this date, pursuant to Section 120.52(7), Florida Statutes, with the designated agency clerk, receipt of which is hereby acknowledged.

(DRAFT)

______________________________________________

(Clerk)                            (Date)

Facility Description

The existing facility consists of the following emissions units.

| Facility ID No. 1050055 |  |
| --- | --- |
| ID No. | Emission Unit Description |  |
| 004 | Sulfuric Acid Plant No. 10 |  |
| 005 | Sulfuric Acid Plant No. 11 |  |
| 030 | Molten Sulfur Storage and Handling |  |
| 048 | Phosphogypsum Stack |  |
| 049 | Fugitive Emissions (Unregulated) |  |
| 054 | Emergency Diesel Engines |  |
| 055 | Stationary CI ICE |  |

PROPOSED PROJECT

To replace the interpass absorber’s (IPA) heat recovery system (HRS) superheater, the IPA tower first stage distributor and upgrade the sulfuric acid mist (SAM) eliminator candles with an auto drain design.  The final absorption tower (FAT) acid distribution system will also be replaced, along with the auto drain design upgrade to the SAM eliminator candle system.  The converter catalyst condition will be evaluated and the fourth bed catalyst will be changed and/or augmented as needed with enhanced cesium catalyst, along with any necessary changes to the catalyst in the other beds.  Other minor work such as changes to associated pumps, ducts, vessels, etc., as determined by turnaround inspections may be performed as part of this project.

This project will modify the following emissions units.

| Facility ID No. 1050055 |  |
| --- | --- |
| ID No. | Emission Unit Description |  |
| 004 | Sulfuric Acid Plant No. 10 |  |

Facility Regulatory Classification

- The facility is a major source of hazardous air pollutants (HAP).

- The facility does not operate units subject to the acid rain provisions of the Clean Air Act (CAA).

- The facility is a Title V major source of air pollution in accordance with Chapter 62-213, F.A.C.

- The facility is a major stationary source in accordance with Rule 62-212.400(PSD), F.A.C.

# Permit History/Affected Permits

Reference current Title V Operation Permit 1050055-025-AV.

- Permitting Authority:  The permitting authority for this project is the Southwest District of the Department of Environmental Protection (Department).  The mailing address, phone number and e-mail address is:

Florida Department of Environmental Protection

Southwest District Office

Air and Solid Waste Permitting Program

13051 North Telecom Parkway, Suite 101

Temple Terrace, Florida 33637-0926

Telephone: 813-470-5700

E-mail: SWD_Air_Permitting@dep.state.fl.us

All documents related to applications for permits shall be submitted to the above e-mail address and/or address.

- Compliance Authority:  All documents related to compliance activities such as reports, tests, and notifications shall be submitted to the Florida Department of Environmental Protection (Department), Southwest District Office’s Compliance Assurance Program.  The mailing address, phone number and e-mail address is:

Florida Department of Environmental Protection

Southwest District Office

Compliance Assurance Program

13051 North Telecom Parkway, Suite 101

Temple Terrace, Florida 33637-0926

Telephone: 813-470-5700

E-mail: SWD_Air@dep.state.fl.us

- Appendices:  The following Appendices are attached as a part of this permit:

- Appendix A (Citation Formats and Glossary of Common Terms);

- Appendix B (General Conditions);

- Appendix C (Common Conditions);

- Appendix D (Common Testing Requirements);

- Appendix E (NSPS 40 CFR 60 - Subpart A - General Provisions);

- Appendix F (NSPS 40 CFR 60 - Subpart H - Standards of Performance for Sulfuric Acid Plants);

- Appendix G (Best Operational Start-up Practices for Sulfuric Acid Plants); and

- Appendix H (BACT, PSD-FL-235/1050055-010-AC).

- Applicable Regulations, Forms and Application Procedures:  Unless otherwise specified in this permit, the construction and operation of the subject emissions units shall be in accordance with the capacities and specifications stated in the application.  The facility is subject to all applicable provisions of: Chapter 403, F.S.; and Chapters 62-4, 62-204, 62-210, 62-212, 62-213, 62-296 and 62-297, F.A.C.  Issuance of this permit does not relieve the permittee from compliance with any applicable federal, state, or local permitting or regulations.

- New or Additional Conditions:  For good cause shown and after notice and an administrative hearing, if requested, the Department may require the permittee to conform to new or additional conditions.  The Department shall allow the permittee a reasonable time to conform to the new or additional conditions, and on application of the permittee, the Department may grant additional time.

[Rule 62-4.080, F.A.C.]

- Modifications:  The permittee shall notify the Compliance Authority upon commencement of construction.  No new emissions unit shall be constructed and no existing emissions unit shall be modified without obtaining an air construction permit from the Department.  Such permit shall be obtained prior to beginning construction or modification.

[Rules 62-210.300(1) and 62-212.300(1)(a), F.A.C.]

- Construction and Expiration.  The expiration date shown on the first page of this permit provides time to complete the physical construction activities authorized by this permit, complete any necessary compliance testing, and obtain an operation permit.  Notwithstanding this expiration date, all specific emissions limitations and operating requirements established by this permit shall remain in effect until the facility or emissions unit is permanently shut down.  For good cause, the permittee may request that that a permit be extended.  Pursuant to Rule 62-4.080(3), F.A.C., such a request shall be submitted to the Permitting Authority in writing before the permit expires.

[Rules 62-4.070(4), 62-4.080 & 62-210.300(1), F.A.C.]

- Source Obligation:

- Authorization to construct shall expire if construction is not commenced within 18 months after receipt of the permit, if construction is discontinued for a period of 18 months or more, or if construction is not completed within a reasonable time.  This provision does not apply to the time period between construction of the approved phases of a phased construction project except that each phase must commence construction within 18 months of the commencement date established by the Department in the permit.

- At such time that a particular source or modification becomes a major stationary source or major modification (as these terms were defined at the time the source obtained the enforceable limitation) solely by virtue of a relaxation in any enforceable limitation which was established after August 7, 1980, on the capacity of the source or modification otherwise to emit a pollutant, such as a restriction on hours of operation, then the requirements of subsections 62-212.400(4) through (12), F.A.C., shall apply to the source or modification as though construction had not yet commenced on the source or modification.

- At such time that a particular source or modification becomes a major stationary source or major modification (as these terms were defined at the time the source obtained the enforceable limitation) solely by exceeding its projected actual emissions, then the requirements of subsections 62-212.400(4) through (12), F.A.C., shall apply to the source or modification as though construction had not yet commenced on the source or modification.

[Rule 62-212.400(12), F.A.C.]

- Actual Emissions Reporting:  This permit is based on an analysis that compared baseline actual emissions with projected actual emissions and avoided the requirements of subsection 62-212.400(4) through (12), F.A.C. for several pollutants.  Therefore, pursuant to Rule 62-212.300(1)(e), F.A.C., the permittee is subject to the following monitoring, reporting and recordkeeping provisions.

- The permittee shall monitor the emissions of any PSD pollutant that the Department identifies could increase as a result of the construction or modification and that is emitted by any emissions unit that could be affected; and, using the most reliable information available, calculate and maintain a record of the annual emissions, in tons per year on a calendar year basis, for a period of 5 years following resumption of regular operations after the change.  Emissions shall be computed in accordance with the provisions in Rule 62-210.370, F.A.C., which are provided in Appendix C of this permit.

- The permittee shall report to the Department within 60 days after the end of each calendar year during the 5-year period setting out the unit’s annual emissions during the calendar year that preceded submission of the report.  The report shall contain the following:

  - The name, address and telephone number of the owner or operator of the major stationary source;

  - The annual emissions calculations pursuant to the provisions of 62-210.370, F.A.C., which are provided in Appendix C of this permit;

  - If the emissions differ from the preconstruction projection, an explanation as to why there is a difference; and

  - Any other information that the owner or operator wishes to include in the report.

- The information required to be documented and maintained pursuant to subparagraphs 62-212.300(1)(e)1 and 2, F.A.C., shall be submitted to the Department which shall make it available for review to the general public.

For this project, the permit requires the annual reporting of actual NOx, SO2 and SAM emissions for the following unit:  EU 004, Sulfuric Acid Plant #10.

[Application 1050055-028-AC; and Rules 62-212.300(1)(e) and 62-210.370, F.A.C.]

- Application for Title V Air Operation Permit: This permit authorizes modification of the permitted emissions unit and initial operation to determine compliance with Department rules.  A Title V air operation permit is required for continued operation of the permitted emissions unit(s).  The permittee shall apply for a Title V air operation permit at least 90 days prior to expiration of this permit, but no later than 180 days after commencing operation or commencing operation as modified.  Commencing operation means setting into operation of any emissions unit for any purpose.  To apply for a Title V air operation permit, the applicant shall submit the following:

- the appropriate permit application form (see current version of Rule 62-210.900, F.A.C. (Forms and Instructions), and/or FDEP Division of Air Resource Management website at: http://www.dep.state.fl.us/air/);

- a copy of the initial compliance test report required by Specific Condition No. A.15., if not previously submitted; and

- copies of the most recent month of records/logs specified in Specific Condition Nos. A.16. to A.20.

The application shall be submitted to the Permitting Authority.

[Rules 62-4.030 and 62-4.050, F.A.C.]

- Electronic Annual Operating Report and Title V Annual Emissions Fees.  The information required by the Annual Operating Report for Air Pollutant Emitting Facility [Including Title V Source Emissions Fee Calculation] (DEP Form No. 62-210.900(5)) shall be submitted by April 1 of each year, for the previous calendar year, to the Department of Environmental Protection’s (DEP) Division of Air Resource Management.  Each Title V source shall submit the annual operating report using the DEP’s Electronic Annual Operating Report (EAOR) software, unless the Title V source claims a technical or financial hardship by submitting DEP Form No. 62-210.900(5) to the DEP Division of Air Resource Management instead of using the reporting software.  Emissions shall be computed in accordance with the provisions of subsection 62-210.370(2), F.A.C.  Each Title V source must pay between January 15 and April 1 of each year an annual emissions fee in an amount determined as set forth in subsection 62-213.205(1), F.A.C.  The annual fee shall only apply to those regulated pollutants, except carbon monoxide and greenhouse gases, for which an allowable numeric emission-limiting standard is specified in the source’s most recent construction permit or operation permit.  Upon completing the required EAOR entries, the EAOR Title V Fee Invoice can be printed by the source showing which of the reported emissions are subject to the fee and the total Title V Annual Emissions Fee that is due.  The submission of the annual Title V emissions fee payment is also due (postmarked) by April 1st of each year.  A copy of the system-generated EAOR Title V Annual Emissions Fee Invoice and the indicated total fee shall be submitted to:  Major Air Pollution Source Annual Emissions Fee, Post Office Box 3070, Tallahassee, Florida 32315-3070.  Additional information is available by accessing the Title V Annual Emissions Fee On-line Information Center at the following Internet web site:  http://www.dep.state.fl.us/air/emission/tvfee.htm.  [Rules 62-210.370(3), 62-210.900 & 62-213.205, F.A.C.; and, §403.0872(11), Florida Statutes (2013)]

{Permitting Note:  Resources to help you complete your AOR are available on the electronic AOR (EAOR) website at:  http://www.dep.state.fl.us/air/emission/eaor.  If you have questions or need assistance after reviewing the information posted on the EAOR website, please contact the Department by phone at (850) 717-9000 or email at eaor@dep.state.fl.us.}

{Permitting Note:  The Title V Annual Emissions Fee form (DEP Form No. 62-213.900(1)) has been repealed.  A separate Annual Emissions Fee form is no longer required to be submitted by March 1st each year.}

This section of the permit addresses the following emissions unit.

| EU No. | Emission Unit Description |
| --- | --- |
| 004 | Sulfuric Acid Plant No. 10: A double absorption process SAP with a design production capacity of 3,000 tons per day expressed as 100% sulfuric acid. Sulfuric Acid Mist (SAM) emissions are controlled by high efficiency mist eliminators. |

Federal Regulations

- Federal Regulatory Requirements: This emission unit is subject to 40 CFR 60, Subpart H – Standards of Performance for Sulfuric Acid Plants, which is adopted by reference in Rule 62-204.800, F.A.C.

[Rule 62-204.800(8), F.A.C.]

Equipment

- SAP #10 HRS superheater, final absorption tower and SAM elimination candles:  The permittee is authorized to replace the interpass absorber’s (IPA) heat recovery system (HRS) superheater, the IPA tower first stage distributor and upgrading the sulfuric acid mist (SAM) eliminator candles with an auto drain design.  The final absorption tower (FAT) acid distribution system will also be replaced, along with the auto drain design upgrade to the SAM eliminator candle system.  The converter catalyst condition will be evaluated and the fourth bed catalyst will be changed and/or augmented as needed with enhanced cesium catalyst, along with any necessary changes to the catalyst in the other beds.  Other minor work such as changes to associated pumps, ducts, vessels, etc., as determined by turnaround inspections may be performed as part of this project.

[Application No. 1050055-028-AC]

Performance Restrictions

- Permitted Capacity, Sulfuric Acid Production:  The maximum allowable production rate of SAP #10 is 3,000 tons/day as 100% sulfuric acid.

[Rule 62-210.200(PTE), F.A.C.; and Permit No. 1050055-010-AC/PSD-FL-235]

- Permitted Capacity, Molten Sulfur Throughput:  The molten sulfur throughput rate shall not exceed 2,300 tons/day on a 5-day rolling average or 725,000 tons/year.

[Rule 62-210.200(PTE), F.A.C.; and Permit No. 1050055-013-AC]

- Restricted Operation:  The hours of operation are not limited (8760 hours per year).

[Rules 62-4.070(3) and 62-210.200(PTE), F.A.C.]

- Not federally enforceable. Best Operational Practices. The permittee shall follow the attached Memorandum of Understanding Regarding Best Operational Start-Up Practices for Sulfuric Acid Plants. (Signed and executed on November 1, 1989.)

[Rule 62-4.070(3), F.A.C.]

##### Emissions Standards

- Emissions Standards:

- Visible Emissions: Visible emissions shall not exceed 10% opacity.

[Rule 62-204.800(8)(b), F.A.C., 40 CFR 60.83(a)(2), 1050055-010-AC/PSD-FL-235 and BACT Determination of September 15, 1997]

- SO2 Emissions.  Sulfur dioxide emissions shall not exceed any of 4 lb/ton 100% sulfuric acid produced, 500 lb/hr or 2,190 ton/yr.

[Rule 62-204.800(8)(b), F.A.C., 40 CFR 60.82(a), 1050055-010-AC/PSD-FL-235 and BACT Determination of September 15, 1997]

- SAM Emissions.  Sulfuric Acid Mist (SAM) emissions shall not exceed any of 0.15 lb/ton sulfuric acid produced, 18.8 lb/hr or 82.1 ton/yr

[Rule 62-204.800(8)(b), F.A.C., 40 CFR 60.82(a), 1050055-010-AC/PSD-FL-235 and BACT Determination of September 15, 1997]

- NOx Emissions.  Nitrogen oxide (NOx) emissions shall not exceed any of 0.12 lb/ton of 100% sulfuric acid produced, 15.0 lb/hr or 65.7 ton/yr:

[1050055-010-AC/PSD-FL-235 and BACT Determination of September 15, 1997]

- Best Operational Practices. This permit acknowledges that leaks of sulfur dioxide and sulfur trioxide, or other fugitive process emissions that do not pass through a stack, may occur as part of routine operations.  Best operational practices to minimize these emissions shall be adhered to and shall include regular inspections and the prompt repair or correction of any leaks or other fugitive emissions. Best operational practices for Sulfuric Acid Plant #10 shall include but not limited to the following:

  - The permittee shall inspect the burner floor for pooled sulfur once per day.

  - On controlled shutdowns, if there is pooled sulfur in the burner, the permittee shall continue to purge with the blower at 1800 RPM.  Once the sulfur is consumed, the permittee shall continue to purge for an additional two minutes.

  - The permittee shall change out and maintain sulfur guns semiannually.  The permittee shall test the spray pattern with water before installing replacement guns.

[Rules 62-210.700(1), F.A.C. and 62-4.070(3), F.A.C.]

##### Testing Requirements

- Initial Compliance Tests:  The emissions unit shall be tested to demonstrate initial compliance with the emissions standards for NOx, SAM and VE.  The initial tests shall be conducted within 60 days after achieving permitted capacity, but not later than 180 days after initial operation of the unit.

[Rules 62-4.070(3) and 62-297.310(8)(b)1, F.A.C.]

- Annual Compliance Tests:  During each calendar year (January 1st to December 31st), the emissions unit shall be tested to demonstrate compliance with the emissions standards for NOx, SAM and VE.

[Rule 62-297.310(8)(a)1, F.A.C.]

- Test Requirements:  The permittee shall notify the Compliance Authority in writing at least 15 days prior to any required tests.  Tests shall be conducted in accordance with the applicable requirements specified in Appendix D (Common Testing Requirements) of this permit.

[Rule 62-297.310(9), F.A.C.]

- Test Methods:  Required tests shall be performed in accordance with the following reference methods.

| Method | Description of Method and Comments |
| --- | --- |
| 1-4 | Traverse Points, Velocity and Flow Rate, Gas Analysis, and Moisture Content |
| 7E | Determination of Nitrogen Oxide Emissions from Stationary Sources |
| 8 | Determination of Sulfuric Acid Mist and Sulfur Dioxide Emissions from Stationary Sources |
| 9 | Visual Determination of the Opacity of Emissions from Stationary Sources |

The above methods are described in Appendix A of 40 CFR 60 and are adopted by reference in Rule 62-204.800, F.A.C.  No other methods may be used unless prior written approval is received from the Department.

[Rules 62-204.800, F.A.C.; and Appendix A of 40 CFR 60]

Monitoring Requirements

- SO2 CEMS.  The continuous emission monitoring system for the measurement of sulfur dioxide shall be calibrated, maintained and operated as specified in 40 CFR 60.84.  The span value of the continuous monitor shall be set at 1000 PPM.

The permittee shall establish a conversion factor for the purpose of converting sulfur dioxide monitoring data into the units of the applicable standard (lb/ton).  The conversion factor shall be determined, as a minimum, three times daily in accordance with 40 CFR 60.84(b) or equivalent method.  A record of all conversion factors and values from which they were calculated shall be maintained.  The permittee may also use the alternative method, including electronic data systems, of determining sulfur dioxide emission rates using the continuous emission monitoring approach and calculation procedures referenced in 40 CFR 60.84(d).

[Rules 62-296.402 and 62-204.800(8)(b), F.A.C., and 40 CFR 60.84]

NOTIFICATION Requirements

- Notification of Operation Commencement: The permittee shall notify the Compliance Authority in writing of the date of recommencing operation of the EU No. 004 after completing the modifications authorized by this permit, no later than fifteen (15) days after that date. Commencing operation means setting into operation of any emissions unit for any purpose.

[Rules 62-4.070, and 62-210.200, F.A.C., (definition of Commence Operation)]

Records and Reports

- Test Reports:  The permittee shall prepare and submit reports for all required tests in accordance with the requirements specified in Appendix D (Common Testing Requirements) of this permit.

[Rule 62-297.310(10), F.A.C.]

- Daily Sulfuric Acid Production Records.  To document compliance with the maximum allowable production rates of Specific Condition No. A.3, the Permittee shall maintain daily records of H2SO4 production for each emissions unit.  Documentation as to how daily production rates were calculated shall be included as part of the records.

[Rule 62-4.070(3), F.A.C.]

- Daily SO2 Emissions Records.  To document ongoing compliance with the sulfur dioxide emissions limits in Specific Condition No. A.7., the permittee shall maintain the following records for each day of the month:

- daily acid production (in tons as 100% H2SO4);

- daily hours operated;

- daily average SO2 emission rate (in lbs/ton of 100% H2SO4 produced);

[Rule 62-4.070(3), F.A.C.]

- Sulfuric Acid Plants Record Requirements (NSPS). In accordance with 40 CFR 60.7(b), the permittee shall maintain records of the occurrence and duration of any startup, shutdown, or malfunction in the operation of the sulfuric acid plants; any malfunction of the air pollution control equipment; or any periods during which a continuous monitoring system or monitoring device is inoperative. Records on monitoring system performance evaluations, calibrations and maintenance shall be maintained in accordance with 40 CFR 60.7(d).

[Rule 62-204.800(8)(b), F.A.C. and 40 CFR 60.7].

- SO2 Excess Emissions Report (NSPS).  The permittee shall submit a written report of excess sulfur dioxide emissions quarterly in accordance with 40 CFR 60.7 (b) and (c).  Periods of excess emissions shall be all three-hour periods (or the arithmetic average of three consecutive one-hour periods) during which the integrated average sulfur dioxide emissions exceed the applicable standard under 40 CFR 60.82.  The excess emission report shall also include a statement of all periods during the quarter when the sulfur dioxide monitoring system was inoperative.  Copies of the quarterly sulfur dioxide excess emission report shall be submitted to the Southwest District Office.

[Rule 62-204.800(8)(b), F.A.C., and 40 CFR 60.7 and 60.84(e)].

- Excess Emissions Quarterly Report. In case of excess emissions resulting from malfunctions, the permittee shall notify the Air Compliance Section of the Southwest District Office of the Department of Environmental Protection in accordance with Rule 62-4.130, F.A.C.  A full written report on the malfunctions shall be submitted in a quarterly report, if requested by the Department.

[Rule 62-210.700(6), F.A.C.]