PERMITTEE:					DRAFT Permit No.: 1050387-004-AC

General Asphalt of Lakeland, Inc.		Effective Date:  XX/XX/XXXX

5640 Lake Grove Drive              			Expiration Date:  12/31/2010

Lakeland, Florida 33809			County: Polk

Project:  Asphalt Batch Plant with Reclaimed Asphalt Pavement (RAP) Crusher

This permit is issued under the provisions of Chapter 403, Florida Statutes, and Florida Administrative Code (F.A.C.) Chapters 62-4, 62-204, 62-210, 62-212, 62-213, 62-296, and 62-297.  The above-named permittee is hereby authorized to perform the work or operate the facility shown on the application and approved drawing(s), plans, and other documents, attached hereto or on file with the Department and made a part hereof and specifically described as follows:

For the addition of a 125-TPH 2002 Mobark 4600 Woodhog horizontal grinder equipped with a 2002 525-HP Caterpillar diesel engine; a process change to allow the blending of non-asbestos containing, virgin asphalt shingles with sand (50%/50% mixture); and a process change to allow ground shingles and the shingle-sand mixture as feed to the existing asphalt batch plant.  The facility contains the following emissions units:

### Permitted Emission Units

Emission Unit ID No. 001 – Asphalt Batch Plant:  This emission unit (EU) is a Cederapids (Model H60C-E, Serial No. 31203) asphalt batch plant having a design capacity of 200 tons per hour (TPH).  The plant is permitted to process either a conventional surface (i.e. virgin) mix or RAP.  The Cederapids asphalt batch plant dryer burner is fired at a maximum heat input rate of 100 million British thermal units (MMBtu)/hour with “on-specification” used fuel oil.  Residual No. 5 fuel oil and natural gas will be the alternate fuels.  The sulfur content of the fuel oils shall not exceed one percent (1.0%), by weight.  The maximum fuel oil consumption rate shall be 1.2 million gallons per year.  Emissions are controlled by a Dustex, Model RA-15-34 DB7, baghouse collection system rated at 45,000 cubic feet per minute, actual (acfm).  The asphalt batch plant is equipped with belt scales and a computer system that displays the production rate.

Emission Unit ID No. 002 – Reclaimed Asphalt Pavement Crusher:  This portable RAP crushing system is equipped with an internal stationary screening unit used to grind/crush reclaimed asphalt to a desired size before use in the asphalt mix; a Chieftain Power Screener is used to further process the RAP from the crusher.  The maximum throughput rate is 250 TPH.  The unit will be powered by two (2) exempt diesel engines (see Exempt Emission Sources below).

Emission Unit ID No. 003 – Asphalt Shingle Grinder:  This E.U. consists of a 125-TPH portable recycle asphalt shingle grinder.  The grinder is a 2002 Mobark 4600 Woodhog horizontal grinder equipped with a 2002 525-HP Caterpillar diesel engine (see Exempt Emission Sources below).  This grinder is used to grind virgin, non-asbestos containing shingles into a smaller size that can be used in the manufacture of new hot mix asphalt.

Exempt Emission Sources

The following items at the facility are exempt/insignificant emission units:

The oil heating system, used to heat liquid asphalt and fuel oil supplied to the plant’s burner system, rated at 1.2 MMBtu/hour.  The heater is fired with No. 2 diesel oil having maximum sulfur content not to exceed 0.5%, by weight;

[Rule 62-4.040(b), F.A.C.]

Two (2) 125-ton capacity Hot Mix Asphalt Storage Silos; and

[Rule 62-4.040(b), F.A.C.]

Four (4) diesel engines to power the asphalt plant’s exhaust fan, RAP crusher, RAP Chieftain Power Screener, and asphalt shingle grinder rated at a maximum heat input rate of 1.24, 2.07, 0.63, and 3.7 MMBtu/hr., respectively.  The maximum sulfur content of the diesel fuel is expected to be 0.5%, by weight.  The three engines are not subject to 40 CFR 60, Subpart IIII – Standards of Performance for Stationary Compression Ignition Internal Combustion Engines.

[Rule 62-210.300(3)(a)36., F.A.C.]

#### Rule Applicability

The asphalt facility is classified as a synthetic minor, non-Title V source with annual throughput limited to a maximum of 480,000 tons/year of asphalt concrete and a facility-wide fuel oil usage limitation of 1.2 million gallons/year.  The sulfur content for all fuel oils shall not exceed one percent (1.0%).  Recordkeeping is required on exempt emission sources using fuel oil based on Rule 62-210.300(3)(c), F.A.C.  (See Specific Condition No. 32.)

The Asphalt Batch Plant (EU No. 001) is not subject to 40 CFR 60, Subpart I (“Standards of Performance for Asphaltic Concrete Plant”) since the plant was constructed prior to June 1973.  The RAP crushing unit (EU No. 002) is subject to the New Source Performance Standards (NSPS) requirements of 40 CFR 60, Subpart OOO  (“Standards of Performance for Nonmetallic Mineral Processing Plants”) and the general provisions of 40 CFR 60, Subpart A, where applicable, which are adopted by reference in Rule 62-204.800(8), F.A.C.

The facility is a synthetic minor, Non-Title V source for the pollutants particulate matter less than 10 microns (PM10), sulfur dioxide, carbon monoxide, nitrogen oxides, and hazardous air pollutants (HAPs).

Attachments to this Permit:

General Conditions

40 CFR 60, Subpart A:  General Provisions

40 CFR 60, Subpart OOO: Standards for Performance for Nonmetallic Mineral Processing Plants

### Facility Information Summary

Location:  4526 Maine Avenue, Lakeland, Polk County

UTM Coordinates:  Zone 17     415.26 East     3098.75 North

Latitude:  28°00'46"	Longitude:  81°52'14"

Facility ID No.:  1050387

| Emission Unit ID No. | Description |
| --- | --- |
| 001 | # Asphalt Batch Plant |
| 002 | # Reclaimed Asphalt Pavement Crusher with associated screening operations |
| 003 | # Asphalt Shingle Grinder |

### Permit History:  This permit replaces and modifies the terms and conditions of Construction Permit 1050387-003-AC.

NOTE: Please reference the Facility Name, Permit No., Facility ID, and Emission Unit ID in all correspondence, test report submittals, applications, etc.

SPECIFIC CONDITIONS:

- General Conditions:  A part of this permit is the attached 15 General Conditions.

[Rule 62-4.160, F.A.C.]

- Other Requirements:  Issuance of this permit does not relieve the permittee from complying with applicable emission limiting standards or other requirements of Chapters 62-4, 62-204, 62-210, 62-212, 62-213, 62-296, and 62-297, F.A.C. or any other requirements under federal, state, or local law.

[Rule 62-210.300, F.A.C.]

- New Source Performance Standards Applicability:  The RAP Crusher (EU ID No. 002) is subject to the attached requirements of 40 CFR 60 Subpart OOO - Standards of Performance for Nonmetallic Mineral Processing Plants and with the general provisions of 40 CFR 60, Subpart A, where applicable, which are adopted by reference in Rule 62-204.800(8), F.A.C.

[Rule 62-204.800(8)(b)64., F.A.C.]

#### Emission and Operating Limitations

- General Standards - Objectionable Odor:  No person shall not cause, suffer, allow, or permit the discharge of air pollutants, which cause or contribute to an objectionable odor.  An objectionable odor is any odor present in the outdoor atmosphere, which by itself or in combination with other odors, is or may be harmful or injurious to human health or welfare, which unreasonably interferes with the comfortable use and enjoyment of life or property, or which creates a nuisance.

[Rules 62-210.200 (Definition “Objectionable Odor”) and 62-296.320(2), F.A.C.]

- General Standards – Visible Emissions:  No person shall cause, let, permit, or allow to be discharged into the atmosphere the emissions of air pollutants from any activity, the density of which is equal to or greater than that designated as Number One on the Ringelmann Chart (20 percent opacity).

[Rule 62-296.320(4)(b)1., F.A.C.]

- Reasonable Precautions of Unconfined Emissions of Particulate Matter:  All reasonable precautions shall be taken to prevent and control generation of unconfined emissions of particulate matter (PM). These provisions are applicable to any source, including but not limited to, vehicular movement, transportation of materials, construction, alteration, demolition or wrecking, or industrial-related activities such as loading, unloading, storing, and handling.  Reasonable precautions to be taken shall include, but not be limited to the following:

  - Paving and maintenance of roads, parking areas, and yards;

  - Application of water or chemicals to control emissions from such activities as demolition of buildings, grading roads, construction, and land clearing;

  - Application of asphalt, water, oil, chemicals, or other dust suppressants to unpaved roads, yards, open stock piles, and similar activities;

  - Removal of PM from roads and other paved areas under the control of the owner or operator of the facility to prevent re-entrainment, and from building(s) or work areas to prevent particulate from becoming airborne;

  - Landscaping or planting of vegetation;

  - Sprinkling with water shall be used as necessary on paved areas, unpaved areas, stockpiles, and during loading/unloading operations;

  - Prevent emissions from material handling by a loader shall be achieved by reducing the material freefall as the bucket dumps the aggregate.  Drop heights shall be minimized from conveyors and screens; and

  - Water shall be applied to the crusher and transfer points, as necessary, to reduce unconfined PM.

If operating experience indicates that these reasonable precautions are not sufficient to control unconfined PM emissions, the Department reserves the right to require additional measures, such as recordkeeping for water truck gallons applied, etc.

[Rules 62-4.070(3), 62-210.300(3)(c)2.e., and 62-296.320(4)(c), F.A.C.; Construction Permit 1050387-003-AC]

- Fugitive Emissions Opacity Standard:  In order to provide reasonable assurance that the precautions and practices taken at the facility are adequate, all sources of unconfined PM not subject to 40 CFR 60, should not exceed five percent (5%) opacity. Exceedance of this limit shall not be considered a violation in and of itself, but an indication that additional control precautions and/or practices beyond those outlined in Specific Condition No. 6 may be necessary.

[Rule 62-4.070(3), F.A.C.; Construction Permit 1050387-003-AC]

- Modification:  Unless otherwise exempt by rule, the permittee shall not initiate any construction, reconstruction, or modification at the facility and shall not install/modify any pollution control device at the facility without obtaining prior authorization from the Department.  Modification is defined as any physical change or changes in the method of operations or addition to a facility that would result in an increase in the actual emissions of any air pollutant subject to air regulations, including any not previously emitted, from any emission unit or facility.  The permittee shall notify the Air Permitting Section of the Department’s Southwest District Office of any modifications and receive written approval from this office of the changes prior to implementing the changes.

[Rules 62-4.070(3), 62-210.200 (Definition “Modification”), and 62-210.300(1), F.A.C.]

- Circumvention of Control Equipment:  No person shall circumvent any air pollution control device, or allow the emission of air pollutants without the applicable air pollution control device operating properly.

[Rule 62-210.650, F.A.C.]

- Particulate Matter Emission Limitations:  PM emissions from the Asphalt Batch Plant (EU ID No. 001) shall not exceed 40.41 pounds per hour (lb/hour) (i.e. 60.62 tons per any consecutive 12-month period), based on a process input rate of 200 TPH.  At a process input rate less than 200 TPH but greater than 30 TPH, Equation 1) shall be used to determine allowable PM emissions.  At a process input rate less than or equal to 30 TPH, Equation 2) shall be used to determine allowable PM emissions.

Equation 1)	E = 17.31P0.16 (P > 30 TPH)

Equation 2)	E = 3.59P0.62 (P < 30 TPH)

where: 	E = emissions in pounds per hour (lbs/hr), and

P = process input weight rate in TPH

[Rule 62-296.320(4)(a)2., F.A.C.; Construction Permit No. 1050387-003-AC]

- Operational Limits:  This facility is limited to the following hours of operation and throughput:

| E.U. ID No. | #### Description | Maximum Hours of Operation [per any consecutive 12-month period] | Daily Average Maximum Throughput [TPH] | Maximum Throughput [tons per any consecutive 12-month period] |
| --- | --- | --- | --- | --- |
| 001 | Asphalt Batch Plant | 3000 | 200.0 | 480,000 |
| 002 | RAP Crusher | 1000 | 250.0 | 125,000 |
| 003 | Shingle Grinder * | 200 | 125.0 | 10,000 |

*  	Only virgin, non-asbestos shingles from the GAF Materials Corporation located at 5138 Madison Avenue, Tampa, Hillsborough County shall be received and processed.

[Rules 62-4.070(3) and 62-210.200 (Definition ‘Potential to Emit’), F.A.C.; Construction Permit No. 1050387-003-AC]

- Visible Emissions:  Visible Emissions (VE) from the facility shall be limited to:

| E.U. ID No. | Emission Point No. | Description | VE Limit (% Opacity) |
| --- | --- | --- | --- |
| 001 | 001 | Asphalt Batch Plant | < 20 |
| 002 | 001 | Outdoor storage pile, front-end loader to RAP hopper | < 20 |
| 002 | 002 | Screen to ground (fines) | 10* |
| 002 | 003 | Screen’s oversize conveyor belt to crusher | 15* |
| 002 | 004 | Crusher to crusher’s exit conveyor belt | 15* |
| 002 | 005 | Crusher exit conveyor to external screen feed hopper | 10* |
| 002 | 006 | External screen hopper to by belt to external screen | 10* |
| 002 | 007 | External screen to coarse conveyor belt | 10* |
| 002 | 008 | External screen to intermediate conveyor belt | 10* |
| 002 | 009 | External screen to fine conveyor belt | 10* |
| 002 | 010 | External screen coarse conveyor belt to coarse storage pile | < 20 |
| 002 | 011 | External screen intermediate conveyor belt to storage pile | < 20 |
| 002 | 012 | External screen fine conveyor belt to fine storage pile | < 20 |
| 003 | 001 | Shingle Grinder | < 20 |

*  	VE associated with the RAP Crusher (EU ID No. 002) are subject to 40 CFR 60, Subpart OOO:  The affected emission sources identified above are subject to the VE limit set forth in 40 CFR 60, Subpart OOO.

[40 CFR 60.672; Rules 62-204.800(8), 62-210.300(3)(c)2.f., and 62-296.320(4)(b)(1), F.A.C.]

- Fuel Limitations:  The asphalt plant dryer shall comply with the following limitations:

- Type of Fuel - The dryer is limited to using the following fuel(s):

  - Natural gas;

  - “On-specification” used fuel oil having a maximum sulfur content of 1.0%, by weight; and

  - New No. 5 fuel oil having a maximum sulfur content of 1.0%, by weight.

- Fuel Usage – The total combined fuel oil usage of the asphalt plant dryer and exempt emission activities that use fuel oil shall not exceed 1,200,000 gallons per any consecutive 12-month period.

(Permitting Note:  It may be necessary to have multiple fuel oil/used oil tanks to comply with these limitations under certain operational scenarios.)

- Used Oil Specifications: The permittee shall not burn “off-specification” used oil. For each delivery of “on-specification” reclaimed/used fuel oil, the vendor shall provide an analysis* documenting the fuel oil meets the following requirements of 40 CFR 761.20(e)(2) and (3) and 40 CFR 279.11 (July 1, 2004):

| Constituent/Property | Allowable Level |
| --- | --- |
| Arsenic | 5 parts per million (ppm) maximum |
| Cadmium | 2 ppm maximum |
| Chromium | 10 ppm maximum |
| Lead | 100 ppm maximum |
| Total Halogens | 1000 ppm maximum |
| Flash Point | 100 degrees Fahrenheit (oF) minimum |
| PCBs | Less than 2 ppm1 |

1Required to be capable of firing on-specification used fuel oil during startup and shutdown.

Firing of 2 ppm or greater PCB containing used fuel oil is prohibited.  Used fuel oil shall not be blended to meet this requirement.

* Copies of the analysis shall be maintained at the facility for a minimum of five (5) years and made available to the Department upon request.

[Rules 62-4.070(3), 62-210.300, and 62-710.210, F.A.C.]

- Fuel Sulfur Content Record:  In order to document the sulfur content of the fuel oil used at the facility, the permittee shall keep records on either vendor provided as-shipped analysis or on analysis of as-received samples taken at the plant.  The above records shall be maintained at the facility for a minimum of five (5) years and made available to the Department upon request.

[Rules 62-4.070(3) and 62-210.300(3), F.A.C.; Construction Permit 1050387-003-AC]

Test Methods and Procedures

Emission Unit No. 001

- Particulate Matter Emission and Visible Emission Test Frequency: In order to document compliance with Specific Condition Nos. 5, 10, and 12, the Asphalt Batch Plant (EU ID No. 001) shall be tested for PM and VE.  The tests from the baghouse of the asphalt batch plant dryer shall be performed during each federal fiscal year (October 1 – September 30).  The VE compliance test shall be conducted concurrently with the PM tests whenever possible.  All sources controlled by the baghouse shall be in operation during testing.  The required test report shall be submitted to the Air Compliance Section of the Department’s Southwest District Office as soon as practical, but no later than 45 days after each test is completed. The test report submitted shall meet all applicable requirements of Chapter 62-297, F.A.C.

[Rules 62-210.300(3)(c), 62-297.310(7)(a), and 62-297.310(8)(b), F.A.C.]

- Particulate Matter Test Method:  Compliance with the PM emission limitations specified in Specific Condition No. 10 shall be determined using EPA Methods 1, 2, 3, 4, and 5 or 5A contained in 40 CFR 60, Appendix A and adopted by reference in Chapter 62-297, F.A.C.  The minimum requirements for stack sampling facilities, source sampling and reporting, shall be in accordance with Chapter 62-297, F.A.C. and 40 CFR 60, Appendix A.

[Rules 62-210.300(3)(c) and 62-297.401, F.A.C.]

- Visible Emissions Test Method: Compliance with the VE limitations, specified in Specific Condition No. 12, shall be conducted using EPA Method 9 contained in 40 CFR 60, Appendix A and adopted by reference in Chapter 62-297, F.A.C.  The minimum requirements for stack sampling facilities, source sampling and reporting, shall be in accordance with Chapter 62-297, F.A.C. and 40 CFR 60, Appendix A.

[Rules 62-210.300(3)(c) and 62-297.401(9), F.A.C.]

- Method 9 Visible Emission Test:  The Method 9 VE compliance test(s) shall be conducted by a certified observer and shall be conducted concurrently with the PM performance test (whenever possible) and be a minimum of 30 minutes in duration.  The opacity test observation period shall include the period during which the highest opacity emissions can reasonably be expected to occur.  If a special VE compliance test for the hot oil heater or engine is requested by the Department, the test shall be at least 30 minutes in duration.

[Rules 62-210.300(3)(c), 62-297.310(4)(a), and 62-297.401(9), F.A.C.]

- Test Requirements:  The permittee shall comply with the following:

  - Testing of emissions of the asphalt batch plant dryer’s baghouse to show compliance shall be conducted within 90 - 100% of the maximum permitted process rate of 200 TPH.  A compliance test submitted at an operating rate less than 90% of the maximum permitted rate will automatically constitute an amended permit at the lesser rate plus ten percent (i.e. 110% of the lesser test rate) until another test, showing compliance at no less than that higher rate is submitted.  Once the unit is so limited, operation at higher capacities is allowed for no more than 15 consecutive days for the purpose of additional compliance testing to regain the authority to operate at the permitted capacity; these test results shall be submitted to the Air Compliance Section of the Southwest District of the Department within 45 days of testing.  Acceptance of the test by the Department will automatically constitute an amended permit at the higher rate plus ten percent (10%), up to the maximum permitted rate of 200 TPH for that EU. The actual rate (in TPH) of the EU for the test period shall be included in the test report for each test. Failure to submit the actual rate for the test period and a copy of the daily log (see Specific Condition No. 32) for the test day in the test report may invalidate the test and fail to provide reasonable assurance of compliance.

  - A compliance test submitted when the dryer is fired with natural gas will automatically constitute an amended permit to allow the dryer to be only fired with natural gas and up to 400 hours of firing fuel oil.  Within 30 days of exceeding the 400th hour of firing the dryer with fuel oil, new compliance tests shall be conducted with the dryer being fired with fuel oil.

  - A compliance test submitted when the dryer is fired with “on-specification” used fuel oil will automatically constitute an amended permit to allow the dryer to be only fired with “on-specification” used fuel oil and/or natural gas and up to 400 hours of firing residual No. 5 fuel oil.  Within 30 days of exceeding the 400th hour of firing the dryer with residual No. 5 fuel oil, new compliance tests shall be conducted with the dryer being fired with residual No. 5 fuel oil.

  - A compliance test submitted when the dryer is fired with residual No. 5 fuel oil will automatically constitute an amended permit to allow the dryer to be only fired with residual No. 5 fuel oil, “on-specification” used fuel oil, or natural gas.

  - Since the Department has determined processing only conventional materials has the greatest potential to generate emissions vs. RAP, a compliance test submitted when processing RAP shall limit the plant to processing only RAP.  If the plant is so limited, within 15 days upon processing virgin materials (conventional hot mix asphalt), new compliance tests (PM and VE) for the asphalt batch plant dryer’s baghouse shall be conducted using only virgin materials.  A compliance test submitted when processing only virgin materials shall also allow the plant to process RAP.

[Rules 62-4.070(3) and 62-297.310(2), F.A.C.]

- Test Report:  The following shall be submitted with any compliance test report for EU ID No. 001:

- Production rate of asphalt batch plant in tons/hr. during the compliance test, including a statement indicating whether virgin materials or RAP was used;

- Type of fuel used;

- Fuel oil analysis of the sulfur content of the fuel oil used;

- Used fuel oil analysis to document compliance with the “on-specification” used fuel oil limits, if “on-specification” used fuel oil was used to fire the dryer; and

- A copy of the records for the month the test was conducted as required by Specific Condition No. 32.

Failure to submit the items mentioned above with any compliance test report for the test period may invalidate the test(s).

[Rules 62-4.070(3) and 62-297.310(8), F.A.C.]

### Emission Unit No. 002

- Visible Emission Test Frequency:  In order to document compliance with Specific Condition No. 12, the RAP Crusher (EU ID No. 002), shall be tested for VE at Emission Points (EPs) 002, 003, 004, 005, 006, 007, 008, and 009 (See Specific Condition No. 12 for detailed description of EPs).  The tests shall be performed during each federal fiscal year (October 1 – September 30).  The required test reports shall be submitted to the Air Compliance Section of the Department’s Southwest District Office as soon as practical, but no later than 45 days after each test is completed. The test report submitted shall meet all applicable requirements of Rule 62-297, F.A.C.

(Permitting Note: The issuance of this construction permit does not alter the requirement from 1050387-003-AC Specific Condition No. 21 for the facility to have tested E.U. 002 for VE on or before November 1, 2008.)

[40 CFR 60.8(a); Rules 62-297.310(7)(a) and 62-297.310(8)(b), F.A.C.; Construction Permit 1050387-003-AC]

- Test Requirements:  Testing of each affected source shall be conducted within 90 - 100% of the maximum permitted process rate of 250 TPH.  A compliance test submitted at an operating rate less than 90% of the maximum permitted rate will automatically constitute an amended permit at the lesser rate plus ten percent (i.e. 110% of the lesser test rate) until another test, showing compliance at no less than that higher rate is submitted.  Once the unit is so limited, operation at higher capacities is allowed for no more than 15 consecutive days for the purpose of additional compliance testing to regain the authority to operate at the permitted capacity; these test results shall be submitted to the Air Compliance Section of the Southwest District of the Department within 45 days of testing.  Acceptance of the test by the Department will automatically constitute an amended permit at the higher rate plus ten percent (10%), up to the maximum permitted rate of 250 TPH for that EU. The actual rate (in TPH) of the EU for the test period shall be included in the test report for each test.  Failure to submit the actual rate for the test period and a copy of the daily log (see Specific Condition No. 33) for the test day in the test report may invalidate the test and fail to provide reasonable assurance of compliance.

[Rules 62-297.310(2) and 62-297.310(8), F.A.C.]

- Test Report:  The following shall be submitted with any compliance test report for EU ID No. 002:

  - The material processing rate, in tons/hr. during that period; and

  - A copy of both the daily record and the monthly record, required by Specific Condition No. 33, which includes the day the test was conducted.

Failure to submit the items mentioned above with any compliance test report for the test period may invalidate the test.

[Rules 62-4-070(3) and 62-297.310(8), F.A.C.]

Emission Unit No. 003

- Visible Emission Test Frequency:  In order to document compliance with Specific Condition No. 12, the shingle grinder (EU ID No. 003), shall be tested for VE.  The tests shall be performed as follows:

Initial Test - The initial compliance test shall be performed within 60 days after the effective date of this permit (See placard page).

Subsequent Tests - Each subsequent test shall be performed during each federal fiscal year (October 1 – September 30).

The required test report shall be submitted to the Air Compliance Section of the Department’s Southwest District Office as soon as practical, but no later than 45 days after each test is completed. The test report submitted shall meet all applicable requirements of Rule 62-297, F.A.C.

[Rules 62-4.070(3) and 62-297.310, F.A.C.]

- Test Requirements:  Testing of the grinder shall be conducted within 90 - 100% of the maximum permitted process rate of 125 TPH.  A compliance test submitted at an operating rate less than 90% of the maximum permitted rate will automatically constitute an amended permit at the lesser rate plus ten percent (i.e. 110% of the lesser test rate) until another test, showing compliance at no less than that higher rate is submitted.  Once the unit is so limited, operation at higher capacities is allowed for no more than 15 consecutive days for the purpose of additional compliance testing to regain the authority to operate at the permitted capacity; these test results shall be submitted to the Air Compliance Section of the Southwest District of the Department within 45 days of testing.  Acceptance of the test by the Department will automatically constitute an amended permit at the higher rate plus ten percent (10%), up to the maximum permitted rate of 125 TPH for that EU. The actual rate (in TPH) of the EU for the test period shall be included in the test report for each test.  Failure to submit the actual rate for the test period and a copy of the daily log (see Specific Condition No. 34) for the test day in the test report may invalidate the test and fail to provide reasonable assurance of compliance.

[Rules 62-297.310(2) and 62-297.310(8), F.A.C.]

- Test Report:  The following shall be submitted with any compliance test report for EU ID No. 003:

  - The material processing rate, in tons/hr. during that period; and

  - A copy of both the daily record and the monthly record, required by Specific Condition No. 34, which includes the day the test was conducted.

Failure to submit the items mentioned above with any compliance test report for the test period may invalidate the test.

[Rules 62-4-070(3) and 62-297.310, F.A.C.]

- Test Method - Visible Emissions:  VE testing for the process emission sources identified in Specific Condition No. 12 shall be conducted using EPA Method 9, and at least 30 minutes in duration.

[Rule 62-297.310(4), F.A.C.]

General Requirements for Testing Methods and Procedures

- Test Notification:  The owner or operator shall notify the Air Compliance Section of the Department’s Southwest District Office at least 15 days prior to the date on which each annual formal compliance test is to begin of the date, time, and place of each such test, and the test contact person who will be responsible for coordinating and having such test conducted.

[40 CFR 60.7(a)6.; Rule 62-297.310(7)(a)9., F.A.C.]

- Special Compliance Testing:  When the Department, after investigation, has good reason (such as complaints, increased VE, or questionable maintenance of control equipment) to believe that any applicable emission standard contained in a Department rule or in a permit issued pursuant to those rules is being violated, it may require the owner or operator of the emissions unit to conduct compliance tests which identify the nature and quantity of pollutant emissions from the emissions unit and to provide a report on the results of said tests to the Air Compliance Section of the Department’s Southwest District Office.

[Rule 62-297.310(7)(b), F.A.C.]

- Stack Sampling:  The permittee shall install and maintain stack sampling facilities (as required), including ports, work platforms, access to work platforms, electrical power, and sampling equipment support.  All stack sampling facilities must meet any Occupational Safety and Health Administration (OSHA) Safety and Health Standards described in 29 CFR Part 1910, Subparts D and E.

[Rule 62-297.310(6), F.A.C.]

- Required Equipment:  The owner or operator of an EU for which compliance tests are required shall install, operate, and maintain equipment or instruments necessary to determine process variables, such as process weight input or heat input, when such data are needed in conjunction with emission data to determine the compliance of emissions units with applicable emission limiting standards.  Equipment or instruments used to directly or indirectly determine process variables, including devices such as belt scales, weigh hoppers, flow meters, and tank scales, shall be calibrated and adjusted to indicate the true value of the parameter being measured with sufficient accuracy to allow the applicable process variable to be determined within ten percent (10%) of its true value.  The permittee, in accordance with the manufacturer’s specifications, shall operate a properly calibrated and maintained device for pressure drop across the dryer baghouse associated with EU No. 001.

[Rules 62-210.650 and 62-297.310(5), F.A.C.; Construction Permit 1050387-003-AC]

Monitoring and Recordkeeping Requirements

- Recordkeeping for EU ID No. 001 and Exempt Emission Units:  In order to demonstrate compliance with Specific Condition Nos. 11, 13, and 19 for the asphalt batch plant, the hot oil heater, the diesel engine that powers the exhaust fan, the diesel engine that powers the RAP Crusher, and the diesel engine that powers the shingle grinder; the permittee shall comply with the following recordkeeping requirements:

Daily Records: Daily, record the following:

    - The pressure drop across the baghouse during normal operation (inches of water);

    - Total asphalt concrete production (tons);

    - Hours of operation while producing asphalt concrete;

    - A calculation of the daily average (TPH) total production rate of asphalt concrete

  - Monthly Records:  Monthly, record the following:

    - Total tons of virgin-, RAP-, and shingle-based asphalt concrete produced for the month (tons), and a calculation of the most recent consecutive 12-month rolling total asphalt production (tons/ consecutive 12-month period);

    - Total operating hours of the asphalt batch plant’s dryer for the month (hours), and a calculation of the most recent consecutive 12-month rolling total dryer operating hours (hours/consecutive 12-month period);

    - The quantity of each of the three (3) types of fuel used to fire the asphalt batch plant’s dryer (EU No. 001);

    - The total usage of all fuel oil, in gallons, used to fire the asphalt batch plant’s dryer, hot oil heater, and the four (4) diesel engines; and

    - The most recent consecutive 12-month rolling total fuel oil usage, gallons, of all fuel oil used to fire the asphalt batch plant’s dryer, hot oil heater, and the four (4) diesel engines.

Additional Fuel Oil Usage Recordkeeping:

    - If the last compliance test was conducted while the dryer was fired with “on-specification” fuel oil, the permittee shall keep a daily record of dryer operating hours while firing residual No. 5 fuel oil, along with the cumulative total of residual No. 5 fuel oil operating hours since the last dryer compliance test.

(Permitting Note:  When residual No. 5 fuel oil dryer operating hours since the last compliance test exceed 400 hours, then additional compliance testing is required (see Specific Condition No. 19).)

    - If the last compliance test was conducted while the dryer was fired with natural gas, the permittee shall keep a daily record of dryer operating hours while firing residual No. 5 fuel oil or “on-specification” fuel oil, along with the cumulative total of residual No. 5 fuel oil and “on-specification” fuel oil operating hours since the last dryer compliance test.

(Permitting Note:  When residual No. 5 fuel oil and/or “on-specification” fuel oil dryer operating hours since the last compliance test exceed 400 hours, then additional compliance testing is required (see Specific Condition No. 19).)

All records shall include the Facility Name, Facility ID, Date, Time, and Operator Initials for each measurement.  The monthly logs shall be completed by the end of the 15th day of the following month.  Daily logs (if required) shall be completed within three (3) business days.  These records shall be available to the Department upon request.

[Rule 62-4.070(3), F.A.C.]

- Recordkeeping for EU ID No. 002:  In order to demonstrate compliance with Specific Condition No. 11 for the RAP Crusher, the permittee shall comply with the following record keeping requirements:

Daily Process Records: Daily record the following:

    - Hours of operation of the RAP crushing unit;

    - Quantity of material processed (tons); and

    - A calculation of the daily average (TPH) processing rate based on (I.) and (II.) above.

Monthly Process Records: Monthly record the following:

    - The total amount of material processed for the month (tons);

    - The most recent consecutive 12-month rolling total of material processed (tons/consecutive 12-months);

    - The month’s total hours of operation of the RAP crushing unit; and

    - The most recent consecutive 12-month rolling total operating hours of the RAP crushing unit.

All records shall include the Facility Name, Facility ID, Date, Time, and Operator Initials for each measurement.  The monthly logs shall be completed by the end of the 15th day of the following month.  Daily logs (if required) shall be completed within three (3) business days.  These records shall be available to the Department upon request.

[Rule 62-4.070(3), F.A.C.; Construction Permit 1050387-003-AC]

- Recordkeeping for EU ID No. 003:  In order to demonstrate compliance with Specific Condition No. 11, the permittee shall comply with the following record keeping requirements:

Daily Process Records: Daily record the following:

    - Hours of operation of the shingle grinder;

    - Quantity of material processed (tons); and

    - A calculation of the daily average (TPH) processing rate based on (I.) and (II.) above.

Monthly Process Records: Monthly record the following:

    - The total amount of material processed for the month (tons);

    - The most recent consecutive 12-month rolling total of material processed (tons/consecutive 12-months);

    - The month’s total hours of operation of the shingle grinder; and

    - The most recent consecutive 12-month rolling total operating hours of the shingle grinder.

- For each load of shingles received:

- Facility Name, Facility ID No. (i.e. 1050387), Emission Unit ID No. (i.e.003);

- The date of delivery;

- The amount of non-asbestos containing shingles received, in tons; and

- A certification signed and dated by the facility’s plant manager that contains the statement, “The non-asbestos containing shingles received are only from the GAF Materials Corporation located at 5138 Madison Avenue, Tampa, Hillsborough County.”

All records shall include the Facility Name, Facility ID, Date, Time, and Operator Initials for each measurement.  The monthly logs shall be completed by the end of the 15th day of the following month.  Daily logs (if required) shall be completed within three (3) business days.  These records shall be available to the Department upon request.

[Rule 62-4.070(3), F.A.C.]

- Recordkeeping - Reasonable Precautions:  In order to demonstrate compliance with Specific Condition No. 6, the permittee shall record the Facility Name, Facility ID, Location, Date, Time, and Operator initials when water application or sweeping is used to control unconfined emissions of PM from the facility.

[Rule 62-4.070(3), F.A.C.]

- Record Retention:  All recordkeeping information, supporting documentation, monitoring data, and all compliance test information required by this permit shall be maintained on-site or electronically for at least five (5) years and be made available to the Department for inspection upon request.

[Rules 62-4.070(3) and 62-210.300(3)(c), F.A.C.]

- Excess Emissions:  Excess emission resulting from startup, shutdown, or malfunction of any emission unit shall be permitted providing:  best operational practices to minimize emissions are adhered to and the duration of excess emissions shall be minimized but in no case exceed two (2) hours in any 24-hour period unless specifically by the Department for longer duration.  Excess emissions which are caused entirely or in part by poor maintenance, poor operation, or any other equipment or process failure which may reasonably be prevented during startup, shutdown, or malfunction shall be prohibited.

[Rule 62-210.700(1) and (4), F.A.C.]

(Permitting Note:  The Excess Emission Rule at Rule 62-210.700, F.A.C. and included in Specific Condition No. 37 cannot vary any requirement of a New Source Performance Standard.)

- Excess Emissions - Reporting:  In the case of excess emissions resulting from malfunctions, each owner or operator shall notify the Department in accordance with Rule 62-4.130, F.A.C.  A full written report on the malfunctions shall be submitted in a quarterly report, if requested by the Department.

[Rule 62-210.700(6), F.A.C.]

- Startup/Shutdown/Malfunction:  For EU 002, the permittee shall maintain records of the occurrence and duration of any startup, shutdown, or malfunction in the operation of an affected facility; or any malfunction of the air pollution control equipment.  The records shall be recorded in a permanent form suitable for inspection and shall be retained for at least five (5) years.

[40 CFR 60.7(b); Rule 62-204.800(8), F.A.C.]

Prohibited Operations

- Asbestos Containing Materials:  This facility shall not process Asbestos Containing Materials (ACM), whether regulated asbestos containing material (RACM), category I or category II, and whether friable or non-friable when received at the facility.

  - “Asbestos” means the asbestiform varieties of serpentinite (chrysotile), riebeckite (crocidolite), cummingtonite-grunerite, anthophyllite, and actinolite-tremolite and includes trade acronyms products such as amosite.

  - “Asbestos-containing materials”, ACM, means any materials that contain more than one percent (1%) asbestos as determined by Polarized Light Microscopy. Based on a representative composite sample.

  - “Asbestos removal project” means renovation or demolition operation in a facility that involves the removal of a threshold amount of regulated asbestos-containing material.

  - “Category I Nonfriable Asbestos-Containing Material (ACM)” means asbestos-containing packings, gaskets, resilient floor covering, and asphalt roofing products containing more than 1 percent (1%) asbestos as determined using the method specified in Appendix A, Subpart F, 40 CFR Part 763, Section 1, Polarized Light Microscopy.

  - “Category II Non-friable ACM” means any material, excluding Category I Nonfriable ACM, containing more than 1 percent (1%) asbestos as determined using the methods specified in Appendix A, Subpart F, 40 CFR Part 763, Section 1, Polarized Light Microscopy, that, when dry, cannot be crumbled, pulverized, or reduced to powder by hand pressure.

[40 CFR 61, Subpart M; Chapter 62-257, F.A.C.; Rules 62-701.520 and 62-730.300, F.A.C.]

Annual Operating Report and Permit Requirement

- Annual Operating Report:  The permittee shall submit to the Air Compliance Section of the Department’s Southwest District Office each calendar year on or before April 1, completed DEP Form 62-210.900(5), “Annual Operating Report for Air Pollutant Emitting Facility” (AOR) for the preceding calendar year.  Note, the annual operating report for calendar year 2008 shall be submitted by May 1, 2009.  The report may be submitted electronically in accordance with the instructions received with the AOR package sent by the Department, or a hardcopy may be submitted.

[Rules 62-210.300(3)(c)2.h. and 62-210.370(3)(a), F.A.C.]

- Operation Permit Application Requirement: The permittee’s previously submitted air pollution permit application (DEP File No. 1050387-002-AO) is incomplete.  In order to complete the previously submitted application, the permittee shall submit to the Air Permitting Section of the Department’s Southwest District Office the initial VE test reports required by Construction Permit 1050387-003-AC, Specific Condition No. 21 and Construction Permit 1050387-004-AC (this permit), Specific Condition No. 24.

[Rules 62-4.030, 62-4.050, 62-4.070(3), 62-210.300(2), 62-210.900, and 62-297.310(7), F.A.C.]

STATE OF FLORIDA DEPARTMENT

OF ENVIRONMENTAL PROTECTION

DRAFT

______________________________

Mara Grace Nasca

District Air Program Administrator

###### Southwest District