TECHNICAL EVALUATION

&

PRELIMINARY DETERMINATION

APPLICANT

Firmenich, Inc.

4330 Drane Field Road

Lakeland, Florida 33811

Lakeland Facility

Facility ID No. 1050409

PROJECT

Project No. 1050409-014-AC

Application for Minor Source Air Construction Permit

Increase HAPs Allowable Emissions and Construction of two Process Tanks

COUNTY

Polk County, Florida

PERMITTING AUTHORITY

Florida Department of Environmental Protection

Air and Solid Waste Permitting

Southwest District Office

Temple Terrace, Florida 33637-0926

August 19, 2015

Prepared by Danny Stubbs

- General Project INFORMATION

Air Pollution Regulations

Projects at stationary sources with the potential to emit air pollution are subject to the applicable environmental laws specified in Section 403 of the Florida Statutes (F.S.).  The statutes authorize the Department of Environmental Protection (Department) to establish regulations regarding air quality as part of the Florida Administrative Code (F.A.C.), which includes the following applicable chapters:  62-4 (Permits); 62-204 (Air Pollution Control – General Provisions); 62-210 (Stationary Sources – General Requirements); 62-212 (Stationary Sources – Preconstruction Review); 62-213 (Operation Permits for Major Sources of Air Pollution); 62-296 (Stationary Sources - Emission Standards); and 62-297 (Stationary Sources – Emissions Monitoring).  Specifically, air construction permits are required pursuant to Chapters 62-4, 62-210 and 62-212, F.A.C.

In addition, the U. S. Environmental Protection Agency (EPA) establishes air quality regulations in Title 40 of the Code of Federal Regulations (CFR).  Part 60 specifies New Source Performance Standards (NSPS) for numerous industrial categories.  Part 61 specifies National Emission Standards for Hazardous Air Pollutants (NESHAP) based on specific pollutants.  Part 63 specifies NESHAP based on the Maximum Achievable Control Technology (MACT) for numerous industrial categories.  The Department adopts these federal regulations in Rule 62-204.800, F.A.C.

Glossary of Common Terms

Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of this permit.

Facility Description and Location

Firmenich, Inc. is an existing juice and beverage ingredient production facility, which is categorized under Standard Industrial Classification Code No. 2037.  The existing facility is located in Polk County at 4330 Drane Field Road in Lakeland, Florida.  The UTM coordinates of the existing facility are Zone 17, 393.2 km East, and 3097.1 km North. This site is in an area that is in attainment (or designated as unclassifiable) for all air pollutants subject to Ambient Air Quality Standards (AAQS).

Facility Regulatory Categories

- The facility is not a major source of hazardous air pollutants (HAP).

- The facility is not a Title V major source of air pollution in accordance with Chapter 62-213, F.A.C.

- The facility is not a major stationary source in accordance with Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

- This facility is a synthetic non-Title V source for the pollutants volatile organic compounds (VOC) and hazardous air pollutants (HAPs).

- Project Description

This project will increase the Total HAPs allowable emissions for Emissions Unit No. 001 from 9.5 to 24.5 tons per year and establish Individual HAP allowable emissions of 9.5 tons per year.

This project is also for the construction of two process tanks in the facility’s Storage and Blending Tanks emissions unit (EU ID No. 001).  Each tank will have a capacity of approximately 640 gallons and will be used to process citrus oils.  See “Appendix E – Tanks Summary” for a detailed summary of the tanks to be constructed.  {Note: This project will replace Construction Permit No. 1050409-013-AC and incorporates the construction of the two process tanks originally included in that permit.}

- Processing Schedule

7-27-2015	Received the application for a minor source air pollution construction permit; application complete.

- PSD Applicability

The project is located in Polk County which is in an area that is currently in attainment with the AAQS or is otherwise designated as unclassifiable.  The facility is not a PSD major source and therefore the proposed project is not subject to a PSD preconstruction review.

- Department review

- This project will increase the Total HAPs allowable emissions for Emissions Unit No. 001 from 9.5 to 24.5 tons per year and establish an Individual HAP allowable emissions of 9.5 tons per year.  This emissions unit include 95 storage and blending tanks that are sources of uncontrolled VOC and HAP emissions.

- List of Emissions Units

| EU ID No. | Emissions Unit Description |
| --- | --- |
| 001 | Storage and Blending Tanks |

- Summary of Emissions

| Pollutant | EU No. | Potential Emissions (tpy) | Allowable Emissions (tpy) |
| --- | --- | --- | --- |
| VOC | 001 | 37.0 | 37.0 |
| Total HAPs | 001 | 24.5 | 24.5 |
| Individual HAPs | 001 | 9.5 | 9.5 |

Note: The VOC, Individual HAPs and Total HAP allowable emissions listed above were requested in the permit application submitted on 7/27/2015.  The VOC, Individual HAPs and total HAPs potential emissions were set based on their corresponding allowable emissions.

| Visible Emissions |  |  |
| --- | --- | --- |
| EU No. | Emission Limiting Standard (% Opacity) | Applicable Rule |
| Facility | 20 % | Rule 62-296.320(4)(b), F.A.C |

- State Requirements and Federal Regulations

This project is subject to the preconstruction review requirements of Chapter 403, Florida Statutes and Chapters 62-204 through 62-297, Florida Administrative Code (F.A.C.), as indicated below.

| Subject to: | Y/N | Comments |
| --- | --- | --- |
| Rule 62-210.300, F.A.C. – Permits Required | Y | subject to general permitting requirements |
| Rule 62-212.400, F.A.C. - Prevention of Significant Deterioration | N | facility is not a PSD major source |
| Rule 62-296.320(4), F.A.C. - General Particulate Emission Limiting Standards | Y | project could be a source of unconfined particulate matter emissions |
| Rules 62-296.320(1) and (2), F.A.C. - General Pollutant Emission Limiting Standards (VOCs and Odor) | Y | project is a source of VOCs, could also be a source of odor |
| Rule 62-296.401-418, F.A.C. - Stationary Source Emission Standards | N | there is no applicable source category |
| Rule 62-296.500, F.A.C. - Reasonably Available Control Technology (VOC) | N | Polk County is not an air quality maintenance area for ozone |
| Rule 62-296.700, F.A.C. - Reasonably Available Control Technology (PM) | N | there is no applicable source category |
| Rule 62-204.800, F.A.C. - Standards of Performance for New Stationary Sources (NSPS) | N | there is no applicable source category |
| Rule 62-204.800, F.A.C. National Emission Standard for Hazardous Air Pollutants (NESHAPS – 40 CFR 61) | N | there is no applicable source category |
| Rule 62-204.800, F.A.C. National Emission Standard for Hazardous Air Pollutants for Source Categories a.k.a. MACT (NESHAPS –40 CFR 63) | N | there is no applicable source category |
| Chapter 62-213, F.A.C. - Operation Permits for Major Sources of Air Pollution | N | facility is a synthetic non-Title V source |
| Rule 62-297.310, F.A.C. - General Compliance Test Requirements, F.A.C. | N | Testing is not required. Compliance is demonstrated via recordkeeping. |

- Preliminary Determination

The Department makes a preliminary determination that the proposed project will comply with all applicable state and federal air pollution regulations as conditioned by the draft permit.  This determination is based on a technical review of the complete application, reasonable assurances provided by the applicant, and the conditions specified in the draft permit.  No air quality modeling analysis is required because the project does not result in a significant increase in emissions.  Danny Stubbs is the project engineer responsible for reviewing the application and drafting the permit.  Additional details of this analysis may be obtained by contacting the project engineer at the Department’s Air and Solid Waste Permitting Section in the Southwest District Office, 13051 North Telecom Parkway, Temple Terrace, Florida 33637-0926, phone 813-470-5729, or by email danny.stubbs@dep.state.fl.us.