TECHNICAL EVALUATION

&

PRELIMINARY DETERMINATION

APPLICANT

Peace Valley Products

3225 Highway 630 West

Fort Meade, FL 33841

Facility ID No. 1050454

PROJECT

Project No. 1050454-004-AC

Application for Minor Source Air Construction Permit

Project Name: Facility relocation and increase in production

COUNTY

Polk County, Florida

PERMITTING AUTHORITY

Florida Department of Environmental Protection

Air and Solid Waste Permitting

Southwest District Office

13051 North Telecom Parkway, Suite 101

Temple Terrace, Florida 33637-0926

June 23,2017

Prepared by Diana Felix

- General Project INFORMATION

Air Pollution Regulations

Projects at stationary sources with the potential to emit air pollution are subject to the applicable environmental laws specified in Section 403 of the Florida Statutes (F.S.).  The statutes authorize the Department of Environmental Protection (Department) to establish regulations regarding air quality as part of the Florida Administrative Code (F.A.C.), which includes the following applicable chapters:  62-4 (Permits); 62-204 (Air Pollution Control – General Provisions); 62-210 (Stationary Sources – General Requirements); 62-212 (Stationary Sources – Preconstruction Review); 62-213 (Operation Permits for Major Sources of Air Pollution); 62-296 (Stationary Sources - Emission Standards); and 62-297 (Stationary Sources – Emissions Monitoring).  Specifically, air construction permits are required pursuant to Chapters 62-4, 62-210 and 62-212, F.A.C.

In addition, the U. S. Environmental Protection Agency (EPA) establishes air quality regulations in Title 40 of the Code of Federal Regulations (CFR).  Part 60 specifies New Source Performance Standards (NSPS) for numerous industrial categories.  Part 61 specifies National Emission Standards for Hazardous Air Pollutants (NESHAP) based on specific pollutants.  Part 63 specifies NESHAP based on the Maximum Achievable Control Technology (MACT) for numerous industrial categories.  The Department adopts these federal regulations in Rule 62-204.800, F.A.C.

Glossary of Common Terms

Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of this permit.

Facility Description and Location

Peace Valley Products is an existing fertilizer recycling facility, which is categorized under Standard Industrial Classification Code No. 2875.  The new site for Peace Valley Products will be located in Polk County at 3225 Highway 630 West in Fort Meade, Florida.  The UTM coordinates of the new facility are Zone 17, 415.9 km East and 3069.2 km North.  This site is in an area that is in attainment (or designated as unclassifiable) for all air pollutants subject to Ambient Air Quality Standards (AAQS).

Facility Regulatory Categories

- The facility is not a major source of hazardous air pollutants (HAP).

- The facility is not a Title V major source of air pollution in accordance with Chapter 62-213, F.A.C.

- The facility is not a major stationary source in accordance with Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

- This facility is a natural minor source of air pollution.

- Project Description

- Peace Valley Products has been in operation of an off-spec fertilizer recycling facility since 2015. Due to growth and surplus of raw materials, they are proposing the relocation of their operation to a larger site and an increase in production. The existing facility is currently located at 800 Pembroke Road in Fort Meade, Florida. This project will relocate the facility’s existing equipment from the existing location to the new warehouse location at 3225 Highway 630 West in Fort Meade, Florida.

The process will be changed from three systems to two systems as described below. The original process had a separate system for the grinding circuit with baghouse of oversized materials, the new process will incorporate this as a grinding loop within the dryer system. The overall process will include drying, screening, grinding and conveying of the raw materials to produce final products which will include semi-granular and granular phosphate products. The semi-granular product will be sold to NPK industry as feed stock and the granular product will be sold as granular fertilizer in the retail market.  This facility is a natural minor source of particulate matter (PM).

- Dryer System – This system dries raw feed materials and screens it into two grades; semi-granular, granular.  Damp raw material with a moisture content between five and seven percent is fed to the dryer feed conveyor (via a front-end loader and Feed Hopper #1) which discharges into a granulator which feeds the dryer. The dryer is a 54-inch diameter by 33 feet long standard rotary dryer that operates with co-current airflow.  The dryer is fired with propane and has a design heat input rate of 3 million Btu per hour.

  - The air stream from the dryer is vented through a cyclone and then through a venturi scrubber system which discharges to the atmosphere from a stack 25 feet tall and 20 inches at the exit diameter.

  - Dry material from the dryer is discharged onto a conveyor, into the dryer discharge elevator and then to the Rotex Screen.  The Rotex Screen is an enclosed system that separates the material into three streams.

    - The semi granular will be transferred directly into an enclosed bin.

    - The granular will be de-dusted in a ribbon mixer and transferred to storage through a conveyor system.

    - The oversize material will be transferred to a grinding circuit. The grinding circuit consists of a chain mill that is enclosed the discharge is feed back into the dryer discharge elevator and becomes one of the other two grades

  - Fugitive emissions from the screen, the transfer points from the dryer discharge and the grinding circuit are vented through a clean air combustion air system vented through a baghouse (Baghouse #1) and then transferred into the dryer combustion chamber for quench air.  Quench air is required to control inlet gas temperature as well as supplying the volume of air to remove moisture from the dried product.

- Shipping System– This system is used to loads final products into trucks for shipping.  The system consists of a hopper (Feed Hopper #2), a feed conveyor, an enclosed oversize screen, a loadout conveyor, an enclosed ribbon mixer (used for applying a de-dusting agent) and a discharge conveyor to discharging final products into trucks.

  - Fugitive emissions from the feed hopper, oversize screen, loadout conveyor and ribbon mixer are vented through a baghouse (Baghouse #2) which discharges into the enclosed warehouse.

  - The de-dusting agent used contains some VOC; however, the undiluted de-dusting agent consist of a hydrotreated naphthenic petroleum distillate similar to motor oil. Because motor oil has a low vapor pressure, the VOC emissions from the system are negligible. It should not evaporate and will be retained on the product.

Notes: The permit application dated June 20, 2017 includes a process flow diagram of the operation described above.

- Processing Schedule

June 20,2017	Received the application for a minor source air pollution construction permit.

- PSD Applicability

The project is located in Polk County which is in an area that is currently in attainment with the AAQS or is otherwise designated as unclassifiable.  The facility is not a PSD major source and therefore the proposed project is not subject to a PSD preconstruction review.

- Department review

- List of Emissions Units

| EU ID No. | Emissions Unit Description |
| --- | --- |
| 001 | Off-Spec Fertilizer Recycling Operation |

- Summary of Emissions

| Pollutant | EU No. | Potential Emissions (tpy) | Allowable Emissions (tpy) |
| --- | --- | --- | --- |
| PM | 001-Dryer System | 1.57 | N/A |
|  | 001-Shipping System | 0.102 | N/A |
|  | Total | 1.67 | N/A |
| VOC | 001-Dryer System | 0.15 | N/A |
|  | Total | 0.15 | N/A |
| NOx | 001-Dryer System | 1.89 | N/A |
|  | Total | 1.89 | N/A |
| SO2 | 001-Dryer System | 0.003 | N/A |
|  | Total | 0.003 | N/A |
| CO | 001-Dryer System | 1.09 | N/A |
|  | Total | 1.09 | N/A |

Note: The potential emissions calculations shown above are based on the following conditions: (1) The Dryer System’s maximum annual throughput is 87,600 tons/year and potential emissions are based on emissions from the discharge of the scrubber control device which is located downstream of the facility’s process dryer and dryer cyclone. (2) The Shipping System’s maximum annual throughput is 25,000 tons/year and the fugitive dust collector is operational.

| Visible Emissions |  |  |
| --- | --- | --- |
| EU No. | Emission Limiting Standard (% Opacity) | Applicable Rule |
| 001 | <20 Percent Opacity | Rule 62-296.320(4)b, F.A.C |

- State Requirements and Federal Regulations

This project is subject to the preconstruction review requirements of Chapter 403, Florida Statutes and Chapters 62-204 through 62-297, Florida Administrative Code (F.A.C.), as indicated below.

| Subject to: | Y/N | Comments |
| --- | --- | --- |
| Rule 62-210.300, F.A.C. – Permits Required | Y | subject to general permitting requirements |
| Rule 62-212.400, F.A.C. - Prevention of Significant Deterioration | N | This facility is not a PSD major source |
| Rule 62-296.320(4), F.A.C. - General Particulate Emission Limiting Standards | Y | This facility is a source of unconfined particulate matter emissions. |
| Rules 62-296.320(1) and (2), F.A.C. - General Pollutant Emission Limiting Standards (VOCs and Odor) | Y | De dusting agent may be a source of VOC and odors |
| Rule 62-296.401-418, F.A.C. - Stationary Source Emission Standards | N | There is no applicable source category. |
| Rule 62-296.500, F.A.C. - Reasonably Available Control Technology (VOC) | N | Polk County is not an air quality maintenance area for ozone. |
| Rule 62-296.700, F.A.C. - Reasonably Available Control Technology (PM) | N | This is a new source and therefore not subject to PM RACT rule. |
| Rule 62-204.800, F.A.C. - Standards of Performance for New Stationary Sources (NSPS) | N | There is no applicable source category. |
| Rule 62-204.800, F.A.C. National Emission Standard for Hazardous Air Pollutants (NESHAPS – 40 CFR 61) | N | There is no applicable source category. |
| Rule 62-204.800, F.A.C. National Emission Standard for Hazardous Air Pollutants for Source Categories a.k.a. MACT (NESHAPS –40 CFR 63) | N | There is no applicable source category. |
| Chapter 62-213, F.A.C. - Operation Permits for Major Sources of Air Pollution | N | This facility is a natural minor source of air pollution. |
| Rule 62-297.310, F.A.C. - General Compliance Test Requirements, F.A.C. | N | Testing is not required. Compliance is demonstrated via recordkeeping. |

- Preliminary Determination

The Department makes a preliminary determination that the proposed project will comply with all applicable state and federal air pollution regulations as conditioned by the draft permit.  This determination is based on a technical review of the complete application, reasonable assurances provided by the applicant, and the conditions specified in the draft permit.  No air quality modeling analysis is required because the project does not result in a significant increase in emissions.  Diana Felix is the project engineer responsible for reviewing the application and drafting the permit.  Additional details of this analysis may be obtained by contacting the project engineer at the Department’s Air and Solid Waste Permitting Section in the Southwest District Office, 13051 North Telecom Parkway, Suite 101, Temple Terrace, Florida 33637-0926, phone 813-470-5708, or by email diana.felix@dep.state.fl.us.