TECHNICAL EVALUATION

&

PRELIMINARY DETERMINATION

APPLICANT

Maverick Boat Company, Inc.

3207 Industrial 29th Street

Ft. Pierce, Florida  34946

Maverick Boat Company

Facility ID No. 1110086

PROJECT

Project No. 1110086-010-AC

Application for Minor Source Air Construction Permit

VOC Production Increase Concurrent Construction Permit

COUNTY

St. Lucie County, Florida

PERMITTING AUTHORITY

Florida Department of Environmental Protection

Southeast District Office

3301 Gun Club Road, MSC 7210-1

West Palm Beach, Florida  33406

June XX, 2017

- General Project INFORMATION

Air Pollution Regulations

Projects at stationary sources with the potential to emit air pollution are subject to the applicable environmental laws specified in Section 403 of the Florida Statutes (F.S.).  The statutes authorize the Department of Environmental Protection (Department) to establish regulations regarding air quality as part of the Florida Administrative Code (F.A.C.), which includes the following applicable chapters:  62-4 (Permits); 62-204 (Air Pollution Control – General Provisions); 62-210 (Stationary Sources – General Requirements); 62-212 (Stationary Sources – Preconstruction Review); 62-213 (Operation Permits for Major Sources of Air Pollution); 62-296 (Stationary Sources - Emission Standards); and 62-297 (Stationary Sources – Emissions Monitoring).  Specifically, air construction permits are required pursuant to Chapters 62-4, 62-210 and 62-212, F.A.C.

In addition, the U. S. Environmental Protection Agency (EPA) establishes air quality regulations in Title 40 of the Code of Federal Regulations (CFR).  Part 60 specifies New Source Performance Standards (NSPS) for numerous industrial categories.  Part 61 specifies National Emission Standards for Hazardous Air Pollutants (NESHAP) based on specific pollutants.  Part 63 specifies NESHAP based on the Maximum Achievable Control Technology (MACT) for numerous industrial categories.  The Department adopts these federal regulations in Rule 62-204.800, F.A.C.

Glossary of Common Terms

Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of this permit.

Facility Description and Location

Maverick Boat Company (MBC) is an existing Boat Building and Repairing Facility, which is categorized under Standard Industrial Classification Code No. 3732 (NAICS No. 336612).  The MBC facility was constructed in 2005 and is located on approximately 12 acres of land in northern St. Lucie County.  The existing facility is operated and owned by Maverick Boat Company, Inc.  Figure 1 shows the general location of the facility, and Figure 2 shows a satellite view.

Figure 1.  Location of the MBC.		Figure 2.  Aerial View of the MBC.

The existing MBC is located in St. Lucie County at 3207 Industrial 29th Street in Ft. Pierce, Florida.  The UTM coordinates of the existing facility are Zone 17, 563.06 km East, and 3,040.61 km North.  This site is in an area that is in attainment (or designated as unclassifiable) for all air pollutants subject to Ambient Air Quality Standards (AAQS).

Facility Regulatory Categories

- The facility is a major source of hazardous air pollutants (HAP).

- The facility is a Title V major source of air pollution in accordance with Chapter 62-213, F.A.C.

- The facility is not a major stationary source in accordance with Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

- The facility operates units subject to the NESHAP of 40 CFR 63, Subparts A – General Provisions and VVVV – NESHAP for Boat Manufacturing.

- Project Description

- The applicant proposes the construction of a new boat production building located at 4551 St. Lucie Blvd., approximately one (1) mile from the existing facility to expand the lamination and assembly capacity of the plant.  The proposed location is shown in Figure 3; the UTM coordinates of the proposed site are Zone 17, 562.02 km East and 3,040.00 km North.  The applicant is requesting an increase of the facility-wide emission limit for volatile organic compounds (VOC) from 99 tons per year (TPY) to 220 TPY.

- Figure 3.  Proposed new building in relation to existing Maverick facility.

- Processing Schedule

May 2, 2017	Received the application for a minor source air pollution construction permit to be processed concurrently with a Title V air operation permit revision.

May 8, 2017	Requested additional information.

June 16, 2017	Received additional information; application complete.

- PSD Applicability

The project is located in St. Lucie County which is in an area that is currently in attainment with the AAQS or is otherwise designated as unclassifiable.

The existing facility is not a major source according to Chapter 62-212.400(PSD), F.A.C., and this project alone does not constitute a major modification because it is below the 250 TPY increment for VOC emissions for sources not on the list of 28 given in 62-212.400, F.A.C.  The proposed project will not violate the emissions increment for any PSD pollutant; therefore, the project is not subject to a PSD preconstruction review.

- Department review

Discussion of proposed project, equipment, controls etc.

The applicant proposes the construction of a new boat production building located at 4551 St. Lucie Blvd., approximately one (1) mile from the existing facility to expand the lamination and assembly capacity of the plant.  According to information provided by the applicant, boat building activities at the new production line will be identical to those of the existing plant.  Upon completion of the construction, the new production line and its boat building operations will be incorporated into the facility’s Title V air operation permit as a new emissions unit (i.e., Emissions Unit ID No. 002).

The applicant requests an increase of the facility-wide emission limit for VOC from 99 TPY to 220 TPY.  According to information provided by the applicant, there will be no equipment changes at the existing production line at 3207 Industrial 29th Street to accommodate this increase.  The existing production line is authorized to operate continuously (8,760 hours/year); therefore, an increase in the number of boats manufactured per 12-month period will account for the increase in VOC emissions.  Because the increase of the VOC emission limit will not require any physical changes or construction at the existing facility, the Title V Air Operation Permit will be revised to reflect this increase as soon as this Air Construction Permit is issued.

Discussion of Emissions

The emissions from the proposed project are expected to increase the VOC emissions by a factor of 2.25 compared with actual emissions in calendar year 2016, as reported in the Annual Operating Report.  A comparison can be seen in Table 1 below.

Table 1.  Comparison of 2016 Actual Emissions against Projected Emissions

| Pollutant | 2016 Actual Emissions (TPY) | Projected Emissions (TPY) |
| --- | --- | --- |
| VOC | 96.36 | 216.81 |
| HAP | 72.56 | 163.26 |

The existing facility is major according to Chapter 62-213, F.A.C., for emissions of HAP, and the proposed project will make the facility major for VOC emissions according to 62-213, F.A.C., because the project will cause VOC emissions to exceed 100 TPY.  The proposed project is also expected to increase HAP emissions by a factor of 2.25.  All VOC and HAP emissions from the new and existing production lines are fugitive emissions.  Emissions will be controlled by HEPA filters, good application practices, and proper handling of VOC and HAP storage containers.

State Requirements

The following state regulations are applicable to the existing and proposed emissions units.

CHAPTER 62-4, F.A.C. Permits

CHAPTER 62-204, F.A.C. Air Pollution Control - General Provisions

CHAPTER 62-210, F.A.C. Stationary Sources - General Requirements

CHAPTER 62-212, F.A.C. Stationary Sources - Preconstruction Review

CHAPTER 62-213, F.A.C. Operation Permits for Major Sources of Air Pollution

CHAPTER 62-296, F.A.C. Stationary Sources - Emission Standards

CHAPTER 62-297, F.A.C. Stationary Sources - Emissions Monitoring

Federal NSPS Provisions

The facility does not operate any units subject to federal NSPS provisions (40 CFR 60).

Federal NESHAP Provisions

Emissions Unit 001 (Fiberglass Boat Building Activities) at the facility is subject to federal NESHAP provisions:  40 CFR 63 Subpart A – General Provisions and Subpart VVVV – NESHAP for Boat Manufacturing.  The boat building activities at the proposed site will be subject to the above listed federal NESHAP regulations.  Emissions units subject to 40 CFR 63 Subpart VVVV must comply with the applicable HAP limit calculated pursuant to this subpart by one of the three following methods:

- MACT model point value averaging (emissions averaging)

- Compliant materials

- Add-on controls.

The facility must submit compliance reports pursuant to 40 CFR 63 Subpart VVVV to demonstrate compliance with the calculated HAP limitation for emissions units subject to this subpart.  The reporting frequency is every 6 months, and the report must contain data for the last 6 12-month rolling periods.

Other Draft Permit Requirements

In order to provide reasonable assurance to the Department that the increased VOC emission limitation of 220 TPY will not be exceeded, the facility will be required to maintain monthly records for VOC emissions tracking in addition to the recordkeeping requirements of 40 CFR 63, Subpart VVVV.  These records shall be made available to the Department upon request.  These tracking requirements will be outlined in the draft air construction permit.

- Preliminary Determination

The Department makes a preliminary determination that the proposed project will comply with all applicable state and federal air pollution regulations as conditioned by the draft permit.  This determination is based on a technical review of the complete application, reasonable assurances provided by the applicant, and the conditions specified in the draft permit.  No air quality modeling analysis is required because the project does not result in a significant increase in emissions.  Eric Dunkelberger is the project engineer responsible for reviewing the application and drafting the permit.  Additional details of this analysis may be obtained by contacting the project engineer at the Department’s Southeast District Office, 3301 Gun Club Road, MSC 7210-1, West Palm Beach, Florida  33406, phone (561) 681-6623, or by email Eric.Dunkelberger@dep.state.fl.us.