# Broward County Solid Waste and Recycling Services

# Broward County Landfill

# Facility ID No. 0112370

# Broward County

#### Title V Air Operation Permit Renewal

#### Permit No. 0112370-007-AV

(Renewal of Title V Air Operation Permit No. 0112370-006-AV)

# Permitting Authority:

State of Florida

Department of Environmental Protection

Division of Air Resource Management

Office of Permitting and Compliance

2600 Blair Stone Road

Mail Station #5505

Tallahassee, Florida  32399-2400

# Telephone:  (850) 717-9000

Email:  DARM_Permitting@dep.state.fl.us

# Compliance Authority:

Southeast District Office

3301 Gun Club Road, MSC 7210-1

West Palm Beach, FL  33406

Telephone:  (561) 681-6600

E-mail:  SED.AIR@dep.state.fl.us

Title V Air Operation Permit Renewal

Permit No. 0112370-007-AV

Table of Contents

Section	Page Number

Placard Page. 	 1

- Facility Information.

- Facility Description. 	 2

- Summary of Emissions Units. 	 2

- Applicable Regulations. 	 2

- Facility-wide Conditions. 	 4

- Emissions Units and Conditions.

- Municipal Solid Waste Landfill and Gas Collection System. 	 7

- 200 kW Diesel Engine Driven Emergency Generator. 	 10

- Gasoline Dispensing Facilities. 	 14

- Appendices. 	 See Appendices Document

Appendix A, Glossary.

Appendix I, List of Insignificant Emissions Units and/or Activities.

Appendix NESHAP, Subpart A - General Provisions.

Appendix NESHAP, Subpart ZZZZ - National Emissions Standards for Hazardous Air Pollutants (NESHAP) for Stationary Reciprocating Internal Combustion Engines.

Appendix RR, Facility-Wide Reporting Requirements.

Appendix TR, Facility-Wide Testing Requirements.

Appendix TV, Title V General Conditions.

Appendix U, List of Unregulated Emissions Units and/or Activities.

Referenced Attachments. 	 At End

Table H, Permit History.

| Permittee: Broward County Solid Waste and Recycling Services 1 North University Drive, Suite 400 Plantation, Florida 33324 | Permit No. 0112370-007-AV Broward County Landfill Facility ID No. 0112370 Title V Air Operation Permit Renewal |
| --- | --- |

The purpose of this permit is to renew the Title V air operation permit for the above referenced facility.  The existing Broward County Landfill is in Broward County at 7101 SW 205th Avenue, Unincorporated Broward County, Florida.  UTM Coordinates are:  Zone 17, 557.95 kilometers (km) East, 2,880.32 km North.  Latitude is:  26° 2’ 26.6615” North; and, Longitude is:  80° 25’ 15.0742” West.

The Title V air operation permit is issued under the provisions of Chapter 403, Florida Statutes (F.S.), and Florida Administrative Code (F.A.C.) Chapters 62-4, 62-210, and 62-213.  The above named permittee is hereby authorized to operate the facility in accordance with the terms and conditions of this permit.

Executed in Tallahassee, Florida.

0112370-007-AV Effective Date:  May 10, 2018

Renewal Application Due Date:  September 27, 2022

Expiration Date:  May 10, 2023

For:

Syed Arif, P.E., Program Administrator

Office of Permitting and Compliance

Division of Air Resource Management

SA/dlr/ead

Subsection A.  Facility Description.

The Broward County Landfill consists of a Municipal Solid Waste (MSW) disposal facility (landfill) having a capacity equal to or greater than 2.5 million megagrams by mass or 2.5 million cubic meters by volume.  The landfill began accepting MSW in September of 1988.  The landfill has installed and operates a landfill gas collection and control system (GCCS) composed of 19 gas extraction wells and an open candlestick flare (nominal capacity 750 standard cubic feet per minute (scfm)).  This GCCS and flare were installed voluntarily by the facility for odor mitigation (i.e., not required by any federal or state regulation).

At this time, the Broward County Landfill is not accepting MSW.  The landfill has not accepted MSW since April 1991.  The waste accepted by the landfill is non-putrescible waste, primarily construction and demolition (C&D) and bulk debris.  The majority of MSW in Broward County is sent to the Wheelabrator South facility to be combusted for power generation and resource recovery.  The Broward County Landfill is filling at a rate such that the amount of nonmethane organic compounds (NMOC) generated is below 50 megagrams per year.

The facility has a 200 kilowatt (kW) emergency generator that is driven by a diesel reciprocating internal combustion engine (RICE).  The manufacture date of this generator is estimated to be 1989 or 1990.

The facility has a vehicle refueling operation and stationary fuel storage tanks that are subject to 40 CFR 63, Subpart CCCCCC - NESHAP for Source Category:  Gasoline Dispensing Facilities.

Subsection B.  Summary of Emissions Units.

| EU No. | ## Brief Description |
| --- | --- |
| Regulated Emissions Units |  |
| 001 | Municipal Solid Waste Landfill and Landfill Gas Collection System |
| 005 | 200 kW Diesel Engine Driven Emergency Generator |
| 006 | Gasoline Dispensing Facilities |
| Unregulated Emissions Units and Activities (see Appendix U, List of Unregulated Emissions Units and/or Activities) |  |
| 007 | 750 scfm Open Candlestick Utility Flare |

Also included in this permit are miscellaneous insignificant emissions units and/or activities (see Appendix I, List of Insignificant Emissions Units and/or Activities).

Subsection C.  Applicable Regulations.

Based on the Title V air operation permit renewal application received September 26, 2017, this facility is not a major source of hazardous air pollutants (HAP).  A summary of applicable regulations is shown in the following table.

| Regulation | ## EU Nos. |
| --- | --- |
| Federal Rule Citations |  |
| 40 CFR 63, Subpart A - General Provisions. | 005 & 006 |
| 40 CFR 63, Subpart ZZZZ - NESHAP for Stationary Reciprocating Internal Combustion Engines. | 005 |
| 40 CFR 63, Subpart CCCCCC - NESHAP for Source Category: Gasoline Dispensing Facilities. | 006 |
| State Rule Citations |  |
| Chapter 62-4, F.A.C.: Permits. | 001, 005, & 006 |
| Rule 62-204.800, F.A.C.: Federal Regulations Adopted by Reference. | 001 & 005 |
| Rule 62-204.800(9)(c), F.A.C.: Municipal Solid Waste Landfills. | 001 |
| Rule 62-210.200, F.A.C.: Definitions. | 001 & 005 |
| Rule 62-210.300, F.A.C.: Permits Required. | 001, 005, & 006 |
| Chapter 62-213, F.A.C.: Operation Permits for Major Sources of Air Pollution. |  |
| Rule 62-296.320, F.A.C.: General Pollutant Emission Limiting Standards. |  |
| Rule 62-297.310, F.A.C.: General Emissions Test Requirements. | 001 & 005 |

Table of Contents.

The following conditions apply facility-wide to all emission units and activities:

- Appendices.  The permittee shall comply with all documents identified in Section IV, Appendices, listed in the Table of Contents.  Each document is an enforceable part of this permit unless otherwise indicated.  [Rule 62-213.440, F.A.C.]

Emissions and Controls

- Not federally Enforceable.  Objectionable Odor Prohibited.  No person shall cause, suffer, allow or permit the discharge of air pollutants, which cause or contribute to an objectionable odor.  An “objectionable odor” means any odor present in the outdoor atmosphere which by itself or in combination with other odors, is or may be harmful or injurious to human health or welfare, which unreasonably interferes with the comfortable use and enjoyment of life or property, or which creates a nuisance.  [Rule 62-296.320(2) and 62-210.200(Definitions), F.A.C.]

- Odor Remediation Plan.  The facility shall be operated to control objectionable odors in accordance with subsection 62-296.320(2), F.A.C.  After being notified by the Department that objectionable odors have been confirmed beyond the landfill property boundary, the owner or operator shall:

- Immediately take steps to reduce the objectionable odors.  Such steps may include applying or increasing initial cover, reducing the size of the working face, and ceasing operations in the areas where odors have been detected;

- Submit to the Department for approval an odor remediation plan for the gas releases.  The plan shall describe the nature and extent of the problem and the proposed long-term remedy.  The remedy shall be initiated within 30 days of approval.

- Implement a routine odor monitoring program to determine the timing and extent of any off-site odors, and to evaluate the effectiveness of the odor remediation plan.

[Rule 62-701.530(3)(b), F.A.C.]

- General Volatile Organic Compounds (VOC) Emissions or Organic Solvents (OS) Emissions.  The permittee shall allow no person to store, pump, handle, process, load, unload or use in any process or installation, volatile organic compounds or organic solvents without applying known and existing vapor emission control devices or systems deemed necessary and ordered by the Department.  [Rule 62-296.320(1), F.A.C.]

{Permitting Note:  Nothing is deemed necessary and ordered at this time.}

- General Visible Emissions.  No person shall cause, let, permit, suffer or allow to be discharged into the atmosphere the emissions of air pollutants from any activity equal to or greater than 20% opacity.  This regulation does not impose a specific testing requirement.  [Rule 62-296.320(4)(b), F.A.C.]

- Unconfined Particulate Matter.  No person shall cause, let, permit, suffer or allow the emissions of unconfined particulate matter from any activity, including vehicular movement; transportation of materials; construction; alteration; demolition or wrecking; or industrially related activities such as loading, unloading, storing or handling; without taking reasonable precautions to prevent such emissions.  Reasonable precautions to prevent emissions of unconfined particulate matter at this facility include:

- Landscaping and vegetation planting to prevent wind erosion of inactive landfill cells.

- Water application and asphalt millings to suppress particulate emissions from unpaved roads at the site.

- Posted and enforced speed limits on unpaved roads to minimize fugitive particulate emissions generated by vehicle traffic.

[Rule 62-296.320(4)(c), F.A.C.; and, proposed by applicant in Title V air operation permit renewal application received September 26, 2017.]

Annual Reports and Fees

See Appendix RR, Facility-wide Reporting Requirements, for additional details and requirements.

- Electronic Annual Operating Report and Title V Annual Emissions Fees.  The information required by the Annual Operating Report for Air Pollutant Emitting Facility [Including Title V Source Emissions Fee Calculation] (DEP Form No. 62-210.900(5)) shall be submitted by April 1 of each year, for the previous calendar year, to the Department of Environmental Protection’s Division of Air Resource Management.  Each Title V source shall submit the annual operating report using the DEP’s Electronic Annual Operating Report (EAOR) software, unless the Title V source claims a technical or financial hardship by submitting DEP Form No. 62-210.900(5) to the DEP Division of Air Resource Management instead of using the reporting software. Emissions shall be computed in accordance with the provisions of subsection 62-210.370(2), F.A.C.  Each Title V source must pay between January 15 and April 1 of each year an annual emissions fee in an amount determined as set forth in subsection 62-213.205(1), F.A.C.  The annual fee shall only apply to those regulated pollutants, except carbon monoxide and greenhouse gases, for which an allowable numeric emission-limiting standard is specified in the source’s most recent construction permit or operation permit.  Upon completing the required EAOR entries, the EAOR Title V Fee Invoice can be printed by the source showing which of the reported emissions are subject to the fee and the total Title V Annual Emissions Fee that is due.  The submission of the annual Title V emissions fee payment is also due (postmarked) by April 1st of each year.  A copy of the system-generated EAOR Title V Annual Emissions Fee Invoice and the indicated total fee shall be submitted to:  Major Air Pollution Source Annual Emissions Fee, P.O. Box 3070, Tallahassee, Florida  32315-3070.  Additional information is available by accessing the Title V Annual Emissions Fee On-line Information Center at the following Internet web site:  http://www.dep.state.fl.us/air/emission/tvfee.htm.  [Rules 62-210.370(3), 62-210.900 & 62-213.205, F.A.C.; and, §403.0872(11), Florida Statutes (2013)]

{Permitting Note:  Resources to help you complete your AOR are available on the electronic AOR (EAOR) website at:  http://www.dep.state.fl.us/air/emission/eaor.  If you have questions or need assistance after reviewing the information posted on the EAOR website, please contact the Department by phone at (850) 717-9000 or email at eaor@dep.state.fl.us.}

{Permitting Note:  The Title V Annual Emissions Fee form (DEP Form No. 62-213.900(1)) has been repealed.  A separate Annual Emissions Fee form is no longer required to be submitted by March 1st each year.}

- Annual Statement of Compliance.  The permittee shall submit an annual statement of compliance to the compliance authority at the address shown on the cover of this permit and to the US. EPA at the address shown below within 60 days after the end of each calendar year during which the Title V air operation permit was effective.  (See also Appendix RR, Conditions RR1 and RR7.)  [Rules 62-213.440(3)(a)2. & 3. and (b), F.A.C.]

U.S. Environmental Protection Agency, Region 4

Atlanta Federal Center

61 Forsyth Street, SW

Atlanta, Georgia  30303

Attn:  Air Enforcement Branch

- Prevention of Accidental Releases (Section 112(r) of CAA).  If, and when, the facility becomes subject to 112(r), the permittee shall:

- Submit its Risk Management Plan (RMP) to the Chemical Emergency Preparedness and Prevention Office (CEPPO) RMP Reporting Center.  Any Risk Management Plans, original submittals, revisions or updates to submittals, should be sent electronically through EPA’s Central Data Exchange system at the following address:  https://cdx.epa.gov.  Information on electronically submitting risk management plans using the Central Data Exchange system is available at:  http://www2.epa.gov/rmp.  The RMP Reporting Center can be contacted at:  RMP Reporting Center, Post Office Box 10162, Fairfax, VA  22038, Telephone:  (703) 227-7650.

- Submit to the permitting authority Title V certification forms or a compliance schedule in accordance with Rule 62-213.440(2), F.A.C.

[40 CFR 68]

- Semi-Annual Monitoring Reports.  The permittee shall monitor compliance with the terms and conditions of this permit and shall submit reports of any deviations from the requirements of these conditions at least every six (6) months.  All instances of deviations from permit requirements must be clearly identified in such reports, including reference to the specific requirement and the duration of such deviation.  All reports shall be accompanied by a certification by a responsible official, pursuant to subsection 62-213.420(4), F.A.C.  (See also Conditions RR2. – RR4. of Appendix RR, Facility-wide Reporting Requirements, for additional reporting requirements related to deviations.)  [Rule 62-213.440(1)(b)3.a., F.A.C.]

{Permitting Note:  EPA has clarified that, pursuant to 40 CFR 70.6(a)(3), the word “monitoring” is used in a broad sense and means monitoring (i.e., paying attention to) the compliance of the source with all emissions limitations, standards, and work practices specified in the permit.}

Table of Contents.

The specific conditions in this section apply to the following emissions unit:

| EU No. | Brief Description |
| --- | --- |
| 001 | Municipal Solid Waste Landfill and Landfill Gas Collection System |

This emissions unit is an existing municipal solid waste landfill for which construction, reconstruction, or modification was commenced before May 30, 1991.  The landfill began accepting waste in September of 1988.  The design capacity of the landfill is equal to or greater than 2.5 million megagrams by mass or 2.5 million cubic meters by volume.  The landfill is filling at a rate such that the amount of NMOC generated is below 50 megagrams per year.  A landfill gas collection and control system (GCCS) with a nominal 750 standard cubic feet per minute open candlestick utility flare was installed for voluntary odor mitigation.  The GCCS became operational in January 1996.

The landfill accepted MSW from September 1988 until April 1991.  At this time, the landfill is not accepting any Class I waste (i.e., MSW).  The landfill is not closed, and it retains the option to accept MSW, if necessary.  The landfill accepts Class III waste (i.e., construction and demolition debris and bulk waste).

Based on samples taken in January 2018, the average hydrogen sulfide concentration of the landfill gas is 19.7 parts per million by volume (ppmv), and the average total reduced sulfur concentration of the landfill gas is 20.5 ppmv.  Both of these values are less than the reference values for landfill gas given in AP-42:  Compilation of Air Emission Factors.

{Permitting Notes:  This emissions unit is regulated pursuant to Rule 62-204.800(9)(c), F.A.C., into which the emissions guidelines contained in NSPS 40 CFR 60, Subpart Cc - Emission Guidelines and Compliance Times for Municipal Solid Waste Landfills, were adopted and incorporated by reference.  This landfill is not an affected facility subject to NSPS 40 CFR 60, Subpart WWW - Standards of Performance for Municipal Solid Waste Landfills (adopted and incorporated by reference in Rule 62-204.800(8)(b)74, F.A.C.) due to the age of the landfill; however, the emissions guidelines of Subpart Cc for existing facilities require the affected facilities to comply with several of the requirements contained in Subpart WWW, by reference.  This emissions unit is not an affected facility subject to NESHAP 40 CFR 63, Subpart AAAA - NESHAP: Municipal Solid Waste Landfills (adopted and incorporated by reference in Rule 62-204.800(11)(b)59, F.A.C.) for the following reasons:  it is not a major source as defined in NESHAP 40 CFR 63, Subpart A - General Provisions; it is not collocated with a major source as defined in Subpart A; and estimated uncontrolled emissions of NMOC are not equal to or greater than 50 megagrams per year.}

Essential Potential to Emit (PTE) Parameters

- Hours of Operation.  This emissions unit may operate continuously (8,760 hours/year).  [Rule 62-210.200(PTE), F.A.C.]

- Emissions Unit Operating Rate Limitation After Testing.  See the related testing provisions in Appendix TR, Facility-wide Testing Requirements.  [Rule 62-297.310(3), F.A.C.]

Emissions Standards

- NMOC Emission Rate.  The owner or operator shall calculate the NMOC emission rate for the landfill using the procedures described in Condition A.6.

  - If the calculated NMOC emission rate is less than 50 megagrams per year, the owner or operator shall:

      - Submit an annual NMOC emissions rate report to the Department, as provided in Condition A.8.

      - Recalculate the NMOC emission rate annually using the procedures described in Condition A.6 until such time as the calculated NMOC emission rate is equal to or greater than 50 megagrams per year, or the landfill is closed.

        - If the NMOC emission rate, upon recalculation, is equal to or greater than 50 megagrams per year, the owner or operator shall install a collection and control system in compliance with 40 CFR 60.752(b)(2).  [Link to 40 CFR 60.752]

        - If the landfill is permanently closed, a closure notification shall be submitted to the Department as provided in Condition A.9.

  - If the calculated NMOC emission rate is equal to or greater than 50 megagrams per year, the owner or operator shall:

      - Submit a collection and control system design plan prepared by a professional engineer to the Department within 1 year that meets the requirements of 40 CFR 60.752(b)(2)(i).  [Link to 40 CFR 60.752]

      - Install a collection and control system that captures the gas generated within the landfill as required by 40 CFR 60.752(b)(2)(ii)(A) or (B) and (b)(2)(iii) within 30 months after the first annual report in which the NMOC emission rate equals or exceeds 50 megagrams per year, unless Tier 2 or Tier 3 sampling demonstrates that the NMOC emission rate is less than 50 megagrams per year, as specified in 40 CFR 60.757(c)(1) or (2).  [Link to 40 CFR 60.757]

[Rule 62-204.800(9)(c), F.A.C., and 40 CFR 60.752, 60.754, & 60.757]

Closure of Landfill

- Landfill Closure.  When an MSW landfill is closed, the owner or operator is no longer subject to the requirement to maintain an operating permit under 40 CFR part 70 or 71 for the landfill if the landfill is not otherwise subject to the requirements of either part 70 or 71 and if either of the following conditions are met:

  - The landfill was never subject to the requirement for a control system pursuant to 40 CFR 60, Subpart Cc, or

  - The owner or operator meets the conditions for control system removal specified in 40 CFR 60, Subpart WWW.

[Rule 62-204.800(9)(c), F.A.C., and 40 CFR 60.32c(d)]

Test Methods and Procedures

- Common Testing Requirements.  Unless otherwise specified, tests shall be conducted in accordance with the requirements and procedures specified in Appendix TR, Facility-wide Testing Requirements, of this permit.  [Rule 62-297.310, F.A.C.]

- Calculation of the NMOC Emission Rate.  The owner or operator shall calculate the NMOC emission rate using the following equation:

MNMOC= i=1n2kLoMi(e-kti)(CNMOC)(3.6 x 10-9)

Where,

MNMOC = Total NMOC emission rate from the landfill, megagrams per year

k = methane generation rate constant, year-1 = 0.05

Lo = methane generation potential, cubic meters per megagram solid waste = 170 cubic meters per megagram

Mi = mass of solid waste in the ith section, megagrams

ti = age of the ith section, years

CNMOC = concentration of NMOC, parts per million (ppm) by volume as hexane = 4,000 ppm by volume as hexane

n = total number of sections at the landfill

3.6 x 10-9 = conversion factor

The mass of nondegradable solid waste may be subtracted from the total mass of solid waste in a particular section of the landfill when calculating the value for Mi, if documentation of the nature and amount of such wastes is maintained.  [Rule 62-204.800(9)(c), F.A.C., and 40 CFR 60.754(a)(1)(i)]

Recordkeeping and Reporting Requirements

- Reporting Schedule.  The following reports and notifications shall be submitted to the Compliance Authority:

| Report | Reporting Deadline | Related Condition(s) |
| --- | --- | --- |
| NMOC Emission Rate Report | Annually (See exemption criteria in Condition A.8.) | A.8. |
| Closure Report | Within 30 days of waste acceptance cessation | A.9. |
| Semi-annual Monitoring Report | Every 6 months | FW10. See Appendix RR |

[Rules 62-204.800(9)(c)5 & 62-213.440(1)(b), F.A.C.; and 40 CFR 60.757(a) & (b) and 60.757(d)]

- NMOC Emission Rate Report.  The owner or operator shall submit a nonmethane organic compounds (NMOC) emission rate report to the Department annually.  The report shall meet the following requirements:

  - The NMOC emission rate report shall contain an annual estimate of the NMOC emission rate calculated using the formula and procedures provided in Condition A.6.

  - If the estimated NMOC emission rate as reported in the annual report is less than 50 megagrams per year in each of the next 5 consecutive years, the owner or operator may elect to submit an estimate of the NMOC emission rate for the next 5-year period in lieu of the annual report.

      - In addition to the NMOC emission rate, this estimate shall include:

        - The current amount of solid waste-in-place.

        - The estimated waste acceptance rate for each year of the 5 years for which an NMOC emission rate is estimated.

- All data and calculations upon which this estimate is based shall be provided to the Department.

- This estimate shall be revised at least once every five years.

- If the actual waste acceptance rate exceeds the estimated waste acceptance rate in any year reported in the 5-year estimate, a revised 5-year estimate shall be submitted to the Department.  The revised estimate shall cover the 5-year period beginning with the year in which the actual waste acceptance rate exceeded the estimated waste acceptance rate.

[Rule 62-204.800(9)(c)5, F.A.C., and 40 CFR 60.757(a) & (b)]

- Closure Report.  The owner or operator shall submit a closure report to the Department within 30 days of waste acceptance cessation.  The Department may request additional information as may be necessary to verify that permanent closure has taken place in accordance with the requirements of 40 CFR 258.60.  [Link to 40 CFR 258.60]  If a closure report has been submitted to the Department, no additional wastes may be placed in the landfill without filing a notification of modification as described under 40 CFR 60.7(a)(4).  [Rule 62-204.800(9)(c)5, F.A.C.; and 40 CFR 60.757(d)]

- Records Retention.  The owner or operator shall keep for at least 5 years, up-to-date, readily accessible, on-site records of the design capacity report, the current amount of solid waste in place, and the year-by-year waste acceptance rate.  Off-site records may be maintained if they are retrievable within 4 hours.  Either paper copy or electronic formats are acceptable.  [Rule 62-204.800(9)(c), F.A.C., and 40 CFR 60.758(a)]

- Other Reporting Requirements.  See Appendix RR, Facility-Wide Reporting Requirements, for additional reporting requirements.  [Rule 62-213.440(1)(b), F.A.C.]

Table of Contents.

The specific conditions in this section apply to the following emissions unit:

| EU No. | Brief Description |
| --- | --- |
| 005 | 200 kW Diesel Engine Driven Emergency Generator |

This emissions unit is a Kohler Fast Response II Generator Model 200ROZ271.  The generator is driven by a Cummins Model NT-855G4 diesel-fuel compression ignition reciprocating internal combustion engine (RICE).  The manufacture date of this generator is estimated to be 1989 or 1990.  The exhaust gas flow rate is estimated to be 2,040 dry standard cubic feet per minute.  The exhaust gas temperature is estimated to be 856 degrees Fahrenheit.

The following table provides important details for this emissions unit:

| EU No. | Engine Brake HP | Date of Construction | Model Year | Primary Fuel | Type of Engine | Displacement liters/cylinder | Serial No. | Date of Last Modification or Reconstruction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 005 | 375 | 1990 | 1990 | Diesel | Emergency | 2.3 | 252242 | Not Applicable |

{Permitting Note:  This compression ignition (CI) engine is regulated under 40 CFR 63, Subpart ZZZZ - NESHAP for Stationary RICE, adopted and incorporated by reference in Rule 62-204.800(11)(b)82, F.A.C.  This RICE is not used for fire pumps.  This permit section addresses an “existing” stationary CI RICE less than or equal to 500 horsepower (HP) that is located at an area source of HAP and that has not been modified or reconstructed after June 12, 2006.  Unless the RICE is modified or reconstructed after July 11, 2005, NSPS 40 CFR 60, Subpart IIII, will not apply.}

Essential Potential to Emit (PTE) Parameters

- Hours of Operation.  The owner or operator shall operate the emergency stationary RICE according to the requirements in paragraphs a. through c. of this specific condition.  In order for the engine to be considered an emergency stationary RICE under 40 CFR 63 Subpart ZZZZ, any operation other than emergency operation, maintenance and testing, emergency demand response, and operation in non-emergency situations for 50 hours per year, as described in paragraphs a. through c. of this specific condition, is prohibited.  If the permittee does not operate the engine according to the requirements in paragraphs a. through c. of this specific condition, the engine will not be considered an emergency engine under this subpart and must meet all requirements for non-emergency engines.

- Emergency Situations.  There is no time limit on the use of emergency stationary RICE in emergency situations.

- Maintenance and Testing.  The permittee may operate this emergency stationary RICE for maintenance and testing purposes for a maximum of 100 hours per calendar year.  Any operation for non-emergency situations as allowed by paragraph c. of this specific condition counts as part of the 100 hours per calendar year allowed by this paragraph b.  Emergency stationary RICE may be operated for maintenance checks and readiness testing, provided that the tests are recommended by federal, state or local government, the manufacturer, the vendor, the regional transmission organization or equivalent balancing authority and transmission operator, or the insurance company associated with the engine. The owner or operator may petition the Department for approval of additional hours to be used for maintenance checks and readiness testing, but a petition is not required if the owner or operator maintains records indicating that federal, state, or local standards require maintenance and testing of emergency RICE beyond 100 hours per calendar year.

- Non-emergency and Other Situations.  Emergency stationary RICE located at area sources of HAP may be operated for up to 50 hours per calendar year in non-emergency situations.  The 50 hours of operation in non-emergency situations are counted as part of the 100 hours per calendar year for maintenance and testing provided in paragraph b. of this specific condition.  The 50 hours per year for non-emergency situations cannot be used for peak shaving or non-emergency demand response, or to generate income for a facility to supply power to an electric grid or otherwise supply power as part of a financial arrangement with another entity.

[Rule 62-204.800(11)(b)82, F.A.C.; and 40 CFR 63.6640(f)]

Emission Limitations and Operating Requirements

- Work or Management Practice Standards.

- Oil.  Change oil and filter every 500 hours of operation or annually, whichever comes first.

- Air Cleaner.  Inspect air cleaner every 1,000 hours of operation or annually, whichever comes first.

- Hoses and Belts.  Inspect all hoses and belts every 500 hours of operation or annually, whichever comes first, and replace as necessary.

[Rule 62-204.800(11)(b)82, F.A.C.; and 40 CFR 63 Table 2d of Subpart ZZZZ]

- Operation and Maintenance.  Operate and maintain the stationary RICE according to the manufacturer’s emission-related operation and maintenance instructions or develop and follow a maintenance plan which must provide, to the extent practicable for the maintenance and operation of the RICE in a manner consistent with good air pollution control practices for minimizing emissions.
[Rule 62-204.800(11)(b)82, F.A.C.; and 40 CFR 63.6625(e), 63.6640(a), and 40 CFR 63 Table 6(9)(a)(i) & (ii) of Subpart ZZZZ]

- Oil Analysis.  The owner or operator has the option of using oil analysis to extend the change requirement in paragraph a, above.  The oil analysis must be performed at the same frequency specified for changing the oil in paragraph a, above.  The analysis program must at a minimum analyze the following three parameters:  Total Base Number, viscosity, and percent of water content.  The condemning limits for these parameters are as follows:  Total Base Number is less than 30 percent of the Total Base Number of the oil when new; viscosity of the oil has changed by more than 20 percent from the viscosity of the oil when new; or percent of water content (by volume) is greater than 0.5.  If all of these condemning limits are not exceeded, the engine owner or operator is not required to change the oil.  If any of the limits are exceeded, the engine owner or operator must change the oil within 2 days of receiving the results of the analysis; if the engine is not in operation when the results of the analysis are received, the engine owner or operator must change the oil within 2 days or before commencing operation, whichever is later.  The owner or operator must keep records of the parameters that are analyzed as part of the program, the results of the analysis, and the oil changes for the engine.  The analysis program must be part of the maintenance plan for the engine that is described in paragraph d, above.  [Rule 62-204.800(11)(b)82, F.A.C.; and 40 CFR 63.6625(i)]

Monitoring of Operations

- Engine Startup.  During periods of startup, the owner or operator must minimize the engine’s time spent at idle and minimize the engine’s startup time to a period needed for the appropriate and safe loading of the engine, not to exceed 30 minutes, after which time the non-startup emission limitations apply.
[Rule 62-204.800(11)(b)82, F.A.C.; and 40 CFR 63.6625(h)]

- Hour Meter.  The owner or operator must install a non-resettable hour meter if one is not already installed.  [Rule 62-204.800(11)(b)82, F.A.C.; and 40 CFR 63.6625(f)]

Compliance

- Continuous Compliance.  This RICE shall be in compliance with the emission limitations and operating standards in this section at all times.  [Rule 62-204.800(11)(b)82, F.A.C.; and 40 CFR 63.6605(a)]

- Operation and Maintenance of Equipment.  At all times, the owner or operator must operate and maintain any affected source, including associated air pollution control equipment and monitoring equipment, in a manner consistent with safety and good air pollution control practices for minimizing emissions.  Determination of whether such operation and maintenance procedures are being used will be based on information available to the compliance authority which may include, but is not limited to, monitoring results, review of operation and maintenance procedures, review of operation and maintenance records, and inspection of the source.  [Rule 62-204.800(11)(b)82, F.A.C.; and 40 CFR 63.6605(b)]

Reporting and Recordkeeping Requirements

- Notification, Performance, and Compliance Records.

- The owner or operator must keep records of the maintenance conducted pursuant to Specific Condition B.2.d.

- The owner or operator must keep records of the hours of operation of the engine that are recorded through the non-resettable hour meter.  The owner or operator must document how many hours are spent for emergency operation, including what classified the operation as emergency and how many hours are spent for non-emergency operation.  If the engine is used for demand response operation, the owner or operator must keep records of the notification of the emergency situation, and the time the engine was operated as part of demand response.

[Rule 62-204.800(11)(b)82, F.A.C.; and 40 CFR 63.6655(e) & (f)]

- Record Retention.

- The owner or operator must keep records in a suitable and readily available form for expeditious review.

- The owner or operator must keep each record readily accessible in hard copy or electronic form for at least 5 years after the date of each occurrence, measurement, maintenance, corrective action, report, or record.

[Rule 62-204.800(11)(b)82, F.A.C.; and 40 CFR 63.6660 & 40 CFR 63.10(b)(1)]

- Emergency Situation.  If an emergency engine is operating during an emergency and it is not possible to shut down the engine in order to perform the work practice requirements on the schedule required in Specific Condition B.2 of this section, or if performing the work practice on the required schedule would otherwise pose an unacceptable risk under federal, state, or local law, the work practice can be delayed until the emergency is over or the unacceptable risk under federal, state, or local law has abated.  The work practice should be performed as soon as practicable after the emergency has ended or the unacceptable risk under federal, state, or local law has abated.  Sources must report any failure to perform the work practice on the schedule required and the federal, state, or local law under which the risk was deemed unacceptable.
[Rule 62-204.800(11)(b)82, F.A.C.; and 40 CFR 63 Table 2d, footnote 2]

General Provisions

- 40 CFR 63 Subpart A, General Provisions.  RICE engines shall comply with all applicable requirements of 40 CFR 63 Subpart A, General Provisions, which have been adopted and incorporated by reference in Rule 62-204.800(11)(d)1, F.A.C., except that the Secretary is not the Administrator for purposes of 40 CFR 63.5(e), 40 CFR 63.5(f), 40 CFR 63.6(g), 40 CFR 63.6(h)(9), 40 CFR 63.6(j), 40 CFR 63.13, and 40 CFR 63.14.  This engine shall comply with the applicable portions of Appendix 40 CFR 63 NESHAP Subpart A included with this permit, as specified below:

| General Provisions Citation | Subject of Citation |
| --- | --- |
| §63.1 | General applicability of the General Provisions |
| §63.2 | Definitions |
| §63.3 | Units and abbreviations |
| §63.4 | Prohibited activities and circumvention |
| §63.5 | Construction and reconstruction |
| §63.6(a) | Applicability |
| §63.6(c)(1)–(2) | Compliance dates for existing sources |
| §63.9(i) | Adjustment of submittal deadlines |
| §63.9(j) | Change in previous information |
| §63.10(a) | Administrative provisions for recordkeeping/reporting |
| §63.10(b)(1) | Record retention |
| §63.10(b)(2)(vi)–(xi) | Records |
| §63.10(b)(2)(xii) | Record when under waiver |
| §63.10(b)(2)(xiv) | Records of supporting documentation |
| §63.10(b)(3) | Records of applicability determination |
| §63.10(d)(1) | General reporting requirements |
| §63.10(d)(4) | Progress Reports |
| §63.10(f) | Waiver for recordkeeping/reporting |
| §63.12 | State authority and delegations |
| §63.13 | Addresses |
| §63.15 | Availability of information |

[Rule 62-204.800(11)(b)82 & 62-204.800(11)(d)1, F.A.C, and 40 CFR 63.6665]

Table of Contents.

The specific conditions in this section apply to the following emissions unit:

| EU No. | Brief Description |
| --- | --- |
| 006 | Gasoline Dispensing Facilities |

This section addresses regulations for on-site gasoline vehicle fueling operations and storage tanks.  These operations are subject to regulation pursuant to 40 CFR 63, Subpart CCCCCC—NESHAP for Source Category:  Gasoline Dispensing Facilities.  The emission sources to which this section applies are gasoline storage tanks and associated equipment components in vapor or liquid gasoline service at this facility.  Pressure/Vacuum vents on gasoline storage tanks and the equipment necessary to unload product from cargo tanks into the storage tanks at gasoline dispensing facilities are covered emission sources.  Gasoline means any petroleum distillate or petroleum distillate/alcohol blend having a Reid vapor pressure of 27.6 kilopascals (kPa) or greater, which is used as a fuel for internal combustion engines.  Distillate fuel oil transfer and storage tanks are not subject to this subpart because of having a vapor pressure lower than 27.6 kPa.

The average throughput of gasoline used in GDF at the Broward County Landfill is approximately 900 gallons per month, with a maximum throughput of less than 1,500 gallons per month.  Therefore, GDF at this facility are subject only to the requirements for facilities with monthly throughput of less than 10,000 gallons of gasoline (see Condition C.2).

The following definitions are applicable to units subject to regulation in this section:

Gasoline means any petroleum distillate or petroleum distillate/alcohol blend having a Reid vapor pressure of 27.6 kilopascals or greater, which is used as a fuel for internal combustion engines.

Gasoline cargo tank means a delivery tank truck or railcar which is loading or unloading gasoline, or which has loaded or unloaded gasoline on the immediately previous load.

Gasoline dispensing facility (GDF) means any stationary facility which dispenses gasoline into the fuel tank of a motor vehicle, motor vehicle engine, nonroad vehicle, or nonroad engine, including a nonroad vehicle or nonroad engine used solely for competition. These facilities include, but are not limited to, facilities that dispense gasoline into on- and off-road, street, or highway motor vehicles, lawn equipment, boats, test engines, landscaping equipment, generators, pumps, and other gasoline-fueled engines and equipment.

Monthly throughput means the total volume of gasoline that is loaded into, or dispensed from, all gasoline storage tanks at each GDF during a month. Monthly throughput is calculated by summing the volume of gasoline loaded into, or dispensed from, all gasoline storage tanks at each GDF during the current day, plus the total volume of gasoline loaded into, or dispensed from, all gasoline storage tanks at each GDF during the previous 364 days, and then dividing that sum by 12.

Link to 40 CFR 63, Subpart CCCCCC – Nation Emission Standards for Hazardous Air Pollutants for Source Category:  Gasoline Dispensing Facilities

General Duties to Minimize Emissions

- Proper Operation.  You must, at all times, operate and maintain any affected source, including associated air pollution control equipment and monitoring equipment, in a manner consistent with safety and good air pollution control practices for minimizing emissions.  Determination of whether such operation and maintenance procedures are being used will be based on information available to the Compliance Authority which may include, but is not limited to, monitoring results, review of operation and maintenance procedures, review of operation and maintenance records, and inspection of the source.  [40 CFR 63.11115(a)]

Requirements for Facilities with Monthly Throughput of Less Than 10,000 Gallons of Gasoline

- Precautions to Prevent Extended Vapor Releases.  You must not allow gasoline to be handled in a manner that would result in vapor releases to the atmosphere for extended periods of time.  Measures to be taken include, but are not limited to, the following:

- Minimize gasoline spills;

- Clean up spills as expeditiously as practicable;

- Cover all open gasoline containers and all gasoline storage tank fill-pipes with a gasketed seal when not in use;

- Minimize gasoline sent to open waste collection systems that collect and transport gasoline to reclamation and recycling devices, such as oil/water separators.

- Portable gasoline containers that meet the requirements of 40 CFR part 59, subpart F, are considered acceptable for compliance with paragraph c., above.  [Link to 40 CFR 59, Subpart F]

[40 CFR 63.11116(a)]

- Throughput Exceedance.  If monthly throughput ever exceeds 10,000 gallons of gasoline, these units become subject to the requirements of 40 CFR 63.11117 and will remain subject to those requirements, even if the monthly throughput later falls below 10,000 gallons of gasoline.  [40 CFR 63.11111(i)]

Link to 40 CFR 63.11117

- Portable Gasoline Tanks.  The dispensing of gasoline from a fixed gasoline storage tank at a GDF into a portable gasoline tank for the on-site delivery and subsequent dispensing of the gasoline into the fuel tank of a motor vehicle or other gasoline-fueled engine or equipment used within the area source is only subject to the requirements of Condition C.2, even if the requirements of Condition C.3 are triggered.  [40 CFR 63.11111(j)]

Records and Reports

- Availability of Records.  You are not required to submit notifications or reports as specified in Subsections 63.11125, 63.11126, or Subpart A of 40 CFR 63, but you must have records available within 24 hours of a request by the Compliance Authority to document your gasoline throughput.  [40 CFR 63.11116(b)]

- 40 CFR 63 Subpart A, General Provisions.  This facility shall comply with the following applicable requirements of 40 CFR 63 Subpart A, General Provisions, which have been adopted by reference in Rule 62-204.800(11)(d)1., F.A.C., except that the Secretary is not the Administrator for purposes of 40 CFR 63.5(e), 40 CFR 63.5(f), 40 CFR 63.6(g), 40 CFR 63.6(h)(9), 40 CFR 63.6(j), 40 CFR 63.13, and 40 CFR 63.14.

| Citation | Subject | Brief description |
| --- | --- | --- |
| §63.1 | Applicability | Initial applicability determination; applicability after standard established; permit requirements; extensions, notifications. Specific requirements given in §63.11111. |
| §63.2 | Definitions | Definitions for part 63 standards. Additional definitions in §63.11132 |
| §63.3 | Units and Abbreviations | Units and abbreviations for part 63 standards |
| §63.4 | Prohibited Activities and Circumvention | Prohibited activities; Circumvention, severability |
| §63.5 | Construction/Reconstruction | Applicability; applications; approvals. Except that these notifications are not required for facilities subject to §63.11116 |
| §63.6(a) | Compliance with Standards/Operation & Maintenance—Applicability | General Provisions apply unless compliance extension; General Provisions apply to area sources that become major |
| §63.6(b)(5) | Notification | Must notify if commenced construction or reconstruction after proposal |
| §63.6(f)(2)-(3) | Methods for Determining Compliance | Compliance based on performance test, operation and maintenance plans, records, inspection |
| §63.6(g)(1)-(3) | Alternative Standard | Procedures for getting an alternative standard |
| §63.7(a)(3) | CAA Section 114 Authority | Administrator may require a performance test under CAA section 114 at any time |
| §63.8(a)(1) | Applicability of Monitoring Requirements | Subject to all monitoring requirements in standard |
| §63.8(b)(1) | Monitoring | Must conduct monitoring according to standard unless Administrator approves alternative |
| §63.9(a) | Notification Requirements | Applicability and State delegation |
| §63.9(b)(1)-(2), (4)-(5) | Initial Notifications | Submit notification within 120 days after effective date; notification of intent to construct/reconstruct, notification of commencement of construction/reconstruction, notification of startup; contents of each |
| §63.9(i) | Adjustment of Submittal Deadlines | Procedures for Administrator to approve change when notifications must be submitted |
| §63.9(j) | Change in Previous Information | Must submit within 15 days after the change |
| §63.10(a) | Recordkeeping/Reporting | Applies to all, unless compliance extension; when to submit to Federal vs. State authority; procedures for owners of more than one source |
| §63.10(b)(1) | Recordkeeping/Reporting | General requirements; keep all records readily available; keep for 5 years |
| §63.10(b)(2)(iii) | Maintenance records | Recordkeeping of maintenance on air pollution control and monitoring equipment |
| §63.10(b)(2)(xiv) | Records | All documentation supporting Initial Notification and Notification of Compliance Status |
| §63.10(b)(3) | Records | Applicability determinations |
| §63.10(d)(1) | General Reporting Requirements | Requirement to report |
| §63.10(f) | Waiver for Recordkeeping/Reporting | Procedures for Administrator to waive |
| §63.12 | Delegation | State authority to enforce standards |
| §63.13 | Addresses | Addresses where reports, notifications, and requests are sent |
| §63.14 | Incorporations by Reference | Test methods incorporated by reference |
| §63.15 | Availability of Information | Public and confidential information |

[40 CFR 63.11130 and Table 3 to Subpart CCCCCC]

Table of Contents.