Title V Air Operation Permit Renewal

Permit No. 0310005-021-AV

Applicant

The applicant for this project is Anchor Glass Container Corporation.  The applicant’s responsible official and mailing address are:  Travis Richins, EHS Manager, Anchor Glass Container Corporation, Jacksonville Plant 07, 4108 Valley Industrial Blvd, Shakopee, MN 55379.

Facility Description

The applicant operates the existing Jacksonville Plant 07, which is located in Duval County at 2121 Huron Street, Jacksonville, Florida.

This existing facility consists of two Glass Melting Furnace Nos. 3 and 4 and a 255 HP Emergency Fire Pump fired by diesel fuel subject to 40 CFR 63, Subpart ZZZZ - National Emissions Standards for Hazardous Air Pollutants for Stationary Reciprocating Internal Combustion Engines; 40 CFR 63, Subpart A, General Provisions; Rule 62-204.800(11), F.A.C.; and Rule 2.201, JEPB.

This facility also includes miscellaneous unregulated/insignificant emissions units and/or activities.

Anchor Glass Container Corporation manufactures various types and colors of container glass primarily for the food and beverage industry.  The facility consists of a raw material batching operation, two glass melting furnaces and six glass container forming and finishing lines.  The raw material batching operation consists of three operations including raw material delivery by truck and rail, raw material transfer by bucket elevator and storage in separate storage silos.  Raw material mixing consists of mixing and precisely weighing raw materials prior to feed into the glass melting furnaces.

The glass furnaces are precisely fed the raw material mix for continuous melting.  The furnaces are fired by natural gas, propane fuel, and electric boost.  After melting, the glass is cooled to forming temperatures in the refiner, alcoves, and forehearths.  The glass melting furnaces are each subject to the process weight table listed in Rule 62-296.320 F.A.C.; and Rule 2.1101, JEPB.  Additional emission limitations, throughput limitations, and production limitations have been placed into the permit to avoid Prevention of Significant Deterioration (PSD) applicability requirements as indicated in Air Construction Permits No. 0310005-008-AC and 0310005-012-AC.

Glass forming consists of shearing mechanisms by which the molten glass is cut into precise increments of hot, viscous glass called gobs, which are gravity fed to the forming machines.  The forming machines shape the glass through mechanical and air blowing processes into containers at the rate of hundreds per minute.

Hot end container coating consists of a vapor deposition hood, which is where a tin based coating is applied to the containers prior to the cold end coating process.

Containers are belt conveyed to the glass annealing lehrs, where residual stresses induced during glass container forming are removed prior to container cooling.

Cold End Container Coating allows the containers to pass through a fog-coating hood which applies a food-grade material for surface lubricity to aid in subsequent handling.  A spray coater applies a polyethylene coating afterwards as a scratch preventative.

A video-jet printing system puts identifying numbers on selected containers and cartons as required by customers.

In the packaging area containers are placed into cardboard boxes or onto pallets for bulk handling and shipping.

regulated Emissions Unit IDentification Numbers and descriptions

| EU No. | Brief Description |
| --- | --- |
| Regulated Emissions Units |  |
| 003 | Glass Melting Furnace No. 3 |
| 004 | Glass Melting Furnace No. 4 |
| 014 | Emergency Diesel Fire Pump |

Applicable Regulations

Based on the Title V air operation permit renewal application received on April 11, 2018, this facility is not a major source of hazardous air pollutants (HAP).  The existing facility is a prevention of significant deterioration (PSD) major source of air pollutants in accordance with Rule 62-212.400, F.A.C.  A summary of applicable regulations is shown in the following table:

| Regulation | EU No(s). |
| --- | --- |
| City of Jacksonville Ordinance Code, Title X, Chapter 360 [Environmental Regulation], Chapter 362 [Air and Water Pollution], Chapter 376 [Odor Control], JEPB Rule 1 [Final Rules with Respect to Organization, Procedure, and Practice]; JEPB Rule 2, Parts I through VII, and Parts IX through XIV. | 003, 004, 014 |
| 40 CFR 63, Subpart A, NESHAP General Provisions. | 014 |
| 40 CFR 63, Subpart ZZZZ. | 014 |

PROCESS FLOW DIAGRAM

Project DESCRIPTION

The purpose of this permitting project is to renew the existing Title V permit for the above referenced facility.  This permit includes the revisions in Permit No. 0310005-020-AV.

Processing Schedule and Related Documents

Application for a Title V Air Operation Permit Renewal received April 11, 2018

Informal Additional Information Request dated April 18, 2018

Informal Additional Information Response received April 25, 2018

Primary Regulatory requirements

Standard Industrial Classification (SIC) Code:  3221 – Glass Containers.

North American Industry Classification System (NAICS): 327213 Glass Container Manufacturing.

HAP:  The facility is not identified as a major source of hazardous air pollutants (HAP).

Title V:  The facility is a Title V major source of air pollution in accordance with Chapter 62-213, Florida Administrative Code (F.A.C.).

PSD:  The facility is not a Prevention of Significant Deterioration (PSD)-major source of air pollution in accordance with Rule 62-212.400, F.A.C.  Additional emission limitations, throughput limitations, and production limitations have been placed into the permit to avoid Prevention of Significant Deterioration (PSD) applicability requirements as indicated in Air Construction Permits No. 0310005-008-AC and 0310005-012-AC.

NSPS:  The facility does not operate units subject to the New Source Performance Standards (NSPS) of 40 Code of Federal Regulations (CFR) 60.

NESHAP:  The facility operates units subject to the National Emissions Standards for Hazardous Air Pollutants (NESHAP) of 40 CFR 63.

CAM:  Compliance Assurance Monitoring (CAM) does not apply to any of the units at the facility because the pollutants are not being controlled with control devices.

GHG:  The facility is not identified as a major source of green house gas (GHG) pollutants.  Facility-wide PTE of CO2e is 64,757 TPY.

The facility is not subject to 40 CFR 60, Subpart CC because both of the Glass Melting Furnace Nos. 3 and 4 commenced construction prior to June 15, 1979.  40 CFR 60.292(c) states that rebricking and the cost of rebricking is not considered as reconstruction for the purposes of 40 CFR 60.15/

PROJECT REVIEW

Renewal with no requested operating changes.

Conclusion

This project renews Title V air operation permit No. 0310005-020-AV, which was effective on May 9, 2013.  This Title V air operation permit renewal is issued under the provisions of Chapter 403, Florida Statues (F.S.), and Chapters 62-4, 62-210, and 62-213, F.A.C.