Title V Air Operation Permit Renewal

Permit No. 0570081-020-AV

Applicant

The applicant for this project is TPSI Terminals, LLC.  The applicant’s responsible official and mailing address are:  Dudley Tarlton, VP, ESOH, 1670 Broadway Ste. 3100, Denver, CO 80202.

Facility Description

The applicant operates the existing Transmontaigne Tampa Terminal, which is located in Hillsborough County at 1523 Port Ave., Tampa, FL 33605.

TPSI Terminals, LLC (TPSI) is a bulk gasoline terminal which receives petroleum products by marine vessel or tanker truck.  The petroleum products are stored in the petroleum storage tanks and re-distributed by loading out to tanker trucks via a loading rack, transporting through pipelines, or occasionally loading back out to barges (diesel only).  The facility consists of eleven (11) storage tanks (tanks) and one (1) truck loading rack.  There is also a slop tank and a petroleum contact water (PCW) tank present at the site.

TPSI blends gasoline with ethanol product.   The terminal has the capability of receiving either denatured or non-denatured ethanol via either marine vessel or tanker truck.  When non-denatured ethanol is received, the denaturant is combined with non-denatured ethanol prior to storage into the storage tanks.  The denaturing process occurs by metering E10 from the truck rack through a line, which has a "T" connection, to the cargo line that runs from the marine vessel to the storage tank.  No additional blending skid is required to denature the ethanol.  All ethanol stored at the Tampa terminal is denatured.

When ready to be loaded out, petroleum products from the storage tanks are loaded into trucks using one loading rack, which consists of eight (8) loading bays.  Ethanol is blended into the gasoline prior to being loaded into a truck.  The truck rack loading system has two identical high pressure pumps to pump the ethanol from the storage tank(s) to the loading rack. The output of these pumps is pressure controlled to deliver a desired pressure and flow rate of ethanol to the blending skid.  The blending skid is located adjacent to the truck loading rack. The blending skid consists of eight (8) meters and eight (8) control valves. Each valve and meter combination, on the skid, is paired with a specific gasoline meter at the truck loading rack. The ethanol blending activity is considered to be an insignificant activity.

Volatile organic compound (VOC) emissions from the storage and handling of the petroleum products are controlled by the storage tank roof type and tank seals.  Emissions are also controlled through limits on the product throughput and the Reid Vapor Pressures (RVP) values.  VOC emissions displaced during truck loading at the loading rack are directed to a knock-out underground tank and routed to the vapor control system, which consists of a primary VCU and a Back-Up Flare.  The primary control device is a John Zink Enclosed Flame Hydrocarbon Vapor Combustion Unit (VCU), Model No. ZCT-3-9-50-X-2/8-2/8-X-X, with natural gas assist for temperature control.  A John Zink Company, Series GV-LH, forced draft open flame flare serves as the back-up control device.

The Tampa Terminal has access to a 10” pipeline for transporting diesel fuel and a second 16” pipeline for transporting gasoline and ethanol.  These pipelines transport the petroleum products to the Orlando area through the Kinder Morgan pipelines.  TPSI also has the capability of loading diesel fuel out to barges, but the activity is considered an insignificant activity due to the relatively low level of emissions generated.  Also identified as an insignificant activity is the handling and storing of biodiesel through two biodiesel storage tanks.

TPSI is subject to the federal regulations under 40 CFR 60, more specifically, the loading rack is subject to 40 CFR 60, Subpart XX - Standards of Performance for Bulk Gasoline Terminals and Rule 62-296.510, F.A.C. - Bulk Gasoline Terminals.  Also, Storage Tank Nos. 1, 5, 8, and 9 are subject to 40 CFR 60, Subpart Kb - Standards of Performance for Volatile Organic Liquid Storage Vessels.   Storage Tank No. 7 is subject to 40 CFR 60, Subpart Ka - Standards of Performance for Storage Vessels for Petroleum Liquids.  In addition, Storage Tank Nos. 3, 4 and 6 are subject to the State regulations under Rule 62-296.508 - Petroleum Liquid Storage (RACT).

This facility is a minor source of hazardous air pollutants (HAPs).  It is therefore, exempt from the NESHAP, Subpart R, Gasoline Distribution Facilities.  However, as an area source (HAP) bulk gasoline terminal, this facility is subject to the NESHAP, Subpart BBBBBB, Gasoline Distribution Bulk Terminals, Bulk Plants, Pipeline Facilities and Gasoline Dispensing Facilities.

Per the FDEP guidance memo “Applicant Enforcement History” dated September 28, 2015, a nationwide enforcement search was performed for all programs using the EPCHC enforcement database and the EPA ECHO database.  The following formal enforcement activities have occurred in the last five years at the TPSI facilities listed.  The TPSI Transmontaigne Tampa Terminal has had no formal enforcement in any program in the last five years.

      - For the facility located at 1478 Highway 61, Jackson, LA 70748, Source ID No. LA0000002212500015, formal enforcement was taken on April 21, 2014 for a Clean Air Act Violation.

- For the facility located at 2630 Broadway, Evansville, IN 47712, Source ID No. IN0000001816300063, formal enforcement was taken on June 26, 2015 for a Clean Air Act Violation.

- For the facility located at 7600 Halifax Ln, Chesapeake, VA 23324, Source ID No. VA0000005155000035, formal enforcement was taken on April 4, 2013, for a Clean Air Act Violation.

Project DESCRIPTION

The purpose of this permitting project is to renew the existing Title V permit for the above referenced facility.

Processing Schedule and Related Documents

Initial Title V Air Operation Permit No. 0570081-008-AV issued October 21, 1999

Title V Air Operation Permit Renewal Permit No. 0570081-011-AV issued February 8, 2006

Title V Air Operation Permit Revision Permit No. 0570081-015-AV issued March 20, 2009

Title V Air Operation Permit Renewal Permit No. 0570081-016-AV issued April 25, 2011

Title V Air Operation Permit Revision Permit No. 0570081-019-AV issued June 30, 2015

Title V Air Operation Permit Renewal Permit No. 0570081-020-AV issued February 5, 2015

Primary Regulatory requirements

Standard Industrial Classification (SIC) Code:  4226 – Special Warehousing and Storage

North American Industry Classification System (NAICS):  493190 - Other Warehousing and Storage

HAP:  The facility is not identified as a major source of hazardous air pollutants (HAP).

Title V:  The facility is a Title V major source of air pollution in accordance with Chapter 62-213, Florida Administrative Code (F.A.C.).

NSPS:  The facility operates units subject to the New Source Performance Standards (NSPS) of 40 Code of Federal Regulations (CFR) 60.

NESHAP:  The facility operates units subject to the National Emissions Standards for Hazardous Air Pollutants (NESHAP) of 40 CFR 63.

CAM:  Compliance Assurance Monitoring (CAM) does apply to the Truck Loading Rack (Emission Unit No. 001) for the controlled emissions of volatile organic compounds (VOC).  VOC emissions are controlled by a primary VCU and a Back-Up Flare.  The primary control device is a John Zink Enclosed Flame Hydrocarbon Vapor Combustion Unit (VCU), Model No. ZCT-3-9-50-X-2/8-2/8-X-X, with natural gas assist for temperature control.  A John Zink Company, Series GV-LH, forced draft open flame flare serves as the back-up control device.

GHG:  The facility is not identified as a major source of greenhouse gas (GHG) pollutants.

PROJECT REVIEW

Changes to the permit made as part of this revision are shown in strike through format for deletions and in double underline format for additions.

- The permit was updated to reflect the most recent TV permit formatting.

- Facility wide Condition No. FW4 was updated to reflect the requirements of Rule 62-296.320(4)(b)1, F.A.C. and Rule 1-3.52, HCEPC.

FW4.  General Particulate Emission Limiting Standards.  General Visible Emissions Standard.  Except for emissions units that are subject to a particulate matter or opacity limit set forth or established by rule and reflected by conditions in this permit, no person shall cause, let, permit, suffer or allow to be discharged into the atmosphere the emissions of air pollutants from any activity, the density of which is equal to or greater than that designated as Number 1 on the Ringelmann Chart (20 percent opacity). EPA Method 9 is the method of compliance pursuant to Chapter 62-297. F.A.C. [Rule 62-296.320(4)(b)1. & 4., F.A.C.]  No person shall cause, let, permit, suffer or allow to be discharged into the atmosphere the emissions of air pollutants from any activity equal to or greater than 20% opacity.  Emissions from the following types of activities in Hillsborough County are further subject to a general 5% opacity standard:  loading or unloading of materials to or from containers such as rail cars, trucks, ships, storage structures and stockpiles; permanent conveyor systems; storage of materials in structures such as silos or enclosed bins, which have a storage capacity of fifty cubic yards or more; crushing, grinding, sizing and screening operations; and, static drop transfer points.  These regulations do not impose a specific testing requirement.  [Rules 62-296.320(4)(b)1, F.A.C. and Rule 1-3.52, HCEPC]

- Facility wide Condition No. FW7. was updated to clarify what should be submitted in the semiannual report.

FW7. Semiannual Report.  The permittee shall include in a semiannual report to the Environmental Protection Commission of Hillsborough County the monitoring and reporting specified in this permit and in Appendix CAM.  All instances of deviations from the permit requirements must be clearly identified in such reports.   [Rule 62-213.440(1)(b)3., F.A.C.]

- Facility wide Condition FW15. was updated to reflect the requirements of Rule 62-297.310(9), F.A.C.

## FW.15   The permittee shall notify the Air Compliance Section of the Environmental Protection Commission of Hillsborough County at least 15 days prior to the date on which each formal compliance test is to begin of the date, time, and place of each such test, and the contact person who will be responsible for coordinating an having such test conducted.  [Rule 62-297.310(7)(a)9., F.A.C.]

FW15.  At least 15 days prior to the date on which each required emissions test is to begin, the owner or operator shall notify the EPC, unless shorter notice is agreed to by the EPC. The notification shall include the date, time, place of each such test, Facility ID Number, Emission Unit ID Number(s) and description(s), Emission Point Number(s) and description(s), test method(s), pollutant(s) to be tested, along with the name and telephone number of the person who will be responsible for conducting such test(s) for the owner or operator. If a scheduled emissions test needs to be re-scheduled, the owner or operator shall submit to the EPC a revised notification at least seven days prior to the re-scheduled emissions test date or arrange a re-scheduled test date with the EPC by mutual agreement..  [Rule 62-297.310(9), F.A.C.]

- Specific Condition Nos. A.1.(a)i), (a)iv), (b)i), and (c)i) and A.3. were removed based on FDEP guidance because the potential VOC emissions are not based on an allowable emission limit from the State or Federal Rules.  In addition, Specific Condition Nos. A.1.(a)iv) is removed because the storage tanks have an overall throughput limit.

- Gasoline Tank Group (EU No. 005):

- Maximum allowable VOC emissions shall not exceed 59.3 tons per twelve consecutive month period

iv) The above VOC emission limit includes 0.1 tons resulting from the slop tank based on gasoline with a RVP 11 annual average as a worse-case estimate.  There is no throughput limitation for the slop tank, but all the petroleum contact water must have been generated at this facility.

- Distillate Tank Group (EU No. 002):

- Maximum allowable VOC emissions shall not exceed 2.4 tons per twelve consecutive month period.

(c)   Connectors, valves, pump seals, open-ended lines, and other components:

- Maximum allowable VOC emissions shall not exceed 0.2 tons per twelve consecutive month period

A.3.  The maximum allowable volatile organic compound (VOC) emissions for the tank farm shall not exceed 137.0 tons per twelve consecutive month period. [Rules 62-212.300, F.A.C. and Permit No. 0570081-017-AC and Title V Revision Application Received February 13, 2015]

{Permit Note:  Overall tank emissions include degassing emissions}

Conclusion

This project renews the Title V air operation permit No. 0570081-019-AV, which was effective on June 30, 2015.  This Title V air operation permit renewal is issued under the provisions of Chapter 403, Florida Statues (F.S.), and Chapters 62-4, 62-210, and 62-213, F.A.C.