| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 006 | No. 5 Power Boiler | Carb | 8,760 | Particulate Matter | 0.3 lb/MMBtu | 137.1 | 598.9 | 62-296.410(1)(b)2., F.A.C. | III.A.5.a. |
|  |  | Fuel Oil |  | Particulate Matter | 0.1 lb/MMBtu |  |  | 62-296.405(1)(b), F.A.C. | III.A.5.b.1 |
|  |  | Fuel Oil |  | Particulate Matter – Soot Blowing and Load Change | 0.3 lb/MMBtu for ≤ 3 hours/24 hours |  |  | 62-210.700(3), F.A.C. | III.A.5.b.2. |
|  |  | All Fuels |  | Sulfur Dioxide | 550.0 lb/hour based on a 24-hour rolling average |  |  | 62-296.340(5)(c), F.A.C. | III.A.6. |
|  |  | NCGs |  | TRS | 5 ppmvd@ 10% O2 (12-hr avg.) | 11.74 | 12.85 | 62-296.404(3)(f), F.A.C. | III.A.7. |
|  |  | Carb |  | VE | 30% Opacity except 40% is permissible for ≤ 2 minutes in any hour |  |  | 62-296.410(1)(b)1., F.A.C. | III.A.8.a. |
|  |  | Fuel Oil |  | VE | 20% Opacity except 40% is permissible for ≤ 2 minutes in any hour |  |  | 62-296.405(1)(a), F.A.C. | III.A.8.b. |
|  |  | Fuel Oil |  | VE – Soot Blowing or Load Change | 60% Opacity ≤ 3 hrs in any 24-hour period, w/ best oper. prctices & minimize excess emissions duration |  |  | 62-210.700(3), F.A.C. | III.A.8.c. |
|  |  | WW Wood Fiber Residuals |  | Mercury | No more than 7.1 pounds per 24 hour period |  |  | 40 CFR 61.52(b) | III.A.9. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 007 | No. 4 Recovery Boiler | BLS, BLS w/ ULSD, No. 2 Fuel Oil, No. 6 Fuel Oil | 8,760 | Particulate Matter | 0.044 gr/dscf@ 8% O2 |  |  | 40 CFR 63.862(a)(1)(i)(A) | III.B.5.a. |
|  |  |  |  | Particulate Matter | 3 lbs/3000 lbs BLS | 137.5 | 602.25 | 62-296.404(2)(a), F.A.C. | III.B.5.b. |
|  |  |  |  | Total Reduced Sulfur | 5 ppmvd @ 8% O2 | 3.24 | 14.19 | 62-296.404(3)(c)1.b., F.A.C. | III.B.6. |
|  |  |  |  | Visible Emissions | ≤ 45% Opacity except up to 60% Opacity no more than one six-minute period during any hour |  |  | 62-296.404(1)(a), F.A.C. | III.B.7.a. |
|  |  |  |  | Visible Emissions | ≤ 35% Opacity for < 6% of operating time w/n any quarterly period |  |  | 40 CFR 63.864(k)(2)(i) | III.B.13. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 011 | No. 5 Recovery Boiler | BLS, BLS w/ ULSD, No. 2 Fuel Oil, No. 6 Fuel Oil | 8,568 | Particulate Matter | 0.044 gr/dscf @ 8% O2 | 83.3 | 356.9 | 40 CFR 63.862(a)(1)(i)(A) | III.C.5. |
|  |  |  |  | Total Reduced Sulfur (Cross Recovery Furnace Operation Mode) | 25 ppmvd @ 8% O2 | 26.3 | 112.67 | 40 CFR 60.283(a)(3) | III.C.6. |
|  |  |  |  | Total Reduced Sulfur (Straight Recovery Furnace Operation Mode) | 5 ppmvd @ 8% O2 | 5.26 | 22.53 | 40 CFR 60.282(a)(1)(i) 40 CFR 60.283(a)(2) | III.C.7. |
|  |  |  |  | Visible Emissions | < 35% Opacity |  |  | 40 CFR 60.282(a)(1)(ii) | III.C.8. |
|  |  |  |  | Visible Emissions | ≤ 35% Opacity for < 6% of operating time w/n any quarterly period |  |  | 40 CFR 63.864(k)(2)(i) | III.C.18. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 013 | No. 4 Smelt Dissolving Tank | N/A | 8,760 | Particulate Matter | ≤ 0.20 lb/ton BLS |  |  | 62-296.320(4)(a)2., F.A.C. 40 CFR 63.862(a)(1)(i)(B) | III.D.4. |
|  |  |  |  | Total Reduced Sulfur | 0.048 lb TRS/3000 lb BLS | 2.2 | 9.64 | 62-296.404(3)(d)1., F.A.C. | III.D.5. |
|  |  |  |  | Visible Emissions | < 20% Opacity |  |  | 62-296.320(4)(b)1., F.A.C. | III.D.6. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 014 | No. 5 Smelt Dissolving Tank | N/A | 8,568 | Particulate Matter | ≤ 0.20 lb/ton BLS | 15.68 | 67.17 | 40 CFR 60.282(a)(2) 40 CFR 63.862(a)(1)(i)(B) | III.E.4. |
|  |  |  |  | Total Reduced Sulfur | 0.033 lb TRS/ton BLS | 2.59 | 11.08 | 40 CFR 60.283(a)(4) | III.E.5. |
|  |  |  |  | Visible Emissions | < 20% Opacity |  |  | 62-296.320(4)(b)1., F.A.C. | III.E.6. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 015 | No. 7 Power Boiler |  | 8,760 | Particulate Matter | 0.10 lb/MMBtu | 102.10 | 447.20 | 40 CFR 60.42(a)(1) | III.F.5. |
|  |  | Coal |  | Sulfur Dioxide | 1.2 lb/MMBtu | 1,225.20 | 5,366.38 | 40 CFR 60.43(a)(2) | III.F.6.a. |
|  |  | No. 6 No. 2 |  | Sulfur Dioxide | 0.8 lb/MMBtu |  |  | 40 CFR 60.43(a)(1) | III.F.6.b. |
|  |  | Coal |  | Nitrogen Oxides | 0.6 lb/MMBtu | 612.6 | 2,683.19 | 10/11/84 BACT Determination | III.F.7.b. |
|  |  | No. 6 No. 2 |  | Nitrogen Oxides | 0.3 lb/MMBtu |  |  | 40 CFR 60.44(a)(2) | III.F.7.a. |
|  |  | Natural Gas |  | Nitrogen Oxides | 0.2 lb/MMBtu |  |  | 40 CFR 60.44(a)(1) | III.F.7.d |
|  |  | Coal No. 6 No. 2 |  | Carbon Monoxide | 93.6 lbs/hr and 409.97 TPY |  |  | EPA/DER Agreement 40 CFR 52.21(j) | III.F.8. |
|  |  | Coal No. 6 No. 2 |  | Visible Emissions | ≤ 20% Opacity except for no more than 27% Opacity for one 6 min period/ hour |  |  | 40 CFR 60.42(a)(2) | III.F.9. |
|  | Coal Prep and Handling | N/A |  | Visible Emissions | < 20% Opacity |  |  | 40 CFR 60.254(a) PSD-FL-062 | III.F.10. |
|  | Fly Ash Handling | N/A |  | Visible Emissions | ≤ 5% Opacity |  |  | 40 CFR 52.21(j) PSD-FL-062 | III.F.10. |
|  |  | N/A |  | Particulate Matter | ≤ 0.5 lbs/hr |  |  | PSD-FL-062 | III.F.11. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 020 | Tall Oil Plant | N/A | 8,760 | Total Reduce Sulfur | 0.05 lb/ton Tall Oil Produced, 12-hr average | 0.43 | 1.9 | 62-296.404(3)(b)1., F.A.C. | III.G.4. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 021 | No. 4 Lime Kiln | No. 6 Fuel Oil, On-Spec Used Oil, Natural Gas; [LPG (Startup Only] | 8,736 | Particulate Matter | 0.064 gr/dscf @ 10% O2 |  |  | 40 CFR 63.862(a)(1)(i)(C) | III.H.5. |
|  |  |  |  | Total Reduced Sulfur | 8 ppmvd @ 10% O2 | 2.63 | 11.5 | 40 CFR 60.283(a)(5) | III.H.6. |
|  |  |  |  | Visible Emissions | ≤ 20% Opacity |  |  | 62-296.320(4)(b)1., F.A.C. | III.H.7. |
|  | No. 1 Lime Bin | N/A | 8,760 | Visible Emissions | ≤ 5% Opacity |  |  | 62-297.620(4), F.A.C. AC45-141878, Permit No. 0890003-036-AC | III.H.8. |
|  |  |  |  | Particulate Matter | ≤ 0.03 gr/dscf | 1.2 | 5.3 | AC45-141878, Permit No. 0890003-036-AC | III.H.9. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 024 | C-Line Brownstock Washer System | N/A | 8,760 | Total Reduced Sulfur | 5 ppmvd uncorrected | 0.16 | 0.7 | 40 CFR 60.283(a)(1)(v) | III.I.4. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 033 | Pulping System MACT I | N/A | 8,760 | HAP Condensate Collection | Collect All Named Stream Option (15-day average) |  |  | 40 CFR 63.446(c)(1) | III.J.5., J.6. |
|  |  |  |  | Total HAP Condensate Treatment | Total HAP destruction of at least 92% by weight |  |  | 40 CFR 63.446(e)(3) | III.J.10. |
|  |  |  |  | HAP Vent Collection | Named Sources enclosed and routed to No. 4 Lime Kiln or No. 5 Power Boiler |  |  | 40 CFR 63.443(a) 40 CFR 63.443(c) 40 CFR 63.443(d) | III.J.2. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or condition s of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 035 | Wide-Web Flexographic Printing Presses | N/A | 8760 | Organic HAP | 400 kg/month-applied usage |  |  | 40 CFR 63.821(b)(2) | III.K.1. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or condition s of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 038, 039, 040 | Reciprocating Internal Combustion Engines (RICE) | Diesel fuel | 8,760 | HAPs | Work Practice Standards- Operation and Maintenance |  |  | 40 CFR 63.6603(a) | III.L.6. |

|  | This table summarizes information for convenience purposes only. This table does not supersede any of the terms or condition s of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | E.P. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 041 | --- | Coal Prep and Handling | N/A | 8,760 | --- | --- | --- | --- | --- | --- |
|  | 03 | Crusher |  |  | Visible Emissions | < 10% Opacity | --- | --- | 40 CFR 60.254(b)(1) | III.O.4.a. |
|  | 01 02 04 05 06 07 08 09 | Screen Hopper Conveyor from Screen/Hopper to Vibrating Feeder Conveyor from Coal Crusher to Coal Silos Coal Silo No. 1 Coal Silo No. 2 Coal Collection Conveyor Coal Bunker Conveyor Coal Bunkers A, B, & C Common Stack |  |  | Visible Emissions | < 20% Opacity |  |  | 40 CFR 60.254(a) PSD-FL-062 | III.O.4.b. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of this permit. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date | Min. Compliance Test Duration | See Permit Condition(s) |
| 006 | No. 5 Power Boiler | Carb | Particulate Matter | N | ESP | EPA Method 5 | FFY | --- | 3, One-hr test runs | III.A.25.a. |
|  |  | Fuel Oil | Particulate Matter | N |  | EPA Method 5 |  | > 400 hrs/yr fuel oil fired | 3, One-hr test runs | IIIA.25.b.1. |
|  |  | Fuel Oil | Particulate Matter (Soot blowing or load change) | N |  | EPA Method 5 | FFY | > 400 hrs/yr fuel oil fired | 3, One-hr test runs | IIIA.25.b.2. |
|  |  | Fuel Oil | Sulfur Dioxide | Y |  | CEMS |  |  |  | IIIA.19.-A.24. |
|  |  | NCGs | Total Reduced Sulfur |  |  | Gases introduced w/primary fuel or into the flame zone, or with the combustion air, min temp of 1200 F, 0.5 sec residence time |  |  |  | III.A.27. |
|  |  | Carb | Visible Emissions |  |  | EPA Method 9 | FFY |  | 60 Minutes | III.A.28.a. |
|  |  | Fuel Oil | Visible Emissions |  |  | EPA Method 9 | FFY |  | 60 Minutes | III.A.28.b. |
|  |  | Fuel Oil | VE – Soot Blowing or Load Change |  |  | EPA Method 9 | FFY |  | 60 Minutes | III.A.28.c. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of this permit. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date | Min. Compliance Test Duration | See Permit Condition(s) |
| 007 | No. 4 Recovery Boiler | BLS, BLS w/ ULSD, No. 2 Fuel Oil, No. 6 Fuel Oil | Particulate Matter | Y COMS | ESP | EPA Method 5 | FFY | --- | 3, One-hr test runs | III.B.11., B.14. |
|  |  |  | Total Reduced Sulfur | Y |  | EPA Method 16 EPA Method 16A, EPA Method 16B, or EPA Method 16C | Prior to permit renewal |  | 3 hours, but no longer than 6 hours | III.B.19.a. |
|  |  |  | Visible Emissions |  |  | EPA Method 9 | FFY |  | 60 Minutes | III.B.18. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of this permit. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date | Min. Compliance Test Duration | See Permit Condition(s) |
| 011 | No. 5 Recovery Boiler | BLS, BLS w/ ULSD, No. 2 Fuel Oil, No. 6 Fuel Oil | Particulate Matter | Y COMS | ESP | EPA Method 5 | FFY | --- | 3, One-hr test runs | III.C.19., C.15. |
|  |  |  | Total Reduced Sulfur | Y |  | EPA Method 16 EPA Method 16A, EPA Method 16B, or EPA Method 16C | Prior to permit renewal |  | 3 hours, but no longer than 6 hours | III.C.13., C.24. |
|  |  |  | Visible Emissions |  |  | EPA Method 9 | FFY |  | 60 Minutes | III.C.23. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of this permit. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date | Min. Compliance Test Duration | See Permit Condition(s) |
| 013 | No. 4 Smelt Dissolving Tank | N/A | Particulate Matter | N | Scrubber | EPA Method 5 | FFY | --- | 3, One-hr test runs | III.D.17. |
|  |  |  | Total Reduced Sulfur | N |  | EPA Method 16 EPA Method 16A EPA Method 16B EPA Method 16C | Prior to permit renewal |  | 3 hours, but no longer than 6 hours | III.D.21. |
|  |  |  | Visible Emissions |  |  | EPA Method 9 | FFY |  | 60 Minutes | III.D.19. |
|  |  |  | TRS Surrogate | Y |  | Weak Wash Flow |  |  |  | III.D.112. |
|  |  |  | PM CPMS | Y |  | Pressure Drop across Scrubber |  |  |  | III.D.13. |
|  |  |  | PM CPMS | Y |  | Scrubbing Liquid Flow rate |  |  |  | III.D.13. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of this permit. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date | Min. Compliance Test Duration | See Permit Condition(s) |
| 014 | No. 5 Smelt Dissolving Tank | N/A | Particulate Matter | N | Scrubber | EPA Method 5 | FFY | --- | 3, One-hr test runs | III.E.18. |
|  |  |  | Total Reduced Sulfur | N | Scrubber | EPA Method 16 EPA Method 16A EPA Method 16B EPA Method 16C | Prior to permit renewal |  | 3 hours, but no longer than 6 hours | III.E.20.a., E.20.b. |
|  |  |  | Visible Emissions |  |  | EPA Method 9 | FFY |  | 60 Minutes | III.E.21., E.22. |
|  |  |  | TRS Surrogate | Y |  | Weak Wash Flow |  |  |  | E.13.c. |
|  |  |  | PM CPMS | Y |  | Pressure Drop across Scrubber |  |  |  | E.13.a., E.13.b. |
|  |  |  | PM CPMS | Y |  | Scrubbing Liquid Flow rate |  |  |  | E.13.a., E.13.b |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of this permit. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date | Min. Compliance Test Duration | See Permit Condition(s) |
| 015 | No. 7 Power Boiler | Coal, No. 6 Fuel oil, No. 2 Fuel Oil, Nat. Gas | Particulate Matter | Y COMS | ESP | EPA Method 5 | FFY | --- | 3, One-hr test runs | III.F.15., F.21. |
|  |  |  | Sulfur Dioxide | N |  | Fuel Sampling Each Coal Delivery Method 16B, t consistent with permit condition language werre EPA Method 6 | Each Coal Delivery FFY |  | 3, One-hr test runs | III.F.17., F.22. |
|  |  |  | Nitrogen Oxides | N |  | EPA Method 7 | FFY |  | 3, One-hr test runs | III.F.23. |
|  |  |  | Carbon Monoxide | N |  | EPA Method 10 | FFY |  | 3, One-hr test runs | III.F.25. |
|  |  |  | Visible Emissions | Y |  | EPA Method 9 | FFY |  | 60 Minutes | III.F.26. |
|  |  |  | NOx Surrogate | Y |  | Maximum O2 set point |  |  |  | III.F.19. |
|  |  |  | CO Surrogate | Y |  | Minimum O2 set point |  |  |  | III.F.20. |
|  | Coal Prep and Handling |  | Visible Emissions | N | Fabric Filter | EPA Method 9 | FFY |  |  | III.F.31. |
|  | Fly Ash Handling |  | Visible Emisisons | N | Fabric Filter | EPA Method 9 | FFY |  |  | III.F.28. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of this permit. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date | Min. Compliance Test Duration | See Permit Condition(s) |
| 020 | Tall Oil Plant | N/A | Total Reduced Sulfur | N | Scrubber | EPA Method 16 EPA Method 16A EPA Method 16B EPA Method 16C | Prior to Permit renewal | --- | 3 hours, but no longer than 6 hours | III.G.6. |
|  |  |  | TRS Surrogate Scrubber Liquid Flow Rate | Y |  | Scrubbing Liquid Flow rate |  |  |  | III.G.5. |
|  |  |  | TRS Surrogate pH | Y |  | 3 grab samples over course of each Tall Oil cook (beginning, middle, end) |  |  |  | III.G.5. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of this permit. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date | Min. Compliance Test Duration | See Permit Condition(s) |
| 021 | No. 4 Lime Kiln | No. 6 Fuel Oil, On-Spec Used Oil, LPG (Startup Only), Nat. Gas | Particulate Matter | Y COMS | ESP | EPA Method 5 | FFY | --- | 3, One-hour test runs | III.H.19., H.25. |
|  |  |  | TRS | Y |  | EPA Method 16 EPA Method 16A EPA Method 16B EPA Method 16C | FFY |  | 3 hours, but no longer than 6 hours | III.H.29. |
|  |  |  | Visible Emissions |  |  | EPA Method 9 | FFY |  | 60 Minutes | III.H.31. |
|  | No. 1 Lime Bin | N/A | Visible Emissions |  | Fabric Filter | EPA Method 9 | FFY |  | 30 Minutes | III.H.32. |
|  | No. 2 Lime Bin | N/A | Visible Emissions |  | Fabric Filter | EPA Method 9 | FFY |  | 30 Minutes | III.H.34. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of this permit. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date | Min. Compliance Test Duration | See Permit Condition(s) |
| 024 | C-Line Brownstock Washer System | N/A | Total Reduced Sulfur | Y | Scrubber | EPA Method 16 EPA Method 16A EPA Method 16B EPA Method 16C | Prior to Permit renewal | --- | 3 hours, but no longer than 6 hours | III.I.9. |
|  |  |  | TRS Surrogate Scrubber Liquid Flow Rate | Y |  | Scrubbing Liquid Flow rate |  |  |  | III.I.8. |
|  |  |  | TRS Surrogate Pressure Drop | Y |  | Pressure Drop across scrubber |  |  |  | III.I.8. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of this permit. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date | Min. Compliance Test Duration | See Permit Condition(s) |
| 033 | Pulping System MACT I | N/A | Condensate Collection | Y |  | All named stream option based on annual 15-day average test; quarterly 3-day test | Annual Quarterly testing |  |  | III.J.6., J.17., J.29. |
|  |  |  | Condensate Treatment | Y |  | Total HAP reduction of 92% or more by weight | Annual Quarterly testing |  |  | III.J.17., J.29. |
|  |  |  | UNOX Vent Gas Purity | Y |  |  |  |  |  | III.J.14., J.15. |
|  |  |  | UNOX Oxygen Feed | Y |  |  |  |  |  | III.J.15. |
|  |  |  | Clean Condensate Alternative Verify that the Effluent Header to the Cooling Tower Valve (Valve No. 3) is maintained in the closed position | Y |  | Replacement of the direct-contact WWTP cooling tower with a non-contact heat exchanger system |  |  |  | III. J.37, J.38, J.39. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of this permit. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date | Min. Compliance Test Duration | See Permit Condition(s) |
| 041 | Coal Prep and Handling | N/A | Coal Crusher | N |  | EPA Method 9 | 90 days or 12 mos. of previous test depending upon test results |  |  | III.O.5., O.10. |
|  |  |  | Screen Hopper Conveyor from Screen/Hopper to Vibrating Feeder Conveyor from Coal Crusher to Coal Silos Coal Silo No. 1 Coal Silo No. 2 Coal Collection Conveyor Coal Bunker Conveyor Coal Bunkers A, B, & C Common Stack | N |  | EPA Method 9 | FFY |  |  | III.O.9., O.10. |