# Hake Marine, LLC

# Hake Marine

# Facility ID No. 1030224

# Pinellas County

# Title V Air Operation Permit Revision

# Permit No. 1030224-014-AV

(Revision of Title V Air Operation Permit No. 1030224-011-AV)

# Permitting Authority:

State of Florida

Department of Environmental Protection

Air and Solid Waste Permitting, Southwest District

13051 North Telecom Parkway

Temple Terrace, Florida  33637-0926

# Telephone:  (813) 470-5700

Fax:  (813) 470-5996

E-mail: SWD_Air_Permitting@dep.state.fl.us

# Compliance Authority:

Pinellas County Air Quality Division

509 East Avenue South, Suite 138

Clearwater, Florida  33756

Telephone: (727) 464-4422

Fax: (727) 464-4420

E-mail: Airquality@pinellascounty.org

Title V Air Operation Permit Revision

Permit No. 1030224-014-AV

Table of Contents

Section	Page Number

Placard Page 	 1

- Facility Information.

- Facility Description. 	 2

- Summary of Emissions Units. 	 2

- Applicable Regulations. 	 2

- Facility-wide Conditions. 	 3

- Emissions Units and Conditions.

- EU No. 001 – Fiberglass Boat Manufacturing. 	 6

- Appendices. 	 14

Appendix A, Glossary.

Appendix I, List of Insignificant Emissions Units and/or Activities.

Appendix NESHAP 40 CFR 63, Subpart A – General Provisions.

Appendix NESHAP 40 CFR 63, Subpart VVVV – National Emissions Standards for Hazardous Air Pollutants for Boat Manufacturing

Appendix RR, Facility-wide Reporting Requirements.

Appendix TV, Title V General Conditions.

Referenced Attachments.

Table H, Permit History.

Permittee:	Permit No. 1030224-014-AV

Hake Marine, LLC	Hake Marine

14300 Clay Terrace Blvd, Ste # 246	Facility ID No. 1030224

Carmel, Indiana 46032	Title V Air Operation Permit Revision

The purpose of this permit is to revise the Title V air operation permit for the above referenced facility.  The existing Traditional Watercraft, Inc. is located in Pinellas County at 1979 and 2080 Wild Acres Road, Largo, Florida.  UTM Coordinates are:  Zone 17, 327.44 km East and 3087.18 km North.  Latitude is:  27053’54” North; and, Longitude is:  82045’12” West.

The Title V air operation permit is issued under the provisions of Chapter 403, Florida Statutes (F.S.), and Florida Administrative Code (F.A.C.) Chapters 62-4, 62-210, and 62-213.  The above named permittee is hereby authorized to operate the facility in accordance with the terms and conditions of this permit.

1030224-014-AV Effective Date:  12-06-2016

Renewal Application Due Date:  09-28-2018

Expiration Date:  05-12-2019

____________________________________________

Pamala Vazquez

Program Administrator

Permitting & Waste Cleanup Program

Southwest District

PV/qn/admin

Subsection A.  Facility Description.

This facility consists of a fiberglass boat manufacturing facility subject to the requirements of 40 CFR 63, Subpart VVVV-National Emission Standards for Hazardous Air Pollutants for Boat Manufacturing.  The facility has 2 sites: 1) 1979 Wild Acres Road and 2) 2080 Wild Acres Road.  Both sites are in Largo, Pinellas County.

Subsection B.  Summary of Emissions Units.

| EU No. | ## Brief Description |
| --- | --- |
| Regulated Emissions Units |  |
| 001 | Fiberglass Boat Manufacturing Facility |

Also included in this permit are miscellaneous insignificant emissions units and/or activities (see Appendix I, List of Insignificant Emissions Units and/or Activities).

Subsection C.  Applicable Regulations.

Based on the Title V air operation permit revision application received 6/20/2016, this facility is a major source of hazardous air pollutants (HAP).  A summary of applicable regulations is shown in the following table.

| Regulation | ## EU No. |
| --- | --- |
| Federal Rule Citations |  |
| 40 CFR 63, Subpart A, NESHAP General Provisions | 001 |
| 40 CFR 63, Subpart VVVV, National Emissions Standards for Hazardous Air Pollutants for Boat Manufacturing |  |
| State Rule Citations |  |
| Chapters 62-4, 62-210.200, 62-213.440, 62-296.320 | 001 |

The following conditions apply facility-wide to all emission units and activities:

- Appendices.  The permittee shall comply with all documents identified in Section IV, Appendices, listed in the Table of Contents.  Each document is an enforceable part of this permit unless otherwise indicated.  [Rule 62-213.440, F.A.C.]

Emissions and Controls

- Objectionable Odor Prohibited.  No person shall cause, suffer, allow or permit the discharge of air pollutants, which cause or contribute to an objectionable odor.  An “objectionable odor” means any odor present in the outdoor atmosphere which by itself or in combination with other odors, is or may be harmful or injurious to human health or welfare, which unreasonably interferes with the comfortable use and enjoyment of life or property, or which creates a nuisance.  [Rule 62-296.320(2) and 62-210.200(Definitions), F.A.C., Pinellas County Code, Section 58-178 and Construction Permit 1030224-010-AC]

- General Volatile Organic Compounds (VOC) Emissions or Organic Solvents (OS) Emissions.  The permittee shall allow no person to store, pump, handle, process, load, unload or use in any process or installation, volatile organic compounds or organic solvents without applying known and existing vapor emission control devices or systems deemed necessary and ordered by the Department.  The permittee shall comply with the following:

- The exhaust fan associated with the Gelcoat Booth, Polycore Booth, Small Parts Lamination activities in the enclosure between the glass building and tooling/mold storage building, and the Varnish Booth shall be operating whenever usage of materials containing VOC/OS occurs and shall remain operating for at least 2 hours after the usage has been completed.  When hull/deck gelcoating/lamination are occurring, the doors on the north and east sides of the building will be closed.  In order to ensure sufficient make up controlled airflow through the building to the exhaust stacks, the doors on the south side of the gelcoat/lamination area and the south side of the building (mill area) will be open to a maximum of 10 feet.  When activities require, one of the internal doors between the two building halves may be opened, but only when the south side (mill area) door is closed.

- When determining the “VOC emission rate”, the percent of solids in the recovered solvents and hazardous waste, shall be documented and subtracted from the amount of recovered solvents and hazardous waste.

- All materials containing VOC/OS shall be stored in closed containers and/or in small automatic closing safety cans.

- Clean-up solvents washings shall be directed into containers that prevent evaporation into the atmosphere.

- All equipment, pipes, hoses, lids, fittings, etc., shall be operated/maintained in such a manner as to minimize leaks, fugitive VOC/OS emissions and spills of materials containing VOC/OS.

- Work practice controls, when practicable, shall include limiting the amount of clean-up solvents issued to employees and employees shall use gloves whenever handling resins.

- Immediately attend to all spills of materials containing VOC/OS as appropriate.

- For each of the two (2) exhaust stacks in the glass shop (gelcoat booth and Polycore booth) and the exhaust stack associated with the small parts lamination in the enclosure, there shall be no device, which (a) reduces the vertical momentum of the stack gas, or (b) reduces the vertical dispersion of the stack gas.

[Rule 62-296.320(1), F.A.C., Construction Permit No. 1030224-010-AC]

- General Visible Emissions.  No person shall cause, let, permit, suffer or allow to be discharged into the atmosphere the emissions of air pollutants from any activity equal to or greater than 20% opacity.  This regulation does not impose a specific testing requirement.  [Rule 62-296.320(4)(b), F.A.C.]

- Unconfined Particulate Matter.  No person shall cause, let, permit, suffer or allow the emissions of unconfined particulate matter from any activity, including vehicular movement; transportation of materials; construction; alteration; demolition or wrecking; or industrially related activities such as loading, unloading, storing or handling; without taking reasonable precautions to prevent such emissions.  Reasonable precautions to prevent emissions of unconfined particulate matter at this facility include:

a.	Using enclosures and/or wind screens as necessary.

b.	Vacuuming and/or sweeping of the heavy sanding/grinding area as necessary.

c.	The buildings shall be kept reasonably clean and the dust controlled to reasonable levels by vacuuming and/or sweeping as necessary.

d.	Use the hoods, fans, filters and similar equipment to contain, capture and/or vent particulate matter, which shall include the use of filters in the Grinding/Trimming activities (enclosure), Gelcoat Spray Booth, Polycore Booth, and Cutting/Grinding Booth.

As an indicator that adequate control measures are being employed, visible emission s from exhaust fans, doors, and vents should not exceed five percent (5%) opacity.  If this level is exceeded, it shall not be considered a violation in and of itself, but may allow the Department to require additional controls or work practices.

[Rule 62-296.320(4)(c), F.A.C.; Construction Permit No. 1030224-010-AC]

Annual Reports and Fees

See Appendix RR, Facility-wide Reporting Requirements for additional details.

- Electronic Annual Operating Report and Title V Annual Emissions Fees.  The information required by the Annual Operating Report for Air Pollutant Emitting Facility [Including Title V Source Emissions Fee Calculation] (DEP Form No. 62-210.900(5)) shall be submitted by April 1 of each year, for the previous calendar year, to the Department of Environmental Protection’s (DEP) Division of Air Resource Management.  Each Title V source shall submit the annual operating report using the DEP’s Electronic Annual Operating Report (EAOR) software, unless the Title V source claims a technical or financial hardship by submitting DEP Form No. 62-210.900(5) to the DEP Division of Air Resource Management instead of using the reporting software.  Emissions shall be computed in accordance with the provisions of subsection 62-210.370(2), F.A.C.  Each Title V source must pay between January 15 and April 1 of each year an annual emissions fee in an amount determined as set forth in subsection 62-213.205(1), F.A.C.  The annual fee shall only apply to those regulated pollutants, except carbon monoxide and greenhouse gases, for which an allowable numeric emission-limiting standard is specified in the source’s most recent construction permit or operation permit.  Upon completing the required EAOR entries, the EAOR Title V Fee Invoice can be printed by the source showing which of the reported emissions are subject to the fee and the total Title V Annual Emissions Fee that is due.  The submission of the annual Title V emissions fee payment is also due (postmarked) by April 1st of each year.  A copy of the system-generated EAOR Title V Annual Emissions Fee Invoice and the indicated total fee shall be submitted to:  Major Air Pollution Source Annual Emissions Fee, Post Office Box 3070, Tallahassee, Florida 32315-3070.  Additional information is available by accessing the Title V Annual Emissions Fee On-line Information Center at the following Internet web site:  http://www.dep.state.fl.us/air/emission/tvfee.htm.  [Rules 62-210.370(3), 62-210.900 & 62-213.205, F.A.C.; and, §403.0872(11), Florida Statutes (2013)]

{Permitting Note:  Resources to help you complete your AOR are available on the electronic AOR (EAOR) website at:  http://www.dep.state.fl.us/air/emission/eaor.  If you have questions or need assistance after reviewing the information posted on the EAOR website, please contact the Department by phone at (850) 717-9000 or email at eaor@dep.state.fl.us.}

{Permitting Note:  The Title V Annual Emissions Fee form (DEP Form No. 62-213.900(1)) has been repealed.  A separate Annual Emissions Fee form is no longer required to be submitted by March 1st each year.}

- Annual Statement of Compliance.  The permittee shall submit an annual statement of compliance to the compliance authority at the address shown on the cover of this permit within 60 days after the end of each calendar year during which the Title V permit was effective.  [Rules 62-213.440(3)(a)2. & 3. and (b), F.A.C.]

FW8.	Prevention of Accidental Releases (Section 112(r) of CAA).  If, and when, the facility becomes subject to 112(r), the permittee shall:

- Submit its Risk Management Plan (RMP) to the Chemical Emergency Preparedness and Prevention Office (CEPPO) RMP Reporting Center.  Any Risk Management Plans, original submittals, revisions or updates to submittals, should be sent electronically through EPA’s Central Data Exchange system at the following address:  https://cdx.epa.gov.  Information on electronically submitting risk management plans using the Central Data Exchange system is available at:  http://www2.epa.gov/rmp.  The RMP Reporting Center can be contacted at:  RMP Reporting Center, Post Office Box 10162, Fairfax, VA  22038, Telephone:  (703) 227-7650.

- Submit to the permitting authority Title V certification forms or a compliance schedule in accordance with Rule 62-213.440(2), F.A.C.

[40 CFR 68]

The specific conditions in this section apply to the following emissions unit:

| EU No. | Brief Description |
| --- | --- |
| 001 | Fiberglass Boat Manufacturing Facility |

Fiberglass Boat Manufacturing Facility - The site at 1979 Wild Acres Road consists of two (2) main buildings (glass building and tooling/mold storage building) and an enclosure between these two (2) buildings.  The main lamination activities and curing are conducted in the glass building.  Inside the glass building is a cutting/grinding booth.  The main production resin (gp resin) is received into a storage tank located within an enclosed structure at the northwest corner of the glass building.  Small parts lamination, grinding, and trimming are conducted in the area between the glass building and tooling/mold storage building.  Hake Marine, LLC will not at present occupy or conduct any activities in the former tooling/mold storage building.

After a fiberglass reinforced boat and other related activities are designed, molds for the fiberglass components are built.  After applying a wax to the surface of the molds, they are sprayed with (typically pigmented) styrene-based gelcoat in the gelcoat spray booth located in the northeast corner of the glass building.  When the gelcoat has sufficiently solidified, fiberglass material is placed in the mold and polyester resin is applied to the fiberglass with hand-held, flow through pressure fed rollers.  The resin is catalyzed at a point just before it reaches the handle of the roller.  The required thickness of fiberglass and resin are built up using this method.  After curing, the hardened fiberglass hull and deck parts are removed from the mold and put on a cradle..  On some of the parts with large, flat horizontal surfaces, non-structural thickness is built up using putty applied by hand or by nonatomized applicator.

Closed molding techniques are used for some parts fabrication.  In this procedure the gelcoat is applied to the surface of the mold in a traditional “open molding” method; fiberglass cloth or mat is then laid over the gelcoated surface and a second “half” of the mold is placed over the fiberglass.  The edges are sealed except for a point where the resin is introduced.  A vacuum is pulled on the part for several hours to verify the seal integrity.  Catalyst is introduced into the resin drums and/or pails and thoroughly mixed prior to infusion into the part.  The vacuum draws the catalyzed resin into the part until it appears at the exit hose at which point the flow will be terminated.  Vacuum will continue to be applied overnight.  After curing over night the part is removed from the mold.  It is expected that closed molding (infusion) construction will be used for more components and will represent an increasing fraction of total manufacturing.  Closed molding operations meet the criteria for insignificant activity specified in Rule 62 -213.430(6)(b), F.A.C., specifically that the activity must emit less than 1000 lbs/yr of any HAP.  Although there is no regulatory-accepted emission factor for infusion, a very conservative value of 0.01 lbs emitted/lb applied, which is one tenth of the open molding factor for nonatomized spay may be used with the MACT maximum allowable styrene content of 35% as an illustration.

Using these values, 100,000 lbs resin x 35 wt % styrene x 0.01 lbs emitted/lb applied = 350 lbs styrene (HAP) emitted.

Excess material is trimmed from the parts in the grinding booth which is equipped with a dust collection system.  Additional, lesser amounts of grinding may be performed in the main building or glass shop.  All of these areas are equipped with dust filters.  No particulates are discharged outside the building from the grinding booth. These grinding and trimming activities are considered exempt.

Emissions from the grinding and trimming activities in the enclosed booth located in the area between the glass building and tooling/mold storage building (to remain at 1979 Wild Acres Road) are controlled by a three-stage filter system, which returns the filtered air back into the enclosure.

There are exhaust points (with fans) from:

The Putty Booth located in the glass building;

The gelcoat booth located in the glass building; and

The lamination activities located in the area between the glass building and tooling/mold storage building.

The site at 2080 Wild Acres Road consists of Assembly Building No. 4 (south) and Assembly Building No. 5 (north), and a separate building (west) with a wood mill (shop).  Sand blast booth and paint booth/plenum are relocated in the Assembly Building No. 4 (south) from 1979 Wild Acres Road Site.

At the assembly buildings, the fiberglass parts are prepped for construction of the boat.  Hatches, deck equipment and wood trim are installed on the deck part while it is still on a cradle.  Hulls are ballasted with steel, and the plywood floor structure is fiberglassed in place.  Then the IGU (internal glass unit), which is a base for the boat's interior, is fiberglassed into the hull.  Wood bulkheads, cabinets and machinery are then fastened and/or fiberglassed to the IGU/hull.

Bulkheads and cabinets are pre-assembled in the wood mill.  Some of these wood components are varnished in the varnish booth located in the northeast corner of Assembly Building No. 4.  When most of the interior is finished, the deck is joined to the hull using stainless steel fasteners and a sealant.  The yacht boat is then cleaned up; the remaining unfinished wood is either varnished or oiled, and preparation for shipping proceeds.

The wood mill utilizes two (2) small baghouses, which exhaust inside the building, to capture particulates from various pieces of equipment.

There is an exhaust point (with a fan) from the varnish booth.

Sources of unconfined particulate emissions at the facility include sawing, grinding, sanding, and similar operations.

{Permitting Note:  This emission unit is/or regulated under 40 CFR 63, Subpart VVVV – National Emissions Standards for Hazardous Air Pollutants for Boat Manufacturing}

Essential Potential to Emit (PTE) Parameters

- Hours of Operation.  This emissions unit may operate continuously (8,760 hours/year).

[Rule 62-210.200(PTE), F.A.C.; and, Construction Permit No. 1030224-010-AC]

Emission Limitations and Standards

- VOC Emissions.  Volatile Organic Compound (VOC) emissions shall not exceed 99 tons per any consecutive 12-month period

[Rule 62-210.200 (Definition “Potential to Emit”), F.A.C.; Construction Permit No. 1030224-010-AC]

- HAP Emissions.  Hazardous Air Pollutants (HAP) emissions shall not exceed 99 tons per any consecutive 12-month period.

[Rule 62-210.200 (Definition “Potential to Emit”), F.A.C.; Construction Permit No. 1030224-010-AC]

Recordkeeping and Reporting Requirements

A.4.	Recordkeeping Requirements – Compliance with the emission limitations of Specific Condition No. A.2. shall be documented on a monthly basis, based upon usage of all VOC/HAP-containing materials (e.g., resin, gelcoat, putties, fillers, etc.).  Monthly usage of materials may be based upon an inventory at the beginning of the month, receipt of material during the month, and inventory of the materials at the end of the month, provided the material is not used in multiple application methods with different emission factors.  All logs shall include the Facility name and Facility ID (1030224).  The emission summary shall show the following:

a.  The total VOC emissions for the month, in tons;

b.  The total HAP emissions for the month, in tons;

c.  The total VOC emissions, in tons, for the most recent consecutive 12-month period;

d.  The total HAP emissions, in tons, for the most recent consecutive 12-month period;

General Recordkeeping Requirements

e.  The method of application (where applicable) for each material used (see Table below);

f.  Emissions of styrene, methyl methacrylate (MMA), and other HAPs and VOCs shall be determined as outlined in Specific Condition No. A4., until notified by the Department or the Pinellas County Air Quality Division.  Note, styrene and MMA are each considered both a VOC and a HAP.

g.  Each log, where applicable, shall have attached the documentation for all chemicals captured for disposal or recycle.

h.  When determining the VOC emissions and HAP emission rates, the percent of solids in the recovered materials shall be documented and accounted for in the emission calculations, if applicable.

i.  The monthly logs shall be completed by the end of the following month.  Supporting documentation (Material Safety Data Sheets, purchase orders, etc.) shall be kept for each material which includes sufficient information to determine VOC emissions and HAP emissions.

j.  The VOC and HAP emission summary, calculations, and records used to determine emissions shall be recorded in a permanent form suitable for inspection by the Department or the Pinellas County Air Quality Division upon request. The logs/records shall be kept at the facility for a minimum of five (5) years.

[Rules 62-4.070(3) and 62-213.440(1)(b), F.A.C.; Construction Permit No. 1030224-010-AC]

A.5.	Emissions Calculation Requirements – Emissions of Styrene, Methyl methacrylate (MMA), and other volatile organic compounds (VOC) shall be determined for open molding activities as follows until notified by the Department.  Note, Styrene and MMA are each considered a VOC and a HAP.

1.	Styrene emissions shall be determined using the following equation:

where:

Ms = amount of styrene containing material used (in tons)

EF = emission factor (lb/ton) for styrene monomer content

(from table below)

The applicable styrene emission factor shall be obtained (interpolated/extrapolated, if applicable) from the following table, in conjunction with the percent of available non-vapor suppressed (NVS) styrene monomer in the resin/gelcoat:

| % Monomer | Resin** Hand (Non-Spray) Layup | Resin** Mechanical Atomized Spray Layup | Resin Mechanical Nonatomized* Spray Layup | Gel Coat** Mechanical Atomized Spray Layup | Gelcoat Mechanical Nonatomized* Spray Layup |
| --- | --- | --- | --- | --- | --- |
| 18 | 39 | 51 | 39 | 138 | 67 |
| 19 | 41 | 54 | 41 | 148 | 70 |
| 20 | 43 | 58 | 43 | 158 | 79 |
| 21 | 45 | 61 | 45 | 168 | 88 |
| 22 | 48 | 64 | 47 | 178 | 97 |
| 23 | 50 | 68 | 49 | 189 | 106 |
| 24 | 52 | 71 | 51 | 200 | 115 |
| 25 | 55 | 75 | 54 | 211 | 124 |
| 26 | 57 | 78 | 56 | 221 | 133 |
| 27 | 59 | 82 | 58 | 233 | 142 |
| 28 | 62 | 86 | 60 | 244 | 151 |
| 29 | 64 | 90 | 62 | 256 | 160 |
| 30 | 67 | 94 | 64 | 269 | 169 |
| 31 | 69 | 97 | 66 | 281 | 178 |
| 32 | 71 | 102 | 68 | 295 | 187 |
| 33 | 74 | 105 | 71 | 308 | 196 |
| 34 | 77 | 112 | 74 | 322 | 205 |
| 35 | 80 | 118 | 77 | 336 | 214 |
| 36 | 83 | 127 | 80 | 354 | 223 |
| 37 | 86 | 136 | 83 | 371 | 232 |
| 38 | 90 | 146 | 86 | 390 | 241 |
| 39 | 92 | 156 | 89 | 408 | 250 |
| 40 | 95 | 166 | 93 | 427 | 259 |
| 41 | 98 | 176 | 96 | 446 | 268 |
| 42 | 100 | 187 | 99 | 466 | 278 |
| 43 | 112 | 192 | 102 | 477 | 287 |
| 44 | 117 | 202 | 105 | 495 | 296 |
| 45 | 122 | 212 | 108 | 513 | 305 |
| 46 | 127 | 221 | 111 | 532 | 314 |
| 47 | 133 | 232 | 115 | 551 | 323 |
| 48 | 138 | 242 | 118 | 571 | 332 |
| 49 | 143 | 252 | 121 | 590 | 341 |
| 50 | 148 | 263 | 124 | 609 | 350 |

*	Nonatomized means any application technology in which the resin is not broken into droplets or an aerosol as it travels from the application equipment to the surface of the part. Nonatomized resin application technology includes, but is not limited to, flowcoaters, chopper flowcoaters, and pressure fed resin rollers.  In addition, the device must be operated according to the manufacturer’s directions, including instructions to prevent the operation of the device at excessive spray pressures.

**	The emission factors in these columns are based on a weighted average using the following assumptions:

-	Using the Unified Emission Factors - 20% by weight of the facility's usage of resins and gelcoats are for reinforced plastic composites production for items such as hatches and doors, but not for hulls and decks.

-	Using the National Marine Manufacturer Association's Factors – 80% by weight of the facility's usage of resins and gelcoats are for items such as decks and hulls.

2.	MMA emissions shall be determined by the following equations:

where:

Mmma = amount of MMA containing material used (in tons)

Cmma = MMA content (percent/100)

3.	VOC and HAP emissions for all other materials shall be determined by the following equation for each material.  These values shall be used in conjunction with other emissions values in this specific condition to determine total VOC/HAP emissions:

where:

Mvoc/HAP = amount of VOC/HAP-containing material used (in tons)

Cvoc/HAP = VOC/HAP content (percent/100)

EF = emission factor for the individual VOC/HAP-containing material

**	Emissions factor shall be 1.0 except for certain reactive species (e.g. isocyanates,   peroxides, and phthalates).  Emissions factors other than 1.0 must be approved by the Department.

[Rule 62-213.440(1)(b), F.A.C.; Construction Permit 1030224-010-AC]

A.6.	Other Reporting Requirements.  See Appendix RR, Facility-Wide Reporting Requirements, for additional reporting requirements.

[Rule 62-213.440(1)(b), F.A.C.]

Other Requirements

A.7.	Federal Rule Requirements - In addition to the specific conditions listed above, this emissions unit is also subject to the applicable provisions of 40 CFR 63, Subparts A - General Provisions to 40 CFR 63 and VVVV - National Emission Standards for Hazardous Air Pollutants for Boat Manufacturing as adopted and incorporated by reference in Rule 62-204.800, F.A.C.  The requirements listed below are based on the Title V permit revision application received on June 29, 2016.  When demonstrating compliance with 40 CFR 63, Subpart VVVV, either the Emissions Averaging Option or the Compliant Materials Option may be used, but each time a compliance option is used, it must be used for a minimum of 12 consecutive months before switching to the other option.

40 CFR 63, Subpart VVVV - National Emission Standards for Hazardous Air Pollutants for Boat Manufacturing - Applicable Provision Requirements

Based on Federal Register dated 8/22/01, revised 12/8/01 to reflect 10/03/01 Federal Register revision.

What This Subpart Covers

§63.5680 What is the purpose of this subpart?

§63.5683 Does this subpart apply to me?

§63.5686 How do I demonstrate that my facility is not a major source?

Not applicable.

§63.5689 What parts of my facility are covered by this subpart?

§63.5692 How do I know if my boat manufacturing facility is a new source or an existing source?

§63.5695 When must I comply with this subpart?

# Standards for Open Molding Resin and Gel Coat Operations

§63.5698 What emission limit must I meet for open molding resin and gel coat operations?

§63.5701 What are my options for complying with the open molding emission limit?

§63.5704 What are the general requirements for complying with the open molding emission limit?

§63.5707 What is an implementation plan for open molding operations and when do I need to prepare one?

§63.5710 How do I demonstrate compliance using emissions averaging?

§63.5713 How do I demonstrate compliance using compliant materials?

§63.5714 How do I demonstrate compliance if I use filled resins?

# Demonstrating Compliance for Open Molding Operations Controlled by Add-on Control Devices

§63.5715 What operating limits must I meet?

Not applicable.

§63.5716 When must I conduct a performance test?

Not applicable.

§63.5719 How do I conduct a performance test?

Not applicable.

§63.5722 How do I use the performance test data to demonstrate initial compliance?

Not applicable.

§63.5725 What are the requirements for monitoring and demonstrating continuous compliance?

Not applicable.

# Standards for Closed Molding Resin Operations

§63.5728 What standards must I meet for closed molding resin operations?

# Standards for Resin and Gel Coat Mixing Operations

§63.5731 What standards must I meet for resin and gel coat mixing operations?

# Standards for Resin and Gel Coat Application Equipment Cleaning Operations

§63.5734 What standards must I meet for resin and gel coat application equipment cleaning operations?

§63.5737 How do I demonstrate compliance with the resin and gel coat application equipment cleaning standards?

# Standards for Carpet and Fabric Adhesive Operations

§63.5740 What emission limit must I meet for carpet and fabric adhesive operations?

# Standards for Aluminum Recreational Boat Surface Coating Operations

§63.5743 What standards must I meet for aluminum recreational boat surface coating operations?

Not applicable.

§63.5746 How do I demonstrate compliance with the emission limits for aluminum wipedown solvents and aluminum coatings?

Not applicable.

§63.5749 How do I calculate the organic HAP content of aluminum wipedown solvents?

Not applicable.

§63.5752 How do I calculate the organic HAP content of aluminum recreational boat surface coatings?

Not applicable.

§63.5753 How do I calculate the combined organic HAP content of aluminum wipedown solvents and aluminum recreational boat surface coatings?

Not applicable.

§63.5755 How do I demonstrate compliance with the aluminum recreational boat surface coating spray gun cleaning work practice standards?

Not applicable.

# Methods for Determining Hazardous Air Pollutant Content

§63.5758 How do I determine the organic HAP content of materials?

# Notifications, Reports, and Records

§63.5761 What notifications must I submit and when?

§63.5764 What reports must I submit and when?

§63.5767 What records must I keep?

§63.5770 In what form and for how long must I keep my records?

# Other Information You Need To Know

§63.5773 What parts of the General Provisions apply to me?

§63.5776 Who implements and enforces this subpart?

# Definitions

§63.5779 What definitions apply to this subpart?

Table 1 to Subpart VVVV - Compliance Dates for New and Existing Boat Manufacturing Facilities

Table 2 to Subpart VVVV - Alternative HAP Content Requirements for Open Molding Resin and Gel Coat Operations

Table 3 to Subpart VVVV - MACT Model Point Value Formulas for Open Molding Operations

Table 4 to Subpart VVVV - Operating Limits If Using an Add-on Control Device for Open Molding Operations

Not applicable.

Table 5 to Subpart VVVV - Default Organic HAP Contents of Solvents and Solvent Blends

Table 6 to Subpart VVVV - Default Organic HAP Contents of Petroleum Solvent Groups

Table 7 to Subpart VVVV - Applicability and Timing of Notifications

Table 8 to Subpart VVVV - Applicability of General Provisions (40 CFR Part 63, Subpart A)

# [Rule 62-204.800, F.A.C.; 40 CFR 63, Subparts A and VVVV]

Appendix A, Glossary.

Appendix I, List of Insignificant Emissions Units and/or Activities.

Appendix NESHAP 40 CFR 63, Subpart A – General Provisions.

Appendix NESHAP 40 CFR 63, Subpart VVVV – National Emissions Standards for Hazardous Air Pollutants for Boat Manufacturing

Appendix RR, Facility-wide Reporting Requirements.

Appendix TV, Title V General Conditions.