Title V Air Operation Permit Revision

Permit No. 1070039-029-AV

Applicant

The applicant for this project is Continental Palatka, LLC.  The applicant’s responsible official and mailing address are:  Debbie Master, Plant Manager, Continental Palatka, LLC, 886 North Highway 17, Palatka, Florida 32177.

Facility Description

The applicant operates the existing Continental Palatka, which is located in Putnam County at 886 North Highway 17 in Palatka, Florida.

The facility is a Title V major source but not a Prevention of Significant Deterioration major source.  The facility is an area source of HAPs.  The facility produces synthetic gypsum wallboard.  Facility activities include receiving feedstock in the form of raw gypsum and other minor materials followed by the processing of feedstock to produce synthetic gypsum wallboard.  Two cage mill dryers reduce from about 10% to about 1% the moisture content of the gypsum.  Two calciner systems convert the gypsum to stucco.  The stucco forms the wallboard.

The existing facility regulated units consist of two 55-ton FGD Surge Bins; two Cage Mill Dryer Systems; two 170-ton Imp Mill Feed Silos; two Imp Mill Flash Calciner Systems; two Imp Mill Air Cooling Systems; two Stucco Silos (690-tons each); Starch Silo; Sawing Systems/Dunnage Machines; Belt Conveyors and Bucket Elevators; Emergency Live Bottom Feed Hopper; Wallboard Dryer - 4 Natural Gas Burners (186 MMBtu/hr); Ball Mills; Landplaster Bin, Additive System and Pin Mixer; Reclaim Processing and Screening; two Perkins diesel fired emergency engines (219 HP and 166 HP); and a John Deere Water Pump (275 HP).

The purpose of this permit is to revise Title V air operation permit to incorporate Construction Permit No. 1070039-027-AC (Railcar unloading station with conveyor transport system of inbound gypsum) for the above referenced facility.  Project PTE PM, PM10 and PM2.5 emissions are 2.03, 0.96 and 0.15 tpy, respectively.

This facility also includes miscellaneous unregulated/insignificant emissions units and/or activities.

regulated Emissions Unit IDentification Numbers and descriptions

| EU | Description | Emission Point | Filter System |
| --- | --- | --- | --- |
| Regulated Emissions Units and associated Emission Points |  |  |  |
| 001 | FGD Surge Bin #1 (55-ton Bin) | EP-01 | FGD Surge Bin Filter |
| 002 | Cage Mill Dryer System #1 | EP-02 | Dust Collector |
| 003 | Imp Mill Feed Silo A (170-ton Bin) | EP-02/023 | Dust Collector/Bin Filter |
| 004 | Imp Mill Flash Calciner System A | EP-04 | Dust Collector |
| 005 | Imp Mill Air Cooling System A | EP-05 | System Collector |
| 006 | Stucco Silo A (690-ton Bin) | EP-06 | Silo Filter |
| 007 | Starch Silo | EP-07 | Starch Silo Vent Filter |
| 008 | Sawing Systems/Dunnage Machines | EP-16 | End Trim Dust Collector |
| 010 | Belt Conveyors and Bucket Elevators. | Fugitive* | None |
| 011 | Emergency Live Bottom Feed Hopper. | EP-011 | BC-9 Nuisance Dust Collector |
| 013 | Wallboard Dryer (4 Natural Gas Burners) | EP-013A EP-013B | None |
|  |  |  | None |
| 014 | Ball Mills | EP-014 | BMA Dust Collector |
| 016 | Landplaster Bin | EP-016 | End Trim Dust Collector |
| 017 | Additives System and Pin Mixer | EP-017 | Nuisance Dust Collector |
| 018 | Imp Mill Feed Silo B (170-ton Bin) | EP-02/023 | Dust Collector/Bin Filter |
| 019 | Imp Mill Flash Calciner System B | EP-019 | Dust Collector |
| EU | Description | Emission Point | Filter System |
| 020 | Imp Mill Cooling System B | EP-020 | System Filter |
| 021 | Stucco Silo B (690-ton Bin) | EP-021 | Silo Filter |
| 022 | FGD Surge Bin (55-ton Bin) #2 | EP-022 | FGD Surge Bin Filter |
| 023 | Cage Mill Dryer System #2 | EP-023 | Dust Collector |
| 024 | Reclaim Processing and Screening | EP-024 | None |
| 028 | Perkins diesel fired emergency engine (219 HP) | EP-028 | N/A |
| 029 | Perkins diesel fired emergency engine (166HP) | EP-029 | N/A |
| 030 | John Deere Water Pump | EP 030 | N/A |
| 031 | Railcar unloading | Fugitive* | N/A |
| 032 | Feed hopper to belt C1 | Fugitive* | N/A |
| 033 | Transfer point belt C1 to C2 | Fugitive* | N/A |
| 034 | Transfer point belt C2 to C3 | Fugitive* | N/A |
| 035 | Transfer point belt C3 to TRS | Fugitive* | N/A |
| 036 | TRS to Stockpile | Fugitive* | N/A |
| Unregulated Emissions Units and Activities |  |  |  |
|  | Paved & unpaved roads (fugitive emissions) |  |  |
|  | Raw material & product storage piles, conveying & handling |  |  |
|  | Wood and metalworking shops |  |  |
|  | Painting operations/paint shop 1 |  |  |
|  | Sandblasting 2 |  |  |
|  | Pipe leaks 3 |  |  |
|  | Pump seals 3 |  |  |
|  | Packing leaks |  |  |
|  | Unconfined particulate from road dust/ miscellaneous sources |  |  |
|  | Lubricating oil reservoirs |  |  |
|  | Fire training |  |  |
|  | Loading/unloading/storage of packaged materials |  |  |
|  | Lab vents |  |  |
|  | Machine shops |  |  |
|  | Refueling |  |  |
|  | Space heaters |  |  |
|  | General purpose painting |  |  |
|  | Gypsum handling and storage system 4 |  |  |
|  | Limestone handling and storage system 4 |  |  |

1 Not to exceed HAP and VOC rule reporting thresholds.

2 With the exception of off property drift.

3 In accordance with manufacturer’s specified allowances under normal operation.

4 Unregulated with the exception of compliance with Facility-Wide Condition No. 3.

* - The term “Fugitive” identifies that emissions do not exit from a stack.  Whether the emissions count for purposes of PSD applicability requires a separate analysis.

Applicable Regulations

Based on the Title V air operation permit revision application received on November 13, 2018, this facility is not a major source of hazardous air pollutants (HAP).  The existing facility is not a prevention of significant deterioration (PSD) major source of air pollutants in accordance with Rule 62-212.400, F.A.C., as the PM emission rate (0.017 gr/dscfm) was requested by Continental for the Cage Mill Dryer Systems #1 and #2 to avoid the facility from previously classified as a PSD major source.  The PM emission rate of 0.017 gr/dscfm effectively limits facility emissions to less than 250 tons per year (tpy).  A summary of applicable regulations is shown in the following table:

| Regulation | ## EU No(s). |
| --- | --- |
| Federal Rule Citations |  |
| 40 CFR 60, NSPS Subpart A - General Provisions | EUs: 001, 002, 003, 004, 010, 011, 014, 016, 018, 019, 022, 023, 024, 031-035 |
| 40 CFR 60, NSPS Subpart OOO - Standards of Performance for Nonmetallic Mineral Processing Plants | EUs: 001, 003, 004, 010, 011, 014, 016, 018, 019, 022, 024, 031-035 |
|  | EU: 036 (the TRS conveyor belt is an NSPS subpart OOO regulated EU, but it is not subject to an emissions standard) |
| 40 CFR 60, NSPS Subpart UUU – Standards of Performance for Calciners and Dryers in Mineral Industries | EUs: 002, 023 |
| 40 CFR 63, NESHAP Subpart A - General Provisions | EUs: 028, 029, 030 |
| 40 CFR 63, NESHAP Subpart ZZZZ - National Emissions Standards for Hazardous Air Pollutants for Stationary Reciprocating Internal Combustion Engines | EUs: 028, 029, 030 |
| State Rule Citations |  |
| State Rule Citations (Rule 62-212.400(12), 62-296; 62-297.310(7); F.A.C.) | EUs: 001, 002, 003, 004, 005, 006, 007, 008, 010, 011, 013, 014, 016, 017,018, 019, 020, 021,022, 023, 024 |

Facility Process Flow Diagram

Project DESCRIPTION

The purpose of this permit is to revise Title V air operation permit to incorporate Construction Permit No. 1070039-027-AC (Railcar unloading station with conveyor transport system of inbound gypsum) for the above referenced facility.

Information:

Continental Palatka constructed pursuant to Air Construction Permit No. 1070039-027-AC a railcar unloading station followed by a conveyor transport system of gypsum received by railcar.  A railcar shaker unloads the gypsum from the railcars into an underground feeder (EU 031).  The feeder meters the gypsum at a maximum rate of 1,200 tons per hour (TPH) onto conveyor belt C1 (EU 032).  Conveyor belt C1 (550 feet long) transfers the gypsum onto conveyor belt C2 (EU 033). Conveyor belt C2 (600 feet long) transfers the gypsum onto conveyor belt C3 (EU 034). Conveyor Belt C3 (180 feet long) transfers the gypsum to a telescoping radial stacker (TRS) (170 feet long) (EU 035). The TRS discharges the gypsum onto the existing outdoor storage pile (EU 036).

The railcar unloading station (EU 031) and transfer points (EUs 032-035) are regulated by 40 CFR 60, NSPS Subpart OOO – Standards of Performance for Nonmetallic Mineral Processing Plants and 40 CFR 60, NSPS Subpart A – General Provisions.  EUs 031-035 are each subject to an opacity standard of 7% that applies to fugitive emissions from NSPS Subpart OOO affected facilities constructed on or after April 22, 2008.

Processing Schedule and Related Documents

Title V Air Operation Permit renewal (1070039-025-AV) issued March 20, 2017

Title V Air Operation Permit revision (1070039-028-AV) issued July 10, 2018 incorporated Air Construction Permit Nos. 1070039-023-AC (Replacement of COMS with Bag Leak Detection System (BLDS) Installation) and 1070039-026-AC (Time Extension) for the above referenced facility

Application for a Title V Air Operation Permit Revision received November 13, 2018 incorporates Air Construction Permit No. 1070039-027-AC (railcar unloading and conveyor system for inbound gypsum feedstock)

Primary Regulatory requirements

Standard Industrial Classification (SIC) Code:  3275 – Gypsum Products.

North American Industry Classification System (NAICS):  327420 - Gypsum Product Manufacturing.

HAP:  The facility is not identified as a major source of hazardous air pollutants (HAP).

Title V:  The facility is a Title V major source of air pollution in accordance with Chapter 62-213, Florida Administrative Code (F.A.C.).

PSD:  The facility is not a Prevention of Significant Deterioration (PSD) major source of air pollution in accordance with Rule 62-212.400, F.A.C.  Potential To Emit (PTE) PM emissions are synthetically limited to less than 250 TPY to avoid PSD regulations.

NSPS:  The facility operates units subject to the New Source Performance Standards (NSPS) of 40 Code of Federal Regulations (CFR) 60.

NESHAP:  The facility operates units subject to the National Emissions Standards for Hazardous Air Pollutants (NESHAP) of 40 CFR 63.

CAM:  Compliance Assurance Monitoring (CAM) does not apply to any of the units at the facility because, either the controls are inherent to the process or the pre-controlled potential to emit is below the major source threshold.

PROJECT REVIEW

This Title V revision adds a new Subsection G. to Section III.  Emission Units and Specific Conditions.  Subsection G Specific Conditions apply to the railcar unloading and conveyor transport of inbound gypsum.

Conclusion

This project revises the Title V air operation permit No. 1070039-025-AV, which was effective on March 20, 2017.  This Title V air operation permit revision is issued under the provisions of Chapter 403, Florida Statues (F.S.), and Chapters 62-4, 62-210, and 62-213, F.A.C.