# Continental Palatka, LLC

# Continental Palatka

# Facility ID No. 1070039

# Putnam County

# Title V Air Operation Permit Revision

# Permit No. 1070039-029-AV

(Revision of Title V Air Operation Permit No. 1070039-025-AV)

# Permitting Authority:

State of Florida

Northeast District Office

Permitting Authority

8800 Baymeadows Way, West, Suite 100

Jacksonville, Florida 32256

# Telephone:  (904) 256-1700

Fax:  (904) 256-1588

# Compliance Authority:

Compliance Assurance

Compliance Authority

8800 Baymeadows Way, West, Suite 100

Jacksonville, Florida 32256

# Telephone:  (904) 256-1700

Fax:  (904) 256-1590

Title V Air Operation Permit Revision

Permit No. 1070039-029-AV

Table of Contents

Section	Page Number

Placard Page 	 1

- Facility Information.

- Facility Description. 	 2-3

- Summary of Emissions Units. 	 4

- Applicable Regulations. 	 5

- Facility-wide Conditions. 	 6-7

- Emissions Units and Conditions.

- EU No. 001- FGD Surge Bin #1 (55-ton). 	 8-10

EU No. 010- Belt Conveyors and Bucket Elevators

EU No. 011- Emergency Live Bottom Feed Hopper (25-ton)

EU No. 014- Ball Mills

EU No. 016- Landplaster Bin

EU No. 022- FGD Surge Bin #2 (40-ton)

- EU No. 003- Imp Mill Feed Silo A (170 ton) 	 11-13

EU No. 004- Imp Mill Flash Calciner System A (55 ton)

EU No. 018- Imp Mill Feed Silo B (170 ton)

EU No. 019- Imp Mill Flash Calciner System B (55 ton)

- EU No. 024- Reclaim Processing and Screening Operation. 	 14-16

- EU No. 002- Cage Mill Dryer System #1 	 17-20

EU No. 023- Cage Mill Dryer System #2

- EU No. 005- Imp Mill Air Cooling System A. ………………………………………………….…… 21-24

EU No. 006- Stucco Silo A (690-ton)

EU No. 007- Starch Silo A

EU No. 008- Sawing Systems/Dunnage Machines

EU No. 013- Wallboard Dryer (4 Natural Gas Burners)

EU No. 017- Additives System and Pin Mixer

EU No. 020- Imp Mill Air Cooling System B

EU No. 021- Stucco Silo B (690-ton)

- EU No. 028- Perkins Emergency Engine (219 HP) 	25-27

EU No. 029- Perkins Emergency Engine (166 HP)

EU No. 030- John Deere Water Pump (275 HP)

- EU No. 031- Railcar unloading 	28-30

EU No. 032- Feed hopper to belt C1

EU No. 033- Transfer point belt C1 to C2

EU No. 034- Transfer point belt C2 to C3

EU No. 035- Transfer point belt C3 to TRS

EU No. 036- TRS drop to stockpile

- Appendices.

Appendix A, Glossary.

Appendix I, List of Insignificant Emissions Units and/or Activities.

Appendix NESHAP, Subpart A – General Provisions.

Appendix NESHAP, ZZZZ - National Emissions Standards for Hazardous Air Pollutants for Stationary Reciprocating Internal Combustion Engines.

Appendix NSPS, Subpart A – General Provisions.

Appendix NSPS, Subpart OOO - Standards of Performance for Nonmetallic Mineral Processing Plants and Subpart UUU, Calciners and Dryers in Mineral Industries

Appendix RR, Facility-wide Reporting Requirements.

Appendix TR, Facility-wide Testing Requirements.

Appendix TV, Title V General Conditions.

Appendix U, List of Unregulated Emissions Units and/or Activities.

Referenced Attachments.

Figure 1, Summary Report-Gaseous and Opacity Excess Emission and

Monitoring System Performance (40 CFR 60, July, 1996).

Table H, Permit History.

Table 1, Summary of Air Pollutant Standards and Terms.

Table 2, Compliance Requirements.

Permittee:	Permit No. 1070039-029-AV

Continental Palatka, LLC	Continental Palatka

886 North Highway 17	Facility ID No. 1070039

Palatka, Florida 32177				Title V Air Operation Permit Revision

The purpose of this permit is to revise Title V air operation permit to incorporate Construction Permit No. 1070039-027-AC (Railcar unloading station followed by conveyor transport system of inbound gypsum) for the above referenced facility.  The existing Continental Palatka, LLC, Continental Palatka is located in Putnam County at 886 North Highway 17 in Palatka, Florida.  The UTM coordinates are Zone 17, 438.13 km East and 3290.36 km North.

The Title V air operation permit is issued under the provisions of Chapter 403, Florida Statutes (F.S.), and Florida Administrative Code (F.A.C.) Chapters 62-4, 62-210, and 62-213.  The above named permittee is hereby authorized to operate the facility in accordance with the terms and conditions of this permit.

1070039-029-AV Effective Date:  TBD

1070039-025-AV Effective Date:  March 20, 2017

Renewal Application Due Date:  August 7, 2021

Expiration Date:  March 20, 2022

Executed in Jacksonville, Florida

STATE OF FLORIDA DEPARTMENT OF ENVIRONMENTAL PROTECTION

___________________________

Thomas G. Kallemeyn

Permitting Program Administrator

TGK/pr

Subsection A.  Facility Description.

This is a synthetic gypsum wallboard production facility.  Major facility activities include raw gypsum material storage and processing, a cage mill dryer system, a calciner, and wallboard production.  The cage mill dryer system reduces from about 10% to about 1% the moisture content of the gypsum.  The calciner converts the gypsum to stucco.

Synthetic gypsum is the primary raw material in wallboard production.  The nearby Seminole Electric Generating Facility delivers gypsum by conveyor(s).  The gypsum is primarily stored in the indoor Active Storage Dome. Synthetic gypsum that arrives by truck unloads into an open stockpile area outside of facility buildings.  Railcars of gypsum also are unloaded (EU 031) and conveyed using a transport system (EUs 032-036) onto the outdoor stockpile.  Raw gypsum has a free moisture content of typically 10% with a feel and consistency of wet fine sand.  These features of the raw gypsum minimize particulate matter emissions during handling and storage.

From the outdoor storage pile the synthetic gypsum (calcium sulfate dihydrate [CaSO4 • 2H2O]) transfers by front end loaders to the Active Storage Dome.  At most about 25,000 tons of gypsum is stored in the Active Storage Dome.  A reclaim system runs underneath the Active Storage Dome to collect and transfer the raw gypsum onto a conveyor.  The raw gypsum transfers along a primary conveyer into the facility process building. When the reclaim system is down for maintenance, front-end loaders deliver the raw gypsum into one of two feed hoppers (EU 011 and an unnumbered unit) that continuously meters the gypsum onto the primary conveyor.  Particulate emissions from each emissions unit vents indoors.  The main conveyor unloads the gypsum into one of two FGD Surge Bins (EUs 001 and 022) that are both controlled by a single control device.

Cage Mill Drying:  From the surge bins, the synthetic gypsum routes to parallel operating Cage Mill Dryers (EU002 or EU023).  The dryers flash dry the gypsum from about 10% to about 1% moisture content.  The dry heat at each Cage Mill Dryer is from a 55 MMBtu/hr NG direct-fired burner, one at each EU.  The dried gypsum includes by-products from the NG fuel combustion.  Two dust collectors recover the dried gypsum.  The collected gypsum is conveyed over an air slide system into one of two (2) large Imp Mill Feed Silos.

A small stream of the dried gypsum is sometimes directed into the Landplaster Bin (EU016).  This stream of dried gypsum is an additive in the Pin Mixer. Before entering the Pin Mixer, the dried gypsum from the Landplaster Bin is sent to Ball Mills (EU014).  Particulate matter emissions at the Landplaster Bin and the Ball Mills are controlled by an End Trim Dust Collector (EP-016) and a BMA Dust Collector (EP-014), respectively.  Both dust collectors vent indoors.

Imp Mill Flash Calcining Systems: From the Imp Mill Feed Silos (EU003 and EU018), the dried gypsum conveys to the Imp Mill Flash Calciners (EU004 and EU019).  The mills grind the 1% moisture dried gypsum to a specified fineness and the calciners remove chemically bound water to form stucco (calcium sulfate hemihydrate [CaSO4 • ½H2O]).  The product stucco water content is 5.5% to 6%.  Heat at each calciner is from separate 40 MMBtu/hr burners.  The stucco product includes NG combustion by-products.  Two dust collectors, one after each calciner, separates the warm gas from the stucco.  The exhaust gas discharges to the atmosphere.  The stucco routes into one of two bins for storing the stucco at the Imp Mill Air Cooling System.

From the bins, the IMP Mill Air Cooling System pneumatically conveys cooled stucco through one of two parallel processing dust collectors (EUs 005 and 020) that separate the cooling air from the stucco.  The cool air discharges to the atmosphere.  The stucco drops to an air slide into one of two 690-ton stucco storage silos (EUs 006 and 021).  From the storage silos, the stucco conveys by air slides and bucket elevators to a screw conveyor and then into the Pin Mixer.

The facility adds wet and dry additives to mix with the stucco at the Pin Mixer.  The major additives are starch, vermiculite, potash, boric acid, sugar, fiberglass, and foaming agents.  The dry additive starch arrives by truck and railcar, and is pneumatically transferred into the starch silo (EU 007).  The starch silo is exterior to plant buildings.  Similarly, vermiculite received by truck gets transferred into a storage bin (EU 015).  The vermiculite bin is inside the plant production building and exhausts air laden dust indoors.  The starch and vermiculite along with other raw materials feed by screw conveyor separate from the stucco screw conveyor into the Pin Mixer.  Wet additives added are a fiberglass slurry, a retardant, a disperser, wax, soap (containing 12% isopropyl alcohol), and edge paste.

Other raw additives received by bag or tote are manually fed into small hopper bins.  These bins continuously discharge additives onto the additives screw conveyor leading to the Pin Mixer.  A Nuisance Dust Collector pulls dust laden air above several emission areas inside the building and discharges the control device exhaust into the building.  Wet additive tanks of various sizes supply the wet additives.

Wallboard Production: The Palatka facility can produce approximately 950 million square feet of wallboard per year.  At the Pin Mixer, which is a small mixing tank, the stucco mixes with wet and dry additives to form a slurry.  The slurry from the Pin Mixer is distributed between a top and a bottom layer of paper to form a gypsum board on the wallboard production line.

The wet gypsum board moves by open conveyor down the length of the production building.  This travel time allows the board to dry set.  At the end of the forming conveyor, the gypsum boards are flipped, cut to desired size, and fed into the Wallboard Dryer (EU013).  Within the Wallboard Dryer are a set of NG burners with a combined heat input rating of approximately 186 MMBtu/hr.  Two stacks are positioned on the Wallboard Dryer.  The first stack is located at the “wet-end” (i.e. entry point) of the Wallboard Dryer (EP-013A) and the second stack is located at the “dry-end” (EP-013B).  Negligible emissions come from the wet-end stack as the combustion exhaust from the Wallboard Dryer burners vent through the dry end stack.

After accepted boards have dried in the Board Dryer, they are cut, trimmed, taped, and stacked.  To ensure appropriate wallboard quality, the paper covering is dried prior to introduction to the forming machine. Six, small 0.1 MMBtu/hr natural gas burners provide the heat to dry the paper as it unwinds from parent paper rolls delivered to the plant.  An inkjet printing system marks the paper with appropriate identification marks and information.  The inkjet printing process is an insignificant source of fugitive VOC and HAP emissions.  Gypsum dusts generated from the sawing and trimming operations are collected in the End Trim Dust Collector (EP-016) and recycled into the process at the Synthetic Gypsum Storage Bin.

A wet board reject system removes flawed wet boards from the line before fed through the Wallboard Dryer.  Front end loaders transfer these wet boards to an outdoor Waste Wallboard Storage pile.  The wet board rejects (or normal boards if there is a low supply of reject boards) are sent to a Dunnage Machine that applies glue to make five-board stacks and saws the five-board stacks into small “two-by-fours” (also called dunnage pieces). The two-by-fours are used as spacers for storing finished wallboards in the warehouse and on trucks.  The stacks of wallboard are safely handled by fork lift.  The End Trim Dust Collector (EP-016) captures the dust generated by the Dunnage Machine (EU008).

Wallboard Trimming Recycling System: Large pieces of leftover wallboard generated by the sawing and trimming operations are transported via front-end loaders to the Active Storage Building.  It is here that a Reclaim Processing and Screening System (EU024) separates debris, soil, and paper from the reclaim wallboard material.  By removing the paper material, a higher percent of reclaim board can be used as raw material.  The Reclaim Processing and Screening System emission unit includes: portable tubular screens (sometimes cited as trommel screens), stationary screens, and bucket separators (modules that attach to the arms of a front-end loader).

The leftover wallboard remnants after removal of debris, dirt and paper are ground up and pulverized by a recycle grinder system.  The recycled wallboard remnants are reintroduced into the process by dropping the material onto the primary conveyor that leads into the Mill Building.

Subsection B.  Summary of Emissions Units.

| EU | Description | Emission Point | Filter System |
| --- | --- | --- | --- |
| Regulated Emissions Units and associated Emission Points |  |  |  |
| 001 | FGD Surge Bin #1 (55-ton Bin) | EP-01 | FGD Surge Bin Filter |
| 002 | Cage Mill Dryer System #1 | EP-02 | Dust Collector |
| 003 | Imp Mill Feed Silo A (170-ton Bin) | EP-02/23 | Dust Collector/Bin Filter |
| 004 | Imp Mill Flash Calciner System A | EP-04 | Dust Collector |
| 005 | Imp Mill Air Cooling System A | EP-05 | System Collector |
| 006 | Stucco Silo A (690-ton Bin) | EP-06 | Silo Filter |
| 007 | Starch Silo | EP-07 | Starch Silo Vent Filter |
| 008 | Sawing Systems/Dunnage Machines | EP-16 | End Trim Dust Collector |
| 010 | Belt Conveyors and Bucket Elevators. | Fugitive | None |
| 011 | Emergency Live Bottom Feed Hopper. | EP-11 | BC-9 Nuisance Dust Collector |
| 013 | Wallboard Dryer (4 Natural Gas Burners) | EP-13A EP-13B | None |
|  |  |  | None |
| 014 | Ball Mills | EP-14 | BMA Dust Collector |
| 016 | Landplaster Bin | EP-16 | End Trim Dust Collector |
| 017 | Additives System and Pin Mixer | EP-17 | Nuisance Dust Collector |
| 018 | Imp Mill Feed Silo B (170-ton Bin) | EP-02/23 | Dust Collector/Bin Filter |
| 019 | Imp Mill Flash Calciner System B | EP-19 | Dust Collector |
| 020 | Imp Mill Cooling System B | EP-20 | System Filter |
| 021 | Stucco Silo B (690-ton Bin) | EP-21 | Silo Filter |
| 022 | FGD Surge Bin (55-ton Bin) #2 | EP-22 | FGD Surge Bin Filter |
| 023 | Cage Mill Dryer System #2 | EP-23 | Dust Collector |
| 024 | Reclaim Processing and Screening | EP-24 | None |
| 028 | Perkins diesel fired emergency engine (219 HP) | EP-28 | N/A |
| 029 | Perkins diesel fired emergency engine (166HP) | EP-29 | N/A |
| 030 | John Deere Water Pump | EP 30 | N/A |
| 031 | Railcar unloading | Fugitive | N/A |
| 032 | Feed hopper to belt C1 | Fugitive | N/A |
| 033 | Transfer point belt C1 to C2 | Fugitive | N/A |
| 034 | Transfer point belt C2 to C3 | Fugitive | N/A |
| 035 | Transfer point belt C3 to TRS | Fugitive | N/A |
| 036 | TRS to Stockpile | Fugitive | N/A |
| Unregulated Emissions Units and Activities |  |  |  |
|  | Paved & unpaved roads (fugitive emissions) |  |  |
|  | Raw material & product storage piles, conveying & handling |  |  |
|  | Wood and metalworking shops |  |  |
|  | Painting operations/paint shop 1 |  |  |
|  | Sandblasting 2 |  |  |
|  | Pipe leaks 3 |  |  |
|  | Pump seals 3 |  |  |
|  | Packing leaks |  |  |
|  | Unconfined particulate from road dust/ miscellaneous sources |  |  |
|  | Machine shops |  |  |

| Unregulated Emissions Units and Activities (continued) |  |
| --- | --- |
|  | Lubricating oil reservoirs |
|  | Fire training |
|  | Loading/unloading/storage of packaged materials |
|  | Lab vents |
|  | Refueling |
|  | Space heaters |
|  | General purpose painting |
|  | Gypsum handling and storage system 4 |
|  | Limestone handling and storage system 4 |

1 Not to exceed HAP and VOC rule reporting thresholds.

2 With the exception of off property drift.

3 In accordance with manufacturer’s specified allowances under normal operation.

4 Unregulated with the exception of compliance with Title V Permit No. 1070039-018, Facility-Wide Condition No. 3.

Also included in this permit are miscellaneous insignificant emissions units and/or activities (see Appendix I, List of Insignificant Emissions Units and/or Activities).

Subsection C.  Applicable Regulations.

Based on the Title V air operation permit revision application received November 13, 2018, this facility is not a major source of hazardous air pollutants (HAP).  The existing facility is not a prevention of significant deterioration (PSD) major source of air pollutants in accordance with Rule 62-212.400, F.A.C., as the PM emission rate (0.017 gr/dscfm) requested by Continental for the Cage Mill Dryer Systems # 1 and 2 avoided the facility from being classified as a PSD major source.  The PM emission rate of 0.017 gr/dscfm effectively limits the emissions to less than 250 tons per year (tpy).  A summary of applicable regulations is shown in the following table.

| Regulation | ## EU No(s). |
| --- | --- |
| Federal Rule Citations |  |
| 40 CFR 60, NSPS Subpart A - General Provisions | EUs: 001, 002, 003, 004, 010, 011, 014, 016, 018, 019, 022, 023, 024, 031-035 |
| 40 CFR 60, NSPS Subpart OOO - Standards of Performance for Nonmetallic Mineral Processing Plants | EUs: 001, 003, 004, 010, 011, 014, 016, 018, 019, 022, 024, 031-035 |
|  | EU: 036 (the TRS conveyor belt is an NSPS subpart OOO regulated EU, but it is not subject to an emissions standard) |
| 40 CFR 60, NSPS Subpart UUU – Standards of Performance for Calciners and Dryers in Mineral Industries | EUs: 002, 023 |
| 40 CFR 63, NESHAP Subpart A - General Provisions | EUs: 028, 029, 030 |
| 40 CFR 63, NESHAP Subpart ZZZZ - National Emissions Standards for Hazardous Air Pollutants for Stationary Reciprocating Internal Combustion Engines | EUs: 028, 029, 030 |
| State Rule Citations |  |
| State Rule Citations (Rule 62-296; 62-297.310(7); Rule 62-212.400(12), F.A.C.) | EUs: 001, 002, 003, 004, 005, 006, 007, 008, 010, 011, 013, 014, 016, 017, 018, 019, 020, 021,022, 023, 024 |

Table of Contents

The following conditions apply facility-wide to all emission units and activities:

- Appendices.  The permittee shall comply with all documents identified in Section IV, Appendices, listed in the Table of Contents.  Each document is an enforceable part of this permit unless otherwise indicated.  [Rule 62-213.440, F.A.C.]

Emissions and Controls

- Not federally Enforceable.  Objectionable Odor Prohibited.  No person shall cause, suffer, allow or permit the discharge of air pollutants, which cause or contribute to an objectionable odor.  An “objectionable odor” means any odor present in the outdoor atmosphere which by itself or in combination with other odors, is or may be harmful or injurious to human health or welfare, which unreasonably interferes with the comfortable use and enjoyment of life or property, or which creates a nuisance.  [Rule 62-296.320(2) and 62-210.200(Definitions), F.A.C.]

- General Volatile Organic Compounds (VOC) Emissions or Organic Solvents (OS) Emissions.  The permittee shall allow no person to store, pump, handle, process, load, unload or use in any process or installation, volatile organic compounds or organic solvents without applying known and existing vapor emission control devices or systems deemed necessary and ordered by the Department.  [Rule 62-296.320(1), F.A.C.]

{Permitting Note:  Nothing is deemed necessary and ordered at this time.}

- General Visible Emissions.  No person shall cause, let, permit, suffer or allow to be discharged into the atmosphere the emissions of air pollutants from any activity equal to or greater than 20% opacity.  This regulation does not impose a specific testing requirement.  [Rule 62-296.320(4)(b), F.A.C.]

- Unconfined Particulate Matter.  No person shall cause, let, permit, suffer or allow the emissions of unconfined particulate matter from any activity, including vehicular movement; transportation of materials; construction; alteration; demolition or wrecking; or industrially related activities such as loading, unloading, storing or handling; without taking reasonable precautions to prevent such emissions.  Reasonable precautions to prevent emissions of unconfined particulate matter at this facility include:

a.	Vehicular traffic on paved and unpaved roads.

b.	Wind-blown dust from yard areas.

c.	Periodic abrasive blasting

The following techniques may be used to control unconfined PM emissions on an as-needed basis:

a. Paving and maintenance of roads, parking areas, and yards.

b. Chemical (dust suppressants) or water application to:

- Unpaved roads

- Unpaved yard areas

c. Landscaping or planting of vegetation

d. Confining abrasive blasting where possible.

e. Other techniques, as necessary.

[Rule 62-296.320(4)(c), F.A.C.; and Permit No. 1070039-001-AC]

Reports and Fees

See Appendix RR, Facility-wide Reporting Requirements for additional details.

- Electronic Annual Operating Report and Title V Annual Emissions Fees.  The information required by the Annual Operating Report for Air Pollutant Emitting Facility [Including Title V Source Emissions Fee Calculation] (DEP Form No. 62-210.900(5)) shall be submitted by April 1 of each year, for the previous calendar year, to the Department of Environmental Protection’s (DEP) Division of Air Resource Management.  Each Title V source shall submit the annual operating report using the DEP’s Electronic Annual Operating Report (EAOR) software, unless the Title V source claims a technical or financial hardship by submitting DEP Form No. 62-210.900(5) to the DEP Division of Air Resource Management instead of using the reporting software.  Emissions shall be computed in accordance with the provisions of subsection 62-210.370(2), F.A.C.  Each Title V source must pay between January 15 and April 1 of each year an annual emissions fee in an amount determined as set forth in subsection 62-213.205(1), F.A.C.  The annual fee shall only apply to those regulated pollutants, except carbon monoxide and greenhouse gases, for which an allowable numeric emission-limiting standard is specified in the source’s most recent construction permit or operation permit.  Upon completing the required EAOR entries, the EAOR Title V Fee Invoice can be printed by the source showing which of the reported emissions are subject to the fee and the total Title V Annual Emissions Fee that is due.  The submission of the annual Title V emissions fee payment is also due (postmarked) by April 1st of each year.  A copy of the system-generated EAOR Title V Annual Emissions Fee Invoice and the indicated total fee shall be submitted to:  Major Air Pollution Source Annual Emissions Fee, Post Office Box 3070, Tallahassee, Florida 32315-3070.  Additional information is available by accessing the Title V Annual Emissions Fee On-line Information Center at the following Internet web site:  http://www.dep.state.fl.us/air/emission/tvfee.htm.  [Rules 62-210.370(3), 62-210.900 and 62-213.205, F.A.C.; and, §403.0872(11), Florida Statutes (2013)]

{Permitting Note:  Resources to help you complete your AOR are available on the electronic AOR (EAOR) website at:  http://www.dep.state.fl.us/air/emission/eaor.  If you have questions or need assistance after reviewing the information posted on the EAOR website, please contact the Department by phone at (850) 717-9000 or email at eaor@dep.state.fl.us.}

{Permitting Note:  The Title V Annual Emissions Fee form (DEP Form No. 62-213.900(1)) has been repealed.  A separate Annual Emissions Fee form is no longer required to be submitted by March 1st each year.}

- Annual Statement of Compliance.  The permittee shall submit an annual statement of compliance to the compliance authority at the address shown on the cover of this permit and to the US. EPA at the address shown below within 60 days after the end of each calendar year during which the Title V air operation permit was effective.  (See also Appendix RR, Conditions RR1 and RR7.)  [Rules 62-213.440(3)(a)2. & 3. and (b), F.A.C.]

U.S. Environmental Protection Agency, Region 4

Atlanta Federal Center

61 Forsyth Street, SW

Atlanta, Georgia  30303

Attn:  Air Enforcement Branch

- Prevention of Accidental Releases (Section 112(r) of CAA).  If, and when, the facility becomes subject to 112(r), the permittee shall:

- Submit its Risk Management Plan (RMP) to the Chemical Emergency Preparedness and Prevention Office (CEPPO) RMP Reporting Center.  Any Risk Management Plans, original submittals, revisions or updates to submittals, should be sent electronically through EPA’s Central Data Exchange system at the following address:  https://cdx.epa.gov.  Information on electronically submitting risk management plans using the Central Data Exchange system is available at:  http://www2.epa.gov/rmp.  The RMP Reporting Center can be contacted at:  RMP Reporting Center, Post Office Box 10162, Fairfax, VA  22038, Telephone:  (703) 227-7650.

- Submit to the permitting authority Title V certification forms or a compliance schedule in accordance with Rule 62-213.440(2), F.A.C.  [40 CFR 68]

Table of Contents

The specific conditions in this section apply to the following emissions unit(s):

| EU | Brief Description | Emission Points |
| --- | --- | --- |
| 001 | FGD Surge Bin #1 (40-ton) | EP-01 - Indoor |
| 010 | Belt Conveyors and Bucket Elevators | Fugitive |
| 011 | Emergency Live Bottom Feed Hopper(25-ton) | EP-11 - Indoor |
| 014 | Ball Mills | EP-14 - Indoor |
| 016 | Landplaster Bin | EP-16 - Indoor |
| 022 | FGD Surge Bin #2 (40-ton) | EP-22 - indoors |

Emission Unit 001 (EU001) and Emission Unit 022 (EU 022): 55-ton capacity gypsum indoor FGD Surge Bins #1 and #2 are controlled by a single FDG surge bin filter for particulate matter control.  The fabric filter’s primary purpose is to recover product and return it to the process.

Emission Unit 011 (EU011): 25-ton capacity Emergency Live Bottom Feed Hopper located indoors is controlled by a BC-9 Nuisance dust collector.

Emission Unit 014 (EU014): Ball Mills located indoors are controlled by a BMA dust collector.

Emission Unit 016 (EU016): Landplaster Bin located indoors is controlled by an end trim dust collector system.

Emission Unit 010 (EU010): 55-ton capacity Emergency Live Bottom Feed Hopper is uncontrolled.

Natural gas is the authorized fuel for these emissions units.

{Permitting Note:  These emission unit(s) are subject to NSPS requirements/standards in 40 CFR 60, Subpart OOO, Standards of Performance for Nonmetallic Mineral Processing Plants and 40 CFR 60 (Subpart A, Standards of Performance for New Stationary Sources – General Provisions) adopted and incorporated by reference in Rule 62-204.800, F.A.C.}

Essential Potential to Emit (PTE) Parameters

- Permitted Capacity.  The maximum allowable operating rate is as follows and shall not be exceeded without prior Department approval.

| Unit No. | Description | Capacity Ton/Hr. Gypsum | Emissions Pt. | Control System |
| --- | --- | --- | --- | --- |
| 001 | FGD Surge Bin #1(40 ton) Note 1 | 100 | 01-Indoor | Bin filter |
| 010 | Belt Conveyors and Bucket Elevators | N/A | Fugitive | N/A |
| 011 | Emergency Live Bottom Feed Hopper (25 ton) | 80 | 011- Indoor | BC-9 Nuisance dust collector |
| 014 | Ball Mills | N/A | 014- Indoors | BMA dust collector |
| 016 | Land plaster Bin | N/A | 016- Indoors | End trim dust collector |
| 022 | FGD Surge Bin #2 (40 ton) Note 1 | 100 | 022- Indoors | Bin Filter |

Note 1: The fabric filter’s primary purpose is to recover product and return it to the process.

[Rules 62-4.160(2), 62-204.800, 62-210.200(PTE), F.A.C.; and, Permit Nos. 1070039-014 and 015-AC.]

- Emissions Unit Operating Rate Limitation After Testing.  See the related testing provisions in Appendix TR, Facility-wide Testing Requirements.  [Rule 62-297.310(3), F.A.C.]

- Methods of Operation Fuels.  Natural Gas is the only fuel authorized to be burned in these units.

[Rule 62-213.410, F.A.C.; Permit Nos. 1070039-014 and 015-AC.]

- Hours of Operation.  These emissions units may operate continuously (8,760 hours/year).  [Rule 62-210.200(PTE), F.A.C.; and, Permit Nos. 1070039-014 and 015-AC]

Emission Limitations and Standards

{Permitting Note:  The attached Table 1, Summary of Air Pollutant Standards, summarizes information for convenience purposes only.  This table does not supersede any of the terms or conditions of this permit.}

Unless otherwise specified, the averaging time(s) for Specific Conditions A.5.–A.6 are based on the specified averaging time of the applicable test method.

- Visible Emissions.  Visible emissions shall not exceed:

| Emission Unit | Emission Pt. | Opacity Limit |
| --- | --- | --- |
| 001, 011, 014, 016, and 022 | Indoors | 7% |
| 010 | fugitive | 10% |

[40 CFR 60.672 (a), (b), (e)(1) and, NSPS Subpart OOO Table 2 and Permit Nos. 1070039-014 and 015-AC]

- PM Emissions.  Particulate matter (PM) emissions shall not exceed:

| Emission Unit (EU) | Emission Point (EP) | Standards | Allowable limit |
| --- | --- | --- | --- |
| 001, 022 | Indoors | 0.02 gr/dscf (0.05 g/dscm) | 0.05 lbs/hr, 0.23 TPY |
| 010, | Fugitive | N/A | N/A |
| 011, 014 | Individually controlled | 0.02 gr/dscf (0.05 g/dscm) | 0.19 lbs/hr, 0.83 TPY |
| 016 | Indoors | 0.02 gr/dscf (0.05 g/dscm) | 5.66 lbs/hr, 24.78 TPY |

[40 CFR 60.672(a), (b), (e)(2), (f) NSPS Subpart OOO Table 2; and Permit Nos. 1070039- 014 and 015-AC]

- Good Air Pollution Control Practices.  At all times, including periods of startup, shutdown, and malfunction, owners and operators shall, to the extent practicable, maintain and operate any affected facility including associated air pollution control equipment in a manner consistent with good air pollution control practice for minimizing emissions.  Determination of whether acceptable operating and maintenance procedures are being used will be based on information available to the Administrator which may include, but is not limited to, monitoring results, opacity observations, review of operating and maintenance procedures, and inspection of the source.  [40 CFR 60.11(d)]

Test Methods and Procedures

{Permitting Note:  The attached Table 2, Summary of Compliance Requirements, summarizes information for convenience purposes only.  This table does not supersede any of the terms or conditions of this permit.}

- Test Methods.  When required, tests shall be performed in accordance with the following reference methods:

| Method | Description of Method and Comments |
| --- | --- |
| 1-4 | Traverse Points, Velocity and Flow Rate, Gas Analysis, and Moisture Content |
| 5B | Method for Determining Particulate Matter Emissions (All PM is assumed to be PM10.) |
| 9 | Visual Determination of the Opacity of Emissions from Stationary Sources |
| 22 | Visual Determination of Fugitive Emissions from Material Sources |

The above methods are described in 40 CFR 60, Appendix A-7, and adopted by reference in Rule 62-204.800, F.A.C.  No other methods may be used unless prior written approval is received from the Department.  [Rule 62-204.800, F.A.C., and, Permit Nos. 1070039-014 and 015-AC]

- Common Testing Requirements.  Unless otherwise specified, tests shall be conducted in accordance with the requirements and procedures specified in Appendix TR, Facility-Wide Testing Requirements, of this permit.  [Rule 62-297.310, F.A.C.]

- Annual Compliance Tests Required Fugitive Emissions (VE).  During each calendar year (January 1st to December 31st), each EU (001, 011, 014, 016 and 022) shall be tested to demonstrate compliance with the emissions limit in Specific Condition A.5. by exhibiting no visible emissions from the building as determined by EPA Method 22.  [Rule 62-297.310(8), F.A.C.  40 CFR 60.675(d)]

Recordkeeping and Reporting Requirements

- Other Reporting Requirements.  See Appendix RR, Facility-Wide Reporting Requirements, for additional reporting requirements.  [Rule 62-213.440(1)(b), F.A.C.]

- Federal Rule Requirements.  In addition to the specific conditions listed above, these emissions units are also subject to the applicable requirements contained in, 40 CFR 60, Subpart A – General Provisions and 40 CFR 60 Subpart OOO – Standards of Performance for Nonmetallic Mineral Processing Plants.  [Rule 62-213.440, F.A.C.]

Table of Contents

The specific conditions in this section apply to the following emissions units:

| EU | Brief Description | Emission Points |
| --- | --- | --- |
| 003 | Imp Mill Feed Silo A (170 ton) | EP-02/023 |
| 004 | Imp Mill Flash Calciner System A (55 ton) | EP-04 |
| 018 | Imp Mill Feed Silo B (170) | EP-02/023 |
| 019 | Imp Mill Flash Calciner System B (170) | EP-19 |

Emission Unit 004 (EU004) and Emission Unit 019 (EU019): 55 TPH Imp Mill Flash Calciner System A and B,   controlled by separate dust collectors.  Each calciner system includes an associated 40 MMbtu/hr natural gas fired burners that provides heat for the calcining process.

Emission Unit 003 (EU003), and Emission Unit 018 (EU018): 170 -ton capacity gypsum Imp Mill Feed Silo A, and B, are ducted to the cage mill dust collector system (EP 02 and 023), which pulls negative draft on the silos.

{Permitting Note:  These emission units are regulated under 40 CFR 60, Subpart OOO, Standards of Performance for Nonmetallic Mineral Processing Plants and 40 CFR 60 (Subpart A, Standards of Performance for New Stationary Sources – General Provisions) adopted and incorporated by reference in Rule 62-204.800, F.A.C.}

Essential Potential to Emit (PTE) Parameters

- Permitted Capacity.  The maximum allowable operating rate is as follows and shall not be exceeded without prior Department approval.

| Unit No. | Description | Capacity Ton/Hr. Gypsum | Emissions Pt. | Control System |
| --- | --- | --- | --- | --- |
| 003 | Imp Mill Feed Silo A (170 ton) | 55 | 02/23 | Dust Collector Ep 02/023 |
| 004 | Imp Mill Flash Calciner System A (55 ton) | 55 | 04 | Dust collector |
| 018 | Imp Mill Feed Silo B (170) | 55 | 02/023 | Dust Collector Ep 02/023 |
| 019 | Imp Mill Flash Calciner System B (170) | 55 | 19 | Dust collector |

[Rules 62-4.160(2), 62-204.800, 62-210.200(PTE), F.A.C.; and, Permit No. 1070039-015-AC]

- Permitted Capacity- Heat Input (EU004) (EU019).  The maximum heat input rate of each emissions units is 40 MMbtu/hr. of natural gas.  [Rules 62-4.160(2), 62-204.800, 62-210.200(PTE), F.A.C.; and Permit No. 1070039-011-AC.]

- Hours of Operation.  These emissions units may operate continuously (8,760 hours/year).  [Rule 62-210.200(PTE), F.A.C]

- Methods of Operation Fuel.  Natural Gas is the only fuel authorized to be burned in these units.

[Rule 62-213.410, F.A.C.; Permit No. 070039-011-AC]

- Emissions Unit Operating Rate Limitation After Testing.  See the related testing provisions in Appendix TR, Facility-wide Testing Requirements.  [Rule 62-297.310(3), F.A.C.]

Emission Limitations and Standards

{Permitting Note:  The attached Table 1, Summary of Air Pollutant Standards, summarizes information for convenience purposes only.  This table does not supersede any of the terms or conditions of this permit.}

Unless otherwise specified, the averaging time(s) for Specific Conditions B.6– B.7 are based on the specified averaging time of the applicable test method.

- Visible Emissions.  Visible emissions shall not exceed:

| Emission Unit | Emission Pt. | Opacity Limit |
| --- | --- | --- |
| 04, 019 | 04, 019 | 7% |
| 03, 018 Note 1 | Indoors | 7% |

Note 1: EU Nos. 03 and 018: Normal operation of the EPs at these emission units is to vent into EU002, which requires a yearly EPA 9.

[40 CFR 60.672(a), (b), (e)(1) and, NSPS Subpart OOO Table 3 and Permit No. 1070039-015-AC]

- PM Emissions.  Particulate matter (PM) emissions shall not exceed:

| Emission Unit | Emission Pt. | Standards | Allowable limit |
| --- | --- | --- | --- |
| 004, 019 | 004, 019 | 0.017 gr/dscf Note 1 | 1.89 lbs/hr, 8.3 TPY |
| 003, 018 Note 2 | Indoors | 0.02 gr/dscf (0.05 g/dscm) | 0.05 lbs/hr, 0.23 TPY |

Note 1:  Requested by applicant to escape PSD

Note 2:  EU Nos. 003 and 018: Normal operation of the EPs at these emission units is to vent into EU002, which requires a yearly EPA Method 5.

[40 CFR 60.672(a), (b), (e)(2), (f) NSPS Subpart OOO Table 2 and Permit No. 1070039-015-AC]

- Good Air Pollution Control Practices.  At all times, including periods of startup, shutdown, and malfunction, owners and operators shall, to the extent practicable, maintain and operate any affected facility including associated air pollution control equipment in a manner consistent with good air pollution control practice for minimizing emissions.  Determination of whether acceptable operating and maintenance procedures are being used will be based on information available to the Administrator which may include, but is not limited to, monitoring results, opacity observations, review of operating and maintenance procedures, and inspection of the source.  [40 CFR 60.11(d)]

Test Methods and Procedures

{Permitting Note:  The attached Table 2, Summary of Compliance Requirements, summarizes information for convenience purposes only.  This table does not supersede any of the terms or conditions of this permit.}

- Test Methods.  When required, tests shall be performed in accordance with the following reference methods:

| Method | Description of Method and Comments |
| --- | --- |
| 1-4 | Traverse Points, Velocity and Flow Rate, Gas Analysis, and Moisture Content |
| 5B | Method for Determining Particulate Matter Emissions (All PM is assumed to be PM10.) |
| 9 | Visual Determination of the Opacity of Emissions from Stationary Sources |
| 22 | Visual Determination of Fugitive Emissions from Material Sources |

The above methods are described in 40 CFR 60, Appendix A-1, and adopted by reference in Rule 62-204.800, F.A.C.  No other methods may be used unless prior written approval is received from the Department.  [Rule 62-204.800, F.A.C. and Permit Nos. 1070039-014 and 015-AC]

- Common Testing Requirements.  Unless otherwise specified, tests shall be conducted in accordance with the requirements and procedures specified in Appendix TR, Facility-Wide Testing Requirements, of this permit.  [Rule 62-297.310, F.A.C.]

- Annual Compliance Tests Required (VE).  During each calendar year (January 1st to December 31st), each EU (003, 004, 018 and 019) shall be tested to demonstrate compliance with Visible Emissions (VE) standards in Specific Condition B.6. {Permitting Note: EU (003) and EU (018): Normal operation is to vent into EU002, which requires a yearly Method 9 VE test.}  [Rule 62-297.310(8), F.A.C.]

- Annual Compliance Tests Required Fugitive Emissions (VE).  During each calendar year (January 1st to December 31st), each EU (003 and 018) shall be tested to demonstrate compliance with the fugitive emissions limit using Method 22 when they are operated such that emissions are vented apart from EU002 (inside of the building).  [Rule 62-297.310(8), F.A.C.  40 CFR 60.675(d)]

- Annual Compliance Tests Required (PM).  During each calendar year (January 1st to December 31st), each EU (003 and 018) shall be tested to demonstrate compliance with the PM emissions limit as specified in Specific Condition B.7. {Permitting Note: EU (003) and EU (018): Normal operation is to vent into EU002, which requires a yearly Method 5 PM test.}  [Rule 62-297.310(8), F.A.C.  40 CFR 60.675(d)]

- Weekly EPA Method 22 Test Required (VE).  A weekly EPA Method 22 visible emissions compliance test for each emissions unit EU (004 and 019) to ensure that emissions are not visible.  The minimum observation period of the weekly EPA Method 22 test shall be twelve minutes.  In the event that the weekly test indicates visible emissions, the permittee shall undertake corrective action as well as conduct a Method 9 test with a minimum observation period of sixty (60) minutes (as incorporated in Chapter 62-297, FAC).  The permittee shall maintain records documenting that the weekly compliance testing has occurred.  The records shall include color, duration, and density of the plume of any abnormal visible emissions detected, as well as the cause and corrective action taken for any abnormal visible emissions.

Recordkeeping and Reporting Requirements

- Other Reporting Requirements.  See Appendix RR, Facility-Wide Reporting Requirements, for additional reporting requirements.  [Rule 62-213.440(1)(b), F.A.C.]

- Federal Rule Requirements.  In addition to the specific conditions listed above, these emissions units are also subject to the applicable requirements contained in, 40 CFR 60, Subpart A – General Provisions and 40 CFR 60 Subpart OOO – Standards of Performance for Nonmetallic Mineral Processing Plants.  [Rule 62-213.440, F.A.C.]

Table of Contents

The specific conditions in this section apply to the following emissions unit:

| EU No. | Brief Description |
| --- | --- |
| 024 | Reclaim Processing Screening and Conveyor System |

Emissions Unit 024 - Reclaim Processing Screening and Conveyor System includes portable tubular screens (trommel screens), stationary screens, and bucket separators (modules that attach to the arms of a front-end loader).  The 200 ton per hour portable McCloskey International 621 RE Trommel Machine includes an associated 174 hp diesel engine.

The Reclaim Processing Screening and Conveyor System separates debris, soil, and paper from the reclaim wallboard material and supplemental supplies of synthetic gypsum delivered to the site.  The removal of the debris, soil, and paper allows for a higher percentage of reclaim board used as the raw material for the plant. The Reclaim Processing Screening and Conveyor System is uncontrolled.

{Permitting Note:  This emission unit is regulated under 40 CFR 60, Subpart OOO, Standards of Performance for Nonmetallic Mineral Processing Plants and 40 CFR 60 (Subpart A, Standards of Performance for New Stationary Sources – General Provisions) adopted and incorporated by reference in Rule 62-204.800, F.A.C.}

Essential Potential to Emit (PTE) Parameters

- Permitted Capacity.  The maximum operating rates are listed below and shall not be exceeded without prior Department approval.

| Unit No. | Description | Capacity Ton/Hr1. Gypsum | Emissions Pt. | Control System |
| --- | --- | --- | --- | --- |
| 024 | Reclaim Processing Screening and Conveyor | 200 | 24 | uncontrolled |

1 174 HP diesel-fuel fired industrial engine

[Rule 62-210.200(PTE) and Permit No. 1070039-024-AC]

- Emissions Unit Operating Rate Limitation After Testing.  See the related testing provisions in Appendix TR, Facility-wide Testing Requirements.  [Rule 62-297.310(3), F.A.C.]

- Hours of Operation.  The emissions unit may operate continuously 8,760 hours/year.  [Rule 62-210.200(PTE) and Permit No. 1070039-024-AC]

Emission Limitations and Standards

{Permitting Note:  The attached Table 1, Summary of Air Pollutant Standards, summarizes information for convenience purposes only.  This table does not supersede any of the terms or conditions of this permit.}

Unless otherwise specified, the averaging time(s) for Specific Condition No. C.4. is based on the specified averaging time of the applicable test method.

- Visible Emissions.  Visible emissions shall not exceed:

| POLLUTANT | STANDARD/LIMIT |
| --- | --- |
| Visible Emissions | 7 % Opacity1 |

1Affected facilities (as defined in 40 CFR 60.670 and 60.671) that commence construction, modification, or reconstruction on or after April 22, 2008

[40 CFR 60.672(b) and Table 3 of Subpart OOO Permit No. 1070039-024-AC]

- Good Air Pollution Control Practices.  At all times, including periods of startup, shutdown, and malfunction, owners and operators shall, to the extent practicable, maintain and operate any affected facility including associated air pollution control equipment in a manner consistent with good air pollution control practice for minimizing emissions.  Determination of whether acceptable operating and maintenance procedures are being used will be based on information available to the Administrator which may include, but is not limited to, monitoring results, opacity observations, review of operating and maintenance procedures, and inspection of the source.  [40 CFR 60.11(d)]

Test Methods and Procedures

{Permitting Note:  The attached Table 2, Summary of Compliance Requirements, summarizes information for convenience purposes only.  This table does not supersede any of the terms or conditions of this permit.}

- Test Methods.  When required, tests shall be performed in accordance with the following reference methods:

| Method | Description of Method and Comments |
| --- | --- |
| 9 | Visual Determination of the Opacity of Emissions from Stationary Sources |

The above methods are described in 40 CFR 60, Appendix A-1, and adopted by reference in Rule 62-204.800, F.A.C.  No other methods may be used unless prior written approval is received from the Department.  [Rule 62-204.800, F.A.C., and, Permit No. 1070039-024-AC]

- Common Testing Requirements.  Unless otherwise specified, tests shall be conducted in accordance with the requirements and procedures specified in Appendix TR, Facility-Wide Testing Requirements, of this permit.  [Rule 62-297.310, F.A.C.]

- Truck Dumping of Nonmetallic Minerals.  Truck dumping of nonmetallic minerals into any screening operation, feed hopper, or crusher is exempt from the requirements of this section.  [40 CFR 60.672(d)]

- Repeat Performance Test.  The permittee shall perform a repeat performance test according to 40 CFR 60.11 for opacity within 5 years from the previous performance test for fugitive emissions from affected facilities without water sprays. Affected facilities controlled by water carryover from upstream water sprays that are inspected according to the requirements in Specific Condition No. C.10. and Specific Condition No. C.11. are exempt from this 5-year repeat testing requirement.  [Permit No. 1070039-024-AC, 40 CFR 60.672(d) and Table 3 of Subpart OOO]

Monitoring Requirements

- Wet Suppression.  The permittee of any affected facility for which construction, modification, or reconstruction commenced on or after April 22, 2008, that uses wet suppression to control emissions from the affected facility must perform monthly periodic inspections to check that water is flowing to discharge spray nozzles in the wet suppression system. The permittee must initiate corrective action within 24 hours and complete corrective action as expediently as practical if the permittee finds that water is not flowing properly during an inspection of the water spray nozzles. The permittee shall record each inspection of the water spray nozzles, including the date of each inspection and any corrective actions taken, in the logbook required under Specific Condition No. C.11.

If an affected facility relies on water carryover from upstream water sprays to control fugitive emissions, then that affected facility is exempt from the 5-year repeat testing requirement specified in Table 3 of this subpart provided that the affected facility meets the criteria in paragraphs (1) and (2) below:

- The permittee of the affected facility conducts periodic inspections of the upstream water spray(s) that are responsible for controlling fugitive emissions from the affected facility.  These inspections are conducted according to this condition Specific Condition No. C.10, and

- The permittee of the affected facility designates which upstream water spray(s) will be periodically inspected at the time of the initial performance test required under 40 CFR 60.1.  If an affected facility that routinely uses wet suppression water sprays ceases operation of the water sprays or is using a control mechanism to reduce fugitive emissions other than water sprays during the monthly inspection (for example, water from recent rainfall), the logbook entry required under Specific Condition No. C.11 shall specify the control mechanism being used instead of the water sprays.

[40 CFR 60.674(b)(1) and (2)]

Recordkeeping and Reporting Requirements

- Periodic Inspection Reports.  The Permittee of affected facilities (as defined in 40 CFR 60.670 and 60.671) for which construction, modification, or reconstruction commenced on or after April 22, 2008, must record each periodic inspection required under Specific Condition C.10., including dates and any corrective actions taken, in a logbook (in written or electronic format).  The permittee shall keep the logbook onsite and make hard or electronic copies (whichever is requested) of the logbook available to the Department upon request.  [40 CFR 60.676(b)(1)]

- Other Reporting Requirements.  See Appendix RR, Facility-Wide Reporting Requirements, for additional reporting requirements.  [Rule 62-213.440(1)(b), F.A.C.]

- Federal Rule Requirements.  In addition to the specific conditions listed above, these emissions units are also subject to the applicable requirements contained in, 40 CFR 60, Subpart A – General Provisions and 40 CFR 60 Subpart OOO – Standards of Performance for Nonmetallic Mineral Processing Plants.  [Rule 62-213.440, F.A.C.]

Table of Contents

The specific conditions in this section apply to the following emissions unit:

| EU No. | Brief Description | Emissions Pt. |
| --- | --- | --- |
| 002 | Cage Mill Dryer System # 1 | 002 |
| 023 | Cage Mill Dryer System # 2 | 023 |

Emissions Units 002 (EU002) and Emissions Units 023 (EU0023):  Cage Mill Dryer Systems #1 and #2 manufactured by ABB Raymond Model No. 3125 with Alstom Power Preheater manufactured by Raymond, September 10 2005.  Each dryer is powered by 55MMBTU/hr natural gas fired burners.  Particulate matter and visible emissions from each emissions unit are controlled by separate 52,000 dscfm baghouse control systems.

The fabric filter’s primary purpose is to recover the dried gypsum and return it to the process.  The EUs each have a bag leak detection system (BLDS) installed.

{Permitting Note:  These emission units are regulated under 40 CFR 60, Subpart UUU, Standards of Performance for Calciners and Dryers in Mineral Industries and 40 CFR 60 (Subpart A, Standards of Performance for New Stationary Sources – General Provisions) adopted and incorporated by reference in Rule 62-204.800, F.A.C.}

Essential Potential to Emit (PTE) Parameters

- Permitted Capacity – Heat Input:  The maximum heat input rate of each emissions unit is 55 MMbtu/hr of natural gas.  [Rules 62-4.160(2), 62-204.800, 62-210.200(PTE), F.A.C.; and Permit No. 1070039-009-AC.]

- Emissions Unit Operating Rate Limitation After Testing.  See the related testing provisions in Appendix TR, Facility-wide Testing Requirements.  [Rule 62-297.310(3), F.A.C.]

- Methods of Operation Fuels:  Natural Gas is the only fuel authorized to be burned in these units.  [Rule 62-213.410, F.A.C.; Permit No. 1070039-011-AC.]

- Hours of Operation:  These emissions units may operate continuously (8,760 hours/year).  [Rule 62-210.200(PTE), F.A.C.; and, Permit Nos. 1070039-011-AC]

Emission Limitations and Standards

{Permitting Note:  The attached Table 1, Summary of Air Pollutant Standards, summarizes information for convenience purposes only.  This table does not supersede any of the terms or conditions of this permit.}

Unless otherwise specified, the averaging time(s) for Specific Condition(s) D.5. – D.6. are based on the specified averaging time of the applicable test method.

- Visible Emissions:  Visible emissions for each cage mill dryer system shall not exceed 10% opacity.  [40 CFR 60.732(b); Permit No. 1070039-011-AC; and Permit No. 1070039-023-AC]

- Particulate Matter Emissions:  Particulate Matter Emissions shall not exceed 0.017 gr/dscfm (7.58 lbs/hr, 33.19 TPY.  [40 CFR 60.732(a), Permit No. 1070039-011-AC; and Permit No. 1070039-023-AC]

{Permitting Note: PM emissions limitations below the NSPS Subpart UUU standard.  The PM emission rate (0.017 gr/dscfm) was requested by Continental to avoid the facility being classified as a PSD major source.}

- Good Air Pollution Control Practices: At all times, including periods of startup, shutdown, and malfunction, owners and operators shall, to the extent practicable, maintain and operate any affected facility including associated air pollution control equipment in a manner consistent with good air pollution control practice for minimizing emissions.  Determination of whether acceptable operating and maintenance procedures are being used will be based on information available to the Administrator which may include, but is not limited to, monitoring results, opacity observations, review of operating and maintenance procedures, and inspection of the source.  [40 CFR 60.11(d)]

Test Methods and Procedures

{Permitting Note:  The attached Table 2, Summary of Compliance Requirements, summarizes information for convenience purposes only.  This table does not supersede any of the terms or conditions of this permit.}

- Test Methods.  When required, tests shall be performed in accordance with the following reference methods:

| Method | Description of Method and Comments |
| --- | --- |
| 1-4 | Traverse Points, Velocity and Flow Rate, Gas Analysis, and Moisture Content |
| 5B | Method for Determining Particulate Matter Emissions (All PM is assumed to be PM10.) |
| 9 | Visual Determination of the Opacity of Emissions from Stationary Sources |
| 22 | Visual Determination of Fugitive Emissions from Material Sources |

The above methods are described in 40 CFR 60, Appendix A-7, and adopted by reference in Rule 62-204.800, F.A.C.  No other methods may be used unless prior written approval is received from the Department.  [Rule 62-204.800, F.A.C., and, Permit Nos. 1070039-014 and 015-AC]

- Common Testing Requirements.  Unless otherwise specified, tests shall be conducted in accordance with the requirements and procedures specified in Appendix TR, Facility-Wide Testing Requirements, of this permit.  [Rule 62-297.310, F.A.C.]

- Annual Compliance Tests Required (VE).  During each calendar year (January 1st to December 31st), each EUs 002 and 023) shall be tested to demonstrate compliance with the visible emissions limit in Specific Condition D.5.  The minimum observation period of the Method 9 test observation shall be sixty (30) minutes (as incorporated in Chapter 62-297, F.A.C.).  [Rules 62-297.310(5)(b); 62-297.310(8), F.A.C.; 40 CFR 60.736(b)(2); and Permit No. 1070039-023-AC]

- Annual Compliance Tests Required (PM).  During each calendar year (January 1st to December 31st), each EU (002 and 023) shall be tested to demonstrate compliance with the PM emissions limit as specified in Specific Condition D.6.  The sampling time and volume for each test run shall be at least 2 hours and 1.70 dscm.  [Rule 62-297.310(8), F.A.C.; 40 CFR 60.736(b)(1); and Permit No. 1070039-023-AC]

- Weekly EPA Method 22 Test Required (VE).  A weekly EPA Method 22 visible emissions compliance test shall be conducted for each emissions unit EU (002 and 023) to ensure that emissions are not visible.  The minimum observation period of the weekly EPA Method 22 test shall be twelve minutes.  In the event that the weekly test indicates visible emissions, the permittee shall undertake corrective action as well as conduct a Method 9 test with a minimum observation period of sixty (60) minutes (as incorporated in Chapter 62-297, F.A.C.).  The permittee shall maintain records documenting that the weekly compliance testing has occurred.  The records shall include color, duration, and density of the plume of any abnormal visible emissions detected, as well as the cause and corrective action taken for any abnormal visible emissions.

{Permitting note:  It is presumed that the threshold of visibility for opacity is equal to 5%.}

[In accordance with the resolution between EPA and LaFarge North America, Inc. (now known as Continental Palatka, LLC on May 9, 2002]

Monitoring Requirements

- Bag Leak Detection System (BLDS) (EU002) & (EU023).  The permittee shall calibrate, maintain, and continuously operate the BLDS on each emissions unit control device to measure and record the opacity of the emissions discharged into the atmosphere from each control device as required by Specific Condition No. D.5.  [40 CFR 60.734(a); and Permit No. 1070039-023-AC]

Recordkeeping and Reporting Requirements

- Bag Leak Detection System (BLDS).  The bag leak detection system must meet the specifications and requirements of a. through f. of this Specific Condition.

- The permittee shall follow the manufacturer installation guidelines of the BLDS.

- The BLDS sensor must provide output of relative particulate matter loadings and the permittee shall continuously record the output from the bag leak detection system using electronic or other means (e.g., using a strip chart recorder or a data logger.)

- The BLDS must be equipped with an alarm system that will sound when an increase in relative particulate loading is detected over the alarm set point established according to (e) of this Specific Condition, and the alarm must be located such that it can be heard by the appropriate plant personnel.

- For each BLDS, the permittee developed and submit to the permitting authority, for approval, a site-specific monitoring plan that addressed the items identified in (a) through (e) of this Specific Condition.  The monitoring plan shall be consistent with the recommendations contained in the U.S. Environmental Protection Agency guidance document ``Fabric Filter Bag Leak Detection Guidance'' (EPA-454/R-98-015 September 1997.).  The permittee shall operate and maintain the bag leak detection system according to the site-specific monitoring plan at all times.  The plan described the following:

- Installation of the bag leak detection system;

- Initial and periodic adjustment of the bag leak detection system including how the alarm set-point will be established;

- Operation of the bag leak detection system including quality assurance procedures;

- How the bag leak detection system will be maintained including a routine maintenance schedule and spare parts inventory list; and,

- How the bag leak detection system output shall be recorded and stored.

- The initial adjustment of the system at a minimum, consisted of establishing the baseline output by adjusting the sensitivity (range) and the averaging period of the device, and establishing the alarm set points and the alarm delay time (if applicable).

- For negative pressure, induced air baghouses, and positive pressure baghouses that are discharged to the atmosphere through a stack, the bag leak detection sensor must be installed downstream of the baghouse.

[Permit No. 1070039-023-AC]

- BLDS Corrective Action (EU 002) & (EU 023).  The permittee shall initiate procedures to determine the cause of all alarms within 1 hour of an alarm.  The cause of the alarm must be alleviated within 3 hours of the time the alarm occurred by taking the necessary corrective action(s).  Corrective actions may include, but are not limited to, the following:

- Inspecting the baghouse fabric filter for air leaks, torn or broken bags or filter media, or any other condition that may cause an increase in particulate emissions

- Replacing defective bags or filter media or otherwise repairing the control device;

- Cleaning the bag leak detection system probe or otherwise repairing the control device;

- Shutting down the process producing the particulate emissions.

[Permit No. 1070039-023-AC]

- Recordkeeping BLDS.  The permittee shall maintain records of the following information:

- All records of inspections, cleanings, repair and replacement of BLDS components.  Records shall include the date, time, personnel conducting the inspection, condition of each sensor as-found, and a description of any actions taken.

- The Operator’s Log contains documentation of alarms and resolution of alarm conditions.

- Monthly records will be maintained for drift checks and response tests performed,

and will be signed by the person conducting the inspection, testing, or maintenance.

- Values for the baseline (sensitivity) setting, response time setting, and alarm level(s) and a description of how each was established.

[Permit No. 1070039-023-AC]

- Records.  Records of the opacity measurements required in Specific Condition No. D.13. shall be retained for at least 2 years.  [Rule 62-4.070(3), F.A.C.; 40 CFR 60.735(a); and Permit No. 1070039-023-AC]

- Exceedances.  Each permittee shall submit written reports semiannually of exceedances of control device opacity required to be monitored by Specific Condition No. D.13.  For the purpose of these reports, exceedances are defined as follows:

All 6-minute periods during which the average opacity from dry control devices is greater than 10 percent.

[40 CFR 60.735(c)(1); and Permit No. 1070039-023-AC]

- Reporting Requirements.  The reporting requirements of Specific Condition No. D.18. remain in force until and unless the Agency, in delegating enforcement authority to a State under section 111(c) of the Clean Air Act, approves reporting requirements or an alternative means of compliance surveillance adopted by such State.  In that event, affected facilities within the State will be relieved of the obligation to comply with this section provided that they comply with the requirements established by the State.  [40 CFR 60.735(d); and Permit No. 1070039-023-AC]

- Other Reporting Requirements.  See Appendix RR, Facility-Wide Reporting Requirements, for additional reporting requirements.  [Rule 62-213.440(1)(b), F.A.C.]

- Federal Rule Requirements.  In addition to the specific conditions listed above, these emissions units are also subject to the applicable requirements contained in, 40 CFR 60, Subpart A – General 40 CFR 60 Subpart UUU – Standards of Performance for Calciners and Dryers in Mineral Industries.  [Rule 62-213.440, F.A.C.]

Table of Contents

The specific conditions in this section apply to the following emissions units:

| EU | Brief Description | Emission Point |
| --- | --- | --- |
| 005 | Imp Mill Air Cooling System A | EP-05 |
| 006 | Stucco Silo A (690-ton) | EP-06 - Indoor |
| 007 | Starch Silo | EP-07 |
| 008 | Sawing Systems/Dunnage Machines | EP-16 - Indoor (See EU016) |
| 013 | Wallboard Dryer (4 Natural Gas Burners) | EP-13A |
|  |  | EP-13B |
| 017 | Additives System and Pin Mixer | EP-17 - Indoors |
| 020 | Imp Mill Air Cooling System B | EP-20 |
| 021 | Stucco Silo B (690-ton) | EP-21- Indoor |

Emission Unit 005 (EU005) and Emission Unit 020 (EU020): 55 TPH Imp Mill Air Cooling System A and B, controlled by separate dust collectors.

Emission Unit 006 (EU006) and Emission Unit 021 (EU021): 100 TPH Stucco Silo A and B controlled by separate bin filters located indoors.

Emission Unit 007 (EU007): 0.66 ton/hr starch silo controlled by a silo vent filter.

Emission Unit 008 (EU008):  Sawing System/Dunnage Machines that exhaust to EU 016- Landplater Bin controlled by an end trim dust collector system.

Emission Unit 013 (EU013): Wallboard Dryer and associated 186 MMBtu/hr (total) natural gas fired burners provides direct heating in the Wallboard Dryer.  There are two stacks on the Wallboard Dryer, the first stack is located at the “wet-end” (i.e. entry point) of the Wallboard Dryer (EP-013A) and the second stack is located at the “dry-end” (EP-013B).

Emission Unit 017 (EU017): Additive System and pin Mixer controlled by a 8000 dscfm Nuisance Dust Collector located indoors.

{Permitting Note:  These emission units are regulated under Rule 62-296.320(4), F.A.C.}

Essential Potential to Emit (PTE) Parameters

- Permitted Capacity.  The maximum allowable operating rate is as follows and shall not be exceeded without prior Department approval.

| EU | Brief Description | Capacity Ton/Hr Stucco | Emissions Pt. | Control System |
| --- | --- | --- | --- | --- |
| 005 | Imp Mill Air Cooling System A | 55 | 05 | Dust collector |
| 006 | Stucco Silo A | 100 | 06 | Bin filter |
| 007 | Starch Silo | 1,320 | 07 | Vent filter |
| 008 | Sawing Systems/Dunnage Machines | N/A | 016 | End Trim Dust Control |
| 013 | Wallboard Dryer (4 Natural Gas Burners) | N/A | 13A/13B |  |
| 017 | Additives System and Pin Mixer | N/A | 017 | Nuisance Dust Collector |
| 020 | Imp Mill Air Cooling System B | 55 |  | Dust collector |
| 021 | Stucco Silo B | 100 | 021 | Bin filter |

Note 1: The fabric filter’s primary purpose is to recover product and return it to the process.

[Rules 62-4.160(2), 62-204.800, 62-210.200(PTE), F.A.C.; and, Permit Nos. 1070039-014 and 015-AC.]

- Emissions Unit Operating Rate Limitation After Testing.  See the related testing provisions in Appendix TR, Facility-wide Testing Requirements.  [Rule 62-297.310(3), F.A.C.]

- Maximum Siloxane Throughput (EU013).  The facility shall limit the 12-month total siloxane usage to 143,885 gallons of siloxane.  [Rules 62-4.070(3), F.A.C. and Permit No. 1070039-22-AC]

- Permitted Capacity - Heat Input (EU013).  The maximum heat input rate of this emissions unit is 186 MMbtu/hr. of natural gas.  [Rules 62-4.160(2), 62-204.800, 62-210.200(PTE), F.A.C.; and Permit No. 1070039-011-AC]

- Methods of Operation Fuel (EU013).  Natural Gas is the only fuel authorized to be burned in this emissions unit.  [Rule 62-213.410, F.A.C.; Permit Nos. 1070039-014 and 015-AC]

- Hours of Operation.  These emissions units may operate continuously (8,760 hours/year).  [Rule 62-210.200(PTE), F.A.C.; and, Permit Nos. 1070039-014 and 015-AC]

Emission Limitations and Standards

{Permitting Note:  The attached Table 1, Summary of Air Pollutant Standards, summarizes information for convenience purposes only.  This table does not supersede any of the terms or conditions of this permit.}

Unless otherwise specified, the averaging time(s) for Specific Conditions E.7–E.8 are based on the specified averaging time of the applicable test method.

- Visible Emissions.  Visible emissions shall not exceed:

| Emission Unit | Emission Pt. | Opacity Limit |
| --- | --- | --- |
| 006, 007, 017, and 021 | Indoors | 5% |
| 008 | 16 | 7% |

[Rule 62-297.620(4), F.A.C. and Permit Nos. 1070039-001, 004 and 011-AC]

- PM Emissions.  Particulate matter (PM) emissions shall not exceed:

| Emission Unit | Emission Pt. | Standards | Allowable limit |
| --- | --- | --- | --- |
| 005 | 05 | 0.02 gr/dscf (48,000 dscfm) | 8.23 lbs/hr. (36.04 TPY) Note 1 |
| 008 | 16 | 0.02 gr/dscf | 4.11 lbs/hr. (18.02 TPY) |
| 017 | 17 | 0.03 gr/dscf (8,000 dscfm) | 2.06 lbs/hr. ( 9.01 TPY) Note1 |
| 020 | 20 | 0.017 gr/dscf | 6.99 lb/hr, (30.63 TPY) |

{Permitting Note: Note 1Limit requested by applicant to escape PSD review} [Permit No. 1070039-015-AC]

Excess Emissions

Rule 62-210.700 (Excess Emissions), F.A.C. cannot vary any requirement of an NSPS, NESHAP or Acid Rain program provision.

- Excess Emissions Allowed.  Excess emissions resulting from startup, shutdown or malfunction of any emissions unit shall be permitted provided that best operational practices to minimize emissions are adhered to and the duration of excess emissions shall be minimized but in no case exceed two hours in any 24 hour period unless specifically authorized by the Department for longer duration.  Excess emissions that are caused entirely or in part by poor maintenance, poor operation, or any other equipment or process failure that may reasonably be prevented during startup, shutdown or malfunction shall be prohibited.  [Rule 62-210.700(1), F.A.C.]

- Excess Emissions Prohibited.  In case of excess emissions resulting from malfunctions, each owner or operator shall notify the Department or the appropriate Local Program in accordance with Rule 62-4.130, F.A.C. A full written report on the malfunctions shall be submitted in a quarterly report, if requested by the Department.  [Rule 62-210.700(5), F.A.C.]

- Excess Emissions.  After May 22, 2020, subsections 62-210.700(1), F.A.C., shall not apply to Emission limits in Chapter 62-296, F.A.C., that have been or that become incorporated into the State Implementation Plan for the State of Florida, identified in 40 C.F.R. Subpart 52.520.  [Rule 62-210.700(6)(a), F.A.C.]

Test Methods and Procedures

{Permitting Note:  The attached Table 2, Summary of Compliance Requirements, summarizes information for convenience purposes only.  This table does not supersede any of the terms or conditions of this permit.}

- Test Methods.  When required, tests shall be performed in accordance with the following reference methods:

| Method | Description of Method and Comments |
| --- | --- |
| 1-4 | Traverse Points, Velocity and Flow Rate, Gas Analysis, and Moisture Content |
| 5B | Method for Determining Particulate Matter Emissions (All PM is assumed to be PM10.) |
| 9 | Visual Determination of the Opacity of Emissions from Stationary Sources |
| 22 | Visual Determination of Fugitive Emissions from Material Sources |

The above methods are described in 40 CFR 60, Appendix A-7, and adopted by reference in Rule 62-204.800, F.A.C.  No other methods may be used unless prior written approval is received from the Department. [Rule 62-204.800, F.A.C., and, Permit Nos. 1070039-014 and 015-AC]

- Common Testing Requirements.  Unless otherwise specified, tests shall be conducted in accordance with the requirements and procedures specified in Appendix TR, Facility-Wide Testing Requirements, of this permit.  [Rule 62-297.310, F.A.C.]

- Annual Compliance Tests Required (VE).  EU008 shall be tested to demonstrate compliance with Visible Emissions (VE) standards in Specific Condition E.7.  [Rule 62-297.310(8), F.A.C.]

{Permitting Note: EU008 vents to EU 016, see Specific Condition A.10.}

- Compliance Tests Required (VE).  EUs (006, 017 and 021) compliance with Visible Emissions (VE) standards in Specific Condition E.7.  [Rule 62-297.310(8), F.A.C.]

{Permitting Note: These emission units are located indoors and testing may be required.  DEP reserves the right to require the modification of construction and operation permits to accommodate testing requirements based on EPA, 04/04/2003 letter and any additional instruction from EPA and/or the Division of Air.  If such determination is made, the permittee will be required to submit all necessary permit applications and publish required "Public Notices"}

- Annual Compliance Tests Required (PM).  During each calendar year (January 1st to December 31st), each EU (005 and 020) shall be tested to demonstrate compliance with the PM emissions limit as specified in Specific Condition E.8.  [Rule 62-297.310(8), F.A.C.]

{Permitting Note: EU (005) and EU (020): Test required due to the requested Particulate Matter maximum allowable emission rate to escape PSD review}

Recordkeeping and Reporting Requirements

- Record Keeping Siloxane Usage.  The permittee shall keep records of siloxane usage on file and made available upon request during the inspection.  [Rules 62-4.070(3), F.A.C.; and Permit No. 1070039-022-AC]

- Excess Emissions.  In the case of excess emissions resulting from malfunctions, each permittee shall notify the Department Air Program immediately in accordance with Rule 62-4.130, F.A.C.  A full written report on the malfunctions shall be submitted in a quarterly report, if requested by the Department.  [Rule 62-210.700(5), F.A.C.]

- Other Reporting Requirements.  See Appendix RR, Facility-Wide Reporting Requirements, for additional reporting requirements.

Table of Contents

The specific conditions in this section apply to the following emissions units:

| EU No. | Description | Model No. | HP | Fuel Type | Year built |
| --- | --- | --- | --- | --- | --- |
| 028 | Perkins Diesel Fired Emergency Generator Engine | 2076/1800 | 219 | diesel | 2000 |
| 029 | Perkins Diesel Fired Emergency Generator Engine | 1797/1500 | 166 | diesel | 2000 |
| 030 | John Deere Water Pump | 6081AF001 | 275 | diesel | 2000 |

This facility operates two Perkins diesel fired emergency generator engines and a John Deere diesel fire water pump engine.  All three engines were manufactured and installed in 2000 and are subject to regulation under 40 CFR 63 Subpart ZZZZ, - National Emissions Standards for Hazardous Air Pollutants for Stationary Reciprocating Internal Combustion Engines.

{Permitting Note: These engines are subject to 40 CFR 63 Subpart ZZZZ National Emissions Standards For Hazardous Air Pollutants For Stationary Reciprocating Internal Combustion Engines.  In accordance with the definitions of this subpart, these engines are existing, stationary RICE}.

Essential Potential to Emit (PTE) Parameters

- NESHAP, 40 CFR 63 Subpart ZZZZ Applicability.  These diesel engines are classified as existing, stationary Reciprocating Internal Combustion Engines (RICE) and shall comply with applicable provisions of 40 CFR 63 Subpart ZZZZ.  These engines are classified as Emergency stationary RICE and are used to pump water for fire suppression.  [40 CFR 63.6675(def); 40 CFR 63.6585(a) and (c)]

- 40 CFR 63, Subpart A-General Provision.  Table 8 of 40 CFR 63 Subpart ZZZZ, shows which parts of the General Provisions in 40 CFR 63.1 through 40 CFR 63.15 are applicable.  [40 CFR 63.6665]

- Method of Operation - Emergency Stationary RICE.  The emergency stationary RICE shall be operated according to the requirements in paragraphs a. through c. of this Condition.  Any operation other than emergency operation, maintenance and testing, and operation in non-emergency situations for 50 hours per year, as described in paragraphs a. through c. of this Condition, is prohibited.  If the permittee does not operate the engine according to the requirements in paragraphs a. through c. of this Specific Condition, the engines will not be considered emergency engines under 40 CFR 63 Subpart ZZZZ and will need to meet all requirements for non-emergency engines.

- There is no time limit on the use of emergency stationary RICE in emergency situations.

- The emergency stationary RICE may be operated for the purpose of maintenance checks and readiness testing, provided that the tests are recommended by Federal, State or local government, the manufacturer, the vendor, or the insurance company associated with the engine.  Maintenance checks and readiness testing of such units is limited to 100 hours per year.  The permittee may petition the Administrator for approval of additional hours to be used for maintenance checks and readiness testing, but a petition is not required if the permittee maintains records indicating that Federal, State, or local standards require maintenance and testing of emergency RICE beyond 100 hours per year.

- The emergency stationary RICE may be operated up to 50 hours per year in non-emergency situations, but those 50 hours are counted towards the 100 hours per year provided for maintenance and testing. The 50 hours per year for non-emergency situations cannot be used for peak shaving or to generate income for a facility to supply power to an electric grid or otherwise supply power as part of a financial arrangement with another entity; except that owners and operators may operate the emergency engine for a maximum of 15 hours per year as part of a demand response program if the regional transmission organization or equivalent balancing authority and transmission operator has determined there are emergency conditions that could lead to a potential electrical blackout, such as unusually low frequency, equipment overload, capacity or energy deficiency, or unacceptable voltage level.  The engine may not be operated for more than 30 minutes prior to the time when the emergency condition is expected to occur, and the engine operation must be terminated immediately after the facility is notified that the emergency condition is no longer imminent.  The 15 hours per year of demand response operation are counted as part of the 50 hours of operation per year provided for non-emergency situations.  The supply of emergency power to another entity or entities pursuant to financial arrangement is not limited by this (iii), as long as the power provided by the financial arrangement is limited to emergency power.

[40 CFR 63.6640(f)(1) - (f)(2)(i); and (3)]

Emission Limitations and Standards

- Each engine shall comply with the following emission standards:

- Change oil and filter every 500 hours of operation or annually, whichever comes first1;

- Inspect air cleaner every 1,000 hours of operation or annually, whichever comes first; and

- Inspect all hoses and belts every 500 hours of operation or annually, whichever comes first, and replace as necessary.

1	Sources have the option to utilize an oil analysis program as described in Specific Condition F.5. in order to extend the specified oil change requirement.

[40 CFR 63.6603(a), Table 2d]

- Oil Analysis Program Option.  The permittee has the option of utilizing an oil analysis program in order to extend the specified oil change requirement stated in Specific Condition F.4.  The oil analysis must be performed at the same frequency specified for changing the oil in Specific Condition F.4.  The analysis program must at a minimum analyze the following three parameters:  Total Base Number, viscosity, and percent water content.  The condemning limits for these parameters are as follows:  Total Base Number is less than 30 percent of the Total Base Number of the oil when new; viscosity of the oil has changed by more than 20 percent from the viscosity of the oil when new; or percent water content (by volume) is greater than 0.5. If all of these condemning limits are not exceeded, the engine permittee is not required to change the oil.  If any of the limits are exceeded, the engine permittee must change the oil within 2 days of receiving the results of the analysis; if the engine is not in operation when the results of the analysis are received, the engine permittee shall change the oil within 2 days or before commencing operation, whichever is later.  The permittee shall keep records of the parameters that are analyzed as part of the program, the results of the analysis, and the oil changes for the engine.  The analysis program must be part of the maintenance plan for the engine.  [40 CFR 63.6625(i)]

General compliance requirements

- NESHAP Subpart ZZZZ.  The permittee shall be in compliance with the emission limitations and operating limitations in 40 CFR 63 Subpart ZZZZ that applies at all times.  [40 CFR 63.6605(a)]

- Good Pollution Control Practices.  At all times the permittee shall operate and maintain any affected source, including associated air pollution control equipment and monitoring equipment, in a manner consistent with safety and good air pollution control practices for minimizing emissions.  The general duty to minimize emissions does not require you to make any further efforts to reduce emissions if levels required by this standard have been achieved.  Determination of whether such operation and maintenance procedures are being used will be based on information available to the Administrator which may include, but is not limited to, monitoring results, review of operation and maintenance procedures, review of operation and maintenance records, and inspection of the source.  [40 CFR 63.6605(b)]

- Manufacturer Related Instructions.  The stationary RICE and after-treatment control device (if any) shall be operated and maintained according to the manufacturer's emission-related written instructions or develop your own maintenance plan which must provide to the extent practicable for the maintenance and operation of the engine in a manner consistent with good air pollution control practice for minimizing emissions.  [40 CFR 63.6625(e) and (e)(3)]

- Non-resettable Hour Meter.  A non-resettable hour meter shall be installed if one is not already installed.  [40 CFR 63.6625(f)]

- Engine Startup.  The engine's time spent at idle during startup and the engine's startup time to a period needed for appropriate and safe loading of the engine shall be minimized, not to exceed 30 minutes, after which time the emission standards applicable to all times other than startup in 40 CFR 63 Subpart ZZZZ Tables 1a, 2a, 2c, and 2d apply.  [40 CFR 63.6625(h)]

Monitoring of Operations

- Continuous Compliance Demonstration.  The permittee shall demonstrate continuous compliance with each emission limitation and operating limitation in Specific Condition F.4. according to methods specified below:

- Operating and maintaining the stationary RICE according to the manufacturer's emission-related operation and maintenance instructions; or

- Develop and follow your own maintenance plan which must provide to the extent practicable for the maintenance and operation of the engine in a manner consistent with good air pollution control practice for minimizing emissions.

[40 CFR 63.6640(a), Table 6, No. 9.a.i.and ii]

Recordkeeping and Reporting Requirements

- Maintenance Records.  Records of the maintenance conducted on the stationary RICE shall be kept in order to demonstrate that the stationary RICE and after-treatment control device (if any) is operated and maintained according to the permittee’s own maintenance plan.  [40 CFR 63.6655(e)(2)]

- Hours of Operation Records.  Records of the hours of operation of the engine shall be kept that is recorded through the non-resettable hour meter.  The permittee shall document how many hours are spent for emergency operation, including what classified the operation as emergency and how many hours are spent for non-emergency operation.  [40 CFR 63.6655(f)]

- Records.  Records shall meet the following:

- Records must be in a form suitable and readily available for expeditious review according to 40 CFR 63.10(b)(1).

- As specified in 40 CFR 63.10(b)(1), each record shall be kept for 5 years following the date of each occurrence, measurement, maintenance, corrective action, report, or record.

- Each record shall be kept readily accessible in hard copy or electronic form for at least 5 years after the date of each occurrence, measurement, maintenance, corrective action, report, or record, according to 40 CFR 63.10(b)(1).

[40 CFR 63.6660]

- Each instance shall be reported in which each emission limitation or operating limitation in Specific Condition F.4. is not met.  These instances are deviations from the emission and operating limitations in this subpart.  These deviations must be reported according to the requirements in 40 CFR 63.6650.  [40 CFR 63.6640(b)]

Table of Contents

The specific conditions in this section apply to the following emissions units:

| EU | Brief Description |
| --- | --- |
| 031 | Railcar unloading |
| 032 | Feed hopper to belt C1 |
| 033 | Transfer point belt C1 to C2 |
| 034 | Transfer point belt C2 to C3 |
| 035 | Transfer point belt C3 to TRS |
| 036 | TRS to Stockpile |

Air pollutant emissions from these units are not captured and therefore do not exit a stack.  But this does not mean the emissions do not count towards PSD applicability.

{Permitting Note:  The emission unit(s) 031-035 are regulated by 40 CFR 60, NSPS Subpart OOO – Standards of Performance for Nonmetallic Mineral Processing Plants and 40 CFR 60, NSPS Subpart A – General Provisions.  EUs 031-035 are each subject to an opacity standard of 7% that applies to fugitive emissions from NSPS Subpart OOO affected facilities constructed on or after April 22, 2008. EU 036, the TRS conveyor drop point to the existing stockpile, is an NSPS Subpart OOO affected facility but is not subject to an NSPS Subpart OOO emission or opacity limit. EU 036 is subject to a Permitted Capacity maximum transfer rate restriction of 1,200 TPH listed in Specific Condition No. G.1. and a 20% fugitive facility-wide standard (Rule 62-296.320(4)).}

Essential Potential to Emit (PTE) Parameters

- Permitted Capacity.  The feed hopper and each conveyor belt (C1, C2, C3, and TRS) a part of EUs 031-036 shall not exceed a maximum transfer rate of 1,200 TPH of gypsum.  [Rule 62-210.200(PTE), F.A.C.; and Permit No. 1070039-027-AC]

- Hours of Operation/Restricted Operation.  The hours of operation are not limited (8,760 hours/year).  [Rule 62-210.200(PTE), F.A.C.; and Permit No. 1070039-027-AC]

Emission Limitations and Standards

{Permitting Note:  The attached Table 1, Summary of Air Pollutant Standards, summarizes information for convenience purposes only.  This table does not supersede any of the terms or conditions of this permit.}

Unless otherwise specified, the averaging time(s) for Specific Condition G.3 is based on the specified averaging time of the applicable test method.

- Visible Emissions.  Visible emissions (VE) shall not exceed 7% opacity from each of EU 031-035. [Permit No. 1070039-027-AC; Table 3 to 40 CFR 60, NSPS Subpart OOO; and Rule 62-204.800, F.A.C.]

Test Methods and Procedures

{Permitting Note:  The attached Table 2, Summary of Compliance Requirements, summarizes information for convenience purposes only.  This table does not supersede any of the terms or conditions of this permit.}

- Test Methods.  When required, tests shall be performed in accordance with the following reference methods:

| Method | Description of Method and Comments |
| --- | --- |
| 9 | Visual Determination of the Opacity of Emissions from Stationary Sources |

The above methods are described in 40 CFR 60, Appendix A-4, and adopted by reference in Rule 62-204.800, F.A.C.  No other methods may be used unless prior written approval is received from the Department.  [Rule 62-204.800, F.A.C., and, Permit No. 1070039-027-AC]

- Common Testing Requirements.  Unless otherwise specified, tests shall be conducted in accordance with the requirements and procedures specified in Appendix TR, Facility-Wide Testing Requirements, of this permit.  [Rule 62-297.310, F.A.C.]

- Repeat Compliance Tests.  Within five years of the previous VE performance test, each EU 031-035 shall each be tested to demonstrate compliance with the emissions standards for VE shown in Specific Condition No. G.3.  [Table 3 to 40 CFR 60 Subpart OOO; and Rules 62-204.800 and 62-297.310(8)(a)5.a. and (8)(b), F.A.C.]

{Permitting Note: The applicant has asserted that EUs 032 and 033 will be below ground.  This means the Initial and Repeat Compliance Tests for EUs 032 and 033 will require the observer to select and observe VE at an above ground appropriate upstream and/or downstream compliance test location from EUs 032 and 033.}

- Test Requirements.  The permittee shall notify the Compliance Authority in writing at least 7 days prior to any required tests.  Tests shall be conducted in accordance with the procedures in 40 CFR 60.11 with the following additions.

- The duration of the Method 9 observations must be 30 minutes (five 6-minute averages). Compliance with the VE standard in Specific Condition No. G.3. shall be based on the average of the five 6-minute averages.

- The minimum distance between the observer and the emission source shall be 4.57 meters (15 feet).

- The observer shall, when possible, select a position that minimizes interference from other fugitive emission sources (e.g., road dust).  The required observer position relative to the sun (Method 9 of appendix A-4 of this part, Section 2.1) must be followed.

- For affected facilities using wet dust suppression for particulate matter control, a visible mist is sometimes generated by the spray. The water mist must not be confused with particulate matter emissions and is not to be considered a visible emission. When a water mist of this nature is present, the observation of emissions is to be made at a point in the plume where the mist is no longer visible.

- If emissions from two or more facilities continuously interfere so that the opacity of fugitive emissions from an individual affected facility cannot be read, either of the following procedures may be used:

  - Use for the combined emission stream the highest fugitive opacity standard applicable to any of the individual affected facilities contributing to the emissions stream.

  - Separate the emissions so that the opacity of emissions from each affected facility can be read.

- A single visible emission observer may conduct visible emission observations for up to three fugitive, stack, or vent emission points within a 15-second interval if the following conditions are met:

  - No more than three emission points may be read concurrently.

  - All three emission points must be within a 70 degree viewing sector or angle in front of the observer such that the proper sun position can be maintained for all three points.

  - If an opacity reading for any one of the three emission points equals or exceeds the applicable standard, then the observer must stop taking readings for the other two points and continue reading just that single point.

[40 CFR 60.675(c) and 40 CFR 60.675(e); and Rule 62-204.800, F.A.C.]

- Enclosed Conveyor Belt Transfer Points.  EUs 033, 034 and 035 are enclosed conveyor belt transfer points that result in no ambient air particulate emissions.  For each repeat (Specific Condition No. G.6.) VE compliance test, the observer is to evaluate opacity at each above ground conveyor belt transfer point (EUs 034 and 035) and at an appropriate upstream and downstream location from each transfer point (EUs 033, 034, 035).  If at any location at any point-in-time the observed opacity reading is above 0% related to a VE compliance test for EUs 033, 034, 035, then the opacity reading shall be reported and the applicant shall file a permit amendment within 180 days of the first opacity value recorded above 0%.  The permit amendment shall incorporate particulate emissions from each enclosed conveyor belt transfer point (EUs 033, 034 and 035).  [Rule 62-210.200(PTE), F.A.C.; and Permit No. 1070039-027-AC]

Recordkeeping and Reporting Requirements

- Test Reports.  The permittee shall prepare and submit reports for all required tests in accordance with the requirements specified in Appendix TR (Facility-wide Testing Requirements) of this permit.  For each test run, the report shall also indicate the gypsum transfer rate in TPH during the test and for test runs for the enclosed conveyor belt transfer points (EUs 033, 034, and 035) the report shall also specify the upstream and/or downstream locations monitored for VE testing.  [Rule 62-297.310(10), F.A.C.]

- Operational Data.  The permittee shall maintain records to document the railcar unloading amount of gypsum by month and the twelve-month rolling total.  [Permit No. 1070039-027-AC]

Table of Contents