##### PERMITTEE

Florida Power & Light Company (FPL)

700 Universe Boulevard, JES/JB

Juno Beach, FL 33408

##### Permitting Authority

Florida Department of Environmental Protection (Department)

Division of Air Resource Management

Office of Permitting and Compliance

2600 Blair Stone Road, MS #5505

Tallahassee, Florida 32399-2400

##### PROJECT

Permit No. 0110037-011-AC (PSD-FL-423)

Major Source PSD Air Construction Permit

Peaker Unit Replacement Project – FPL Lauderdale Plant

The combined 1,512 megawatt (MW) generation capacity of the 36 existing gas turbine (GT) peaking units at the FPL Lauderdale and Port Everglades Plants will be replaced with five nominal 200 MW combustion turbines (CTs) at the Lauderdale Plant to provide continuing peaking resources while also reducing emissions and providing far superior emission profiles and efficiency.  The new CTs will be designated Units 6A through 6E at the Lauderdale Plant.  No new CTs will be installed at the Port Everglades Plant.  In addition, four nominal 3,100 kilowatt emergency generators firing ultra-low sulfur distillate (ULSD) fuel oil may be added to provide black start capability, i.e., these emergency generators will be used when electric power is not available to start the CTs.  Alternatively, two existing GTs may be retained for the purposes of black start capability.  The project will include a 300 horsepower fire pump engine using ULSD oil along with two new 3-million gallon ULSD fuel oil storage tanks.

This project was subject to PSD preconstruction review in accordance with Rule 62-212.400, F.A.C.  Best Available Control Technology (BACT) determinations were required for nitrogen oxides (NOX), carbon monoxide (CO), particulate matte (PM), PM with a mean diameter of 10 microns or less (PM10), PM with a mean diameter of 2.5 microns or less (PM2.5) and volatile organic compounds (VOC).  The Department’s BACT determinations for these pollutants are based on:  Lean premix Dry Low NOX technology while firing natural gas and water injection while firing fuel oil to control NOX emissions; good combustion practices to control CO and VOC emissions; and clean, low sulfur fuels to control to control PM/PM10/PM2.5 emissions.  Low sulfur fuels provide the added benefit of controlling SO2 and sulfuric acid mist emissions.

##### NOTICE AND PUBLICATION

The Department distributed a draft Major Source PSD air construction permit package on February 27, 2014.  The applicant published the Public Notice in the Sun Sentinel on March 19, 2014.  No requests for administrative hearings or requests for extensions of time to file a petition for administrative hearing were received.

# COMMENTS

No comments on the Draft Permit were received from the public, the EPA Region 4 Office or the applicant.

# CONCLUSION

The final action of the Department is to issue the permit with no changes.