Permittee:

G.A.P. Roofing of Florida, LLC			      Permit/Cert Number:	0470015-005-AO

Mid-America Industrial Park, 4444 Hunt Street	      Date of Issue:		July 22, 2014

Pryor, Oklahoma 74361				      Expiration Date:	             July 22, 2019

Authorized Representative:

Mr. Glen Passmore, President			Project:	Air Operation Permit Renewal

#### Project And Location

This permit authorizes the operation of Emission Unit 001, which is an Asphalt Saturated Felt Manufacturing Operation (Standard Industrial Classification No. 2952).  The facility is located in Hamilton County at 12120 Southwest County Road 137, Jasper, Florida.  The UTM coordinates are Zone 17, 317.01 km East, and 3372.32 km North.

This final permit is organized by the following sections.

Section 1.  General Information

Section 2.  Administrative Requirements

Section 3.  Emissions Unit Specific Conditions

Section 4.  Appendices

Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of Section 4 of this permit.

Upon issuance of this final permit, any party to this order has the right to seek judicial review of it under Section 120.68 of the Florida Statutes by filing a notice of appeal under Rule 9.110 of the Florida Rules of Appellate Procedure with the clerk of the Department of Environmental Protection in the Office of General Counsel (Mail Station #35, 3900 Commonwealth Boulevard, Tallahassee, Florida, 32399-3000) and by filing a copy of the notice of appeal accompanied by the applicable filing fees with the appropriate District Court of Appeal.  The notice must be filed within 30 days after this order is filed with the clerk of the Department.

Executed in Jacksonville, Florida.

Richard S. Rachal III, P.G.

Program Administrator

Waste and Air Resource Management Program

CERTIFICATE OF SERVICE

The undersigned duly designated deputy agency clerk hereby certifies that this Final Air Permit package (including the Notice of Issuance and Final Permit) was sent by electronic mail (or a link to these documents made available electronically on a publicly accessible server) with received receipt requested before the close of business on July 22, 2014 to the persons listed below.

# Mr. Glen Passmore, G.A.P. Roofing (e-mail address: glenpassmore@yahoo.com)

# Mr. Dave Swaroop, Plant Manager, G.A.P. Roofing of Florida, LLC (e-mail address: dave.swaroopgap@gmail.com)

# Ms. Kristin C. Ruyle, Blackshare Environmental Solutions (e-mail address: KRuyle@Blackshare-env.com)

# Clerk Stamp

# FILING AND ACKNOWLEDGMENT FILED, on this date, pursuant to Section 120.52(7), Florida Statutes, with the designated agency clerk, receipt of which is hereby acknowledged.

___________________________       July 22, 2014

(Clerk)				(Date)

Facility and Project Description

GAP Roofing of Florida, LLC is an asphalt-roofing manufacturer. The facility operates line 1 for asphalt saturated felt.

#### Line 1 – Asphalt Saturated Felt

At Line 1, organic felt is unrolled from an unwind stand onto a dry looper, which maintains a constant tension on the material. From the dry looper the felt will pass into a saturator dip section where the felt is drawn over a series of rollers, with the bottom rollers submerged in hot asphalt at 400º F to 480 º F. During the next step, heated drying-in drums provide the heat for the asphalt to penetrate the felt. The saturated felt then passes through water-cooled rolls onto the finish floating looper, and then is rolled and cut to product size on the roll winder. The product is then stored.

Incoming asphalt for the saturation process is stored in a 19,500-gallon receiving/saturator run tank (Tank A). The tank contents are maintained at a temperature of approximately 425 º F. Heated asphalt from the tank will be conveyed to the saturator at Line 1. Heat for the process is supplied by a 3.0 MMBtu/hour burner.

The emissions from Line 1 are controlled by a CECO Mist Collector System with a fiberglass/charcoal pre-filter. A sample picture of the unit is as shown below.

## Regulated Emission Units:

EU No.	     EU Description					     Control Device

001	    Line #1 – Asphalt saturated felt manufacturing 	      CECO Mist Collector System

operation consisting of asphalt receiving/saturator

run tank, saturator, drying-in drums and cooler.

Generic Emissions Unit Exemptions: The following emission units or pollutant-emitting activities meet the criteria for the Generic Emissions Unit Exemption pursuant to Rule 62-210.300(3)(b) 1., F.A.C:

3.0 MMBTU/hr Natural gas fired heater

1.8 MMBTU/hr Natural gas fired heater

Facility Regulatory Classifications

- The facility is not a major source of hazardous air pollutants (HAP).

- The facility has no units subject to the acid rain provisions of the Clean Air Act (CAA).

- The facility is not a Title V major source of air pollution in accordance with Chapter 213, F.A.C.

- The facility is not a major stationary source in accordance with Rule 62-212.400(PSD), F.A.C.

- The facility has one or more emissions units subject to NSPS.

The facility is subject to Appendix A	40 CFR 60, Subpart A – General Provisions (NSPS) and 40 CFR 60, Subpart UU—Standards of Performance for Asphalt Processing and Asphalt Roofing Manufacture.

The facility is a minor source of air pollution because the potential emissions of regulated air pollutants are less than 100 tons per year, and individual and total hazardous air pollutants are less than 10 and 25 tons per year, respectively in accordance with Chapter 62-210, F.A.C.

##### Relevant Documents

The documents listed below are the basis of the permit.  They are specifically related to this permitting action.  These documents are on file with the Department:

# Application for Air Permit – Non-Title V Source received July 21, 2014.

- Permitting Authority:  The permitting authority for this project is the Florida Department of Environmental Protection (Department), Northeast District Office, Waste and Air Resource Management Program.  The Northeast District Office’s mailing address is 8800 Baymeadows Way West, Suite 100, Jacksonville, Florida 32256.  All documents related to applications for permits to operate an emissions unit shall be submitted to the Northeast District Office.

- Compliance Authority:  All documents related to compliance activities such as reports, tests, and notifications shall be submitted to the Northeast District Office, Compliance Assurance.  The mailing address and phone number of the Northeast District Office is:  8800 Baymeadows Way West, Suite 100, Jacksonville, Florida 32256 and Phone Number (904) 256-1700.

- Appendices:  The following Appendices are attached as part of this permit:

- Appendix A.  Citation Formats and Glossary of Common Terms;

- Appendix B.  General Conditions;

- Appendix C.  Common Conditions; and

- Appendix D.  Common Testing Requirements.

- Appendix E.  40 CFR 60 Subpart A – General Provisions

- 40 CFR 60 Subpart UU – Standards of Performance for Asphalt Processing and Asphalt Roofing Manufacture

- Applicable Regulations, Forms and Application Procedures:  Unless otherwise specified in this permit, the construction and operation of the subject emissions units shall be in accordance with the capacities and specifications stated in the application.  The facility is subject to all applicable provisions of: Chapter 403, F.S.; and Chapters 62-4, 62-204, 62-210, 62-212, 62-213, 62-296 and 62-297, F.A.C.  Issuance of this permit does not relieve the permittee from compliance with any applicable federal, state, or local permitting or regulations.

- New or Additional Conditions:  For good cause shown and after notice and an administrative hearing, if requested, the Department may require the permittee to conform to new or additional conditions.  The Department shall allow the permittee a reasonable time to conform to the new or additional conditions, and on application of the permittee, the Department may grant additional time.

[Rule 62-4.080, F.A.C.]

- Modifications:  The permittee shall notify the Compliance Authority upon commencement of construction.  No new emissions unit shall be constructed and no existing emissions unit shall be modified without obtaining an air construction permit from the Department.  Such permit shall be obtained prior to beginning construction or modification.

[Rules 62-210.300(1) and 62-212.300(1)(a), F.A.C.]

- Construction Permit Required.  Unless exempt from permitting pursuant to Rule 62-210.300(3)(a) or (b), F.A.C., or Rule 62-4.040, F.A.C., an air construction permit shall be obtained by the owner or operator of any proposed new, reconstructed, or modified facility or emissions unit, or any new pollution control equipment prior to the beginning of construction, reconstruction pursuant to CFR 60.15 or 63.2, or modification of the facility or emissions unit or addition of the air pollution control equipment; or to establish a PAL; in accordance with all applicable provisions of Chapter 62-210, F.A.C., Chapter 62-212, F.A.C., and Chapter 62-4, F.A.C.

[Rule 62-210.300(1)(a), F.A.C.]

- Application for Non-Title V Operation Permit Renewal:  A completed Application for Air Permit –Non-Title V Source (DEP Form No. 62-210.900(3), F.A.C.), shall be submitted to the Department at least 60 days prior to the expiration date of this operation permit.  To properly apply for an operation permit, the permittee shall submit the appropriate application form, processing fee, and compliance test reports as required by this permit.

[Rules 62-4.030, 62-4.055, 62-4.220, 62-210.300(1)(a), and 62-4.210.300(2), F.A.C.]

- The ID Number and Project Name for this source shall be used on all correspondences.

The following specific conditions apply to the Emissions Unit and Emission Points listed below:

| Emissions Unit | Brief Description |
| --- | --- |
| 001 | Line #1 – Asphalt saturated felt manufacturing operation consisting of asphalt receiving/saturator run tank, saturator, drying-in drums and cooler. The emissions from Line 1 are controlled by a CECO Mist Collector System with a fiberglass/charcoal pre-filter. |

##### The facility is subject to the following regulations: Florida Administrative Code Chapters 62-4; 62-103; 62-204; 62-210; 62-212, 62-296, 62-297 and 40 CFR 60, Subpart UU—Standards of Performance for Asphalt Processing and Asphalt Roofing Manufacture.  Issuance of this permit does not relieve the facility owner or operator from compliance with any applicable federal, state, or local permitting requirements or regulations.

Operational Parameters

A.1.	Hours of Operation. The hours of operation are not restricted: 24 hours/day; 7 days/week; 52 weeks/year (8760 hours/year and 8784 during a Leap Year).

[Rules 62-4.160(2) & (14)(b), 62-210.200(PTE), F.A.C.]

A.2. 	Maximum Operation Rate. The maximum saturated felt production rate is 70,200 tons per year.

[Permit No. 0470015-002-AC, Rules 62-4.160(2) and 62-210.200(PTE), F.A.C.]

A.3. 	General Provision of 40 CFR 60. The permittee shall comply with the requirements of 40 CFR Part 60, Subpart A- General Provisions. See attached Appendix E. [40 CFR 60.1 (a)]

Emission Limits and Standards

{Permitting Note:  Unless otherwise specified, the averaging times for these conditions are based on the specified averaging time of the applicable test method.}

A.4. 	Emissions Limit(s). The permitted emissions limits for the saturator are as follows:

| Affected Source | Pollutant | Emission Limit | Rule |
| --- | --- | --- | --- |
| Saturator | Particulate Matter (PM) | 0.8 lb/ton of saturated felt or smooth-surfaced roll roofing produced | 40 CFR 60.472(a)(1)(ii) |
|  |  | 5.85 lb/hr; 28.08 tons/year | Permit No. 0470015-002-AC |
|  | Visible Emissions (VE) | Not more than 20% opacity | 40 CFR 60.472(a)(2) |

A.5.    Opacity Limit for Asphalt Storage Tank A. The owner or operator shall not cause to be discharged into the atmosphere from any asphalt storage tank exhaust gases with opacity greater than 0 percent, except for one consecutive 15-minute period in any 24-hour period when the transfer lines are being blown for clearing. The control device shall not be bypassed during this 15-minute period. If, however, the emissions from the asphalt storage tank(s) are ducted to a control device for a saturator, the combined emissions shall meet the emission limit contained in Specific Condition A.4. of this permit during the time the saturator control device is operating. At any other time the asphalt storage tank(s) must meet the opacity limit specified above for storage tanks.

[40 CFR 60.472 (c)]

A.6.    No owner or operator shall cause to be discharged into the atmosphere from any mineral handling and storage facility emissions with opacity greater than 1 percent.

[40 CFR 60.472 (d)]

###### MONITORING OF OPERATIONS

A.7.    Pressure Drop Monitoring. The owner or operator shall monitor and record the pressure drop across the mist collector once every 8-hour period when operating. The normal operating range for the pressure drop shall be between 2 to 10 inches of water (max 12 inches).

[40 CFR 60.473 (c), Permit No. 0470015-002-AC]

Test Methods and Procedures

A.8.  Operating Rate During Compliance Test. For saturators, if the final product is saturated felt or smooth-surfaced roll roofing, the tests shall be conducted while 6.8-kg (15-lb) felt is being produced.

[40 CFR 60.474 (a)(2)]

A.9.  PM/VE Compliance Test. The owner or operator shall determine compliance with the particulate matter and VE standards as follows:

(a) The emission rate (E) of particulate matter shall be computed for each run using the following equation:

E=(cs Qsd)/(PK)

where:

E=emission rate of particulate matter, kg/Mg (lb/ton).

cs=concentration of particulate matter, g/dscm (gr/dscf).

Qsd=volumetric flow rate of effluent gas, dscm/hr (dscf/hr).

P=asphalt roofing production rate or asphalt charging rate, Mg/hr (ton/hr).

K=conversion factor, 1000 g/kg [7000 (gr/lb)].

(b) Method 5A, as stated in 40 CFR 60 Appendix A shall be used to determine the particulate matter concentration (cs) and volumetric flow rate (Qsd) of the effluent gas. For a saturator, the sampling time and sample volume for each run shall be at least 120 minutes and 3.00 dscm (106 dscf).

A.9. continued:

(c) 	For the saturator, the asphalt roofing production rate (P) for each run shall be determined as follows: The amount of asphalt roofing produced on the saturated felt process lines shall be obtained by direct measurement. The asphalt roofing production rate is the amount produced divided by the time taken for the run.

(d) Method 9, as stated in 40 CFR 60 Appendix A and the procedures in 40 CFR 60.11 shall be used to determine opacity.

[40 CFR 60.474 (b) &(c); Rule 62-297.320, F.A.C.]

A.10.  Compliance Test Frequency. The owner or operator shall conduct the performance test for PM emissions as stated in Specific Condition A.9. on a 5-year basis, prior to obtaining a renewed operation permit. The owner or operator shall conduct VE test as stated in Specific Condition A.9. annually, during each federal fiscal year (October 1 – September 30) to demonstrate compliance with the opacity limit.

[Rule 62-297.310(7)(a) 3 & 4, F.A.C.]

A.11.  EU001 was last operated in July 2009. Therefore, Permit No. 0470015-005-AO extends EU001 expiration date only until July 22, 2019. The permit cannot be renewed again in 2019 should the facility not operate prior to the 2019 expiration date.

[Rule 62-210.300(2)(a)3.b. & c.]