M E M O R A N D U M

DATE:	March 14, 2012

TO:	Richard D. Garrity, Ph.D.

FROM:	Stephen R. Hathaway, P.E.		THRU:	Sterlin K. Woodard, P.E.

SUBJECT:	FINAL Air Construction Permit

Florida Rock Industries, Inc.

Permit No. 0570018-019-AC

Attached is FINAL Air Construction Permit No. 0570018-019-AC which is being issued to Florida Rock Industries, Inc. (“Florida Rock”) located at 2001 Maritime Blvd., Tampa, FL.  This permit is being issued as a minor modification to Permit No. 0570018-017-AC/018-AV in response to the written request dated November 17, 2011 from Florida Rock.  The letter requested the authorization of multiple changes at the facility which did not require substantial technical review; therefore, this permit is being issued even though a formal permit application was not submitted for the requested changes.  The facility requested several minor changes in the method of operation of existing emission units in the blending plant, packhouse, bulk silos, and bulk coarse/granular material handling operations. The facility’s process description and emission unit descriptions have been revised to address the requested changes.  Initial visible emissions tests are being required upon initial operation under the modified configuration of each emission unit being affected by this permit.

The DRAFT Air Construction Permit package was issued on December 30, 2011.  Florida Rock requested an extension of time to file petition on January 5, 2012 and was granted the extension until March 12, 2012.  One written comment was received from Florida Rock Industries on January 18, 2012 regarding the Bulk Silo operations.  The change was made as requested, and the REVISED DRAFT permit package was issued on February 28, 2012.  The facility withdrew their request for extension and published the Notice of Intent in The Tampa Tribune on February 28, 2012.  No comments were received from the general public since the NOI was issued.

Based upon our review, we recommend the issuance of the FINAL AC permit as drafted.

SRH: 	0570018-019-AC

FINAL DETERMINATION

FOR

Florida Rock Industries, Inc.

Hillsborough County

Air Construction Permit

Application Number

0570018-019-AC

Environmental Protection Commission of

Hillsborough County

Tampa, FL

March 14, 2012

The Environmental Protection Commission of Hillsborough County mailed a public notice package on December 30, 2011 that included the Intent to Issue AC Permit No. 0570018-019-AC to Florida Rock Industries, Inc., located at 2001 Maritime Boulevard, Tampa, FL 33605.  The AC Permit is for the minor modification of the blending plant, packhouse, bulk silos, and bulk coarse/granular material handling operations.  Florida Rock requested an extension of time to file petition on January 5, 2012 and was granted the extension until March 12, 2012.  One written comment was received from Florida Rock Industries on January 18, 2012 regarding the Bulk Silo operations.  The change was made as requested, and the REVISED DRAFT permit package was issued on February 28, 2012.

The Public Notice of Intent to Issue was published on February 28, 2012 in The Tampa Tribune.

No comments were received from the public since the Intent was published.

The final action of the Environmental Protection Commission of Hillsborough County is to issue the FINAL AC permit.

ENVIRONMENTAL PROTECTION COMMISSION OF

HILLSBOROUGH COUNTY, as Delegated by

STATE OF FLORIDA

DEPARTMENT OF ENVIRONMENTAL PROTECTION

NOTICE OF PERMIT

Harrinarine Mootoor

Florida Rock Industries, Inc.

2001 Maritime Blvd.

Tampa, FL 33605

Dear Mr. Mootoor:						Re:  Hillsborough County - AP

Enclosed is Permit Number No. 0570018-019-AC to authorize multiple minor changes at the facility that were requested as part of the minor modification request dated November 17, 2011.  The facility requested minor changes in the operations of the blending plant, packhouse silos, bulk silos, and material handling operations.  This permit is being issued pursuant to Section 403.087, Florida Statutes.  Additional details are documented in the Technical Evaluation attached to the REVISED DRAFT permit.

Any party to this order (permit) has the right to seek judicial review of the permit pursuant to Section 120.68, Florida Statutes, by the filing of a Notice of Appeal pursuant to Rule 9.110, Florida Rules of Appellate Procedure, with the Clerk of the EPC in the Legal Department at 3629 Queen Palm Drive, Tampa, FL  33619; and by filing a copy of the Notice of Appeal accompanied by the applicable filing fees with the appropriate District Court of Appeal.  The Notice of Appeal must be filed within 30 days from the date this Notice is filed with the clerk of the EPC.

Executed in Tampa, Florida.

Sincerely,

Richard D. Garrity, Ph.D.

Executive Director

cc:  	FDEP Southwest District (via e-mail)

James Burkholder, Florida Rock Industries, Inc.

Florida Rock Industries, Inc.          						                Page 2

Tampa, FL 33605

CERTIFICATE OF SERVICE

This is to certify that this NOTICE OF PERMIT and all copies were mailed before the close of business on _________________________ to the listed persons.

Clerk Stamp

FILED, on this date, pursuant to Section 120.52(7), Florida Statutes, with the designated clerk, receipt of which is hereby acknowledged.

__________________________   _____________

Clerk                     	    Date

| PERMITTEE: | PERMIT/CERTIFICATION |
| --- | --- |
| Florida Rock Industries, Inc. 2001 Maritime Blvd. Tampa, FL 33605 | Permit No.: 0570018-019-AC County: Hillsborough Expiration Date: February 24, 2013 Project: Minor Modification (Multiple EU’s) |

This permit is issued under the provisions of Chapter 403, Florida Statutes, and Florida Administrative Code Rules 62-204, 62-210, 62-212, 62-296, 62-297, and 62-4.  The above named permittee is hereby authorized to perform the work or operate the facility shown on the application and approved drawing(s), plans, and other documents, attached hereto or on file with the EPC and made a part hereof and specifically described as follows:

The facility (formerly known as LaFarge) is a bulk cement and aggregate material handling and packaging facility currently comprised of 42 different emission units.  The plant receives various cementitious and aggregate products from ships or trucks for processing and shipment off-site by truck or ship.  The raw products are dried in a dryer, ground, and blended in finish mills, as needed.  The materials are then further blended to the desired cementitious mixture in the blending plant, if needed.  The final product can be bulk loaded into trucks or ships for shipment off-site, or packaged into various sized bags for shipment via truck or ship.  This Air Construction permit authorizes the following modifications:

- During the operation and investigation of the blending plant process (EU 072 and 073), Florida Rock found that certain cementitious products that are blended and loaded into the jumbo bag holding bin segregate due to the length of travel to the bin. This results in an inconsistent sand content when loading jumbo bags yielding a sub-standard product.

Permit Nos. 0570018-017-AC/018-AV authorized Florida Rock to construct and operate one jumbo bag filling station consisting of two (2) jumbo bag filling spouts for loadout of up to two 3-ton jumbo bags simultaneously at an approximate loadout rate of 60 ton/hr. The blending plant currently has a single spout jumbo bag filling station that is fed from the jumbo bag holding bin.  In order to prevent material segregation, Florida Rock proposed to relocate the second jumbo bag loading spout from the jumbo bag filling station to the end of the existing take-away screw conveyor that is fed by the mixer.

The take away screw conveyor will be extended approximately 5 feet and a flexible pipe will connect the screw conveyor to the second loading spout for loading jumbo bags.  The loading spouts will be vented to Baghouse 73 (EU 073) as originally stated in the aforementioned permits.  Since the jumbo bag filling is an enclosed operation, there is no increase in emissions as the permit currently accounts for two jumbo bag loading spouts and the loadout rate of 60 tons/hr will remain the same.

2.  Florida Rock requested the capability to alternatively control Old Bulk Silo #24 using Baghouse 8 (EU 008) to avoid product contamination.  Old Bulk Silo #24 is currently controlled by Baghouse 6 (EU 006).  There is currently an existing line interconnecting Silos 23 and 24.  Florida Rock would like to activate this interconnection in order to alternatively control emissions from Silo 24 using Baghouse 8 to prevent product contamination.  Both EU 006 and 008 are equipped with 6,250 ACFM baghouses, which will each control 3 bulk silos under the proposed alternative scenario.  The potential emissions for EU 006 and 008 were based on the baghouse air flow rate, gr/dscf limit, and 8,760 hours of operation, so there is no increase in potential emissions as a result of this change.

3.  Currently, bulk coarse/granular materials can be loaded out to trucks from stockpiles for movement around the yard or for delivery off-site.  Front-end loaders retrieving material from the piles can load directly to trucks or to a hopper feeding a movable conveyor which drops the material into trucks.  This emission unit is identified as EU 083.

Florida Rock requested the flexibility to load bulk coarse/granular materials into ships via EU 083.  Under the proposed modification, bulk coarse/granular material will be retrieved from stockpiles via front-end loader and loaded into a hopper which feeds a movable conveyor which will load the material into a ship or barge.  This process will be executed in the same manner permitted for loading a truck via a movable conveyor. Florida Rock did not request an increase in throughput, but would like any material loaded into a ship/barge to count against the existing throughput limit for EU 083.  According to Florida Rock, the proposed process will remain similar to the originally permitted operation and there should not be an increase in emissions.  All reasonable precautions will be utilized to maintain compliance with the visible emission standards (5% for material transfer and 10% for shiphold opening) along with initial and annual visible emissions testing.  The process and emission unit descriptions have been revised to reflect this modification.

4.  Florida Rock is currently permitted to bring in white cement ships equipped with self-unloading systems for loadout into the packhouse silos and old bulk silos.  It was authorized in Permits Nos. 0570018-017-AC/018-AV for Packhouse Silo 7 to be alternatively controlled by Baghouse 31 (EU 031) to avoid product contamination.  Baghouse 31 is a 12,000 ACFM baghouse, and primarily controls the automated packers.  Florida Rock requested in the minor modification letter that Silos 13 and 14 may also be alternatively controlled by Baghouse 31 to avoid product contamination.  Silos 13 and 14 will be used in conjunction with Silo 7 to store white cement for packing.

The potential to emit for EU 031 was based on the baghouse air flow rate, the gr/dscf limit, and 8,760 hours of operation.  There is no increase in potential emissions for alternatively controlling Silos 13 and 14 with Baghouse 31, therefore, the change has been made as requested.

Process Description

This facility is a cement and aggregate processing, handling, and packaging operation that receives various cementitious products from ships or trucks for processing and shipment off-site.  The facility handles two distinct types of products: 1) Bulk Fine Materials; and 2) Bulk Coarse/Granular Materials.  Fine materials (e.g. cement, flyash, milled limestone, etc.) are handled pneumatically, stored in enclosed silos, and controlled by baghouses, with the exception of transfer of material on enclosed Belt Conveyor #7 during the ship unloading transfer to Silos 27 and 28.  Coarse/Granular materials (e.g. gypsum (natural and synthetic), slag (blast furnace, white, black, other), limestone, sand, pumice, emathlite, kitty litter, bauxite, etc.) are typically handled with conveyors and elevators, stored in open stockpiles, and controlled though moisture content, enclosures on conveyor belts, tarp enclosures, and water sprays or dust suppressants, as necessary.  The operations at the facility have been grouped into sections based on the physical location of the various distinct operations around the site.  Where possible, the units are described based on the process flow of the materials as follows: 1) Receiving; 2) Transfer; 3) Processing; 4) Storage; and 5) Shipping. Some emission units are mentioned in multiple groups because their operations carryover to several processes.

Bulk Coarse/Granular Material Handling/Processing – (Emission Unit (EU) 053, 054, 058, 059, 074, 075, 076, 077, 078, 083)

Bulk coarse/granular materials are received from ships or trucks.  Coarse/Granular materials are unloaded from ships (EU 053) using self unloading equipment or clamshells.  The material is currently loaded into two hoppers (EU 054) that empty onto dedicated covered conveyor belts that direct the product onto a single covered conveyor (Conveyor #6) or directly to a loadout chute.  A third hopper is also scheduled to be constructed, which will operate similar to the existing two.  A loadout chute is also located on the southern end of Conveyor #6.  Material can be delivered through the loadout chutes directly to trucks or a mobile stacker conveyor hopper, followed by a series of up to three moveable stick conveyors if necessary, so a stockpile can be formed near the ship unloading area.  The trucks can deliver the products to the desired stockpile on-site or take the material off-site.  Alternatively, the material on Conveyor #6 is transferred to an enclosed conveyor (Conveyor #7), which ascends to an enclosed tower where the material is transferred to a covered conveyor (Conveyor #8) through a diverter.  A baghouse controls emissions at this transfer point; however, it is typically only utilized during bulk fine unloading.  Conveyor #8 empties onto a series of up to three moveable stick conveyors, which empty onto an automatic radial stacker for formation of the storage piles on the north side of the site.  The fixed portion of the radial stacker is covered.  All of the transfer of material from the dockside hoppers via conveyors or truck to the stacker conveyors is identified as EU 059 and estimated potential emissions were based on 8 transfer points.

Trucks arriving with coarse/granular materials from off-site can drop the products directly to storage piles located around the yard or into hoppers or grates feeding the various processes.  The drop of material to a pile from conveyor or truck is identified as EU 077.  The material received from the ships and trucks is typically moist and water sprays are required as needed to reduce PM emissions.  Bulk coarse/granular materials can also be loaded out to trucks for movement around the yard or for off-site delivery.  Furthermore, bulk coarse/granular materials can be loaded out to ship or barge for off-site delivery.  Front-end loaders retrieving material from the storage piles can load directly to trucks or to a hopper feeding a movable conveyor which drops the material into trucks or ships.  Emissions from this loadout are identified as EU 083.  Particulate matter emissions are controlled using reasonable precautions such as partial enclosures and water or dust suppressant as needed.

The bulk coarse/granular materials can be processed through a natural gas-fired parallel-flow rotary drum dryer at a maximum rate of 150 ton/hr to reduce the moisture content prior to further processing and/or blending.  Material sent to the dryer is retrieved from the storage piles and dropped into the dryer feed hopper via front-end loader, and then transferred via open conveyors into the dryer.  The handling of material prior to the dryer is identified as EU 074.  Material exiting the dryer is passed though an enclosed drag conveyor and then up an elevator where it feeds either the bulk coarse/granular silo (EU 075) or Finish Mill Feed Silos 9C, 10A and 10B.  Emissions from the dryer and drag conveyor are controlled by Baghouse 58 (EU 058).  Collected dust from this baghouse is returned to the enclosed drag conveyor.  Material from the bulk coarse/granular silo is bulk loaded into trucks (EU 076).  The bulk coarse/granular silo is primarily used to store dried sand and its emissions are controlled through partial enclosures.

Bulk coarse/granular materials can also be processed through a crusher/screener to recover the desired sizes for further processing.  Bulk coarse/granular materials can be reclaimed from the storage piles via front-end loader and placed into a hopper.  From the hopper, the material is fed via conveyor into a crusher.  From the crusher, the material is transferred via conveyor and dropped into a triple-deck screen to separate the product by size at an estimated maximum process rate of 50 ton/hr.  Three conveyors will radiate out from the various levels of the screen to convey the product to form three distinct piles based on the screen size.  Water sprays are required throughout this operation to ensure that that product retains sufficient moisture to minimize particulate matter emissions.  This operation is identified as EU 078.

Bulk Fine Material Receiving and Silos 27 & 28 – (EU 027, 042, 043, 044, 045, 053, 060, 061, 066)

Bulk fine materials are received from ships or trucks.  This facility receives bulk fine materials from two types of ships - bulk carrier vessels with open holds and self unloading vessels with onboard pumps and closed holds (EU 053).  The bulk fine materials can be self-unloaded into the packhouse or bulk silos, or can be removed from ships by vacuum via two cyclone separators with dedicated unloading arms onto the air slide/conveyor system.  Emissions from each unloading arm/separator are controlled with dedicated baghouses, designated as Baghouse 44 and Baghouse 45 (EU 044 and EU 045).  The material travels down independent air slides from each separator and is delivered onto Conveyor #7 in an enclosed area controlled by Baghouse 27 (EU 027).  The material ascends to an enclosed tower where the bulk fine material is transferred via diverter to an air slide.  Baghouse 66 (EU 066) controls this transfer and is located on top of the transfer house.  The air slide discharges into a screen that feeds a bucket elevator that empties onto another air slide leading to Silos 27 and 28.  There are also four truck unloading lines at ground level of Silos 27 and 28 that allow trucks to pump material directly into the base of the bucket elevator that leads to the silos.  Emissions generated during the transfer of the product from Conveyor #7 air slide to the screen, the truck unloading lines to the base of the elevator, the bucket elevator, and the transfer of material from the elevator to the next air slide are controlled by Baghouse 42 (EU 042).  The elevator has a maximum process rate of 1,000 ton/hr.

The air slide from the bucket elevator delivers the product to a diverter leading to two air slides.  One air slide empties the material into Silo 27.  The other air slide leads to a diverter that routes the material to either Silo 28 or to an air slide leading directly to the Finish Mill Feed Silos.  Material in Silo 27 and Silo 28 can be loaded directly into trucks, or transferred to the finish mills.  The product from the silos is transferred by dedicated air slides from each silo to the Silo Transfer Elevator which feeds the final air slide leading to the Finish Mill Feed Silos.  In addition, there are four lines that run from the ground directly into the top of the two silos that allow up to four trucks to pneumatically unload simultaneously to either Silo 27 or 28, or a combination of the two.  Emissions from the transfers through these various air slides, from the loading of Silos 27 and 28, and from the Silo Transfer Elevator are controlled by Baghouse 43 (EU 043).

Beneath Silos 27 and 28 are individual truck loadout stations which allow for bulk loadout of fine material to trucks from the silos through a telescopic chute.  Also located beneath Silo 28 (north silo) is a jumbo bag filling station consisting of 4 jumbo bag filling spouts.  Up to 4 jumbo bags can be filled simultaneously.  Emissions from Silo 27 truck loading is controlled by Baghouse 60 (EU 060).  Emissions from Silo 28 truck loading and the jumbo bag filling station is controlled by Baghouse 61 (EU 061).  Both baghouses are located within the base of the corresponding silo.

Finish Mills – (EU 012, 013, 016, 017, 018, 019, 020, 021, 080, 081)

In the finish mill building, bulk coarse/granular and fine materials are initially stored, processed through the grinding mills and/or separators to be sized as needed, and then pumped to storage silos for eventual blending and/or loadout.  The Finish Mill Feed Silos are comprised of 12 silos that can be loaded from the air slide connected to Silos 27 and 28, directly from trucks (selective silos), from a bucket elevator (Silos 9C, 10A, and 10B), or from the exit of the finish mills (reclaimed).  Several of the Finish Mill Feed Silos can receive fine materials directly from trucks.  Silos 7A, 7B, 8A, 8C, and KL have single truck unloading line, and Silos 9A and 9B have two truck unloading lines.  Emissions from Silos 7A, 7B, 7C, 8A, 8B, 8C, 9A, 9B, and KL are all controlled by Baghouse 12 (EU 012).  A bucket elevator is located on the east side of the silos that is fed directly from the dryer or from a short conveyor beneath a truck unloading grate.  The elevator can feed finish mill Silos 9C, 10A and 10B.  Emissions from Silos 10A and 10B are controlled by Baghouse 19 (EU 019).  Silo 9C is not controlled by a baghouse, and its emissions are controlled through partial enclosures and material moisture content.  Silo 9C is identified as EU 081.

Products from the Finish Mill Feed Silos are transferred via drag conveyors to the No. 9 finish mill, the No. 10 finish mill, the No. 8 mixing separator, or directly to an outgoing surge pump via the No. 7 transfer line.  For Finish Mill 9, the #9 drag conveyor transfers materials from the feed silos to the Finish Mill #9 direct feed elevator.  The elevator feeds a drag conveyor which unloads the material into Finish Mill 9.  Material exiting the mill is transferred through the #9 main floor screw conveyor, then up the #9 main elevator, and then into the two #9 separators.  Rejects from the separators are fed back into the finish mill.  The desired products from the separators are fed into the #4 surge bin.  From the #4 surge bin, material is fed to either Pump A or Pump C for pneumatic transport to the packhouse silos, the bulk silos, or back to the finish mill feed silos.  Emissions from the drag conveyors, the direct feed elevator, the base of the main elevator, the surge bin, and the Pumps A and C are controlled by Baghouse 17 (EU 017).  Emissions from Finish Mill #9 and the Finish Mill #9 to screw conveyor transfer are controlled with a cyclone in series with Baghouse 18 (EU 018).  Collected material from the cyclone and Baghouse 18, along with reclaimed product from the vacuum truck cleaning system, is dropped into the floor screw conveyor.  Emissions from the #9 main elevator and the two #9 separators are controlled by Baghouse 16 (EU 016).  Collected material from Baghouse 16 is transferred via screw conveyor to the #4 surge bin.

For Finish Mill 10, the handling of material is very similar to Finish Mill 9.  Material is received and processed through Finish Mill 10 and then directed to two separators using similar transfer equipment to Finish Mill 9.  Desired products from the separators are fed into the #3 surge bin.  From the #3 surge bin, material is fed to either Pump A, Pump B, or Pump C for transport.  Emissions from the #10 drag conveyors, the #10 direct feed elevator, and the base of the #10 main elevator are controlled by Baghouse 20 (EU 020).  Emissions from the finish mill and finish mill to screw conveyor transfer are controlled by Baghouse 21 (EU 021).  Collected material from Baghouse 21 is dropped into the floor screw conveyor.  Emissions from the #10 main elevator, the two #10 separators, and the #3 surge bin are controlled by Baghouse 19 (EU 019).  Collected material from Baghouse 19 is transferred via air slide to the #3 surge bin.  Pump B is controlled by a small standalone baghouse identified as Baghouse 80 (EU 080).

For Line #8, the #8 drag conveyor transfers materials from the feed silos to the #8 main elevator.  The elevator delivers material to the #8 separator which serves as a mixer (no materials are separated or removed).  The desired products from the separator are fed into the #2 surge bin via screw conveyor.  Line #7 is similar to Line #8, but there is no separator.  Line #7 is only used to transfer material from the feed silos to the #2 surge bin.  However, the facility has the flexibility to divert product flow from Line #7 to Line #8 through use of an air slide from the #7 drag conveyor to the #8 main elevator.  From the #2 surge bin, material is fed to either Pump B or Pump C for pneumatic transport.  Emissions from the #7 and #8 drag conveyors, the main elevators, the #7 transfer air slide, and the screw conveyors from both lines, along with the #2 surge bin and #8 separator, are all controlled by Baghouse 13 (EU 013).

Bulk Silos – (EU 006, 007, 008)

The Bulk Silos are comprised of 8 silos (6 exterior silos and 2 interstitial silos).  Material is loaded into the silos from the finish mills, or directly from ships or trucks (selective silos).  Material from ships can be transferred directly to any of the Bulk Silos.  A single truck unloading line can only feed Silos 19, 20, or 21 at one time.  Beneath the three silos on the east side (Silos 19-21) is the east bulk truck loading spout.  Trucks can be loaded by a telescopic spout that extends down to the truck opening.  The emissions are vented to Silo 21.  Beneath the three silos on the west side (Silos 22-24) is the west bulk truck loading station with a telescopic loading spout, similar to the east side.  Beneath Silo 24 are four jumbo bag filling stations.  These stations can fill up to 3-ton jumbo bags from Silo 24 for shipment off-site.  An additional two jumbo bag filling stations are located beneath Silo 22.  Emissions from the 4 jumbo bag filling stations (beneath Silo 24), the east truck loading spout, and Silos 19, 20, 21, and 24 are controlled by Baghouse 6 (EU 006).  Emissions from the 2 jumbo bag filling stations (beneath Silo 22), the west truck loading spout, and Silos 22 and 23 are controlled by Baghouse 8 (EU 008).  As an alternative method of operation, emissions from Silo 24 may be controlled by Baghouse 8 to prevent product contamination.  Emissions from the 2 interstitial silos, Silos 25 and 26, are controlled by Baghouse 7 (EU 007).  Silo 19 can also be controlled by Baghouse 7 (EU 007) as an alternative operation if necessary in order to help prevent product contamination.

Packhouse Silos/ Blending Plant/Packing Operations – (EU 001, 002, 003, 005, 031, 032, 071)

Bulk fine materials are stored in the Packhouse Silos and transferred to either the blending plant or a series of packers for bagging of material for shipment off-site.  The Packhouse Silos are comprised of 18 silos.  Material is loaded into the silos from the finish mills, or directly from trucks or ships.  Material from ships can be transferred directly to any of the Packhouse Silos.  The Packhouse Silos have four distinct truck unloading operations.  Silos 11, 12, 17, and 18 can be loaded by a single truck line.  Silos 1, 2, 3 can be loaded by a single truck line, and Silos 7 and 10 each have single individual truck unloading lines.  Packhouse Silos 1, 2, and 3 are controlled by Baghouse 2 (EU 002).  Packhouse Silos 4, 5, and 6 are controlled by Baghouse 1 (EU 001).  Packhouse Silos 7-10 and 13-16 are controlled by Baghouse 3 (EU 003).  Silo 17 can also be controlled by Baghouse 3 as an alternative operation if necessary in order to help prevent product contamination.  Packhouse Silos 11, 12, 17 and 18 are controlled by Baghouse 5 (EU 005).  Silos 7, 13, and 14 can also be controlled by Baghouse 31 as an alternative mode of operation, if necessary, to prevent product contamination.

Packhouse Silos 2-6, 11, 12, and 14-18 can feed the rotary packing system on the north end of the building.  Material from these silos are either fed directly to the primary bucket elevator or through a secondary elevator followed by a screen that feeds the primary elevator.  From the primary elevator, the material drops through a screen and into the holding bin above the packer.  The rotary packer was manufactured by Haver Filling Systems, Inc. and can load different sizes of bags up to approximately 100 pounds at various rates, with a maximum estimated loading rate of 141 ton/hr and 4,000 bags/hr.  Packaged materials are discharged onto belt conveyors that transport the bags to the storage warehouse.  A quality control system on the finished bag conveyor examines each bag and shreds off-spec bags.  There is also a reclaim hopper beneath the packer to collect spilled material.  Reclaimed material from packing and damaged bags is collected through a series of screw conveyors and sent to the secondary elevator.  The secondary elevator can discharge back to the primary elevator or to a series of isolation bins that temporarily store material to prevent product contamination.  Emissions from all of the handling operations associated with the rotary packer are controlled by Baghouse 71 (EU 071).

Packhouse Silos 7-10 and 13-16 can feed the automated packers, the manual packer, and the Haver inline packer.  Packhouse Silos 1-6 can feed the manual packer and the Haver inline packer, but not the automated packers in order to prevent product contamination.  For the automated packers, material is transferred by air slides or screw conveyors to an elevator, and then through a rotary screen which discharges into one of three chutes.  One chute leads directly to an automated packer bin, the second chute is into a screw conveyor to the second packer bin, and the third chute leads to the #6 screw conveyor which can feed any of the packer bins (manual, automated and Haver inline).  The material from each bin is fed through a rotary air lock to the packers.  Emissions from the air slides, elevator, screw conveyors, screen, storage bins, and packers are controlled by Baghouse 31 (EU 031).  For the manual and Haver inline packers, material is fed directly to the packers from the packhouse silos via air slide to an elevator, screen, and then storage bin above the packers.  The packers can also receive material processed through the blending plant.

Packaged materials from all 4 packers are discharged onto belt conveyors that transport the bags to the storage warehouse.  Beneath the 4 packers is an open reclaim screw conveyor to collect loose material from the bagging operation.  Emissions from the bag handling system and the material reclaim system are controlled by Baghouse 32 (EU 032).

Blending Plant – (EU 072, 073, 082)

The blending plant is located east of the 4 packers.  All materials enter the blending plant from the packhouse silos via air slides, screw conveyors and an elevator that direct the material to the blending plant bucket elevator or directly to the bulk jumbo bag holding bin.  Material can also be pneumatically unloaded from trucks through 3 unloading lines directly to the 4 elevated storage bins or the jumbo bag filling holding bin.  Coarse/Granular materials can also be bulk loaded into the blending plant elevator by truck through a small hopper connected with a short screw conveyor.  The loadout from truck to the blending plant hopper is identified as EU 082.  The material exiting the blending plant elevator goes through a diverter gate and can be directed to any of the 4 storage bins, to the manual and Haver inline packer hopper for bagging, or to the bulk jumbo bag holding bin.  From the jumbo bag holding bin, the blended product is sent to a jumbo bag filling station consisting of one jumbo bag filling spout.  A second jumbo bag filling spout is located at the end of the blending plant product (take away) screw conveyor.  Each jumbo bag filling spout can load one 3-ton jumbo bag at a time at an approximate loadout rate of 60 ton/hr.  Materials stored in the 4 storage bins flow downwards (by gravity) to the weighing bins (for proportioning), to the mixer, and last to the take away screw conveyor.  The mixer is a Model 2250 SKAKO blender where dry product is mixed at a maximum rate of approximately 45 ton/hr.  The take away screw conveyor returns the blended materials back to the blending plant elevator or diverts the material to the pneumatic pump for transfer to bulk or packhouse storage silos.  Emissions from the air slides, screw conveyors, elevators, 4 storage bins, and blender are controlled by Baghouse 72 (EU 072).  Emissions from the manual packer, Haver inline packer, jumbo bag filling stations, and associated holding bins are controlled by Baghouse 73 (EU 073).

Bulk Fine Ship Loading – (EU 079)

Bulk fine materials can also be loaded to ships through the following two distinct scenarios, or any combination of these scenarios simultaneously not to exceed 420 ton/hr.  In the first scenario, material being transferred to the finish mill feed silos from Silos 27 and 28 will be diverted from the existing transfer air slide to a new air slide that will discharge into a chute leading directly to the existing surge bins and adjoining pumps in the finish mill building.  The new construction will provide for a more direct path to the surge bins and bypass unnecessary handling through the finish mill feed silos.  The transfer of material from the new air slide leading to the surge bins/pumps will be controlled by the following 3 existing baghouses in the finish mill: Baghouse 13 (EU 013), Baghouse 17 (EU 017), and Baghouse 20 (EU 020).  The combined capacity of the 3 pumps is approximately 210 ton/hr (70 ton/hr each).  From the pumps, the material will then be pneumatically transferred through the three respective lines that currently lead to the packhouse silos near the dock.  However, new connections to each of the three pneumatic lines will be installed prior to the material arriving at the packhouse building.  The new lines will allow for the products to be diverted directly to the shiphold for loadout.  Particulate matter emissions from the ship loading will be controlled by up to two of the existing dockside baghouses: Baghouse 45 and/or Baghouse 27.  A flexible pipe can be added from the South cyclone separator (EU 045) during ship loading of bulk fine materials in order to allow collected dust from the separator/baghouse to be routed back to the ship rather than the air slides to help prevent product contamination.

In the second scenario, a new 12-line truck loadout bay will be constructed near the dock.  Each line is expected to be 5” in diameter.  Trucks will arrive to the loadout bay from off-site, from the two truck loadout stations beneath Silos 27 and 28, or from other existing cement truck loadout stations located at the facility.  All trucks will contain their own power sources for the pump (i.e. self unloading), and have a typical individual unloading rate of 35 ton/hr.  The 12 lines will each lead to the ship for loadout, so up to 12 trucks could unload to the shiphold at one time (i.e. maximum ship loading capacity is 420 ton/hr).  Particulate matter emissions from the ship loading will be controlled by up to three of the existing dockside baghouses: Baghouse 44 and/or Baghouse 45 and/or Baghouse 27.  In order to help assure that an individual baghouse is not overloaded during ship loading, Baghouses 44 or 45 cannot solely control emissions from the ship.  As an alternative option for the second scenario, particulate matter emissions from ship loading may be controlled by a single portable dust collector with a minimum airflow of 10,500 acfm and a maximum airflow of 39,000 acfm.  The make and model number of the dust collector may change as long as the minimum specifications and testing requirements in the permit are met.  Flexible pipes can be added from the North and South cyclone separators (EU 044-45) during ship loading of bulk fine materials in order to allow collected dust from the separator/baghouse to be routed back to the ship rather than the air slides to help prevent product contamination.

Also present at the facility are several insignificant sources/activities.  Included as part of the insignificant sources is a collection of 4 steel storage tanks approximately 15,000 gallons each located south of the finish mills.  The tanks contain a variety of additives and grinding aids utilized as needed in the finish mills.

Based on the Title V Air Operation Permit Construction/Renewal application received April 20, 2010, this facility is not a major source of hazardous air pollutants (HAPs).  The facility is not subject to Rule 62-212.400, F.A.C. – Prevention of Significant Deterioration (PSD) since the facility-wide emissions are below the applicability threshold.  Multiple emission units are subject to Compliance Assurance Monitoring (CAM) requirements.  The facility is also subject to PM RACT (Rule 62-296.700, F.A.C.)

Location:  2001 Maritime Blvd., Tampa

UTM:  17-357.9E  3090.7N    FACILITY ID NO: 0570018

| EU ID No. | Brief Description |
| --- | --- |
| Subsection A | Bulk Coarse/Granular Material Handling/Processing |
| 083 | Bulk Coarse/Granular Material Shipping - Loadout to Truck/Ship |
| Subsection D | Bulk Silos |
| 006 | Bulk Fine Storage – Bulk Silos 19, 20, 21, and 24 |
| 008 | Bulk Fine Storage – Bulk Silos 22, 23 |
| Subsection E | Packhouse Silos/Blending Plant/Packing Operations |
| 003 | Bulk Fine Storage – Packhouse Silos 7, 8, 9, 10, 13, 14, 15, 16 |
| 031 | Packhouse - Automated Packers |
| 073 | Blending Plant/Packhouse - Manual Packer, Haver Inline Packer, Jumbo Baggers |

Replaces Permit Nos.: 	N/A

References Permit Nos.:   	0570018-005-AC

0570018-006-AC

0570018-010-AV

0570018-014-AC

0570018-017-AC/018-AV

Facility-wide Conditions:

The following conditions apply facility-wide:

1.  A part of this permit is the attached General Conditions. [Rule 62-4.160, F.A.C.]

2. All applicable rules of the Environmental Protection Commission of Hillsborough County including design discharge limitations specified in the application shall be adhered to.  The permit holder may also need to comply with county, municipal, federal, or other state regulations prior to construction. [Rule 62-4.070(7), F.A.C.]

3.  Issuance of this permit does not relieve the permittee from complying with applicable emission limiting standards or other requirements of Chapters 62-204, 62-210, 62-212, 62-296 and 62-297, F.A.C., or any other requirements under federal, state, or local law. [Rule 62-210.300, F.A.C.]

4.  The permittee shall not cause, suffer, allow, or permit the discharge of air pollutants which cause or contribute to an objectionable odor. [Rule 62-296.320, F.A.C.]

5.  General Particulate Emission Limiting Standards.  General Visible Emissions Standard.  Except for emissions units that are subject to a particulate matter or opacity limit set forth or established by rule and reflected by conditions in this permit, no person shall cause, let, permit, suffer or allow to be discharged into the atmosphere the emissions of air pollutants from any activity, the density of which is equal to or greater than that designated as Number 1 on the Ringelmann Chart (20 percent opacity).  EPA Method 9 is the method of compliance pursuant to Chapter 62-297, F.A.C.

[Rules 62-296.320(4)(b)1. & 4., F.A.C.]

6.  Prevention of Accidental Releases (Section 112(r) of CAA).

- The permittee shall submit its Risk Management Plan (RMP) to the Chemical Emergency Preparedness and Prevention Office (CEPPO) RMP Reporting Center when, and if, such requirement becomes applicable.  Any Risk Management Plans, original submittals, revisions or updates to submittals, should be sent to:

RMP Reporting Center

Post Office Box 10162

Fairfax, VA  22038

Telephone:  (703) 227-7650

and,

- The permittee shall submit to the permitting authority Title V certification forms or a compliance schedule in accordance with Rule 62-213.440(2), F.A.C.

[40 CFR 68]

7.  [Reserved.]

8.  Emissions of Unconfined Particulate Matter.  Pursuant to Rules 62-296.320(4)(c)1., 3. & 4., F.A.C., reasonable precautions to prevent emissions of unconfined particulate matter at this facility include the following requirements:

- Limiting vehicle movement in the area of unpaved roads and storage piles;

- Providing adequate moisture to the storage piles while loading and unloading;

- Providing adequate moisture to storage piles to control windblown particulate matter during storage;

- Providing a dust suppressant, as necessary, to the unpaved road;

- Posting of speed limit sign to control the speed of vehicles on the roads to 10 mph or less;

- Exercising good housekeeping practices at all times, including the removal of material spillage promptly;

- Making facility personnel aware of the environmental requirements in this permit.

[Rules 62-4.070(3) and 62-296.320(4)(c)2., F.A.C.]

9.  When appropriate, any recording, monitoring, or reporting requirements that are time-specific shall be in accordance with the effective date of the permit, which defines day one.  The annual testing requirement is defined as calendar year (January – December).

[Rule 62-213.440, F.A.C.]

10.  Statement of Compliance.  The annual statement of compliance pursuant to Rule 62-213.440(3)(a)2., F.A.C., shall be submitted to the Department and EPA within 60 (sixty) days after the end of the calendar year using DEP Form No. 62-213.900(7), F.A.C.

[Rules 62-213.440(3) and 62-213.900, F.A.C.]

11.  The permittee shall provide timely notification to the Environmental Protection Commission of Hillsborough County prior to implementing any changes that may result in a modification to this permit pursuant to Rule 62-210.200, F.A.C., Modification.  The changes may include, and are not limited to the following, and may also require prior authorization before implementation: [Rules 62-210.300, 62-4.060, and 62-4.070(3), F.A.C.]

- Alteration or replacement of any equipment or major component of such equipment listed in the process description of this permit.

- Installation or addition of any equipment which is a source of air pollution.

- The storage or handling of any products other than those authorized by this permit.

12.  If the permittee wishes to transfer this permit to another owner, an "Application for Transfer of Permit" (DEP Form 62-210.900(7)) shall be submitted, in duplicate, to the Environmental Protection Commission of Hillsborough County within 30 days after the sale or legal transfer of the permitted facility. [Rule 62-4.120, F.A.C.]

13.  The permittee shall operate and maintain the baghouse cleaning systems (including maintaining the air cleaning pressure where applicable) and a device to measure, within ten percent accuracy, the gas pressure differential across each baghouse.  Particulate matter emissions generated during material handling and processing operations must be vented to and controlled by the corresponding baghouse, where applicable.

[Rules 62-213.440(1)(b) and 62-210.650, F.A.C.; Permit No. 0570018-018-AV]

14.  [Reserved.]

15.  When the Environmental Protection Commission of Hillsborough County, after investigation, has good reason (such as complaints, increased visible emissions or questionable maintenance of control equipment) to believe that any applicable emission standard contained in a Department rule or in a permit issued pursuant to those rules is being violated, it may require the permittee to conduct compliance tests which identify the nature and quantity of pollutant emissions from the emissions unit and to provide a report on the results of said tests to the Environmental Protection Commission of Hillsborough County.  Stack sampling facilities must meet the requirements of Appendix SS-1, Stack Sampling Facilities.

[Rules 62-297.310(6) and (7)(b), F.A.C.]

16.    The permittee must submit to the Environmental Protection Commission of Hillsborough County each calendar year, a completed DEP Form 62-210.900(5), "Annual Operating Report (AOR) for Air Pollutant Emitting Facility", for the preceding calendar year.  The AOR shall be submitted by April 1 of the following year.

[Rule 62-210.370(3), F.A.C.]

17.  The permittee shall notify the Air Compliance Section of the Environmental Protection Commission of Hillsborough County at least 15 days prior to the date on which each formal compliance test is to begin of the date, time, and place of each such test, and the contact person who will be responsible for coordinating and having such test conducted.

[Rule 62-297.310(7)(a)9., F.A.C.]

18.  The permittee shall file a report with the Environmental Protection Commission of Hillsborough County on the results of each compliance test as soon as practical but no later than 45 days after the last sampling run of each test is completed.  The test report shall provide sufficient detail on the emissions unit tested and the test procedures used to allow the Environmental Protection Commission of Hillsborough County to determine if the test was properly conducted.

[Rule 62-297.310(8), F.A.C.]

19.  The permittee shall retain records of all monitoring data and support information for a period of at least 5 years from the date of the monitoring sample, measurement, report, or application.  Support information includes all calibration and maintenance records and all original strip-chart recordings for continuous monitoring instrumentation, and copies of all reports required by the permit.  [Rule 62-213.440(1)(b)2.b., F.A.C.]

20.  Unless otherwise specified in this permit, the unit “ton” shall be defined as a short ton, equivalent to 2,000 pounds (avdp).

[Rule 62-4.070, F.A.C.]

21.  In order to limit the potential to emit for the facility and remain below the applicability threshold for Rule 62-212.400, F.A.C. – Prevention of Significant Deterioration (PSD), the maximum allowable particulate matter (PM) emissions for the facility described in this permit shall not exceed 244.0 tons per twelve consecutive month period.

[Rules 62-4.070(3) and 62-210.200 – “Potential to Emit”, F.A.C., and Construction Permit Application received April 20, 2010]

22.  Annual Emissions Fee Form and Fee.  The annual Title V emissions fees are due (postmarked) by March 1st of each year.  The completed form and calculated fee shall be submitted to:  Major Air Pollution Source Annual Emissions Fee, P.O. Box 3070, Tallahassee, Florida  32315-3070.  The forms are available for download by accessing the Title V Annual Emissions Fee On-line Information Center at the following Internet web site:  http://www.dep.state.fl.us/air/emission/tvfee.htm.  [Rule 62-213.205, F.A.C.]

23.  Relaxations of Restrictions on Pollutant Emitting Capacity.  At such time that a particular source or modification becomes a major stationary source or major modification (as these terms were defined at the time the source obtained the enforceable limitation) solely by virtue of a relaxation in any enforceable limitation which was established after August 7, 1980, on the capacity of the source or modification otherwise to emit a pollutant, such as a restriction on hours of operation, then the requirements of subsections 62-212.400(4) through (12), F.A.C., shall apply to the source or modification as though construction had not yet commenced on the source or modification.

[Rule 62-212.400(12)(b), F.A.C.]

24.  The use of property, facilities, equipment, processes, products, or compounds, or the commission of paint overspraying or any other act, that causes or materially contributes to a public nuisance is prohibited.

[Hillsborough County Environmental Protection Act, Section 16, Chapter 84-446, Laws of Florida, as Amended.]

25.  The permittee shall submit all compliance related notifications and reports, data, certifications and requests required of this permit to the Environmental Protection Commission of Hillsborough County at:

Environmental Protection Commission of Hillsborough County

#### Air Management Division

3629 Queen Palm Drive

Tampa, FL  33619

Phone: (813) 627-2600

Fax: (813) 627-2660

26.  Any reports, data, notifications, certifications and requests required to be sent to the United States Environmental Protection Agency Region 4 should be sent to:

United States Environmental Protection Agency

Region 4

#### Air, Pesticides and Toxics Management Division

Air and EPCRA Enforcement Branch

Air Enforcement Section

61 Forsyth Street

Atlanta, Georgia  30303-8960

Telephone:  404/562-9155; Fax:  404/562-9163

27.  Certification by Responsible Official (RO).  In addition to the professional engineering certification required for applications by Rule 62-4.050(3), F.A.C., any application form, report, compliance statement, compliance plan and compliance schedule submitted pursuant to Chapter 62-213, F.A.C., shall contain a certification signed by a responsible official that, based on information and belief formed after reasonable inquiry, the statements and information in the document are true, accurate, and complete.  Any responsible official who fails to submit any required information or who has submitted incorrect information shall, upon becoming aware of such failure or incorrect submittal, promptly submit such supplementary information or correct information.  [Rule 62-213.420(4), F.A.C.]

28.  Prior to the Title V Renewal permit application due date (225 days prior to Title V permit expiration date), the permittee shall conduct particulate matter compliance tests on one finish mill, one bulk fine material ship unloading operation, and the bulk coarse/granular material dryer.

[Rule 62-297.310(7)(a)3. F.A.C.]

29.  The permittee shall submit a report of all monitoring required at least every six months. All instances of deviations from permit requirements, including those attributable to upset conditions, must be clearly identified in such reports. The reports shall be in accordance with the requirements of Rule 62-210.700(6), F.A.C. for excess emissions resulting from malfunctions and Rule 62-4.130, F.A.C. for plant operations – problems. The reports shall include the probable cause of such deviations, any corrective actions or preventive measures taken, and shall be accompanied by a certification by a responsible official.

[Rule 62-213.440(1)(b)3, F.A.C.]

30.  The permittee shall provide at least the minimum requirements for stack sampling facilities as specified in Rule 62-297, F.A.C.  Sources sampling platforms, platform access, and other associated work areas, whether permanent or temporary, shall be in accordance with Occupational Safety and Health Administration standards per 29 CFR 1910, Subparts D and E.

31.  A minimum of two copies of an operating permit application for a Title V Operating Permit Revision shall be submitted to the Environmental Protection Commission of Hillsborough County within 90 days of completion of the initial compliance testing of the emission units being modified by this permit but no later than 90 days prior to the expiration date of this permit.  A copy of the compliance tests shall be submitted with the permit application. [Rules 62-213.420(1)(a)4. and 62-210.300(2), F.A.C.]

### Subsection A - Bulk Coarse/Granular Material Handling/Processing

### This section addresses the following emissions unit(s).

| EU ID No. | Brief Description |
| --- | --- |
| 083 | Bulk Coarse/Granular Material Shipping - Loadout to Truck/Ship |

{Permitting note(s): This emission unit is regulated under: PM-RACT (Rule 62-296.711, F.A.C.); and Chapter 1-3.52 - Rules of the Environmental Protection Commission of Hillsborough County.}

The following specific conditions apply to the emissions unit(s) listed above:

A.1.  Any bulk coarse/granular material may be handled, as long as the material is handled in accordance with the permit and in compliance with the visible emission standards specified in Specific Condition No. A.6.  [Rule 62-4.070(3), F.A.C.]

Enforceable Potential to Emit (PTE) Parameters

A.2.  [RESERVED]

A.3. In order to limit the potential to emit and remain below the applicability threshold for Rule 62-212.400, F.A.C. – Prevention of Significant Deterioration (PSD), the following operating limits shall not be exceeded for the loadout of bulk coarse/granular material to ships:

| EU | Bulk Coarse/Granular Material Throughput (tons/year)1 | Maximum Handling Rate (ton/hr) | Minimum Moisture Content (% by wt.) |
| --- | --- | --- | --- |
| 083 | 820,0002 | 400 (truck or ship) | 4.8% |

1tons per consecutive twelve month period

2throughput limit is the combined limit for truck loading and ship loading operations

[Rule 62-210.200 – “Potential to Emit”, F.A.C.; Construction Permit No. 0570018-014-AC and 017-AC]

A.4.  In order to limit the potential to emit and remain below the applicability threshold for Rule 62-212.400, F.A.C. – Prevention of Significant Deterioration (PSD), the following operating limits shall not be exceeded for the bulk coarse/granular material handling operations:

| EU | Bulk Coarse/Granular Material Throughput (tons/year)1 | Maximum Handling Rate (ton/hr) |
| --- | --- | --- |
| 083 | 820,000 2 | 400 (truck or ship) |

1- tons per consecutive twelve month period

2- throughput limit is the combined limit for truck loading and ship loading operations

[Rule 62-210.200 – “Potential to Emit”, F.A.C.; Construction Permit No. 0570018-014-AC; and Construction Permit Application received April 20, 2010]

Emission Limitations and Standards

A.5.  [RESERVED]

A.6.  The permittee shall not cause, permit, or allow any visible emissions (five percent opacity) from EU 083 except when loading a ship with material from a conveyor system.  An opacity standard of ten percent shall apply when the shiphold hatch covering has been moved. [Rule 62-296.711(2), F.A.C.; Section 1-3.52 Rules of the Environmental Protection Commission of Hillsborough County; Construction Permit No. 0570018-014-AC and 017-AC]

A.7.  [RESERVED]

A.8.  In order to ensure compliance with Specific Condition Nos. A.3, A.4, and Facility-wide Condition 21, the following reasonable precautions to prevent emissions of unconfined particulate matter at this facility shall apply:

- Apply water via water sprays to the bulk coarse/granular materials as needed whenever it is handled prior to the dryer to ensure compliance with the opacity limit.

- Enclose all conveyor belts on the top and sides.

- When material is discharged into a receiving hopper from a clamshell, the free fall distance shall not exceed 4 ft. and the clamshell shall not be opened until it is below the sides of the hopper.

- Water sprays, or similar chemical coating applicators, shall be used as necessary on the bulk coarse/granular materials at the receiving hoppers, transfer points, storage piles and plant grounds to ensure sufficient dust suppression to meet opacity standards while material is transferred or stored.

- Material spillage or leakage from the clamshell shall be kept at a minimum.  The clamshell shall be maintained so that it has a tight lip.

- The clamshell shall be kept clean at all times, and accumulation of material in the clamshell is prohibited.

- Use of front-end loaders and bulldozers in the marine vessel hold shall be minimized to the greatest extent possible.

- No blowers shall be used in the marine vessel holds to gather material for the final clean out of the cargo hold.

- Tarps or similar coverings shall be used as needed to minimize particulate emissions from the shiphold and other uncontrolled emission points.

- Vacuum or wet areas used by vehicles and machines as needed to minimize emissions.

- Curtailing or ceasing operations when necessary, especially in situations with high winds, in order to comply with the opacity standard.

- Minimize the drop height for all conveyor drops to piles.

[Rules 62-296.320(4)(c) and 62-4.070(3), F.A.C.; Permit No. 0570018-010-AV Construction Permit No. 0570018-014-AC; and Construction Permit Application received April 20, 2010]

Test Methods and Procedures

A.9.  Prior to the beginning of each ship loading or unloading operation of any bulk coarse/granular material, test each outgoing shipment or incoming shipload of bulk coarse/granular material, respectively, for moisture content.  The permittee shall also test any slag fed to the dryer for moisture content on any date that the dryer is processing slag.

[Rule 62-4.070(3), F.A.C.; Permit Nos. 0570018-006-AC, 014-AC, and 017-AC]

A.10. Test the following emission unit for visible emissions as follows:

EU 083 – Bulk Coarse/Granular Material Shipping - Loadout to Truck/Ship

- Truck Loadout - Test the loadout of bulk coarse/granular materials to trucks from front-end loaders at the point of highest opacity annually during each federal fiscal year.

- Ship – Test the loadout of bulk coarse/granular materials to ship at the point of highest opacity during the initial shiphold loading operation after completion of the modification and annually during each federal fiscal year.  The Method 9 test shall include separate observations of the shiphold and the material transfer points.

Submit two copies of the test data to the Air Compliance Section of the Air Management Division of the Environmental Protection Commission of Hillsborough County (EPCHC) within 45 days of testing.  The visible emissions should be observed from the highest point of opacity during the period in which the highest opacity emissions can reasonably be expected.   [Rules 62-297.310 and 62-4.070(3), F.A.C.]

A.11. The test method for moisture content shall be ASTM Method D-2216 or an equivalent method.  The test method for visible emissions shall be shall be EPA Method 9, incorporated and adopted by reference in Rule 62-297, F.A.C.  The test method for particulate matter shall be EPA Methods 1, 2, 4, and 5, incorporated and adopted by reference in Rule 62-297, F.A.C.  The minimum EPA Method 5 sample volume shall be 30 dry standard cubic feet.  The permittee shall record and report the gas pressure differential across the baghouse during each compliance test.

[Rules 62-4.070(3), 62-296.711 and 62-213.440(1)(b), F.A.C.; Section 1-3.52 Rules of the EPC; and Permit No. 0570018-006-AC]

A.12. Testing of emissions shall be conducted with the sources operating at capacity.  Capacity is defined as 90-100% of the material handling rates in Specific Condition Nos. A.3. and A.4.  If it is impracticable to test at capacity, then the source may be tested at less than capacity; in this case subsequent source operation is limited to 110% of the test load until a new test is conducted.  Once the unit is so limited, then operation at higher capacities is allowed for no more than fifteen days for purposes of additional compliance testing to regain the rated capacity in the permit, with prior notification to the Environmental Protection Commission of Hillsborough County.  Failure to submit the input rates and actual operating conditions, including pressure differential across the baghouse, may invalidate the test.

[0570018-006-AC and Rule 62-297.310(2), F.A.C.]

A.13. The required minimum period of observation for an EPA Method 9 compliance test shall be thirty (30) minutes.

[Rule 62-297.310(4)(a), F.A.C.]

A.14.  Test emission unit EU 083 for visible emissions upon the first instance of handling a coarse/granular material, already identified in the permit, that has not previously been tested for visible emissions at the facility.  Each VE observation shall be made at the point of highest opacity from each handling operation and should be in accordance with Specific Condition Nos. A.11, A.12 and A.13.  Submit two copies of the test data to the Air Management Division of the Environmental Protection Commission of Hillsborough County within 45 days of such testing.  Testing procedures shall be consistent with the requirements of Rule 62-297.310, F.A.C. [Rules 62-297.310(7)(a)4., 62-297.310(8)(b), and 62-4.070(3), F.A.C.]

A.15. [RESERVED]

Monitoring, Recordkeeping and Reporting Requirements

A.16. [RESERVED]

A.17.  The permittee shall maintain records to demonstrate compliance with the requirements of Specific Condition Nos. A.3, A.4., and A.6.  The records shall include, as a minimum, the following information and shall be made available for inspection by the Environmental Protection Commission of Hillsborough County for at least five years:

- Daily (if operating)

- Day, Month, Year

- Amount and type of bulk coarse/granular materials loaded into and unloaded from ships (tons/day)

- The number of moveable stick conveyors in operation during ship unloading

- Monthly

- Month, Year

- Summary of bulk coarse/granular material moisture content for each material shipment, and slag moisture content at the beginning of slag drying, as required by Specific Condition No. A.9. [% weight]

- Amount and type of bulk coarse/granular material loaded into and unloaded from ships during each month [tons] and the most recent consecutive twelve month period [tons/12 months]

- Amount and type of bulk coarse/granular material loaded into and unloaded from trucks during each month [tons] and the most recent consecutive twelve month period [tons/12 months]

[Rules 62-213.440(1)(b), 62-4.070(3) and 62-4.160(14), F.A.C.; Permit No . 0570018-005-AC, 014-AC, and 017-AC

Subsection D - Bulk Silos

### This section addresses the following emissions unit(s).

| EU ID No. | Brief Description |
| --- | --- |
| 006 | Bulk Fine Storage – Bulk Silos 19, 20, 21, and 24 |
| 008 | Bulk Fine Storage – Bulk Silos 22, 23 |

{Permitting note(s): These emissions units are regulated under: CAM – 40 CFR 64, Compliance Assurance Monitoring, adopted and incorporated by reference in Rule 62-204.800, F.A.C.; PM-RACT (Rule 62-296.711, F.A.C.); and Chapter 1-3.52 - Rules of the Environmental Protection Commission of Hillsborough County.}

The following specific conditions apply to the emissions unit(s) listed above:

Enforceable Potential to Emit (PTE) Parameters

D.1.  In order to limit the potential to emit and remain below the applicability threshold for Rule 62-212.400, F.A.C. – Prevention of Significant Deterioration (PSD), the following operating limits and equipment ratings shall not be exceeded:

| EU | Hours of Operation (hours/12 months)1 | Material Handling/Loading Rate (tons/hour) 2 | Baghouse ID No. | Baghouse Design Flowrate (acfm) |
| --- | --- | --- | --- | --- |
| 006 | 8,760 | 150 (by transfer) 35 (by truck) 500 (by ship) | 6 | 6,250 |
| 008 | 8,760 | 150 (by transfer) 500 (by ship) | 8 | 6,250 |

1hours per consecutive twelve month period

2daily average

[Rule 62-210.200 – “Potential to Emit”, F.A.C.; Construction Permit No. AC29-249252; and Permit No. 0570018-017-AC]

Emission Limitations and Standards

D.2.  In order to limit the potential to emit for these emission units and remain below the applicability threshold for Rule 62-212.400, F.A.C. – Prevention of Significant Deterioration (PSD), the following maximum allowable particulate matter (PM) emissions and opacity emissions standards shall not be exceeded:

|  | Emissions Standard |  |  |
| --- | --- | --- | --- |
| EU | (grains/dscf) | ton/year1 | Opacity |
| 006 | 0.02 | 4.7 | 5% |
| 008 | 0.02 | 4.7 | 5% |

1tons per consecutive twelve month period

[Rules 62-210.200 – “Potential to Emit” and 62-296.711(2), F.A.C.; Section 1-3.52 Rules of the Environmental Protection Commission of Hillsborough County; and Construction Permit Application received April 20, 2010]

D.3.  The following work practice standards apply:

- Particulate emissions generated during bulk fine material handling must be vented through the corresponding baghouse.  As an alternative method of operation, emissions from Silo 24 may be vented to Baghouse 8 (EU 008) to prevent product contamination.

- Any bulk fine material may be handled, as long as the material is handled in accordance with the permit and in compliance with the visible emission standards listed in Specific Condition No. D.2.

- No more than 1 truck can unload to Silos 19, 20 or 21 at one time.

- The jumbo bag filling stations shall be maintained in proper order to function as designed.  Ensure proper operation of dust control system prior to filling any jumbo bags.

Test Methods and Procedures

D.4.  Test EU 008 upon initial operation for visible emissions under the modified configuration (while controlling Bulk Silos 22, 23, and 24).  Test all the emission units in this section (EU 006-008) for visible emissions annually during each federal fiscal year (October 1 – September 30).  Testing shall include: 1) the exhaust of each baghouse while corresponding silos are being loaded; 2) each truck loading spout at loadout point to trucks; and 3) each jumbo bagging operation at filling points to bags.

Submit two copies of the test data to the Air Compliance Section of the Air Management Division of the Environmental Protection Commission of Hillsborough County (EPCHC) within 45 days of testing.  The test observations shall be made from the highest point of opacity during the period in which the highest opacity emissions can reasonably be expected.

[Rules 62-297.310 and Rule 62-4.070(3), F.A.C.]

D.5.  The test method for visible emissions shall be shall be EPA Method 9, incorporated and adopted by reference in Rule 62-297, F.A.C.  The test method for particulate matter shall be EPA Methods 1, 2, 4, and 5, incorporated and adopted by reference in Rule 62-297, F.A.C.  The minimum EPA Method 5 sample volume shall be 30 dry standard cubic feet.  The permittee shall record and report the gas pressure differential across each baghouse during each required compliance test.

[Rules 62-4.070(3), 62-296.711 and 62-213.440(1)(b), F.A.C.; Section 1-3.52 Rules of the EPC; and Permit No. 0570018-005-AC]

D.6. Testing of emissions shall be conducted with the source operating at capacity.  Capacity is defined as 90-100% of the material handling rates in Specific Condition No. D.1. If it is impracticable to test at capacity, then the source may be tested at less than capacity; in this case subsequent source operation is limited to 110% of the test load until a new test is conducted.  Once the unit is so limited, then operation at higher capacities is allowed for no more than fifteen days for purposes of additional compliance testing to regain the rated capacity in the permit, with prior notification to the Environmental Protection Commission of Hillsborough County.  Failure to submit the input rates and actual operating conditions, including the pressure differential across the baghouse, may invalidate the test.

[Rule 62-297.310(2), F.A.C.]

D.7.  The required minimum period of observation for an EPA Method 9 compliance test shall be thirty (30) minutes.

[Rule 62-297.310(4)(a), F.A.C.]

D.8.  A visible emissions test indicating no visible emissions (5 percent opacity) may be submitted in lieu of a particulate stack test for the bulk silo operations since they are equipped with baghouses.  Because of the expense and complexity of conducting a stack test on minor sources of particulate matter, the Environmental Protection Commission of Hillsborough County pursuant to the authority granted under Rule 62-297.620(4), F.A.C. hereby allows the particulate testing to be waived in lieu of a visible emission standard not to exceed an opacity of 5%.  If the EPC has reason to believe that the particulate weight emission standard is not being met, it shall require that compliance be demonstrated by Method 5 testing.

[Rules 62-296.711, 62-297.620, and 62-297.310(7)(b), F.A.C.]

Continuous Monitoring Requirements

D.9. These emissions units are subject to the CAM requirements contained in the attached combined Operation and Maintenance (O&M) Plan/Compliance Assurance Monitoring (CAM) Plan.  Failure to adhere to the monitoring requirements specified in the CAM portion of the appendix does not necessarily indicate an exceedance of a specific emissions limitation; however, it may constitute good reason to require compliance testing pursuant to Rule 62-297.310(7)(b), F.A.C.

[40 CFR 64; and Rules 62-204.800 and 62-213.440(1)(b)1.a., F.A.C.]

Monitoring, Recordkeeping and Reporting Requirements

D.10. The permittee is subject to the Operation and Maintenance Plan requirements for particulate matter for all the emission units in this section controlled by a baghouse as specified in the attached combined Operation and Maintenance (O&M) Plan/Compliance Assurance Monitoring (CAM) Plan.

[Rules 62-296.700 and 62-213.440(1)(b), F.A.C.; Construction Permit Application received April 20, 2010]

D.11.  The permittee shall maintain records to demonstrate compliance with the requirements of specific conditions D.1., D.2., and D.10.  The records shall include, at a minimum, the following information and shall be made available for inspection by the Environmental Protection Commission of Hillsborough County for at least five years:

- Daily

- Day, Month, Year

- Pressure drop [inches H2O] across each baghouse as required by the Operation and Maintenance Plan

- Result of each instantaneous visual emissions determination on each emissions unit with a baghouse as required by the Operation and Maintenance Plan

(b) Monthly

- Month, Year

- The amount of bulk fine materials handled by the Bulk Silos during the month [tons] and the most recent consecutive twelve month period [tons/12 months]

- The amount of bulk fine materials loaded into jumbo bags during the month [tons] and the most recent consecutive twelve month period [tons/12 months]

- The amount of bulk fine materials loaded into trucks during the month [tons] and the most recent consecutive twelve month period [tons/12 months]

[Rule 62-4.070(3), 62-4.160(14), and 62-213.440(1)(b), F.A.C.; and Construction Permit Application received April 20, 2010]

Subsection E - Packhouse Silos/Blending Plant/Packing Operations

### This section addresses the following emissions unit(s).

| EU ID No. | Brief Description |
| --- | --- |
| 003 | Bulk Fine Storage – Packhouse Silos 7, 8, 9, 10, 13, 14, 15, 16 |
| 031 | Packhouse - Automated Packers |
| 073 | Blending Plant/Packhouse - Manual Packer, Haver Inline Packer, Jumbo Baggers |

{Permitting note(s): These emissions units are regulated under: CAM – 40 CFR 64, Compliance Assurance Monitoring, adopted and incorporated by reference in Rule 62-204.800, F.A.C. (except EU 082); PM-RACT (Rule 62-296.711, F.A.C.); and Chapter 1-3.52 - Rules of the Environmental Protection Commission of Hillsborough County.}

The following specific conditions apply to the emissions unit(s) listed above:

Enforceable Potential to Emit (PTE) Parameters

E.1.  In order to limit the potential to emit and remain below the applicability threshold for Rule 62-212.400, F.A.C. – Prevention of Significant Deterioration (PSD), the following operating limits and equipment ratings shall not be exceeded:

| EU | Hours of Operation (hours/12 months)1 | Material Handling/Loading Rate (tons/hour) 2 | Baghouse ID No. | Baghouse Design Flowrate (acfm) |
| --- | --- | --- | --- | --- |
| 003 | 8,760 | 150 (transfer, truck, or ship) | 3 | 9,260 |
| 031 | 8,760 | 150 | 31 | 12,000 |
| 073 | 8,760 | 210 | 73 | 8,000 |

1hours per consecutive twelve month period

2daily average

[Rule 62-210.200 – “Potential to Emit”, F.A.C.; Construction Permit No. 0570018-014-AC and 017-AC]

E.2.  [RESERVED]

Emission Limitations and Standards

E.3.  In order to limit the potential to emit for these emission units and remain below the applicability threshold for Rule 62-212.400, F.A.C. – Prevention of Significant Deterioration (PSD), the following maximum allowable particulate matter (PM) emissions and opacity emissions standards shall not be exceeded:

|  | Emissions Standard |  |  |
| --- | --- | --- | --- |
| EU | (grains/dscf) | ton/year1 | Opacity |
| 003 | 0.02 | 7.0 | 5% |
| 031 | 0.02 | 9.0 | 5% |
| 073 | 0.015 | 4.5 | 5% |

1tons per consecutive twelve month period

[Rules 62-210.200 – “Potential to Emit” and 62-296.711(2), F.A.C.; Section 1-3.52 Rules of the Environmental Protection Commission of Hillsborough County; Construction Permit No. 0570018-014-AC and 017-AC]

E.4.  In order to ensure compliance with Specific Condition Nos. E.1 and E.3, the following reasonable precautions to prevent emissions of unconfined particulate matter at this facility shall apply:

- All bulk fine material handling and packaging operations shall be vented to the designated control device (baghouse) as designed.

- The control devices shall be in operation at all times when any material is being processed through the related handling/packaging systems.

- The manual and Haver inline packing operations may operate simultaneously with the jumbo bagging system on the blending plant, as long as the combined process rate does not exceed 210 ton/hr.

- Any bulk fine material may be handled, as long as the material is handled in accordance with the permit and in compliance with the visible emission standards listed in Specific Condition No. E.3.

- Water sprays, or similar chemical coating applicators, shall be used as necessary on the bulk coarse/granular materials loaded into the blending plant hopper to ensure sufficient dust suppression to meet opacity standards while material is transferred or stored.

- The permittee shall maintain the integrity of the enclosures around the hopper, conveyors and bucket elevator handling bulk materials delivered to the blending plant.

- The jumbo bag filling stations shall be maintained in proper order to function as designed.  Ensure proper operation of dust control system prior to filling any jumbo bags.

- No more than 3 trucks can unload simultaneously to the elevated storage bins on the blending plant at one time.

[Rules 62-296.320(4)(c) and 62-4.070(3), F.A.C.; and Air Construction Permit No. 0570018-017-AC]

Test Methods and Procedures

E.5.  Test the jumbo bag filling stations and the exhaust of Baghouse 073 for visible emissions upon initial operation under the modified configuration (with the second jumbo bag loading spout connected to the take away screw conveyor).  If practicable, the manual and Haver packers shall be in operation at capacity during the test.  In addition, test all the emission units in this subsection for visible emissions annually during each federal fiscal year (October 1 – September 30).

Furthermore, test the exhaust of Baghouse 31 upon initial operation under the alternative configuration (while Packhouse Silos 7, 13, and 14 are being controlled by Baghouse 31). If practicable, the automated packers shall be in operation at capacity during the visible emissions test.

Submit two copies of the test data to the Air Compliance Section of the Air Management Division of the Environmental Protection Commission of Hillsborough County (EPCHC) within 45 days of testing of each emission unit.  The visible emissions should be observed at the highest point of opacity during the period in which the highest opacity emissions can reasonably be expected to occur.   [Rules 62-297.310 F.A.C., and Rule 62-4.070(3), F.A.C.]

E.6.  [RESERVED]

E.7.  The test method for visible emissions shall be shall be EPA Method 9, incorporated and adopted by reference in Rule 62-297, F.A.C.  The test method for particulate matter shall be EPA Methods 1, 2, 4, and 5, incorporated and adopted by reference in Rule 62-297, F.A.C.  The minimum EPA Method 5 sample volume shall be 30 dry standard cubic feet.  The permittee shall record and report the gas pressure differential across each baghouse during each required compliance test.

[Rules 62-4.070(3), 62-296.711 and 62-213.440(1)(b), F.A.C.; and Section 1-3.52 Rules of the EPC]

E.8.  Testing of emissions shall be conducted with the sources operating at capacity.  Capacity is defined as 90-100% of the material handling rates in Specific Condition No. E.1.  For the blending plant bagging/packing (EU 073) operation, the manual packer, Haver inline packer and jumbo bag filling stations should be in operation simultaneously, including normal material transfer to the holding bins, during the test.  If it is impracticable to test at capacity, then the source may be tested at less than capacity; in this case subsequent source operation is limited to 110% of the test load until a new test is conducted.  Once the unit is so limited, then operation at higher capacities is allowed for no more than fifteen days for purposes of additional compliance testing to regain the rated capacity in the permit, with prior notification to the Environmental Protection Commission of Hillsborough County.  Failure to submit the input rates and actual operating conditions, including the pressure differential across the baghouse, may invalidate the test.

[Rule 62-297.310(2), F.A.C.]

E.9.  The required minimum period of observation for an EPA Method 9 compliance test shall be thirty (30) minutes.

[Rule 62-297.310(4)(a), F.A.C.]

E.10.  Except for the PM test required prior to renewal, a visible emissions test indicating no visible emissions (5 percent opacity) may be submitted in lieu of a particulate stack test for the remaining emission units, since they are equipped with a baghouse.  Because of the expense and complexity of conducting a stack test on minor sources of particulate matter, the Environmental Protection Commission of Hillsborough County pursuant to the authority granted under Rule 62-297.620(4), F.A.C. hereby allows the particulate testing to be waived in lieu of a visible emission standard not to exceed an opacity of 5%.  If the EPC has reason to believe that the particulate weight emission standard is not being met, it shall require that compliance be demonstrated by Method 5 testing.

[Rules 62-296.711 and 62-297.310(7)(b), F.A.C.]

Continuous Monitoring Requirements

E.11.  These emissions units are subject to the CAM requirements contained in the attached combined Operation and Maintenance (O&M) Plan/Compliance Assurance Monitoring (CAM) Plan.  Failure to adhere to the monitoring requirements specified in the CAM portion of the appendix does not necessarily indicate an exceedance of a specific emissions limitation; however, it may constitute good reason to require compliance testing pursuant to Rule 62-297.310(7)(b), F.A.C.

[40 CFR 64; and Rules 62-204.800 and 62-213.440(1)(b)1.a., F.A.C.]

Monitoring, Recordkeeping and Reporting Requirements

E.12. The permittee is subject to the Operation and Maintenance Plan requirements for particulate matter for all the emission units in this section controlled by a baghouse as specified in the attached combined Operation and Maintenance (O&M) Plan/Compliance Assurance Monitoring (CAM) Plan.

[Rules 62-296.700 and 62-213.440(1)(b), F.A.C.; Construction Permit No. 0570018-006-AC; and Construction Permit Application received April 20, 2010]

E.13.  The permittee shall maintain records to demonstrate compliance with the requirements of Specific Condition Nos. E.1., E.3., and E.12.  The records shall include, as a minimum, the following information and shall be made available for inspection by the Environmental Protection Commission of Hillsborough County for at least five years:

- Daily (if operating)

- Day, Month, Year

- Pressure drop [inches H2O] across each baghouse as required by the current Operation and Maintenance Plan

- Result of each instantaneous visual emissions determination on each emissions unit as required by the current Operation and Maintenance Plan

(b) Monthly

- Month, Year

- The amount of blended bulk fine materials packaged through the blending plant during the month [tons] and the most recent consecutive twelve month period [tons/12 months]

- The amount of bulk fine materials packaged through the automatic packers during the month [tons] and the most recent consecutive twelve month period [tons/12 months]

- The amount of bulk fine materials handled by the Packhouse Silos during the month [tons] and the most recent consecutive twelve month period [tons/12 months]

- Amount and type of bulk coarse/granular material loaded into the blending plant hopper during each month [tons] and the most recent consecutive twelve month period [tons/12 months]

[Rules 62-213.440(1)(b), 62-4.070(3) and 62-4.160(14), F.A.C.]

ENVIRONMENTAL PROTECTION COMMISSION

OF HILLSBOROUGH COUNTY

_________________________________

Richard D. Garrity, Ph.D.

Executive Director