FINAL DETERMINATION

FOR

Mosaic Fertilizer, LLC – Big Bend Terminal

Hillsborough County

Construction Permit

Application Number

0570094-011-AC

Environmental Protection Commission of

Hillsborough County

Tampa, FL

February 16, 2012

FINAL DETERMINATION

The Environmental Protection Commission of Hillsborough County mailed a public notice package on January 26, 2012 that included Intent to Issue Air Construction Permit No. 0570094-011-AC to Mosaic Fertilizer, LLC-Big Bend Terminal Facility located at the 12839 Wyandotte Road, Gibsonton, Hillsborough County, FL 33534.  The air construction permit is to (1) modify the current AC Permit No. 0570094-009-AC to allow the facility to use the existing wet phosphate rock conveyor between Transfer Tower A (EU 110) and the stacker reclaimer and the existing wet phosphate rock conveyor between the stacker reclaimer and Transfer Point No. 2 (EU 002) to convey ammoniated phosphate (AP) products, and (2) extend the expiration date of the AC Permit No. 0570094-009-AC from November 30, 2011 to November 30, 2014.

Permit No. 0570094-009-AC authorized the following construction/modifications at the facility; (1) modification of the existing Wet Phosphate Rock Railcar Unloading Operation to handle AP products;  (2) modification of the existing Wet Phosphate Rock Transfer Tower A;  (3) construction of a new Truck Unloading Station to unload AP product;  (4) construction of a new 220,313 ton Phosphate Product Warehouse No. 2 Storage Building, with a reclaim and belt conveyor system;  (5) construction of a new Transfer Tower B to transfer AP product from the new Truck Unloading Station to the new Conveyor B, and the installation of a 1,450 DSCFM design flow rate cartridge filter to control PM emissions;  (6) construction of three new belt conveyors to transfer AP products from Transfer Tower A to the new Phosphate Product Warehouse No. 2 Storage Building and (7) modification of the Outgoing Transfer Point No. 3 to accommodate the new Conveyor B.  According to Mosaic, the facility has not yet constructed the units as authorized by the AC Permit due to the market conditions, however, the AC project is moving forward.

The Public Notice of Intent to Issue was published on February 1, 2012 in the Tampa Tribune.

No comments have ever received since the Intent was published.

The final action of the Environmental Protection Commission of Hillsborough County is to issue the final permit.

ENVIRONMENTAL PROTECTION COMMISSION OF

HILLSBOROUGH COUNTY, as Delegated by

STATE OF FLORIDA

DEPARTMENT OF ENVIRONMENTAL PROTECTION

NOTICE OF PERMIT

Jeffrey M. Stewart

Environmental Superintendent

Mosaic Fertilizer, LLC

13830 Circa Crossing Drive

Lithia, FL 33547

Re:  Hillsborough County - AP

Dear Mr. Stewart:

Enclosed is Construction Permit Number 0570094-011-AC, issued pursuant to Section 403.087, Florida Statutes, for the modification of the current Permit No. 0570094-009-AC to allow Mosaic Big Bend to move ammoniated phosphate (AP) products via the railcar rotary dump through Transfer Tower A, the stacker reclaimer, Transfer Point No. 2, Transfer Point No. 3, and the shipping terminal gantry, and finally into ships/barges.

Any party to this order (permit) has the right to seek judicial review of the permit pursuant to Section 120.68, Florida Statutes, by the filing of a Notice of Appeal pursuant to Rule 9.110, Florida Rules of Appellate Procedure, with the Clerk of the EPC in the Legal Department at 3629 Queen Palm Dr, Tampa, FL  33619; and by filing a copy of the Notice of Appeal accompanied by the applicable filing fees with the appropriate District Court of Appeal.  The Notice of Appeal must be filed within 30 days from the date this Notice is filed with the clerk of the EPC.

Executed in Tampa, Florida.

Sincerely,

Richard D. Garrity, Ph.D.

Executive Director

RDG/KRZ/krz

cc:  Rama Iyer, P.E. - Mosaic Fertilizer, LLC (e-mail)

Mosaic Big Bend Terminal										        Page Two

Lithia, FL 33547

CERTIFICATE OF SERVICE

This is to certify that this NOTICE OF PERMIT and all copies were mailed before the close of business on _________________________ to the listed persons.

Clerk Stamp

FILED, on this date, pursuant to Section 120.52(7), Florida Statutes, with the designated clerk, receipt of which is hereby acknowledged.

___________________________   ____________									Clerk                     			Date

PERMITTEE:                         				PERMIT/CERTIFICATION

Jeffrey M. Stewart					Permit No:  0570094-011-AC

Environmental Superintendent   			County:  Hillsborough

Mosaic Fertilizer, LLC 				Expiration Date:  November 30, 2014

13830 Circa Crossing Drive		      		Project:  Minor Modification

Lithia, FL 33547

This permit is issued under the provisions of Chapter 403, Florida Statutes, and Florida Administrative Code Rules 62-204, 62-210, 62-212, 62-296, 62-297, and 62-4.  The above named permittee is hereby authorized to perform the work or operate the facility shown on the application and approved drawing(s), plans and other documents, attached hereto or on file with the EPC and made a part of hereof and specifically described as follows:

The permit authorizes the modification of Permit No. 0570094-009-AC to allow Mosaic Big Bend Terminal to move ammoniated phosphate (AP) products via the railcar rotary dump through Transfer Tower A, the stacker reclaimer, Transfer Point No. 2, Transfer Point No. 3, and the shipping terminal gantry, and finally into ships/barges.

The facility is currently permitted (0570094-010-AO) for the operation of a barge loading facility handling phosphate materials.  The plant currently receives up to 2,233,800 ton/yr of GTSP, MAP, DAP, or wet phosphate rock and transports it for delivery to ships.  Particulate matter emissions are generated from the handling of the materials through various batch drops and conveyor transfers.  GTSP, MAP, DAP, and AFI are unloaded at the Truck and Railcar Unloading Station (EU No. 103) at a maximum rate of 300 tph.  The facility has the capability of adding a dust suppressant at the underground receiving hopper as material is conveyed up Belt #8.  The material is transferred from Belt #8 to Belt #9, which is part of EU No. 103, and then from Belt #9 to Belt #10 (EU No. 001 - Incoming Transfer Point No. 1) at a maximum rate of 300 tph.  PM emissions from EU No. 001 are controlled by a 4,153 DSCFM Mikro-Pulsaire Model No. 81S-8-20 Baghouse (Baghouse #1).  From Belt #10, the material is transferred to Belt #11 (EU No. 104-Warehouse Transfer Point), and then into the Warehouse Storage Building (EU No. 100), which is controlled by enclosures.

Front-end loaders place the product on conveyor belts through one of four openings on the north and south sides of the Warehouse Storage Building (EU No. 100).  Inside the warehouse, the north belt (Belt #12) transfers to the east belt (Belt #13), which then transfers to the south belt (Belt #14).  The products are conveyed from Belt #14 to Belt #4 (EU No. 002 - Outgoing Transfer Point No. 2) at a maximum rate of 1,300 tph through a partially enclosed covering which is connected to a 2,538 DSCFM Mikro-Pulseaire Model No. 49S-8-20 Baghouse (Baghouse #2).  Material is then transferred from Belt #4 to Belt #5 (EU No. 003 - Outgoing Transfer Point No. 3) at a maximum rate of 2,000 tph, with PM emissions controlled by a 2,538 DSCFM Mikro-Pulsaire Model No. 49S-8-20 Baghouse (Baghouse #3).  The material is then transferred from Belt #5 to Belt #6 (EU No. 004 - Shipping Terminal Gantry-Transfer Point No. 4) at a maximum rate of 2,000 tph.  PM emissions from the Belt #5 to Belt # 6 Transfer point (EU No. 004) are controlled by a 18,456 DSCFM Mikro-Pulseaire Model No. 384K-8-20 Baghouse (Baghouse #4).  The material is then conveyed over the ship and dropped through a telescopic chute and into the ship or barge (EU No. 106 – Ship/Barge Hold Loading) at a maximum rate of 2,000 tph.  PM emissions from the Barge Hold (EU No. 106) are controlled by dust suppressants, the telescopic choke chute, tarps (as necessary), and a pick-up point that leads to Baghouse #4.

The facility is authorized to utilize the wet phosphate rock operation to handle ammoniated phosphate product, which includes DAP and MAP.  Wet phosphate rock and ammoniated phosphate product is unloaded via an existing Rotary Railcar Unloading Station (EU No. 109) at a rate of 2,000 tph, and transferred to a hopper to the existing Incoming Transfer Tower A (EU No. 110) via belt conveyor (Conveyor No. 1).  Transfer Tower A consists of the Conveyor No. 1 to Conveyor A Transfer Point.  PM emissions from the ammoniated phosphate product Railcar Unloading Station (EU No. 109) will be controlled by a 36,654 DSCFM JET-AIRE Model JA-378-CG Pulse-Jet Baghouse, while PM emissions from ammoniated phosphate product loading into Incoming Transfer Tower A (EU No. 110) will be controlled by a 1,450 DSCFM Dust Control and Loading Systems VMV Cartridge Filter Baghouse.  Ammoniated phosphate product will also be unloaded via truck from the Truck Unloading Station (EU No. 107) at a maximum rate of 500 tph through a below grade hopper to a feed belt and on to a new belt conveyor (Conveyor F) and new Transfer Tower B (EU No. 111).  PM emissions from the new Truck Unloading Station will be controlled by enclosure in a building, while PM emissions from Transfer Tower B (Conveyor F to Conveyor A Transfer Point) will be controlled by 1,450 DSCFM Dust Control and Loading Systems VMV Cartridge Filter Baghouse.  Ammoniated phosphate product will be transferred from Transfer Tower A to Transfer Tower B via belt conveyor (Conveyor A). From Transfer Tower B, ammoniated phosphate product is belt conveyed to a new 220,313 ton Phosphate Product Warehouse No. 2 Storage Building (EU No. 108), and then at a maximum rate of 2,000 tph to Outgoing Transfer Point No. 3 (EU No. 003) via Conveyor B before final loadout into ships and barges through the existing shipping terminal gantry and ship/barge loading (EU Nos. 004 and 106).  PM emissions from Phosphate Product Warehouse No. 2 Storage Building (EU No. 108) and Conveyor B are controlled by enclosure.

Wet phosphate rock or ammoniated phosphate product can also be conveyed through a belt conveyor between Transfer Tower A (EU No. 110) and the stacker reclaimer and then to storage piles (only for wet phosphate rock).  The wet phosphate rock from the storage piles, or ammoniated phosphate product from the stacker reclaimer is conveyed from Belt #14 to Belt #4 (EU No. 002-Outgoing Transfer Point No. 2), and then through Belt #4 to Belt #5 (EU No. 003-Outgoing Transfer Point No. 3).  Wet rock is then transferred from Belt #5 to Belt #6 (EU No. 004-Shipping Terminal Gantry-Transfer Point No. 4 for final loadout into a ship or barge (EU No. 106-Ship/Barge Hold Loading).

The facility will no longer handle GTSP, but will be allowed to handle up to 2,233,800 ton/yr of AP products and AFI (Animal Feed Ingredient, which is a form of granular calcium phosphate), as long as the moisture content is  ≥ 0.72 % and the silt  is ≤ 19%.  The facility will also be allowed to handle the permitted materials without operation of their respective baghouses or cartridge filters, as long as a dust suppressant is applied, and the opacity is < 5%.

Location: 12839 Wyandotte Road, Gibsonton, FL 33534

UTM:	17-361.0 E, 3076.2 N    			NEDS NO:   0570094

Emission Unit ID:	001 		Incoming Transfer Point #1

002 		Outgoing Transfer Point #2

003 		Outgoing Transfer Point #3

004 		Shipping Terminal Gantry (Transfer Point #4)

100		Warehouse Storage Building

103		Railcar and Truck Unloading Station for AP and AFI

104		Warehouse Transfer Point

106		Ship/Barge Hold Loading

107		AP Truck Unloading

108		Phosphate Product Warehouse No. 2 Storage Building

109		Wet Phosphate Rock /AP Railcar Unloading Station

110		Wet Phosphate Rock/AP Incoming Transfer Tower A

111		AP Incoming Transfer Tower B

References Permit No.:  	0570094-009-AC

1.	A part of this permit is the attached General Conditions. [Rule 62-4.160, F.A.C.]

2.	All applicable rules of the Environmental Protection Commission of Hillsborough County including design discharge limitations specified in the application shall be adhered to.  The permit holder may also need to comply with county, municipal, federal, or other state regulations prior to construction.

[Rule 62-4.070(7), F.A.C.]

3.	Issuance of this permit does not relieve the permittee from complying with applicable emission limiting standards or other requirements of Chapters 62-204, 62-210, 62-212, 62-296 and 62-297, F.A.C., or any other requirements under federal, state, or local law. [Rule 62-210.300, F.A.C.]

4.	The permittee shall not cause, suffer, allow, or permit the discharge of air pollutants which cause or contribute to an objectionable odor. [Rule 62-296.320, F.A.C.]

5.	The permittee shall not cause, permit, or allow visible emissions greater than or equal to the following opacity limits: [Rule 62-296.711(2)(a), F.A.C. and 1-3.52 Rules of the Environmental Protection Commission of Hillsborough County]

Source								Opacity Standard

- Railcar and Truck Unloading Station (Hoppers and 				5%

Belt #8 to Belt #9 Transfer Point)

- Incoming Transfer Point #1 (Belt #9 to Belt #10 Transfer Point)

Enclosure and Baghouse #1 Exhaust 					5%

- Warehouse Transfer Point (Belt #10 to Belt #11 Transfer Point) 		5%

- Warehouse Storage Building No. 1						5%

- Outgoing Transfer Point #2 (Belt #14 to Belt #4 Transfer Point) 		5%

Enclosure and Baghouse #2 Exhaust

- Outgoing Transfer Point #3 (Belt #4 to Belt #5 Transfer Point)		5%

Enclosure and Baghouse #3 Exhaust

- Shipping Terminal Gantry-Transfer Point #4 				5%

(Belt #5 to Belt #6 Transfer Point) and Baghouse #4 Exhaust

- Ship/Barge Hold Loading*							5/10%

- Wet Phosphate Rock/AP Railcar Unloading Station				5%

and Baghouse Exhaust

- AP Truck Unloading Station and Building Enclosure			5%

- Phosphate Product Warehouse No. 2 Storage Building			5%

- Wet Phosphate Rock/AP Incoming Transfer Tower A,			5%

stacker reclaimer, and Baghouse Exhaust

- Wet Phosphate Rock/AP Incoming Transfer Tower B			5%

and Baghouse Exhaust

* 10% opacity is allowed from the shiphold during loading only when the conveyor discharging material to the shiphold is moving.

- In order to limit the potential to emit and establish the facility as a Synthetic Non-Title V source, particulate matter emissions shall not exceed 83.6 tons per twelve consecutive month period, and the facility shall not exceed the limitations of Specific Condition Nos. 7 and 9.

[Rules 62-210.200 – “Potential to Emit” and 62-4.070(3), F.A.C.; and Air Construction Permit No. 0570094-009-AC]

7.	For each of the emission units listed in Specific Condition No. 5 that is equipped with a baghouse, the permittee shall not exceed the maximum allowable particulate matter emissions from each baghouse specified below:    [Rules 62-4.070(3) and 62-296.711, F.A.C.; and Permit No. 0570094-009-AC]

Source 				       (gr/dscf)	      (DSCFM)

Incoming Baghouse #1 Exhaust 		   0.03		 4,153

Outgoing Baghouse #2 Exhaust 		   0.03		 2,538

Outgoing Baghouse #3 Exhaust 		   0.03		 2,538

Shipping Terminal Gantry	    	          0.03 	        18,456

Baghouse #4 Exhaust

Wet Phosphate Rock/AP Railcar 	          0.03	        38,654

Unloading Baghouse

Wet Phosphate Rock/AP Incoming	          0.03	        1,450

Transfer Tower A Baghouse

Wet Phosphate Rock/AP Incoming	          0.03	        1,450

Transfer Tower B Baghouse

8.	[Reserved]

9.	In order to ensure compliance with Specific Condition Nos. 5, 6, and 7, the following restrictions shall apply: [Rule 62-4.070(3), F.A.C., and AC Permit No. 0570094-009-AC]

- The hours of operation for the emission units are not limited.

- Only wet phosphate rock, ammoniated phosphate products (AP) and Animal Feed Ingredient (AFI) may be transferred. Wet phosphate rock is defined as follows: “Material produced in a phosphate beneficiation plant by washing and flotation, both of which are wet processes.  Material produced in a fuel fired kiln or dryer, in which water is then applied, is not wet phosphate rock.”

- No more than 2,233,800 tons of AP and AFI combined shall be transferred through the facility in any twelve consecutive month period.  Records of the same shall be kept on-site and be made available upon request.

- Maximum transfer rate of Transfer Point #1 is 300 tph.

- Maximum transfer rate for Transfer Point #2 is 1,300 tph.

- Maximum transfer rate for Transfer Point #3 and the Shipping Terminal Gantry-Transfer Point #4 is 2,000 tph.

- Maximum transfer rate for the Wet Phosphate Rock/AP Unloading Station, Incoming Transfer Tower A and Phosphate Products Warehouse No. 2 Storage Building  is 2,000 tph.

- Maximum transfer rate for Transfer Point #2 is 1,300 tph.

- Maximum transfer rate for AP Truck Unloading and Incoming Transfer Tower B is 500 tph.

- The baghouses shall be maintained in good working order.

- The permittee is authorized to perform truck and railcar unloading operations simultaneously.

- All doors, vents, fans, etc. leading into or out of the warehouses during periods of material transfer must remain closed, except for the openings associated with the conveyor(s) in use at that time.

- All ship and barge loading shall utilize the telescopic chute.

- Oil application or dust suppressant application to the products is done at the processing plant prior to delivery of the product to the transfer facility, or the product must be sufficiently treated with non-hazardous oil or dust suppressant on site as needed to meet the opacity limit.

- A chemical coating shall be applied to all AFI on the outgoing conveyor from the warehouse (Belt #14).

- A chemical coating should be applied as necessary to AFI at the receiving hopper (Belt #8) to ensure sufficient dust suppression to meet opacity standards while material is transferred to the warehouse.

- A daily visible emission observation shall be performed and recorded on any dates that material is received or loaded to ships to provide reasonable assurance of continued compliance with the opacity standard.  The observations should be an instantaneous reading during active material transport using EPA Method 22 procedures.  At least one reading should be made on the incoming conveyor systems and one reading on the outgoing conveyor systems at the point of highest observed opacity from the various transfer/loadout points.

10.	The permittee is allowed to handle products with the baghouses turned off provided that:

[Rules 62-296.711(2)(a) and 62-4.070(3), F.A.C.; and Permit No. 0570094-009-AC]

- At no time shall the visible emissions limit of 5% opacity be exceeded.

- Oil application or dust suppressant application to the products is done at the processing plant prior to delivery of the product to the transfer facility, or the product must be sufficiently treated with non-hazardous oil or dust suppressant on site as needed to meet the opacity limit.

- Before handling any material with the control equipment turned off, an initial VE test demonstrating compliance must be performed for AFI on all points controlled by baghouses while the baghouses are not in operation.

- The visible emission tests conducted in C) of this condition shall indicate the approximate usage of dust suppressant (gals/ton) and approximate date the product was oiled, or treated.

11.	All reasonable precautions shall be taken to prevent emissions of unconfined particulate matter.  Reasonable precautions shall include, but are not limited to, the following:

[Rule 62-296.320(4)(c)3, F.A.C.]

- Paving or maintenance of roads, parking areas, and yards.

- Application of water when necessary to control emissions.

- Removal of particulate matter from roads and other paved areas under the control of the owner or operator.

- Maintenance of the material transport system, if necessary, including coverings or enclosures.

- Curtailing operations if winds are entraining unconfined particulate matter.

12.	The permittee shall use tarps or canvas, as necessary, to cover the shipholds when loading material into the shipholds in order to meet the required opacity limitations. [Rule 62-4.070(3), F.A.C.]

13.	The maximum allowable emission rate for particulate matter for the baghouses is set by Specific Condition No. 7.  However, because of the expense and complexity of conducting a stack test on minor sources of particulate matter, the Environmental Protection Commission of Hillsborough County pursuant to the authority granted under Rule 62-297.620(4), F.A.C. hereby allows the particulate matter testing on all baghouses, except the one that controls the Wet Phosphate Rock/AP Railcar Unloading Station, to be waived in lieu of a visible emission standard not to exceed an opacity of 5%.

[Rules 62-297.620(4) and 62-296.711(3)(c), F.A.C.]

        - Test EU Nos. 001, 002, 003, 004, 100, 103, 104, and 106 for visible emissions annually while handling AFI.  Both the pickup points and baghouse exhausts must be observed separately if the baghouses are being utilized to demonstrate compliance.  Submit two copies of the test data to the Air Compliance Section of the Air Management Division of the Environmental Protection Commission of Hillsborough County (EPCHC) within 45 days of testing.

[Rules 62-297.310 F.A.C., and Rule 62-4.070(3), F.A.C.]

        - Within 180 days of completion of construction, test EU Nos. 107, 108, 109, 110, and 111 for visible emissions, and annually thereafter while handling AP product.  Within 180 days of completion of construction and prior to permit renewal, test EU No. 109 for particulate matter while handling AP product. Both the pickup points and baghouse exhausts must be observed separately if the baghouses are being utilized to demonstrate compliance.  Submit two copies of the test data to the Air Compliance Section of the Air Management Division of the Environmental Protection Commission of Hillsborough County (EPCHC) within 45 days of testing.  [Rules 62-297.310 F.A.C., and Rule 62-4.070(3), F.A.C.]

        - The following testing conditions shall be adhered to: [Rule 62-4.070(3), F.A.C.]

- If Baghouse #4 is utilized during testing, testing of emissions from the Shipping Terminal Gantry Baghouse exhaust and Transfer Point #4 shall be conducted concurrently with ship loading operation.

- If the baghouses are utilized during testing, testing of emissions from the baghouse exhaust and corresponding pickup points shall be conducted concurrently.

- If testing is conducted without the baghouses in operation, then the test report must indicate the product, date, time and location of coating application, and the approximate usage of dust suppressant (lb/ton).

17.	Testing of emissions shall be conducted with the source operating at capacity.  Capacity is defined as 90-100% of rated capacity listed in Specific Condition No. 9.  If it is impracticable to test at capacity, then the source may be tested at less than capacity; in this case subsequent source operation is limited to 110% of the test load until a new test is conducted.  Once the unit is so limited, then operation at higher capacities is allowed for no more than fifteen days for purposes of additional compliance testing to regain the rated capacity in the permit, with prior notification to the EPC.  Failure to submit the input rates and actual operating conditions may invalidate the test. [Rules 62-4.070(3) and 62-297.310(2)(b), F.A.C.]

  - Compliance with the emission limitations of Specific Condition Nos. 5, 6 and 7 shall be determined using EPA Methods 1, 2, 4, 5, and 9 contained in 40 CFR 60, Appendix A, and adopted by reference in Rule 62-297, F.A.C.  The Method 9 observation period on each source shall be at least 30 minutes.  The minimum requirements for stack sampling facilities, source sampling and reporting shall be in accordance with Rule 62-297, F.A.C. and 40 CFR 60, Appendix A. [Rule 62-297, F.A.C.]

  - [Reserved]

20.	Submit to the Environmental Protection Commission of Hillsborough County each calendar year on or before April 1, completed DEP Form 62-210.900(5), "Annual Operating Report for Air Pollutant Emitting Facility", for the preceding calendar year.  For the purposes of this permit, annual emissions shall be determined and reported in the Annual Operating Report (AOR) using stack test data, or other appropriate emission factors.  [Rule 62-210.370(3), F.A.C.]

21.	The permittee shall minimize emissions at all times, including periods of startup, shutdown, and malfunction in a manner consistent with good air pollution control practice.

[Rules 62-210.700 and 62-4.070(3), F.A.C.]

22. 	The permittee shall install, operate, and maintain a measuring device to determine the air pressure differential across each baghouse within 10% accuracy.  [Rule 62-4.070(3), F.A.C.]

23.	The permittee shall record the pressure differential across the operating baghouses during any required compliance test. [Rule 62-4.070(3), F.A.C.]

24.  	When the Environmental Protection Commission of Hillsborough County (EPC) after investigation, has good reason (such as complaints, increased visible emissions or questionable maintenance of control equipment) to believe that any applicable emission standard contained in Rules 62-204, 62-210, 62-212, 62-296, or 62-297, F.A.C., or in a permit issued pursuant to those rules is being violated, it may require the owner or operator of the source to conduct compliance tests which identify the nature and quantity of pollutant emissions from the source and to provide a report on the results of said tests to the EPC.

[Rule 62-297.310(7)(b), F.A.C.]

25.  	In order to demonstrate compliance with Specific Condition Nos. 6 and 9, the permittee shall maintain daily records of operations for the most recent three year period.  The records shall be made available to the Environmental Protection Commission of Hillsborough County, state or federal air pollution agency upon request.  The records shall include, but not limited to, the following:

[Rules 62-4.070(3) and 62-4.160(14), F.A.C.]

- Month, Day, and Year

- Amount of AP and AFI received individually and combined in tons per day.

- Amount of AP and AFI loaded out individually and combined in tons per day.

- Total of items C) and D) above for the most recent 12 consecutive month period in tons.

- Records of daily visible emission observations required by Specific Condition No. 9.

  - The permittee shall notify the Compliance Section of the Air Management Division of the Environmental Protection Commission of Hillsborough County at least 15 days prior to the date on which each formal visible emissions test is to begin of the time, date, and place of each such test, and the contact person who will be responsible for coordinating and having such test conducted.

[Rule 62-297.310(7)(a)9., F.A.C.]

  - The permittee shall provide timely notification to the Environmental Protection Commission of Hillsborough County prior to implementing any changes that may result in a modification to this permit pursuant to Rule 62-210.200(Modification), F.A.C.  The changes do not include normal maintenance and may also require prior authorization before implementation.  The changes may include, and are not limited to, the following: [Rules 62-210.300 and 62-4.070(3), F.A.C.]

- Alteration or replacement of any equipment or major component of such equipment listed on Page 1 through 3 of this permit.

- Installation or addition of any equipment that is a source of air pollution.

- The use or handling of materials or fuels other than those authorized by this permit.

28.  	If the permittee wishes to transfer this permit to another owner, an “Application for Transfer of Permit” (DEP Form 62-210.900(7)) shall be submitted, in duplicate, to the Environmental Protection Commission of Hillsborough County within 30 days after the sale or legal transfer of the permitted facility.

[Rule 62-4.120, F.A.C.]

29.  	A minimum of two copies of an application for an operation permit shall be submitted to the Environmental Protection Commission of Hillsborough County within 60 days of completion of compliance testing while handling AP or at least 90 days prior to the expiration date of this permit, whichever occurs first. At least one copy of the particulate matter and VE tests required by Specific Condition Nos. 14 and 15 shall be included with the application. [Rules 62-4, 62-210, 62-296.700, F.A.C.]

ENVIRONMENTAL PROTECTION COMMISSION

OF HILLSBOROUGH COUNTY

_________________________________

Richard D. Garrity, Ph.D.

Executive Director