M E M O R A N D U M

DATE:	November 6, 2013

TO:	Richard D. Garrity, Ph.D.

FROM:	Stephen Hathaway, P.E.		THRU:	Diana M. Lee, P.E.

SUBJECT:	REVISED DRAFT Air Construction Permit No. 0570412-007-AC

Vulcan Materials Company, Florida Rock Division

Tampa Sales Yard Expansion Project

Attached is REVISED DRAFT Air Construction Permit No. 0570412-007-AC which is being issued to Vulcan Materials Company, Florida Rock Division for the expansion of the bulk material sales yard located at 3510 Pendola Point Road, Tampa, FL.  Vulcan Materials is proposing to construct an expansion of the existing facility to the east, as a result of acquiring the neighboring property from the Tampa Port Authority.   Vulcan Materials is also requesting to increase the annual throughput limit of the expanded facility from 1.8 to 2.5 million tons per year, in order to accommodate the increased capacity.

The DRAFT Air Construction Permit package, including the Intent to Issue, Public Notice of Intent to Issue, and the Technical Evaluation and Preliminary Determination was sent to the applicant on October 18, 2013.  The applicant submitted comments on October 24, 2013 regarding the DRAFT AC Permit.  The applicant’s comments and EPC Staff responses are outlined below:

Comment Nos. 1 and 2:  Page 2 footer – Suggested Clarification:  The footer should be changed from “Page 2 of 60” to “Page 2 of 8”.

Page 3 footer – Suggested Clarification:  The footer should be changed from “Page 3 of 60” to “Page 3 of 8”.

EPC Staff Response:  These were formatting errors and have been corrected accordingly.

Comment No. 3:  Process Description:

Page 2 of 8, first paragraph, suggested re-wording:  “The new ship unloading system will have up to seven (7) material transfer points and each material transfer point will have a water spray system.”  Each material transfer point will have a water spray system, and the portable conveyors will have partial enclosures.

Comment Rationale:  The material moisture content will be a minimum of 4.8% and each transfer point will be equipped with water spray bars at the material transfer points.  Vulcan Materials Company would like to remove the requirement to have partial enclosures on the portable conveyors.

EPC Staff Response:  Based on the meeting between Vulcan Materials Company and EPC Staff on October 24, 2013, Vulcan representatives explained that in the original facility expansion proposal, existing equipment was to be used for the expansion which was already equipped with partial enclosures.  Consequently, EPC Staff included this equipment in the DRAFT AC permit.  However, since it was explained that Vulcan would no longer be using existing equipment, and the material is expected to maintain a minimum of 4.8% moisture content with water sprays at each of the material transfer points for water application as needed, EPC Staff does not have any objections in removing the requirement to have partial enclosures on the conveyors.  Therefore, the changes are being made as requested.

Comment No. 4:  Process Description:

Page 2 of 8, third paragraph, suggested re-wording:  “The screened material will be gravity fed onto conveyor belts and into storage piles.  Particulate matter emissions from the screening operation will be controlled by water application as necessary sprays and material moisture content.

Comment Rationale:  The portable screeners may not be equipped with water sprays when brought into the facility.  Vulcan Materials Company would like to request flexibility for the type of emission control water application should a portable screener not be equipped with spray bars.

EPC Staff Response:  According to Vulcan Materials, the material tends to retain moisture based on the facility’s periodic moisture content sampling.  In addition, the facility will water, as needed, the material stockpile prior to screening.  Based on this, and the fact that there will be a relatively small amount of material to be screened on an annual basis, EPC Staff does not have any objections to providing Vulcan Materials the operational flexibility for water application as needed to comply with the PM-RACT and Chapter 1-3, Rules of the EPC 5% opacity standards.  The changes have been made as requested.

Comment No. 5:  Process Description:

Page 2 of 8, third paragraph, suggested re-wording:  “From the material storage piles, front-end loaders will either transfer the material to a different storage pile, to the existing washing station, or directly into open-bed trucks for shipment offsite or relocation of the material inside the expanded facility.  The facility may also receive off-site material by truck which will be gravity fed to open stockpiles.”

Comment Rationale:  The permit application identified a new emission unit for truck unloading to storage piles from on-site and off-site trucks.  Vulcan Materials Company would like this language added to the permit description.

EPC Staff Response:  While this emission unit, EU 016 Truck Dump to Pile, was included in the DRAFT AC Permit, EPC staff inadvertently omitted the clarification from the process description that the facility could also receive shipments of material from off-site.  The changes have been made as requested.

Comment No. 6:  Material unloaded from ships shall have a minimum in-bound moisture content of 4.8% by weight.  The material moisture content analysis is conducted when the material is loaded onto the ship for export.  These records will be retained by Vulcan for compliance with the moisture requirement.

EPC Staff Response:  EPC Staff does not have an objection to Vulcan Materials demonstrating compliance with the moisture content requirement for each shipment of material in this manner, provided copies of the moisture content analyses are maintained at the facility for inspection upon request.

Based on the changes proposed in this AC project, the facility-wide PTE for particulate matter will increase from 23.0 tons/yr to 69.1 tons/yr.  Particulate matter emissions will be controlled by material moisture content and water application as needed.  The facility is subject to PM RACT (Rule 62-296.711, Materials Handling, Sizing, Screening, Crushing and Grinding Operations, F.A.C.) and Chapter 1-3, Rules of the EPCHC.

Based upon our review, we recommend the issuance of the REVISED DRAFT AC permit.

SRH: 	0570412-007-AC

TECHNICAL EVALUATION

AND

PRELIMINARY DETERMINATION

FOR

Vulcan Materials Company, Florida Rock Division

Hillsborough County

REVISED DRAFT Air Construction Permit

Application Number

0570412-007-AC

Environmental Protection Commission of

Hillsborough County

Tampa, FL

November 6, 2013

I.  Project Description

| A. Applicant: | Hershel Burton Vulcan Materials Company, Florida Rock Division 3510 Pendola Point Road Tampa, FL 33619 |
| --- | --- |
| B. Engineer: | Michael A. Sussman, P.E. Kleinfelder Southeast, Inc. 1174 Camp Avenue Mt. Dora, FL 32757 |
| P.E. No.: | 69286 |

C.  Project and Location:

Attached is DRAFT Air Construction Permit No. 0570412-007-AC which is being issued to Vulcan Materials Company, Florida Rock Division for the expansion of the bulk material sales yard located at 3510 Pendola Point Road, Tampa, FL.  Vulcan Materials is proposing to construct an expansion of the existing facility to the east, as a result of acquiring the neighboring property from the Tampa Port Authority.   Vulcan Materials is also requesting to increase the annual throughput limit of the expanded facility from 1.8 to 2.5 million tons per year, in order to accommodate the increased capacity.

Vulcan Materials will use existing equipment currently located on the new property, in addition to new equipment, to expand its current facility’s operations.  The expanded facility will include a new bulk material ship unloading and conveying system rated at 2,500 tons per hour and up to seven (7) material transfer points, a new portable screening system to screen up to 100,000 tons per year of bulk material, stockpiling of materials, and a truck tire wash to minimize fugitive dust emissions from truck traffic.  According to Vulcan, the expanded facility will handle the same types of materials as the existing sales yard facility.

Based on the changes proposed in this AC project, the facility-wide PTE for particulate matter will increase from 23.0 tons/yr to 69.1 tons/yr.  Particulate matter emissions will be controlled by material moisture content, water sprays as needed, and best management practices.

The project has been assigned Source Classification Code No. 3-05-020-06 - Industrial Processes, Mineral Products, Stone Quarrying, Miscellaneous: Conveying/Screening/Handling.  The Standard Industrial Code for the project is No. 4491 (Marine Cargo Handling).  The project is located at 3510 Pendola Point Road, Tampa, FL, 33619.  UTM Coordinates of the location are 17-357.9 E and 3090.7 N.

- Process and Controls:

Vulcan Materials Company is a marine terminal facility which handles limestone, granite, gravel, silica sand, aragonite, granulated blast furnace slag, and black slag.

At the existing facility, material is received by ship and loaded onto a shipboard conveyor and belt conveyed to an on-site receiving hopper (H-1) via a ship mounted conveyor boom. The receiving hopper has a water spray system along the top perimeter of the hopper.  From the receiving hopper, the material is gravity fed to a conveyor (C-1) which belt conveys the material to a second hopper (H-2), where it is gravity fed to the radial stacker (R-1), which forms the storage piles.  Each transfer point has a water spray system.  From the storage piles, front-end loaders either transfer the material to a different storage pile, to the washing station, or directly into open-bed trucks for shipment offsite.

Some of the limestone at the facility is washed prior to being loaded into trucks. If the limestone is to be washed, front-end loaders load the limestone from a storage pile into a hopper (H-3), which gravity feeds to a conveyor (C-2) that belt conveys the material into the washing station where it is thoroughly washed with water. After washing, the limestone is transferred to a conveyor belt (C-3) and then into a truck loading bin.  Any oversized limestone is sent back to the storage pile using an oversize conveyor belt (C-4). From the truck loading bin, limestone is gravity fed into a truck. Limestone is only processed through the washing station when a truck is ready to be loaded. Limestone is not stored in the truck loading bin.

At the newly expanded facility, material will be received by ship and loaded onto a shipboard conveyor and belt conveyed to the east on-site receiving hopper (EH-1) via a ship mounted conveyor boom.  The receiving hopper will have a water spray system along the top perimeter of the hopper.  From the receiving hopper, the material will be gravity fed to a conveyor (EC-1) which will convey the material to a radial stacker (ER-1).  From the radial stacker, material will be conveyed to either a second radial stacker (ER-2) to a storage pile, or to up to two (2) portable conveyors (EC-2 and EC-3) prior to feeding the second radial stacker (ER-2), which will then feed the storage pile.  The new ship unloading system will have up to seven (7) material transfer points and each material transfer point will have a water spray system, for water application as needed.

Also at the newly expanded facility will be a portable screener which will be authorized to screen up to 100,000 tons per year of material.  The screening operation will involve the loading of material into a hopper via front-end loader which will gravity feed onto a conveyor belt.  From the conveyor belt, the material will be fed to a vibrating screen where the material will be separated by size.  The screened material will be gravity fed onto conveyor belts and into storage piles.  Particulate matter emissions from the screening operation will be controlled by water application, as necessary and material moisture content.

From the material storage piles, front-end loaders will either transfer the material to a different storage pile, to the existing washing station, or directly into open-bed trucks for shipment offsite or relocation of the material inside the expanded facility.  The facility may also receive off-site material by truck which will be gravity fed to open stockpiles.

Particulate matter emissions from the conveyors, hoppers, radial stackers, transfer points, mobile screener, and storage piles will be controlled by material moisture content, water application as needed, and the use of best management practices.  This facility is subject to PM RACT (Rule 62-296.711, Materials Handling, Sizing, Screening, Crushing and Grinding Operations, F.A.C.) and Chapter 1-3, Rules of the EPCHC.

- Application Information:

Received on:  September 11, 2013

Information Requested:  N/A

Application Complete:  September 11, 2013

II.  Rule Applicability

This project is subject to the preconstruction review requirements of Chapter 403, Florida Statutes, Chapters, 62-204, 62-4, 62-210, 62-212, 62-296, and 62-297, Florida Administrative Code (F.A.C.) and Chapter 1-3 of the Rules of the Environmental Protection Commission of Hillsborough County.

This project is subject to the requirements of Rule 62-212.300, General Preconstruction Review Requirements, F.A.C., since the project is a source of air pollution.

This project is not subject to the requirements of Rule 62-212.400, Prevention of Significant Deterioration, F.A.C. or Rule 62-212.500, New Source Review for Nonattainment Areas, F.A.C., since the facility is a minor source of air pollution.

This project is not subject to the requirements of Rule 62-213, Operation Permits for Major Sources of Air Pollution, F.A.C., since the facility is a Synthetic non-Title V source by state definition.

This project is subject to the requirements of Rule 62-296.320, F.A.C., General Pollutant Emission Limiting Standards, since the operation is a source of Particulate Matter.

This project is not subject to the requirements of Rule 62-296.401 through 62-296.417, Specific Emission Limiting and Performance Standards, F.A.C., since there is not a defined category for this project.

This project is not subject to the requirements of Rule 62-296.500, Reasonably Available Control Technology (RACT) - Volatile Organic Compounds (VOC) and Nitrogen Oxide (NOx) Emitting Facilities, F.A.C., since there is not a defined category for this project.

This project is not subject to the requirements of Rule 62-296.600, Reasonably Available Control Technology (RACT) - Lead, F.A.C., since there is not a defined category for this project.

This project is subject to the requirements of Rule 62-296.700, Reasonably Available Control Technology (RACT) Particulate Matter, F.A.C., since it is located within the Hillsborough County Maintenance Area for the pollutant particulate matter and the potential to emit for PM emissions for the facility are greater than 15 TPY.

This project is not subject to the requirements of Rule 62-204.800(8), F.A.C., Title 40, Code of Federal Regulations, Part 60, Standards of Performance for New Stationary Sources, or Rule 62-204.800, F.A.C., Title 40, Code of Federal Regulations, Part 63, National Emission Standards for Hazardous Air Pollutants for Source Categories.

This project is subject to the requirements of Chapter 84-446, Laws of Florida and Chapter 1-3, Rules of the Environmental Protection Commission of Hillsborough County.

III. Summary of Emissions

| Emission Unit (EU) No(s). | EU Description | Regulated Pollutant | Actual Emissions (tons/yr)(1) | Potential Emissions (tons/yr)(2) | Increase in Emissions (tons/yr) | Allowable Emissions |
| --- | --- | --- | --- | --- | --- | --- |
| 006-010 | West Ship Unloading System 1) Ship Conveyor to Receiving Hopper 2) Hopper to Conveyor (C-1) 3) Conveyor (C-1) to Hopper (H-2) 4) Hopper (H-2) to Radial Stacker (R-1) 5) Radial Stacker (R-1) to Pile | PM | 1.7 | 22.1(3) | 20.4 | 5% opacity |
| 011 | Frontend Loader to 2nd Storage Pile; or, | PM | 0.10 | 8.1 | 7.7 | 5% opacity |
| 012 | Frontend Loader to Truck; | PM | 0.32 |  |  | 5% opacity |
| 013 | Frontend Loader to Washing Receiving Hopper (H-3) | PM | 0.88 | 8.1 | 7.2 | 5% opacity |
| 014 | Washing Receiving Hopper (H-3) to Conveyor | PM | 0.88 | 8.1 | 7.2 | 5% opacity |
| 015 | Conveyor to Washing Station | PM | 0.88 | 8.1 | 7.2 | 5% opacity |
| 016 | East Ship Unloading System (Seven material transfer points) | PM | 0 | 30.9 (3) | 30.9 | 5% opacity |
| 017 | Portable Screening Operation (Four material transfer points) | PM | 0 | 1.9 | 1.9 | 5% opacity |
| 018 | Truck Dump to Pile | PM | 0 | 3.9 | 3.9 | 5% opacity |
|  | TOTALS | PM | 4.76 | 69.1 | 66.0 |  |

- Past actual emissions were based off of the average of emissions from the 2011-2012 AOR’s.

- Potential emissions for the ship unloading material transfer operations were estimated using a calculated emission factor of 0.0014 lb/ton based on a moisture content of 4.8%, and a calculated emission factor of 0.0043 lb/ton for the transfer of material after either of the ship unloading systems based on a moisture content of 2.1%.  Potential emissions from the screening operation were estimated based on an emission factor of 0.025 lb/ton from AP-42 Chapter 11.19.2.  Potential emissions from the handling of slag was calculated by using an emission factor (1.1 lbs/ton) derived from previous stack test data for a similar facility in Hillsborough County.  Based on the analysis provided by Vulcan, material unloaded by ship comes in with a high moisture content, as the material is washed at the point of origin.  The material is expected to maintain a high moisture content as it is handled from the ship through the conveying systems, and into the storage pile.

- The worst-case potential emissions from the ship unloading operations is represented by the East Ship Unloading System (EU 016) which has seven (7) material transfer points, rather than the West Ship Unloading System (EU’s 006-010), which only has five (5) material transfer points.  The overall facility throughput used in the calculations was 2.5 million tons per year. Therefore, the total PM PTE includes only the greater of the two ship unloading systems.

Inventory of Title III pollutants is estimated to be less than 10 TPY individually and less than 25 TPY collectively.

IV.  Conclusions:

The emission limits proposed by the applicant will meet all of the requirements of Chapters 62-204, 62-210, 62-212, 62-296, and 62-297, F.A.C., and Chapter 1-3, Rules of the Commission.

The General and Specific Conditions listed in the proposed permit (attached) will assure compliance with all the applicable requirements of Chapters 62-204, 62-210, 62-212, 62-296, and 62-297, F.A.C.

V.   Proposed Agency Action:

Pursuant to Section 403.087, Florida Statutes and Rule 62-4.070, Florida Administrative Code the Environmental Protection Commission of Hillsborough County hereby gives notice of its intent to issue a permit to operate the aforementioned air pollution source in accordance with the DRAFT permit and its conditions as stipulated (see attached).

In the Matter of an                         	File No.:  0570412-007-AC

Application for Permit by:                   	County:   Hillsborough

Mr. Hershel Burton

Vulcan Materials Company,

Florida Rock Division

3510 Pendola Point Road

Tampa, FL 33619

__________________________________/

INTENT TO ISSUE

The Environmental Protection Commission of Hillsborough County (EPC), as delegated by the Florida Department of Environmental Protection (DEP) gives notice of its intent to issue an Air Construction Permit for the Synthetic non-Title V source detailed in the application specified above, for the reasons stated below.

The applicant, Vulcan Materials Company, Florida Rock Division, applied on September 11, 2013 to the permitting authority for an air construction permit to expand their bulk material handling operation located at 3510 Pendola Point Road, Tampa, Hillsborough County.  Vulcan Materials is proposing to construct a new ship unloading and material handling operation at their facility as a result of the acquisition of the neighboring property to the east.  Also included in this project is the addition of a portable screening operation.

The EPC has permitting jurisdiction under Chapter 403 Florida Statutes (F.S.) and Florida Administrative Code (F.A.C.) Chapters 62-4 and 62-210.  The project is not exempt from permitting procedures.  The EPC has determined that an air pollution construction permit is required to commence or continue operations at the described facility.  The EPC intends to issue this permit based on the belief that reasonable assurances have been provided to indicate that operation of the source will comply with the appropriate provisions of Florida Administrative Code (F.A.C.) Chapters 62-204 through 62-297 and 62-4.

Pursuant to Section 403.815 and 403.0872, F.S. and Rules 62-103.150 and 62-210.350(3), F.A.C., you (the applicant) are required to publish at your own expense the enclosed Notice of Intent to Issue Permit.  The notice shall be published one time as soon as possible, in the legal advertisement section of a newspaper of general circulation in the area affected.  For the purpose of this rule, "publication in a newspaper of general circulation in the area affected" means publication in a newspaper meeting the requirements of Sections 50.011 and 50.031, F.S., in the county where the activity is to take place.  Where there is more than one newspaper of general circulation in the county, the newspaper used must be one with significant circulation in the area that may be affected by the permit.  If you are uncertain that a newspaper meets these requirements, please contact the EPC at the address or telephone number listed below.  The applicant shall provide proof of publication to the EPC, Air Permitting Section, at 3629 Queen Palm Dr., Tampa, Florida 33619 (Phone 813-627-2600 - FAX 813-627-2660) within 7 (seven) days of publication.    Failure to publish the notice and provide proof of publication within the allotted time may result in the denial of the permit pursuant to Rule 62-103.150(6), F.A.C.

Mediation under section 120.573, F.S. is not available in this proceeding.

A person subject to regulation has a right to apply to the Department of Environmental Protection for a variance from or waiver of the requirements of particular rules, on certain conditions, under Section 120.542, F.S.  The relief provided by this state statute applies only to state rules, not statutes, and not to any federal regulatory requirements.  Applying for a variance or waiver does not substitute or extend the time for filing a petition for an administrative hearing or exercising any other right that a person may have in relation to the action proposed in this notice of intent.

The application for a variance or waiver is made by filing a petition with the Office of General Counsel of the Department of Environmental Protection, 3900 Commonwealth Boulevard, Mail Station #35, Tallahassee, FL 32399-3000.  The petition must specify the following information:

- The name, address, and telephone number of the petitioner,

- The name, address, and telephone number of the attorney or qualified representative of the petitioner, if any,

- Each rule or portion of a rule from which a variance or waiver is requested,

- The citation to the statute underlying (implemented by) the rule identified in (c) above,

- The type of action requested,

- The specific facts that would justify a variance or waiver for the petitioner,

- The reason by the variance or waiver would serve the purposes of the underlying statute (implemented by the rule), and

- A statement whether the variance or waiver is permanent or temporary and, if temporary, a statement of the dates showing the duration of the variance or waiver requested.

The Department will grant a variance or waiver when the petition demonstrates both that the application of the rule would create a substantial hardship or violate principles of fairness, as each of the those terms is defined in Section 120.542(2), F.S., and that the purpose of the underlying statute will be or has been achieved by other means by the petitioner.

Persons subject to regulation pursuant to any federally delegated or approved air program should be aware that Florida is specifically not authorized to issue variances or waivers from any requirements of any such federally delegated or approved program.  The requirements of the program remain fully enforceable by the Administrator of United States Environmental Protection Agency and by any person under the Clean Air Act unless and until the Administrator separately approves any variance or waiver in accordance with the procedures of the federal program.

Any person listed below may request to obtain additional information, a copy of the application (except for information entitled to confidential treatment pursuant to Section 403.111, F.S.), all relevant supporting materials, and all other materials available to the EPC that are relevant to the permit decision.  Interested persons may contact Diana M. Lee, P.E., at the above address or call (813) 627-2600, for additional information.

Any party to this order has the right to seek judicial review of it under Section 120.68 of the Florida Statues, by filing a notice of appeal under rule 9.110 of the Florida rules of Appellate Procedure with the EPC’s Legal Office at 3629 Queen Palm Dr., Tampa, Florida 33619 and with the clerk of the Department of Environmental Protection in the Office of General Counsel, Mail Station 35, 3900 Commonwealth Boulevard, Tallahassee, Florida 32399-3000, and by filing a copy of the notice of appeal accompanied by the applicable filing fees with the appropriate district court of appeal.  The notice must be filed within thirty days after this order is filed with the clerk of the Department.

Executed in Tampa, Florida

ENVIRONMENTAL PROTECTION COMMISSION

OF HILLSBOROUGH COUNTY

__________________________________

Richard D. Garrity, Ph.D.

Executive Director

## CERTIFICATE OF SERVICE

The undersigned duly designated deputy agency clerk hereby certifies that this INTENT TO ISSUE AN AIR CONSTRUCTION PERMIT (including the PUBLIC NOTICE and the REVISED DRAFT permit package) and all copies were sent by certified mail or electronically (with read receipt) before the close of business on ___________________ to the person(s) listed:

Hershel Burton / Vulcan Materials Company, Florida Rock Division

In addition, the undersigned duly designated deputy agency clerk hereby certifies that copies of this INTENT TO ISSUE AN AIR CONSTRUCTION PERMIT (including the PUBLIC NOTICE and the REVISED DRAFT permit package) were sent by U.S. mail or electronically (with Read Receipt) on the same date to the person(s) listed or as otherwise noted:

James Burkholder / Vulcan Materials Company, Florida Rock Division

Clerk Stamp

FILING AND ACKNOWLEDGEMENT

FILED, on this date, pursuant to Section 120.52(7), Florida Statutes, with the designated clerk, receipt of which is hereby acknowledged.

_________________________   	_____________

Clerk                  			Date

ENVIRONMENTAL PROTECTION COMMMISSION

OF HILLSBOROUGH COUNTY

PUBLIC NOTICE OF INTENT TO ISSUE PERMIT

# REVISED DRAFT Air Construction Permit No:  0570412-007-AC

Vulcan Materials Company, Florida Rock Division

Hillsborough County

The Environmental Protection Commission of Hillsborough County (EPC), as delegated by the Florida Department of Environmental Protection (DEP) gives notice of its intent to issue an Air Construction Permit to Vulcan Materials Company, Florida Rock Division to expand their bulk material handling operation located at 3510 Pendola Point Road, Tampa, Hillsborough County.  Vulcan Materials is proposing to construct a new ship unloading and bulk material handling operation at their expanded facility.  Also included in this project is the addition of a portable screening operation.  Vulcan Materials is a Synthetic non-Title V source of particulate matter emissions.  Emissions will be controlled by water application as needed, material moisture content, and best management practices.

A Best Available Control Technology (BACT) determination was not required.

The EPC will issue the FINAL permit with the conditions of the DRAFT permit unless a timely petition for an administrative hearing is filed pursuant to Sections 120.569 and 120.57, F.S. before the deadline for filing a petition.  The procedures for petitioning for hearing are set forth below.

A person whose substantial interests are affected by the proposed permitting decision may petition for an administrative proceeding (hearing) under Sections 120.569 and 120.57, F.S.  The petition must contain the information set forth below and must be filed (received) in the Legal Department of the EPC at 3629 Queen Palm Dr., Tampa, Florida 33619, Phone 813-627-2600, Fax 813-627-2602.  Petitions filed by any persons other than those entitled to written notice under Section 120.60(3), F.S. must be filed within 14 (fourteen) days of publication of the public notice or within 14 (fourteen) days of receipt of this notice of intent, whichever occurs first.  Under Section 120.60(3), however, any person who asked the EPC for notice of agency action may file a petition within 14 (fourteen) days of receipt of that notice, regardless of the date of publication.  A petitioner shall mail a copy of the petition to the applicant at the address indicated above, at the time of filing.  The failure of any person to file a petition within the appropriate time period shall constitute a waiver of that person's right to request an administrative determination (hearing) under Sections 120.569 and 120.57, F.S., or to intervene in this proceeding and participate as a party to it.  Any subsequent intervention will be only at the approval of the presiding officer upon the filing of a motion in compliance with Rule 28-106.205 of the F.A.C.

A petition that disputes the material facts on which the EPC’s action is based is required to contain the following information:

- The name and address of each agency affected and each agency’s file or identification number if known;

- The name, address, and telephone number of the petitioner, and the name, address, and telephone number of each petitioner’s representative, if any, which shall be the address for service purposes during the course of the proceedings; and an explanation of how the petitioner’s substantial interests will be affected by the agency determination;

- A statement of how and when petitioner received notice of the EPC action;

- A statement of all disputed issues of material fact.  If there are none, the petition must so indicate;

- A concise statement of the ultimate facts alleged, including the specific facts the petitioner contends warrant reversal or modification of the EPC proposed action;

- A statement of specific rules or statutes the petitioner contends requires reversal or modification of the EPC’s proposed action; and

- A statement of the relief sought by the petitioner, stating precisely the action petitioner wishes the EPC to take with respect to the EPC’s proposed action.

A petition that does not dispute the material facts upon which the EPC’s action is based shall state that no such facts are in dispute and otherwise shall contain the same information as set forth above as required by Rule 28-106.301.

Because the administrative hearing process is designed to formulate final agency action, the filing of a petition means that the EPC's final action may be different from the position taken by it in this notice of intent.  Persons whose substantial interests will be affected by any such final decision of the EPC on the application have the right to petition to become a party to the proceeding, in accordance with the requirements set forth above.  Mediation under section 120.573, F.S. is not available in this proceeding.

This action is final and effective on the date filed with the Clerk of the EPC unless a petition is filed in accordance with above.  Upon the timely filing of a petition this order will not be effective until further order of the EPC.

Any party to this order has the right to seek judicial review of it under Section 120.68 of the Florida Statues, by filing a notice of appeal under rule 9.110 of the Florida rules of Appellate Procedure with the EPC’s Legal Office at 3629 Queen Palm Dr., Tampa, Florida 33619 and with the clerk of the Department of Environmental Protection in the Office of General Counsel, Mail Station 35, 3900 Commonwealth Boulevard, Tallahassee, Florida 32399-3000, and by filing a copy of the notice of appeal accompanied by the applicable filing fees with the appropriate district court of appeal.  The notice must be filed within thirty days after this order is filed with the clerk of the Department.

The complete project file is available for public inspection during normal business hours, 8:00 a.m. to 5:00 p.m., Monday through Friday, except legal holidays, at the Environmental Protection Commission of Hillsborough County, 3629 Queen Palm Dr., Tampa, FL 33619.  The complete project file includes the proposed Permit, the application, and the information submitted by the responsible official, exclusive of confidential records under Section 403.111, F.S.  Interested persons may contact Diana M. Lee, P.E., at the above address, or call 813-627-2600, for additional information.  Any written comments filed shall be available for public inspection.  If written comments received result in a significant change in the proposed agency action, the EPC shall revise the proposed permit and require, if applicable, another Public Notice.

ENVIRONMENTAL PROTECTION COMMISSION OF

HILLSBOROUGH COUNTY, as Delegated by

STATE OF FLORIDA

DEPARTMENT OF ENVIRONMENTAL PROTECTION

NOTICE OF PERMIT

Mr. Hershel Burton

Vulcan Materials Company,

Florida Rock Division

3510 Pendola Point Road

Tampa, FL 33619

Dear Mr. Burton:						Re:  Hillsborough County - AP

Enclosed is Air Construction Permit Number No. 0570412-007-AC to expand the Tampa sales yard and construct a new ship unloading and bulk material handling operation located at 3510 Pendola Point Road in Tampa.  This permit is being issued pursuant to Section 403.087, Florida Statutes.  Additional details are documented in the Technical Evaluation attached to the REVISED DRAFT permit.

Any party to this order (permit) has the right to seek judicial review of the permit pursuant to Section 120.68, Florida Statutes, by the filing of a Notice of Appeal pursuant to Rule 9.110, Florida Rules of Appellate Procedure, with the Clerk of the EPC in the Legal Department at 3629 Queen Palm Drive, Tampa, FL  33619; and by filing a copy of the Notice of Appeal accompanied by the applicable filing fees with the appropriate District Court of Appeal.  The Notice of Appeal must be filed within 30 days from the date this Notice is filed with the clerk of the EPC.

Executed in Tampa, Florida.

Sincerely,

Richard D. Garrity, Ph.D.

Executive Director

cc:  	FDEP Southwest District (via e-mail)

Vulcan Materials Company, Florida Rock Division					              Page 2

Tampa, FL 33619

CERTIFICATE OF SERVICE

This is to certify that this NOTICE OF PERMIT and all copies were mailed before the close of business on _________________________ to the listed persons.

Clerk Stamp

FILED, on this date, pursuant to Section 120.52(7), Florida Statutes, with the designated clerk, receipt of which is hereby acknowledged.

__________________________   _____________

Clerk                     	    	       Date

| PERMITTEE: | PERMIT/CERTIFICATION: |
| --- | --- |
| Vulcan Materials Company, Florida Rock Division 3510 Pendola Point Road Tampa, FL 33619 | Permit No.: 0570412-007-AC County: Hillsborough Expiration Date: November 6, 2015 Project: Tampa Sales Yard Expansion |

This permit is issued under the provisions of Chapter 403, Florida Statutes, and Florida Administrative Code Rules 62-204, 62-210, 62-212, 62-296, 62-297, and 62-4.  The above named permittee is hereby authorized to perform the work or operate the facility shown on the application and approved drawing(s), plans, and other documents, attached hereto or on file with the EPC and made a part hereof and specifically described as follows:

Project Description

Vulcan Materials Company is an existing marine terminal facility which handles bulk limestone, granite, gravel, silica sand, aragonite, granulated blast furnace slag, and black slag.

Vulcan Materials is proposing to construct an expansion of the existing Tampa Sales Yard to the east, as a result of the acquisition of the neighboring property from the Tampa Port Authority.   Vulcan Materials is also requesting to increase the annual throughput limit of the expanded facility from 1.8 to 2.5 million tons per year, in order to accommodate the increased capacity.

Vulcan Materials will use existing equipment currently located on the new property, in addition to new equipment, to expand its current facility’s operations.  The expanded facility will include a new bulk material ship unloading and conveying system rated at 2,500 tons per hour and up to seven (7) material transfer points, a new portable screening system to screen up to 100,000 tons per year of bulk material, stockpiling of materials, and a truck tire wash to minimize fugitive dust emissions from truck traffic.  According to Vulcan Materials, the expanded facility will handle the same types of materials as the existing sales yard facility.

Process Description

At the newly expanded facility, material will be received by ship and loaded onto a shipboard conveyor and belt conveyed to the east on-site receiving hopper (EH-1) via a ship mounted conveyor boom. The receiving hopper will have a water spray system along the top perimeter of the hopper.  From the receiving hopper, the material will be gravity fed to a conveyor (EC-1) which will convey the material to a radial stacker (ER-1).  From the radial stacker, material will be conveyed to either a second radial stacker (ER-2) to a storage pile, or to up to two (2) portable conveyors (EC-2 and EC-3) prior to feeding the second radial stacker (ER-2), which will then feed the storage pile.  The new ship unloading system will have up to seven (7) material transfer points and each material transfer point will have a water spray system, for water application as-needed.

Also at the newly expanded facility will be a portable screener which will be authorized to screen up to 100,000 tons per year of material.  The screening operation will involve the loading of material into a hopper via front-end loader which will gravity feed onto a conveyor belt.  From the conveyor belt, the material will be fed to a vibrating screen where the material will be separated by size.  The screened material will be gravity fed onto conveyor belts and into storage piles.  Particulate matter emissions from the screening operation will be controlled by water application as-needed and material moisture content.

From the material storage piles, front-end loaders will either transfer the material to a different storage pile, to the existing washing station, or directly into open-bed trucks for shipment offsite or relocation of the material inside the expanded facility.  The facility may also receive off-site material by truck which will be gravity fed to open stockpiles.

Particulate matter emissions from the conveyors, hoppers, radial stackers, transfer points, mobile screener, and storage piles will be controlled by water application as-needed, material moisture content, and the use of best management practices.  This facility is subject to PM RACT (Rule 62-296.711, Materials Handling, Sizing, Screening, Crushing and Grinding Operations, F.A.C.) and Chapter 1-3, Rules of the EPCHC.

Based on the changes proposed in this AC permit, the facility-wide PTE for particulate matter will increase from 23.0 tons/yr to 69.1 tons/yr.

Location:  3510 Pendola Point Road, Tampa

UTM:  17-359.10 E  3086.90 N    FACILITY ID NO: 0570412

### The following emissions unit(s) are being constructed as part of this project:

| EU ID No. | Emission Unit Description |
| --- | --- |
| 016 | East Ship Unloading System Ship Conveyor to Receiving Hopper (EH-1) Hopper (EH-1) to Conveyor (EC-1) Conveyor (EC-1) to Radial Stacker (ER-1) Radial Stacker (ER-1) to Radial Stacker (ER-2); or, Radial Stacker (ER-1) to Portable Conveyor (EC-2) Portable Conveyor (EC-2) to Portable Conveyor (EC-3) Portable Conveyor (EC-2 or EC-3) to Radial Stacker (ER-2) Radial Stacker to Storage Pile |
| 017 | Portable Screening Operation |
| 018 | Truck Dump to Pile |

Replaces Permit Nos.:  N/A

References Permit No.:  0570412-006-AO

1.	A part of this permit is the attached 15 General Conditions.  [Rule 62-4.160, F.A.C.]

2.	All applicable rules of the Environmental Protection Commission of Hillsborough County including design discharge limitations specified in the application shall be adhered to.  The permit holder may also need to comply with county, municipal, federal, or other state regulations prior to construction.  [Rule 62-4.070(7), F.A.C.]

3.	Issuance of this permit does not relieve the permittee from complying with applicable emission limiting standards or other requirements of Chapters 62-204, 62-210, 62-212, 62-296 and 62-297, F.A.C., or any other requirements under federal, state, or local law.  [Rule 62-210.300, F.A.C.]

4.	The permittee shall not cause, suffer, allow, or permit the discharge of air pollutants which cause or contribute to an objectionable odor.  [Rule 62-296.320(2), F.A.C.]

5.	[RESERVED]

6.	The permittee shall not cause, permit, or allow any visible emissions (five (5%) percent opacity) from any point in the aggregate handling operation including, but not limited to, the following emission points:  [Rules 62-296.711(2)(a) and 62-4.070(3), F.A.C.]

- Shiphold

- Ship Unloading Conveyors

- All Hoppers

- All Conveyor Transfer Points

- Radial Stacker to Storage Piles

- Front-end Loader to Storage Pile

- Front-end Loader to Truck

- Front-end Loader to Washer Receiving Hopper

- Washer Receiving Hopper to Conveyor

- Conveyor to Washing Station

- Washing Station

- Truck Loading Bin

- Truck Loading Bin to Truck

- Truck to Storage Pile

- Screening Operations

7.	The following limitations and restrictions shall apply during any twelve consecutive month period:  [Rule 62-4.070(3), F.A.C., Permit No. 0570412-006-AO, and Air Construction Permit Application Received September 11, 2013]

- The permittee shall handle only the following materials:

Material					Tons/12 Consecutive

Month Period

- Limestone, Granite, Gravel,		      2,450,000

Silica Sand	, and Aragonite

- Granulated Blast Furnace Slag	      	      50,000

and Black Slag

| Operation | Maximum Material Handling Rate |  |
| --- | --- | --- |
|  | (tons/hour) | (tons/12 month period) |
| West Ship Unloading | 6,000 | 2,500,000 |
| East Ship Unloading | 2,500 |  |
| Front-end Loader to Storage Pile and Truck Loading | 600 | 2,500,000 |
| Truck to Storage Pile | 1,000 | 1,800,000 |
| Portable Screening Operation | 600 | 100,000 |

- Material unloaded from ships shall have a minimum in-bound moisture content of 4.8% by weight.

- Only saturated limestone shall be processed through the truck loading bin.

- Maintain a water spray system to reach the storage piles as needed.

8.	Test each emission point associated with Emission Units 006 through 018 for visible emissions as follows.  Submit two copies of the test results to the Environmental Protection Commission of Hillsborough County within 45 days of testing.  Each test shall be at least 30 minutes in duration unless the operation is completed in less than 30 minutes and does not reoccur during the observation period.  In that case, the visible emissions test duration shall be equal to the duration of the operation.  The visible emission tests shall be conducted in accordance with Rule 62-297.310, F.A.C.  [Rules 62-297.310(4)(a)2.a. and (7), F.A.C.]

- Test Emission Units 016, 017, and 018 for visible emissions at the point of highest opacity, the first time each material listed in Specific Condition No. 7.A is received and handled.  Test annually thereafter (October 1 – September 30) while handling any of the materials listed in Specific Condition No. 7.A.

- Test Emission Units 006 through 015 for visible emissions at the point of highest opacity, the first time a new material listed in Specific Condition No. 7.A is received and handled.  Test annually thereafter (October 1 – September 30) while handling any of the materials listed in Specific Condition No. 7.A.

9.	Compliance with the emission limitation of Specific Condition No. 6 shall be determined using EPA Method 9 contained in 40 CFR 60, Appendix A and adopted by reference in Rule 62-297, F.A.C.  The EPA Method 9 test shall be at least 30 minutes in duration or as otherwise specified in Specific Condition No. 8.  The minimum requirements for stack sampling facilities, source sampling and reporting, shall be in accordance with Rule 62-297, F.A.C. and 40 CFR 60, Appendix A. [Rules 62-297.310, 62-297.310(4)(a)2, and 62-296.711(3)(a) F.A.C.]

10.	Testing of emissions shall be conducted with the source operating at capacity.  Capacity is defined as 90-100% of rated capacity specified in Specific Condition No. 7.  If it is impracticable to test at capacity, then the source may be tested at less than capacity; in this case subsequent source operation is limited to 110% of the test load until a new test is conducted.  Once the unit is so limited, then operation at higher capacities is allowed for no more than fifteen days for purposes of additional compliance testing to regain the rated capacity in the permit, with prior notification to the EPC.  Failure to submit the input rates and actual operating conditions may invalidate the test.  [Rules 62-4.070(3) and 62-297.310(2)(b), F.A.C.]

11.	The owner or operator shall notify the Environmental Protection Commission of Hillsborough County, at least 15 days prior to the date on which each formal compliance test is to begin, of the date, time, and place of each such test, and the test contact person who will be responsible for coordinating and having such test conducted.  [Rule 62-297.310(7)(a)9., F.A.C.]

12.	In order to demonstrate compliance with Specific Condition Nos. 5 and 7, the permittee shall maintain monthly records of operations for the most recent three year period.  The records shall be made available to the Environmental Protection Commission of Hillsborough County, state or federal air pollution agency upon request and shall include, but not limited to, the following:   [Rules 62-4.070(3) and 62-4.160, F.A.C.; Permit No. 0570412-006-AO, and Air Construction Permit Application Received September 11, 2013]

- Month, Year

- Amount and type of material unloaded from ships

- Amount and type of unwashed material loaded into trucks

- Amount of washed limestone processed through the washing station and loaded into trucks

- Type and amount of material processed through the portable screening operation

- Type and amount of material transferred from truck to storage pile.

- Monthly summary and rolling twelve month total of B) through F) above.

- Records as required by Specific Condition No. 13.B)ii.

- Moisture content analysis for material unloaded from each ship and from stockpiles.

13.	All reasonable precautions shall be taken to prevent and control generation of unconfined emissions of particulate matter in accordance with the provision in Rule 62-296.320, F.A.C.  These provisions are applicable to any source, including, but not limited to, vehicular movement, transportation of materials, construction, alterations, demolition or wrecking, or industrial related activities such as loading, unloading, storing and handling.  These reasonable precautions shall include, but are not limited to, the following:  [Rules 62-4.070(3) and 62-296.320(4)(c), F.A.C.]

A)  Daily (When in operation)

- Operate water spray heads on hoppers and conveyor drops, as needed.

- Maintain roadways and unpaved areas free of material build-up.

- Maintain vehicular speed to a minimum (10 mph or less).  Post signs if necessary.

- Inspect material drop points and maintain a minimum drop height distance as reasonably

possible.

- Minimize the pile height as needed.

- Properly operate and maintain the truck tire wash.

B)  Weekly

- Inspect road and other paved surfaces.  Clean as necessary.

- Perform maintenance inspections on the sprinkler systems at the storage piles and conveyor drop points.  Record the date the inspection was performed and any maintenance work performed.

14.	When the Environmental Protection Commission of Hillsborough County (EPC) after investigation, has good reason (such as complaints, increased visible emissions or questionable maintenance of control equipment) to believe that any applicable emission standard contained in Rules 62-204, 62-210, 62-212, 62-296, or 62-297, F.A.C., or in a permit issued pursuant to those rules is being violated, it may require the owner or operator of the source to conduct compliance tests which identify the nature and quantity of pollutant emissions from the source and to provide a report on the results of said tests to the EPC. [Rule 62-297.310(7)(b), F.A.C.]

15.	The permittee shall provide timely notification to the Environmental Protection Commission of Hillsborough County prior to implementing any changes that may result in a modification to this permit pursuant to Rule 62-210.200(205), F.A.C., Modification.  The changes do not include normal maintenance, but may include, and are not limited to, the following, and may also require prior authorization before implementation: [Rules 62-210.300 and 62-4.070(3), F.A.C.]

A)	Alteration or replacement of any equipment or major component of such equipment.

B)	Installation or addition of any equipment which is a source of air pollution.

C)	Handling any new materials not previously handled at the facility.

16.  	Submit to the Environmental Protection Commission of Hillsborough County each calendar year on or before April 1, completed DEP Form 62-210.900(5), "Annual Operating Report for Air Pollutant Emitting Facility", for the preceding calendar year.  [Rule 62-210.370(3), F.A.C.]

17.	If the permittee wishes to transfer this permit to another owner, an "Application for Transfer of Air Permit" (DEP Form 62-210.900(7)) shall be submitted, in duplicate, to the Environmental Protection Commission of Hillsborough County within 30 days after the sale or legal transfer of the permitted facility.  [Rule 62-4.120, F.A.C.]

18.	Within sixty (60) days of completion of visible emissions compliance testing on EU’s 016 and 018 as required by Specific Condition No. 8.A) of this permit, but no later than sixty days prior to the expiration of this permit, the permittee shall apply for a non-Title V permit revision using the current version of the permit application form along with the proper fee.  The application shall also include a copy of the required compliance tests.  [Rules 62-4.050(2) and 62-210, F.A.C.]

ENVIRONMENTAL PROTECTION COMMISSION

OF HILLSBOROUGH COUNTY

__________________________________

Richard D. Garrity, Ph.D.

Executive Director