FINAL DETERMINATION

FOR

Mutual Scrap Processing, Inc.

Hillsborough County

Construction Permit

Application Number

0571273-004-AC

Environmental Protection Commission of

Hillsborough County

Tampa, FL

September 23, 2013

FINAL DETERMINATION

The Environmental Protection Commission of Hillsborough County mailed a public notice package on August 23, 2013 that included Intent to Issue Air Construction Permit No. 0571273-004-AC to Mutual Scrap Processing, Inc. located at 4902 South 50th Street, Tampa, Hillsborough County, FL 33619.

Mutual Scrap Processing operates an aluminum scrap recycling facility.  There are three (3) types of scrap aluminum material collected at the site: cast aluminum parts that may include car transmissions, radiators and other aluminum materials like sheets, siding, etc.  The scrap aluminum material is either delivered by or picked-up from commercial customers by open-bed trucks or roll-offs. The scrap is first weighed and then dumped onto the ground where it lies until it is time to process.  From these piles a crane is used to lift the scrap into a shredder which reduces the material to a smaller workable size and discharges it to a pile.  The shredded material is inspected to remove non-aluminum materials like plastic and also a magnet is used to remove steel.  The prepped material is then processed in a "dirt screen".  The screen discharges to two different piles, an aluminum pile and a dirt pile. The material is then ready to load onto a truck and is sent to the client.   There are also four (4) bobcats and two (2) front end loaders on site to handle the scrap material.

The Public Notice of Intent to Issue was published on September 6, 2013 in the La Gaceta.

No comments have ever received since the Intent was published.

The final action of the Environmental Protection Commission of Hillsborough County is to issue the final permit.

ENVIRONMENTAL PROTECTION COMMISSION OF

HILLSBOROUGH COUNTY, as Delegated by

STATE OF FLORIDA

DEPARTMENT OF ENVIRONMENTAL PROTECTION

NOTICE OF PERMIT

Jon Hale, Owner

Mutual Scrap Processing, Inc.

4902 South 50th Street

Tampa, FL 33619

Dear Mr. Hale:

Re:  Hillsborough County - AP

Enclosed is Permit Number 0571273-004-AC to operate an aluminum scrap processing facility located at 4902 South 50th Street, Tampa, FL 33619, issued pursuant to Section 403.087, Florida Statutes.

Any party to this order (permit) has the right to seek judicial review of the permit pursuant to Section 120.68, Florida Statutes, by the filing of a Notice of Appeal pursuant to Rule 9.110, Florida Rules of Appellate Procedure, with the Clerk of the EPC in the Legal Department at 3629 Queen Palm Drive, Tampa, Florida, 33619 and by filing a copy of the Notice of Appeal accompanied by the applicable filing fees with the appropriate District Court of Appeal.  The Notice of Appeal must be filed within 30 days from the date this Notice is filed with the clerk of the EPC.

Executed in Tampa, Florida.

Sincerely,

Richard D. Garrity, Ph.D.

Executive Director

cc:  Kenneth E. Given, P.E. - Air Testing & Consulting, Inc.

CERTIFICATE OF SERVICE

This is to certify that this NOTICE OF PERMIT and all copies were mailed before the close of business on _________________________ to the listed persons.

Clerk Stamp

FILED, on this date, pursuant to Section 120.52(7), Florida Statutes, with the designated clerk, receipt of which is hereby acknowledged.

___________________________  _____________

Clerk                                                Date

PERMITTEE:	PERMIT/CERTIFICATION

Mutual Scrap Processing, Inc.	Permit No.: 0571273-004-AC

4902 South 50th Street	County: Hillsborough

Tampa, FL 33619	Expiration Date:  September 1, 2014

Project: Aluminum Scrap Processing Facility

This permit is issued under the provisions of Chapter 403, Florida Statutes, and Florida Administrative Code Rules 62-204, 62-210, 62-212, 62-296, 62-297, and 62-4.  The above named permittee is hereby authorized to perform the work or operate the facility shown on the application and approved drawing(s), plans, and other documents, attached hereto or on file with the EPC and made a part hereof and specifically described as follows:

- This project is for an “after-the-fact” air construction permit to operate an aluminum scrap processing facility located at 4902 South 50th Street, Tampa, Florida.  There are three (3) types of scrap aluminum material collected at the site: cast aluminum parts that may include car transmissions, radiators and other aluminum materials like sheets, siding, etc.  The scrap aluminum material is either delivered by or picked-up from commercial customers by open-bed trucks or roll-offs. The scrap is first weighed and then dumped onto the ground where it lies until it is time to process.  From these piles a crane is used to lift the scrap into a shredder which reduces the material to a smaller workable size and discharges it to a pile.  The shredded material is inspected to remove non-aluminum materials like plastic and a magnet is used to remove steel.  The prepped material is then processed in a "dirt screen".  The screen discharges to two different piles, an aluminum pile and a dirt pile. The material is then ready to load onto a truck and is sent to the client.   There are also four (4) bobcats and two (2) front end loaders on site to handle the scrap material.  Emissions will be controlled by using best management practices, which can include applying water, as needed.

- The facility is not subject to Rule 62-296.700, F.A.C., PM RACT, since the facility emits less than 15 TPY and 5 lbs/hr of PM emissions.  However, the visible emissions (VE) from the scrap aluminum handling operations (i.e., transfer point or drop point) are limited to 5% opacity standard, pursuant to Chapter 1-3.52(1), Rules of the EPC.  Visible emissions from the diesel engines is limited to a 20% opacity standard pursuant to Rule 62-296.320(4)(b)1., F.A.C.

- The Shredder is driven by a 700 HP diesel fired reciprocating internal combustion engine (RICE) and the Dirt Screener is driven by a 110 HP diesel fired RICE.  Both RICEs are not subject to 40 CFR 60, Subpart IIII or 40 CFR 63, Subpart ZZZZ since they are non-road engines instead of stationary RICEs as defined under 40 CFR 1068.30.  According to the permit application, the Shredder is moved around the site to the location of the piles of scrap aluminum and is also operated at other facility sites.

Location:  4902 South 50th Street, Tampa, FL 33619

UTM Coordinates:		17- 3596.5 E, 30877.4 N

Latitude:	27° 54' 26" N	Longitude:	82° 25' 34" W

Facility ID No.:		0571404

Emission Unit (EU) Nos.:	EU 005 – Material Handling

EP 1 - Truck unloading to pile

EP 2 - Pile to pile transfer

EP 3 - Crane to Shredder

EP 4 - Shredding

EP 5 - Shredder discharge to pile

EP 6 - Pile to Dirt Screener

EP 7 - Screening

EP 8 - Dirt Screener to aluminum pile

EP 9 - Dirt Screener to dirt pile

EP 10 - Dirt pile to truck

EP 11 - Aluminum pile to truck

EU 006 – Diesel Powered Shredder

EU 007 - Diesel Powered Dirt Screener

- A part of this permit is the attached General Conditions. [Rule 62-4.160, F.A.C.]

- Issuance of this permit does not relieve the permittee from complying with applicable emission limiting standards or other requirements of Chapters 62-204, 62-210, 62-212, 62-296 and 62-297, F.A.C., or any other requirements under federal, state, or local law. [Rule 62-210.300, F.A.C.]

- All applicable rules of the Environmental Protection Commission of Hillsborough County including design discharge limitations specified in the application shall be adhered to.  The permit holder may also need to comply with county, municipal, federal, or other state regulations prior to construction.

[Rule 62-4.070(7), F.A.C.]

- The use of property, facilities, equipment, processes, products, or compounds, or any other act that causes or materially contributes to a public nuisance is prohibited, pursuant to the Hillsborough County Environmental Protection Act, Section 16, Chapter 84-446, Laws of Florida, as Amended.

- Operating hours for the material handling at this facility are not limited.  Therefore, this facility is authorized to operate 8,760 hours per consecutive 12 month period.

[Rule 62-4.070(3), F.A.C., and Air Construction (AC) Permit Application submitted July 22, 2013]

- As requested by the permittee, the facility shall not receive or process more than 20,000 tons of scrap aluminum and consume more than 52,000 gallons of diesel fuel in any consecutive 12 month period.

[Rule 62-4.070(3), F.A.C., and AC Permit Application submitted July 22, 2013]

- In order to limit the potential to emit and establish the facility as a synthetic minor for criteria pollutants, the facility-wide particulate matter (PM) emissions shall not exceed 12.1 tons in any consecutive 12 month period (TPY).

[Rules 62-210.200 – “Potential to Emit” and 62-4.070(3), F.A.C. and AC Permit Application submitted July 22, 2013]

- The permittee shall maintain a truck scale to determine the amount of scrap aluminum received with an error of less than or equal to 10 percent.  [Rule 62-4.070(3), F.A.C.]

- Visible emissions from all scrap material handling activities shall not exceed 5% opacity.  Visible emissions from the diesel engines that power the equipment shall not be equal to or greater than 20% opacity.

[Rule 62-296.320, F.A.C., and Chapter 1-3.52, Rules of the EPCHC and AC Permit Application submitted July 22, 2013]

- The permittee shall not cause, suffer, allow or permit the discharge of air pollutants which cause or contribute to an objectionable odor.  An objectionable odor is any odor present in the outdoor atmosphere which by itself or in combination with other odors, is or may be harmful or injurious to human health or welfare, which unreasonably interferes with the comfortable use and enjoyment of life or property, or which creates a nuisance. [Rule 62-210.200 (Objectionable Odor) and Rule 62-296.320, F.A.C.]

- In order to assure compliance with the emission limiting standards in Specific Condition No. 9, the permittee shall implement the following work practices:  [Rule 62-4.070(3)]

- Crane & loader operators shall minimize the material drop heights as much as possible;

- Apply water to material stockpiles, as needed, prior to processing.

- All reasonable precautions shall be taken to prevent and control generation of unconfined emissions of particulate matter in accordance with the provision in Rule 62-296.320, F.A.C.  These provisions are applicable to any source, including, but not limited to, vehicular movement, transportation of materials, construction, alterations, demolition or wrecking, or industrial related activities such as loading, unloading, storing and handling.  Reasonable precautions shall include, but not be limited to, the following:

[Rules 62-4.070(3) and 62-296.320, F.A.C.]

- Vehicle speeds will be limited to 10 mph in unpaved areas and 15 mph in paved areas of the facility.

- Wet roadways and unpaved areas as necessary to prevent excessive fugitive emissions.  Paved areas shall be kept free of dirt and or debris by sweeping or vacuuming (no blowers permitted).  If paved areas are not kept free of dirt and debris, they shall be maintained wet to the extent necessary to prevent excessive fugitive emissions.

- Equipment shall be properly maintained to perform its designed function.

- Maintain piles wet to the extent necessary to operate in compliance with the applicable visible emission standard.

- Operator of the crane, bobcats and front end loaders used to move scrap in the site must have a clear, unobstructed view of the area of hazardous operations at all times.

- In order to ensure compliance with Specific Condition No. 9, the permittee shall perform visible emission (VE) tests for EU 005 including all the emission points (EP 1 thru EP 11) listed in this permit, EU 006 and EU 007, within 90 days of receipt of this final permit, and annually thereafter, between October 1 and September 30 (once per federal fiscal year).  Testing procedures shall be consistent with the requirements of Rule 62-297.340, F.A.C. [Rules 62-296.711(3), and 62-297.310(7), F.A.C.]

- All Visible emissions shall be tested by a certified observer in accordance with EPA Method 9 for a minimum of thirty (30) minutes and shall be taken at the point of highest opacity.

[Rules 62-297.310(4)(a)2, and (7)(a), and 62-297.320, F.A.C.]

- The permittee shall notify the Air Compliance Section of the Environmental Protection Commission of Hillsborough County at least 15 days prior to the date on which each formal compliance test is to begin.  The notification will include the date, time, and place of each such test, and the contact person who will be responsible for coordinating and having such test conducted. [Rule 62-297.310(7)(a)9., F.A.C]

- The permittee will submit two copies of the compliance test report to the Air Management Division of the Environmental Protection Commission of Hillsborough County within 45 days of such testing.  The test report shall contain sufficient detail on the source tested and the test procedures used to allow the EPC to determine if the test was properly conducted and the test results properly computed.  The operational data shall include the truck unloading and loading rates as determined by the weight of material processed during the visible emission test observation period and the specific work practices utilized to control particulate emissions. [Rules 62-297.310(8), F.A.C.]

- The permittee shall provide timely notification to the Environmental Protection Commission of Hillsborough County prior to implementing any changes that may result in a modification to this permit pursuant to Rule 62-210.200, F.A.C., Modification.  The changes do not include normal maintenance, but may include, and are not limited to, the following, and may also require prior authorization before implementation: [Rules 62-210.300 and 62-4.070(3), F.A.C.]

  - Alteration or replacement of any equipment* or major component of such equipment;

  - Installation or addition of any equipment* which is a source of air pollution.

*Not applicable to routine maintenance, repair, or replacement of component parts of an air emissions unit.

- When the Environmental Protection Commission of Hillsborough County (EPC) after investigation, has good reason (such as complaints, increased visible emissions or questionable maintenance of control equipment) to believe that any applicable emission standard contained in Rules 62-204, 62-210, 62-212, 62-296, or 62-297, F.A.C., or in a permit issued pursuant to those rules is being violated, it may require the owner or operator of the source to conduct compliance tests which identify the nature and quantity of pollutant emissions from the source and to provide a report on the results of said tests to the EPC.

[Rule 62-297.310(7)(b), F.A.C.]

- In order to demonstrate continuous compliance with Specific Conditions 6, 7 and 8, the permittee shall maintain a recordkeeping system for the most recent three-year period.  The records shall be made available to the EPC upon request.  The records shall include, but not limited to, the following:

[Rule 62-4.070(3) and 62-4.160(14)(b), F.A.C.]

- Day, month and year of the data recorded;

- Monthly and 12-month rolling total of aluminum scrap received and processed (tons);

- Monthly and 12-month rolling total of diesel fuel usages (consumed) in gallons;

- Record of the annual certification of the truck scale;

- The permittee must submit to the Environmental Protection Commission of Hillsborough County each calendar year, a completed DEP Form 62-210.900(5), "Annual Operating Report (AOR) for Air Pollutant Emitting Facility", for the preceding calendar year.  The AOR shall be submitted by April 1 of the following year.  [Rule 62-210.370(3), F.A.C.]

- If the permittee wishes to transfer this permit to another owner, an "Application for Transfer of Permit" (DEP Form 62-210.900(7)) shall be submitted, in duplicate, to the Environmental Protection Commission of Hillsborough County within 30 days after the sale or legal transfer of the permitted facility. [Rule 62-4.120, F.A.C.]

22.	A minimum of two copies of an operating permit application with applicable compliance test report for incorporating this construction Permit No. 0571273-004-AC into an operating permit shall be submitted to the Environmental Protection Commission of Hillsborough County within 60 days of completion of testing or 90 days before the expiration date of this permit, whichever comes first.

[Rules 62-4.070(3) and 62-210.300(2), F.A.C.]

ENVIRONMENTAL PROTECTION COMMISSION

OF HILLSBOROUGH COUNTY

_________________________________

Richard D. Garrity, Ph.D.

Executive Director

P.E. Certification Statement

Permittee:

Mutual Scrap Processing, Inc.				DRAFT Permit No.:  0571273-004-AC

Tampa, Florida						Facility ID No.:  0571273

Project Type:  Air Construction

I HEREBY CERTIFY that the engineering features described in the above referenced application and subject to the proposed permit conditions provide reasonable assurance of compliance with applicable provisions of Chapter 403, Florida Statutes, and Florida Administrative Code Chapters 62-4 and 62-204 through 62-297.  However, I have not evaluated and I do not certify aspects of the proposal outside of my area of expertise (including but not limited to the electrical, mechanical, structural, hydrological, and geological features).

(Seal)

__________________________  _______________

Diana M. Lee, P.E.                       Date

Registration No.: 56082

Permitting Authority:

Environmental Protection Commission

of Hillsborough County

Air Management Division

3629 Queen Palm Dr

Tampa, FL  33619

Telephone:  (813) 2627-2600

Fax:  (813) 627-2660