M E M O R A N D U M

DATE:	February 29, 2012

TO:	Richard D. Garrity, Ph.D.

FROM:	Jeff Sims	Thru:	Stephen R. Hathaway, P.E.

Sterlin K. Woodard, P.E.

SUBJECT:	Initial Operation Permit No. 0571342-003-AO

Blacklidge Emulsions, Inc. – Plant #11

Attached is initial operating Permit No. 0571342-003-AO being issued to Blacklidge Emulsions, Inc. for the operation of an asphalt blending facility.  The facility is located at 5010 Montgomery Street, Tampa, FL  33619.

Blacklidge Emulsions, Inc., Tampa Plant #11 currently manufactures asphalt products used in the paving industry.  The facility receives liquid asphalt cement and stores the material in heated tanks.  Ground Tire Rubber (GTR), other aggregate materials, and flux oil are mixed with the asphalt cement in blending tanks and the finished asphalt products are directly loaded out to trucks or transferred to other heated storage tanks.  The heat is provided by diesel-fired burners on the tanks and a central hot oil heater.

This operating permit incorporates construction Permit Nos. 0571342-002-AC and 0571342-004-AC.  Permit Nos. 0571342-002-AC authorized the construction of 7 storage tanks, 8 associated burners, and the Ground Tire Rubber (GTR) receiving, storage, and mixing operation.  However, the GTR silos were never constructed and the application indicated that they are no longer part of the facility plans and should not be included in the operating permit.  GTR is still used in the operation, but it is only loaded into hoppers from supersacks and transferred via enclosed conveyor to the top of the mixing tanks.  The loading of GTR into the hoppers and its transfer up and into the tanks were determined to be exempt from permitting due to the low level of emissions pursuant to Rule 62-210.300(3)(b), F.A.C., and is therefore no longer considered a regulated emission unit.

Construction Permit No. 0571342-002-AC also identified the performance of a temporary trial study using a modified asphalt blend that was exempt from permitting based on a limitation of hours of operation.  Tank #7 (and ultimately Tank #8) were added to support this test trial and were originally intended to be removed following the study.  However, following completion of the initial stages of temporary trial, the facility realized that the other party involved may not likely resume the trials soon, if ever.  Therefore, the facility elected to keep the tanks permanently on-site and utilize them as part of their normal operation, primarily for asphalt product storage.  Permit No. 0571342-004-AC was processed to recognize the permanent after-the-fact addition of Tanks #7 and #8 along with the heater dedicated to providing heat for Tank #8.  Tank #7 is currently used to store flux oil at ambient temperatures as an additive to the asphalt mixtures, but may also store asphalt.  A steam boiler was added to provide heat for various parts of the facility during the trial, including Tank #7.  However, the application indicated that the facility plans to remove the boiler and expand the existing hot oil heater loop to provide the necessary heat for Tank #7.  The boiler was determined to be exempt from permitting pursuant to Rule 62-210.300(3)(a)34, F.A.C.

Construction Permit No. 0571342-004-AC is being issued concurrently with this initial operating permit, which will incorporate the additions from the construction permit since all initial testing relating to Tanks #7 and #8 have already been performed.  Visible emission testing was performed on Tank #7 on December 27, 2010 and on Tank #8 on December 19, 2011, and both were found to be in compliance.

Each storage tank, except Tank #7, has at least one diesel-fired burner to help maintain the temperature of the tank contents.  However, based on information provided by the facility’s consultant, the previously stated ratings are for nominal heat output and don’t necessarily reflect the true maximum fuel combustion capacity from the burners.  Therefore, the burner ratings were changed in the permit from 0.68 MMBtu/hr to 0.8 MMBtu/hr for the tank burners and from 1.35 MMBtu/hr to 2.2 MMBtu/hr for the hot oil heater in order to reflect the maximum heat input for each burner.  The potential emissions were recalculated based on a maximum fuel usage of 5.5 gal/hr for each of the tank burners and 15.7 gal/hr for the hot oil heater to better reflect the maximum emissions from fuel combustion.

Based on our review, we recommend issuance of the permit as drafted.

JDS: 0571342-003-AO

ENVIRONMENTAL PROTECTION COMMISSION OF

HILLSBOROUGH COUNTY, as Delegated by

STATE OF FLORIDA

DEPARTMENT OF ENVIRONMENTAL PROTECTION

NOTICE OF PERMIT

Mr. Danny Sison, Plant Manager

Blacklidge Emulsions, Inc. – Tampa Plant #11

5010 Montgomery Street

Tampa, Florida 33619

Re:	Hillsborough County – Initial Operation Permit No. 0571342-003-AO

Dear Mr. Sison:

Enclosed is Permit Number 0571342-003-AO for the operation of an asphalt blending facility located at 5010 Montgomery Street, Tampa, Florida, 33619, issued pursuant to Section 403.087, Florida Statutes.  This initial operating permit incorporates Construction Permit Nos. 0571342-002-AC and 0571342-004-AC.

Any party to this order (permit) has the right to seek judicial review of the permit pursuant to Section 120.68, Florida Statutes, by the filing of a Notice of Appeal pursuant to Rule 9.110, Florida Rules of Appellate Procedure, with the Clerk of the EPC in the Legal Department at 3629 Queen Palm Drive, Tampa, Florida, 33619 and by filing a copy of the Notice of Appeal accompanied by the applicable filing fees with the appropriate District Court of Appeal.  The Notice of Appeal must be filed within 30 days from the date this Notice is filed with the clerk of the EPC.

Executed in Tampa, Florida.

Sincerely,

Richard D. Garrity, Ph.D.

Executive Director

cc:  	Florida Department of Environmental Protection (via e-mail)

Dean H. Myers., P.E., General Environmental Engineering, Inc. (via e-mail)

CERTIFICATE OF SERVICE

This is to certify that this NOTICE OF PERMIT and all copies were mailed before the close of business on _________________________ to the listed persons.

Clerk Stamp

FILED, on this date, pursuant to Section 120.52(7), Florida Statutes, with the designated clerk, receipt of which is hereby acknowledged.

___________________________  _____________

Clerk                  		            Date

PERMITTEE:	PERMIT/CERTIFICATION

Blacklidge Emulsions, Inc. – Tampa Plant #11	Permit No.: 0571342-003-AO

5010 Montgomery Street	County: Hillsborough

Tampa, Florida 33619	Expiration Date:  February 28, 2017

Project: Asphalt Blending Facility

This permit is issued under the provisions of Chapter 403, Florida Statutes, and Florida Administrative Code Rules 62-204, 62-210, 62-212, 62-296, 62-297, and 62-4.  The above named permittee is hereby authorized to perform the work or operate the facility shown on the application and approved drawing(s), plans, and other documents, attached hereto or on file with the EPC and made a part hereof and specifically described as follows:

- This air permit is for the operation of an asphalt blending facility.  Blacklidge Emulsions, Inc., Tampa Plant #11 manufactures asphalt products used in the paving industry.  The facility receives liquid asphalt cement, GTR, and other aggregate materials by tank truck and in 2,100 lb super sacks.  GTR from the super sacks is loaded into dedicated hoppers, via forklift, and screw-conveyed into a corresponding mixing tank.  The liquid asphalt cement is stored in storage tanks and the desired additives are added and mixed with the asphalt in blending tanks.

- From the heated blending tanks, the finished product is loaded into tanker trucks by a loading rack as finished asphalt product or transferred into a storage tank.  The truck loading operations, blending tanks, asphalt cement storage tanks, and each compartment of the horizontal paving asphalt reclaiming tank (Tank #4) are heated by a central, 2.2 MMBTU/hr, hot oil system manufactured by GENCOR, Model No. C2-GO-15.  Each tank, except Tank #7, also uses a diesel-fired burner with a maximum heat input of 0.8 MMBTU/hr manufactured by R.W. Beckett Corporation (Model SF Oil Burner) to maintain operating temperatures from ~200 °F, however, not to exceed 375 °F.  The burner ratings were increased slightly from the values reflected in previous construction permits in order to reflect the maximum heat input for each burner and maximum associated fuel usage.  Tank #4 has two of the described burners.  The facility receives off-spec paving asphalt and stores it in a duel-compartment, horizontal “cook-off” tank (Tank #4), which is heated to remove moisture and reclaim the paving asphalt.

- Tank #7 can be used to store flux oil at ambient temperatures as an additive to the asphalt mixtures, but may also store asphalt.  Tank #7 can be heated from a steam boiler previously added to provide heat for various parts of the facility, or may be heated through use of the central hot oil system.  The boiler was determined to be exempt from permitting pursuant to Rule 62-210.300(3)(a)34., F.A.C.

- Particulate Matter (PM) and Volatile Organic Compounds (VOC) emissions from the heated storage and blending tanks, along with the truck loading rack, are uncontrolled.  However, the vents from the mixing tanks are routed down to the ground and through a quench drum (barrel) to help reduce emissions and odors where practical.  PM emissions from the screw-conveyors are controlled by covers, while PM emissions from GTR loading into the hoppers are controlled by the use of reasonable precautions.  The loading of GTR into the hoppers and its transfer up and into the tanks were determined to be exempt from permitting due to the low level of emissions pursuant to Rule 62-210.300(3)(b), F.A.C., based on a maximum anticipated usage of 4,800 tons/yr.  Silos previously planned for construction to store GTR were never built and, therefore, were not included in this operation permit.

Facility Information Summary

Location:  5010 Montgomery Street, Tampa, Hillsborough County

UTM Coordinates:		17- 362.4 E, 3087.5 N

Latitude:	27° 54' 21" N	Longitude:	82° 23' 55" W

Facility ID No.:		0571342

| Emission Unit ID | Description | Capacity |
| --- | --- | --- |
| 002 | Tanks and Truck Loading as follows: GTR Asphalt Blending Tank (T1) GTR Asphalt Blending Tank (T2) GTR Asphalt Blending Tank (T3) Horizontal Asphalt “Cook-off” Tank (T4) Asphalt Cement Storage Tank (H5) Asphalt Cement Storage Tank (H6) Asphalt Cement Storage Tank (T7) Asphalt Cement Storage Tank (T8) Tank Truck Loading | 10,000 gallons 10,000 gallons 10,000 gallons 20,000 gallons 20,000 gallons 20,000 gallons 10,000 gallons 20,000 gallons Single-arm, top-loading |
| 003 | Diesel-Oil Heaters:* GENCOR Hot Oil Heater GTR Asphalt Blending Tank (T1) GTR Asphalt Blending Tank (T2) GTR Asphalt Blending Tank (T3) Horizontal Asphalt “Cook-off” Tank (T4) Asphalt Cement Storage Tank (H5) Asphalt Cement Storage Tank (H6) Asphalt Cement Storage Tank (T8) | (maximum heat input) 2.2 MMBtu/hr 0.8 MMBtu/hr 0.8 MMBtu/hr 0.8 MMBtu/hr 0.8 MMBtu/hr (x2) 0.8 MMBtu/hr 0.8 MMBtu/hr 0.8 MMBtu/hr |

* - If necessary, Tank #7 is heated by the exempt boiler or by the central hot oil heater.

- General Conditions.  A part of this permit is the attached General Conditions.  [Rule 62-4.160, F.A.C.]

- Applicable Requirements.  Issuance of this permit does not relieve the permittee from complying with applicable emission limiting standards or other requirements of Chapters 62-204, 62-210, 62-212, 62-296 and 62-297, F.A.C., or any other requirements under federal, state, or local law.  [Rule 62-210.300, F.A.C.]

- Rules of the EPC.  All applicable rules of the Environmental Protection Commission of Hillsborough County including design discharge limitations specified in the application shall be adhered to.  The permit holder may also need to comply with county, municipal, federal, or other state regulations prior to construction.  [Rule 62-4.070(7), F.A.C.]

- Chapter 84-446, Laws of Florida.  The use of property, facilities, equipment, processes, products, or compounds, or any other act that causes or materially contributes to a public nuisance is prohibited, pursuant to the Hillsborough County Environmental Protection Act, Section 16, Chapter 84-446, Laws of Florida, as Amended.

Operation and Emission Limitations

- Hours of Operation.  The facility is permitted to operate continuously; 8,760 hours per 12-consecutive month period, except for the fuel oil heaters (EU003) which are limited to 2,080 hours per 12-consecutive month period.  [Rule 62-210.200 (Potential to Emit), F.A.C; and Permit Nos. 0571342-002-AC and 0571342-004-AC]

- Production Limitation.  In order to establish this facility as a Synthetic Non-Title V Source, and as requested by the permittee, the facility shall not produce more than 80,000 tons of GTR paving asphalt product in any 12-consecutive month period.  [Rule 62-4.070(3), F.A.C.; and Permit Nos. 0571342-002-AC and 0571342-004-AC]

- Operational Limitations.  In order to help ensure proper operation of the units and establish this facility as a Synthetic Non-Title V Source, the facility operations shall be limited as follows:

  - The temperature of the asphalt binder storage tanks and the GTR paving asphalt blending tanks shall not exceed 375 °F;

  - The Fuel Oil Heaters (EU003) shall only be fired with No. 2 fuel oil containing a maximum sulfur content of 0.5% by weight.  The heat input rates for the hot oil heater and the individual tank heaters shall not exceed 2.2 MMBtu/hr and 0.8 MMBtu/hr, respectively.

[Rules 62-4.070(3), F.A.C.; and Permit Nos. 0571342-002-AC and 0571342-004-AC]

- Visible Emissions (VE).  Visible emissions from the Tanks & Truck Loading activities (EU002) and Fuel Oil Heaters (EU003) shall not be equal to or greater than 20% opacity.  [Rule 62-296.320, F.A.C.; and Chapter 1-3.52, Rules of the Environmental Protection Commission of Hillsborough County (EPC)]

- Objectionable Odor.  The permittee shall not cause, suffer, allow or permit the discharge of air pollutants which cause or contribute to an objectionable odor.  An objectionable odor is any odor present in the outdoor atmosphere which by itself or in combination with other odors, is or may be harmful or injurious to human health or welfare, which unreasonably interferes with the comfortable use and enjoyment of life or property, or which creates a nuisance.  During tank loading/loadout and tank to tank/blending operations, if objectionable odors are noted, the operation(s) shall be curtailed or halted.  If the Environmental Protection Commission of Hillsborough County receives odor complaints attributed to this facility and they are determined to be a nuisance, the permittee will need to evaluate their operations and propose corrective actions that may include installing additional odor control measures for this facility.  [Rules 62-4.070(3) and 62-296.320(2), F.A.C.; and Chapter 1-3.22.3., Rules of the EPC]

- Control of Unconfined PM.  All reasonable precautions shall be taken to prevent and control generation of unconfined emissions of particulate matter in accordance with the provision in Rule 62-296.320, F.A.C.  These provisions are applicable to any source, including, but not limited to, vehicular movement, transportation of materials, construction, alterations, demolition or wrecking, or industrial related activities such as loading, unloading, storing and handling.  Reasonable precautions shall include, but not be limited to, the following:

  - Vehicle speeds will be limited to 10 mph in unpaved areas and 15 mph in paved areas of the facility.

  - Wet roadways and unpaved areas as necessary to prevent excessive fugitive emissions.  Paved areas shall be kept free of dirt and or debris by sweeping or vacuuming (no blowers permitted).  If paved areas are not kept free of dirt and debris, they shall be maintained wet to the extent necessary to prevent excessive fugitive emissions.

[Rules 62-4.070(3) and 62-296.320, F.A.C.]

- Open Burning.  Open burning is prohibited unless it is specifically authorized by a separate open burn permit issued by the EPC pursuant to Rule 62-296.320(3), F.A.C.

Compliance Testing

- Annual Compliance Testing.  Visible emissions tests for each emission unit shall be performed annually during each federal fiscal year (October 1 through September 30), with a target date of September 28th.  [Rule 62-297.310(7)(a), F.A.C.; and Chapter 1-3.52.3., Rules of the EPC]

- Testing Requirements.  Visible emissions testing for opacity shall be performed by a certified observer in accordance with EPA Method 9 for a minimum of thirty (30) minutes and shall include the period during which the highest opacity emissions can reasonably be expected to occur.  Testing of emissions shall be conducted at 90-100% of the maximum permitted process rates as stated in Specific Condition No. 7.  If it is impracticable to test at capacity, then the source may be tested at less than capacity; in this case subsequent source operation is limited to 110% of the test load until a new test is conducted.  Once the unit is so limited, then operation at higher capacities is allowed for no more than fifteen days for purposes of additional compliance testing to regain the rated capacity in the permit, with prior notification to the EPC.  Failure to submit the input rates and actual operating conditions may invalidate the tests.  [Rules 62-297.310(2)(b), 62-297.310(4)(a)2, F.A.C.; and Chapter 1-3.52.3., Rules of the EPC]

- Compliance Test Notification.  The permittee shall notify the Air Compliance Section of the EPC at least 15 days prior to the date on which each formal compliance test is to begin, of the date, time, and place of each such test, and the test contact person who will be responsible for coordinating and having such test conducted.  [Rule 62-297.310(7)(a)(9), F.A.C.]

- Test Report Submittal.  The permittee will submit two (2) copies of the compliance test reports required by this permit to the Air Compliance Section of the EPC within forty-five (45) days of completion of compliance testing.  The test report shall contain sufficient detail on the source tested and the test procedures used to allow the EPC to determine if the test was properly conducted and the test results properly computed.  [Rule 62-297.310(8), F.A.C.]

- Special Compliance Testing.  When the Environmental Protection Commission of Hillsborough County (EPC) after investigation, has good reason (such as complaints, increased visible emissions, or questionable maintenance of control equipment) to believe that any applicable emission standard contained in Rules 62-204, 62-210, 62-212, 62-296, or 62-297, F.A.C., or in a permit issued pursuant to those rules is being violated, it shall require the owner or operator of the source to conduct compliance tests which identify the nature and quantity of pollutant emissions from the source and to provide a report on the results of said tests to the EPC. [Rule 62-297.310(7)(b), F.A.C.]

Recordkeeping and Reporting Requirements

- Records.  In order to demonstrate compliance with Specific Condition Nos. 5, 6 and 7, the permittee shall maintain records of the GTR paving asphalt processing operations.  The facility records shall include, but not limited to, the following:  [Rules 62-4.070(3), and 62-4.160(14), F.A.C.]

  - Month, and Year of the data recorded;

  - The total weight of paving asphalt produced, in short or metric tons (specified), on a monthly basis and the rolling total for the previous 12-consecutive month period;

  - The total hours of operation of the fuel oil heaters, on a monthly basis and the rolling total for the previous 12-consecutive month period;

  - The total amount of diesel fuel consumed, in gallons on a monthly basis and the rolling total for the previous 12-consecutive month period.  Records of fuel oil sulfur content shall also be maintained.

- Records Retention.  All records shall be completed by the end of the following month, shall be maintained onsite and made available to the Environmental Protection Commission of Hillsborough County, state or federal air pollution agency upon request, and kept on file for at least three years from the date of measurement.  [Rules 62-4.070(3), and 62-4.160(14)(b), F.A.C.]

- Annual Reporting.  The permittee shall submit to the Environmental Protection Commission of Hillsborough County, by April 1st of each calendar year, a completed DEP Form 62-210.900(5), "Annual Operating Report (AOR) for Air Pollutant Emitting Facility", for the preceding calendar year.  [Rule 62-210.370(3), F.A.C.]

Permit Applications and Transfers

- Operation Permit Application.  Prior to sixty days before the expiration of this operating permit, the permittee shall apply for a renewal of the permit using the current version of the permit renewal application form.  A renewal application shall be timely and sufficient.  If the application is submitted prior to sixty days before the expiration of the permit, it will be considered timely and sufficient.  If the renewal application is submitted at a later date, it will not be considered timely and sufficient unless it is submitted and made complete prior to the expiration of the operation permit.  When the application for renewal is timely and sufficient, the existing permit shall remain in effect until the renewal application has been finally acted upon by the EPC or, if there is court review of the final agency action, until a later date is required by Section 120.60, Florida Statutes. [Rules 62-4.090, 62-210.300(2), and 62-210.900, F.A.C.]

- Transfer of Ownership.  If the permittee wishes to transfer this permit to another owner, an "Application for Transfer of Permit" (DEP Form 62-210.900(7)) shall be submitted, in duplicate, to the Environmental Protection Commission of Hillsborough County within 30 days after the sale or legal transfer of the permitted facility.  [Rule 62-4.120, F.A.C.]

- Modifications.  The permittee shall provide timely notification to the Environmental Protection Commission of Hillsborough County prior to implementing any changes that may result in a modification to this permit pursuant to Rule 62-210.200, F.A.C., Modification.  The changes do not include normal maintenance, but may include, and are not limited to, the following, and may also require prior authorization before implementation:

- Alteration or replacement of any equipment or major component of such equipment;

- Installation or addition of any equipment which is a source of air pollution;

Note:	Items A and B are not applicable to routine maintenance, repair, or replacement of component parts of an air emissions unit.  [Rules 62-210.300 and 62-4.070(3), F.A.C.]

ENVIRONMENTAL PROTECTION COMMISSION

OF HILLSBOROUGH COUNTY

_________________________________

Richard D. Garrity, Ph.D.

Executive Director