M E M O R A N D U M

DATE:	July 21, 2014

TO:	Richard D. Garrity, Ph.D.

FROM:	Lora Webb		THRU:  Diana M. Lee, P.E.

Sterlin K. Woodard, P.E.

SUBJECT:	Draft Construction Permit – Tampa Marine Terminals, LLC

Permit No. 0571418-007-AC

Attached is Draft Construction Permit No. 0571418-007-AC for Tampa Marine Terminals, LLC a bulk material handling facility located at 1650 Hemlock Street, Tampa, FL.  This permit authorizes the facility to handle prilled sulfur in addition to the materials the facility is currently authorized to handle.

Material is received by either truck or ship.  Material received by ship is clamshelled into two on-shore hoppers.  The material from each hopper is gravity fed into open bed trucks. The trucks drive the material to the northeast portion of the property and dump the bulk materials into storage piles.  Alternatively, the trucks may drive directly to off-site customers.  Material received by truck is unloaded directly onto the storage piles.  When ready to be shipped offsite, front-end loaders load the material from the storage piles into trucks.  Prilled sulfur will be stored on an impervious surface and all contact water will be directed to the water treatment facility adjacent to the site.

In addition to shipping the material offsite by truck, material can be loaded into ships.  Front-end loaders load the material from the storage piles into trucks.  The trucks drive the material to a storage pile closer to the dock.  From the dock, the material is transferred into a ship using the ship’s loading equipment.

Water sprays on the on-shore hoppers, truck loadouts, and storage pile areas are utilized to minimize unconfined PM emissions. In addition, the on-shore hoppers are equipped with wind screens to minimize unconfined PM emissions from the ship unloading operation.  Also, EPC expressed concerns about the flammability of the prilled sulfur material.  According to Tampa Marine Terminals, they discussed this concern with the City of Tampa Fire Marshal, who stated that the material has a low potential for flammability.  However, the material is combustible when exposed to heat or flame.

Therefore, as a precaution, the permit requires fire hydrants will be located near the piles which will be used in the unlikely event of a fire.

As requested by the permittee, the permit includes a limit on the PM PTE, but does not include a limit on the annual throughput.  In order to demonstrate compliance with the PTE limit, the permittee is required to maintain daily records detailing the type of material handled, material throughputs, and calculate their PM emissions on a monthly basis using the approved emission factors for each material.

The facility is subject to Rule 62-296.711, F.A.C. Materials Handling, Sizing, Screening, Crushing and Grinding Operations and Chapter 1-3.52, Rules of the EPC, which limit the visible emissions to a 5% opacity standard for all of the material handling operations.

In addition to the above operation, Permit No. 0571418-003-AC, which expires on November 30, 2014, authorizes the construction of a conveyor system that will be used to unload and load ships.  The conveying system has not been constructed at this time.

Based on our review, we recommend issuance of the permit as drafted.

LAW: 0571418-007-AC

TECHNICAL EVALUATION

AND

PRELIMINARY DETERMINATION

FOR

Tampa Marine Terminals, LLC

Hillsborough County

Construction Permit

Application Number

0571418-007-AC

Environmental Protection Commission of

Hillsborough County

Tampa, FL

July 21, 2014

I.  Project Description

A.  Applicant:

Billy J. Roy

Operations Manager

Tampa Marine Terminals, LLC

1650 Hemlock Street

Tampa, FL 33605

B.  Engineer:

Tom John, P.E.

P.E. No.:  33157

Tom John, P.E., Inc.

6250 Cape Hatteras Way No. 2

St. Petersburg, FL 33702

- Project and Location:

This permit authorizes the facility to handle prilled sulfur in addition to the other materials that the facility is currently authorized to handle.

The project has been assigned NEDS Source Classification Code Nos. 3-05-104-99 for Bulk Material Unloading Operation and 3-05-105-99 for Bulk Material Loading Operation.  The Standard Industrial Code for the project is No. 32 for Stone, Clay, Glass, and Concrete Products.  The project is located at 1650 Hemlock Street, Tampa, Hillsborough County, FL, 33605.  UTM Coordinates of the location are 17- 358.15 E and 3091.20 N.

- Process and Controls:

Tampa Marine Terminals, LLC is a bulk material handling facility which receives bulk materials by either truck or ship.  Material received by ship is clamshelled into two on-shore hoppers.  The material from each hopper is gravity fed into open bed trucks. The trucks drive the material to the northeast portion of the property and dump the bulk materials into storage piles.  Alternatively, the trucks may drive directly to off-site customers.  Material received by truck is unloaded directly onto the storage piles.  When ready to be shipped offsite, front-end loaders load the material from the storage piles into trucks.

In addition to shipping the material offsite by truck, material can be loaded into ships.  Front-end loaders load the material from the storage piles into trucks.  The trucks drive the material to a storage pile closer to the dock.  From the dock, the material is transferred into a ship using the ship’s loading equipment.

Water sprays on the on-shore hoppers, truck loadouts, and storage pile areas are utilized to minimize unconfined PM emissions. In addition, the on-shore hoppers are equipped with

wind screens to minimize unconfined PM emissions from the ship unloading operation.  Also, fire hydrants are located near the piles which will be used in the unlikely event of a fire.

In order to provide operational flexibility, the facility requested that the permit not include a limit on the annual material throughput.  The facility suggested calculating emissions on a monthly basis using the approved emission factor for each material to demonstrate compliance with the annual PM PTE.  Each type of material handled at the facility has a different emission factor.  The facility may be able to handle a larger amount of one material and still maintain compliance with the PM PTE.  Therefore, in order to provide the facility maximum flexibility, the permit includes a limit on the PM PTE but does not include a limit on the annual throughput.  In addition, in order to provide reasonable assurance that the PTE will not be exceeded, the facility will be required to maintain a daily tracking system, which shall include records detailing the type of material handled, material throughputs, and emission calculations.

As requested by the permittee, prilled sulfur is included in the material group number V – Non-Fertilizer Products.  The materials in the group include coal, petcoke, coke, syngypsum, iron ore, clay, and magnetite/ferrous oxides.  Prilled sulfur will be stored on an impervious surface and all contact water will be directed to the water treatment facility adjacent to the site.

The facility is subject to Rule 62-296.711, F.A.C. Materials Handling, Sizing, Screening, Crushing and Grinding Operations and Chapter 1-3.52, Rules of the EPC, which limit the visible emissions to a 5% opacity standard for all of the material handling operations.

E.  Application Information:

Received on:  May 8, 2014

Information Requested:  June 5, 2014

Application Complete:  July 3, 2016

II.  Rule Applicability

This project is subject to the preconstruction review requirements of Chapter 403, Florida Statutes, Chapters, 62-204, 62-210, 62-212, 62-296, and 62-297, Florida Administrative Code (F.A.C.) and Chapter 1-3 of the Rules of the Environmental Protection Commission of Hillsborough County.

This project is subject to the requirements of Rule 62-212.300, General Preconstruction Review Requirements, F.A.C., since the project is not exempt from the permit requirements in Rule 62-210.300, F.A.C.

This project is not subject to the requirements of Rule 62-212.400, Prevention of Significant Deterioration, F.A.C. or Rule 62-212.500, New Source Review for Nonattainment Areas, F.A.C., since the facility is a Synthetic Non-Title V Source by state definition.

This project is not subject to the requirements of Rule 62-213, Operation Permits for Major Sources of Air Pollution, F.A.C., since the facility is a Synthetic Non-Title V Source by state definition.

This project is subject to the requirements of Rule 62-296.320, General Pollutant Emission Limiting Standards, F.A.C., since the project is a potential source of particulate matter (PM) and visible emissions.

This project is not subject to the requirements of Rule 62-296.401, through 62-296.470, Specific Emission Limiting and Performance Standards, F.A.C., since there is not an applicable source specific category in this rule.

This project is not subject to the requirements of Rule 62-296.500, Volatile Organic Compounds and Nitrogen Oxides Reasonably Available Control Technology, F.A.C., since there is not an applicable source specific category in this rule.

This project is not subject to the requirements of Rule 62-296.600, Reasonably Available Control Technology - Lead, F.A.C., since there is not an applicable source specific category in this rule.

This project is subject to the requirements of Rule 62-296.700, Reasonably Available Control Technology – Particulate Matter, F.A.C., since there is an applicable source specific category in this rule, specifically Rule 62-296.711, F.A.C. – Materials Handling, Sizing, Screening, Crushing and Grinding Operations.

This project is not subject to the requirements of Rule 62-204.800, Federal Regulations Adopted by Reference, F.A.C., since there is not an applicable source specific category in this rule.

This project is subject to the requirements of Chapter 84-446, Laws of Florida and Chapter 1-3, Rules of the Environmental Protection Commission of Hillsborough County.

III. Summary of Particulate Matter Emissions

| Emission Unit (EU) No. | EU Description | Actual PM Emissions (tons/year) | Potential PM Emissions (tons/yr) | Increase in Emissions | Allowable Visible Emissions |
| --- | --- | --- | --- | --- | --- |
| 001 | Ship Unloading to Onshore Hopper | 0.6 | 3.5 | 2.9 | 5% opacity |
| 002 | Hopper to Ship Unloading Conveyor 1 | 0 | 3.5 | 3.5 | 5% opacity |
| 003 | Ship Unloading Conveyor 1 to Radial Stacker | 0 | 3.5 | 3.5 | 5% opacity |
| 004 | Radial Stacker to Pile | 0 | 3.5 | 3.5 | 5% opacity |
| 005 | Onshore Hopper to Truck | 0.6 | 3.5 | 2.9 | 5% opacity |
| 006 | Truck to Pile | 0.6 | 3.5 | 2.9 | 5% opacity |
| 007 | Pile to Pile Movement or Movement of Material within Storage Pile | 0 | 3.5 | 3.5 | 5% opacity |
| 008 | Front-end Loader to Truck | 0.6 | 3.5 | 2.9 | 5% opacity |
| 009 | Truck to Ship Loading Pile | 0 | 3.5 | 3.5 | 5% opacity |
| 010 | Front-end Loader to Ship Loading Conveyor 3 | 0 | 3.5 | 3.5 | 5% opacity |
| 011 | Ship Loading Conveyor 3 to Ship Loading Pile | 0 | 3.5 | 3.5 | 5% opacity |
| 012 | Ship Equipment to Ship | 0 | 3.5 | 3.5 | 5% opacity |
| 013 | Front-end Loader to Ship Loading Conveyor 2 | 0 | 3.5 | 3.5 | 5% opacity |
| 014 | Ship Loading Conveyor 2 to Ship | 0 | 3.5 | 3.5 | 5% opacity |
|  | Total Facility-wide PM Emissions | 2.3 | 49 | 46.6 |  |

- Actual emissions are based on the average of 2012 and 2013 AOR data.

- Potential PM emissions are calculated by using a PM emission factor based on the worst emitting material from each of the eleven group categories, a material throughput, and a maximum of nine transfer points (worst case scenario).  A 70% control efficiency is applied for transfer points that use of a water spray system.

- The facility-wide emissions are limited to 49 tons/year of PM.  The emission units are not limited to a PTE.  However, this table shows a breakdown of the facility-wide emissions by emission unit for annual emissions reporting purposes.

IV.  Conclusions:

The emission limits proposed by the applicant will meet all of the requirements of Chapters 62-204, 62-210, 62-212, 62-296, and 62-297, F.A.C., and Chapter 1-3, Rules of the Commission.

The General and Specific Conditions listed in the proposed permit (attached) will assure compliance with all the applicable requirements of Chapters 62-204, 62-210, 62-212, 62-296, and 62-297, F.A.C.

V.   Proposed Agency Action:

Pursuant to Section 403.087, Florida Statutes and Rule 62-4.070, Florida Administrative Code the Environmental Protection Commission of Hillsborough County hereby gives notice of its intent to issue a permit to construct the aforementioned air pollution source in accordance with the draft permit and its conditions as stipulated (see attached).

CERTIFIED MAIL

In the Matter of an

Application for Permit by:

Billy J. Roy					File No.:  0571418-007-AC

Operations Manager				County:  Hillsborough

Tampa Marine Terminals, LLC

1650 Hemlock Street

Tampa, FL 33605

INTENT TO ISSUE

The Environmental Protection Commission of Hillsborough County (EPC), as delegated by the Florida Department of Environmental Protection (DEP) gives notice of its intent to issue a permit (copy attached) for the proposed project as detailed in the application specified above, for the reasons stated below.

The applicant, Tampa Marine Terminals, LLC, applied on May 8, 2014 to the EPC for a construction permit to authorize the handling of prilled sulfur in addition to the other materials that the facility is authorized to handle.  Particulate matter emissions from the handling operation will be controlled through the use of a water spray system.  The facility is located at 1650 Hemlock St., Tampa, Hillsborough County, FL, 33605.

The EPC has permitting jurisdiction under Chapter 403 Florida Statutes (F.S.) and Florida Administrative Code (F.A.C.) Chapters 62-4 and 62-210.  The project is not exempt from permitting procedures.  The EPC has determined that an air pollution construction permit is required to commence or continue operations at the described facility.

The EPC intends to issue this permit based on the belief that reasonable assurances have been provided to indicate that operation of the source will comply with the appropriate provisions of Florida Administrative Code (F.A.C.) Chapters 62-204 through 62-297 and 62-4.

Pursuant to Pursuant to Section 403.815 and 403.0872, F.S. and Rules 62-103.150 and 62-210.350(3), F.A.C., you (the applicant) are required to publish at your own expense the enclosed Notice of Intent to Issue Permit.  The notice shall be published one time as soon as possible, in the legal advertisement section of a newspaper of general circulation in

the area affected.  For the purpose of this rule, "publication in a newspaper of general circulation in the area affected" means publication in a newspaper meeting the requirements of Sections 50.011 and 50.031, F.S., in the county where the activity is to take place.  Where there is more than one newspaper of general circulation in the county, the newspaper used must be one with significant circulation in the area that may be affected by the permit.  If you are uncertain that a newspaper meets these requirements, please contact the EPC at the address or telephone number listed below.  The applicant shall provide proof of publication to the EPC, Air Permitting Section, at 3629 Queen Palm Drive, Tampa, Florida  33619 (Phone 813-627-2600 - FAX 813-627-2660) within 7 (seven) days of publication.  Failure to publish the notice and provide proof of publication within the allotted time may result in the denial of the permit pursuant to Rule 62-103.150(6), F.A.C.

The EPC will issue the final permit with the attached conditions unless a timely petition for an administrative hearing is filed pursuant to Section 120.569 and 120.57 F.S. before the deadline for filing a petition.  The procedures for petitioning for a hearing are set forth below.

A person whose substantial interests are affected by the proposed permitting decision may petition for an administrative proceeding (hearing) under Sections 120.569 and 120.57, F.S.  The petition must contain the information set forth below and must be filed (received) in the Legal Department of the EPC at 3629 Queen Palm Drive, Tampa, Florida 33619, Phone 813-627-2600, Fax 813-627-2602.  Petitions filed by the permit applicant or any of the parties listed below must be filed within 14 (fourteen) days of receipt of this notice of intent.  Petitions filed by any persons other than those entitled to written notice under Section 120.60(3), F.S. must be filed within 14 (fourteen) days of publication of the public notice or within 14 (fourteen) days of receipt of this notice of intent, whichever occurs first.  Under Section 120.60(3), however, any person who asked the EPC for notice of agency action may file a petition within 14 (fourteen) days of receipt of that notice, regardless of the date of publication.  A petitioner shall mail a copy of the petition to the applicant at the address indicated above, at the time of filing.  The failure of any person to file a petition within the appropriate time period shall constitute a waiver of that person's right to request an administrative determination (hearing) under Sections 120.569 and 120.57, F.S.; or to intervene in this proceeding and participate as a party to it.  Any subsequent intervention will be only at the approval of the presiding officer upon the filing of a motion in compliance with Rule 28-106.205, F.A.C.

A petition that disputes the material facts on which the EPC’s action is based must contain the following information:

- The name and address of each agency affected and each agency’s file or identification number if known;

- The name, address, and telephone number of the petitioner and the name, address, and telephone number of each petitioner’s representative, if any, which shall be the address for service purposes during the course of the proceedings; and an explanation of how the petitioner’s substantial interests will be affected by the EPC’s determination;

- A statement of how and when the petitioner received notice of the EPC action;

(d) A statement of all disputed issues of material fact.  If there are none, the petition must so indicate;

- A concise statement of the ultimate facts alleged, including the specific facts the petitioner contends warrant reversal or modification of the EPC’s proposed action;

- A statement of specific rules or statutes the petitioner contends requires reversal or modification of the EPC’s proposed action; and

- A statement of the relief sought by the petitioner, stating precisely the action petitioner wishes the EPC to take with respect to the EPC’s proposed action.

A petition that does not dispute the material facts upon which the EPC’s action is based shall state that no such facts are in dispute and otherwise shall contain the same information as set forth above as required by Rule 28-106.301, F.A.C.

Because the administrative hearing process is designed to formulate final agency action, the filing of a petition means that the EPC's final action may be different from the position taken by it in this notice of intent.  Persons whose substantial interests will be affected by any such final decision of the EPC on the application have the right to petition to become a party to the proceeding, in accordance with the requirements set forth above.

Mediation under section 120.573, F.S. is not available in this proceeding.

This action is final and effective on the date filed with the Clerk of the EPC unless a petition is filed in accordance with above.  Upon the timely filing of a petition, this order will not be effective until further order of the EPC.

In addition to the above, a person subject to regulation has a right to apply to the Department of Environmental Protection for a variance from or waiver of the requirements of particular rules, on certain conditions, under Section 120.542, F.S.  The relief provided by this state statute applies only to state rules, not statutes, and not to any federal regulatory requirements.  Applying for a variance or waiver does not substitute or extend the time for filing a petition for an administrative hearing or exercising any other right that a person may have in relation to the action proposed in this notice of intent.

The application for a variance or waiver is made by filing a petition with the Office of General Counsel of the Department of Environmental Protection, 3900 Commonwealth Boulevard, Mail Station #35, Tallahassee, FL  32399-3000.  The petition must specify the following information:

(a) The name, address, and telephone number of the petitioner,

(b) The name, address, and telephone number of the attorney or qualified representative of the petitioner, if any,

(c) Each rule or portion of a rule from which a variance or waiver is requested,

- The citation to the statute underlying (implemented by) the rule identified in (c) above,

- The type of action requested,

- The specific facts that would justify a variance or waiver for the petitioner,

- The reason by the variance or waiver would serve the purposes of the underlying statute (implemented by the rule), and

- A statement whether the variance or waiver is permanent or temporary and, if temporary, a statement of the dates showing the duration of the variance or waiver requested.

The Department will grant a variance or waiver when the petition demonstrates both that the application of the rule would create a substantial hardship or violate principles of fairness, as each of the those terms is defined in Section 120.542(2), F.S., and that the purpose of the underlying statute will be or has been achieved by other means by the petitioner.

Persons subject to regulation pursuant to any federally delegated or approved air program should be aware that Florida is specifically not authorized to issue variances or waivers from any requirements of any such federally delegated or approved program.  The requirements of the program remain fully enforceable by the Administrator of United States Environmental Protection Agency and by any person under the Clean Air Act unless and until the Administrator separately approves any variance or waiver in accordance with the procedures of the federal program.

Any person listed below may request to obtain additional information, a copy of the application (except for information entitled to confidential treatment pursuant to Section 403.111, F.S.), all relevant supporting materials, and all other materials available to the EPC that are relevant to the permit decision.  Interested persons may contact Diana M. Lee, P.E., at the above address or call (813) 627-2600, for additional information.

Any party to this order has the right to seek judicial review of it under Section 120.68 of the Florida Statues, by filing a notice of appeal under rule 9.110 of the Florida rules of Appellate Procedure with the EPC’s Legal Office at 3629 Queen Palm Dr., Tampa, Florida 33619 and with the clerk of the Department of Environmental Protection in the Office of General Counsel, Mail Station 35, 3900 Commonwealth Boulevard, Tallahassee, Florida 32399-3000, and by filing a copy of the notice of appeal accompanied by the applicable filing fees with the appropriate district court of appeal.  The notice must be filed within thirty days after this order is filed with the clerk of the Department.

Executed in Tampa, Florida

ENVIRONMENTAL PROTECTION COMMISSION

OF HILLSBOROUGH COUNTY

__________________________________

Richard D. Garrity, Ph.D.

Executive Director

cc:  Tom John, P.E. – Tom John, P.E., Inc. (via email)

CERTIFICATE OF SERVICE

This is to certify that this NOTICE OF PERMIT ISSUANCE and all copies were mailed before the close of business on _________________ to the listed persons.

FILING AND ACKNOWLEDGEMENT FILED, on this

date, pursuant to Section 120.52(7), Florida Statues,

with the clerk, receipt of which is hereby acknowledged.

_____________________________ ___________

Clerk                 	Date

ENVIRONMENTAL PROTECTION COMMISSION

# OF HILLSBOROUGH COUNTY

NOTICE OF INTENT TO ISSUE PERMIT

The Environmental Protection Commission of Hillsborough County (EPC), as delegated by the Florida Department of Environmental Protection (DEP), gives notice of its intent to issue an air pollution permit No. 0571418-007-AC to Tampa Marine Terminals, LLC.  The applicant applied on May 8, 2014 to the EPC for a construction permit to authorize the handling of prilled sulfur.  Particulate matter emissions will be controlled through the use of a water spray system.  The facility, a Synthetic Non-Title V Source, is located at 1650 Hemlock St., Tampa, FL, 33605.

A Best Available Control Technology (BACT) determination was not required.

The EPC will issue the Final permit with the conditions of the DRAFT permit unless a timely petition for an administrative hearing is filed pursuant to Sections 120.569 and 120.57, F.S. before the deadline for filing a petition.  The procedures for petitioning for hearing are set forth below.

A person whose substantial interests are affected by the proposed permitting decision may petition for an administrative proceeding (hearing) under Sections 120.569 and 120.57, F.S.  The petition must contain the information set forth below and must be filed (received) in the Legal Department of the EPC at 3629 Queen Palm Drive, Tampa, Florida 33619, Phone 813-627-2600, Fax 813-627-2602.  Petitions filed by the permit applicant or any of the parties listed below must be filed within 14 (fourteen) days of receipt of this notice of intent.  Petitions filed by any persons other than those entitled to written notice under Section 120.60(3), F.S. must be filed within 14 (fourteen) days of publication of the public notice or within 14 (fourteen) days of receipt of this notice of intent, whichever occurs first.  Under Section 120.60(3), however, any person who asked the EPC for notice of agency action may file a petition within 14 (fourteen) days of receipt of that notice, regardless of the date of publication.  A petitioner shall mail a copy of the petition to the applicant at the address indicated above, at the time of filing.  The failure of any person to file a petition within the appropriate time period shall constitute a waiver of that person's right to request an administrative determination (hearing) under Sections 120.569 and 120.57, F.S., or to intervene in this proceeding and participate as a party to it.  Any subsequent intervention will be only at the approval of the presiding officer upon the filing of a motion in compliance with Rule 28-106.205 of the F.A.C.

A petition that disputes the material facts on which the EPC’s action is based must contain the following information:

- The name and address of each agency affected and each agency’s file or identification number if known;

- The name, address, and telephone number of the petitioner, and the name, address, and telephone number of each petitioner’s representative, if any, which shall be the address for service purposes during the course of the proceedings; and an explanation of how the petitioner’s substantial interests will be affected by the agency determination;

- A statement of how and when petitioner received notice of the EPC action;

- A statement of all disputed issues of material fact.  If there are none, the petition must so indicate;

- A concise statement of the ultimate facts alleged, including the specific facts the petitioner contends warrant reversal or modification of the EPC proposed action;

- A statement of specific rules or statutes the petitioner contends requires reversal or modification of the EPC’s proposed action; and

- A statement of the relief sought by the petitioner, stating precisely the action petitioner wishes the EPC to take with respect to the EPC’s proposed action.

A petition that does not dispute the material facts upon which the EPC’s action is based shall state that no such facts are in dispute and otherwise shall contain the same information as set forth above as required by Rule 28-106.301.

Because the administrative hearing process is designed to formulate final agency action, the filing of a petition means that the EPC's final action may be different from the position taken by it in this notice of intent.  Persons whose substantial interests will be affected by any such final decision of the EPC on the application have the right to petition to become a party to the proceeding, in accordance with the requirements set forth above.

Mediation under section 120.573, F.S. is not available in this proceeding.

This action is final and effective on the date filed with the Clerk of the EPC unless a petition is filed in accordance with above.  Upon the timely filing of a petition this order will not be effective until further order of the EPC.

Any party to this order has the right to seek judicial review of it under Section 120.68 of the Florida Statues, by filing a notice of appeal under rule 9.110 of the Florida rules of Appellate Procedure with the EPC’s Legal Office at 3629 Queen Palm Drive, Tampa, Florida 33619 and with the clerk of the Department of Environmental Protection in the Office of General Counsel, Mail Station 35, 3900 Commonwealth Boulevard, Tallahassee, Florida 32399-3000, and by filing a copy of the notice of appeal accompanied by the applicable filing fees with the appropriate district court of appeal.  The notice must be filed within thirty days after this order is filed with the clerk of the Department.

The complete project file is available for public inspection during normal business hours, 8:00 a.m. to 5:00 p.m., Monday through Friday, except legal holidays, at the Environmental Protection Commission of Hillsborough County, 3629 Queen Palm Drive, Tampa, Florida 33619.  The complete project file includes the proposed Permit, the application, and the information submitted by the responsible official, exclusive of confidential records under Section 403.111, F.S. Interested persons may contact Diana M. Lee, P.E., at the above address, or call 813-627-2600, for additional information.  Any written comments filed shall be available for public inspection.  If written comments received result in a significant change in the proposed agency action, the EPC shall revise the proposed permit and require, if applicable, another Public Notice.

ENVIRONMENTAL PROTECTION COMMISSION OF

HILLSBOROUGH COUNTY, as Delegated by

STATE OF FLORIDA

DEPARTMENT OF ENVIRONMENTAL PROTECTION

NOTICE OF PERMIT

Billy J. Roy

Operations Manager

Tampa Marine Terminals, LLC

1650 Hemlock St.

Tampa, FL 33605

Re:  Hillsborough County - AP

Dear Mr. Roy:

Enclosed is Permit Number 0571418-007-AC to authorize the handling of prilled sulfur in addition to the other materials that the facility is authorized to handle, issued pursuant to Section 403.087, Florida Statutes.

Any party to this order (permit) has the right to seek judicial review of the permit pursuant to Section 120.68, Florida Statutes, by the filing of a Notice of Appeal pursuant to Rule 9.110, Florida Rules of Appellate Procedure, with the Clerk of the EPC in the Legal Department at 3629 Queen Palm Drive, Tampa, Florida 33619; and by filing a copy of the Notice of Appeal accompanied by the applicable filing fees with the appropriate District Court of Appeal.  The Notice of Appeal must be filed within 30 days from the date this Notice is filed with the clerk of the EPC.

Executed in Tampa, Florida.

Sincerely,

Richard D. Garrity, Ph.D.

Executive Director

RDG/LAW/law

Tampa Marine Terminals, LLC								Page 2 of 2

Tampa, FL 33605

cc:  Florida Department of Environmental Protection via email

Tom John, P.E. – Tom John, P.E., Inc. via email

CERTIFICATE OF SERVICE

This is to certify that this NOTICE OF PERMIT ISSUANCE and all copies were mailed before the close of business on _________________ to the listed persons.

FILING AND ACKNOWLEDGEMENT FILED, on this

date, pursuant to Section 120.52(7), Florida Statues,

with the clerk, receipt of which is hereby acknowledged.

_____________________________ ___________

Clerk                 	Date

PERMITTEE:                       			PERMIT/CERTIFICATION:

Tampa Marine Terminals, LLC			Permit No.:  0571418-007-AC

1650 Hemlock St. 					County:  Hillsborough

Tampa, FL 33605		          			Expiration Date:  August 1, 2015

Project: Prilled Sulfur Handling

This permit is issued under the provisions of Chapter 403, Florida Statutes, and Florida Administrative Code Rules 62-204, 62-210, 62-212, 62-296, 62-297, and 62-4.  The above named permittee is hereby authorized to perform the work or operate the facility shown on the application and approved drawing(s), plans, and other documents, attached hereto or on file with the EPC and made a part hereof and specifically described as follows:

Tampa Marine Terminals, LLC is a bulk material handling facility. This permit authorizes the handling of prilled sulfur in addition to the other materials that the facility is authorized to handle.  Tampa Marine Terminals, LLC receives bulk materials by either truck or ship.  Material received by ship is clamshelled into two on-shore hoppers.  The material from each hopper is gravity fed into open bed trucks. The trucks drive the material to the northeast portion of the property and dump the bulk materials into storage piles.  Alternatively, the trucks may drive directly to off-site customers.  Material received by truck is unloaded directly onto the storage piles.  When ready to be shipped offsite, front-end loaders load the material from the storage piles into trucks.  Prilled sulfur will be stored on an impervious surface and all contact water will be directed to the water treatment facility adjacent to the site.

In addition to shipping the material offsite by truck, material can be loaded into ships.  Front-end loaders load the material from the storage piles into trucks.  The trucks drive the material to a storage pile closer to the dock.  From the dock, the material is transferred into a ship using the ship’s clamshell.

Water sprays on the on-shore hoppers, truck loadouts, and storage pile areas are utilized to minimize unconfined PM emissions. In addition, the on-shore hoppers are equipped with wind screens to minimize unconfined PM emissions from the ship unloading operation. Also, fire hydrants are located near the piles which will be used in the unlikely event of a fire.

Location:  1650 Hemlock St., Tampa, Hillsborough County, FL

UTM:  17- 358.15 E and 3091.20 N

NEDS NO:  1418

Emission Units Nos.:

001 - Ship Unloading to Onshore Hopper(s)

005 - Onshore Hopper(s) to Truck

006 - Truck to Pile

007 - Pile to Pile Movement or Movement of Material within Storage Pile

008 - Front-end Loader to Truck

009 - Truck to Ship Loading Pile

012 - Ship Equipment to Ship

References Permit No.:  0571418-003-AC

Replaces Permit No.:  N/A

1.	A part of this permit is the attached General Conditions.  [Rule 62-4.160, F.A.C.]

2.	All applicable rules of the Environmental Protection Commission of Hillsborough County including design discharge limitations specified in the application shall be adhered to.  The permit holder may also need to comply with county, municipal, federal, or other state regulations prior to construction.  [Rule 62-4.070(7), F.A.C.]

3.	Issuance of this permit does not relieve the permittee from complying with applicable emission limiting standards or other requirements of Chapters 62-204, 62-210, 62-212, 62-296 and 62-297, F.A.C., or any other requirements under federal, state, or local law.  [Rule 62-210.300, F.A.C.]

4.	The permittee shall not cause, suffer, allow or permit the discharge of air pollutants which cause or contribute to an objectionable odor.  [Rule 62-296.320(2), F.A.C.]

5.	The use of property, facilities, equipment, processes, products, or compounds, or the commission of paint overspraying or any other act, that causes or materially contributes to a public nuisance is prohibited, pursuant to the Hillsborough County Environmental Protection Act, Section 16, Chapter 84-446, Laws of Florida, as Amended.

6.	As requested by the permittee, in order to limit the potential to emit, the facility-wide potential particulate matter emissions shall not exceed 49 tons per twelve consecutive month period.  [Rules 62-210.200(PTE), 62-212.300, 62-4.070(3), F.A.C. and Permit No. 0571418-003-AC]

7.	Visible emissions from any emission unit, transfer point, or activity shall not exceed 5% opacity.  [Rule 62-296.711(2)(a), F.A.C. and Ch. 1-3.52.2. of the Rules of the EPCHC]

8.	The following limitations and restrictions shall apply per any twelve consecutive month period: [Rule 62-4.070(3), F.A.C. and Permit No. 0571418-003-AC]

A)  The facility is authorized to operate 8,760 hours/year.

B)  Only the following materials shall be handled:

| Group I – Animal Feed Ingredients |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Animal Feed Ingredients* |  |  |  |  |  |
| Group II – Phosphate Rock |  |  |  |  |  |
| Phosphate Products Rock* |  |  |  |  |  |
| Group III –Fertilizers |  |  |  |  |  |
| Ammonium Nitrate |  | Sodium Potassium Nitrate |  |  | MAP |
| Group III –Fertilizers (continued) |  |  |  |  |  |
| Calcium Ammonium Nitrate |  | Compound Fertilizers |  |  | DAP |
| Potassium Nitrate |  | Dried Sludge |  |  | GTSP* |
| Calcium Nitrate |  | Urea |  |  | Kieserite |
| Magnesium Sucrate |  | Ammonium Sulfate |  |  | Granular Sulfate of Potash |
| Standard Muriate of Potash |  | Standard Sulfate of Potash |  |  | Standard Potassium Nitrate |
| Granular Muriate of Potash |  | Granular Sulfate of Potash-Magnesium |  |  | Standard Sulfate of Potash-Magnesium |
| Ammonium Nitrate with Potassium |  |  |  |  |  |
| Group I V – Millscale |  |  |  |  |  |  |
| Millscale* |  |  |  |  |  |  |
| Group V–Non-Fertilizer Products |  |  |  |  |  |  |
| Coal* |  | Syngypsum |  |  | Clay |  |
| Petcoke |  | Iron Ore |  |  | Magnetite/Ferrous Oxides |  |
| Coke |  | Prilled Sulfur |  |  |  |  |
| Group VI –Non-Fertilizer Products |  |  |  |  |  |  |
| Gypsum* |  | Aluminum Hydrate |  |  | Clinker (treated or screened) |  |
| Pumice |  | Vermiculite |  |  | Peanut Hulls |  |
| Wood Chips |  | Dolomite |  |  | Salt (solar) |  |
| Ferrous Sulfate |  | Glass |  |  | Bio-Mass |  |
| Ferro Alloys |  |  |  |  |  |  |
| Group VII – Slag and Metal Scrap |  |  |  |  |  |  |
| Coal Slag* | Granulated Furnace Slag |  |  | Ladle Slag |  |  |
| Industrial Slag | Iron Scrap (including shredded scrap metal) |  |  |  |  |  |
| Group VIII – Agricultural Products |  |  |  |  |  |  |
| Grains* |  | Grain Meals |  |  | Seeds |  |
| Soybeans |  | Grain Byproducts |  |  | Peas |  |
| Wheat, Rye, Barley |  | Citrus Pellets |  |  |  |  |
| Group IX – Bauxite/Alumina and similar Cement-like Products |  |  |  |  |  |  |
| Bauxite* |  | Alumina |  |  | Slag Cement |  |
| Group X – Aggregate |  |  |  |  |  |  |
| Limestone |  | Granite |  |  | Rocks |  |
| Gravel |  | Stone |  |  | Limestone Fines* |  |
| Group XI – Sand and Similar Materials |  |  |  |  |  |  |
| Sand* |  | Soil |  |  |  |  |
| Dirt |  | Sugar |  |  |  |  |

*Represents the highest emitting material (dustiest) in each group.

C)  All materials in Group IX shall be treated with a dust suppressant at the first material transfer point and subsequent material transfer points as necessary to meet the 5% opacity standard.

D)	All materials, except for Group III materials, shall have water applied or a dust suppressant applied in order to demonstrate compliance with the 5% opacity standard.

E)  Materials shall be adequately wet prior to transferring from the pile to a truck.

F) 	When calculating particulate matter emissions in order to demonstrate compliance with Specific Condition No. 6 the emission factors listed below for each material shall be used.

| Group Number | Emission Factor |
| --- | --- |
| I | 0.016 |
| II | 0.24 |
| III | 0.01 |
| IV | 6.8 |
| V | 0.2 |
| VI | 0.067 |
| VII | 0.155 |
| VIII | 0.086 |
| IX | 1.1 |
| X | 0.12 |
| XI | 0.001 |

G) When calculating particulate matter emissions in order to demonstrate compliance with Specific Condition No. 6 the control efficiency listed below for transfer point shall be used.

| Emission Unit No. | Control Efficiency |
| --- | --- |
| 001 | 70% |
| 005 | 70% |
| 006 | 70% |
| 007 | 70% |
| 008 | 70% |
| Emission Unit No. | Control Efficiency |
| 009 | 70% |
| 012 | 70% |

9.	In order to ensure compliance with the emission limitation specified in Specific Condition No. 7, the materials listed in Groups II, IV, V, VII, and IX  in Specific Condition No. 8.B), shall be only handled using the hoppers and the covered conveyor system, except when transferring material from the pile to trucks.  [Rule 62-4.070(3), F.A.C. and Permit No. 0571418-003-AC]

10.	In order to ensure compliance with the emission limitations in Specific Condition Nos. 6 and 7, the following restrictions and limitations shall apply:  [Rule 62-4.070(3), F.A.C. and Permit No. 0571418-003-AC]

- The permittee shall install an atomized water spray system at each transfer point and in the storage area(s).  The water spray system shall be capable of reaching the storage piles to adequately wet the material at all times.  Adequately wet means the presence of visible moisture on the surface of the pile.

- Fire hydrants shall be located in the vicinity of the prilled sulfur storage piles and shall be capable of reaching each prilled sulfur pile.

- The maximum combined number of material transfer points shall not exceed seven.

- The ship to truck unloading rate and the truck to pile unloading rate shall not exceed 300 tons/hour per operation.

- The front-end loader to truck loading rate shall not exceed 400 tons/hour per operation.

- The truck loading rate and the ship loading shall not exceed 400 tons/hour per operation.

- Onshore hopper(s) equipped with wind screens shall be used during all ship unloading activities.

- Each onshore hopper shall have a water spray system installed along the top of the hopper.

- The water spray system on the onshore hopper(s) shall be in operation while the ship off-loading operation is occurring, except when handling Group III materials.

- The drop height between the ship unloading clamshell and the onshore hopper shall be minimized as necessary to ensure compliance with the 5% opacity standard.

- Monthly maintenance inspections shall be performed on each water spray system.

11.	The permittee shall perform the following observations and checks on the daily, when in operation.  The permittee shall perform any necessary maintenance work in order to keep the equipment in good working order to minimize fugitive particulate emissions.  [Rules 62-296.320 and 62-4.070(3), F.A.C., and Permit No. 0571418-003-AC]

  - Inspect each water spray system for proper operation, including, but not limited to, adequate water pressure, pluggage, leaks, and adequate wetness of material.

  - Perform a 12-minute EPA Method 9 visible emissions observation during each day of ship unloading and ship loading operation on EU Nos. 001, 005, 009, and 012.

  - Inspect the paved and unpaved areas at the facility to ensure they are adequately wet.  Re-water as necessary if visible fugitive dust emissions are observed.

12.	Test each emission unit for visible emissions the first time prilled sulfur is handled through the emission unit and annually thereafter.  Testing shall be performed the point of highest opacity.  Submit two copies of the test data to the Air Management Division within 45 days of such testing.  Testing procedures shall be consistent with the requirements of 40 CFR 60 and Rule 62-297, F.A.C.  [Rules 62-297.310(7)(a)4. and 297.310(8)(b), F.A.C.]

13.	Testing of emissions shall be conducted with the source operating at capacity.  Capacity is defined as 90-100% of rated capacity listed in Specific Condition Nos. 10.D) – F).  If it is impracticable to test at capacity, then the source may be tested at less than capacity; in this case subsequent source operation is limited to 110% of the test load until a new test is conducted.  Once the unit is so limited, then operation at higher capacities is allowed for no more than fifteen (15) days or the next instance that material is received and handled for purposes of additional compliance testing to regain the rated capacity in the permit, with prior notification to the EPC.  Failure to submit the type of material unloaded or loaded, the throughput rates, and actual operating conditions may invalidate the test.  [Rules 62-4.070(3) and 62-297.310, F.A.C.]

14.	Compliance with the emission limitations of Specific Condition No. 7 shall be determined using EPA Method 9 contained in 40 CFR 60, Appendix A and adopted by reference in Rule 62-297, F.A.C.  The EPA Method 9 test shall be at least 30 minutes in duration.  The minimum requirements for stack sampling facilities, source sampling and reporting, shall be in accordance with Rule 62-297, F.A.C. and 40 CFR 60, Appendix A. [Rules 62-297, 62-297.310(4)(a)2., and 62-296.711(3)(a) F.A.C.]

15.	The permittee shall notify the Air Compliance Section of the Environmental Protection Commission of Hillsborough County at least 15 days prior to the date on which each formal compliance test is to begin of the date, time, and place of each such test, and the contact person who will be responsible for coordinating and having such test conducted.  [Rules 62-297.310(7)(a)9., F.A.C.]

16.	In order to demonstrate compliance with the limits established in Specific Condition Nos. 6, 8, and 10, the permittee shall maintain daily records for the most recent three-year period.  The records shall be made available to the Environmental Protection Commission of Hillsborough County, State and Federal officials upon request and shall include, but not limited to, the following: [Rules 62-4.160(14)(b) and 62-4.070(3), F.A.C. and Permit No. 0571418-003-AC]

- Day, Month, Year

- Amount and type of material unloaded from ships and trucks (tons)

- Amount and type of material loaded into trucks for shipment offsite (tons)

- Amount and type of material loaded into ships for shipment offsite (tons)

- Monthly and twelve consecutive month rolling totals of B), C), and D) above (tons)

- Monthly and twelve consecutive month rolling total of particulate matter emissions.  When calculating particulate matter emissions, the emission factors and control efficiencies listed in Specific Condition No. 8 shall be used.

- Records of the water spray system inspections as required in Specific Condition No. 9.

17.	All reasonable precautions shall be taken to prevent and control generation of unconfined emissions of particulate matter in accordance with the provision in Rule 62-296.320, F.A.C.  These provisions are applicable to any source, including, but not limited to, vehicular movement; transportation of materials; construction, alteration, demolition or wrecking; or industrially related activities such as loading, unloading, storing or handling; without taking reasonable precautions to prevent such emissions.  Reasonable precautions shall include, but not limited to, the following:  [Rule 62-296.320(4)(c), F.A.C.]

- Minimize material drop heights as much as possible.

- Cease all material handling operations during wind gusts greater than 12 miles per hour.

- Limit the height of the prilled sulfur storage pile(s) to a maximum of 30 feet.

- Installation of wind breaks to mitigate wind entrainment of particulate matter from storage piles, as necessary.

- Exercise good housekeeping at all times.

- Paving or maintenance of roads, parking area, and yards.

- Application of asphalt, water, oil, chemicals or other dust suppressants to unpaved roads, yards, open stock piles and similar activities as needed.

- Removal of particulate matter from roads and other paved areas under control of the owner or operator to prevent re-entrainment.

- Landscaping or planting of vegetation.

- Clean up spills promptly, during each shift (eight hours).  The permittee shall use a vacuum sweeper to clean up spills of prilled sulfur.

- Posting of vehicle speed limits, as necessary.

- Placing a tarp between the ship and the dock to prevent material from falling into the water during ship unloading operations.

- During the handling of prilled sulfur keep the material away from heat, sparks, hot surfaces, and sources of ignition

- During the handling of prilled sulfur use non-ferrous tools and non-ferrous fittings

18.	The permittee shall provide timely notification to the Environmental Protection Commission of Hillsborough County prior to implementing any changes that may result in a modification to this permit pursuant to Rule 62-210.200(185), F.A.C., Modification.  The changes do not include normal maintenance, but may include, and are not limited to the following, and may also require prior authorization before implementation:  [Rule 62-210.300 and 62-4.070(3), F.A.C.]

- Alteration or replacement of any equipment or major component of such equipment listed in the process description of this permit.

- Installation or addition of any equipment which is a source of air pollution.

- The handling of materials other than those allowed by this permit.

- The construction of storage silos, buildings, or other permanent enclosures.

19.	When the Environmental Protection Commission of Hillsborough County (EPC) after investigation, has good reason (such as complaints, increased visible emissions or questionable maintenance of control equipment) to believe that any applicable emission standard contained in Rules 62-204, 62-210, 62-212, 62-296, or 62-297, F.A.C., or in a permit issued pursuant to those rules is being violated, it may require the owner or operator of the source to conduct compliance tests which identify the nature and quantity of pollutant emissions from the source and to provide a report on the results of said tests to the EPC.  [Rule 62-297.310(7)(b), F.A.C.]

20.	If the permittee wishes to transfer this permit to another owner, an "Application for Transfer of Air Permit" (DEP Form 62-210.900(7)) shall be submitted, in duplicate, to the Environmental Protection Commission of Hillsborough County within 30 days after the sale or legal transfer of the permitted facility.  [Rule 62-4.120, F.A.C.]

21.	Submit to the Environmental Protection Commission of Hillsborough County each calendar year on or before April 1, completed DEP Form 62-210.900(5), "Annual Operating Report for Air Pollutant Emitting Facility", for the preceding calendar year.  [Rule 62-210.370(2), F.A.C.]

22.	A minimum of two copies of an air operating permit application shall be submitted to the Environmental Protection Commission of Hillsborough County within 60 days of completion of  compliance testing while handling prilled sulfur, or at least 90 days prior to the expiration date of this permit, whichever occurs first.  [Rules 62-4.050(2) and 62-4.090, F.A.C.]

ENVIRONMENTAL PROTECTION

COMMISSION OF HILLSBOROUGH COUNTY

_________________________________

Richard D. Garrity, Ph.D.

Executive Director