##### Table 4

Oil Storage Tanks & Containers Data

| Container /Tank Description | SPCC ID From Site Plan | Cont.1/Tank Type | Cont.1 /Tank Content | Estimated Cont.1 /Tank Vol. (gallons) | Constr. Matrl. | 1 or 2 Walls | Other 2ndary Contn. (yes/no) | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gas Tank # 2 | 3 | AST | Unld. Vehcl. Gasoline | 400 | Steel | 1 | Yes | Secondarily Contained Fuel AST Farm |
| Diesel Tank # 1 | 3 | AST | Diesel Fuel | 550 | Steel | 2 | Yes | Secondarily Contained Fuel AST Farm |
| Diesel Tank #2 | 3 | AST | Diesel Fuel | 270 | Steel | 1 | Yes | Secondarily Contained Fuel AST Farm |
| Fuel bowser 01 | 1 | Bowser | JP-8 Fuel | 400 | Steel | 1 | Yes | Secondary Contained Bldg. 13A |
| Fuel bowser 002 | 1 | Bowser | JP-8 Fuel | 400 | Steel | 1 | Yes | Secondary Contained Bldg. 13A |
| Oil Bowser OB001 (rectangular white) /Bleed Air Cart | 1 | Bowser | Oil | 450 | Steel | 1 | Yes | Secondary Contained Bldg. 13A |
| DG01 Generator | 7 | AST | Diesel Fuel | ~50 ea. | Steel | 1 | No | Tarmac south of proposed Hangar 6 |
| GPU03 Generator | 7 | AST | Diesel Fuel | ~50 ea. | Steel | 1 | No | Tarmac south of proposed Hangar 6 |
| GPU04 Generator | 7 | AST | Diesel Fuel | ~50 ea. | Steel | 1 | No | Tarmac south of proposed Hangar 6 |
| GPU05 Generator | 7 | AST | Diesel Fuel | ~50 ea. | Steel | 1 | No | Tarmac south of proposed Hangar 6 |
| GPU06 Generator | 7 | AST | Diesel Fuel | ~50 ea. | Steel | 1 | No | Tarmac south of proposed Hangar 6 |
| GPU07 Generator | 7 | AST | Diesel Fuel | ~50 ea. | Steel | 1 | No | Tarmac south of proposed Hangar 6 |
| Clark Tug (SN# 66-440, white) | 7 | Tug | Gas or Diesel | <15 ea. | Steel | 1 | No | Tarmac south of proposed Hangar 6 |
| Gray Harlan Tug (SN# 4157) | 7 | Tug | Gas or Diesel | <15 ea. | Steel | 1 | No | Tarmac south of proposed Hangar 6 |
| Tug (SN# T-15XA-101, white) | 7 | Tug | Gas or Diesel | <25 ea. | Steel | 1 | No | Tarmac south of proposed Hangar 6 |
| Large Clark Tug (SN# 70AH193) | 7 | Tug | Gas or Diesel | <40 ea. | Steel | 1 | No | Tarmac south of proposed Hangar 6 |
| Air Start Cart AS-01 | 7 | OFOE | JP-8 | 190 | Steel | 1 | No | Tarmac south of proposed Hangar 6 |
| Fuel Dolly FD-01 | 1 | AST | Gas or Diesel | 35 | Steel | 1 | Yes | Secondary Contained Bldg. 13A |
| Fuel Dolly FD-02 | 1 | AST | Gas or Diesel | 35 | Steel | 1 | Yes | Secondary Contained Bldg. 13A |
| Oil Cart (white, cylindrical) OC-03 | 1 | AST | Oil | 450 | Steel | 1 | Yes | Secondary Contained Bldg. 13A |
| Oil Cart | 7 | AST | Oil | 350 | Steel | 1 | No | Tarmac south of proposed Hangar 6 |
| Portable Light Stand | 7 | OFOE | Diesel | 30 | Steel | 1 | No | Tarmac south of proposed Hangar 6 |
| Waste or Material Tote | 2 | Tote | JP-8/water | 7*250 ea. | Plastic | 1 | Yes | W/in Chem & HazMat Building (Bldg. # 13) |
| Waste or Material Drum | 2 | Drum | Waste paint | 15*55 ea. | Steel | 1 | Yes | W/in Chem & HazMat Building (Bldg. # 13) |
| Waste or Material Drum | 2 | Drum | Hyspin AWS 32 | 4*55 ea. | Steel | 1 | Yes | W/in Chem & HazMat Building (Bldg. # 13) |
| Waste or Material Drum | 2 | Drum | Hyspin AWS 32 | 2*55 ea. | Steel | 1 | Yes | W/in Chem & HazMat Building (Bldg. # 13) |
| Waste or Material Tote | 2 | Tote | Hyspin AWS 32 | 2*250 ea. | Plastic | 1 | Yes | W/in Chem & HazMat Building (Bldg. # 13) |
| Waste or Material Drum | 2 | Drum | Used Oil | 7*55 ea. | Steel | 1 | Yes | W/in Chem & HazMat Building (Bldg. # 13) |
| Waste or Material Drum | 2 | Drum | Hydraulic Oil | 6*55 ea. | Steel | 1 | Yes | W/in Chem & HazMat Building (Bldg. # 13) |
| Waste or Material Drum | 4 | Drum | Motor Oil | 2*55ea. | Steel | 1 | Yes | Maintenance Building 14B |
| Waste or Material Drum | 4 | Drum | Used Oils | 55 | Steel | 1 | Yes | Maintenance Building 14B |
| Waste or Material Drum | 5 | Drum | Motor Oil | 55 | Steel | 1 | Yes | Hangar 3 Chem. & Oil Storage Area |
| Waste or Material Drum | 5 | Drum | Synth. Hydr. Oil | 55 | Steel | 1 | Yes | Hangar 3 Chem. & Oil Storage Area |
| ASG-025 Bun Press | 6 | OFOE | Hyspin AWS 32 | 300 | Steel | 1 | No | Building 16 |
| ASG-066 Bun Press | 6 | OFOE | Hyspin AWS 32 | 300 | Steel | 1 | No | Building 16 |
| Hydro Press/Verson | 6 | OFOE | Hyspin AWS 32 | 1500 | Steel | 1 | No | Building 16 |
| Electrical Transformer | 10 | OFOE | Mineral Oil | 450 | Steel | 1 | No | Building 1 E |
| Electrical Transformer | 11 | OFOE | Mineral Oil | 450 | Steel | 1 | No | Building 2B N |
| Electrical Transformer | 12 | OFOE | Mineral Oil | 450 | Steel | 1 | No | Building 3 south |
| Electrical Transformer | 13 | OFOE | Mineral Oil | 450 | Steel | 1 | No | Building 16 North |
| Electrical Transformer | 14 | OFOE | Mineral Oil | 450 | Steel | 1 | No | cafeteria |