TECHNICAL EVALUATION

&

PRELIMINARY DETERMINATION

APPLICANT

Ritchie Bros. Auctioneers (America), Inc.

4000 Pine Lake Road

Lincoln, Nebraska 68516

Orlando Auction Facility

Facility ID No. 1050358

PROJECT

Project No. 1050358-006-AO, 1050358-007-AC

Application for Minor Air Construction Permit

Project Name: Air Construction Permit and Operation Permit Revision

COUNTY

Polk County, Florida

PERMITTING AUTHORITY

Florida Department of Environmental Protection

Air Resource Management

Southwest District Office

Temple Terrace, Florida 33637-0926

March 5, 2014

Prepared by Nwankaku Onwunli

General Project INFORMATION

Air Pollution Regulations

Projects at stationary sources with the potential to emit air pollution are subject to the applicable environmental laws specified in Section 403 of the Florida Statutes (F.S.).  The statutes authorize the Department of Environmental Protection (Department) to establish regulations regarding air quality as part of the Florida Administrative Code (F.A.C.), which includes the following applicable chapters:  62-4 (Permits); 62-204 (Air Pollution Control – General Provisions); 62-210 (Stationary Sources – General Requirements); 62-212 (Stationary Sources – Preconstruction Review); 62-213 (Operation Permits for Major Sources of Air Pollution); 62-296 (Stationary Sources - Emission Standards); and 62-297 (Stationary Sources – Emissions Monitoring).  Specifically, air construction permits are required pursuant to Rules 62-4, 62-210 and 62-212, F.A.C.

In addition, the U. S. Environmental Protection Agency (EPA) establishes air quality regulations in Title 40 of the Code of Federal Regulations (CFR).  Part 60 specifies New Source Performance Standards (NSPS) for numerous industrial categories.  Part 61 specifies National Emission Standards for Hazardous Air Pollutants (NESHAP) based on specific pollutants.  Part 63 specifies NESHAP based on the Maximum Achievable Control Technology (MACT) for numerous industrial categories.  The Department adopts these federal regulations on a quarterly basis in Rule 62-204.800, F.A.C.

Glossary of Common Terms

Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of this permit.

- Facility Description:

Ritchie Bros. Auctioneers (America), Inc. (RBA) refurbishes and sells used industrial equipment 	including construction, transportation, mining, forestry, petroleum, and agricultural equipment.  RBA 	refurbishes equipment by washing, abrasive blasting, and painting (mostly heavy construction 	equipment).  Refurbishment operations generally occur throughout the year with increased activity 6 	weeks prior to each auction.

II.  	Project Description:

A.	Applicant:

Karl Werner, Chief Auction Operations Officer

Ritchie Bros. Auctioneers (America), Inc.

4000 Pine Lake Road

Lincoln, Nebraska 68506

B.	Professional Engineer:

John Malkowski

Terracon Consultants, Inc.

1675 Lee Road

Winter Park, Florida 32789

C.	Project Location:

700 Ritchie Road, Davenport, Florida 33897

D. 	Project Summary:

This project is for the increase in the maximum throughput rate of abrasive material (from 100.0 tons per year (TPY) to 11,037.0 TPY) and the potential emissions of particulate matter less than 10 micrometers (PM10) (from 0.001 TPY to 0.11 TPY) from Emissions Unit No. 001 – Abrasive Blast Booth.

E.	Application Information:

Application Received on:   February 18, 2014

Additional Information Requested on: Not Applicable

Additional Information Received on: Not Applicable

Application Complete on:

III.	Applicable State and Federal Regulations

This project is subject to the preconstruction review requirements of Chapter 403, Florida Statutes and Chapters 62-204 through 62-297, Florida Administrative Code (F.A.C.), as indicated below.

| Subject to: | Y/N | Comments |
| --- | --- | --- |
| Rule 62-210.300, F.A.C. – Permits Required | Y | subject to general permitting requirements |
| Rule 62-212.400, F.A.C. - Prevention of Significant Deterioration | N | facility is not a PSD major source |
| Rule 62-296.320(4), F.A.C. - General Particulate Emission Limiting Standards | Y | abrasive blasting operations (EU No. 001) could be a source of unconfined particulate matter emissions |
| Rules 62-296.320(1) and (2), F.A.C. - General Pollutant Emission Limiting Standards (VOCs and Odor) | Y | painting operations (EU No. 002) could be a source of VOCs and odor |
| Rule 62-296.401-418, F.A.C. - Stationary Source Emission Standards | N | there is no applicable source category |
| Rule 62-296.500, F.A.C. - Reasonably Available Control Technology (VOC) | N | Polk County is not an air quality maintenance area for ozone |
| Rule 62-296.700, F.A.C. - Reasonably Available Control Technology (PM) | N | there is no applicable source category |
| Rule 62-204.800, F.A.C. - Standards of Performance for New Stationary Sources (NSPS) | N | there is no applicable source category |
| Rule 62-204.800, F.A.C. National Emission Standard for Hazardous Air Pollutants (NESHAPS – 40 CFR 61) | N | there is no applicable source category |
| Rule 62-204.800, F.A.C. National Emission Standard for Hazardous Air Pollutants for Source Categories a.k.a. MACT (NESHAPS –40 CFR 63) | N | there is no applicable source category |
| Chapter 62-213, F.A.C. - Operation Permits for Major Sources of Air Pollution | N | facility is a synthetic non-Title V source |
| Rule 62-297.310, F.A.C. - General Compliance Test Requirements, F.A.C. | Y | VE testing is required for EU ID No. 001 |

V.	Summary of Emissions

| Pollutant | EU No. and brief description | Potential Emissions (TPY) | Allowable Emissions (TPY) |
| --- | --- | --- | --- |
| PM/ PM10 | 001 – Abrasive Blast Booth | 0.11 TPY1 | N/A |
| VOC | 002- Paint Spray Booths #1 and #2 | 24.5 TPY | 24.5 TPY2 |
| Individual HAP | 002- Paint Spray Booths #1 and #2 | --- | 9.9 TPY3 |
| Cobalt Compounds | 002- Paint Spray Booths #1 and #2 | 0.08 TPY | 9.9 TPY |
| Cumene | 002- Paint Spray Booths #1 and #2 | 0.15 TPY | 9.9 TPY |
| Ethylbenzene | 002- Paint Spray Booths #1 and #2 | 1.98 TPY | 9.9 TPY |
| Naphthalene | 002- Paint Spray Booths #1 and #2 | 2.47 TPY | 9.9 TPY |
| Styrene | 002- Paint Spray Booths #1 and #2 | 0.49 TPY | 9.9 TPY |
| Xylenes | 002- Paint Spray Booths #1 and #2 | 3.89 TPY | 9.9 TPY |
| Total HAPs | Total | --- | 24.5 TPY3 |
| VE Limitation |  |  |  |
| VE | 001 – Abrasive Blast Booth |  | 5% opacity |
|  | 002- Paint Spray Booths #1 and #2 |  | 20% opacity |

1 8760 hrs/yr x 1260 lbs/hr = 5518.8 TPY x 2 hoses = 11037.6 TPY (0.00001 baghouse efficiency 	(99.999%) x 11037) = 0.11 TPY

2Total VOCs are limited to 24.5 TPY

3Individual HAPs are limited to 9.9 TPY, but collectively cannot exceed 24.5 TPY

V.	Conclusions

The emission limits proposed by the applicant will meet all of the requirements of Chapters 62-204 through 297, F.A.C.

The General and Specific Conditions listed in the proposed permit (attached) will assure compliance with all the applicable requirements of Chapters 62-204 through 297, F.A.C.

VI.	Preliminary Determination

The Department makes a preliminary determination that the proposed project will comply with all applicable state and federal air pollution regulations as conditioned by the draft permit.  This determination is based on a technical review of the complete application, reasonable assurances provided by the applicant, and the conditions specified in the draft permit.  Additional details of this analysis may be obtained by contacting the project engineer at the Florida Department of Environmental Protection Air Resource Management, Southwest District 13051 North Telecom Parkway, Temple Terrace, Florida 33637-7600.

Pursuant to Section 403.087, Florida Statutes and Section 62-4.070, Florida Administrative Code, the Department hereby gives notice of its intent to issue a permit to construct the aforementioned air pollution source in accordance with the draft permit and its conditions as stipulated (see attached).