TECHNICAL EVALUATION

&

PRELIMINARY DETERMINATION

APPLICANT

Owens Corning Insulating Systems, LLC

2222 W. Bella Vista Street

Lakeland, FL 33810

Lakeland Facility

Facility ID No. 1050375

PROJECT

Project No. 1050375-011-AC

Application for Minor Air Construction Permit

Project Name: Production Line Expansion

COUNTY

Polk County, Florida

PERMITTING AUTHORITY

Florida Department of Environmental Protection

Air & Solid Waste Permitting Program

Southwest District Office

Temple Terrace, Florida 33637-0926

June 30, 2014

Prepared by Richard Spaulding, PE

General Project INFORMATION

Air Pollution Regulations

Projects at stationary sources with the potential to emit air pollution are subject to the applicable environmental laws specified in Section 403 of the Florida Statutes (F.S.).  The statutes authorize the Department of Environmental Protection (Department) to establish regulations regarding air quality as part of the Florida Administrative Code (F.A.C.), which includes the following applicable chapters:  62-4 (Permits); 62-204 (Air Pollution Control – General Provisions); 62-210 (Stationary Sources – General Requirements); 62-212 (Stationary Sources – Preconstruction Review); 62-213 (Operation Permits for Major Sources of Air Pollution); 62-296 (Stationary Sources - Emission Standards); and 62-297 (Stationary Sources – Emissions Monitoring).  Specifically, air construction permits are required pursuant to Rules 62-4, 62-210 and 62-212, F.A.C.

In addition, the U. S. Environmental Protection Agency (EPA) establishes air quality regulations in Title 40 of the Code of Federal Regulations (CFR).  Part 60 specifies New Source Performance Standards (NSPS) for numerous industrial categories.  Part 61 specifies National Emission Standards for Hazardous Air Pollutants (NESHAP) based on specific pollutants.  Part 63 specifies NESHAP based on the Maximum Achievable Control Technology (MACT) for numerous industrial categories.  The Department adopts these federal regulations on a quarterly basis in Rule 62-204.800, F.A.C.

Glossary of Common Terms

Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of this permit.

- Facility Description:

Fiberglass insulation manufacturing consisting of raw material handling, glass melting/refining and glass fiber forming to produce low density insulation products for the construction industry.

II.  	Project Description:

A.	Applicant:

Jerry Lewis, Plant Leader

Owens Corning Insulating Systems, LLC

2222 W. Bella Vista Street

Lakeland, FL 33810

B.	Professional Engineer:

Michael Ballenger, PE

Trinity Consultants

919 Lake Baldwin Land, Suite B

Orlando, FL 32814

C.	Project Location:

2222 W. Bella Vista Street

Lakeland, FL 33810

D. 	Project Summary:

Construction of additional unbonded loosefill fiberglass insulation (ULF) fiberizers and expansion of related equipment, including an additional electric glass furnace, as further described in the permit.

E.	Application Information:

Application Received on:  June 5, 2014

Application Complete on:  June 5, 2014

III.	Applicable State and Federal Regulations

This project is subject to the preconstruction review requirements of Chapter 403, Florida Statutes and Chapters 62-204 through 62-297, Florida Administrative Code (F.A.C.), as indicated below.

| Subject to: | Y/N | Comments |
| --- | --- | --- |
| Rule 62-210.300, F.A.C. – Permits Required | Y | subject to general permitting requirements |
| Rule 62-212.400, F.A.C. - Prevention of Significant Deterioration | N | This facility is not currently subject to PSD requirements; however, after the completion of this project, this facility will be classified as PSD major source. This facility will have a potential to emit more than 100 tons of the PSD pollutants PM, PM10, PM2.5 and CO and is classified as a glass fiber processing plant. |
| Rule 62-296.320(4), F.A.C. - General Particulate Emission Limiting Standards | Y | Manufacturing operations could be a source of unconfined particulate matter emissions, and melt furnaces are subject to process weight table. |
| Rules 62-296.320(1) and (2), F.A.C. - General Pollutant Emission Limiting Standards (VOCs and Odor) | Y | Manufacturing operations are a source of VOC and odors. |
| Rule 62-296.401-418, F.A.C. - Stationary Source Emission Standards | N | There is no applicable source category. |
| Rule 62-296.500, F.A.C. - Reasonably Available Control Technology (VOC) | N | Polk County is not an air quality maintenance area for ozone. |
| Rule 62-296.700, F.A.C. - Reasonably Available Control Technology (PM) | N | Based on its date of start of operation, this facility is not subject to PM RACT rule. |
| Rule 62-204.800, F.A.C. - Standards of Performance for New Stationary Sources (NSPS) | Y | EU No. 003 is subject to 40 CFR 60 Subpart PPP (Wool Fiberglass Insulation Manufacturing). EU No. 004 is subject to 40 CFR 60 Subpart IIII (Stationary Compression Ignition Internal Combustion Engines). |
| Rule 62-204.800, F.A.C. National Emission Standard for Hazardous Air Pollutants (NESHAPS – 40 CFR 61) | N | There is no applicable source category. |
| Rule 62-204.800, F.A.C. National Emission Standard for Hazardous Air Pollutants for Source Categories a.k.a. MACT (NESHAPS –40 CFR 63) | N | There is no applicable source category. Note: This facility is not subject to 40 CFR 63, Subpart DDD (Mineral Wool Production) or Subpart NNN (Wool Fiberglass Manufacturing) as it is not a major source of HAP. For EU No. 004 compliance with 40 CFR 63 Subpart ZZZZ (Reciprocating Internal Combustion Engines) is achieved by compliance with 40 CFR 60 Subpart IIII (Stationary Compression Ignition Internal Combustion Engines). |
| Chapter 62-213, F.A.C. - Operation Permits for Major Sources of Air Pollution | Y | After completion of this project, this facility will be a major Title V source. |
| Rule 62-297.310, F.A.C. - General Compliance Test Requirements, F.A.C. | Y | VE testing is required for EU Nos. 001 and 002. PM Testing is required for EU No. 003. |

IV.	Summary of Emissions

| Pollutant | EU No. and brief description | Potential Emissions (tpy) | Allowable Emissions (tpy) |
| --- | --- | --- | --- |
| PM | 001, Material Handling | 1.75 | N/A |
|  | 002, Electric Furnaces | 10.36 | N/A |
|  | 003, ULF forming processes | 129.9 | N/A |
|  | 004, Emergency Diesel | 0.37 | N/A |
|  | Total Insignificant Emissions | 1.12 | N/A |
|  | Total | 143.5 | N/A |
| PM10 | 001, Material Handling | 1.75 | N/A |
|  | 002, Electric Furnaces | 10.36 | N/A |
|  | 003, ULF forming processes | 129.9 | N/A |
|  | 004, Emergency Diesel | 0.37 | N/A |
|  | Total Insignificant Emissions | 1.12 | N/A |
|  | Total | 143.5 | N/A |
| PM2.5 | 001, Material Handling | 1.06 | N/A |
|  | 002, Electric Furnaces | 5.54 | N/A |
|  | 003, ULF forming processes | 118.9 | N/A |
|  | 004, Emergency Diesel | 0.37 | N/A |
|  | Total Insignificant Emissions | 0.75 | N/A |
|  | Total | 126.6 | N/A |
| VOC | 001, Material Handling | 0.0 | N/A |
|  | 002, Electric Furnaces | 0.0 | N/A |
|  | 003, ULF forming processes | 45.4 | N/A |
|  | 004, Emergency Diesel | 0.42 | N/A |
|  | Total Insignificant Emissions | 0.07 | N/A |
|  | Total | 45.9 | N/A |
| NOx | 001, Material Handling | 0.0 | N/A |
|  | 002, Electric Furnaces | 0.0 | N/A |
|  | 003, ULF forming processes | 8.07 | N/A |
|  | 004, Emergency Diesel | 5.19 | N/A |
|  | Total Insignificant Emissions | 0.17 | N/A |
|  | Total | 13.4 | N/A |
| SO2 | 001, Material Handling | 0.0 | N/A |
|  | 002, Electric Furnaces | 6.67 | N/A |
|  | 003, ULF forming processes | 0.06 | N/A |
|  | 004, Emergency Diesel | 0.34 | N/A |
|  | Total Insignificant Emissions | 0.0 | N/A |
|  | Total | 7.07 | N/A |
| CO | 001, Material Handling | 0.0 | N/A |
|  | 002, Electric Furnaces | 1.42 | N/A |
|  | 003, ULF forming processes | 145.9 | N/A |
|  | 004, Emergency Diesel | 1.12 | N/A |
|  | Total Insignificant Emissions | 0.14 | N/A |
|  | Total | 148.6 | N/A |
| Total HAP | 001, Material Handling | 0.0 | N/A |
|  | 002, Electric Furnaces | 1.11 | N/A |
|  | 003, ULF forming processes | 6.63 | N/A |
|  | 004, Emergency Diesel | 0.0 | N/A |
|  | Total Insignificant Emissions | 0.0 | N/A |
|  | Total | 7.74 | N/A |
| VE Limitation |  |  |  |
| VE | 001 |  | <5% opacity (Application & Rules 62-4.070(3) & 62.210.650, F.A.C.) |
|  | 002 |  | <5% opacity (Application & Rules 62-4.070(3) & 62.210.650, F.A.C.) |
|  | 003 |  | <20% opacity (Rule 62-296.320(4)(b)1 F.A.C. |
|  | 004 |  | <20% opacity (Rule 62-296.320(4)(b)1 F.A.C. |

V.	Conclusions

The emission limits proposed by the applicant will meet all of the requirements of Chapters 62-204 through 297, F.A.C.

The General and Specific Conditions listed in the proposed permit (attached) will assure compliance with all the applicable requirements of Chapters 62-204 through 297, F.A.C.

VI.	Preliminary Determination

The Department makes a preliminary determination that the proposed project will comply with all applicable state and federal air pollution regulations as conditioned by the draft permit.  This determination is based on a technical review of the complete application, reasonable assurances provided by the applicant, and the conditions specified in the draft permit.  Additional details of this analysis may be obtained by contacting the project engineer at the Florida Department of Environmental Protection Air & Solid Waste Permitting Program, Southwest District 13051 North Telecom Parkway, Temple Terrace, Florida 33637-7600.

Pursuant to Section 403.087, Florida Statutes and Section 62-4.070, Florida Administrative Code, the Department hereby gives notice of its intent to issue a permit to construct the aforementioned air pollution source in accordance with the draft permit and its conditions as stipulated (see attached).