# Title V Air Operation Permit Renewal

Florida Gas Transmission Company

Compressor Station No. 16

Permit No.:  0070012-015-AV

Applicant

The applicant for this project is Florida Gas Transmission Company.  The applicant’s responsible official and mailing address are:  David W. Shellhouse, Vice President Southeast Operations, Florida Gas Transmission Company, 2405 Lucien Way, Suite 200, Maitland, Florida 32751-7047.

Facility Description

The applicant operates Compressor Station No. 16, which is one of the existing compressor stations for the natural gas pipeline serving Florida.  The compressor station is located in Bradford County at 14369 SW State Road 231, Brooker, Florida.

The existing compressor station consists of seven; natural gas fired Spark Ignition (SI) Reciprocating Internal Combustion Engines (RICE):

Five Internal Combustion Engines, No. 1601, 1602, 1603, 1604, and 1605 all classified as EU 001. The five identical four- stroke lean -burn (4SLB) Worthington manufactured engines, model SEHG-8 are each rated at 2,000 bhp.

One Internal Combustion Engine No. 1606 identified as EU 002, which is a two- stroke lean -burn (2SLB) Cooper-Bessemer, manufactured engine model 8W-330-C2 rated at 4,000 bhp.

One Internal Combustion gas turbine No. 1607 identified as EU 003, manufactured by Cooper-Rolls 501-KC7 DLE rated at 7,200 bhp.

One  GEN03 Waukesha Model No. H24GL natural gas fired emergency generator classified as EU 004.  The engine is a four- cycle lean burn, spark ignition, internal combustion natural gas engine manufactured prior to December 19, 2002.

Also included in this permit are miscellaneous unregulated emissions units and/or activities.

Project DESCRIPTION

The purpose of this permitting project is to renew the existing Title V Air Operation Permit for the above referenced facility. This permit also revises the annual Visible Emissions requirement of EU002 & EU003 to a 5 year testing upon permit renewal in accordance with  Rules 62-297.310(7) a 3, F.A.C. and 62-4.070(3)

Processing Schedule and Related Documents

Application for a Title V Air Operation Permit Renewal/Revision received May 30, 2013

Notice of Intent to Issue Air Permit issued July 25, 2013

Public Notice Published August 22, 2013

Primary Regulatory requirements

Title III:  The facility is identified as a major source of hazardous air pollutants (HAP).

Title V:  The facility is a Title V major source of air pollution in accordance with Chapter 62-213, Florida Administrative Code (F.A.C.).

NSPS:  The facility does operate units subject to the New Source Performance Standards (NSPS) of 40 Code of Federal Regulations (CFR) 60.  The combustion turbine EU 003 is subject to the New Source Performance Standards in 40 CFR 60, Subpart GG.

NESHAP:  The facility does operate unit(s) subject to the National Emissions Standards for Hazardous Air Pollutants (NESHAP) of 40 CFR 63.

The combustion turbine No. 1607 EU 003 is subject to 40 CFR 63, Subpart YYYY- NESHAP for Stationary Combustion Turbines. In accordance with 40 CFR 63.6090(b)(4), this emissions unit does not have to meet the requirements of this subpart or subpart A of this part.

The Five Internal Combustion Engines, No. 1601, 1602, 1603, 1604, and 1605 all classified as EU 001 and the internal Combustion Engine No. 1606 identified as EU 002, combustion engines are classified as existing stationary RICE by 40 CFR 63.6590(a)(1)(i) of Subpart ZZZZ—National Emissions Standards for Hazardous Air Pollutants for Stationary Reciprocating Internal Combustion Engines since they each commenced construction on or before December 19, 2002.  As such, in accordance with 40 CFR 63.6590(b)(3)(i) and (ii), these engines do not have to meet the requirements of Subpart ZZZZ or 40 CFR 63 Subpart A.  These engines presently do not meet the applicability requirements of 40 CFR 60 Subpart JJJJ – Standards of Performance for Stationary Spark Ignition Internal Combustion Engines.

EU 002 incorporates “lean burn” technology to minimize exhaust NOx emissions. This emissions unit is regulated in accordance with F.A.C. Rule 62-212.400, Prevention of Significant Deterioration, which required a BACT determination approved May 8, 1991.

In accordance with 40 CFR 63.6590(b)(3)(iii), the  GEN03 Waukesha Model No. H24GL natural gas fired 585 bhp emergency generator classified as EU 004, does not have to meet the requirements of Subpart ZZZZ and of 40 CFR 63 Subpart A due to classification as an existing emergency stationary RICE (40 CFR 63.6590(a)(1)(i)), with a site rating of more than 500 brake HP located at a major source of HAP emissions. These engines presently do not meet the applicability requirements of 40 CFR 60 Subpart JJJJ – Standards of Performance for Stationary Spark Ignition Internal Combustion Engines.

CAIR:  The facility is not subject to the Clean Air Interstate Rule (CAIR) set forth in Rule 62-296.470, F.A.C.

CAM:  Compliance Assurance Monitoring (CAM) does not apply to any of the units at the facility.

PROJECT REVIEW

To revise the annual Visible Emissions requirement of EU002 & EU003  from annual emissions testing to a 5 year testing prior to obtaining a renewed operation permit in accordance with  Rules 62-297.310(7) a 3, F.A.C. and 62-4.070(3)

Changes that were made as part of this revision include:

- FROM:

Test Methods and Procedures

B.8.  Annual Testing.  Compliance with each of the emissions limits stated in Specific Condition B.6. shall be demonstrated on an annual basis during each federal fiscal year (October 1st to September 30th).

[Comments from Florida Gas Transmission Company received on September 29, 2008]

TO:

B.9.  Visible Emissions Test EU 002: A visible emissions test shall be conducted every 5 years prior to obtaining a renewed operation permit.

[Rules 62-297.310(7) a 3, F.A.C. and 62-4.070(3)]

In addition corrections were made to emissions factor that determine the maximum allowable Emissions rate for sulfur dioxide emissions limit  for EU 002 from 10 gr. S/100 Scf to 7.80 gr. S/ 100 Scf base on FGT Amendment to Permit No. AC04-189454 (September 17, 1993).

FROM:

Test Methods and Procedures

C.8.  Annual Testing.  During each federal fiscal year (October 1st to September 30th) the gas turbine shall be tested to demonstrate compliance with the emission standards for CO, NOX and VE.

[Rule 62-297.310(7)(a)4, F.A.C.; Air Permit No. 0070012-004-AC]

TO:

C.8.	Annual Testing:  Compliance with Carbon Monoxide (CO) and Nitrogen Oxides (NOx), emissions limits stated in Specific Condition C.4. shall be demonstrated on an annual basis during each federal fiscal year (October 1st to September 30th).

[Rule 62-297.310(7)(a)4, F.A.C.; Air Permit No. 0070012-004-AC Permit Application received May 30th, 2013]

C.12.	Visible Emissions Test EU 003: A visible emissions test shall be conducted every 5 years prior to obtaining a renewed operation permit.

[Rules 62-297.310(7) a 3, F.A.C. and 62-4.070(3)]

Conclusion

This project for the renewal of Title V air operation permit No. 0070012-013-AV, which was issued on November 17, 2008.  This Title V Air Operation Permit Renewal is issued under the provisions of Chapter 403, Florida Statutes (F.S.), and Florida Administrative Code (F.A.C.) Chapters 62-4, 62-210 and 62-213.  The above named permittee is hereby authorized to operate the facility shown on the application and approved drawing(s), plans, and other documents, attached hereto or on file with the permitting authority, in accordance with the terms and conditions of this permit.