| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 006 | No. 5 Power Boiler | Carb | 8,760 | Particulate Matter | 0.3 lb/MMBtu | 137.1 | 598.9 | 62-296.410(1)(b)2., F.A.C. | III.A.5.a. |
|  |  | Fuel Oil |  | Particulate Matter | 0.1 lb/MMBtu |  |  | 62-296.405(1)(b), F.A.C. | III.A.5.b.1 |
|  |  | Fuel Oil |  | Particulate Matter – Soot Blowing and Load Change | 0.3 lb/MMBtu for ≤ 3 hours/24 hours |  |  | 62-210.700(3), F.A.C. | III.A.5.b.2. |
|  |  | All Fuels |  | Sulfur Dioxide | 550.0 lb/hour based on a 24-hour rolling average |  |  | 62-296.340(5)(c), F.A.C. | III.A.6. |
|  |  | NCGs |  | TRS | 5 ppmvd@ 10% O2 (12-hr avg.) | 11.74 | 12.85 | 62-296.404(3)(f), F.A.C. | III.A.7. |
|  |  | Carb |  | VE | 30% Opacity except 40% is permissible for ≤ 2 minutes in any hour |  |  | 62-296.410(1)(b)1., F.A.C. | III.A.8.a. |
|  |  | Fuel Oil |  | VE | 20% Opacity except 40% is permissible for ≤ 2 minutes in any hour |  |  | 62-296.405(1)(a), F.A.C. | III.A.8.b. |
|  |  | Fuel Oil |  | VE – Soot Blowing or Load Change | 60% Opacity ≤ 3 hrs in any 24-hour period, w/ best oper. prctices & minimize excess emissions duration |  |  | 62-210.700(3), F.A.C. | III.A.8.c. |
|  |  | WW Wood Fiber Residuals |  | Mercury | No more than 7.1 pounds per 24 hour period |  |  | 40 CFR 61.52(b) | III.A.9. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 007 | No. 4 Recovery Boiler | BLS, BLS w/ ULSD, No. 2 Fuel Oil, No. 6 Fuel Oil | 8,760 | Particulate Matter | 0.044 gr/dscf@ 8% O2 |  |  | 40 CFR 63.862(a)(1)(i)(A) | III.B.5.a. |
|  |  |  |  | Particulate Matter | 3 lbs/3000 lbs BLS | 137.5 | 602.25 | 62-296.404(2)(a), F.A.C. | III.B.5.b. |
|  |  |  |  | Total Reduced Sulfur | 5 ppmvd @ 8% O2 | 3.24 | 14.19 | 62-296.404(3)(c)1.b., F.A.C. | III.B.6. |
|  |  |  |  | Visible Emissions | ≤ 45% Opacity except up to 60% Opacity no more than one six-minute period during any hour |  |  | 62-296.404(1)(a), F.A.C. | III.B.7.a. |
|  |  |  |  | Visible Emissions | ≤ 35% Opacity for < 6% of operating time w/n any quarterly period |  |  | 40 CFR 63.864(k)(2)(i) | III.B.13. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 011 | No. 5 Recovery Boiler | BLS, BLS w/ ULSD, No. 2 Fuel Oil, No. 6 Fuel Oil | 8,568 | Particulate Matter | 0.044 gr/dscf @ 8% O2 | 83.3 | 356.9 | 40 CFR 63.862(a)(1)(i)(A) | III.C.5. |
|  |  |  |  | Total Reduced Sulfur (Cross Recovery Furnace Operation Mode) | 25 ppmvd @ 8% O2 | 26.3 | 112.67 | 40 CFR 60.283(a)(3) | III.C.6. |
|  |  |  |  | Total Reduced Sulfur (Straight Recovery Furnace Operation Mode) | 5 ppmvd @ 8% O2 | 5.26 | 22.53 | 40 CFR 60.282(a)(1)(i) 40 CFR 60.283(a)(2) | III.C.7. |
|  |  |  |  | Visible Emissions | < 35% Opacity |  |  | 40 CFR 60.282(a)(1)(ii) | III.C.8. |
|  |  |  |  | Visible Emissions | ≤ 35% Opacity for < 6% of operating time w/n any quarterly period |  |  | 40 CFR 63.864(k)(2)(i) | III.C.18. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 013 | No. 4 Smelt Dissolving Tank | N/A | 8,760 | Particulate Matter | ≤ 0.20 lb/ton BLS |  |  | 62-296.320(4)(a)2., F.A.C. 40 CFR 63.862(a)(1)(i)(B) | III.D.4. |
|  |  |  |  | Total Reduced Sulfur | 0.048 lb TRS/3000 lb BLS | 2.2 | 9.64 | 62-296.404(3)(d)1., F.A.C. | III.D.5. |
|  |  |  |  | Visible Emissions | < 20% Opacity |  |  | 62-296.320(4)(b)1., F.A.C. | III.D.6. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 014 | No. 5 Smelt Dissolving Tank | N/A | 8,568 | Particulate Matter | ≤ 0.20 lb/ton BLS | 15.68 | 67.17 | 40 CFR 60.282(a)(2) 40 CFR 63.862(a)(1)(i)(B) | III.E.4. |
|  |  |  |  | Total Reduced Sulfur | 0.033 lb TRS/ton BLS | 2.59 | 11.08 | 40 CFR 60.283(a)(4) | III.E.5. |
|  |  |  |  | Visible Emissions | < 20% Opacity |  |  | 62-296.320(4)(b)1., F.A.C. | III.E.6. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 015 | No. 7 Power Boiler |  | 8,760 | Particulate Matter | 0.10 lb/MMBtu | 102.10 | 447.20 | 40 CFR 60.42(a)(1) | III.F.5. |
|  |  | Coal |  | Sulfur Dioxide | 1.2 lb/MMBtu | 1,225.20 | 5,366.38 | 40 CFR 60.43(a)(2) | III.F.6.a. |
|  |  | No. 6 No. 2 |  | Sulfur Dioxide | 0.8 lb/MMBtu |  |  | 40 CFR 60.43(a)(1) | III.F.6.b. |
|  |  | Coal |  | Nitrogen Oxides | 0.6 lb/MMBtu | 612.6 | 2,683.19 | 10/11/84 BACT Determination | III.F.7.b. |
|  |  | No. 6 No. 2 |  | Nitrogen Oxides | 0.3 lb/MMBtu |  |  | 40 CFR 60.44(a)(2) | III.F.7.a. |
|  |  | Natural Gas |  | Nitrogen Oxides | 0.2 lb/MMBtu |  |  | 40 CFR 60.44(a)(1) | III.F.7.d |
|  |  | Coal No. 6 No. 2 |  | Carbon Monoxide | 93.6 lbs/hr and 409.97 TPY |  |  | EPA/DER Agreement 40 CFR 52.21(j) | III.F.8. |
|  |  | Coal No. 6 No. 2 |  | Visible Emissions | ≤ 20% Opacity except for no more than 27% Opacity for one 6 min period/ hour |  |  | 40 CFR 60.42(a)(2) | III.F.9. |
|  | Coal Prep and Handling | N/A |  | Visible Emissions | < 20% Opacity |  |  | 40 CFR 60.254(a) PSD-FL-062 | III.F.10. |
|  | Fly Ash Handling | N/A |  | Visible Emissions | ≤ 5% Opacity |  |  | 40 CFR 52.21(j) PSD-FL-062 | III.F.10. |
|  |  | N/A |  | Particulate Matter | ≤ 0.5 lbs/hr |  |  | PSD-FL-062 | III.F.11. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 020 | Tall Oil Plant | N/A | 8,760 | Total Reduce Sulfur | 0.05 lb/ton Tall Oil Produced, 12-hr average | 0.43 | 1.9 | 62-296.404(3)(b)1., F.A.C. | III.G.4. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 021 | No. 4 Lime Kiln | No. 6 Fuel Oil, On-Spec Used Oil, Natural Gas; [LPG (Startup Only] | 8,736 | Particulate Matter | 0.064 gr/dscf @ 10% O2 |  |  | 40 CFR 63.862(a)(1)(i)(C) | III.H.5. |
|  |  |  |  | Total Reduced Sulfur | 8 ppmvd @ 10% O2 | 2.63 | 11.5 | 40 CFR 60.283(a)(5) | III.H.6. |
|  |  |  |  | Visible Emissions | ≤ 20% Opacity |  |  | 62-296.320(4)(b)1., F.A.C. | III.H.7. |
|  | No. 1 Lime Bin | N/A | 8,760 | Visible Emissions | ≤ 5% Opacity |  |  | 62-297.620(4), F.A.C. AC45-141878, Permit No. 0890003-036-AC | III.H.8. |
|  |  |  |  | Particulate Matter | ≤ 0.03 gr/dscf | 1.2 | 5.3 | AC45-141878, Permit No. 0890003-036-AC | III.H.9. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 024 | C-Line Brownstock Washer System | N/A | 8,760 | Total Reduced Sulfur | 5 ppmvd uncorrected | 0.16 | 0.7 | 40 CFR 60.283(a)(1)(v) | III.I.4. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 033 | Pulping System MACT I | N/A | 8,760 | HAP Condensate Collection | Collect All Named Stream Option (15-day average) |  |  | 40 CFR 63.446(c)(1) | III.J.5., J.6. |
|  |  |  |  | Total HAP Condensate Treatment | Total HAP destruction of at least 92% by weight |  |  | 40 CFR 63.446(e)(3) | III.J.10. |
|  |  |  |  | HAP Vent Collection | Named Sources enclosed and routed to No. 4 Lime Kiln or No. 5 Power Boiler |  |  | 40 CFR 63.443(a) 40 CFR 63.443(c) 40 CFR 63.443(d) | III.J.2. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or condition s of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 035 | Wide-Web Flexographic Printing Presses | N/A | 8760 | Organic HAP | 400 kg/month-applied usage |  |  | 40 CFR 63.821(b)(2) | III.K.1. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or condition s of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 038, 039, 040 | Reciprocating Internal Combustion Engines (RICE) | Diesel fuel | 8,760 | HAPs | Work Practice Standards- Operation and Maintenance |  |  | 40 CFR 63.6603(a) | III.L.6. |

|  | This table summarizes information for convenience purposes only. This table does not supersede any of the terms or condition s of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | E.P. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 041 | --- | Coal Prep and Handling | N/A | 8,760 | --- | --- | --- | --- | --- | --- |
|  | 03 | Crusher |  |  | Visible Emissions | < 10% Opacity | --- | --- | 40 CFR 60.254(b)(1) | III.O.4.a. |
|  | 01 02 04 05 06 07 08 09 | Screen Hopper Conveyor from Screen/Hopper to Vibrating Feeder Conveyor from Coal Crusher to Coal Silos Coal Silo No. 1 Coal Silo No. 2 Coal Collection Conveyor Coal Bunker Conveyor Coal Bunkers A, B, & C Common Stack |  |  | Visible Emissions | < 20% Opacity |  |  | 40 CFR 60.254(a) PSD-FL-062 | III.O.4.b. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or condition s of the permit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard or Allowable Emissions | Equivalent Emissions* |  | Regulatory Citation(s) | See Permit Condition(s) |
|  |  |  |  |  |  | lbs/hr | TPY |  |  |
| 042 | Reciprocating Internal Combustion Engines (RICE) | Diesel fuel | 8,760 | HAPs | Work Practice Standards- Operation and Maintenance |  |  | 40 CFR 63.6602 | III.P.6. |
| 043 | Reciprocating Internal Combustion Engines (RICE) | Gasoline fuel | 8,760 | HAPs | Work Practice Standards- Operation and Maintenance |  |  | 40 CFR 63.6602 | III.Q.6. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of this permit. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date | Min. Compliance Test Duration | See Permit Condition(s) |
| 006 | No. 5 Power Boiler | Carb | Particulate Matter | N | ESP | EPA Method 5 | FFY | --- | 3, One-hr test runs | III.A.25.a. |
|  |  | Fuel Oil | Particulate Matter | N |  | EPA Method 5 |  | > 400 hrs/yr fuel oil fired | 3, One-hr test runs | IIIA.25.b.1. |
|  |  | Fuel Oil | Particulate Matter (Soot blowing or load change) | N |  | EPA Method 5 | FFY | > 400 hrs/yr fuel oil fired | 3, One-hr test runs | IIIA.25.b.2. |
|  |  | Fuel Oil | Sulfur Dioxide | Y |  | CEMS |  |  |  | IIIA.19.-A.24. |
|  |  | NCGs | Total Reduced Sulfur |  |  | Gases introduced w/primary fuel or into the flame zone, or with the combustion air, min temp of 1200 F, 0.5 sec residence time |  |  |  | III.A.27. |
|  |  | Carb | Visible Emissions |  |  | EPA Method 9 | FFY |  | 60 Minutes | III.A.28.a. |
|  |  | Fuel Oil | Visible Emissions |  |  | EPA Method 9 | FFY |  | 60 Minutes | III.A.28.b. |
|  |  | Fuel Oil | VE – Soot Blowing or Load Change |  |  | EPA Method 9 | FFY |  | 60 Minutes | III.A.28.c. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of this permit. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date | Min. Compliance Test Duration | See Permit Condition(s) |
| 007 | No. 4 Recovery Boiler | BLS, BLS w/ ULSD, No. 2 Fuel Oil, No. 6 Fuel Oil | Particulate Matter | Y COMS | ESP | EPA Method 5 | FFY | --- | 3, One-hr test runs | III.B.11., B.14. |
|  |  |  | Total Reduced Sulfur | Y |  | EPA Method 16 EPA Method 16A, EPA Method 16B, or EPA Method 16C | Prior to permit renewal |  | 3 hours, but no longer than 6 hours | III.B.19.a. |
|  |  |  | Visible Emissions |  |  | EPA Method 9 | FFY |  | 60 Minutes | III.B.18. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of this permit. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date | Min. Compliance Test Duration | See Permit Condition(s) |
| 011 | No. 5 Recovery Boiler | BLS, BLS w/ ULSD, No. 2 Fuel Oil, No. 6 Fuel Oil | Particulate Matter | Y COMS | ESP | EPA Method 5 | FFY | --- | 3, One-hr test runs | III.C.19., C.15. |
|  |  |  | Total Reduced Sulfur | Y |  | EPA Method 16 EPA Method 16A, EPA Method 16B, or EPA Method 16C | Prior to permit renewal |  | 3 hours, but no longer than 6 hours | III.C.13., C.24. |
|  |  |  | Visible Emissions |  |  | EPA Method 9 | FFY |  | 60 Minutes | III.C.23. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of this permit. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date | Min. Compliance Test Duration | See Permit Condition(s) |
| 013 | No. 4 Smelt Dissolving Tank | N/A | Particulate Matter | N | Scrubber | EPA Method 5 | FFY | --- | 3, One-hr test runs | III.D.17. |
|  |  |  | Total Reduced Sulfur | N |  | EPA Method 16 EPA Method 16A EPA Method 16B EPA Method 16C | Prior to permit renewal |  | 3 hours, but no longer than 6 hours | III.D.21. |
|  |  |  | Visible Emissions |  |  | EPA Method 9 | FFY |  | 60 Minutes | III.D.19. |
|  |  |  | TRS Surrogate | Y |  | Weak Wash Flow |  |  |  | III.D.112. |
|  |  |  | PM CPMS | Y |  | Pressure Drop across Scrubber |  |  |  | III.D.13. |
|  |  |  | PM CPMS | Y |  | Scrubbing Liquid Flow rate |  |  |  | III.D.13. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of this permit. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date | Min. Compliance Test Duration | See Permit Condition(s) |
| 014 | No. 5 Smelt Dissolving Tank | N/A | Particulate Matter | N | Scrubber | EPA Method 5 | FFY | --- | 3, One-hr test runs | III.E.18. |
|  |  |  | Total Reduced Sulfur | N | Scrubber | EPA Method 16 EPA Method 16A EPA Method 16B EPA Method 16C | Prior to permit renewal |  | 3 hours, but no longer than 6 hours | III.E.20.a., E.20.b. |
|  |  |  | Visible Emissions |  |  | EPA Method 9 | FFY |  | 60 Minutes | III.E.21., E.22. |
|  |  |  | TRS Surrogate | Y |  | Weak Wash Flow |  |  |  | E.13.c. |
|  |  |  | PM CPMS | Y |  | Pressure Drop across Scrubber |  |  |  | E.13.a., E.13.b. |
|  |  |  | PM CPMS | Y |  | Scrubbing Liquid Flow rate |  |  |  | E.13.a., E.13.b |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of this permit. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date | Min. Compliance Test Duration | See Permit Condition(s) |
| 015 | No. 7 Power Boiler | Coal, No. 6 Fuel oil, No. 2 Fuel Oil, Nat. Gas | Particulate Matter | Y COMS | ESP | EPA Method 5 | FFY | --- | 3, One-hr test runs | III.F.15.a., F.213. |
|  |  |  | Sulfur Dioxide | N |  | Fuel Sampling Each Coal Delivery Method 16B, t consistent with permit condition language werre EPA Method 6 | Each Coal Delivery FFY |  | 3, One-hr test runs | III.F.179., F.224. |
|  |  |  | Nitrogen Oxides | NYa |  | EPA Method 7 | FFY |  | 3, One-hr test runs | III.F.15.b., F.16., F.235. |
|  |  |  | Carbon Monoxide | N |  | EPA Method 10 | FFY |  | 3, One-hr test runs | III.F.257. |
|  |  |  | Visible Emissions | Y |  | EPA Method 9 | FFY |  | 60 Minutes | III.F.268. |
|  |  |  | NOx Surrogate | Y |  | Maximum O2 set point NOx CEMS |  |  |  | III.F.19. |
|  |  |  | CO Surrogate | Y |  | Minimum O2 set point |  |  |  | III.F.202. |
|  | Coal Prep and Handling |  | Visible Emissions | N | Fabric Filter | EPA Method 9 | FFY |  |  | III.F.31. |
|  | Fly Ash Handling |  | Visible Emisisons | N | Fabric Filter | EPA Method 9 | FFY |  |  | III.F.2830. |

a  CEMS applicable no later than July 17, 2014.  Emissions monitoring requirements applicable upon installation, calibration, certification, and operation of the NOx CEMS pursuant to the requirements of 40 CFR 60 Subpart D, 60.45.

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of this permit. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date | Min. Compliance Test Duration | See Permit Condition(s) |
| 020 | Tall Oil Plant | N/A | Total Reduced Sulfur | N | Scrubber | EPA Method 16 EPA Method 16A EPA Method 16B EPA Method 16C | Prior to Permit renewal | --- | 3 hours, but no longer than 6 hours | III.G.6. |
|  |  |  | TRS Surrogate Scrubber Liquid Flow Rate | Y |  | Scrubbing Liquid Flow rate |  |  |  | III.G.5. |
|  |  |  | TRS Surrogate pH | Y |  | 3 grab samples over course of each Tall Oil cook (beginning, middle, end) |  |  |  | III.G.5. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of this permit. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date | Min. Compliance Test Duration | See Permit Condition(s) |
| 021 | No. 4 Lime Kiln | No. 6 Fuel Oil, On-Spec Used Oil, LPG (Startup Only), Nat. Gas | Particulate Matter | Y COMS | ESP | EPA Method 5 | FFY | --- | 3, One-hour test runs | III.H.19., H.25. |
|  |  |  | TRS | Y |  | EPA Method 16 EPA Method 16A EPA Method 16B EPA Method 16C | FFY |  | 3 hours, but no longer than 6 hours | III.H.29. |
|  |  |  | Visible Emissions |  |  | EPA Method 9 | FFY |  | 60 Minutes | III.H.31. |
|  | No. 1 Lime Bin | N/A | Visible Emissions |  | Fabric Filter | EPA Method 9 | FFY |  | 30 Minutes | III.H.32. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of this permit. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date | Min. Compliance Test Duration | See Permit Condition(s) |
| 024 | C-Line Brownstock Washer System | N/A | Total Reduced Sulfur | Y | Scrubber | EPA Method 16 EPA Method 16A EPA Method 16B EPA Method 16C | Prior to Permit renewal | --- | 3 hours, but no longer than 6 hours | III.I.9. |
|  |  |  | TRS Surrogate Scrubber Liquid Flow Rate | Y |  | Scrubbing Liquid Flow rate |  |  |  | III.I.8. |
|  |  |  | TRS Surrogate Pressure Drop | Y |  | Pressure Drop across scrubber |  |  |  | III.I.8. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of this permit. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date | Min. Compliance Test Duration | See Permit Condition(s) |
| 033 | Pulping System MACT I | N/A | Condensate Collection | Y |  | All named stream option based on annual 15-day average test; quarterly 3-day test | Annual Quarterly testing |  |  | III.J.6., J.17., J.29. |
|  |  |  | Condensate Treatment | Y |  | Total HAP reduction of 92% or more by weight | Annual Quarterly testing |  |  | III.J.17., J.29. |
|  |  |  | UNOX Vent Gas Purity | Y |  |  |  |  |  | III.J.14., J.15. |
|  |  |  | UNOX Oxygen Feed | Y |  |  |  |  |  | III.J.15. |
|  |  |  | Clean Condensate Alternative Verify that the Effluent Header to the Cooling Tower Valve (Valve No. 3) is maintained in the closed position | Y |  | Replacement of the direct-contact WWTP cooling tower with a non-contact heat exchanger system |  |  |  | III. J.37, J.38, J.39. |

| This table summarizes information for convenience purposes only. This table does not supersede any of the terms or conditions of this permit. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date | Min. Compliance Test Duration | See Permit Condition(s) |
| 041 | Coal Prep and Handling | N/A | Coal Crusher | N |  | EPA Method 9 | 90 days or 12 mos. of previous test depending upon test results |  |  | III.O.5., O.10. |
|  |  |  | Screen Hopper Conveyor from Screen/Hopper to Vibrating Feeder Conveyor from Coal Crusher to Coal Silos Coal Silo No. 1 Coal Silo No. 2 Coal Collection Conveyor Coal Bunker Conveyor Coal Bunkers A, B, & C Common Stack | N |  | EPA Method 9 | FFY |  |  | III.O.9., O.10. |