| E.U. ID No. | Brief Description | Fuels | Hrs/Yr | Pollutant Name | Standard Allowable Emissions |  | Equivalent Emissions1 |  | Regulatory Citations | See Permit Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | lbs/hr | TPY Each unit |  |  |
|  |  |  |  |  | Standards | TPY |  |  |  |  |
| 012 and 027 | Turbines | Natural Gas | 8,760 | VE | 10% |  |  |  | 1050002-005-AC | D.4. |
| 012 |  |  |  | NOX | 168 ppmvd @ 15% O2 | 96.4 | 22.0 |  |  | D.5. |
| 027 |  |  |  |  | 30 ppmvd @ 15% O2 | 39.4 | 9.0 |  |  |  |
| 012 |  |  |  | SO2 | Pipeline Natural Gas |  | 21.63 | 27.16 |  | D.6. |
| 027 |  |  |  |  |  |  | 32.43 | 310.68 |  |  |

Notes:

1 Equivalent Emissions are for information l purposes only.

2 Equivalent Emissions are based on operating 8760 hours/year, max heat input of 51.15 MMBtu/hr and 0.034% sulfur in natural gas.

3 Equivalent Emissions are based on operating 8760 hours/year, max heat input of 76.26 MMBtu/hr and 0.034% sulfur in natural gas.