| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard Allowable Emissions Standard(s) lbs/hr TPY |  |  | Equivalent Emissions* |  | See Permit Condition(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | lbs/hr | TPY |  |
| -002 & 003 | Nos. 1 & 2 SAP |  | 8,760 | VE | Not equal to or greater than 10% opacity | N/A | N/A |  |  | III.A.6. |
|  |  |  | 8,760 | SO2 | 4 lbs/ton (3- hour avg.) 3.5 lbs/ton (24-hour avg.) | 496, 24-hr avg |  |  | 2,172 | III.A.3.2. |
|  |  |  | 8,760 | SAM | 0.05 lb/ton of 100% acid produced | 7.1 |  |  | 31.1 | III.A.4.2. |
|  |  |  | 8,760 | NO2 | 0.12 lb/ton of 100% acid produced | 17 |  |  | 75 | III.A.5.2. |
| 004 | No. 3 SAP |  | 8,760 | VE | Not equal to or greater than 10% opacity | N/A | N/A |  |  | III.A.6. |
|  |  |  | 8,760 | SO2 | 4 lbs/ton (3- hour avg.) 3.5 lbs/ton (24-hour avg.) | 496, 24-hr avg |  |  |  | III.A.3.3. |
|  |  |  | 8,760 | SAM | 0.05 lb/ton of 100% acid produced | 7.1 |  |  | 31.1 | III.A.4.3. |
|  |  |  | 8,760 | NO2 | 0.12 lb/Ton of H2SO4, 3-hr rolling average | 17 |  |  | 75 | III.A.5.3. |
| -042 & 044 | Nos. 4 & 5 SAP |  | N/A | VE | Not equal to or greater than 10% opacity | N/A | N/A |  |  | III.A.6. |
|  |  |  | 8,760 | SO2 | 4 lbs/ton (3-hour avg.) | 483.3 | 2, 117 |  |  | III.A.3.1. |
|  |  |  | 8,760 | (SAM) | 0.15 lbs/ton of 100% acid produced | 18.1 | 79.4 |  |  | III.A.4.1. |
|  |  |  | 8,760 | NO2 | 0.12 lbs/ton of 100% acid produced | 14.5 | 63.5 |  |  | III.A.5.1. |

Notes:  *The “Equivalent Emissions” listed are for informational purposes only.

NA:  Not Applicable		EBA:  Established by Applicant

| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard Allowable Emissions Standard(s) lbs/hr TPY |  |  | Equivalent Emissions* |  | See Permit Condition(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | lbs/hr | TPY |  |
| -008 & 017 | PAP East & West |  | 8,760 | F | 0.02 lbs/ton of 100% P2O5 feed |  |  | 3.00 | 13.55 | III.B.3. |
| -039 | PAP No. 3 |  | 8,760 | F | 0.02 lbs/ton of 100% P2O5 feed | N/A | N/A | 1.8 | 7.85 | III.C.2. |
| -053 | Phos Acid Clarification & Storage Area |  | 8760 | F |  | 25 lbs/day (1.04 lbs/hr) |  |  |  | III.D.3.a. |
| -048 | 30% Clarification Area (Area 10) |  | 8760 | F |  | 0.28 |  |  |  | III. D.3.b |
| -009 | DAP Plant No. 1 |  | 8760 | F | 0.06 lbs/ton of 100% P2O5 input | 2.92 |  |  |  | III.E.4. |
| -009 |  |  | 8,760 | PM |  | 15 |  |  |  | III.E.5. |
| -009 |  |  | 8,760 | VE | Shall not be equal to or greater than 20% Opacity |  |  |  |  | III.E.6. |

Notes:  *The “Equivalent Emissions” listed are for informational purposes only		DAP:  Diammonium Phosphate

NA:  Not Applicable		EBA:  Established by Applicant		PAP:  Phosphoric Acid Plant

| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard Allowable Emissions Standard(s) lbs/hr TPY |  |  | Equivalent Emissions* |  | See Permit Condition(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | lbs/hr | TPY |  |
| -045 & -046 | DAP 2 East & West Train |  | 8,760 | F | 0.0417 lb/ton of P2O5 input | 3.34(per train) | 14.6(per train) |  |  | III.F.3.. |
|  |  |  | 8,760 | PM | 0.08 lb PM/ton of P2O5 input | 6.40(per train) | 28.0(per train) |  |  | III.F.4.. |
|  |  |  | 8,760 | SO2 |  | 22.2(per train) | 87.0(per train) |  |  | III.F.8. |
|  |  |  | 8,760 | NOx |  | 12.6(per train) | 55.2(per train) |  |  | III.F.9. |
|  |  |  |  | VE | ≤ 15% Opacity |  |  |  |  | III.F.7. |
| -056 | DAP 2 – East Product Cooler |  | 8,760 | PM | 0.011 gr/dscf @ 64,300 scfm | 6.06 | 26.5 |  |  | III.F.5. |
| -047 | DAP 2 – West Product Cooler |  | 8,760 | PM | 0.010 gr/dscf@49,200 scfm | 4.22 | 8.5 |  |  | III.F.6. |
|  |  |  |  | VE | ≤ 5% Opacity |  |  |  |  | III.F.7. |

Notes:  *The “Equivalent Emissions” listed are for informational purposes only.

NA:  Not Applicable		EBA:  Established by Applicant

| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard Allowable Emissions Standard(s) lbs/hr TPY |  |  | Equivalent Emissions* |  | See Permit Condition(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | lbs/hr | TPY |  |
| -011 | MAP Prill Plant |  | 8,760 | PM |  | 7 |  |  |  | III.G.3. |
|  |  |  |  | F |  | 0.83 |  |  |  | III.G.3. |
|  |  |  |  | VE | 20% opacity, 6-min av. basis | N/A | N/A | N/A | N/A | III.G.3. |
| -055 | MAP Plant Cooler |  | 8,760 | PM | 0.02 gr/dscf | 4.0 |  |  |  | III.G.4. |
|  |  |  | N/A | VE | 5% opacity’ 6-min av. basis | N/A | N/A | N/A | N/A | III.G.4. |
| -015 | AFI Shipping/Truck Loadout |  | 8,760 | PM |  | 3.6 |  |  |  | III.H.3. |
| -023 | AFI Storage Silos(3) – North Side |  | 8,760 | PM |  | 4.75 |  |  |  | III.H.3. |
| -024 | AFI Storage/shipping |  | 8,760 | PM |  | 3.6 |  |  |  | III.H.3. |
| -025 | 2 Limestone storage silos |  | 8,760 | PM |  | 3.6 |  |  |  | III.H.3. |
| -026 | Silica Storage Bin |  | 8,760 | PM |  | 1.6 |  |  |  | III.H.3. |
| -028 | 3 AFI storage silo |  | 8,760 | PM |  | 4.75 |  |  |  | III.H.3. |
| -052 | Limestone Feed Bin |  | 8,760 | PM |  | 3.6 |  |  |  | III.H.3. |
| 015, 023-026, 028, 030& 052 |  |  | 8,760 | VE | 20% Opacity |  |  |  |  | III.H.4. |

Notes:  *The “Equivalent Emissions” listed are for informational purposes only		AFI:  Animal Feed Ingredients.

NA:  Not Applicable		EBA:  Established by Applicant

| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard Allowable Emissions Standard(s) lbs/hr TPY |  |  | Equivalent Emissions* |  | See Permit Condition(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | lbs/hr | TPY |  |
| -027 | AFI Plant |  | 8,760 | PM |  | 36.8 |  |  |  | III.I.3. |
|  |  |  | N/A | VE | 20% opacity | N/A | N/A |  |  | III.I.4. |
| -063, -067, -068 | No. 1 Truck Pit |  | 8,760 | PM/PM10 | Emissions Inventory | 0.2 | 0.8 |  |  | III.J.2. |
|  |  |  |  | SO2 | Emissions Inventory | 0.3 | 1.2 |  |  | III.J.2. |
|  |  |  |  | TRS/H2S | Emissions Inventory | 0.1 | 0.6 |  |  | III.J.2. |
|  |  |  |  | VOC | Emissions Inventory |  |  |  |  | III.J.2. |
| -064, 069 | No. 2 Truck Pit |  |  | PM/PM10 | Emissions Inventory | 0.1 | 0.3 |  |  | III.J.2. |
|  |  |  | 8,760 | SO2 | Emissions Inventory | 0.1 | 0.5 |  |  | III.J.2. |
|  |  |  |  | TRS/H2S | Emissions Inventory | 0.1 | 0.2 |  |  | III.J.2. |
|  |  |  |  | VOC | Emissions Inventory | 0.1 | 0.3 |  |  | III.J.2. |

Notes:  *The “Equivalent Emissions” listed are for informational purposes only.

NA:  Not Applicable		EBA:  Established by Applicant

| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard Allowable Emissions Standard(s) lbs/hr TPY |  |  | Equivalent Emissions* |  | See Permit Condition(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | lbs/hr | TPY |  |
| -065 | Rail Pit |  | 8,760 | PM/PM10 | Emissions Inventory | 0.2 | 0.8 |  |  | III.J.2. |
|  |  |  |  | SO2 | Emissions Inventory | 0.3 | 1.2 |  |  | III.J.2. |
|  |  |  |  | TRS/H2S | Emissions Inventory | 0.1 | 0.6 |  |  | III.J.2. |
|  |  |  |  | VOC | Emissions Inventory | 0.1 | 0.3 |  |  | III.J.2. |
| -066 | Transfer Pit |  | 8,760 | PM/PM10 | Emissions Inventory | 0.1 | 0.4 |  |  | III.J.2. |
|  |  |  |  | SO2 | Emissions Inventory | 0.1 | 0.4 |  |  | III.J.2. |
|  |  |  |  | TRS/H2S | Emissions Inventory | 0.1 | 0.2 |  |  | III.J.2. |
|  |  |  |  | VOC | Emissions Inventory | 0.1 | 0.2 |  |  | III.J.2. |

Notes:  *The “Equivalent Emissions” listed are for informational purposes only.

NA:  Not Applicable		EBA:  Established by Applicant

| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard Allowable Emissions Standard(s) lbs/hr TPY |  |  | Equivalent Emissions* |  | See Permit Condition(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | lbs/hr | TPY |  |
| -060 | Rail Storage Tank |  | 8,760 | PM/PM10 | Emissions Inventory | 0.6 | 2.6 |  |  | III.J.2. |
|  |  |  |  | SO2 | Emissions Inventory | 0.5 | 2.1 |  |  | III.J.2. |
|  |  |  |  | TRS/H2S | Emissions Inventory | 0.2 | 1.0 |  |  | III.J.2. |
|  |  |  |  | VOC | Emissions Inventory | 0.3 | 1.2 |  |  | III.J.2. |
| -062 | No. 3 Storage Tank |  | 8,760 | PM/PM10 | Emissions Inventory | 0.6 | 1.1 |  |  | III.J.2. |
|  |  |  |  | SO2 | Emissions Inventory | 0.5 | 2.2 |  |  | III.J.2. |
|  |  |  |  | TRS/H2S | Emissions Inventory | 0.2 | 1.0 |  |  | III.J.2. |
|  |  |  |  | VOC | Emissions Inventory | 0.3 | 1.0 |  |  | III.J.2. |
|  | Total |  |  | PM/PM10 | Emissions Inventory | 1.8 | 6.0 |  |  | III.J.2. |
|  |  |  |  | SO2 | Emissions Inventory | 3.7 | 7.658 |  |  | III.J.2. |
|  |  |  |  | TRS/H2S | Emissions Inventory | 0.8 | 3.6 |  |  | III.J.2. |
|  |  |  |  | VOC | Emissions Inventory | 1.2 | 4.1 |  |  | III.J.2. |

Notes:  *The “Equivalent Emissions” listed are for informational purposes only.

NA:  Not Applicable		EBA:  Established by Applicant

| E.U. ID No. | Brief Description | Fuel(s) | Hrs/Yr | Pollutant Name | Standard Allowable Emissions Standard(s) lbs/hr TPY |  |  | Equivalent Emissions* |  | See Permit Condition(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | lbs/hr | TPY |  |
| -070 | Limestone Storage Silo/Rock Grinding |  | N/A | VE | 20% opacity | N/A | N/A | N/A | N/A | III.K.2. |
|  |  |  |  | PM | 0.03 gr/dscf @ 2700 dscfm | 0.7 | 3.0 |  |  | III.K.4. |
| -078 | GMAP Plant | NG, No. 6 or better fuel oil | N/A | F | 0.032 lb F/ton of P2O5 input | 2.40 | 10.5 | N/A | N/A | III.M.7. |
|  |  |  |  | PM | 0.0645 lb PM/ton of product | 9.68 | 42.4 |  |  | III.M.8. |

Notes:  *The “Equivalent Emissions” listed are for informational purposes only.		GMAP: Granular Mono Ammonium Phosphate

NA:  Not Applicable		EBA:  Established by Applicant