| E.U. ID No. | Brief Description | Fuel(s) | Pollutant Name | Emission Monitoring1 | Emission Control | Compliance Method EPA Method | Testing Time Frequency | Frequency Base Date 2 | Min. Compliance Test Duration | See Permit Condition(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -002, 003, 004, 042, 044 | Nos. 1, 2, 3, 4, 5 SAP |  | VE |  |  | 9 | FFY |  | 1 hour | III.A13.1 |
| -042, 044 | Nos. 4, 5 SAP |  | SO2 |  |  | 6 | FFY |  | 3 1-hour runs | III.A13.1 |
| -002, 003, 004, | Nos. 1, 2, 3 SAP |  | SO2 |  |  | CEMS | continuous |  | 24-hr rolling avg. for short term limit of lb/ton 24-hour (daily) block CEMS average for lbs/hour limit | III.A3.2&3.3 and III.A13.1 |
|  |  |  | H2SO4 acid mist |  |  | 8 | FFY |  | 3 1-hour runs | III.A13.1 |
|  |  |  | NOx |  |  | 7E | Every 5 years, prior to renewal |  | 3 1-hour runs | III.A.13.2. |
| -008, 017 | PAP East & West |  | F |  |  | 13A, 13B | FFY |  | 3 1-hour runs | III.B.5. |
| 039 | PAP No. 3 |  | F |  |  | 13A, 13B | FFY |  | 3 1-hour runs | III.C.5. |
| -048 | 30% Clarification Area (Area 10) |  | F |  |  | 13A, 13B | FFY |  | 3 1-hour runs | III.D.8. |
| -053 | Phos Acid Clarification and Storage Area |  | F |  |  | 13A, 13B | FFY |  | 3 1-hour runs | III.D.8. |
| -009 | DAP No. 1 |  | F |  |  | 13A, 13B | FFY |  | 3 1-hour runs | III.E.8. |
|  |  |  | PM |  |  | 5 | FFY |  | 3 1-hour runs | III.E.8. |
|  |  |  | VE |  |  | 9 | FFY |  | 1 hour | III.E.8. |
| -045, 046 | DAP No. 2 East Train, West Train |  | PM |  |  | 5 | FFY |  | 3 1-hour runs | F.11. & F.13.1. |
|  |  |  | SO2 |  |  | 6 | FFY |  | 3 1-hour runs | F.11. & F.13.1. |
|  |  |  | F |  |  | 13A, 13B | FFY |  | 3 1-hour runs | F.11. & F.13.1. |
|  |  |  | NOx |  |  | 7E | FFY |  | 3 1-hour runs | F.11. & F.13.1. |
|  |  |  | VE |  |  | 9 | FFY |  | 1 hour | F.11. & F.13.1. |
| -056 | DAP No. 2 – East Product Cooler |  | VE |  |  | 9 | FFY |  | 1 hour | III. F.11 & F.13.2. |
|  |  |  | PM |  |  | 5 | FFY |  | 3 1-hour runs | III. F.11 & F.13.2. |
|  |  |  | F |  |  | 13A, 13B | FFY |  | 3 1-hour runs | III. F.11 & F.13.2. |
| -047 | DAP No. 2 – West Product Cooler |  | VE |  |  | 9 | FFY |  | 1 hour | III. F.11 & F.13.3. |
|  |  |  | F |  |  | 13A, 13B | Every 5 years |  | 3 1-hour runs | III. F.11 & F.13.3. |
| -011 | MAP Prill Plant |  | F (Fluorides) |  |  | 13A or 13B | FFY |  | 3 1-hour runs | III.G.6 & III.G.8. |
|  |  |  | PM |  |  | 5 | FFY |  | 3 1-hour runs | III.G.6 & III.G.8. |
|  |  |  | VE |  |  | 9 | FFY |  | 1 hour | III.G.6 & III.G.8. |
| -055 | MAP Plant Cooler |  | VE |  |  | 9 | FFY |  | 1 hour | III.G.6 & III.G.9. |
| -015, 023, 024, 025, 026, 028, 030 & 052 | AFI Shipping, storage silos, storage, shipping, railcar loadout, Lime stone silo, Storage Silo – Southside, Limestone feed bin |  | VE |  |  | 9 | FFY |  | 30-min | III.H.6 & III.H.8. |
|  |  |  | PM |  |  | 5 | FFY, except for EU 030 (Not required for EU 030) |  | 3 1-hour runs | III.H.6 & III.H.8. |
| -027 | AFI Plant |  | PM |  |  | 5 | FFY |  | 3 1-hour runs | III.I.6 & III.I.8. |
|  |  |  | VE |  |  | 9 | FFY |  | 30 min | III.I.6 & III.I.8. |
| -070 | Limestone Storage Silo / Rock Grinding |  | VE |  |  | 9 | FFY |  | 30 min | III.K.5. & III.K.7. |
|  |  |  | PM |  |  | 5 | Every 5 years |  | 3 1-hour runs | III.K.5. & III.K.7. |
|  |  |  | Mass flow rate |  |  |  |  |  |  |  |
| -078 | GMAP |  | PM |  |  | 5 | Every 5 years |  | 3 1-hour runs | III.M.11. & III.M.14. |
|  |  |  | F (Fluorides) |  |  | 13A or 13B | FFY |  | 3 1-hour runs | III.M.11 & III.M.13 |

Notes:	*Testing date is once in Federal Fiscal Year (FFY); see Rule 62-297.310, F.A.C.

**CMS [=] continuous monitoring system