Initial Title V Air Operation Permit

Permit No. 1070049-001-AV

Applicant

The applicant for this project is Putnam County Sanitation Department.  The applicant’s responsible official and mailing address are:  Larry Gast, Solid Waste Director, Putnam County Sanitation Department, Putnam County Central Landfill, P.O. Box 2499, Palatka, Florida  32178.

Facility Description

The applicant operates the existing Putnam County Central Landfill, which is located in Putnam County at 140 County Landfill Road, Palatka, Florida.

The Putnam County Central Landfill is comprised of the closed Class I Phase I landfill, the active Phase II Class I landfill, and the closed Class III landfill.  The closed Class I Phase I landfill is an unlined landfill that accepted waste from 1972 until its final closure was completed in September 1991.  The closed Class III landfill accepted a combination of processed tires, horticultural and yard waste, and industrial waste from approximately 1992 to 2005.  The Class III landfill was officially closed in 2007.  The Phase II Class I landfill is a lined facility that began accepting municipal solid waste in 1991 and is still active.  In January 2014, a Solid Waste permit was issued that expanded the Phase II Class I landfill by 21.5 acres.  This expansion causes the facility to exceed the 2.5 million megagram design capacity threshold, making the landfill subject to the requirements of 40 CFR 60 Subpart WWW – Standards of Performance for Municipal Solid Waste Landfills.  Tier II sampling and analyses show the NMOC emission rate to be below the 50 megagram threshold requiring the installation of a collection and control system.  Asbestos containing material is also disposed in the landfill and these activities are subject to the requirements of 40 CFR 61 Subpart M (Set A).  This facility does not contain any cells that qualify as a bioreactor.

This facility also includes miscellaneous unregulated/insignificant emissions units and/or activities.

Project DESCRIPTION

The purpose of this permitting project is to issue an initial Title V permit for the above referenced facility.

Processing Schedule and Related Documents

Application for an Initial Title V Air Operation Permit received April 11, 2014

Primary Regulatory requirements

Standard Industrial Classification (SIC) Code:  4953 – Refuse Systems.

North American Industry Classification System (NAICS):  562212 – Solid Waste Landfill.

HAP:  The facility is not identified as a major source of hazardous air pollutants (HAP).

Title IV:  The facility operates units subject to the acid rain provisions of the Clean Air Act.

Title V:  The facility is a Title V major source of air pollution in accordance with Chapter 62-213, Florida Administrative Code (F.A.C.).

PSD:  The facility is a Prevention of Significant Deterioration (PSD)-major source of air pollution in accordance with Rule 62-212.400, F.A.C.

NSPS:  The facility operates units subject to the New Source Performance Standards (NSPS) of 40 Code of Federal Regulations (CFR) 60.

CAM:  Compliance Assurance Monitoring (CAM) does not apply to any of the units at the facility.

GHG:  The facility is identified as a major source of greenhouse gas (GHG) pollutants.

PROJECT REVIEW

This project issues the initial Title V air operation permit for an existing minor source which triggered the Title V permitting requirement in 40 CFR 60 Subpart WWW due to an increase in design capacity.  The existing facility consists of closed Class I Phase I and Class III waste disposal areas and an active Class I Phase II waste disposal area.  The vertical and lateral, 21.5 acres, expansion of the Class I Phase II landfill increases the total landfill permitted design capacity to approximately 4.0 million megagrams.

Conclusion

This Initial Title V air operation permit is issued under the provisions of Chapter 403, Florida Statues (F.S.), and Chapters 62-4, 62-210, and 62-213, F.A.C.