Title V Air Operation Permit Renewal

Permit No. 1130172-007-AV

Applicant

The applicant for this project is Santa Rosa County Board of County Commissioners.  The applicant’s responsible official and mailing address are:  Ron Hixson, Environmental Supervisor, Santa Rosa County, Santa Rosa County Central Landfill, 6065 Old Bagdad Highway, Milton, Florida  32583.

Facility Description

This facility consists of a municipal solid waste landfill (Class I and Class III); a stationary air curtain incinerator; and a voluntarily installed gas collection system with an open candlestick flare.

The landfill area contains a Class I Municipal Solid Waste (MSW) Landfill and a Class III Landfill.  The Class I Landfill disposal area started operating in 1978 and is approximately 39.03 acres, of which 18.02 acres are new and 21.01 acres high-rise over an existing closed Class I Landfill with a single liner.  The Class III Landfill disposal area is approximately 44.27 acres of which 24.47 acres are operating and 19.8 acres are new.  The landfill design capacity is 2.56 million Mg by mass as reported in the Initial Design Capacity Report dated October 5, 2006.

The air curtain incinerator is refractory-lined chamber with a Phelps Fan Company Model B-61 air curtain system.  The air supply system is a squirrel case type fan driven by a diesel engine.  The refractory-lined pit measuring 10 feet wide by 30 feet long by 10 feet deep is a modular based design and was fabricated by Simonds Manufacturing Corporation.

The open “candlestick” flare is designed to continuously burn landfill gas.  The flare is manufactured by LFG Specialties, L.L.C., utility flare Model No. PCF62216.  The maximum flow rate of landfill gas to the flare is 750 SCFM.  The flare’s height is 22 feet with an exit diameter of 1.5 feet.

Other activities occurring at the landfill includes wood waste processing and temporary storage, white goods temporary storage, household hazardous waste collection and temporary storage, used oil collection and temporary storage, diesel fuel storage tanks, scrap tire temporary storage, asbestos disposal pit, equipment maintenance building with hydraulic and motor oil storage, electronic waste collection and temporary storage, oil, gas, and battery removal from lawn mowers and similar equipment, and lead-acid battery temporary storage.

regulated Emissions Unit IDentification Numbers and descriptions

| EU No. | Brief Description |
| --- | --- |
| 001 | Air Curtain Incinerator |
| 002 | Open Flare |
| 003 | Municipal Solid Waste Landfill |

Applicable Regulations

Based on the initial Title V air operation permit application received August 21, 2014, this facility is not a major source of hazardous air pollutants (HAP).  A summary of applicable regulations is shown in the following table.

| Regulation | ## EU No(s). |
| --- | --- |
| Federal Rule Citations |  |
| 40 CFR 60, Subpart A, NSPS General Provisions | 001, 003 |
| 40 CFR 60, Subpart WWW | 003 |
| 40 CFR 60, Subpart DDDD | 001 |
| 40 CFR 61 Subpart A – General Provisions - NESHAP | 003 |
| 40 CFR 61 Subpart M (Set A) – NESHAP For Asbestos | 003 |
| State Rule Citations |  |
| Rule 62-4, Florida Administrative Code (F.A.C.) (Permitting Requirements) | 001, 002, 003 |
| Rule 62-204, F.A.C. (Ambient Air Quality Requirements, PSD Increments, and Federal Regulations Adopted by Reference) | 001, 002, 003 |
| Rule 62-296.320(4), F.A.C. (Unconfined Particulate Matter and Visible Emissions) | 001, 002, 003 |
| Rule 62-210, F.A.C. (Permits Required, Public Notice, Reports, Stack Height Policy, Circumvention, Excess Emissions, and Forms) | 001, 002, 003 |
| Rule 62-213, F.A.C. (Title V Air Operation Permits for Major Sources of Air Pollution) | 001, 002, 003 |
| Rule 62-297, F.A.C. (Test Methods and Procedures, Continuous Monitoring Specifications, and Alternate Sampling Procedures) | 001, 002, 003 |

This facility also includes miscellaneous unregulated/insignificant emissions units and/or activities.

Project DESCRIPTION

The purpose of this permitting project is to issue an initial Title V permit for the above referenced facility.

Processing Schedule and Related Documents

Application for an Initial Title V Air Operation Permit received August 21, 2014.

Primary Regulatory requirements

Standard Industrial Classification (SIC) Code:  4953 – Refuse Systems.

North American Industry Classification System (NAICS):  562212 – Solid Waste Landfill.

HAP:  The facility is not identified as a major source of hazardous air pollutants (HAP).

Title V:  The facility is a Title V major source of air pollution in accordance with Chapter 62-213, Florida Administrative Code (F.A.C.).

NSPS:  The facility operates units subject to the New Source Performance Standards (NSPS) of 40 Code of Federal Regulations (CFR) 60.

NESHAP:  The facility operates units subject to the National Emissions Standards for Hazardous Air Pollutants (NESHAP) of 40 Code of Federal Regulations (CFR) 61.

CAM:  Compliance Assurance Monitoring (CAM) does not apply to any of the units at the facility.

GHG:  The facility is not identified as a major source of greenhouse gas (GHG) pollutants.

PROJECT REVIEW

This project issues the initial Title V air operation permit for an existing minor source which triggered the Title V permitting requirement in 40 CFR 60 Subpart WWW due to an increase in design capacity.  The Class I Landfill disposal area started operating in 1978 and is approximately 39.03 acres, of which 18.02 acres are new and 21.01 acres high-rise over an existing closed Class I Landfill with a single liner.  The Class III Landfill disposal area is approximately 44.27 acres of which 24.47 acres are operating and 19.8 acres are new.  The vertical and lateral expansion of the Class I Phase II landfill increased the total landfill permitted design capacity to approximately 2.56 million megagrams.

Conclusion

This Initial Title V air operation permit is issued under the provisions of Chapter 403, Florida Statues (F.S.), and Chapters 62-4, 62-210, and 62-213, F.A.C.