TECHNICAL EVALUATION

&

PRELIMINARY DETERMINATION

APPLICANT

Boston Whaler, Inc.

100 Whaler Way

Edgewater, Florida

Edgewater facility

Facility ID No. 1270063

PROJECT

Project No. 1270063-007-AV

Application for Title V Air Operation Permit Revision

Project Name: Title V Air Operation Permit Revision

COUNTY

Volusia County, Florida

PERMITTING AUTHORITY

Florida Department of Environmental Protection

Air Resource Management

Central District Office

3319 Maguire Blvd., Ste. 232, Orlando, FL  32803-3767

March 6, 2015

Prepared by Stephen Amirault, Project Engineer

General Project INFORMATION

Air Pollution Regulations

Projects at stationary sources with the potential to emit air pollution are subject to the applicable environmental laws specified in Section 403 of the Florida Statutes (F.S.).  The statutes authorize the Department of Environmental Protection (Department) to establish regulations regarding air quality as part of the Florida Administrative Code (F.A.C.), which includes the following applicable chapters:  62-4 (Permits); 62-204 (Air Pollution Control – General Provisions); 62-210 (Stationary Sources – General Requirements); 62-212 (Stationary Sources – Preconstruction Review); 62-213 (Operation Permits for Major Sources of Air Pollution); 62-296 (Stationary Sources - Emission Standards); and 62-297 (Stationary Sources – Emissions Monitoring).  Specifically, air construction permits are required pursuant to Rules 62-4, 62-210 and 62-212, F.A.C.

In addition, the U. S. Environmental Protection Agency (EPA) establishes air quality regulations in Title 40 of the Code of Federal Regulations (CFR).  Part 60 specifies New Source Performance Standards (NSPS) for numerous industrial categories.  Part 61 specifies National Emission Standards for Hazardous Air Pollutants (NESHAP) based on specific pollutants.  Part 63 specifies NESHAP based on the Maximum Achievable Control Technology (MACT) for numerous industrial categories.  The Department adopts these federal regulations on a quarterly basis in Rule 62-204.800, F.A.C.

Glossary of Common Terms

Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of this permit.

I.  	Project Description:

A.	Applicant:

Ms. Rebecca Crews, Vice President of Manufacturing Operations

Boston Whaler, Inc.

100 Whaler Way

Edgewater Florida, 32141

rcrews@whaler.com

B.	Professional Engineer:

Selina Cantelou, P. E.

Cantelou Consulting, Inc.

360 Baytree Drive

Melbourne, Florida, 32940

scantelou@aol.com

C.	Project Location:

100 Whaler Way

Edgewater Florida, 32141

D. 	Project Summary:

The purpose of this permit is to revise the Title V air operation permit for the above referenced facility.  Changes include incorporating the addition of an emergency fire pump (Emission Unit EU 016) to the facility, as well as applicable regulatory updates and changes to the permit template format.

The installation of this emergency fire pump is exempt from Construction Permitting Requirements in accordance with Rule 62-210.300(3) and the Categorical Exemption for Stationary Reciprocating Internal Combustion Engines (RICE) provided at Rule 62-210.300(3)(a)35.

However, such stationary RICE are subject to the permitting requirements of Chapter 62-213, F.A.C., if contained within a Title V source.

Since the RICE is subject to an applicable unit specific requirement (in this case, the federal standards, 40 CFR 63 Subpart ZZZZ, and 40 CFR 60 Subpart IIII), the RICE is required to be incorporated into the Title V Operation Permit within 180 days of commencing operation.

E.	Application Information:

Application Received: 02/16/15

Application Complete Determination: 03/02/15

No additional information was requested.

II.	PSD Applicability for Project

As provided in the application total project emissions will not exceed the PSD significant emissions rates; therefore, the project is not subject to PSD preconstruction review.

III.	Rule Applicability

This project is subject to the preconstruction review requirements of Chapter 403, Florida Statutes and Chapters 62-204 through 62-297, Florida Administrative Code (F.A.C.), as indicated below.

| Subject to: | Y/N | Comments |
| --- | --- | --- |
| Rule 62-210.300, F.A.C. – Stationary Source General Requirements | Y | Not exempt from general permitting requirements |
| Rule 62-212.400, F.A.C. - Prevention of Significant Deterioration | N | Facility is not a PSD major source |
| Rule 62-296.320(4), F.A.C. - General Particulate Emission Limiting Standards | Y | Boat manufacturing operations could be a source of unconfined particulate matter emissions |
| Rules 62-296.320(1) and (2), F.A.C. - General Pollutant Emission Limiting Standards (VOCs and Odor) | Y | Boat manufacturing operations are a source of VOC and odors |
| Rule 62-296.400, F.A.C. - Stationary Source Emission Standards | N | There is no applicable source category |
| Rule 62-296.500, F.A.C. - Reasonably Available Control Technology (VOC) | N | Volusia County is an attainment area for ozone |
| Rule 62-296.700, F.A.C. - Reasonably Available Control Technology (PM) | N | There is no applicable source category |
| Rule 62-204.800, F.A.C. - Standards of Performance for New Stationary Sources (NSPS– 40 CFR 60) | Y | The new emergency fire pump (Emission Unit EU 016) added to the facility as part of this revision, is subject to the NSPS 40 CFR 60, Subpart IIII, Standards of Performance for Stationary Compression Ignition Internal Combustion Engines for Stationary CI RICE. |
| Rule 62-204.800, F.A.C. National Emission Standard for Hazardous Air Pollutants (NESHAPS – 40 CFR 61) | Y | Boat manufacturing operations are subject to 40 CFR 63, Subpart VVVV – National Emission Standards for Hazardous Air Pollutants for Boat Manufacturing. The new emergency fire pump (EU 016) added to the facility as part of this revision, is subject to 40 CFR 63, Subpart ZZZZ, NESHAP for Stationary RICE; however, meeting the requirements of 40 CFR 60, Subpart IIII, satisfies compliance with the requirements of Subpart ZZZZ. |
| Chapter 62-213, F.A.C. - Operation Permits for Major Sources of Air Pollution | Y | Facility is a Title V source for volatile organic compounds (VOCs) and a major source of hazardous air pollutants (HAPs) |

IV.	Summary of Emissions

| Pollutant | EU No. and brief description | Allowable Emissions (tons per year) |
| --- | --- | --- |
| VOC | Facility wide | 249 |
| Total HAP | Facility wide | 249 |
| Individual HAP (styrene) | Facility wide | Less than 249 |
| VE | Facility wide | Less than 20% |

V.	Federal NSPS and/or NESHAP Provisions

Boat manufacturing operations are subject to 40 CFR 63, Subpart VVVV – National Emission Standards for Hazardous Air Pollutants for Boat Manufacturing.

The new emergency fire pump (Emission Unit EU 016) added to the facility as part of this revision, is subject to the NSPS 40 CFR 60, Subpart IIII, Standards of Performance for Stationary Compression Ignition Internal Combustion Engines for Stationary CI RICE.  The fire pump is also subject to 40 CFR 63, Subpart ZZZZ, NESHAP for Stationary RICE.  However, meeting the requirements of 40 CFR 60, Subpart IIII, satisfies compliance with the requirements of Subpart ZZZZ.

VI.	Conclusions

The emission limits proposed by the applicant will meet all of the requirements of Chapters 62-204 through 297, F.A.C.

The General and Specific Conditions listed in the proposed permit (attached) will assure compliance with all the applicable requirements of Chapters 62-204 through 297, F.A.C.

VII.	Preliminary Determination

The Department makes a preliminary determination that the proposed project will comply with all applicable state and federal air pollution regulations as conditioned by the draft permit.  This determination is based on a technical review of the complete application, reasonable assurances provided by the applicant, and the conditions specified in the draft permit.  Additional details of this analysis may be obtained by contacting the project engineer, Stephen Amirault, at the Florida Department of Environmental Protection, Waste & Air Resource Programs, Central District Office, 3319 Maguire Blvd., Suite 232, Orlando, Florida 32803-3767, 407/897-4100.