### TECHNICAL EVALUATION

AND

PRELIMINARY DETERMINATION

#### APPLICANT

Orlando Utilities Commission

Reliable Plaza

100 West Anderson Street

Orlando, Florida  32801

Indian River Plant

Facility ID No. 0090008

#### PROJECT

# Permit Revision No. 0090008-009-AC

Extended Cold Shutdown

#### COUNTY

Brevard County, Florida

PERMITTING AUTHORITY

Florida Department of Environmental Protection

Division of Air Resource Management

Office of Permitting and Compliance

2600 Blair Stone Road, MS #5505

Tallahassee, Florida  32399-2400

July 2, 2014

- General Project INFORMATION

  - Air Pollution Regulations

Projects at stationary sources with the potential to emit air pollution are subject to the applicable environmental laws specified in Section 403 of the Florida Statutes (F.S.).  The statutes authorize the Department of Environmental Protection (Department) to establish regulations regarding air quality as part of the Florida Administrative Code (F.A.C.), which includes the following applicable chapters:  62-4 (Permits); 62-204 (Air Pollution Control – General Provisions); 62-210 (Stationary Sources – General Requirements); 62-212 (Stationary Sources – Preconstruction Review); 62-213 (Operation Permits for Major Sources of Air Pollution); 62-296 (Stationary Sources - Emission Standards); and 62-297 (Stationary Sources – Emissions Monitoring).  Specifically, air construction permits are required pursuant to Rules 62-4, 62-210 and 62-212, F.A.C.

In addition, the U. S. Environmental Protection Agency (EPA) establishes air quality regulations in Title 40 of the Code of Federal Regulations (CFR).  Part 60 specifies New Source Performance Standards (NSPS) for numerous industrial categories.  Part 61 specifies National Emission Standards for Hazardous Air Pollutants (NESHAP) based on specific pollutants.  Part 63 specifies NESHAP based on the Maximum Achievable Control Technology (MACT) for numerous industrial categories.  The Department adopts these federal regulations on a quarterly basis in Rule 62-204.800, F.A.C.

  - Facility Description and Location

This facility is an existing power plant, which is categorized under Standard Industrial Classification Code No. 4911.  .  The existing Indian River Plant is located at US 1 and Kings Highway, Titusville, in Brevard County.  UTM Coordinates are:  Zone 17; 521.3 km East and 3151.7 km North.  Latitude is:  28° 29’ 36” North; and, Longitude is:  80° 46’ 57” West.

This site is in an area that is in attainment (or designated as unclassifiable) for all air pollutants subject to state and federal Ambient Air Quality Standards (AAQS).

The existing facility consists primarily of three boilers and four combustion turbines.  Emissions units 001, 002 and 003 consist of three Combustion Engineering Steam Generators (boilers), with a combined electrical generation rating of 603 megawatt (MW) while firing natural gas or propane gas.  These emissions units are not equipped with add-on pollution control devices.  Unit 001 began operation on February 1, 1960; Unit 002 on September 1, 1964; and Unit 003 on February 1, 1974.

Emissions units 004 and 007 (combustion turbines A and B) consist of simple cycle General Electric Frame 6 combustion turbines, each with a 35 MW rating.  Although the turbines primarily fire natural gas, distillate oil may be fired during periods of curtailed or uneconomical natural gas supply.  Nitrogen oxides emissions are reduced by using water injection.  Both turbines began commercial operation on August 1, 1990.

Emissions units 005 and 006 (combustion turbines C and D) consist of simple cycle Westinghouse Model Number 501-D5 combustion turbines, each with a 129 MW rating.  The turbines primarily fire natural gas. Distillate oil may be fired during periods of curtailed or uneconomical natural gas supply.  Nitrogen oxides emissions are controlled by water injection.  Both turbines began commercial operation on November 1, 1991.

  - Facility Regulatory Categories

- The facility is a major source of hazardous air pollutants (HAP).

- The facility is a Title V major source of air pollution in accordance with Chapter 213, F.A.C.

- The facility is major stationary source in accordance with Rule 62-212.400, F.A.C., for the Prevention of Significant Deterioration (PSD) of Air Quality.

  - Project Description

OUC has confirmed that the three electric steam generating units (EU001, EU002 and EU003) are currently "not commercially available".  Another term typically used to describe this status is “extended cold shutdown”. OUC has requested the Department’s concurrence with the practice that routine compliance activities such as fuel sampling, emissions monitoring, and stack testing are not required while the facility is not operating.  It was further proposed that the Department would be notified at least 60 days before resuming commercial operation.

- OUC has indicated that they will continue to submit quarterly reports, semi-annual monitoring reports, annual statements of compliance, annual operating reports, and the annual Title V fee submissions. OUC requested that an air construction permit revision and the concurrent Title V permit renewal reflect the extended cold shutdown status of these units.

  - Application Processing Schedule

Application for an Air Construction Permit Revision received on May 20, 2014.  Application was deemed complete.

- PSD Applicability

  - General PSD Applicability

The Department regulates major stationary sources in accordance with Florida’s PSD program pursuant to Rule 62-212.400, F.A.C.  PSD preconstruction review is required in areas that are currently in attainment with the state and federal ambient air quality standards (AAQS) or areas designated as “unclassifiable” for these regulated pollutants.  Commonly addressed PSD pollutants in the power industry include: carbon monoxide (CO), NOX, particulate matter (PM), PM with a mean diameter of 10 microns or less (PM10), PM with a mean diameter of 2.5 microns or less (PM2.5), sulfur dioxide (SO2), volatile organic compounds (VOC), lead (Pb), fluorides (F), sulfuric acid mist (SAM), and mercury (Hg).  According to state and federal rules, six greenhouse gases (GHG), are also subject to PSD review.

Additional PSD pollutants that are more common to certain other industries include: hydrogen sulfide (H2S), TRS including H2S, reduced sulfur compounds (RSC) including H2S, municipal waste combustor (MWC) organics measured as total tetra- through octa-chlorinated dibenzo-p-dioxins and dibenzofurans (dioxin/furan), MWC metals measured as PM; MWC acid gases measured as SO2 and HCl, and MSW landfill emissions as non-methane organic compounds (NMOC).

As defined in Rule 62-210.200(189)(a)1, F.A.C., a stationary source is a “major stationary source” (major PSD source) if it emits or has the potential to emit (PTE):

- 250 tons per year (TPY) or more of any PSD pollutant; or

- 100 TPY or more of any PSD pollutant and the facility belongs to one of the 28 listed PSD major facility categories.

The list given in the citation includes the category of “fossil fuel-fired steam electric plants of more than 250 million British thermal units per hour heat input”.  This category applies to the Indian River Plant before and after the proposed project.  The Indian River Plant is a major stationary source based on actual emissions of and potential to emit 100 TPY or more of several individual PSD pollutants.

For major stationary sources such as the Indian River Plant, PSD applicability for modification projects is based on thresholds known as the significant emission rates (SER) as defined in Rule 62-210.200(274), F.A.C.  Any “net emissions increase” as defined in Rule 62-210.200(204), F.A.C. of a PSD pollutant from the project that equals or exceeds the respective SER is considered “significant”.  SER also means any emissions rate or any net emissions increase of a PSD pollutant associated with a major stationary source or major modification which would construct within 10 km of a Class I area and have an impact on such area equal to or greater than 1 gram per cubic meter, 24-hour average.

Although a facility may be “major” (i.e. emits or has the potential to emit 100 or 250 TPY as applicable) for only one PSD pollutant, a project must include BACT controls for any PSD pollutant that exceeds the corresponding SER given in Table 1.

Table 1 – List of SER by PSD-Pollutant. 1

| Pollutant | SER (TPY) | Pollutant | SER (TPY) |
| --- | --- | --- | --- |
| CO | 100 | NOX | 40 |
| PM/PM10/PM2.5 | 25/15/10 | Ozone (VOC) 2 | 40 |
| PM2.5 (NOX) | 40 | PM2.5 (SO2) | 40 |
| Ozone (NOX) 2 | 40 | SAM | 7 |
| SO2 | 40 | Pb | 0.6 |
| Hg | 0.1 | GHGs | 0 3 |
| - Excluding fluoride and pollutants specific to the Pulp and Paper industry, MWCs, MSW landfills. - Ozone (O3) is regulated by its precursors (VOC and NOX). PSD for PM2.5 can be triggered by its precursors (NOX and SO2). - Pursuant to 40 CFR 52.21(b)(23)(ii), pollutants with no SER listed at 40 CFR 52.21(b)(23)(i) have a SER of zero tons/year. |  |  |  |

  - PSD Applicability for Project

The project will revise permit conditions to reflect the extended cold shutdown status of Units 001, 002 and 003 at the facility.  There will be no emissions increases and the project is not subject to PSD preconstruction review.

- Department review

  - Response to Requested Revisions

The applicant requested changes to previous air construction permits as explained below:

OUC has requested the Department’s concurrence with the practice that routine compliance activities such as fuel sampling, emissions monitoring, and stack testing are not required while the facility is not operating (i.e., under an “extended cold shutdown”.  The Department agrees with this revision request.  To effect these changes, the specific condition of two prior air construction permits are revised as follows:

- Specific Condition 2. of 0090008-008-AC is changed as follows:

2.  Method of Operation – Fuels.  The only fuels allowed to be fired in Boilers 1, 2 & 3 are natural gas and propane gas containing a maximum of 2 grains of sulfur per 100 standard cubic feet.  Compliance with this limitation shall be demonstrated through information provided by the fuel vendor(s).  This limitation supersedes all previously issued permits or established conditions (whether issued to OUC, GenOn or RRI Energy Florida, LLC) that allowed the firing of No. 2 fuel oil, No. 6 fuel oil and on-specification used oil in these boilers.

Under “extended cold shutdown” status for the production suspension:  Fuel shall be sampled on the last day fuel is fired and shall resume on the first day fuel is fired following the period of not operating.

[Application No. 0090008-008-AC]

- Specific Condition 2. of 0090196-010-AC is changed as follows:

2.  Fuel Monitoring:  The permittee shall calibrate, operate and maintain devices to continuously monitor and record the fuel flow rate and heat input rates.

Under “extended cold shutdown” status for the production suspension:  Fuel will be sampled on the last day fuel is fired and will resume on the first day fuel is fired following the period of not operating.

[Application No. 0090196-010-AC and Rule 62-4.070(3), F.A.C.]

- Preliminary Determination

The Department makes a preliminary determination that the proposed project will comply with all applicable state and federal air pollution regulations as conditioned by the draft permit.  This determination is based on a technical review of the complete application, reasonable assurances provided by the applicant, and the conditions specified in the draft permit.  No air quality modeling analysis is required because the project does not result in a significant increase in emissions.

Tom Cascio is the project engineer responsible for reviewing the application and drafting the permit revision.  Additional details of this analysis may be obtained by contacting him by telephone at 850/717-9077 or by e-mail at tom.cascio@dep.state.fl.us in the Department’s Office of Permitting and Compliance at Mail Station #5505, 2600 Blair Stone Road, Tallahassee, Florida  32399-2400.