Florida Department of Environmental Protection

Division of Air Resource Management, Office of Permitting and Compliance

# Draft Air Permit No. 0110037-011-AC / PSD-FL-423

Florida Power & Light Company (FPL), Lauderdale Plant

Broward County, Florida

Applicant:  The applicant for this project is the Florida Power & Light Company (FPL).  The applicant’s authorized representative and mailing address is:  Mr. Randall R. LaBauve, Vice President of Environmental Services, Florida Power & Light Company (FPL), 700 Universe Boulevard, JES/JB, Juno Beach, Florida 33408.

Facility Location:  The existing Lauderdale Plant is located within the city of Dania Beach in Broward County, Florida.

Project:  The project involves the replacement of 24 existing gas turbines (GT) peaking units (GT1 to GT24) at the FPL Lauderdale Plant.  The 24 GTs are composed of two banks of 12 each, with each bank having a nominal capacity of 504 megawatts (MW).  In addition, one bank of 12 existing GT peaking units (GT1 through GT12) at the FPL Port Everglades Plant will also be retired as a result of this project.  The 36 GTs will be replaced with five nominal 200 MW combustion turbines (CTs) at the Lauderdale Plant to provide equivalent peaking resources while reducing emissions and providing better efficiency.  The new CTs will be designated Units 6A through 6E at the Lauderdale Plant.  There will be no overlap of the operation between the existing GTs and new CTs and a condition of the permit will so stipulate.  The CTs will fire natural gas and ultra low sulfur (ULSD) distillate fuel oil.  FPL has requested that each CT be allowed to operate 3,300 hours per year (hr/yr) at base load conditions, including up to 500 hr/yr while utilizing ULSD fuel oil.

FPL has indicated that they will install four nominal 3,100 kilowatt reciprocating internal combustion engine (RICE) emergency generators firing ULSD fuel oil for black start capability, i.e., these emergency generators will be used when electrical power is not available to start the CTs.  This primarily would occur during catastrophic events such as hurricanes.  The generators would be limited to 100 hr/yr of operation for maintenance and reliability testing.  Alternatively, FPL has requested that two of the existing GTs may remain on-site to provide the necessary black start capability.  These GTs would also be limited to 100 hr/yr of operation.  In addition, the project will add a 300 horsepower fire pump engine that uses ULSD fuel oil.  The fire pump engine will also be limited to 100 hr/yr of operation for maintenance and reliability testing.  Finally, the Lauderdale project will add two 3-million gallon ULSD fuel oil storage tanks.

Based on the air permit application submitted by FPL, and taking into account the reduction in emissions from the retired and replaced GT units, the project will result in emissions increases of:  475 tons per year carbon monoxide (CO); 749 tons per year of nitrogen oxides (NOX); 132 tons per year of particulate matter (PM); 132 tons per year of PM with a mean diameter of 10 micrometer (µm) or less (PM10); 132 tons per year of PM with a mean diameter of 2.5 µm or less (PM2.5); 34 tons per year of sulfur dioxide (SO2); and 70 tons per year of volatile organic compounds (VOC).  As defined in Rule 62-210.200, F.A.C., the project results in significant net emissions increases for CO, NOX, VOC, PM, PM10 and PM2.5 thereby requiring a Best Available Control Technology (BACT) determination for these pollutants.  In addition, the project triggers PSD for greenhouse gases and the applicant has submitted a PSD GHG application to EPA Region 4 in Atlanta.

The Department is required to determine the BACT and evaluate the applicant’s Air Quality Analysis regarding ambient impacts due to the project.  The Department’s preliminary BACT determinations for the pollutants listed in the previous paragraph are based on:  Lean premix Dry Low NOX technology while firing natural gas and water injection while firing fuel oil to control NOX emissions; good combustion practices to control CO and VOC emissions; and clean, low sulfur fuels to control to control PM/PM10/PM2.5 emissions.  Low sulfur fuels provide the added benefit of controlling SO2 and sulfuric acid mist emissions.

The applicant performed ambient air quality analyses as required.  Based on the modeling results, the maximum predicted ambient air quality impacts for both PM10 and PM2.5 exceeded the Class I significant impact levels (SILs) applicable at the nearest PSD Class I area; the Everglades National Park.  Consequently, comprehensive multi-source modeling was necessary to show that concentrations of these pollutants did not exceed the PSD Class I increments.  Modeling for Class I increment consumption in terms of micrograms per cubic (μg/m3) meter percent (%) was conducted for these pollutants and for the relevant averaging periods as detailed below:

| Pollutant | Averaging Time | Allowable Increment (μg/m3) | Increment (μg/m3) | Consumed Percent |
| --- | --- | --- | --- | --- |
| PM10 | 24-hour | 4.0 | 1.29 | 32.3 |
| PM2.5 | 24-hour | 2.0 | 0.16 | 8.0 |

The maximum predicted ambient air quality impacts are greater than the respective SILs applicable to the PSD Class II areas in Broward County nearest the project for the following pollutants:  PM2.5 and nitrogen dioxide (NO2).  There is no established PSD increment for the 1-hour NO2 standard but an increment analysis was required for the 24-hour PM2.5 standard.  The combined impacts of all increment-consuming sources in the area of the new source cannot exceed the PSD increment.  Modeling for increment consumption for this pollutant and for the specified averaging period is detailed below:

| Pollutant | Averaging Time | Allowable Increment (μg/m3) | Increment (μg/m3) | Consumed Percent |
| --- | --- | --- | --- | --- |
| PM2.5 | 24-hour | 9.0 | 2.0 | 22.2 |

Based on the analyses, emissions from the project will not cause, or significantly contribute to, a violation of any ambient air quality standards or applicable increments.

Permitting Authority:  Applications for air construction permits are subject to review in accordance with the provisions of Chapter 403, Florida Statutes (F.S.) and Chapters 62-4, 62-210 and 62-212, F.A.C.  The proposed project is not exempt from air permitting requirements and an air permit is required to perform the proposed work.  The Division of Air Resource Management’s (DARM) Office of Permitting and Compliance is the Permitting Authority responsible for making a permit determination for this project.  The Permitting Authority’s physical address is:  2600 Blair Stone Road, Tallahassee, Florida.  The Permitting Authority’s mailing address is:  2600 Blair Stone Road, MS #5505, Tallahassee, Florida 32399-2400.  The Permitting Authority’s telephone number is 850/717-9000.

# Project File:  A complete project file is available for public inspection during the normal business hours of 8:00 a.m. to 5:00 p.m., Monday through Friday (except legal holidays), at address indicated above for the Permitting Authority.  The complete project file includes the Draft Permit, the Technical Evaluation and Preliminary Determination, the application, and the information submitted by the applicant, exclusive of confidential records under Section 403.111, F.S.  Interested persons may contact the Permitting Authority’s project review engineer for additional information at the address and phone number listed above.  In addition, electronic copies of these documents are available on the following web site by entering draft permit number:  http://www.dep.state.fl.us/air/emission/apds/default.asp.

Notice of Intent to Issue Air Permit:  The Permitting Authority gives notice of its intent to issue an air permit to the applicant for the project described above.  The applicant has provided reasonable assurance that operation of the proposed equipment will not adversely impact air quality and that the project will comply with all applicable provisions of Chapters 62-4, 62-204, 62-210, 62-212, 62-296 and 62-297, F.A.C.  The Permitting Authority will issue a Final Permit in accordance with the conditions of the proposed Draft Permit unless a timely petition for an administrative hearing is filed under Sections 120.569 and 120.57, F.S. or unless public comment received in accordance with this notice results in a different decision or a significant change of terms or conditions.

Comments:  The Permitting Authority will accept written comments concerning the proposed Draft Permit and requests for a public meeting for a period of 30 days from the date of publication of the Public Notice.  Written comments must be received by the Permitting Authority by close of business (5:00 p.m.) on or before the end of this 30-day period.  In addition, if a public meeting is requested within the 30-day comment period and conducted by the Permitting Authority, any oral and written comments received during the public meeting will also be considered by the Permitting Authority.  If timely received comments result in a significant change to the Draft Permit, the Permitting Authority shall revise the Draft Permit and require, if applicable, another Public Notice.  All comments filed will be made available for public inspection.

Petitions:  A person whose substantial interests are affected by the proposed permitting decision may petition for an administrative hearing in accordance with Sections 120.569 and 120.57, F.S. Petitions filed by the applicant or any of the parties listed below must be filed within 14 days of receipt of this written notice of Intent to Issue Air Permit.  Petitions filed by any persons other than those entitled to written notice under Section 120.60(3), F.S., must be filed within 14 days of publication of the attached Public Notice or within 14 days of receipt of this written notice of Intent to Issue Air Permit, whichever occurs first.  Under Section 120.60(3), F.S., however, any person who asked the Permitting Authority for notice of agency action may file a petition within 14 days of receipt of that notice, regardless of the date of publication.  A petitioner shall mail a copy of the petition to the applicant at the address indicated above, at the time of filing.  A petition for administrative hearing must contain the information set forth below and must be filed (received) with the Agency Clerk in the Office of General Counsel, 3900 Commonwealth Boulevard, MS 35, Tallahassee, Florida 32399-3000, Agency.Clerk@dep.state.fl.us, before the deadline.  The failure of any person to file a petition within the appropriate time period shall constitute a waiver of that person’s right to request an administrative determination (hearing) under Sections 120.569 and 120.57, F.S., or to intervene in this proceeding and participate as a party to it.  Any subsequent intervention (in a proceeding initiated by another party) will be only at the approval of the presiding officer upon the filing of a motion in compliance with Rule 28-106.205, F.A.C.

A petition that disputes the material facts on which the Permitting Authority’s action is based must contain the following information: (a) The name and address of each agency affected and each agency’s file or identification number, if known; (b) The name, address, any email address, telephone number and any facsimile number of the petitioner; the name, address any email address, telephone number, and any facsimile number of the petitioner’s representative, if any, which shall be the address for service purposes during the course of the proceeding; and an explanation of how the petitioner’s substantial interests will be affected by the agency determination; (c) A statement of when and how each petitioner received notice of the agency action or proposed decision; (d) A statement of all disputed issues of material fact.  If there are none, the petition must so state; (e) A concise statement of the ultimate facts alleged, including the specific facts the petitioner contends warrant reversal or modification of the agency’s proposed action; (f) A statement of the specific rules or statutes the petitioner contends require reversal or modification of the agency’s proposed action including an explanation of how the alleged facts relate to the specific rules or statutes; and, (g) A statement of the relief sought by the petitioner, stating precisely the action the petitioner wishes the agency to take with respect to the agency’s proposed action.  A petition that does not dispute the material facts upon which the Permitting Authority’s action is based shall state that no such facts are in dispute and otherwise shall contain the same information as set forth above, as required by Rule 28-106.301, F.A.C.

Because the administrative hearing process is designed to formulate final agency action, the filing of a petition means that the Permitting Authority’s final action may be different from the position taken by it in this Public Notice of Intent to Issue Air Permit.  Persons whose substantial interests will be affected by any such final decision of the Permitting Authority on the application have the right to petition to become a party to the proceeding, in accordance with the requirements set forth above.

Mediation:  Mediation is not available in this proceeding.