Permittee

| United States Navy 6801 Roosevelt Boulevard Jacksonville, Florida 3221 Authorized Representative: Mr. Ryan Dexter, Commanding Officer | Air Permit No. 0310215-076-AC Permit Expires: DRAFT Minor Air Construction Permit Naval Air Station (NAS) Jacksonville Abrasive Blasting Booth Hanger Building (Bldg.) 101S (EU131) |
| --- | --- |

Project

This air construction permit, which authorizes construction of a new abrasive blasting booth, emission unit (EU) 131, to be located at the FRCSE Paint Stripping Shop, Hanger Building (Bldg.) 101S.  The particulate matter emissions (PM) will be controlled by two dust collectors: the Donaldson Torit model DFE 4-16 and the Clemco CDF-4.  The proposed work will be conducted at the existing Naval Air Station (NAS) Jacksonville, which is a National Security facility categorized under Standard Industrial Classification No. 9711.  The existing facility is located in Duval County at 6801 Roosevelt Boulevard, Jacksonville, Florida.  The UTM coordinates are Zone 17, 434.186 km East and 3343.245 km North; Latitude: 30° 13 14 North and Longitude: 81° 41 02 West.

This final permit is organized into the following sections:  Section 1 (General Information); Section 2 (Administrative Requirements); Section 3 (Emissions Unit Specific Conditions); and Section 4 (Appendices).  Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of Section 4 of this permit.

Statement of Basis

This air pollution construction permit is issued under the provisions of:  Chapter 403 of the Florida Statutes (F.S.) and Chapters 62-4, 62-204, 62-210, 62-212, 62-296 and 62-297 of the Florida Administrative Code (F.A.C.).  The permittee is authorized to conduct the proposed work in accordance with the conditions of this permit.  This project is subject to the general preconstruction review requirements in Rule 62-212.300, F.A.C. and is not subject to the preconstruction review requirements for major stationary sources in Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

Upon issuance of this final permit, any party to this order has the right to seek judicial review of it under Section 120.68 of the Florida Statutes by filing a notice of appeal under Rule 9.110 of the Florida Rules of Appellate Procedure with the clerk of the Department of Environmental Protection in the Office of General Counsel (Mail Station #35, 3900 Commonwealth Boulevard, Tallahassee, Florida, 32399-3000) and by filing a copy of the notice of appeal accompanied by the applicable filing fees with the appropriate District Court of Appeal.  The notice must be filed within 30 days after this order is filed with the clerk of the Department.

Executed in Jacksonville, Florida

(DRAFT)

Katie Sula Miller

Permitting Program Administrator

CERTIFICATE OF SERVICE

The undersigned duly designated deputy agency clerk hereby certifies that this Air Permit package was sent by electronic mail, or a link to these documents made available electronically on a publicly accessible server, with received receipt requested before the close of business on the date indicated below to the following persons.

# Captain Ryan Dexter, Naval Air Station Jacksonville, ryan.p.dexter.mil@us.navy.mil

# Tori Rohde, Naval Air Station Jacksonville, tori.rohde.civ@us.navy.mil

# Bradley Uhlmann, P.E., WSP USA Inc., brad.uhlmann@wsp.com

Leslie Maybin, NED FDEP

Katie Miller, NED FDEP

Brian Durden, NED FDEP

Julia Boesch, NED FDEP

Zachary Winkler, NED FDEP

# Clerk Stamp

# FILING AND ACKNOWLEDGMENT FILED, on this date, pursuant to Section 120.52(7), Florida Statutes, with the designated agency clerk, receipt of which is hereby acknowledged.

(DRAFT)

_______________	___________(DRAFT)

Clerk				Date

Facility Description

The Naval Air Station (NAS), Jacksonville is located in Duval County, in the northeastern sector of Florida.  This naval base occupies about 3400 acres along the west bank of the St. Johns River.  The base is approximately 30 miles inland from the Atlantic Ocean and 13 miles south of downtown Jacksonville.  The main gate to the base can be accessed at about 3 miles north of the intersection of U.S. Highway 17 (US 17) and Interstate 295 (I-295) on US 17 (also known as Roosevelt Boulevard).  Although the NAS Jacksonville consists of over 115 individual commands, for the purpose of this permit, the air pollutant emissions units (EUs) can be roughly grouped as belonging to one of three organizations as follows:

- NAS provides training of aircraft crews and commands, supports fleet and shore-based personnel, maintains and operates facilities, and provides services and materials to support operation of the aviation activities.  Air pollutant emitting activities associated with NAS primarily result from operation and maintenance of military aircraft.

- Public Works Department (PWD) maintains support facilities, such as the main boiler plants and other utilities.  PWD contracts facility maintenance operations with an outside contractor.

- The Fleet Readiness Center Southeast (FRCSE) Jacksonville has been in operation since 1940.  With over 4,000 employees, it occupies more than 50 buildings in over 102 acres at the east end of NAS.  FRCSE personnel perform in-depth (“depot-level”) and intermediate level maintenance, repair, and rework on naval aircraft, engines, components, accessories, and ground support equipment.  Most of the air pollutant emitting activities at the NAS Jacksonville are located within FRCSE and consist of activities such as aircraft surface coating and de-painting operations, solvent use operations, a chrome electroplating facility, abrasive blasting operations, engine testing and intermediate maintenance of military aircraft.

“Intermediate” level maintenance is also performed by FRCSE.  This operation consists of several divisions that perform various maintenance activities:

  - 400 Division performs maintenance on aircraft engines.

  - 500 Division is responsible for airframes maintenance.

  - 600 Division performs maintenance on aircraft electronic equipment.

  - 700 Division performs maintenance on aircraft weapons and ordnance systems.

  - 800 Division maintains life support equipment on the aircraft.

  - 900 Division performs maintenance on aircraft ground support equipment.

The existing facility consists of the following emissions units (EU).

| EU No. | ## Brief Description |
| --- | --- |
| Regulated Emissions Units |  |
| Abrasive Blasting Operations |  |
| 065 | Abrasive Blasting Booth, Bldg. 794 |
| 066 | Abrasive Blasting Booth, Bldg. 794 |
| 103 | Three Abrasive Blasting Booths, Bldg. 794 |
| 105 | Abrasive Blasting Booth, Hangar 101S |
| 113 | Abrasive Blasting Booth No. 1, Bldg. 101 |
| 114 | Abrasive Blasting Booth No. 2, Bldg. 101 |
| 116 | Abrasive Blasting Booth, Bldg. 1952 |
| 128 | Abrasive Blasting Booth, Bldg. 101C |
| Steam Generating Boilers |  |
| 072 | Steam Boiler No. 2, 300 HP Johnston Boiler Co., (EC030) [Bldg. H-2032] |
| 076 | Steam Boiler No. 3, 300 HP Johnston Boiler Co., (EC194) [Bldg. H-2032] |
| 087 | Steam Plant A, (EC048, EC048, EC049) |
| 088 | Steam Plant C, Bldg. 86 |
| 089 | Steam Plant F, Bldg. 88 |
| 115 | Boiler No. F-1, Bldg. H2032 |
| Chrome Plating Operation |  |
| 043 | Chrome Plating Shop, Bldg. 794 |
| Surface Coating and Solvent Use Operations |  |
| 117 | Three Paint Spray Booths, Bldg. 794 |
| 011 | Aircraft Painting Hangar, Bldg. 868 |
| 052 | Bearing Shop, Bldg. 101 |
| 056 | Aircraft Paint Stripping Hangar 101S |
| 084 | Fugitive VOC and HAP Emissions from Organizational Level (Squadrons) Aircraft Maintenance Activities |
| 085 | Fugitive VOC and HAP Emissions from Paints and Solvents used at FRCSE |
| 086 | Paint Spray Booth, Bldg. 101S |
| 093 | Binks Paint Booth |
| 094 | Two Paint Spray Booths |
| 106 | Paint Spray Booth, Bldg. 724, FRCSE Shop 430 |
| 107 | Paint Spray Booth, Hangar 1000, FRCSE Shop 51A |
| 111 | Paint Spray Booth, Hangar 124 |
| 112 | Paint Spray Booth, Bldg. 101W |
| Emergency Engines |  |
| 120 | >500 hp Existing Emergency Stationary CI IC Engines |
| 121 | >500 hp New Emergency Stationary CI IC Engines |
| 122 | ≤ 500 hp Existing Emergency Stationary CI IC Engines |
| 123 | Subpart IIII Emergency Stationary CI ICE Pre-2007 Engines |
| 124 | Subpart IIII Emergency Stationary CI ICE 2007 and after Engines |
| 125 | Subpart JJJJ Emergency Stationary SI ICE 2007 and after Engines |
| Gasoline Dispensing Facilities (GDF) |  |
| 119 | Gasoline Dispensing Facilities (GDF) |
| Boilers Subject to 40 CFR 63 Subpart DDDDD |  |
| 126 | Boilers Subject to 40 CFR 63 Subpart DDDDD |

PROPOSED PROJECT

Construction of a new abrasive blasting booth EU131 that will be located at the FRCSE Paint Stripping Shop, Hanger Building (Bldg.) 101S.  The blasting booth will be manufactured by Clemco Industries CORP and will use glass beads as blast media for surface preparation and cleaning target surfaces from aircraft parts (including wings and stabilizers).  The particulate matter (PM) emissions generated from the blasting will be vented to filters and cartridges.

This project will add the following emissions unit.

| EU No. | Emission Unit Description |
| --- | --- |
| EU131 | Abrasive Blasting Booth Hangar Building (Bldg.) 101S |

Facility Regulatory Classification

- The facility is a major source of hazardous air pollutants (HAP).

- The facility is a Title V major source of air pollution in accordance with Chapter 62-213, F.A.C.

- The facility is a major stationary source in accordance with Rule 62-212.400, F.A.C. for the PSD of Air Quality.

- The facility operates units subject to the New Source Performance Standards (NSPS) of Title 40 Part 60 of the Code of Federal Regulations (40 CFR 60).

- The facility operates units subject to the National Emissions Standards of Hazardous Air Pollutants (NESHAP) of 40 CFR 63.

- Permitting Authority:  The permitting authority for this project is the Northeast District Office of the Department of Environmental Protection (Department).  The Northeast District mailing address is 8800 Baymeadows Way West, Suite 100, Jacksonville, Florida 32256.

- Compliance Authority:  All documents related to compliance activities such as reports, tests, and notifications shall be submitted to the Northeast District Office, Compliance Assurance at:  8800 Baymeadows Way West, Suite 100, Jacksonville, Florida 32256.

- Appendices:  The following Appendices are attached as a part of this permit:  Appendix A (Citation Formats and Glossary of Common Terms); Appendix B (General Conditions); Appendix C (Common Conditions); and Appendix D (Common Testing Requirements); Appendix E (LR-1, Local Index), Appendix NESHAP 40 CFR 63 Subpart A (General Provisions); and Appendix NESHAP, 40 CFR 63 Subpart GG (National Emission Standards for Aerospace Manufacturing and Rework Facilities).

- Applicable Regulations, Forms and Application Procedures:  Unless otherwise specified in this permit, the construction and operation of the subject emissions units shall be in accordance with the capacities and specifications stated in the application.  The facility is subject to all applicable provisions of: Chapter 403, F.S.; and Chapters 62-4, 62-204, 62-210, 62-212, 62-213, 62-296 and 62-297, F.A.C.  Issuance of this permit does not relieve the permittee from compliance with any applicable federal, state, or local permitting or regulations.

- New or Additional Conditions:  For good cause shown and after notice and an administrative hearing, if requested, the Department may require the permittee to conform to new or additional conditions.  The Department shall allow the permittee a reasonable time to conform to the new or additional conditions, and on application of the permittee, the Department may grant additional time.  [Rule 62-4.080, F.A.C.]

- Modifications:  The permittee shall notify the Compliance Authority upon commencement of construction.  No new emissions unit shall be constructed and no existing emissions unit shall be modified without obtaining an air construction permit from the Department.  Such permit shall be obtained prior to beginning construction or modification.  [Rules 62-210.300(1) and 62-212.300(1)(a), F.A.C.]

- Construction and Expiration:  The expiration date shown on the first page of this permit provides time to complete the physical construction activities authorized by this permit, complete any necessary compliance testing, and obtain an operation permit.  Notwithstanding this expiration date, all specific emissions limitations and operating requirements established by this permit shall remain in effect until the facility or emissions unit is permanently shut down.  For good cause, the permittee may request that a permit be extended.  Pursuant to Rule 62-4.080(3), F.A.C., such a request shall be submitted to the Permitting Authority in writing before the permit expires.  [Rules 62-4.070(3) & (4), 62-4.080 & 62-210.300(1), F.A.C.]

- Source Obligation:

- At such time that a particular source or modification becomes a major stationary source or major modification (as these terms were defined at the time the source obtained the enforceable limitation) solely by virtue of a relaxation in any enforceable limitation which was established after August 7, 1980, on the capacity of the source or modification otherwise to emit a pollutant, such as a restriction on hours of operation, then the requirements of subsections 62-212.400(4) through (12), F.A.C., shall apply to the source or modification as though construction had not yet commenced on the source or modification.

- At such time that a particular source or modification becomes a major stationary source or major modification (as these terms were defined at the time the source obtained the enforceable limitation) solely by exceeding its projected actual emissions, then the requirements of subsections 62-212.400(4) through (12), F.A.C., shall apply to the source or modification as though construction had not yet commenced on the source or modification.

[Rule 62-212.400(12), F.A.C.; and Rule 2.401, JEPB]

- Application for Title V Permit:  This permit authorizes construction of the permitted emissions units and initial operation to determine compliance with Department rules.  A Title V air operation permit is required for regular operation of the permitted emissions unit.  The permittee shall apply for a Title V air operation permit at least 90 days prior to expiration of this permit, but no later than 180 days after commencing operation.  To apply for a Title V operation permit, the applicant shall submit the appropriate application form, compliance test results, and such additional information as the Department may by law require.  The application shall be submitted to the appropriate Permitting Authority with copies to the Compliance Authority.  [Rules 62-4.030, 62-4.050 and Chapter 62-213, F.A.C.; and 2.501, JEPB]

OTHER REQUIREMENTS

- Facility-Wide Potential-to-Emit (PTE) Records.  The permittee shall record and maintain a log identifying the facility-wide PTE.

- The log shall include a description of the source of emissions, regulated pollutants and HAP, emissions factors and supporting documentation, and calculations used in determining the facility-wide PTE.  This includes facility-wide sources emitting any regulated pollutant and/or HAP, including sources exempt from permitting (i.e., emergency engines, fire pumps, boilers/heaters, etc.).

- The log shall be updated within one-month of operating any new air emitting activities and/or equipment.

- The facility-wide PTE of all regulated pollutants and HAP (individual and total) shall be provided to the Department upon request and with all applications for an air construction permit modification and air operation permit renewals.

[Rules 62-4.070, 62-210.200(155) and 62-213.440, F.A.C.; Rule 2.301, JEPB & 2.501, JEPB and Application No. 0310215-076-AC]

- City of Jacksonville Ordinance Code.  The facility shall be subject to City of Jacksonville Ordinance Code, Title X, Chapter 360 [Environmental Regulation], Chapter 362 [Air and Water Pollution], Chapter 376 [Odor Control], and JEPB Rule 1 [Final Rules with Respect to Organization, Procedure, and Practice].

- JEPB Rules.  The facility shall be subject to JEPB Rule 2, Part Nos. I through IV, VI through VII and Part Nos. IX through XIV.

This section of the permit addresses the following emissions units.

| EU No. | Emission Unit Description |
| --- | --- |
| 131 | Abrasive Blasting Booth, Hangar Building (Bldg.), 101S |

The abrasive blasting booth will be located at the FRCSE Paint Stripping Shop, Bldg., 101S.  EU131 dimensions will be 35’ L x 14’ W x 13’ H, and it will be a column bolt blast room with monorail, full floor recovery, and a 50 CF Hopper manufactured by Clemco Industries CORP.  Glass beads will be the blasting media used.  There is one existing vertical stack on roof of Hangar Building Bldg. 101S.

Glass beads recycling process:  As the beads impact the surface, they remove contaminants, rust, or paint.  The used glass beads then fall to the floor, where it is gathered and transported to a recovery system.  There, the glass beads pass through a reclaimer that filters out dust, debris, and broken beads, ensuring only clean, reusable glass beads are retained.  These cleaned beads are then stored in a hopper and recirculated back into the blasting system for continued use.

To control emissions of PM from this EU, two dust collectors will be installed: the Donaldson Torit Model DFE 4-16 with a HEPA filter, and the Clemco CDF-4.  The filtration efficiencies for both filers, often exceeding 99.97%.

Discharge Type: Vertical. Stack Height: 71 feet.  Exit Diameter: 3.33 feet. Exit Temperature:  75°F.  Actual Volumetric Flow Rate (acfm): 1,600.

{Permitting Notes:  This emission unit is regulated under Rule 62-210.300, F.A.C. - Permit Required; 40 CFR 63, Subpart GG, National Emission Standards for Aerospace Manufacturing and Rework Facilities, and 40 CFR 63, Subpart A, General Provisions, as delineated in Table 1 to Subpart GG, shall apply to this emission unit.}

Equipment

- Abrasive Blasting Booth and Dust Collectors with Filters:  The permittee is authorized to install:

  - A 35’ L X 14’ W X 13’ H abrasive blasting booth manufactured by Clemco Industries CORP, at Bldg. 101S, EU131.

  - Two dust collectors:  Donaldson Torit model DFE 4-16 with a HEPA filter system, and the Clemco CDF-4 with high filtration efficiencies for both filers 99.97%.

[Rules 62-4.070(3), F.A.C. (Reasonable Assurance rule) & 62-210.200, F.A.C. (Potential to Emit (PTE)); and Application No. 0310215-076-AC].

Performance Restrictions

- Maximum Process Rate:  The maximum process material flow rate of abrasive blast media (glass beads) to the blasting operation shall be 1,284,216 pounds per year (lbs/yr).  [Rules 62-4.070(3) & 62-210.200 (PTE), F.A.C.; Rule 2.301, JEPB; and Application No. 0310215-076-AC].

- Hours of Operation:  The hours of operation are not limited (8,760 hours/year).  [Rule 62-4.070(3), F.A.C.; Rule 2.301, JEPB; and 62-210.200(PTE), F.A.C.]

- Method of Operation – Blasting Media:  The abrasive blasting media shall consist of Glass beads (Potters - Ballontini impact beads, or equivalent glass beads).  [Rule 62-4.070(3) & 62-210.200, F.A.C.; and Application No. 0310215-076-AC].

- No Circumvention:  No person shall circumvent any air pollution control device, or allow the emission of air pollutants without the applicable air pollution control device operating properly..  [Rules 62-4.070(3), 62-210.200 (PTE) and 62-210.650, F.A.C.; Rule 2.301, JEPB; and Application No. 0310215-076-AC]

- Control Devices:  The dust collectors will be operated in parallel and located outside the building.  [Rule 62-4.070(3), F.A.C. (Reasonable Assurance rule); and Application No. 0310215-076-AC]

Emissions Standards

- 40 CFR 63 Subpart GG and 40 CFR 63 Subpart A - Applicability:  40 CFR 63, Subpart GG, National Emission Standards for Aerospace Manufacturing and Rework Facilities and 40 CFR 63, Subpart A, General Provisions shall apply to this emissions unit.  Table 1 to Subpart GG of Part 63 defines the applicable parts of the General Provisions which apply to affected emission units in 40 CFR, Subpart GG.  [Rule 62-204.800, F.A.C.; Rule 2.201, JEPB; and 40 CFR 63.741(a),(b); and 40 CFR 63 Subpart GG Table 1]

- Visible Emissions (VE): VE shall be limited to 5% opacity.  [Rule 62-4.070, F.A.C., Rule 2.1101, JEPB; and Application No. 0310215-076-AC]

##### Testing Requirements

- Initial Compliance Tests:  The emissions unit shall be tested to demonstrate initial compliance with the emissions standards for the following emissions and pollutants:  Visible emissions (VE) (no more than 5% opacity).  The initial tests shall be conducted within 60 days after achieving permitted capacity, but not later than 180 days after initial operation of the unit.  [Rules 62-4.070(3) & 62-297.310(8)(b)1, F.A.C.; and Application No. 0310215-076-AC]

- Annual Compliance Tests:  During each calendar year (January 1st to December 31st), the emissions unit shall be tested to demonstrate compliance with the emissions standards for VE.  VE testing shall be conducted for a minimum period of 30 minutes.  The opacity test observation period shall include the period during which the highest opacity emissions can reasonably be expected to occur.  [Rules 62-297.310(5), 62-297.310(8)(a)1, and 62-297.310(8)(a)4, F.A.C.; Rule 2.1201, JEPB and Application No. 0310215-076-AC]

- Permit Renewal Test:  The emissions unit shall be tested to demonstrate compliance with the emissions standards for visible emissions (no more than 5 percent (%) opacity) prior to obtaining a renewed operation permit.  The opacity test observation period shall include the period during which the highest opacity emissions can reasonably be expected to occur [Rules 62-297.310(5) & 62-297.310(8)(b)1, F.A.C.; and Rule 2.1201, JEPB]

- Test Requirements:  The permittee shall notify the Compliance Authority in writing at least 15 days prior to any required tests.  Tests shall be conducted in accordance with the applicable requirements specified in Appendix D (Common Testing Requirements) of this permit.  [Rule 62-297.310(9), F.A.C.]

{Permitting Note:  Air compliance test notifications can now be completed online in the Department’s Business Portal.  To access this online process, go to http://www.fldepportal.com/go/home and sign in (or register if you’re a new user) from the link in the upper right corner of the page.  On the Welcome page select the Submit option, then select Registration/Notification, and then click on Air Compliance Test Notifications.  Once in the process, just carefully read the instructions on each screen (and under the Help tabs) to complete the notification.}

- Test Methods:  Required tests shall be performed in accordance with the following EPA reference methods.

| Method | Description of Method and Comments |
| --- | --- |
| 9 | Visual Determination of the Opacity of Emissions from Stationary Sources |

The above methods are described in Appendix A of 40 CFR 60 and are adopted by reference in Rule 62-204.800, F.A.C.  No other methods may be used unless prior written approval is received from the Department.  [Rule 62-204.800, F.A.C.; and Appendix A of 40 CFR 60]

Monitoring AND Maintenance and Compliance Requirements

- Operate and Maintain Equipment According to Manufacturer’s Specifications:  Where non-chemical based equipment is used the permittee shall operate and maintain the equipment according to the manufacturer's specifications.  During periods of malfunctions of such equipment, they may use substitute materials during the repair period provided the substitute materials used are those available that minimize organic HAP emissions.  In no event shall substitute materials be used for more than 15 days annually, unless such materials are organic HAP-free. [Rule 62-204.800, F.A.C.; and 40 CFR 63.746(b)(2)]

- Depainting Operations Standards:  The new depainting operation that generates airborne inorganic HAP emissions from dry media blasting equipment shall comply with Specific Condition 14 and shall also comply with the requirements as stated in below paragraphs a, b, c, d and e  of this Specific Condition. [Rule 62-204.800, F.A.C.; Rule 2.201, JEPB; and 40 CFR 63.746(b)(4)]

- Enclosed Blasting Booth:  The permittee shall only perform the depainting operation in the enclosed blasting booth.  The airsteam from the blast booth shall be passed through the dust collector(s) before discharge to the atmosphere.  [Rule 62-204.800, F.A.C.; Rule 2.201, JEPB; and 40 CFR 63.746(b)(4)(i)]

- New Source Dry Particulate Filter System:  For new sources, pass any air stream removed from the enclosed area or closed-cycle depainting system through a dry particulate filter system certified using the method described in Specific Condition 26 to meet or exceed the efficiency data points specified in the Tables 1 and 2 included below.  [Rule 62-204.800, F.A.C.; Rule 2.201, JEPB; 40 CFR 63.745, and 40 CFR 63.746(b)(4)ii(B); and 40 CFR 63.750(o)]

[Rule 62-204.800, F.A.C.; Rule 2.201, JEPB; and 40 CFR 63.746(b)(4)(ii)(B); and 40 CFR 63.745 (Table 4 ); and 40 CFR 63.750(o)]

[Rule 62-204.800, F.A.C.; Rule 2.201, JEPB and 40 CFR 63.746(b)(4)(ii)(B) and 40 CFR 63.745 (Table 5)]

- Particulate Filter System Compliance:  If a dry particulate filter system is used, the following requirements shall be met:

  - Maintain the system in good working order;

- Install a differential pressure gauge across the filter banks;

- Continuously monitor the pressure drop across the filter and read and record the pressure drop once per shift; and

- Take corrective action when the pressure drop exceeds or falls below the filter manufacturer's recommended limits.

[Rule 62-4.070(3), F.A.C.; Rule 62-204.800, F.A.C.; and 40 CFR 63.746(b)(4)(iii)]

- Pressure Drop:  If the pressure drop, as recorded pursuant to Specific Condition 20, is outside the limit(s) specified by the filter manufacturer or in locally prepared operating procedures, whichever is more stringent, the permittee shall shut down the operation immediately and take corrective action.  The operation shall not be resumed until the pressure drop is returned within the specified limit(s).  [Rule 62-4.070(3), F.A.C.; Rule 62-204.800, F.A.C.; and 40 CFR 63.746(b)(4)(v) and 63.752(e)(7)]

- Particulate filters:  The permittee shall use the dry particulate filter in accordance with the requirements as stated in above paragraphs a, b, c, and d of this Specific Condition and shall, while depainting operations are occurring, continuously monitor the pressure drop across the particulate filters and read and record the pressure drop per shift following the recordkeeping requirements as specified in  Specific Condition 20.  [Rule 62-4.070(3) & Rule 62-204.800, F.A.C.; 40 CFR 63.746(b)(4) & 40 CFR 63.751(d); and Application No. 0310215-076-AC]

- Inorganic HAP emissions—depainting operations:  Each depainting operation is in compliance when:

- The operating requirements, as stated in above paragraphs a, b, c, d  and e of  this Specific Condition are followed.  [Rule 62-204.800, F.A.C.; 40 CFR 63.746(b)(4); and 40 CFR 63.749(g)] and

- It is shut down immediately whenever the pressure drop is outside the limit(s) established for them and is not restarted until the pressure drop is returned within these limit(s), as required as stated in above paragraph d. of this Specific Condition.  [Rule 62-204.800, F.A.C.; 40 CFR 63.746(b)(4)(v) and 40 CFR 63.749(g)]

- Handling and Storage of Waste: The permittee of each facility subject to this subpart that produces a waste that contains organic HAP from aerospace primer, topcoat, specialty coating, chemical milling maskant, or chemical depainting operations must be handled and stored as specified in the below paragraphs i. and ii. of this Specific Condition:

  - Conduct the handling and transfer of the waste to or from containers, tanks, vats, vessels, and piping systems in such a manner that minimizes spills.

  - Store all waste that contains organic HAP in closed containers.

[Rule 62-204.800, F.A.C.; Rule 2.201, JEPB; and 40 CFR 63.748(a)]

- Safety and Good Air Pollution Control Practices:  At all times, the permittee shall operate and maintain the affected source, including associated air pollution control equipment and monitoring equipment, in a manner consistent with safety and good air pollution control practices for minimizing emissions.  The general duty to minimize emissions does not require the permittee to make any further efforts to reduce emissions if levels required by the applicable standard have been achieved.  Determination of whether a source is operating in compliance with operation and maintenance requirements will be based on information available to the Compliance Authority which may include, but is not limited to, monitoring results, review of operation and maintenance procedures, review of operation and maintenance records, and inspection of the source.  [Rule 62-204.800, F.A.C.; Rule 2.201, JEPB and 40 CFR 63.743(e)]

- Original Equipment Manufacturers (OEM) Guidelines and Recommendations:  The dust collectors shall have filters and cartridges, that shall be operated, inspected and maintained per the OEM guidelines and recommendations.  [Rule 62-4.070(3), F.A.C.; and Application No. 0310215-076-AC]

Records and Reports

- Test Reports:  The permittee shall prepare and submit reports for all required tests in accordance with the Test requirements specified in Appendix D (Common Testing Requirements) of this permit.  For each test run, including initial compliance tests, the report shall indicate the visible emissions.  The permittee shall submit a written test report to the compliance authority specified by permit, on the results of each such test as soon as practicable but no later than 45 days after the last run of each test is completed. Test reports may be submitted electronically.  [Rule 62-297.310(10), F.A.C.]

- Type of Aircraft De-painted Records:  The permittee shall record, for each type of aircraft depainted at the facility, a listing of the parts, subassemblies, and assemblies normally removed from the aircraft before depainting.  Prototype, test model or aircraft that exist in low numbers (i.e., less than 25 aircraft of any one type) are exempt from this requirement. [Rule 62-204.800, F.A.C.; and Rule 2.201, JEPB; and 40 CFR 63.752(e)4; and Application No. 0310215-076-AC]

- Dry Media Blasting Equipment Records - Non-chemical based equipment:  The permittee shall maintain the following records during periods of malfunction of the dry media blasting equipment.

  - The names and types of non-chemical based equipment; and

  - For periods of malfunction the following information/data:

- The non-chemical method or technique that malfunctioned;

- The date that the malfunction occurred;

- A description of the malfunction;

- The methods used to depaint aerospace vehicles during the malfunction period;

- The dates that these methods were begun and discontinued; and

- The date that the malfunction was corrected.

[Rule 62-204.800, F.A.C.; Rule 2.201, JEPB; 40 CFR 63.752(e)(5); and Application No. 0310215-076-AC]

- Recordkeeping for Inorganic HAP Emissions:  The permittee shall record the actual pressure drop across the particulate filters once each shift in which the depainting process is in operation.  This log shall include the acceptable limit(s) of the pressure drop as specified by the filter manufacturer, as specified by the booth manufacturer or in locally prepared operating procedures.  [Rule 62-204.800, F.A.C.; 40 CFR 63.752(e)(7) and Application No. 0310215-076-AC]

- Recordkeeping – General:  The permittee shall record and maintain the information specified in below paragraphs  a, b and c of this Specific Condition:

  - In the event that an affected unit fails to meet an applicable standard, record the number of failures. For each failure record the date, time, and duration of each failure.

  - For each failure to meet an applicable standard, record and retain a list of the affected sources or equipment, an estimate of the quantity of each regulated pollutant emitted over any emission limit and a description of the method used to estimate the emissions.

  - Record actions taken to minimize emissions in accordance with Specific Condition 15.h., and any corrective actions taken to return the affected unit to its normal or usual manner of operation.

[Rule 62-204.800, F.A.C.; Rule 2.201, JEPB; 40 CFR 63.10(b)(1) & 40 CFR 63.752(a); and application 0310215-076-AC]

- Semi-Annual Reports:  The permittee shall submit semi-annual reports (every 6 months) for the period(s) September 1 through February 28 or 29, and March 1 through August 31.  The reports shall be due May 1 for the September through February reporting period and November 1 for the March through August reporting period.  The reports shall contain the following information:

- Identification of any 24-hour period where organic HAP was emitted from the de-painting of aerospace vehicles;

- Any new non-chemical de-painting technique in use since the notification of compliance status or any subsequent semiannual report was filed;

- For Periods of Malfunction:

- The non-chemical method or technique that malfunctioned;

- The date that the malfunction occurred;

- A description of the malfunction;

- The methods used to de-paint aerospace vehicles during the malfunction period;

- The dates that these methods were begun and discontinued; and

- The date that the malfunction was corrected;

- All periods when a non-chemical de-painting operation subject to above Specific Conditions 14 and 15 for the control of inorganic HAP emissions was not immediately shut down when the pressure drop was outside the limits specified by the filter or booth manufacturer or locally prepared operational procedures;

- A list of new and discontinued aircraft models de-painted at the facility over the last 6 months and a list of the parts normally removed for de-painting for each new aircraft model being de-painted; and

- If the de-painting operation has been in compliance for the reporting period, a statement signed by the responsible official stating that the operation was in compliance with the applicable standards.

[Rule 62-204.800, F.A.C.; Rule 2.201, JEPB; 40 CFR 63.753(d)(1); and application 0310215-076-AC]

- Dry Media Blasting System Records and Semi-Annual Reports:  The permittee shall maintain records of the monthly and annual use of abrasive blasting media/material.  These records shall be kept and maintained for a minimum period of five (5) years.  Records shall be made available to the Compliance Authority upon request.  Additionally, semi-annual reports of this information shall be submitted to the Compliance Authority with the semi-annual reports required in the above Semi-Annual Specific Condition 22.  [Rules 62-4.070(3), 62-210.200, and 62-213.440(1)(b), F.A.C.; and Rule 2.501, JEPB]

- Annual Reports:  The permittee shall submit annual reports (every 12 months) from the date of the notification of compliance status that identify: the number of times the pressure drop limit(s) for each filter system were outside the limit(s) specified by the filter or booth manufacturer or in locally prepared operating procedures.  [Rule 62-204.800, F.A.C.; Rule 2.201, JEPB; 40 CFR 63.753(d)(2)((ii); and Application No. 0310215-076-AC].

- Record Retention and Availability:  The above records and reports requested in Specific Conditions 17, 18, 19 20, 21, 22, 23 and 24. shall be retained for a minimum period of five (5) years and shall be made available to the Compliance Authority upon request. [Rule 62-4.070(3), F.A.C.; and 2.201, JEPB]

- Dry Particulate Filter Certification Requirements:  Dry particulate filters used to comply with Specific Condition 15 must be certified by the filter manufacturer or distributor, paint/depainting booth supplier, and/or the permittee using method 319 in Appendix A to Part 63 - Test Methods, adopted and incorporated by reference in Rule 62-204.800, F.A.C., to meet or exceed the efficiency data points found in Tables 4 and 5 of above Specific Condition 15.b. for existing or new sources respectively.  [Rule 62-204.800(11), F.A.C; 40 CFR 63.745 and 63.750(o); and Application No. 0310215-076-AC]

- Commencement of Construction and Operation:  The permittee shall submit to the Air Compliance Authority of this Office written notification to the Northeast District Office mailing address: 8800 Baymeadows Way West, Suite 100, Jacksonville, Florida 32256 or send electronically within 15 days after such date to: DEP_NED@floridadep.gov, of both the date of commencement of construction and commencement of operation of the new Abrasive Blasting Booth - Hangar Building (Bldg.) 101S (EU 131).  [Rule 62-4.070(3), F.A.C.; and Rules 2.1401], JEPB