TECHNICAL EVALUATION

&

PRELIMINARY DETERMINATION

APPLICANT

United States Navy

6801 Roosevelt Boulevard

Jacksonville, FL  32212

Naval Air Station (NAS) Jacksonville

Facility ID No. 0310215

PROJECT

Project No. 0310215-076-AC

Application for Minor Source Air Construction Permit

Abrasive Blasting Booth Bldg. 101S (EU131)

COUNTY

Duval County, Florida

PERMITTING AUTHORITY

Florida Department of Environmental Protection

Northeast District Office

8800 Baymeadows Way West, Suite 100

Jacksonville, Florida  32256

October 31, 2025

- General Project INFORMATION

Air Pollution Regulations

Projects at stationary sources with the potential to emit air pollution are subject to the applicable environmental laws specified in Section 403 of the Florida Statutes (F.S.).  The statutes authorize the Department of Environmental Protection (Department) to establish regulations regarding air quality as part of the Florida Administrative Code (F.A.C.), which includes the following applicable chapters:  62-4 (Permits); 62-204 (Air Pollution Control – General Provisions); 62-210 (Stationary Sources – General Requirements); 62-212 (Stationary Sources – Preconstruction Review); 62-213 (Operation Permits for Major Sources of Air Pollution); 62-296 (Stationary Sources - Emission Standards); and 62-297 (Stationary Sources – Emissions Monitoring).  Specifically, air construction permits are required pursuant to Chapters 62-4, 62-210 and 62-212, F.A.C.

In addition, the U. S. Environmental Protection Agency (EPA) establishes air quality regulations in Title 40 of the Code of Federal Regulations (CFR).  Part 60 specifies New Source Performance Standards (NSPS) for numerous industrial categories.  Part 61 specifies National Emission Standards for Hazardous Air Pollutants (NESHAP) based on specific pollutants.  Part 63 specifies NESHAP based on the Maximum Achievable Control Technology (MACT) for numerous industrial categories.  The Department adopts these federal regulations in Rule 62-204.800, F.A.C.

Glossary of Common Terms

Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of this permit.

  - Facility Description and Location

The Naval Air Station (NAS), Jacksonville, is located in Duval County, in the northeastern sector of Florida.  This naval base occupies about 3400 acres along the west bank of the St. Johns River.  The base is approximately 30 miles inland from the Atlantic Ocean and 13 miles south of downtown Jacksonville.  The main gate to the base can be accessed at about 3 miles north of the intersection of U.S. Highway 17 (US 17) and Interstate 295 (I-295) on US 17 (also known as Roosevelt Boulevard).  Naval Air Station (NAS) Jacksonville, which is a National Security facility (Standard Industrial Classification No. 9711).  The existing Naval Air Station (NAS) Jacksonville facility is located in Duval County at 6801 Roosevelt Blvd., in Jacksonville, Florida.  The UTM coordinates of the existing facility are Zone 17, 434.2 km East, and 3342.80 km North.  This site is in an area that is in attainment (or designated as unclassifiable) for all air pollutants subject to Ambient Air Quality Standards (AAQS).  The location of Duval County is shown in red in Figure 1.  Figure 2. Satellite view of the facility is shown.  Figure 3. Shows the Facility Plot Plan.

Figure 1:  Location Duval County, Florida 					Figure 2: Satellite view of NAS Jacksonville

Figure 3.  Facility Plot Plan

Although the NAS consists of over 115 individual commands, for the purpose of this application, the air pollutant emissions units (EUs) can be roughly grouped as belonging to one of three organizations as follows:

- NAS provides training of aircraft crews and commands, supports fleet and shore-based personnel, maintains and operates facilities, and provides services and materials to support operation of the aviation activities.  Air pollutant emitting activities associated with NAS primarily result from operation and maintenance of military aircraft.

- Public Works Department (PWD) maintains support facilities, such as the main boiler plants and other utilities.  PWD contracts facility maintenance operations with an outside contractor.

- The Fleet Readiness Center Southeast (FRCSE) Jacksonville has been in operation since 1940.  With over 4,000 employees, it occupies more than 50 buildings in over 102 acres at the east end of NAS. FRCSE personnel perform in-depth (“depot-level”) and intermediate level maintenance, repair, and rework on naval aircraft, engines, components, accessories, and ground support equipment.  Most of the air pollutant emitting activities at the NAS Jacksonville are located within FRCSE and consist of activities such as aircraft surface coating and de-painting operations, solvent use operations, a chrome electroplating facility, abrasive blasting operations, engine testing and intermediate maintenance of military aircraft.

“Intermediate” level maintenance is also performed by FRCSE.  This operation consists of several divisions that perform various maintenance activities:

  - 400 Division performs maintenance on aircraft engines.

  - 500 Division is responsible for airframes maintenance.

  - 600 Division performs maintenance on aircraft electronic equipment.

  - 700 Division performs maintenance on aircraft weapons and ordnance systems.

  - 800 Division maintains life support equipment on the aircraft.

  - 900 Division performs maintenance on aircraft ground support equipment.

1.2		Facility Regulatory Categories

- The facility is a major source of hazardous air pollutants (HAP).

- The facility is a Title V major source of air pollution in accordance with Chapter 62-213, F.A.C., and Rule 2.501, JEPB.

- The facility is a major stationary source in accordance with Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

- The facility does operate units subject to the NSPS of 40 CFR 60.

- The facility does operate units subject to the NESHAP of 40 CFR 63.

1.3		Project Description

The United States Navy proposes to construct a new abrasive blasting booth.

EU131 will be located in the same building as EU 105 at the FRCSE Paint Stripping Shop, Hangar Building (Bldg.) 101S.  The new abrasive blasting booth dimensions will be 35’ L X 14’ W X 13’ H, and it will be a column bolt blast room with monorail, full floor recovery, and a 50 CF Hopper using glass beads as the abrasive media used.  The operator will direct a nozzle emitting a high-pressure stream of air mixed with glass beads onto the surface of a workpiece to clean it.  The workpieces targeted surfaces will have a low inorganic HAP content.

The facility may conduct the following glass beads recycling process:  As the beads impact the surface, they remove contaminants, rust, or paint.  The used glass beads then fall to the floor, where it is gathered and transported to a recovery system.  There, the glass beads pass through a reclaimer that filters out dust, debris, and broken beads, ensuring only clean, reusable glass beads are retained.  These cleaned beads are then stored in a hopper and recirculated back into the blasting system for continued use.

To effectively control emissions of particulate matter (PM) from EU131 two dust collectors will be installed: the Donaldson Torit model DFE 4-16 with a HEPA filter, and the Clemco CDF-4 each with a high filtration efficiency of 99.97%.  The dust collectors will be operated in parallel and will be located outside the building.  The dust collectors with filters and cartridges, will be operated, inspected and maintained per the Original Equipment Manufacturers guidelines and recommendations.  There is one existing vertical stack on the roof of Hangar Building 101S.

Discharge Type: Vertical. Stack Height: 71 feet.  Exit Diameter: 3.33 feet. Exit Temperature:  75°F.  Actual Volumetric Flow Rate actual cubic feet per minute (acfm): 1,600. On Hangar Building 101 S

Figure 4.  Process Flow

- Table 1 shows the following new emission unit will be added by this project.

- TABLE 1. List of New Emission Units

| - EU No. | - Description |
| --- | --- |
| - EU131 | - Abrasive Blasting Booth, Hangar Building (Bldg.), 101S |

- Processing Schedule

May 12, 2025  Received the application for a minor source air pollution construction permit.

May 29, 2025  Requested additional information.

August 22, 2025  Received additional information;

September 19, 2025  Received supplemental information to the August 22, 2025 submittal; application complete.

- PSD Applicability

General PSD Applicability

For areas currently in attainment with the AAQS or areas otherwise designated as unclassifiable, the Department regulates major stationary sources of air pollution in accordance with Florida’s PSD preconstruction review program as defined in Rule 62-212.400, F.A.C.  Under preconstruction review, the Department first must determine if a project is subject to the PSD requirements (“PSD applicability review”) and, if so, must conduct a PSD preconstruction review.  A PSD applicability review is required for projects at new and existing major stationary sources.  In addition, proposed projects at existing minor sources are subject to a PSD applicability review to determine whether potential emissions from the proposed project itself will exceed the PSD major stationary source thresholds.  A facility is considered a major stationary source with respect to PSD if it emits or has the potential to emit:

- 250 tons per year or more of any regulated air pollutant; or

- 100 tons per year or more of any regulated air pollutant and the facility belongs to one of the following 28 PSD-major facility categories:  fossil fuel-fired steam electric plants of more than 250 million British thermal units per hour heat input, coal cleaning plants (with thermal dryers), Kraft pulp mills, Portland cement plants, primary zinc smelters, iron and steel mill plants, primary aluminum ore reduction plants, primary copper smelters, municipal incinerators capable of charging more than 250 tons of refuse per day, hydrofluoric, sulfuric, and nitric acid plants, petroleum refineries, lime plants, phosphate rock processing plants, coke oven batteries, sulfur recovery plants, carbon black plants (furnace process), primary lead smelters, fuel conversion plants, sintering plants, secondary metal production plants, chemical process plants, fossil fuel boilers (or combinations thereof) totaling more than 250 million British thermal units per hour heat input, petroleum storage and transfer units with a total storage capacity exceeding 300,000 barrels, taconite ore processing plants, glass fiber processing plants and charcoal production plants.

Once it is determined that a project is subject to PSD preconstruction review, the project emissions are compared to the “significant emission rates” defined in Rule 62-210.200, F.A.C. for the following pollutants:  carbon monoxide (CO); nitrogen oxides (NOX); sulfur dioxide (SO2); particulate matter (PM); particulate matter with a mean particle diameter of 10 microns or less (PM10); particulate matter with a mean particle diameter of 2.5 microns or less (PM2.5); volatile organic compounds (VOC); lead (Pb); fluorides (F); sulfuric acid mist (SAM); hydrogen sulfide (H2S); total reduced sulfur (TRS), including H2S; reduced sulfur compounds, including H2S; municipal waste combustor organics measured as total tetra- through octa-chlorinated dibenzo-p-dioxins and dibenzofurans; municipal waste combustor metals measured as particulate matter; municipal waste combustor acid gases measured as SO2 and hydrogen chloride (HCl); municipal solid waste landfills emissions measured as non-methane organic compounds (NMOC); and mercury (Hg).  In addition, significant emissions rate also means any emissions rate or any net emissions increase associated with a major stationary source or major modification which would construct within 10 kilometers of a Class I area and have an impact on such area equal to or greater than 1 μg/m3, 24-hour average.

If the potential emission equals or exceeds the defined significant emissions rate of a PSD pollutant, the project is considered “significant” for the pollutant and the applicant must employ the Best Available Control Technology (BACT) to minimize the emissions and evaluate the air quality impacts.  Although a facility or project may be major with respect to PSD for only one regulated pollutant, it may be required to install BACT controls for several “significant” regulated pollutants.

PSD Applicability for Project

As provided in the application, the following table summarizes potential emissions and PSD applicability for the project.

TABLE 2.  Summary of the Applicant’s PSD Applicability Analysis

| Pollutant | PTE | Increase | SER Threshold | Subject to PSD |
| --- | --- | --- | --- | --- |
| PM | 0.017 | 0.017 | 25 | No |
| PM10 | 0.014 | 0.014 | 15 | No |
| PM2.5 | 0.013 | 0.013 | 10 | No |
| VOC | 0.00035 | 0.00035 | 40 | No |

As shown in the above table 2, total project emissions will not exceed the PSD significant emissions rates; therefore, the project is not subject to PSD preconstruction review.

- Department review

Based on the application and the Departments review, the Department will issue the draft permit authorizing the installation of EU131, Abrasive Blasting Booth and two dust collectors, Donaldson Torit model DFE 4-16 with a HEPA filter, and the Clemco CDF-4 to control PM emissions.

The hours of operation will not be limited, and calculations are based on operating 8,760 hours per year.

Brief Discussion of Emissions

Visible emissions (VE) will be limited to 5% opacity, since the control devices are proposed to have a 99.97% control efficiency, and based on the reasonable assurance rule, Rule 62-4.070, F.A.C.  The facility agreed to this VE cap and to an initial VE test and a VE test prior to permit renewal in the Request for Additional Information response.  In the application form the Facility proposed to conduct annual observation (testing) for VE.

The maximum process throughput/material flow rate of abrasive blast media to the blasting operation will be limited to 1,284,216 pounds per year (lbs/yr), which is the maximum rate indicated in the application form and upon which the PTE calculations are based.

The operator will direct a nozzle emitting a high-pressure stream of air mixed with glass beads onto the surface of a workpiece to be cleaned.  The August 22, 2025, RAI response, indicated the workpieces targeted surfaces will be have a low inorganic HAP content with annual emissions of 0.00035 tons per year.

Table 3. Applicant’s Factors and Annual Emissions

| Nozzles | Media Type | Nozzle Diameters 1 | Nozzle Pressure 1 | Material Density 2 | Flowrate 3 lb/hour | Shop Hours (hr/year) |
| --- | --- | --- | --- | --- | --- | --- |
| Nozzle 1 | Glass Beads | 1/8 | 30 | 156.07 | 44.1 | 8760 |
| Nozzle 2 | Glass Beads | 3/16 | 30 | 156.07 | 102.5 | 8760 |
| Emission Factor (lb/hr)4 Nozzle 1 |  |  | Control Efficiency 5 (%) | Annual Emissions (TPY) Nozzle 1 |  |  |
| PM10 | PM2.5 | PM |  | PM10 | PM2.5 | PM |
| 0.008 | 0.007 | 0.009 | 99.7 | 0.004 | 0.004 | 0.005 |
| Emission Factor (lb/hr)4 Nozzle 2 |  |  | Control Efficiency5 (%) | Annual Emissions (TPY) Nozzle 2 |  |  |
| PM10 | PM2.5 | PM |  | PM10 | PM2.5 | PM |
| 0.008 | 0.007 | 0.009 | 99.7 | 0.010 | 0.009 | 0.012 |
| Total Annual Emissions (TPY) both Nozzles |  |  |
| PM10 | PM2.5 | PM |
| 0.014 | 0.013 | 0.017 |

Notes:

1 Nozzle diameters and pressures that will be used at the Abrasive Blasting Booth are provided by NAS JAX.

2 The abrasive media that will be used, Glass beads, density was provided by manufacturer

3 Flowrate calculated based on South Coast Air Quality Management District, Section 2: Unconfined Abrasive Blasting

4 Emission Factors are from Table 7-1 from the Air Emissions Guide for Air Force Stationary Sources

5 Standard control efficiency for Clemco CDF dust collectors is 99.7%. The Donaldson Torit model DFE 4-16 dust collector with a HEPA   filter typically has a filtration efficiency of 99.97%. A 99.7% control efficiency is used for conservative emission calculations.

The HAP PTE is below the thresholds of 10 TPY for a single HAP and 25 TPY for HAP aggregate.

Potential to Emit Emissions from the new abrasive blasting booth:

PM: 0.017 TPY; PM10: 0.014 TPY; PM2.5: 0.013 TPY; VOC:  0.00035 TPY

State Requirements

- This project is subject to the applicable rules and regulations defined in the following Chapters of the Florida Administrative Code (F.A.C.).

Chapter 62-4, F.A.C.

Rule 62-4.070(1), F.A.C., Standards for Issuing or Denying Permits; Issuance; Denial.

A permit shall be issued to the applicant upon such conditions as the Department may direct, only if the applicant affirmatively provides the Department with reasonable assurance based on plans, test results, installation of pollution control equipment, or other information, that the construction, expansion, modification, operation, or activity of the installation will not discharge, emit, or cause pollution in contravention of Department standards or rules.

Projects at stationary sources with the potential to emit air pollution are subject to the applicable environmental laws specified in Section 403 of the Florida Statutes (F.S.).  The statutes authorize the Department of Environmental Protection (Department) to establish regulations regarding air quality as part of the Florida Administrative Code (F.A.C.), which includes the following applicable chapters: 62-4 (Permits); 62-204 (Air Pollution Control – General Provisions); 62-210 (Stationary Sources – General Requirements); 62-212 (Stationary Sources – Preconstruction Review); 62-213 (Operation Permits for Major Sources of Air Pollution); 62-296 (Stationary Sources - Emission Standards); and 62-297 (Stationary Sources – Emissions Monitoring).  Specifically, air construction permits are required pursuant to Rules 62-4, 62-210 and 62-212, F.A.C.

Rule 62-210.300, F.A.C., Permits Required

Unless exempted, the owner or operator of any facility or emissions unit which emits or can reasonably be expected to emit any air pollutant shall obtain appropriate authorization from the Department prior to undertaking any activity at the facility or emissions unit for which such authorization is required.

Rule 62-212.300, F.A.C. - General Preconstruction Review Requirements

This rule generally applies to the construction or modification of air pollutant emitting facilities in those parts of the state in which the state ambient air quality standards are being met.

Chapter 62-213, F.A.C.

Because the facility is a Title V source, the applicant shall be required to apply for and obtain a revision to its Title V operation permit in accordance with the applicable provisions of Chapter 62-213, subsequent to the construction authorized by the air construction permit, and demonstration of compliance with the conditions of the air construction permit.

Federal NSPS Provisions

EU131 is not subject to NSPS requirements.

Federal NESHAP Provisions

Since the facility qualifies as a major source of HAPs and does not meet the exemption for research or laboratory facilities: EU131 is subject to 40 CFR 63, Subpart A, National Emission Standards (NESHAP) General Provisions, and to 40 CFR 63, Subpart GG Aerospace Manufacturing and Rework Facilities as per 40 CFR 63.741(a) & (b).

Jacksonville Environmental Protection Board Rules

EU131 is subject to the City of Jacksonville Ordinance Code.  The facility shall be subject to City of Jacksonville Ordinance Code, Title X, Chapter 360 [Environmental Regulation], Chapter 362 [Air and Water Pollution], Chapter 376 [Odor Control], and JEPB Rule 1 [Final Rules with Respect to Organization, Procedure, and Practice].

JEPB Rules.  The facility shall be subject to JEPB Rule 2, Part Nos. I through IV, VI through VII and Part Nos. IX through XIV.

Other Draft Permit Requirements

Performance restrictions:

- The maximum process throughput of abrasive blast media to the blasting operation shall be 1,284,216 pounds per year (lbs/yr).

- The permittee shall only perform the depainting operation in the enclosed blasting booth.  The airsteam from the blast booth shall be passed through the control devices (dust collectors) before discharge to the atmosphere.

The emissions are limited as follows:

- Visible Emissions (VE) shall be limited to 5% opacity.

Test required

- The emissions unit shall be tested initially, annually, and prior to permit renewal to demonstrate compliance with the VE limits.

monitoring requirements:

- The permittee shall operate and maintain the equipment, including but not limited to the abrasive blasting booth, control devices, and filters, according to the manufacturer's specifications and the applicable requirements of 40 CFR 63, Subpart GG.

- The pressure drop across the dry particulate filter(s) shall be continuously monitored via a differential pressure gauge to be installed across the filter banks.

Records and Reports:

- The permittee shall maintain records of the monthly and annual use of abrasive blasting media/material.  The reports shall be submitted semi-annually, and at the request of the compliance authority.

- The permittee shall prepare and submit reports for all VE tests (e.g., initial, annual and prior to permit renewal).

- The pressure drop across the dry particulate filter(s) shall be read and recorded once per shift.

- The permittee shall submit semi-annual reports, including but not limited to, addressing any malfunctions. If in compliance for that 6-month reporting period, a statement signed by the responsible official stating that the operation was in compliance with the applicable standards.

- The permittee shall submit annual reports (every 12 months) that identify: the number of times the pressure drop limit(s) for each filter system were outside the limit(s) specified by the filter or booth manufacturer or in locally prepared operating procedures.

4.	Preliminary Determination

The Department makes a preliminary determination that the proposed project will comply with all applicable state and federal air pollution regulations as conditioned by the draft permit.  This determination is based on a technical review of the complete application, reasonable assurances provided by the applicant, and the conditions specified in the draft permit.  No air quality modeling analysis is required because the project does not result in a significant increase in emissions.  Julia Boesch is the project engineer responsible for reviewing the application and drafting the permit.  Additional details of this analysis may be obtained by contacting the project engineer at the Department’s Northeast District  Office, 8800 Baymeadows Way West, Suite 100, Jacksonville, Florida, 322256, phone (904) 256-1577 or by email Julia.Boesch@FloridaDEP.gov.