### TECHNICAL EVALUATION

&

PRELIMINARY DETERMINATION

#### APPLICANT

Fiberstar, Inc.

1700 County Road 833

Clewiston, Florida 33440

Fiberstar, Inc.

Facility ID No. 0510022

#### PROJECT

# Project No. 0510022-004-AC

Application for Minor Source Air Construction Permit

Fiberstar, Inc.

#### COUNTY

Hendry, Florida

PERMITTING AUTHORITY

Florida Department of Environmental Protection

Air Resource Section

South District Office

2295 Victoria Avenue, Suite 364

P.O. Box 2549

Fort Myers, Florida  33901-2549

April 30, 2013

1.  General Project INFORMATION

Air Pollution Regulations

Projects at stationary sources with the potential to emit air pollution are subject to the applicable environmental laws specified in Section 403 of the Florida Statutes (F.S.).  The statutes authorize the Department of Environmental Protection (Department) to establish regulations regarding air quality as part of the Florida Administrative Code (F.A.C.), which includes the following applicable chapters:  62-4 (Permits); 62-204 (Air Pollution Control – General Provisions); 62-210 (Stationary Sources – General Requirements); 62-212 (Stationary Sources – Preconstruction Review); 62-213 (Operation Permits for Major Sources of Air Pollution); 62-296 (Stationary Sources - Emission Standards); and 62-297 (Stationary Sources – Emissions Monitoring).  Specifically, air construction permits are required pursuant to Rules 62-4, 62-210 and 62-212, F.A.C.

In addition, the U. S. Environmental Protection Agency (EPA) establishes air quality regulations in Title 40 of the Code of Federal Regulations (CFR).  Part 60 specifies New Source Performance Standards (NSPS) for numerous industrial categories.  Part 61 specifies National Emission Standards for Hazardous Air Pollutants (NESHAP) based on specific pollutants.  Part 63 specifies NESHAP based on the Maximum Achievable Control Technology (MACT) for numerous industrial categories.  The Department adopts these federal regulations on a quarterly basis in Rule 62-204.800, F.A.C.

Glossary of Common Terms

Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of this permit.

Facility Description and Location

Fiberstar, Inc. is an existing citrus pulp and peel processing facility, which is categorized under Standard Industrial Classification Code No. 2034.  The facility is located in Hendry County at 1700 County Road 833 in Clewiston, Florida.  The UTM coordinates of the existing facility are Zone 17, 481.6 km East, and 2957.7 km North.  This site is in an area that is in attainment (or designated as unclassifiable) for all air pollutants subject to state and federal Ambient Air Quality Standards (AAQS).

Facility Regulatory Categories

- The facility is not a major source of hazardous air pollutants (HAP).

- The facility has no units subject to the acid rain provisions of the Clean Air Act.

- The facility is not a Title V major source of air pollution in accordance with Chapter 213, F.A.C.

- The facility is not a major stationary source in accordance with Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

- Project Description

This project is a NEW air construction permit, (No. 0510022-004-AC). This project adds an additional process system (System No. 2) to the facility and includes the addition of one (1) Citrus Dryer, (EU008) with one (1) baghouse (EU012). Two (2) electric grinders  (EU009 &  EU010) and one (1) silo (EU011) with baghouse. The grinders emit 3.8 TPY of particulate matter (PM) each and are EXEMPT by generic categorical exemption and the silo emits less than 5.0 TPY and is also EXEMPT. The dryer produces 39.4 TPY particulate matter (PM), 10.68 TPY nitrogen oxides (NOx), 8.97 TPY carbon monoxide (CO), 42.30 TPY volatile organic compounds (VOC) and insignificant amounts of SO2. The dryer Particulate matter (PM) control device consists of a series of small cyclones directed to a final larger cyclone for venting to atmosphere. The particulate matter produced by this new system is much larger in particle size than what is produced by the existing system (system No.1), due to the type of raw citrus material that is being processed, and therefore produces less visible emissions (VE) than system No.1.

The dryer (EU008) is serviced by the existing 8.4 MMBtu/hr boiler (EU005) which is exempt, unregulated by categorical exemption. (The boiler is NOT subject to NESHAP Industrial, Commercial, and Institutional Boilers and Process Heaters (Subpart DDDDD of Part 63) “(Boiler MACT”) or NESHAP for Area Sources, Subpart JJJJJJ of Part 63) (“Boiler Area Source Rule”)

With the addition of this project, the combined TOTAL emissions (TPY) for the facility are:

PM/PM10     SO2       NOx        CO        VOC

Citrus Dryer No. 1            51.07          0.06       25.40       8.97      28.21

Citrus Dryer No. 2            41.30          0.06       25.40       8.97      42.31

Boiler No. 1                         0.27          0.02         8.54        3.02        0.24

Silo No.1                             0.01            N/A        N/A         N/A       N/A

Silo No. 2                            0.01            N/A        N/A         N/A       N/A

Silo No. 3                            0.01            N/A        N/A         N/A       N/A

TOTALS                             92.67          0.14        59.34      20.96     70.76

This project requests unrestricted hours of operation @ 8760 hours per year.

Confidential Information

Certain description of the process and specific documents provided for the purpose of application definition are requested to be CONFIDENTIAL. This specific information is to be kept in DEP confidential files and is NOT subject to public access.

Processing Schedule

April 5, 2013	Received the application for a minor source air pollution construction permit.

2.  PSD Applicability

General PSD Applicability

For areas currently in attainment with the state and federal AAQS or areas otherwise designated as unclassifiable, the Department regulates major stationary sources of air pollution in accordance with Florida’s PSD preconstruction review program as defined in Rule 62-212.400, F.A.C.  Under preconstruction review, the Department first must determine if a project is subject to the PSD requirements (“PSD applicability review”) and, if so, must conduct a PSD preconstruction review.  A PSD applicability review is required for projects at new and existing major stationary sources.  In addition, proposed projects at existing minor sources are subject to a PSD applicability review to determine whether potential emissions from the proposed project itself will exceed the PSD major stationary source thresholds.  A facility is considered a major stationary source with respect to PSD if it emits or has the potential to emit:

- 250 tons per year or more of any regulated air pollutant; or

- 100 tons per year or more of any regulated air pollutant and the facility belongs to one of the following 28 PSD-major facility categories:  fossil fuel-fired steam electric plants of more than 250 million British thermal units per hour heat input, coal cleaning plants (with thermal dryers), Kraft pulp mills, portland cement plants, primary zinc smelters, iron and steel mill plants, primary aluminum ore reduction plants, primary copper smelters, municipal incinerators capable of charging more than 250 tons of refuse per day, hydrofluoric, sulfuric, and nitric acid plants, petroleum refineries, lime plants, phosphate rock processing plants, coke oven batteries, sulfur recovery plants, carbon black plants (furnace process), primary lead smelters, fuel conversion plants, sintering plants, secondary metal production plants, chemical process plants, fossil fuel boilers (or combinations thereof) totaling more than 250 million British thermal units per hour heat input, petroleum storage and transfer units with a total storage capacity exceeding 300,000 barrels, taconite ore processing plants, glass fiber processing plants and charcoal production plants.

Once it is determined that a project is subject to PSD preconstruction review, the project emissions are compared to the “significant emission rates” defined in Rule 62-210.200, F.A.C. for the following pollutants:  carbon monoxide (CO); nitrogen oxides (NOX); sulfur dioxide (SO2); particulate matter (PM); particulate matter with a mean particle diameter of 10 microns or less (PM10); volatile organic compounds (VOC); lead (Pb); fluorides (Fl); sulfuric acid mist (SAM); hydrogen sulfide (H2S); total reduced sulfur (TRS), including H2S; reduced sulfur compounds, including H2S; municipal waste combustor organics measured as total tetra- through octa-chlorinated dibenzo-p-dioxins and dibenzofurans; municipal waste combustor metals measured as particulate matter; municipal waste combustor acid gases measured as SO2 and hydrogen chloride (HCl); municipal solid waste landfills emissions measured as non-methane organic compounds (NMOC); and mercury (Hg).  In addition, significant emissions rate also means any emissions rate or any net emissions increase associated with a major stationary source or major modification which would construct within 10 kilometers of a Class I area and have an impact on such area equal to or greater than 1 μg/m3, 24-hour average.

If the potential emission exceeds the defined significant emissions rate of a PSD pollutant, the project is considered “significant” for the pollutant and the applicant must employ the Best Available Control Technology (BACT) to minimize the emissions and evaluate the air quality impacts.  Although a facility or project may be major with respect to PSD for only one regulated pollutant, it may be required to install BACT controls for several “significant” regulated pollutants.

PSD Applicability for Project

This facility is NOT considered a major stationary source with respect to PSD because it does not emit and does not have the potential to emit 250 tons per year or more of any regulated air pollutant; or 100 tons per year or more of any regulated air pollutant and the facility belongs to one of the following 28 PSD-major facility categories.

As provided in the application, the following table summarizes potential emissions and PSD applicability for the project.

Table A.  Potential Emissions (Tons/Year) and PSD Applicability (This project, System No. 2).

| Pollutant | Project Potential Emissions (TPY) |
| --- | --- |
| CO | 8.97 |
| NOX | 25.40 |
| PM/PM10 | 41.30 |
| SO2 | 0.06 |
| VOC | 42.31 |

As shown in the above table, total project emissions will not exceed the PSD significant emissions rates; therefore, the project is NOT subject to PSD preconstruction review.

3.  Application Review

Discussion of Emissions

In March 2003 the Department found that based on information provided from the facility’s consultant that the facility’s emissions activities complied with the Generic Facility Exemption under 62-210.300, F.A.C..  Fiberstar is located on adjacent contiguous property owned by Southern Gardens.  The facility at the time received all its raw product (pulp) from Southern Gardens and had the same major SIC code.  In March 2003 the Department also found that the facility was NOT under common ownership or control of Southern Gardens (facility ID 0510015).  This was again the opinion in a Department letter dated April 6, 2012 letter.

The previous project (0510022-003-AC) reduced EU001’s permitted capacity and heat input, removed EU002 (no longer proposed to be constructed), removed EU003, EU004, EU006 and EU007 (meets criteria for generic emissions unit exemption) and un-regulates EU005 [meets the criteria for categorical exemption under 62-210.300(3)(a)33., F.A.C.]  This project requests unrestricted hours of operation.  This modification resulted in a decrease in potential emissions for the construction of the initial project (System No. 1).  The potential emissions  of that project (0510022-003-AC) are 51.07 TPY particulate matter, 33.94 TPY nitrogen oxides, 11.99 TPY carbon monoxide and 28.45 TPY volatile organic compounds and insignificant amounts of SO2.

This NEW project is air construction permit, (No. 0510022-004-AC). This project adds an additional process system (System No. 2) to the facility and includes the addition of one (1) Citrus Dryer, (EU008), one (1) baghouse (EU012), two (2) grinders  (EU009 &  EU010) and one (1) silo (EU011) with baghouse. The grinders emit 3.8 TPY of particulate matter (PM) each and are EXEMPT by generic categorical exemption and the silo emits less than 5.0 TPY and is also EXEMPT. The dryer produces 41.3 TPY particulate matter (PM), 25.40 TPY nitrogen oxides (NOx), 8.97 TPY carbon monoxide (CO), 42.31 TPY volatile organic compounds (VOC) and insignificant amounts of SO2. The dryer Particulate matter (PM) control device consists of a series of small cyclones directed to a final larger cyclone and then to a 55,000 cfm rated baghouse for venting to atmosphere. The particulate matter produced by this new system is much larger in particle size than what is produced by the existing system (system No.1), due to the type of raw citrus material that is being processed, and therefore produces less visible emissions (VE) than system No.1, however an alternate of other raw material of smaller particulate matter may be used in this process, and that is why the baghouse is added to the discharge of the primary cyclone.

It is noted that the total particulate matter of both System No. 1 and System No. 2 is computed at 92.67 tpy

and is only 7.33 tpy less that the threshold of 100 tpy that is required for a Title V classification.

4.  Preliminary Determination

The Department makes a preliminary determination that the proposed project will comply with all applicable state and federal air pollution regulations as conditioned by the draft permit.  This determination is based on a technical review of the complete application, reasonable assurances provided by the applicant, and the conditions specified in the draft permit.  Carter B. Endsley, PE. is the project engineer responsible for reviewing the application and drafting the permit.  Additional details of this analysis may be obtained by contacting the project engineer at the Department’s South District Office at 2295 Victoria Avenue, Suite 364, Fort Myers, Florida  33902.