Permittee

| Riverview Facility Mosaic Fertilizer, LLC 13830 Circa Crossing Drive Lithia, Florida 33547 Authorized Representative: Mr. Robert Fredere, Plant Manager | Permit No. 0570008-080-AC Permit Expires: August 1, 2018 Riverview Facility SO2 Emissions Reduction Project Hillsborough County |
| --- | --- |

Project

This is the final air construction permit for the SO2 Emissions Reduction Project at the Riverview Facility, which include physical and operational changes to reduce ambient SO2 emissions and ambient impacts from the facility.

The Riverview Facility is an existing phosphate fertilizer manufacturer categorized under Standard Industrial Classification Number (No.) 2874.  The existing facility is located in Hillsborough County at 8813 U.S. Highway 41 South in Riverview, Florida.  UTM Coordinates are:  Zone 17, 363.23 kilometers (km) East and 3082.56 km North.  Latitude is:  27o51’39” North; and, Longitude is:  82o23’21” West.

This final permit is organized into the following sections:  Section 1 (General Information); Section 2 (Administrative Requirements); Section 3 (Emissions Unit Specific Conditions); and Section 4 (Appendices).  Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix CF of Section 4 of this permit

Statement of Basis

This air pollution construction permit is issued under the provisions of:  Chapter 403 of the Florida Statutes (F.S.) and Chapters 62-4, 62-204, 62-210, 62-212, 62-296 and 62-297 of the Florida Administrative Code (F.A.C.).  This project is subject to the general preconstruction review requirements in Rule 62-212.300, F.A.C. and is not subject to the preconstruction requirements for major new source review in Chapter 62-212, F.A.C.

Upon issuance of this final permit, any party to this order has the right to seek judicial review of it under Section 120.68 of the Florida Statutes by filing a notice of appeal under Rule 9.110 of the Florida Rules of Appellate Procedure with the clerk of the Department of Environmental Protection in the Office of General Counsel (Mail Station #35, 3900 Commonwealth Boulevard, Tallahassee, Florida, 32399-3000) and by filing a copy of the notice of appeal accompanied by the applicable filing fees with the appropriate District Court of Appeal.  The notice must be filed within 30 days after this order is filed with the clerk of the Department.

Executed in Tallahassee, Florida

for:	Jeffery F. Koerner, Program Administrator

Office of Permitting and Compliance

Division of Air Resource Management

JFK/dlr/sms

## CERTIFICATE OF SERVICE

The undersigned duly designated deputy agency clerk hereby certifies that this final air permit package (including the Final Determination and Final Permit with Appendices) was sent by electronic mail, or a link to these documents made available electronically on a publicly accessible server, with received receipt requested before the close of business on the date indicated below to the following persons.

Mr. Robert Fredere, Mosaic Fertilizer, LLC:  robert.fredere@mosaicco.com

Mr. Rama Iyer, P.E., Mosaic Fertilizer, LLC:  rama.iyer@mosaicco.com

Mr. Jeff Stewart, Mosaic Fertilizer, LLC:  jeff.stewart@mosaicco.com

Ms. Kelley Boatwright, DEP SWD:  kelley.boatwright@dep.state.fl.us

Ms. Heather Ceron, U.S. EPA Region 4:  ceron.heather@epa.gov

# Ms. Diana Lee, P.E., HCEPC:  lee@epchc.org

Mr. Thomas G. Rogers, DEP OBP:  tom.rogers@dep.state.fl.us

# Clerk Stamp

# FILING AND ACKNOWLEDGMENT FILED, on this date, pursuant to Section 120.52(7), Florida Statutes, with the designated agency clerk, receipt of which is hereby acknowledged.

Facility Description

This existing Mosaic Riverview facility consists of several industrial processes that convert insoluble rock containing phosphorus ore into a soluble form suitable for agricultural use.  The processes consist of one phosphoric acid plant (two trains), two ammoniated phosphate (AP) plants, three sulfuric acid plants (SAP), one material handling system, one auxiliary boiler, two animal feed plants, a molten sulfur storage and handling system and emergency compression ignition reciprocating internal combustion engines.  This facility consists of the emissions units shown below.  The emission units affected by this permitting action are highlighted in yellow.

LIST OF emission units.

| EU No. |  | ## Brief Description |
| --- | --- | --- |
| Regulated Emissions Units |  |  |
| 004 | No. 7 Sulfuric Acid Plant |  |
| 005 | No. 8 Sulfuric Acid Plant |  |
| 006 | No. 9 Sulfuric Acid Plant |  |
| 007 | No. 6 AP Plant |  |
| 051 | Conveyor No. 9 Transfer Points and Railcar Unloading |  |
| 052 | Conveyor No. 9 to Shipping Belt Conveyor |  |
| 053 | Vessel Loading Operation |  |
| 055 | No. 5 Granulation Plant |  |
| 058 | Conveyor No. 6 to Conveyor No. 7 |  |
| 059 | Conveyor No. 7 to Conveyor No. 8 |  |
| 060 | Screening Tower and Conveyor No. 8 to Conveyor No. 9 |  |
| 061 | East Vessel Loading Facility - Shiphold/Chokefeed |  |
| 063 | Molten Sulfur Storage and Handling System – Tank #1, 2 and 3 |  |
| 066 | Molten Sulfur Storage and Handling System -- Pit #7 |  |
| 067 | Molten Sulfur Storage and Handling System -- Pit #8 |  |
| 068 | Molten Sulfur Storage and Handling System -- Pit #9 |  |
| 073 | Phosphoric Acid Production Facility |  |
| 074 | Molten Sulfur Storage and Handling System -- Truck Loading Station |  |
| 075 | AP Storage Building Nos. 2, 4, 5 & 6 |  |
| 076 | Loadout between Buildings 2 & 4 |  |
| 077 | Loadout adjacent to Building 6 |  |
| 078 | Animal Feed Ingredient (AFI) Plant No. 1 |  |
| 079 | Diatomaceous Earth (DE) Silo |  |
| 080 | Limestone Silo |  |
| 081 | Animal Feed Plant (AFI) Loadout System |  |
| 103 | Animal Feed Ingredient (AFI) Plant No. 2 |  |
| 104 | Phosphogypsum Stack North (No. 1) |  |
| 108 | Phosphogypsum Stack South (No 2.) |  |
| 109 | Clarifier and Storage Tanks |  |
| 111 | Existing Emergency Stationary RICE < or equal to 500 HP |  |
| 112 | Auxiliary Steam Boiler |  |
| 113 | Non-Emergency CI ICE |  |
| Miscellaneous Emissions Units and Activities |  |  |
| 105 | Facility-Wide Fugitive Emissions: - Sulfur dioxides (SO2), sulfur trioxides (SO3), sulfuric acid mist (SAM) emissions from the 7, 8 & 9 Sulfuric Acid Plants - Fluoride emissions from the Phosphoric Acid Plant - Fluoride, ammonia (NH3), particulate matter (PM) emissions from the Nos. 5 & 6 Granulation Plants - Hydrogen Fluoride emissions from the Phosphogypsum Stacks and Cooling Ponds Note: For this emissions unit, Annual Operation Report (AOR) emissions estimates are required only for the Hydrogen Fluoride emissions from the Phosphogypsum Stacks and Cooling Ponds. |  |

PROPOSED PROJECT

The purpose of the project is to reduce SO2 emissions and ambient impacts from the facility.  Specifically, the permit requires:

- Elimination of the use of oil at the plant except during periods of natural gas curtailment or disruption.

- Changing/augmenting the catalysts in the converters in SAP Nos. 7, 8 and 9, which will lower SO2 emissions while not increasing sulfuric acid mist (SAM) emissions.  Note that the existing permitted capacities of the plants will remain the unchanged.

- Increasing the stack height of each SAP to no lower than 65 meters (213.25 feet) which is equivalent to approximately a 60-foot increase per stack.

- Comply with specific SO2 emissions standards and caps based on a 24-hour block average as determined by CEMS data.

Facility Regulatory Classification

- The existing facility is a major source of HAP.

- The existing facility is a Title V major source of air pollution in accordance with Chapter 62-213, F.A.C.

- The existing facility is a major stationary source in accordance with Rule 62-212.400 (PSD), F.A.C.

- This facility does not operate units subject to the acid rain provisions of the Clean Air Act (CAA)

- The facility operates units that are subject to the New Source Performance Standards (NSPS) at 40 Code of Federal Regulations, Part 60 (40 CFR 60), and the National Emissions Standards for Hazardous Air Pollutants (NESHAP) at 40 CFR 63.

.

- Permitting Authority:  The permitting authority for this project is the Office of Permitting and Compliance, Division of Air Resource Management, Florida Department of Environmental Protection (Department).  The mailing address for the Office of Permitting and Compliance is 2600 Blair Stone Road (MS #5505), Tallahassee, Florida 32399-2400.

- Compliance Authority:  All documents related to compliance activities such as reports, tests, and notifications shall be submitted to the Compliance Authority, the Environmental Protection Commission of Hillsborough County (EPCHC).  The Compliance Authority’s mailing address is:

Environmental Protection Commission Hillsborough County

3629 Queen Palm Drive

Tampa, Florida 33619

Telephone:  813/627-2600, Fax:  813/627-2660

- Appendices:  The following Appendices are attached as a part of this permit and the permittee must comply with the requirements of the appendices:

- Appendix CC	Common Conditions;

- Appendix CF	Citation Formats and Glossary of Common Terms; and

- Appendix GC	General Conditions.

- Applicable Regulations, Forms and Application Procedures:  Unless otherwise specified in this permit, the construction and operation of the subject emissions units shall be in accordance with the capacities and specifications stated in the application.  The facility is subject to all applicable provisions of: Chapter 403, F.S.; and Chapters 62-4, 62-204, 62-210, 62-212, 62-213, 62-296 and 62-297, F.A.C.  Issuance of this permit does not relieve the permittee from compliance with any applicable federal, state, or local permitting or regulations.

- New or Additional Conditions:  For good cause shown and after notice and an administrative hearing, if requested, the Department may require the permittee to conform to new or additional conditions.  The Department shall allow the permittee a reasonable time to conform to the new or additional conditions, and on application of the permittee, the Department may grant additional time.  [Rule 62-4.080, F.A.C.]

- Modifications:  No emissions unit shall be constructed or modified without obtaining an air construction permit from the Department.  Such permit shall be obtained prior to beginning construction or modification.  [Rules 62-210.300(1) and 62-212.300(1)(a), F.A.C.]

- Title V Permit:  This permit authorizes specific modifications and/or new construction on the affected emissions units as well as initial operation to determine compliance with conditions of this permit.  A Title V operation permit is required for regular operation of the permitted emissions units.  The permittee shall apply for a Title V operation permit at least 90 days prior to expiration of this permit.  To apply for a Title V operation permit, the applicant shall submit the appropriate application form, compliance test results, and such additional information as the Department may by law require.  The application shall be submitted to the appropriate Permitting Authority.  [Rules 62-4.030, 62-4.050, 62-4.220, and Chapter 62-213, F.A.C.]

- Objectionable Odors Prohibited:  No person shall cause, suffer, allow or permit the discharge of air pollutants which cause or contribute to an objectionable odor.  [Rule 62-296.320(2), F.A.C.]

{Note:  An objectionable odor is defined in Rule 62-210.200(Definitions), F.A.C., as any odor present in the outdoor atmosphere which by itself or in combination with other odors, is or may be harmful or injurious to human health or welfare, which unreasonably interferes with the comfortable use and enjoyment of life or property, or which creates a nuisance.}

- Unconfined Emissions of Particulate Matter:  No person shall cause, let, permit, suffer or allow the emissions of unconfined particulate matter from any activity, including vehicular movement; transportation of materials; construction, alteration, demolition or wrecking; or industrially related activities such as loading, unloading, storing or handling; without taking reasonable precautions to prevent such emissions.  Any permit issued to a facility with emissions of unconfined particulate matter shall specify the reasonable precautions to be taken by that facility to control the emissions of unconfined particulate matter.  General reasonable precautions include the following: a. Paving and maintenance of roads, parking areas and yards; b. Application of water or chemicals to control emissions from such activities as demolition of buildings, grading roads, construction, and land clearing; c. Application of asphalt, water, oil, chemicals or other dust suppressants to unpaved roads, yards, open stock piles and similar activities; d. Removal of particulate matter from roads and other paved areas under the control of the owner or operator of the facility to prevent re-entrainment, and from buildings or work areas to prevent particulates from becoming airborne; e. Landscaping or planting of vegetation; f. Use of hoods, fans, filters, and similar equipment to contain, capture and/or vent particulate matter; g. Confining abrasive blasting where possible; and h. Enclosure or covering of conveyor systems. 
[Rule 62-296.320(4)(c), F.A.C.]

Previous Applicable Requirements

- Effect on Other Permits:  The conditions of this permit supplement all previously issued air construction and operation permits for these emissions units.  Unless otherwise specified, these conditions are in addition to all other applicable permit conditions, rules and regulations.  [Rule 62-4.070(1) & (3), Reasonable Assurance, F.A.C.]

This subsection of the permit addresses the following emission units:

| EU No. | Brief Description |
| --- | --- |
| 004 | No. 7 Sulfuric Acid Plant |
| 005 | No. 8 Sulfuric Acid Plant |
| 006 | No. 9 Sulfuric Acid Plant |
| Descriptions: Sulfuric Acid Plants (SAPs) Nos. 7, 8 and 9 have a design capacity of 3,200 tons per day (TPD), 2,700 TPD, and 3,400 TPD of 100% sulfuric acid, respectively. Each SAP recovers a portion of the waste heat (steam) for process use and to generate electricity. Waste heat recovery reduces plume visibility. There are two electrical generators at each SAP, rated at 35 megawatts (MW) and 36 MW for a total of 71 MW. These plants are sulfur burning, double-conversion, and double-absorption plants based on the Leonard-Monsanto design. Sulfur is burned with dried atmospheric oxygen to produce sulfur dioxide (SO2). The sulfur dioxide is catalytically oxidized to sulfur trioxide (SO3) over a catalyst bed. The sulfur trioxide is then absorbed in sulfuric acid. The remaining sulfur dioxide, not previously oxidized, is passed over a final converter bed of catalyst and the sulfur trioxide produced is then absorbed in sulfuric acid. SAP Nos. 7, 8 and 9 began operating in 1961, 1965 and 1974, respectively. Air Pollution Control Systems and Measures: The control of SO2 emissions is primarily by the process itself. Currently, a double-conversion, double-absorption plant efficiently converts SO2 to SO3 then SO3 reacts in a mixture of water and sulfuric acid to produce sulfuric acid. In a double-absorption system, the conversion efficiency from SO2 to SO3 is at least 99.7%. All three plants currently use vanadium and cesium catalysts in the converters. Monitors: Each SAP is equipped with an existing SO2 continuous emissions monitoring system (CEMS). The SO2 CEMS are required by the Standards of Performance for New Stationary Sources (NSPS). Stack Parameters: Emissions not absorbed by each double absorption system are currently vented through 150-foot tall stack for each SAP. The stack exhaust gas characteristics for SAP Nos. 7, 8 and 9 are, respectively: exhaust gas temperatures of 170° F, 150° F and 152 ºF; exhaust gas flow rates of 122,000, 105,000 and 149,000 actual cubic feet per minute and stack diameters of 7.5, 8.0 and 9.0 feet. |  |

{Permitting notes:  The SAPs are currently regulated under NSPS 40 CFR 60, Subpart H, Standards of Performance for Sulfuric Acid Plants, adopted and incorporated by reference in Rule 62-204.800(7)(b)10., F.A.C.; Rule 62-212.400, F.A.C., Prevention of Significant Deterioration (PSD), PSD-FL-209 (AC29-241660) - 1st BACT for SAP Nos. 8 and 9, PSD-FL-250 (0570008-025-AC) - 1st BACT for SAP No. 7, PSD-FL-315 (0570008-036-AC) - 2ndBACT for SAP Nos. 8 and 9; Rule 62-296.402, F.A.C., Sulfuric Acid Plants; and Rule 62-296.320, F.A.C., General Pollutant Emission Limiting Standards.}

Physical Changes

- Stack Height Increase:  The SAP Nos. 7, 8 and 9 all have stacks that are currently 150 feet high, with diameters of 7.5, 8.0 and 9.0 feet, respectively.  In accordance with the work schedule in Condition 3 of this subsection, all three stack heights shall be increased to a height not less than 65 meters (213.25 feet), with the discharge diameters remaining essentially the same or less at the exhaust point (same or better exhaust characteristics for dispersion).  Within 45 days of commencing operation with the increased stack heights, the permittee shall notify the Division and the Compliance Authority that the work is complete and provide the final stack dimensions and design.  [Rules 62-4.070(1) and (3) and 62-4.080, F.A.C.; and SO2 Attainment SIP]

- Converter Catalyst:  All three SAPs are double-absorption plants with a four catalyst bed converter and waste heat boiler, cold and hot pass heat exchangers, economizers, and heat recovery systems.  In accordance with the work schedule specified in Condition 3 of this subsection, the permittee shall change/augment the catalyst to comply with the SO2 emissions caps specified in Conditions 4, 5, and 6 of this subsection.  The permitted SAP capacities will remain unchanged; the purpose of the catalyst work is to reduce SO2 emissions and ambient impacts.  Within 45 days of commencing operation following the turnaround (including catalyst installation and arrangement for each SAP), the permittee shall provide the following information to the Division and the Compliance Authority:  the type of catalyst; the amount of catalyst, the catalyst arrangement within the convertor; and the initial expected SO2 emissions rate in “lb/ton of 100% sulfuric acid produced” to be achieved by the catalyst formulation.  [Rules 62-4.070(1) and (3) and 62-4.080, F.A.C.; and SO2 Attainment SIP]

- Work Schedule:  The permittee shall conduct the required work in accordance with the following schedule, which is based on the facility’s planned turnarounds.

| Date Completed | SAP | SAP Project Work - Activity |
| --- | --- | --- |
| November 2014 | SAP No. 9 (EU No. 006) | Catalyst change/augmentation |
| November 2015 | SAP No. 8 (EU No. 005) | Catalyst change/augmentation Increase stack height to a minimum of 213.25 feet |
| November 2016 | SAP No. 7 (EU No. 004) | Catalyst change/augmentation Increase stack height to a minimum of 213.25 feet |
| November 2017 | SAP No. 9 (EU No. 006) | Increase stack height to a minimum of 213.25 feet |

Prudent planning will allow the permittee to conduct the catalyst changes/augmentation by the deadlines specified above.  With regard to the stack height increases, the permittee shall notify the Department as soon as possible regarding any issues with the design, materials, labor, etc. that would cause substantive delays in meeting the given deadline.  The Department may approve alternative deadlines for the stack height increases.  [Rules 62-4.070(1) and (3) and 62-4.080, F.A.C.; SO2 Attainment SIP]

SO2 Emission Standards and Caps for SAPs

- Individual SAP Emissions Standards:  As determined by existing CEMS data, each SAP shall continue to comply with the following individual emissions standards, which are enforceable standards that allowed these units to avoid Rule 62-296.340(5)(c), F.A.C. (BART):

- SAP No. 7:  400 lb SO2/hour, 24-hour block average (midnight to midnight).

- SAP No. 8:  315 lb SO2/hour, 24-hour block average (midnight to midnight).

- SAP No. 9:  425 lb SO2/hour, 24-hour block average (midnight to midnight).

These emissions standards apply at all times including periods when two or three SAPs are in operation.  [Rule 62-296.340(5)(c), F.A.C. and Permit No. 0570008-061-AC to avoid BART; and SO2 Attainment SIP]

- Emission Cap when Two SAPs are Operating:  Effective December 15, 2016, when any two SAPs operate within a 24-hour block averaging period and the third SAP is not in operation, the following SO2 emissions cap also applies:  550 lb SO2/hour, 24-hour block average (6:00 am to 6:00 am).  Note that the individual emissions standards in Condition 4 of this subsection remain in effect.  {Permitting Note:  This new emissions cap reduces SO2 emissions and ambient impacts in and around the SO2 non-attainment area in Hillsborough County.} [Rules 62-4.070(1) and (3) and 62-4.080, F.A.C.; and SO2 Attainment SIP]

- Emission Caps when Three SAPs are Operating:  Effective December 15, 2016, when all three SAPs are in operation within the same 24-hour block averaging period, the following SO2 emission cap also applies:  
575 lb SO2/hour, 24-hour block average (6:00 am to 6:00 am).  Note that the individual emissions standards in Condition 4 and the emissions cap for two operating SAPs in Condition 5 of this subsection remain in effect.  {Permitting Note:  This new emissions cap reduces SO2 emissions and ambient impacts in and around the SO2 non-attainment area in Hillsborough County.}  [Rules 62-4.070(1) and (3) and 62-4.080, F.A.C.; and SO2 Attainment SIP]

- Previous SO2 Emission Limits:  These new SO2 limits and caps are in addition to previous permit limits.  The SO2 emissions standards and caps specified in this permit cannot vary or waive any SO2 emissions standards related to any previous NSPS, NESHAP, maximum achievable control technology (MACT), best available control technology (BACT), prevention of significant deterioration (PSD), state implementation plan (SIP) requirements, or any previously issued air construction permit requirements.  [Rules 62-4.070(1) and (3) and 62-4.080, F.A.C.; and SO2 Attainment SIP]

Compliance Demonstration

- Continuous Compliance Demonstration:  The permittee shall demonstrate continuous compliance with the SO2 emissions standards and caps established in Specific Conditions 4, 5, and 6 of this subsection based on data collected by the existing SO2 CEMS.  The emissions standards and caps apply during all periods of operation including startup and shutdown.  [Rules 62-4.070(3) and 62-4.080, F.A.C.; and SO2 Attainment SIP]

- SO2 CEMS Requirements:  The existing SO2 CEMS shall comply with the quality assurance and quality control requirements specified in the most recent Title V air operation permit.  [Rules 62-4.070(1) and (3) and 62-4.080, F.A.C.; and SO2 Attainment SIP]

- Semi-Annual Progress Reports:  The permittee shall provide semiannual progress reports for the six-month periods ending June 30th and December 31st of each year on the status of the physical and operational changes required by the this permit.  Reports shall be submitted to the Division and Compliance Authority within 30 days following the reporting period.  The first progress report is due by January 30, 2015 and the last progress report is due by January 30, 2018.  [Rules 62-4.070(1) and (3) and 62-4.080, F.A.C.; and SO2 Attainment SIP]

Ongoing Compliance and Test Reports

- Semiannual SO2 CEMS Data Reports:  The permittee shall submit semiannual emissions reports for the six-month periods ending June 30th and December 31st of each year summarizing the SO2 data and demonstrating compliance with the SO2 emissions standards and caps specified in Specific Conditions 4, 5, and 6 of this subsection.  Reports shall be submitted within 30 days following the six-month reporting period.  The first report is due by July 30, 2018.  Each report shall summarize each 24-hour block average SO2 emissions rates showing compliance with the standards and caps during the reporting period along with any background information necessary to explain the emissions.  [Rules 62-4.070(1) and (3) and 62-4.080, F.A.C.; and SO2 Attainment SIP]

- Emissions Exceedance Reporting:  Within one business day of occurrence, the permittee shall notify the Compliance Authority of any exceedance of the SO2 emissions standards and/or caps.  Within 15 days of occurrence, the permittee shall submit a report to the Compliance Authority detailing the exceedance, identifying the likely cause, describing any corrective actions taken, and noting when the unit was returned to compliance.  This reporting requirement is effective December 15, 2016.  [Rules 62-4.070(1) and (3) and 62-4.080, F.A.C.; and SO2 Attainment SIP]

This subsection of the permit addresses the following emission units:

| EU No. | Brief Description |  |
| --- | --- | --- |
| 004 | No. 7 Sulfuric Acid Plant |
| 005 | No. 8 Sulfuric Acid Plant |
| 006 | No. 9 Sulfuric Acid Plant |
| 007 | No. 6 AP Plant |
| 055 | No. 5 Granulation Plant |

Fuel Oil Limitation

- Elimination of Fuel Oil Use:  Effective July 1, 2015, the Mosaic Riverview facility shall cease firing fuel oil in Emission Units 004, 005, 006, 007 and 055 except during periods of natural gas curtailment or disruption.  [Rules 62-4.070(1) and (3) and 62-4.080, F.A.C.; and SO2 Attainment SIP]